3GPP TSG-RAN WG1 Meeting #104-e	R1-2100930

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.15.1

Source:	Ericsson

Title:	On scenarios and evaluations for eMTC and NB-IoT based NTN

Document for:	Discussion

# 1	Introduction

In RAN#86, a Rel-17 Study Item (SI) on IoT NTN was approved to evaluate the feasibility of NTN for NB-IoT and eMTC and the Study Item Description (SID) is updated in [1]. It was agreed to use the existing work on NR NTN captured in TR 38.821 [2] as a baseline. The first objective of this study is as follows.

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

In this contribution, we present our views on scenarios relevant for IoT NTN and discuss what evaluations are needed to verify the feasibility of NTN for eMTC and NB-IoT.

# 2	Background

3GPP has actively worked on MTC related improvements for LTE already since Release 8. Delay-tolerant access radio resource control (RRC) establishment was introduced in LTE Release 10. It allows the network to deprioritize connection requests from delay tolerant UEs. Rel-11 specified enhanced access barring (EAB), which allows barring of delay tolerant UEs in an overload scenario. In Rel-12 the power-saving mode (PSM) feature was specified. It allows a UE to remain registered to a network while powering down. Together with the Rel-13 extended DRX (eDRX) feature, PSM allows UEs to optimize their time spent in the power efficient RRC Idle mode.

In Rel-13, 3GPP specified eMTC and NB-IoT to support the massive machine type communications (mMTC) use case. The mMTC use case is characterized by requirements such as support of a massive number of UEs, low UE complexity to provide low UE cost, long UE battery life to limit the need for battery charging and replacement, and coverage enhancements to provide ubiquitous coverage.

## 2.1	eMTC

3GPP Rel-12 initiated the work on eMTC, also often referred to as LTE-M [4], and specified the first low-complexity UE category 0 (Cat-0). Cat-0 supports a reduced peak data rate of 1 Mbps, single antenna and half-duplex frequency-division duplex (HD-FDD) operation.

In Rel-13, the work accelerated with the introduction of the Cat-M1 UE category. It supports a further reduced complexity, and coverage enhanced (CE) operation. The additional cost reduction came from a reduced transmission and reception bandwidth of 1.08 MHz, equivalent to six 180 kHz physical resource blocks (PRBs). The introduction of a lower UE power class of 20 dBm, in addition to the 23 dBm power class, further facilitates a lower UE complexity.

Because of the reduction in bandwidth, a new narrowband physical downlink control channel, the MTC physical downlink control channel (MPDCCH), was introduced as a substitute for the wideband legacy physical downlink control channel (PDCCH) and the Enhanced PDCCH (EPDCCH). The Cat-M1 UEs monitor MPDCCH in a narrowband (NB), which is defined by 6 adjacent PRBs.

eMTC supports an MCL that is 20 dB larger than the normal MCL of LTE. This is achieved mainly through time repetition and a relaxed acquisition time of the physical channels and signals. The primary and secondary synchronization signals (PSS and SSS) are fully reused from LTE and extended coverage is achieved by means of an increased acquisition time.

For the physical broadcast channel (PBCH), the MPDCCH, the physical uplink control channel  (PUCCH) and the data channels, that is, the physical uplink shared channel (PUSCH) and physical downlink shared channel (PDSCH), the desired coverage enhancement is achieved through so-called time repetition of a transmission block.

In LTE Releases 14 and 15, eMTC was further enhanced to support a more diversified set of applications and services. A new UE category Cat-M2 was e.g. specified. To support power efficient UE operation, since Rel-15 eMTC supports uplink transmissions using 2-of-3, 3, 6 or 12 sub-carriers. The performance of eMTC Rel-15 meets the IMT-2020 5G requirements for the massive IoT use case.

The work in 3GPP on eMTC was continued in Rel-16 and is further evolved also in Rel-17.

## 2.2	NB-IoT

The work on NB-IoT- started in Rel-13 [5]. Compared to eMTC, the target was even lower UE complexity, and a design facilitating high deployment flexibility.

This resulted in a system design that in short can be described as a narrowband version of LTE. A NB-IoT downlink carrier is defined by 12 OFDM sub-carriers, each of 15 kHz, giving a total baseband bandwidth of 180 kHz. This design gives NB-IoT a high deployment flexibility: the system can operate standalone, in the guardband of an LTE carrier or within an LTE carrier.

Similar to eMTC, NB-IoT also makes use of increased acquisition times and time repetitions to extend the system coverage. The repetitions can be seen as a third level of retransmissions added at the physical layer as a complement to those at MAC HARQ and RLC ARQ.

NB-IoT supports anchor and non-anchor carriers. The anchor carrier supports synchronization to the downlink frame structure, system information transmissions and mobile terminated and originated system access in addition to control and data transmissions. The system capacity may be increased through the deployment of non-anchor carriers. These support control and data transmissions and since Rel-14 mobile terminated and originated system access.

To support power efficient UE operation, NB-IoT supports uplink transmissions using 1, 3, 6 or 12 sub-carriers. The single-subcarrier modulation supports a close to constant-envelope waveform which enables power efficient operation. The small scheduling granularity also supports a high uplink capacity when the system is operating in extended coverage.

# 3	Scenarios for IoT NTN

## 3.1	Cellular IoT evolution for NTN

Typical uses cases for eMTC include connected vehicles, wearable devices, trackers and alarm panels. Most common use cases of NB-IoT include utility meters and sensors. eMTC and NB-IoT are suitable for different applications. For example, if one has an oil tank in the basement of a building that needs a sensor to check its level from time to time, NB-IoT will be the choice (the elevator servicing that basement, however, will use eMTC). From an operator perspective, NB-IoT also creates more deployment flexibility due to guard-band deployment. If the operator’s available frequency assets allow, NB-IoT can also be deployed as stand-alone access.

eMTC and NB-IoT are complementary technologies that can address different types of IoT use cases based on their unique capabilities. NB-IoT supports ultra-low complexity devices with very narrow bandwidth, while eMTC can achieve higher data rates, more accurate device positioning, and supports voice calls and connected mode mobility. Both are considered future-proof and viewed as 5G technologies. They can efficiently co-exist with 5G NR in the same spectrum and already fulfil all 5G massive MTC requirements.

The approved Rel-17 IoT NTN SID mentions LEO and GEO, but not other NTN systems such as HAPS/HIBS and A2G (which are mentioned in the Rel-17 NR NTN WI). With this observation, it is our understanding that the  Rel-17 IoT NTN SI  focuses on LEO and GEO systems.

Additionally, the Rel-17 IoT NTN SI should focus on essential adaptations to minimize additional network and UE complexity and benefit from the economy of scale of the existing IoT ecosystem. This also means that the enhancements should be limited to adapting eMTC and NB-IoT to NTN, while generic enhancements motivated by non-NTN are outside the scope.

Lastly, the IoT NTN should support connectivity to EPC as the baseline. This is a low hanging fruit and can reduce time-to-market. This was agreed at RAN2#112e which assumes support for EPC, while support for 5GC is TBD.

eMTC and NB-IoT can address different types of IoT use cases based on their unique capabilities and thus complement each other.

NB-IoT supports ultra-low complexity devices with very narrow bandwidth, while eMTC can achieve higher data rates, more accurate device positioning, and supports voice calls and connected mode mobility.

The approved Rel-17 IoT NTN SID is dedicated to LEO and GEO satellite communication, while HAPS/HIBS and A2G are not in the scope.

Rel-17 IoT NTN study should equally treat eMTC and NB-IoT. The study item will be incomplete unless each of them is properly studied for its feasibility for NTN.

It was agreed at RAN2#112e that support for EPC is assumed for IoT NTN.

IoT NTN study should focus on essential adaptations for NTN, while generic enhancements motivated by non-NTN are outside the scope.

## 3.2	Bands of interest in sub 6 GHz

The SID clearly indicates that the bands of interest for IoT NTN are in sub 6 GHz, which is a sensible target because both eMTC and NB-IoT have only been designed for sub 6 GHz. For bands above 6 GHz, significant changes are required for adapting eMTC and NB-IoT for NTN, which is neither a desirable evolution direction nor in the scope of IoT NTN SI.

Regarding which specific bands in sub 6 GHz are of interest, this will be a topic for RAN4 to discuss in a potential normative phase. For the purpose of this SI, it is sufficient to consider the nominal S band for various evaluation purposes, as was done during the Rel-16 NR NTN SI.

Additionally, the focus can be limited to FDD, as TDD is inefficient in NTN. This is consistent with the recommendation in the Rel-17 NR NTN WI.

Identifying specific bands of interest in sub 6 GHz can be a topic for RAN4 to discuss when a potential normative phase begins.

In Rel-17 IOT NTN SI, consider nominal S band (2 GHz) for evaluation purposes.

In Rel-17 IOT NTN SI, limit the focus to FDD only.

## 3.3	Architecture

The SID clearly indicates that the focus is on transparent payload, i.e., bent-pipe architecture, which is sensible and can help time-to-market.

In addition, we would like to suggest focusing on earth fixed beams for IoT NTN to limit the impact on specifications and implementation. As it has become evident from the discussions in both the Rel-16 NR NTN SI and the ongoing Rel-17 NR NTN WI, supporting earth moving beams is complicated and involves many challenges that are difficult for NR already, let alone the supposedly much simpler eMTC and NB-IoT devices.

The approved Rel-17 IoT NTN SID is dedicated to transparent payload.

In Rel-17 IOT NTN SI, prioritize earth fixed beams.

# 4	Evaluations for IoT NTN

eMTC and NB-IoT target the massive IoT sector, which is the deployment of an immense number of low-complexity devices that do not need to communicate with great frequency. Performance does not need to be high, and low transmission latency is not a requirement. Typical use cases include low-cost sensors, meters, wearables and trackers. Many of these can be deployed in challenging radio environments, such as a basement of a building or on a moving piece of machinery, and in certain scenarios will be relied upon to send occasional signals for up to 10 years, without a change of battery. This makes power consumption and conservation critical aspects.

Note that eMTC and NB-IoT were carefully studied at the initial design stage to ensure they could meet the various design targets, see 3GPP TR 36.888 [6] TR 45.820 [7]. Important design targets include:

- Improved coverage to support 164 dB MCL.

- Support of a massive number of low throughput devices

  - A system that can support a large number of devices, each generating a small amount of data is required. The original traffic model was based on TR 36.888. In Rel-15, both eMTC and NB-IoT were further evaluated and shown to fulfil 5G device density requirement of 1,000,000 devices per square km.

- Reduced complexity and cost:

  - MTC applications require devices that are very cheap (so that they can be deployed on a mass scale or in a disposable manner), requiring techniques that significantly reduce complexity and cost, e.g., reduced device bandwidth, small number of antennas, etc.

- Power consumption:

  - The power consumption of MTC devices should be reduced to support up to ten years of battery life with a battery capacity of 5 Wh (Watt-hours), even in locations with adverse coverage conditions with up to 164 dB MCL.

- Latency:

  - MTC devices may in general support relaxed delay characteristics. Certain applications (e.g., alarms) may however require a reasonably strict delay profile. For devices supporting such applications, a delay requirement of 10 seconds is appropriate for the uplink when measured from the application 'trigger event' to the packet being ready for transmission from the base station towards the core network.

To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for NB-IoT and eMTC in the new context of NTN, taking into account factors such as the additional complexity, cost, and power consumption associated with GNSS operation.

Additionally, the study item description clearly indicates that device types with PC3 and PC5 are in the scope and thus it will be necessary to evaluate both device types.

In our companion contribution [8], we discuss in more detail about evaluation assumptions for  eMTC and NB-IoT based NTN, and provide some initial evaluation results including link budget calculations.

To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for eMTC and NB-IoT in the new context of NTN, taking into account factors such as the additional complexity, cost, and power consumption associated with GNSS operation.

In Rel-17 IOT NTN SI, evaluate eMTC and NB-IoT in the context of NTN at least for the following targets: (1) coverage performance through link budget analysis; (2) supported device density; (3) complexity and cost of equipping eMTC/NB-IoT devices with NTN capability; (4) power consumption performance of eMTC/NB-IoT devices with NTN connectivity; and (5) latency performance of eMTC/NB-IoT devices in NTN systems.

# Conclusion

In the previous sections we made the following observations:

Observation 1	eMTC and NB-IoT can address different types of IoT use cases based on their unique capabilities and thus complement each other.

Observation 2	NB-IoT supports ultra-low complexity devices with very narrow bandwidth, while eMTC can achieve higher data rates, more accurate device positioning, and supports voice calls and connected mode mobility.

Observation 3	The approved Rel-17 IoT NTN SID is dedicated to LEO and GEO satellite communication, while HAPS/HIBS and A2G are not in the scope.

Observation 4	Rel-17 IoT NTN study should equally treat eMTC and NB-IoT. The study item will be incomplete unless each of them is properly studied for its feasibility for NTN.

Observation 5	It was agreed at RAN2#112e that support for EPC is assumed for IoT NTN.

Observation 6	Identifying specific bands of interest in sub 6 GHz can be a topic for RAN4 to discuss when a potential normative phase begins.

Observation 7	The approved Rel-17 IoT NTN SID is dedicated to transparent payload.

Observation 8	To study the feasibility of NTN for eMTC and NB-IoT, it is important to properly evaluate the various design targets originally envisioned for eMTC and NB-IoT in the new context of NTN, taking into account factors such as the additional complexity, cost, and power consumption associated with GNSS operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	IoT NTN study should focus on essential adaptations for NTN, while generic enhancements motivated by non-NTN are outside the scope.

Proposal 2	In Rel-17 IOT NTN SI, consider nominal S band (2 GHz) for evaluation purposes.

Proposal 3	In Rel-17 IOT NTN SI, limit the focus to FDD only.

Proposal 4	In Rel-17 IOT NTN SI, prioritize earth fixed beams.

Proposal 5	In Rel-17 IOT NTN SI, evaluate eMTC and NB-IoT in the context of NTN at least for the following targets: (1) coverage performance through link budget analysis; (2) supported device density; (3) complexity and cost of equipping eMTC/NB-IoT devices with NTN capability; (4) power consumption performance of eMTC/NB-IoT devices with NTN connectivity; and (5) latency performance of eMTC/NB-IoT devices in NTN systems.

# References

RP-202689, Study on NB-IoT/eMTC support for Non-terrestrial Network, RAN#90, Dec 2020.

TR 38.821, Solutions for NR to support Non-terrestrial Networks (NTN), 3GPP, V16.0.0, Jan 2016.

3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies,” version 16.0.0, July 2020.

A. Hoglund, J. Bergman, X. Lin, O. Liberg, A. Ratilainen, H. S. Razaghi, T. Tirronen, E. Yavuz, "Overview of 3GPP Release 14 Further Enhanced MTC," IEEE Communications Standards Magazine, vol. 2, no. 2, pp. 84-89, June 2018.

Y.P.E. Wang, X. Lin, A. Adhikary, A. Grovlen, Y. Sui, Y. Blankenship, J. Bergman, and H.S. Razaghi, "A Primer on 3GPP Narrowband Internet of Things," IEEE Communications Magazine, vol. 55, no. 3, pp. 117-123, March 2017.

TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, 3GPP, V12.0.0, June 2013.

TR 45.820, Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT), 3GPP, V13.1.0, November 2015.

R1-2100934, On evaluation assumptions for eMTC and NB-IoT based NTN, RAN1#104-e, Ericsson.