3GPP TSG RAN WG1 #104-e		       R1-2100940

e-Meeting, , January 25th – February 5th, 2021

Source:	NEC

Title:	PDSCH enhancements on supporting NR from 52.6GHz to 71 GHz

Agenda Item:	8.2.5

Document for:	Discussion and Decision

Introduction

At the previous RAN1#103-e meeting, the following agreement has been made [1],

Agreement:

For operation in 52-71 GHz:

- 120 kHz should be supported

- Up to two additional SCS may be considered and at least one should be supported

- FFS: Applicability of additional SCS to particular signals and channels

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- It was identified that new subcarrier spacing, if agreed, may require further investigation of multi-PDSCH/PUSCH scheduling and standardization, if needed. The following aspects should be at least investigated for multi-PDSCH/PUSCH scheduling:

  - whether to support a single TB and/or multiple TBs scheduled over multiple slots

  - applicable DCI format(s) (including potential new formats, if needed) for multi-PDSCH and multi-PUSCH scheduling

  - Enhancement on multiple beam indication and association with multiple PDSCH/PUSCH scheduling

  - DM-RS enhancements such as DM-RS bundling, or changes to the time-domain pattern

  - HARQ enhancements for multi-PDSCH

  - Applicability of Rel-16 multi-PUSCH scheduling

In this contribution, we would like to share our view on PDSCH enhancements on supporting NR from 52.6GHz to 71 GHz.

Discussion

  - Multi-PDSCH scheduling

From the Chairman’s notes of RAN1#103e, the agreement said it was identified that new subcarrier spacing, if agreed, may require further investigation of multi-PDSCH/PUSCH scheduling and standardization, if needed. On one hand, the exist PUSCH scheduling scheme in R16 can be propagated to multi-PDSCH scheduling, on the other hand, there are special parts in multi-PDSCH scheduling, especially HARQ Feedback.

With our observation，below issues should be discussed for multi-PDSCH scheduling:

1. How many PDSCHs are scheduled in a single DCI?

2. How many PDSCH-to-HARQ_feedback timing indicators should be set in a single DCI?

3. How many DAI fields should be set in a single DCI?

4. Are current 2 bit c-DAI and 2 bit t-DAI enough for the HARQ Feedback of the multi-PDSCH?

5. The multi-PDSCH scheduled by a single DCI are continuous or non-continuous?

6. The HARQ Feedback of the multi-PDSCH is in an uplink slot or can be spread to multiple slots?

7. How to determine the following fields in a single DCI for multi-PDSCH which are newly added for NR-U: PDSCH group index, number of requested PDSCH group(s), new feedback indicator, etc.?

About some of the above questions, we’d like to share our view. Below is an illustration for a scaled TDD configuration with numerology increasing, there are several alternatives can be considered:

- Alt 1: Schedule multiple PDSCHs in a single DCI, and UE reports their HARQ Feedback in the same uplink slot. In this case, the PDSCH-to-HARQ_feedback timing indicator in the single DCI can be used to indicate the slot offset from the first or the last PDSCH to the corresponding uplink slot. If it’s a single cell, the c-DAI field in the DCI is the DAI value of the first PDSCH, and the c-DAI is then incremented by 1 for each subsequent PDSCH in the scheduled order, with modulo 4 operation applied.

Figure 1: Illustration of Alt 1

- Alt 2: Schedule multiple PDSCHs in a single DCI, and UE reports their HARQ Feedback in different uplink slots. In this case, the PDSCH-to-HARQ_feedback timing indicator in the single DCI can be used to indicate the slot offset from the first PDSCH to its corresponding uplink slot, and it’s an index of a set of numbers, the PDSCH-to-HARQ_feedback timing indicator is then incremented by 1 for each subsequent PDSCH in the scheduled order. For example, in 480kHz example of figure2, the set of numbers can be {16,15,14,13,13,12,11,10,10,9,8,7,7,6,5,4}. If multiple PDSCHs are scheduled by a PDCCH in slot 0, the PDSCH-to-HARQ_feedback timing indicator in the single DCI is 0, which points to the first value of the set; if multiple PDSCHs are scheduled by a PDCCH in slot 8, the PDSCH-to-HARQ_feedback timing indicator in the single DCI is 8, which points to the 8th value of the set. The c-DAI and t-Dai fields need to be extended accordingly for the second uplink, for example, a 2nd DAI field is added for the second uplink.

Figure 2: Illustration of Alt 2

- Alt 3: Combine the multi-PDSCH scheduling with monitoring PDCCH every multiple slots. If the super span of PDCCH monitoring period is the slot duration of 120 kHz, then the number of multi-PDSCH scheduled by a single DCI will be 4 for 480 kHz and 8 for 960 kHz. In this case, depending on the gNB scheduling strategy, one or two uplink slots will be used to feedback HARQ results of the multi-PDSCH, and the PDSCH-to-HARQ_feedback timing indicator and the c-DAI signaled in the single DCI may be Alt1 or Alt2 as we described above.

Proposal 1: The multi-PDSCH scheduling scheme should be discussed and decided

Proposal 2: The multi-PDSCH scheduling by a single DCI can report HARQ Feedback in an uplink slot or multiple uplink slots.

  - HARQ enhancement

For the type-1 HARQ-ACK codebook determination, it seems the original framework can be reused by multiple PDSCHs scheduling, but there is a potential issue. If 960 kHz is adopted, there will be maximum 64 slots in a subframe, the symbol and slot duration is very short. For operation in unpaired spectrum, in order to avoid DL-UL switch frequently, the downlink slots may last more than 10 slots in a transmission period. In this case, if the HARQ information is reported in different uplink slots with Type-1 HARQ-ACK codebook determination, there will be a lot of overlapping bits in each HARQ-ACK codebook. It will be harmful to the efficiency and performance of the system.

Below are some examples:

Figure 4: overlapping window

From Figure 4, we can see the overlapping window in each type-1 HARQ-ACK codebook in a TDD transmission period. With the incensement of the number of downlink slots in a period, the overlapping window size will be greater.

Proposal 3: Consider optimization for type-1 HARQ-ACK codebook overlapping issue when higher SCS is supported

For the type-2 HARQ-ACK codebook determination, if it’s a single cell, according to our alternatives above, the c-DAI value of the subsequent PDSCHs can be derived from the c-DAI value of the first PDSCH, then the original framework of codebook determination can be reused by multiple PDSCHs scheduling. But for the CA case, there is a potential issue if using current 2 bit c-DAI and 2 bit t-DAI:

Figure 5: PDCCH missing in Scell1

If the PDCCH of Scell1 is missing, the UE can’t be aware of this case by using 2 bit c-DAI and 2 bit t-DAI. It will cause the ambiguous understanding between the UE and gNB.

Proposal 4: Consider increasing the bit length of c-DAI and t-DAI for type-2 HARQ-ACK codebook determination in CA case.

  - changes to time units

In current TS38.211[2], the time unit is based on Δfmax=480∙103 Hz. According to the WID [3], in addition to 120kHz SCS, new SCS, 480kHz and 960kHz will be supported. Then Δfmax should be changed accordingly. So we propose

Proposal 5: For 120kHz and 240KHz, Δfmax=480∙103 Hz. For 960kHz, Δfmax=960∙103Hz.

Conclusion

In this contribution, we shares our views on PDSCH enhancements on supporting NR from 52.6GHz to 71 GHz, and the following proposals are made:

Proposal 1: The multi-PDSCH scheduling scheme should be discussed and decided

Proposal 2: The multi-PDSCH scheduling by a single DCI can report HARQ Feedback in an uplink slot or multiple uplink slots.

Proposal 3: Consider optimization for type-1 HARQ-ACK codebook overlapping issue when higher SCS is supported

Proposal 4: Consider increasing the bit length of c-DAI and t-DAI for type-2 HARQ-ACK codebook determination in CA case.

Proposal 5: For 120kHz and 240kHz, Δfmax=480∙103 Hz. For 960kHz, Δfmax=960∙103Hz.

References

- 3GPP RAN1#103e Chairman’s notes

- TS 38.211

- RP-202925, Extending current NR operation to 71GHz