3GPP TSG RAN WG1 Meeting #104e	R1-2101019

e-Meeting, January 25th – February 5th, 2021

Agenda Item: 8.15.1

Source: Thales, Sateliot, GateHouse

Title: Feasibility of the large, single-beam small sats / CubeSats scenario

Document for: Discussion

# Introduction

This document contains the description of a use-case of IoT-NTN over LEO small satellite, which use a single large beam pointed at nadir. It aims at showing that one can find a working point, and that this use case should be included in 3GPP studies.

# IoT NTN Scenarios, Parameters, Link Budget

## IoT NTN Scenario

The scenario considered here is based on scenario C described R1-2008868 [1], Agenda Item 8.15.1, untitled “Summary #4 of 8.15.1 IoT NTN Scenarios”:

Figure 1 : View of the studied scenario

- The setup is in FDD mode

- Devices have GNSS capabilities

- Transparent payload is assumed

- Frequency band of 2 GHz

- LEO based non-terrestrial access network generating fixed beams which footprint move with the satellite

## IoT NTN Parameters

### UE

We consider an IoT device, as defined in TR 38.821, clause 6.1.1.1.

Table 1 : UE characteristics for system level simulations

| Characteristics | IoT device |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 5 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |
| Note 1: The noise figure of 5 dB is chosen because a duplexer-free UE is assumed (i.e. half-duplex) Note 2: The uplink is assumed to be set to work with 3.75 kHz subcarrier spacing. |  |

### Satellite

Table 2: Satellite parameters for system level simulator calibration

| Parameter |  | Value |
| --- | --- | --- |
| Satellite orbit |  | LEO-600 |
| Satellite altitude |  | 600 km |
| Satellite antenna pattern |  | Elliptical pattern formed by a linear array of 3 antennas |
| Payload characteristics for DL transmissions |  |  |
| Satellite max EIRP density (Note 1) | S-band (2 GHz) | 21.4 dBW/MHz |
| Satellite Tx max Gain |  | 11 dBi (Note 4) |
| 3 dB beamwidth |  | 104.7 deg |
| Satellite beam diameter (Note 2) |  | 1700 km |
| Payload characteristics for UL transmissions |  |  |
| Max G/T (Note 1) | S-band (2 GHz) | -17.9 dB K-1 |
| Satellite Rx max Gain |  | 11 dBi |
| NOTE 1: Max value available at boresight NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 4 : We consider here an linear array of 3 antennas of 6.2 dB gain each. So the equation 6.1.3.1-6 of TR 38.821 applies. |  |  |

### Propagation channel

Table 3 : System Level Simulation assumptions for calibration

| Parameter | Value |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) |
| Maximum Bandwidth per beam (DL + UL) | S-band: DL 180 kHz and UL 3.75 kHz |
| Satellite characteristics (G/T, EIRP density) | See Table 1 and Table 2 |
| Beam layout definition | One single large beam (104,7 deg HPBW) |
| Channel model | AWGN |
| Propagation conditions | Base-line: Clear Sky |
| UE configuration | S-band |
| UE orientation | Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line: Coupling loss, Geometry Note: Coupling loss is defined as the signal loss from the antenna port to the antenna port |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

## IoT NTN Link budget

The link budget is calculated for:

- An UE at the edge of the beam (at 30° of elevation angle),

- An UE at satellite’s nadir (at 90° of elevation angle)

| Case | Transmission mode | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [kHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Pointing losses [dB] | SNR [dB] | RSRP [dBm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beam edge, 30° elev | DL | 2,0 | 44,0 | -29,6 | 180 | 159,1 | 0,1 | 3,0 | 2,2 | 3,0 | 3,0 | -10,0 | -137,2 |
|  | UL | 2,0 | 23,0 | -17,9 | 3,75 | 159,1 | 0,1 | 3,0 | 2,2 | 3,0 | 3,0 | -2,5 | -136,4 |
| Nadir, 90° elev | DL | 2,0 | 44,0 | -29,6 | 180 | 154,0 | 0,1 | 3,0 | 2,2 | 3,0 | 0,0 | -1,9 | -129,2 |
|  | UL | 2,0 | 23,0 | -17,9 | 3,75 | 154,0 | 0,1 | 3,0 | 2,2 | 3,0 | 0,0 | 5,6 | -128,4 |

Simulations show that at -10 dB SNR for downlink, we can close the link budget with a number of repetition of 64 The study [2] (figure 13) also shows a setup with a MCL of 164 dB which can achieve 11 kbps in DL, and 2 kbps in UL.

| SNR [dB] | Datarate [bps] | iTBS | nSF | Nrep |
| --- | --- | --- | --- | --- |
| -10,2 | 2800 | 0 | 10 | 8 |
| -10,1 | 2800 | 2 | 6 | 16 |
| -10 | 3500 | 7 | 2 | 32 |
| -9,9 | 3500 | 1 | 8 | 8 |
| -9,8 | 3500 | 3 | 5 | 16 |

## Agreement / Conclusion

Independent work at GateHouse and in [2] show that at -12 dB SNR, a data rate of 3.500 kbps (resp. 2 kbps in [2]) can be achieved. These figures are given in a terrestrial configuration, but provided that:

- the link budget is closed in [2] at -14 dB SNR (compared to our -10 dB),

- at UE side, optimizations can be made to better track the phase of the eNodeB,

- the link budget is given at beam edge (worst case),

one can expect the setup to be more favourable in the LEO case.

# ANNEX

Void

# References

- R1-2008868, Eutelsat, Summary #4 of 8.15.1 IoT NTN Scenarios, RAN1#103e, November 2020

- A Systematic Analysis of Narrowband IoT Quality of Service, Sensors 2020, 20, 1636; doi:10.3390/s20061636