3GPP TSG RAN WG1 #104-e	R1-2101173

e-Meeting, January 25th – February 5th, 2021

Agenda item:	7.2.2

Source:	Samsung

Title:	Correction on the use of subCarrierSpacingCommon in NR-U

Document for:	Discussion and Decision

# Background and Justification

In Rel-16 NR-U, it has been agreed to use subCarrierSpacingCommon to indicate the QCL parameter NSSBQCL, and the corresponding procedure has been specified in TS 38.213. Note that this change is only applicable to NR-U, i.e., FR1 unlicensed band, and UE should assume the subcarrier spacing of CORESET#0 is same as the associated SS/PBCH block. TS 38.213 has distinguished the different ways of determining the subcarrier spacing of CORESET#0 for licensed and unlicensed bands, correspondingly, and TS 38.211 should clarify the different use case accordingly.

For this purpose, we propose the following two TPs to clarify.

================================ Start of TP1 for TS 38.211 ==================================

#### 4.4.4.2	Point A

Point A serves as a common reference point for resource block grids and is obtained from:

-	offsetToPointA for a PCell downlink where offsetToPointA represents the frequency offset between point A and the lowest subcarrier of the lowest resource block, which overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2;

-	for operation without shared spectrum channel access, the lowest resource block has the subcarrier spacing provided by the higher layer parameter subCarrierSpacingCommon;

-	for operation with shared spectrum channel access, the lowest resource block has the subcarrier spacing same as the SS/PBCH block used by the UE for initial cell selection;

-	absoluteFrequencyPointA for all other cases where absoluteFrequencyPointA represents the frequency-location of point A expressed as in ARFCN.

================================ End of TP1 for TS 38.211 ==================================

================================ Start of TP2 for TS 38.211 ==================================

#### 7.4.3.1	Time-frequency structure of an SS/PBCH block

================================ Unchanged Text Omitted ==================================

For an SS/PBCH block, the UE shall assume

-	antenna port  is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH,

-	for SS/PBCH block type A,  and  with the quantities , and NCRBSSB expressed in terms of 15 kHz subcarrier spacing, and

-	for SS/PBCH block type B,  and  with the quantity  expressed in terms of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and NCRBSSB expressed in terms of 60 kHz subcarrier spacing;

-	the centre of subcarrier 0 of resource block NCRBSSB  coincides with the centre of subcarrier 0 of a common resource block with the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon for operation without shared spectrum channel access and same as the subcarrier spacing of the SS/PBCH block for operation with shared spectrum channel access. This common resource block overlaps with subcarrier 0 of the first resource block of the SS/PBCH block.

================================ End of TP2 for TS 38.211 ==================================

# Conclusion

The proposal made in this contribution is summarized below:

Proposal 1:

- Adopt TP1 and TP2 for TS 38.211.

================================ Start of TP1 for TS 38.211 ==================================

#### 4.4.4.2	Point A

Point A serves as a common reference point for resource block grids and is obtained from:

-	offsetToPointA for a PCell downlink where offsetToPointA represents the frequency offset between point A and the lowest subcarrier of the lowest resource block, which overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2;

-	for operation without shared spectrum channel access, the lowest resource block has the subcarrier spacing provided by the higher layer parameter subCarrierSpacingCommon;

-	for operation with shared spectrum channel access, the lowest resource block has the subcarrier spacing same as the SS/PBCH block used by the UE for initial cell selection;

-	absoluteFrequencyPointA for all other cases where absoluteFrequencyPointA represents the frequency-location of point A expressed as in ARFCN.

================================ End of TP1 for TS 38.211 ==================================

================================ Start of TP2 for TS 38.211 ==================================

#### 7.4.3.1	Time-frequency structure of an SS/PBCH block

================================ Unchanged Text Omitted ==================================

For an SS/PBCH block, the UE shall assume

-	antenna port  is used for transmission of PSS, SSS, PBCH and DM-RS for PBCH,

-	the same cyclic prefix length and subcarrier spacing for the PSS, SSS, PBCH and DM-RS for PBCH,

-	for SS/PBCH block type A,  and  with the quantities , and NCRBSSB expressed in terms of 15 kHz subcarrier spacing, and

-	for SS/PBCH block type B,  and  with the quantity  expressed in terms of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and NCRBSSB expressed in terms of 60 kHz subcarrier spacing;

-	the centre of subcarrier 0 of resource block NCRBSSB  coincides with the centre of subcarrier 0 of a common resource block with the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon for operation without shared spectrum channel access and same as the subcarrier spacing of the SS/PBCH block for operation with shared spectrum channel access. This common resource block overlaps with subcarrier 0 of the first resource block of the SS/PBCH block.

================================ End of TP2 for TS 38.211 ==================================