3GPP TSG RAN WG1#104-e			R1-2101182

e-Meeting, January 25th – February 5th, 2021

Agenda item:	7.2.6

Source: 	Samsung

Title: 	On Rel.16 multi-TRP/panel transmission

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s views on Rel.16 multi-TRP/Panel transmission.

Remaining issue in Single-PDCCH based NC-JT

PDSCH default beam for cross-carrier scheduling

In Rel. 16, PDSCH default beam behaviors have been enhanced for two different operation scenarios, single-PDCCH based NC-JT and cross carrier scheduling in MR-DC, as captured in the table below.

| Multi-TRP Agreement For single-DCI based Multi-TRP/panel transmission with at least one configured TCI states for the serving cell of scheduled PDSCH containing 'QCL-TypeD', - If the offset between the reception of the PDCCH and the corresponding PDSCH is less than timeDurationForQCL and after the reception of activation command of TCI states for UE specific PDSCH, the UE may assume that DMRS ports of PDSCH follows QCL parameters indicated by default TCI state(s) as following: - Use the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states which are activated for PDSCH. - If all the TCI codepoints are mapped to a single TCI state, then Rel-15 behavior is followed The support of this feature is part of UE capability. MR-DC Agreements: When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < Threshold-Sched-Offset timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell Agreement: Adopt a new RRC configuration parameter with which the network selects either Rel-15 or Rel-16 behaviour for the default beam selection | 5.1.5 Antenna ports quasi co-location ... Independent of the configuration of tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains the 'QCL-TypeD', - the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, - the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol and they are associated with same CORESETpoolindex, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). - If a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA' or is configured with higher layer parameter repetitionNumber-r16, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion. In this case, if the 'QCL-TypeD' in both of the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers) - In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enableDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. … |
| --- | --- |

Although there has been no intention to make above two features be mutually exclusive to each other from RAN1 discussions so far, it seems that the current specification does not capture any UE behavior when single PDCCH based NC-JT operation is scheduled by cross carrier scheduling. To address this issue, the following TP can be considered:

Proposal 1: On PDSCH default beam for single-PDCCH based NC-JT, adopt the following TP in TS 38.214 V16.3.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enabledDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; - When a UE is configured with enableTwoDefaultTCIStates-r16 and at least one TCI codepoint indicates two TCI states for the component carrier of scheduled PDSCH, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of the component carrier are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

Type-1 HARQ-ACK codebook determination for SPS PDSCH

If a SPS PDSCH configuration is activated by a DCI format indicating an entry containing repetitionNumber-r16, the SPS PDSCH repeats in repetitionNumber-r16 consecutive slots. Current Rel-16 specification defines the slots where the repeated SPS PDSCH belongs and the HARQ-ACK is reported, as highlighted below.

9.1.2 Type-1 HARQ-ACK codebook determination … If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or - from slot n-repetitionNumber-r16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16, or - in slot n, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

The first bullet describes the case for PDSCH (including SPS PDSCH) configured with pdsch-AggregationFactor. The second bullet contains only the case that PDSCH repetitions with repetitionNumber-r16 is scheduled dynamically by DCI, but misses the case of SPS PDSCH repetition with repetitionNumber-r16. Instead, such case is captured erroneously in the third bullet. To fix the error, the following TP can be considered:

Proposal 2: If a SPS PDSCH configuration is activated by a DCI format indicating an entry containing repetitionNumber-r16, the UE reports HARQ-ACK information for a SPS PDSCH reception from slot n-repetitionNumber-r16+1 to slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k.

Proposal 3: Adopt the following TP in TS 38.213 V16.3.0:

9.1.2 Type-1 HARQ-ACK codebook determination --- start of TP --- If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or - from slot n-repetitionNumber-r16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16 or the PDSCH is a SPS PDSCH with repetitionNumber-r16 repetitions, or - in slot n, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK. --- end of TP ---

Remaining issue in Multi-PDCCH based NC-JT

In current specification the association of SPS PDSCH to the TRPs is not clearly defined. In our understanding the SPS PDSCH with the activation DCI is associated with the CORESETPoolIndex value of the CORESET in which the activation DCI is received. The subsequent SPS PDSCHs without DCI should also follow the same association.

Proposal 4: For a SPS configuration, SPS PDSCH with or without DCI is associated with a TRP according to the value of CORESETPoolIndex in a CORESET in which the activation DCI of the SPS configuration is received.

In RAN1#99 an agreement was reached to allow reception of two overlapping DG PDSCHs each from a different TRP. UE may declare a capability to support such a feature.

Agreement For multi-DCI based multi-TRP, when PDCCHs schedule two PDSCHs/PUSCHs across TRPs, i.e. PDCCHs are associated with different values of CORESETpoolIndex, following operations are allowed: - PDCCH to PDSCH: For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i.

Rel-16 has also introduced support of multiple active SPS configuration in a cell. For a UE that operates in a cell with multiple active SPS configuration and Multi-DCI M-TRP, and declares a capability of receiving two overlapping DG PDSCH, reception of two overlapping SPS PDSCHs should also be supported if the two PDSCHs are associated with different CORESETPoolIndex values. In fact receiving two overlapping SPS PDSCHs may be even less of a challenge as the UE is aware of the PDSCH resources semi-statically, once the configurations have been activated.

Proposal 5: Extend the agreement in RAN1#99 to include two SPS PDSCHs which overlap in time domain and are associated to two different TRPs according to the value of CORESETPoolIndex.

Proposal 6: Adopt the following TP in TS 38.214 V16.3.0:

5.1 UE procedure for receiving the physical downlink shared channel --- start of TP --- When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1. For any SPS configuration, the corresponding SPS PDSCH with or without PDCCH is associated with a value of CORESETPoolIndex of the CORESET in which the activation DCI of the SPS configuration is received. If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot by applying the following pseudo-code to SPS PDSCHs associated with the same value of CORESETPoolIndex as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to N, where N is the maximum number of unicast PDSCHs per CORESETPoolIndex per slot if UE is provided CORESETPoolIndex with the value of 1, or the number of unicast PDSCHs in a slot supported by the UE otherwise --- end of TP ---

UE capability for multi-TRP/Panel

For Rel-16 M-TRP features, FG16-2c below describes if a UE supports simultaneous reception with different QCL-D. One thing to think about is that support of this FG may be interpreted as supporting any combination of QCL-D with simultaneous reception when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH due to lack of further condition mentioned.

| 16-2c | Simultaneous reception with different Type-D | Supports simultaneous reception with different QCL Type-D RSs. |  | Yes | N/A |  | Per band | N/A | FR2 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

However, such interpretation is unrealistic since a UE would unlikely be able to receive different QCL-D unconditionally. Without such consideration, most UE’s would need to decline support of FG16-2c, and this can be a considerable under reporting.

Ideally, Rel-16 signaling could have been designed to explicitly acknowledge this aspect. However, given the timing, we would like to propose exploiting the existing Rel-15 FG2-29a described below.

| 2-29a | Group based beam reporting | Support of beam group RSRP reporting for group of 2 beams |  | groupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In our understanding, one of objectives of group based reporting is to enable information exchange from a UE to gNB regarding which QCL-D can be simultaneously received. Hence, this can also be utilized for support of receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH in principle. Along this line, we provide two alternatives.

Alt1: Support of FG16-2c is interpreted as supporting simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH. Support of FG2-29a and group based beam reporting (if configured) can also be utilized for receiving rel-16 S-DCI and M-DCI M-TRP PDSCH when a UE does not support FG16-2c.

In our view, alt1 may have a minimal impact in maintaining independency between FG16-2c and FG2-29a. When a UE supports FG16-2c, it supports simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH irrespective of FG2-29a. When a UE does not support FG16-2c, but supports FG2-29a, receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH with simultaneous different QCL-D can still happen based on group based reporting. Also, this is pretty natural interpretation given lack of clear limitation for applicability of FG2-29a. However, we believe confirmation is beneficial especially given the existence of Rel-16 signaling with potentially similar functionality.

Alt2:

| FG2-29a | FG16-2c | Interpretation for rel-16 M-TRP operation |
| --- | --- | --- |
| Support | Support | A UE supports simultaneous reception with different QCL-D which are in the same group in group based reporting when receiving rel-16 S-DCI and M-DCI M-TRP PDSCH. |
| Support/not support | Not support | A UE does not support simultaneous reception with different QCL-D. |
| Not support | Support | A UE supports simultaneous reception with different QCL-D without further condition when receiving rel-16 S-DCI and M-DCI M-TRP PDSCH. |

Alt2 introduces more dependency between FG2-29a and FG16-2c while limiting usability of FG16-2c. Hence, we provide a proposal below based on alt1.

Proposal 7: Support of FG16-2c is interpreted as supporting simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH. Support of FG2-29a and group based beam reporting (if configured) can also be utilized for receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH when a UE does not support FG16-2c.

Conclusion

In this contribution, the following proposals are given:

Proposal 1: On PDSCH default beam for single-PDCCH based NC-JT, adopt the following TP in TS 38.214 V16.3.0:

5.1.5 Antenna ports quasi co-location --- start of TP --- If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enabledDefaultBeamForCCS]: - The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero; - For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell; - When a UE is configured with enableTwoDefaultTCIStates-r16 and at least one TCI codepoint indicates two TCI states for the component carrier of scheduled PDSCH, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of the component carrier are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. --- end of TP ---

Proposal 2: If a SPS PDSCH configuration is activated by a DCI format indicating an entry containing repetitionNumber-r16, the UE reports HARQ-ACK information for a SPS PDSCH reception from slot n-repetitionNumber-r16+1 to slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k.

Proposal 3: Adopt the following TP in TS 38.213 V16.3.0:

9.1.2 Type-1 HARQ-ACK codebook determination --- start of TP --- If the UE is provided pdsch-AggregationFactor in SPS-Config or PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 includes repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor in SPS-Config or PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat is provided by pdsch-AggregationFactor [6, TS 38.214], or - from slot n-repetitionNumber-r16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber-r16 or the PDSCH is a SPS PDSCH with repetitionNumber-r16 repetitions, or - in slot n, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK. --- end of TP ---

Proposal 4: For a SPS configuration, SPS PDSCH with or without DCI is associated with a TRP according to the value of CORESETPoolIndex in a CORESET in which the activation DCI of the SPS configuration is received.

Proposal 5: Extend the agreement in RAN1#99 to include two SPS PDSCHs which overlap in time domain and are associated to two different TRPs according to the value of CORESETPoolIndex.

Proposal 6: Adopt the following TP in TS 38.214 V16.3.0:

5.1 UE procedure for receiving the physical downlink shared channel --- start of TP --- When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry in pdsch-TimeDomainAllocationList which contain RepNumR16 in PDSCH-TimeDomainResourceAllocation, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1. For any SPS configuration, the corresponding SPS PDSCH with or without PDCCH is associated with a value of CORESETPoolIndex of the CORESET in which the activation DCI of the SPS configuration is received. If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot by applying the following pseudo-code to SPS PDSCHs associated with the same value of CORESETPoolIndex as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to N, where N is the maximum number of unicast PDSCHs per CORESETPoolIndex per slot if UE is provided CORESETPoolIndex with the value of 1, or the number of unicast PDSCHs in a slot supported by the UE otherwise --- end of TP ---

Proposal 7: Support of FG16-2c is interpreted as supporting simultaneous reception with different QCL-D without any further condition when receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH. Support of FG2-29a and group based beam reporting (if configured) can also be utilized for receiving Rel-16 S-DCI and M-DCI M-TRP PDSCH when a UE does not support FG16-2c.