EGPP TSG RAN WG1 #104-e	                                                                 R1-2101246

e-Meeting, January 25th – February 5th, 2021

Agenda Item:		8.16

Source:		Samsung

Title:		Discussion on DL 1024QAM for NR FR1

Document for:		Discussion and decision

Introduction

In RAN#90e, 1024QAM WID was updated with clarifications on MCS/CQI table as follows.

- Specify high order modulation for PDSCH [RAN1] - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH - Specify corresponding 5-bit MCS table with 1024QAM entries as defined in E-UTRA, with 5 bit DCI overhead for MCS indication - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA, with no changes to the CQI field and table sizes

This contribution discusses RAN1 specification impact related to DL 1024QAM including others than 1024QAM constellation, MCS and CQI tables based on agreements [1] and the above updated WID.

CQI table and MCS table

Regarding CQI table, it is noted that CQI field and table sizes should be reused in LTE based on WID. Therefore, it should reuse following CQI table for NR.

Proposal 1: Support following 4 bits CQI table for 1024 QAM in TS 38.214.

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

Regarding MCS table, it is observed that the following potential combinations of (modulation order, code rate) are providing reasonable SNR spacing gap between 256QAM and 1024QAM with the target BLER of 10% as shown in table 1. Related evaluation assumptions are given in [4].

Table 1. Evaluation results

| (Modulation, code rate) | SNR (dB) | Spectral efficiency (bps/Hz) |
| --- | --- | --- |
| (8, 916.5) | 11.1 | 5.422 |
| (8, 948) | 12.0 | 5.689 |
| (10, 806) | 12.4 | 6.044 |
| (10, 853) | 13.6 | 6.4 |

Therefore, it is preferable to consider that NR 1024QAM MCS table should consider entries of LTE 1024QAM CQI table as much as possible, and for entries which were not in the CQI table, it is reasonable to apply mean values in between in order to provide throughput evenly.

Proposal 2: Support following MCS table for 1024 QAM for TS 38.214.

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.377 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.877 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 434 | 1.6953 |
| 6 | 4 | 553 | 2.1602 |
| 7 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 9 | 6 | 567 | 3.3223 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.332 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 806 | 7.8711 |
| 24 | 10 | 853 | 8.3321 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

Other issues

PTRS time density

In TS 38.214, time density of PT-RS is determined based on scheduled MCS as well as MCS table. So, it needs additional sentence for 1024 QAM. Considering that MCS table for 1024 QAM is not defined in NR, it is sufficient to add some sentence as placeholder, and then it can be finalized when MCS table for 1024 QAM is available in NR.

Proposal 3: Support following text proposal for PT-RS for TS 38.214.

5.1.6.3 PT-RS reception procedure Unchanged parts are omitted - otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or [- the scheduled MCS from Table 5.1.3.1-x is smaller than yy, or] - the number of scheduled RBs is smaller than 3, or - otherwise, if the RNTI equals RA-RNTI, [MsgB-RNTI], SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present Unchanged parts are omitted

Processing time capability

Since there is no restriction about frequency or time domain resource allocation, it is no doubt that maximum TBS size for 1024 QAM is approximated 1.25 times larger than current maximum TBS size that NR is supporting. From UE side, it would be challenging processing this larger TBS size within the same amount of processing timeline (e.g., N1). It is noted that a UE falls back to capability 1 (slow processing time) if the scheduled PRB is larger than 136 RBs and 30kHz is configured in case that capability 2 (fast processing time) is configured, in order to give a sufficient processing time to process large TBS size. In this sense, it needs to consider processing time relaxation for a UE supporting 1024 QAM. .

Proposal 4: Consider processing time relaxation for a UE supporting 1024 QAM.

Conclusions

In this contribution, it was discussed on how to support 1024 QAM in NR specification.

Proposal 1: Support following 4 bits CQI table for 1024 QAM in TS 38.214.

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

Proposal 2: Support following MCS table for 1024 QAM for TS 38.214.

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.377 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.877 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 434 | 1.6953 |
| 6 | 4 | 553 | 2.1602 |
| 7 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 9 | 6 | 567 | 3.3223 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.332 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 806 | 7.8711 |
| 24 | 10 | 853 | 8.3321 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

Proposal 3: Support following text proposal for PT-RS for TS 38.214.

5.1.6.3 PT-RS reception procedure Unchanged parts are omitted - otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or [- the scheduled MCS from Table 5.1.3.1-x is smaller than yy, or] - the number of scheduled RBs is smaller than 3, or - otherwise, if the RNTI equals RA-RNTI, [MsgB-RNTI], SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present Unchanged parts are omitted

Proposal 4: Consider processing time relaxation for a UE supporting 1024 QAM.

Appendix

- [1] Agreements

Agreements: - Introduce new RRC signaling to indicate use of 1024-QAM CQI table. - For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation. - 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI. Agreements: - Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1 - FFS : support of 1024-QAM MCS table for DCI format 1_2 - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config Agreements: - RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config - When UE is configured with mcs-Table-r17 set to ‘qam1024’ in PDSCH-Config, - UE uses 1024-QAM MCS table for PDSCH scheduled with a DCI format 1_1 with CRC scrambled by C-RNTI, - UE uses 1024-QAM MCS table for PDSCH scheduled with the DCI format 1_1 with CRC scrambled by CS-RNTI if the UE is not configured with mcs-Table in SPS-Config - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, similar approach is used for 1024-QAM MCS table usage with DCI format 1_2 Agreements: - Adopt following TP for in 38.212, subclause 5.4.2.1 for TBS_LBRM determination. Agreements: - Adopt following text proposal for TS 38.201, subclause 4.2.2. -------------------------------------------------------- Omitted ----------- 4.2.2 Physical channels and modulation -------------------------------------------------------- Omitted --------------- The modulation schemes supported are - in the downlink, QPSK, 16QAM, 64QAM, and 256QAM and 1024 QAM - in the uplink, QPSK, 16QAM, 64QAM and 256QAM for OFDM with a CP and π/2-BPSK, QPSK, 16QAM, 64QAM and 256QAM for DFT-s-OFDM with a CP -------------------------------------------------------- Omitted ------. Agreements: - Adopt following TP for 38.211 to reflect the agreed 1024-QAM constellation. Agreements: - Adopt following TP to 38.211, subclause 7.3.1.2, to reflect 1024-QAM support for PDSCH Section 7.3.1.2 Table 7.3.1.2-1: Supported modulation schemes. Modulation scheme; Modulation order. QPSK; 2. 16QAM; 4. 64QAM; 6. 256QAM; 8. 1024QAM; 10. Agreements: - Adopt following TP for 38.214, subclause 5.2.2.1, reflecting the 1024-QAM CQI table usage based on corresponding RRC parameter as follows. - Note : RAN1 to further align with the RAN2 signaling design 5.2.2.1 Channel quality indicator (CQI) -------------------------------------------------------- Omitted ---------------------------------------------------------------------------------------------------------------------- The CQI indices and their interpretations are given in Table 5.2.2.1-2 or Table 5.2.2.1-4 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-3 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-5 for reporting CQI based on QPSK, 16QAM, 64QAM, 256QAM and 1024 QAM. Based on an unrestricted observation interval in time unless specified otherwise in this Clause, and an unrestricted observation interval in frequency, the UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition: - A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding: - 0.1, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table1’ (corresponding to Table 5.2.2.1-2), or ‘table2’ (corresponding to Table 5.2.2.1-3), or if the higher layer parameter cqi-Table-r17 in CSI-ReportConfig configures ‘table4’ (corresponding to Table 5.2.2.1-5) - 0.00001, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table3’ (corresponding to Table 5.2.2.1-4). -------------------------------------------------------- Omitted ------------------------------------------------------------------------------------------------------------------------- Agreements: - Companies are encouraged to use below link-level simulation assumptions for assessing at least transition point between 256-QAM and 1024-QAM. Parameter; Value. Carrier frequency, SCS, System BW; 3.5GHz, 30kHz, 100 MHz. Channel model; AWGN, CDL-B or CDL-C in TR 38.901 with up to 30ns delay spread. UE speed; 3km/h, 0km/h. Number of UE antennas; 1T4R, 2T4R or 4T4R. Number of gNB antennas; 32T32R or 64T64R or 2T or 8T. Tx EVM; 0, 2%. Rx EVM; 0, 3%. MCS; 256 QAM, 1024 QAM Coding Rate*: 0.70, 0.75, 0.80, 0.85, 0.90, 0.925 Other coding rates are not precluded and, if simulated, to be reported by each company. DMRS type; DM-RS type 1. Number of DMRS symbols; 1. Number of scheduled RBs; 273. PDSCH mapping; Type A, Start symbol 2, Duration 12. Rank; Rank1, Rank 2,. Channel estimation; Realistic channel estimation. Metric; Crossover SNR at transition points between 256-QAM and 1024-QAM. Note*: Coding rates are used for 1024QAM, while coding rates for 256QAM are selected from TS38.214 MCS table 2.

- [2] 1024QAM MCS table (TS 36.213 v16.1.0)

Table 7.1.7.1-1B. Modulation and TBS index table 3 for PDSCH MCS Index; Modulation Order; Modulation Order; TBS Index. 0; 2; 2; 0. 1; 2; 2; 2. 2; 2; 2; 4. 3; 2; 4; 6. 4; 2; 4; 8. 5; 4; 6; 11. 6; 4; 6; 13. 7; 4; 8; 15. 8; 6; 8; 16. 9; 6; 8; 18. 10; 6; 8; 20. 11; 6; 8; 21. 12; 6; 8; 22. 13; 6; 8; 23. 14; 6; 8; 24. 15; 8; 8; 25. 16; 8; 8; 27. 17; 8; 8; 28. 18; 8; 8; 29. 19; 8; 8; 30. 20; 8; 8; 31. 21; 8; 8; 32. 22; 8; 8; 33/33A/33B. 23; 10; 10; 34A. 24; 10; 10; 35. 25; 10; 10; 36. 26; 10; 10; 37A/37. 27; 2; 2; reserved. 28; 4; 4. 29; 6; 6. 30; 8; 8. 31; 10; 10.

- [3] 1024QAM CQI table (TS 36.213 v16.1.0)

Table 7.2.3-4: 4-bit CQI Table 4 CQI index; modulation; code rate x 1024; Efficiency. 0; out of range. 1; QPSK; 78; 0.1523. 2; QPSK; 193; 0.3770. 3; QPSK; 449; 0.8770. 4; 16QAM; 378; 1.4766. 5; 16QAM; 616; 2.4063. 6; 64QAM; 567; 3.3223. 7; 64QAM; 666; 3.9023. 8; 64QAM; 772; 4.5234. 9; 64QAM; 873; 5.1152. 10; 256QAM; 711; 5.5547. 11; 256QAM; 797; 6.2266. 12; 256QAM; 885; 6.9141. 13; 256QAM; 948; 7.4063. 14; 1024QAM; 853; 8.3321. 15; 1024QAM; 948; 9.2578.

- [4] Evaluation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100MHz |
| Channel model | CDL-B (30ns) |
| UE speed | 3km/h |
| Number of UE antennas | 4Rx |
| Number of gNB antennas | 32Tx |
| MCS | LTE 1024QAM table |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Channel estimation | Realistic channel estimation |