3GPP TSG RAN WG1 #104b-e			R1-2103132

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.15.1

Source:	Apple

Title:	Link Budget Analysis of IoT NTN

Document for:	Discussion/Decision

# Introduction

The study item description of NB-IoT/eMTC support for NTN was provided in [1]. One objective of this study item is to identify scenarios applicable to NB-IoT/eMTC. The following agreements have been made in RAN1 #104-e meeting [2], [3].

Agreement:

The following assumptions are agreed for a common set of link budget parameters:

- UE power class (PC5=20 dBm)

- UE Noise Figure (NF=9 dB)

- Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e

  - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz

  - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL)

- Other losses

| Other Losses | GEO (35786 km) | LEO (1200 km) | LEO (600 km) |
| --- | --- | --- | --- |
| Scintillation losses | 2.2 | 2.2 | 2.2 |
| Atmospheric losses | 0.2 | 0.1 | 0.1 |
| Polarization loss | 3 | 3 | 3 |
| Shadow margin | 3 | 3 | 3 |

NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL.

NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL.

NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR.

NOTE 4: These parameters are only for the purpose of link budget calculations.

NOTE 5: Atmospheric losses are a function of elevation angle.

Agreement:

Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4

Agreement:

Include in TR 36.763, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations – (Corresponding satellite parameter Set 3 and Set 4 are given in Section 9.4)

| SET 3 | GEO 35786 km | LEO-600 km | LEO-1200 km |
| --- | --- | --- | --- |
| 3 dB Beam width (HPBW) | 0.735 degree | 22.0631 degree | 22.0631 degree |
| Central beam center elevation | 20.88 degree | 43.78 degree | 46.05 degree |
| Central beam edge elevation | 12.5 degree | 30 degree | 30 degree |
| Central beam edge satellite-UE distance | 40316 km | 1074 km | 1998 km |

| SET 4 | LEO-600 km |
| --- | --- |
| 3 dB Beam width (HPBW) | 104.7 degree |
| Central beam center elevation | 90 degree |
| Central beam edge elevation | 30 degree |
| Central beam edge satellite-UE distance | 1076 km |

NOTE 1: The 3 dB beam width (HPBW) is already included in satellite parameter set 1 and Set 2 in TR 38.821 Table 6.1.1.1-1 and Table 6.1.1.1-2 respectively. The central beam center elevation for Set-1 and Set-2 is defined as the target elevation angle that is included in in TR 38.821 Table 6.1.3.2-1. The central beam edge satellite-UE distance can be derived from the central beam edge elevation and does not need to be included.

NOTE 2: Central beam center elevation is the beam center elevation of the central beam in the beam layout.

NOTE 3: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

NOTE 4 In SLS evaluation with a multiple beam layout, the central beam is the serving beam for UEs. The outer beams have beam center elevation that is different from the central beam center elevation.  For the interference modelling, the interference due to the outer beams is determined by using their respective beam center elevations.

NOTE 5: For the multiple-beam satellite cell, the longest beam edge distance will correspond to the minimum beam edge elevation of the most outer beam as illustrated in figure below.

Agreement:

Include the following tables in TR 36.763:

- Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1)

- Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2)

- Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degree for GEO, and 30 degree for LEO-600 km and 1200 km)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Central beam edge elevation |  | 12.5 degree | 30 degree | 30 degree |
| Central beam center elevation |  | 20.9 degree | 46.05 degree | 43.8 degree |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4m | 0.4 m |
| Satellite EIRP density |  | 59.8 dBW/MHz | 33.7 dBW/MHz | 28.3 dBW/MHz |
| Satellite Tx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |
| 3dB beam width (HPBW) |  | 0.7353 degree | 22.1 degree | 22.1 degree |
| Satellite beam diameter (NOTE 2) |  | 459km | 470 km | 234 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 12 m | 0.4 m | 0.4 m |
| G/T |  | 16.7dB K-1 | -12.8 dB K-1 | -12.8 dB K-1 |
| Satellite Rx max Gain |  | 45.7 dBi | 16.2 dBi | 16.2 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

- Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019)

| Satellite orbit |  | LEO-600 |
| --- | --- | --- |
| Satellite altitude |  | 600 km |
| Central beam edge elevation |  | 30 degree |
| Central beam center elevation |  | 90 degree |
| Payload characteristics for DL transmissions |  |  |
| Equivalent satellite antenna aperture (NOTE 1) | S-band (i.e. 2 GHz) | 0.097 m |
| Satellite EIRP density |  | 21.45 dBW/MHz |
| Satellite Tx max Gain |  | 11 dBi |
| 3dB beam width (HPBW) |  | 104.7 degree |
| Satellite beam diameter (Note 2) |  | 1700 km |
| Payload characteristics for UL transmissions |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 0.097 m |
| G/T |  | - 18.6 dB·K-1 |
| Satellite Rx max Gain |  | 11 dBi |

NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811

NOTE 2: Satellite beam diameter is at Nadir point

NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821

NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.

In this contribution, we provide link budget analysis based on the agreed IoT NTN reference scenario parameters.

# Discussion

Three different IoT NTN scenarios and their respective reference parameters have been agreed [4]. A common set of link budget parameters (e.g., UE power class, UE noise figure, scintillation loss, atmospheric loss, polarization loss, shadow margin, etc) have been agreed [2]. Furthermore, besides two existing satellite parameter sets (i.e., Set 1 and Set 2 in [5]), two additional satellite parameter sets (i.e., Set 3 and Set 4) have been agreed to be used for IoT NTN link budget calculations.

These parameters are used in our link budget analysis.

For set 1 satellite parameters in [5], the link budget evaluations for downlink are provided in Table 1 and the link budget evaluations for uplink is provided in Table 2.

Observation 1: For set 1 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.98, 2.22 and 1.60 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 2: For set 1 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -3.12/-13.91, 5.18/-5.61 and 10.56/-0.23 dB for GEO, LEO-1200 and LEO-600, respectively.

Table 1: DL NB-IoT/eMTC link budget based on set 1 satellite parameters in [5]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 59 |  | 40 |  | 34 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 | 0.18 | 1.08 | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 81.55 | 89.33 | 62.55 | 70.33 | 56.55 | 64.33 |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.58 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| IoT antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  | -174 |  | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 | -121.45 | -113.67 | -121.45 | -113.67 |
| IoT noise figure (dB) | 9 |  | 9 |  | 9 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| CNR (dB) | -4.98 | -4.98 | 2.22 | 2.22 | 1.60 | 1.60 |

Table 2: UL NB-IoT/eMTC link budget based on set 1 satellite parameters in [5]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| IoT device max Tx power (dBm) | 20 |  | 20 |  | 20 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| IoT device EIRP (dBm) | 20 |  | 20 |  | 20 |  |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.58 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| Antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| G/T (dB/K) | 19 |  | 1.1 |  | 1.1 |  |
| Satellite Rx gain (dBi) | 43.63 |  | 25.72 |  | 25.72 |  |
| Channel bandwidth (MHz) | 0.015 | 0.18 | 0.015 | 0.18 | 0.015 | 0.18 |
| CNR (dB) | -3.12 | -13.91 | 5.18 | -5.61 | 10.56 | -0.23 |

For set 2 satellite parameters in [5], the link budget evaluations for downlink are provided in Table 3 and the link budget evaluations for uplink is provided in Table 4.

Observation 3: For set 2 satellite parameters, the CNR for DL NB-IoT/eMTC is -10.48, -3.78 and -4.40 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 4: For set 2 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -8.12/-18.91, -0.82/-11.61 and 4.56/-6.23 dB for GEO, LEO-1200 and LEO-600, respectively.

Table 3: DL NB-IoT/eMTC link budget based on set 2 satellite parameters in [5]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 53.5 |  | 34 |  | 28 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 | 0.18 | 1.08 | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 76.05 | 83.83 | 56.55 | 64.33 | 50.55 | 58.33 |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.58 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| IoT antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  | -174 |  | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 | -121.45 | -113.67 | -121.45 | -113.67 |
| IoT noise figure (dB) | 9 |  | 9 |  | 9 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| CNR (dB) | -10.48 | -10.48 | -3.78 | -3.78 | -4.40 | -4.40 |

Table 4: UL NB-IoT/eMTC link budget based on set 2 satellite parameters in [5]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| IoT device max Tx power (dBm) | 20 |  | 20 |  | 20 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| IoT device EIRP (dBm) | 20 |  | 20 |  | 20 |  |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.58 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| Antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| G/T (dB/K) | 14 |  | -4.9 |  | -4.9 |  |
| Satellite Rx gain (dBi) | 38.62 |  | 19.72 |  | 19.72 |  |
| Channel bandwidth (MHz) | 0.015 | 0.18 | 0.015 | 0.18 | 0.015 | 0.18 |
| CNR (dB) | -8.12 | -18.91 | -0.82 | -11.61 | 4.56 | -6.23 |

For set 3 satellite parameters in [2], the link budget evaluations for downlink are provided in Table 5 and the link budget evaluations for uplink is provided in Table 6.

Observation 5: For set 3 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.18, -4.08 and -4.10 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 6: For set 3 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -5.42/-16.21, -8.72/-19.51 and -3.34/-14.13 dB for GEO, LEO-1200 and LEO-600, respectively.

Table 5: DL NB-IoT/eMTC link budget based on set 3 satellite parameters in [2]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 59.8 |  | 33.7 |  | 28.3 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 | 0.18 | 1.08 | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 82.35 | 90.13 | 56.25 | 64.03 | 50.85 | 58.63 |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.20 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| IoT antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  | -174 |  | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 | -121.45 | -113.67 | -121.45 | -113.67 |
| IoT noise figure (dB) | 9 |  | 9 |  | 9 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| CNR (dB) | -4.18 | -4.18 | -4.08 | -4.08 | -4.10 | -4.10 |

Table 6: UL NB-IoT/eMTC link budget based on set 3 satellite parameters in [2]

| Satellite orbit | GEO |  | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB-IoT | eMTC | NB-IoT | eMTC | NB-IoT | eMTC |
| IoT device max Tx power (dBm) | 20 |  | 20 |  | 20 |  |
| IoT device antenna gain (dBi) | 0 |  | 0 |  | 0 |  |
| IoT device EIRP (dBm) | 20 |  | 20 |  | 20 |  |
| Central beam edge elevation (degree) | 12.5 |  | 30 |  | 30 |  |
| Max. distance between satellite and IoT device (km) | 40308 |  | 1998 |  | 1075 |  |
| Carrier frequency (GHz) | 2 |  | 2 |  | 2 |  |
| Free space path loss (dB) | 190.58 |  | 164.48 |  | 159.10 |  |
| Shadowing (dB) | 3 |  | 3 |  | 3 |  |
| Atmospheric path loss (dB) | 0.2 |  | 0.1 |  | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  | 2.2 |  | 2.2 |  |
| Polarization loss (dB) | 3 |  | 3 |  | 3 |  |
| Antenna temperature (K) | 290 |  | 290 |  | 290 |  |
| G/T (dB/K) | 16.7 |  | -12.8 |  | -12.8 |  |
| Satellite Rx gain (dBi) | 41.32 |  | 11.82 |  | 11.82 |  |
| Channel bandwidth (MHz) | 0.015 | 0.18 | 0.015 | 0.18 | 0.015 | 0.18 |
| CNR (dB) | -5.42 | -16.21 | -8.72 | -19.51 | -3.34 | -14.13 |

For set 4 satellite parameters in [2], the link budget evaluations for downlink are provided in Table 7 and the link budget evaluations for uplink is provided in Table 8.

Observation 7: For set 4 satellite parameters, the CNR for DL NB-IoT/eMTC is -10.95 dB.

Observation 8: For set 4 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -9.14/-19.93 dB.

Table 7: DL NB-IoT/eMTC link budget based on set 4 satellite parameters in [2]

| Satellite orbit | LEO-600 (DL) |  |
| --- | --- | --- |
|  | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 21.45 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 44.00 | 51.78 |
| Central beam edge elevation (degree) | 30 |  |
| Max. distance between satellite and IoT device (km) | 1075 |  |
| Carrier frequency (GHz) | 2 |  |
| Free space path loss (dB) | 159.10 |  |
| Shadowing (dB) | 3 |  |
| Atmospheric path loss (dB) | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  |
| Polarization loss (dB) | 3 |  |
| IoT antenna temperature (K) | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 |
| IoT noise figure (dB) | 9 |  |
| IoT device antenna gain (dBi) | 0 |  |
| CNR (dB) | -10.95 | -10.95 |

Table 8: UL NB-IoT/eMTC link budget based on set 4 satellite parameters in [2]

| Satellite orbit | LEO-600 |  |
| --- | --- | --- |
|  | NB-IoT | eMTC |
| IoT device max Tx power (dBm) | 20 |  |
| IoT device antenna gain (dBi) | 0 |  |
| IoT device EIRP (dBm) | 20 |  |
| Central beam edge elevation (degree) | 30 |  |
| Max. distance between satellite and IoT device (km) | 1075 |  |
| Carrier frequency (GHz) | 2 |  |
| Free space path loss (dB) | 159.10 |  |
| Shadowing (dB) | 3 |  |
| Atmospheric path loss (dB) | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  |
| Polarization loss (dB) | 3 |  |
| Antenna temperature (K) | 290 |  |
| G/T (dB/K) | -18.6 |  |
| Satellite Rx gain (dBi) | 6.02 |  |
| Channel bandwidth (MHz) | 0.015 | 0.18 |
| CNR (dB) | -9.14 | -19.93 |

# Conclusion

In this contribution, we provided our link budget analysis on IoT over NTN. Our observations are as follows:

Observation 1: For set 1 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.98, 2.22 and 1.60 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 2: For set 1 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -3.12/-13.91, 5.18/-5.61 and 10.56/-0.23 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 3: For set 2 satellite parameters, the CNR for DL NB-IoT/eMTC is -10.48, -3.78 and -4.40 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 4: For set 2 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -8.12/-18.91, -0.82/-11.61 and 4.56/-6.23 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 5: For set 3 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.18, -4.08 and -4.10 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 6: For set 3 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -5.42/-16.21, -8.72/-19.51 and -3.34/-14.13 dB for GEO, LEO-1200 and LEO-600, respectively.

Observation 7: For set 4 satellite parameters, the CNR for DL NB-IoT/eMTC is -10.95 dB.

Observation 8: For set 4 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 15 kHz/180 kHz is -9.14/-19.93 dB.

# References

- RP-202689, “New study WID on NB-IoT/eMTC support for NTN,” Dec. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104-e Meeting, Feb. 2021.

- R1-2102102, “Summary #4 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC,” Feb. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.