3GPP TSG RAN WG1 #104bis-e		R1-2103461

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.8.1.2 TB Processing over Multi-Slot PUSCH

Source: 		Sierra Wireless

Title: 		Design Considerations for TB Processing over Multi-Slot PUSCH

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for the following coverage enhancement objectives in the WID [2]:

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots

Agreements from previous meetings:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

- The same number of PRBs per symbol is allocated across slots for TBoMS transmission.

- Consecutive physical slots for UL transmission can be used for TBoMS for paired spectrum and the SUL band

  - FFS if non-consecutive physical slots for UL transmission are also supported for paired spectrum and the SUL band

For TBoMS, the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers.

- FFS: Details and further constraints on the applicability of TBoMS.

One or two of the following approaches will be considered as a starting point to decide howNinfo NInfo for TBoMS is calculated (aiming for down selection in RAN1 #104-bis-e):

- Approach 1: Based on all REs determined across the symbols or slots (FFS whether symbols or slots are used) over which the TBoMS transmission is allocated

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K

Note: L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

One or two of the following options will be considered (aiming for down-selection in RAN1#104b-e) to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols or slots (FFS whether symbol or slot are used) over which the TBoMS transmission is allocated.

  - FFS: if either the number of symbols or the number of slots is used.

  - FFS: if xOverhead is separately configured from the one in Rel-15/16.

# Multi-Slot Encoding

A common solution to achieve a lower code rate for a multi-slot transmission is to encode transport blocks over multiple slots. This is especially useful for a larger TBS which are needed when a large number of repetitions are used. For example, in [1] the increased code rate going from TBS=888 with 8 repeats to TBS=1800 with 16 repeats lead to a decline in performance. To determine the performance of multi-slot encoding, LLS simulations were conducted with different TBS, number of repetitions, and multi-slot encoding options. For a fair comparison, all results have a data rate of 100kbps. Note: doppler frequency was set at 2Hz and 4 PRB were used for all data points. Full simulation assumptions this paper can be found in Appendix A.

Figure 1: LLS Results of Multi-Slot Encoding vs. no Multi-Slot Encoding at Fdop = 2Hz

As the graph clearly shows, there is no coverage gain between TBS=888 encoded over 1 slot and TBS=1780 encoded over >=2 slots at low mobility.  The gain between the black line (no multi-slot encoding, TBS 1780) and Green and Red lines (with multi-slot encoding) only occurs because the code rate improved but improving code rate can also be done by increasing the number of PRBs used for encoding (e.g. from 4 to 8).

Either multi-slot Encoding or increasing number of PRBs used for encoding can improve the code rate for larger TBS

The above results were simulated at a low UE speed. To show the performance of multi-slot encoding for higher UE speeds, the LLS simulations were conducted at different doppler frequencies (2Hz, 19Hz, 78Hz corresponding to UE speeds of 3km/h, 30km/h, 120km/h respectively at the carrier frequency of 700MHz). Note: The TBS has been adjusted to maintain data rates of 100kbps.

Figure 2: LLS Simulation of No Multi-Slot Encoding vs. Multi-Slot Encoding for dop = 2, 19, 78Hz

Encoding over multiple slots with a higher TBS of 1780 results in a longer transmission time than no multi-slot encoding with a TBS of 888 (16ms vs. 8ms). This increased transmission time leads to improved time diversity which leads to marginal coverage gain at higher doppler frequencies.

Multi-slot encoding provides no coverage gain for indoor and low mobility scenarios (e.g. 2Hz doppler).

Multi-slot encoding provides only marginal coverage gain (e.g. 0.5dB) for high mobility scenarios (e.g. 19, 78Hz doppler)

# Multi-Slot Encoding with Gaps

As show above, as multi-slot encoding itself provides little gain for indoor scenarios. Due to inbuilding penetration loss, indoors is the most likely place to find coverage issue and is the hardest scenario to achieve good time diversity since there is usually little UE movement, resulting in a low doppler and large coherence times. As discussed in [1], adding gaps between repeats is a simple and effective way to increase time diversity.

To see determine the gain that can be achieved using gaps, LLS were conducted for gap sizes of 9ms, 15ms, and 31ms and with 8 slots of transmission.

Fig 3: LLS results using, Fdop=2Hz, 8 slots

The above results indicate that adding gaps of 31ms can improve coverage by 1.5dB. To realize such a meaningful gain from increased time diversity, the transmission time should be about half of the coherence time. A 2Hz doppler has a coherence time of ~500ms thus to see meaningful gains a 250ms transmission time is required.  The gap of 31ms provides a transmission time of 32*8=256ms which is why it provides so much gain.  Note that similar time diversity and coverage gains would occur with a smaller gap of 15ms if the transmission time was 16 ms (e.g. TBs encoded over 4 slots , repeats=4).

Adding gaps between repeats can provide more than 1.5dB of coverage gain even for indoor/low dopplers scenarios because of increase time diversity.

Multi-slot encoding can also achieve the desired time diversity but requires a extremely long continuous transmission (e.g. 256ms long). To determine the efficacy of multi-slot encoding with and without gaps, LLS were conducted.

Figure 4: LLS result of comparison of Multi-Slot Encoding with and without gaps

Figure 4 demonstrates that increasing time diversity through either multi-slot encoding with gaps or multi-slot encoding without gaps can provides similar performance. However, there are several advantages of multi-slot encoding with gaps over multi-slot encoding without gaps.

Smaller resources:  To achieve time diversity gains for multi-slot encoding without gaps (black line), the 256ms transmission was achieved by encoding over 64 slots with 4 repeats which required 256 slots of resources.  To achieve time diversity gains for multi-slot encoding with 15ms gaps (red line), the 256ms transmission was achieved by encoding over 4 slots with 4 repeats which required only 16 slots of resources – 16X less.

Support small TBS:  To achieve time diversity gains for multi-slot encoding without gaps (black line), the 256ms transmission requires a TBS=28480 bits to achieve 100kbps.  To achieve time diversity gains for multi-slot encoding with 16ms gaps (red line), the 256ms transmission required a TBS=1780 to achieve 100kbps (this assumes 15 other TBS of 1780 are schedule in the gaps). multi-slot encoding with gaps can provide time diversity for small TBS in a spectrally efficient way where multi-slot encoding without gaps cannot. This is important when lower data rates for eMBB are needed and is especially important for the VoIP use case where small TBS like 320 bits are often required.

Scheduling flexibility:  As mentioned, to achieve time diversity gains for multi-slot encoding without gaps, the 256ms transmission requires 256 continuous slots of resources where multi-slot encoding with gaps requires only 16 slots of non-contiguous resources.   Allowing the gaps to be filled with TBs for other users improves the gNB’s scheduling flexibility which will result in lower latency for other users and improved capacity.  Also, in general, scheduling around smaller resource allocations is easier than larger ones.

Given the above observations, the following proposal is made:

Multi-slot encoding with gaps between repeats should be specified.

FFS: sizes of gaps

# TDRA Considerations

The following agreement was made last meeting WRT TDRA design:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

The results in this paper show that for indoor scenarios (2Hz doppler) TBoMS does not provide any coverage gain where TBoMS with gaps provide 1.5 dB of gain. Thus the ability to specify gaps should be taken into consideration when choosing a TDRA design direction.

No strong view on Type A or B TDRA but the ability to specify gaps should be taken into consideration when choosing a TDRA design direction.

# Maximum number of slots to encode over

The maximum number of slots to encode over is an open issue with many facets to consider include whether repetition with TBoMS is supported:

FFS: impacts and further details if repetitions of TBoMS is supported.

Transmission size:

The transmission size is a function of both the maximum number of repeats and maximum number of slots to encode over. As discussed in [3], there are many reasons why the maximum number of slots of a transmission should be limited to 32ms:

TR 38.830[4] gave guidance as a conclusion that repeats should be increased up to 32.

For a data rate of >=100kbps (eMBB), increasing slots of transmission beyond 8 does not provide any coverage gain

For FR1 TDD VoIP with 100ms latency, the maximum number of slots that can be used is 24.

For FDD VoIP with 100ms latency, HARQ can be effectively used to transmit more slots when needed .

The SID[5] states “LPWA services are not included” thus limiting the maximum number of slots of transmission.

The last reason being the most important – it is outside the scope of this WI.

Complexity:

All the LLS results in this paper have shown that multi-slot encoding is a not a necessary technique. The issue of high coding rate can also be circumvented either by appropriate TB sizing or by using more PRBs. Though gain can be achieved through additional time diversity using multi-slot encoding with gaps. Thus although support for Multi-Slot Encoding should be added, the very minimum amount is all that is necessary.

Multi-slot encoding should be specified with a maximum of 2 slots of encoding.

- Given such a low number of TBoMS slot of encoding, TBoMS with repetition should be supported:

Repetition of TBoMS should be specified.

# Conclusions

- Either multi-slot encoding or increasing number of PRBs used for encoding can improve the code rate for larger TBS

Multi-slot encoding provides no coverage gain for indoor and low mobility scenarios (e.g. 2Hz doppler).

Multi-slot encoding provides only marginal coverage gain (e.g. 0.5dB) for high mobility scenarios (e.g. 19, 78Hz doppler)

Adding gaps between repeats can provide more than 1.5dB of coverage gain even for indoor/low dopplers scenarios because of increase time diversity.

- Multi-slot encoding with gaps between repeats should be specified.

- FFS: sizes of gaps

No strong view on Type A or B TDRA but the ability to specify gaps should be taken into consideration when choosing a TDRA design direction.

Multi-slot encoding should be specified with a maximum of 2 slots of encoding.

Repetition of TBoMS should be specified.

# References

R1-2007930 Potential techniques for NR coverage enhancement, Sierra Wireless

RP-202928 China Telecom, “New WID on NR coverage enhancements”

R1-2103459 Sierra Wireless, “Design Considerations for PUSCH repetition Type A Enhancements”

TR 38830 “Study on NR coverage enhancements”

RP-200861 China Telecom, “Study on NR coverage enhancements”

# Appendix A: LLS Simulation Assumptions:

| Parameter | Value |
| --- | --- |
| UE Speed | As specified |
| Bandwidth Part | 20 MHz |
| Configuration | FDD |
| Carrier frequency | 700MHz |
| RX Antenna ports | 2 |
| Channel model | TDL-C 300ns |
| Number of PRBs | 4 |
| UE Speed | 3, 30,100 km/H As specified |
| Carrier Frequency Offset | None |
| Channel Estimation | Perfect |
| Frequency Hopping | Disabled |
| Frequency Averaging Window | 3 PRBs |
| DMRS Configuration | 2 DMRS per slot |
| Transform Pre-coding | Yes |
| SCS | 15 kHz |
| Modulation | QPSK |
| TBS | As specified |
| Repeats | As specified |
| HARQ | Disabled |
| iBLER Target | 10% |
| Gaps | 0,9, 15, 31ms As specified |
| Multi- Slots encoding | 1,2,4 As specified |