3GPP TSG RAN WG1 #104b-e	R1-210xxxx

e-Meeting, April 12th – 20th, 2021

Source:	Moderator (OPPO)

Title:	Summary#1 of email thread [104b-e-NR-L1enh-URLLC-02]

Agenda Item:	7.2.5

Document for:	Discussion and Decision

# Introduction

In this paper, discussions under the following email thread in RAN1#104b-e are summarized.

[104b-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO):

·        Issue #1: Correction for sub-slot based PUCCH

·        Issue #2: Conflict between the first PUCCH repetition and semi-static configuration

·        Issue #3: Clarification of the configuration for one PUCCH-Config with subslotLengthForPUCCH-r16

·        Issue #6: Msg3 or MsgA PUSCH overlapping with a high-priority PUCCH

·        Issue #4:  Clarification that Type 1 CB is not supported for sub-slot HARQ-ACK

·        Issue #5:  Number of PUCCH resource sets per PUCCH-config (simple fix)

·        (editorial/clarification):  Editorial clarifications related to DCI format 1_2

·        Discussion/decision by April 15 and TP(s) by April 20

# Remaining issues

## Issue#1: Correction for sub-slot based PUCCH

## Inputs from Tdocs

This issue was discussed in the last meeting. The following TP was proposed but not agreed before the meeting was closed.

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause except for section 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

In this meeting, some companies re-submitted the proposal. The observation is: Not all the slots in section 9 should be replaced by sub-slot when a UE is provided subslotLength-ForPUCCH

OPPO proposal:

Proposal 1: Adopt the following text proposal for corrections for sub-slot based PUCCH transmission:

9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause except for section 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit CSI reports and for overlapping PUCCH(s) and PUSCH(s) in a slot, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH. *** Unchanged text is omitted *** ----------------------------------------------------- End of text proposal ------------------------------------------------------

CATT proposal:

Proposal 2: Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

<Unchanged text omitted>

In the remaining of this Clause except for section 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Nokia proposal:

Proposal 2: Adopt the following draft CR to clarify the meaning of “slot” when sub-slot based PUCCH is configured (changes in green, to be shown as track changes in the final CR).

*****************************************

9   UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause, unless stated otherwise, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource for PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH. Providing subslotLengthForPUCCH does not change UE’s processing timelines of MAC-CE commands.

*** Unchanged text is omitted ***

vivo proposal:

Proposal 1: Adopt the following text proposal for sub-slot based PUCCH transmission in 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this Clause except for Clauses 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

## 1st round proposal and discussion

It is suggested to start the discussion from the status of last meeting for this proposal. Companies are encouraged to express your views:

---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause except for section 9.2.2, 9.2.4 and 9.2.5 for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0-----------------------

| Company | Comments |
| --- | --- |
| CATT | We support the TP. |
| vivo | We support the TP. |
| HW/HiSi | We support the TP |
| Nokia, NSB | We would not object to the TP, but would like to point out, that Aris & Souror in the last meeting had been drafting some alternative TP (which seem to be the last discussed version during RAN1#104-e). Has there been any reason to not start from the latest proposal from RAN1#104-e (i.e. from where we seem to have stopped last time)!? ---------------------------------Start of Text Proposal on TS 38.213 v16.4.0----------------------- 9 UE procedure for reporting control information *** Unchanged text is omitted *** In the remaining of this Clause, unless stated otherwise, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource for PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH. *** Unchanged text is omitted *** ---------------------------------End of Text Proposal on TS 38.213 v16.4.0----------------------- |
| ZTE | Fine with the proposal |
| Samsung | Agree with Nokia – the cited text from RAN1#104-e is simpler and cleaner. |
| Apple | We are fine with the proposal in principle. One question: I understand if subslotLength-ForPUCCH is configured, PUCCH for HARQ-ACK, SR and CSI should all be contained within a subslot as we agreed. But for the UCI multiplexing procedure in section 9.2.5, do we really need to say a slot should be replaced by a subslot? Should not the procedure still work on a per-slot basis? That is, Section 9.2.5 still works if “slot” is always interpreted as a slot, not a sub-slot. Basically I wonder if “for multiple PUCCH resources in a slot to transmit PUCCH with CSI reports” can be removed from the TP for simplicity. On the TP from Nokia (or from Aris/Sorour in the last meeting), we feel “unless stated otherwise” is a bit unclear. E.g. is there any place that states otherwise? |
| Qualcomm | We slightly prefer the alternative TP from Aris/Sorour, because it’s simpler and cleaner. To Apple’s question, the UCI mux between CSI and HARQ-ACK in Section 9.2.5 may depend on the sub-slot length. For example, the Issue #3 in [104e-NR-L1enh-URLLC-05] arises because HARQ-ACK operates in sub-slot, but CSI mux operates in slot. Therefore, we can not interpret all “slot” in Section 9.2.5 as a slot. |
| Intel | Slightly prefer the proposed TP as it spells out the exception clauses, but we would also be fine with the proposal from Samsung/Ericsson. |

## Issue#2: Conflict between the first PUCCH repetition and semi-static configuration

## Inputs from Tdocs

This issue was discussed in the last meeting. The following conclusion was proposed but not agreed before the meeting was closed.

Potential conclusion:

It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition for RRC configured UCI transmission) is postponed to the next available UL slot if the PUCCH repetition collides with DL symbol(s) (including semi-static DL symbol(s)) or SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

In this meeting, some companies re-submitted the contribution for clarify this issue.

CATT proposal:

Proposal 1: It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with  SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Nokia proposal:

Observation 1: Different handling for RRC configured and scheduled UCI transmission would lead to unnecessary HARQ-ACK delay for dynamically scheduled PDSCH.

Therefore, we propose to agree the same handling for RRC configured and scheduled UCI transmission along the lines of the RAN1#104-e discussed conclusion by clarifying that this really applies to both cases – RRC configured and scheduled UCI transmission.

Proposal 1: RAN1 to clarify the common / unified handling for RRC configured and scheduled UCI transmission for PUCCH repetition by endorsing the following proposed conclusion:

Proposed conclusion:

It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition for RRC configured and scheduled UCI transmission) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

## 1st round proposal and discussion

It is suggested to start the discussion from the status of last meeting for this issue. Companies are encouraged to express your views:

Potential conclusion:

It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) for RRC configured UCI transmission) is postponed to the next available UL slot if the PUCCH repetition collides with  SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

| Company | Comments |
| --- | --- |
| CATT | Although we are fine with the proposal (adding a missing parenthesis), it seems that companies prefer a unified solution for both configured and scheduled UCI transmission. We are fine with either way. |
| vivo | Not support the potential conclusion. It can be misleading that for DCI scheduled PUCCH can’t be postponed to the next available UL slot. For DCI based PUCCH, even if gNB doesn’t mean to do such scheduling, it is still possible in some cases the first repetition is collided with DL symbols. For example, the k1 timing is RRC configured and there is no PDSCH-to-HARQ_feedback timing indicator field in DCI. or there is no a suitable k1 in the configured k1 set to indicate a UL slot that not collided with semi-static DL symbol. So we prefer to delete “for RRC configured UCI transmission” |
| HW/HiSi | We should also discuss the situation for scheduled PUCCH, the currently proposed conclusion does not capture this. It would be good if a unified solution for configured and scheduled by DCI can be achieved. For a scheduled PUCCH that is colliding with SSB or another DL symbol, would this transmission be cancelled or postponed? We prefer a unified solution, i.e. to postpone. |
| Nokia, NSB | Do not support the proposal. A unified solution for configured and scheduled UCI transmission is better not to cause HARQ-ACK delay for dynamically scheduled PDSCH. Either remove “for RRC configured UCI transmission” from the proposal (based on the CATT input) or mention both situations “for RRC configured and scheduled UCI transmission” (as in Nokia input) |
| ZTE | Fine with the proposal. |
| Samsung | Agree with previous comments (CATT/Vivo/HW/Nokia) – the “for RRC configured UCI transmission” should be removed as no such differentiation is made in the specs. |
| Apple | It is unclear whether the proposed conclusion intends to include the first repetition for scheduled PUCCH transmission also. We are fine to adopt the same behavior for configured and scheduled PUCCH transmission. |
| Qualcomm | We are fine with the proposal. One additional comment, should we delete R-15 in the conclusion, given that we are only clarifying the UE behavior in Rel-16? |
| Intel | Support in principle, and we also prefer to remove the part about “RRC configured UCI transmission”. |

## Issue#3: Clarification of the configuration for one PUCCH-Config with subslotLengthForPUCCH-r16

## Inputs from Tdocs

This issue was addressed in a contribution for the last meeting, but was not scoped into the email discussion.

CATT proposal:

It is possible that UE is provided one PUCCH-config and subslotLengthForPUCCH-r16 is provided in the PUCCH-Config, the PUCCH configuration restriction should also be captured for this case.

Proposal 3: Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

<Unchanged text omitted>

If a UE is provided one PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in the PUCCH-Config is within the subslotLengthForPUCCH symbols in the PUCCH-Config

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

----------------------------------------------------- End of text proposal ------------------------------------------------------

Nokia proposal:

Proposal 3: Adopt the following TP from [CATT R1-2100337] to clarify the PUCCH resource configuration for SR and CSI when only one PUCCH-config has been configured and that PUCCH-config is provided with subslotLengthForPUCCH-r16  (changes in green, to be shown as track changes in the final CR).

------------------------------------------ Start of text proposal on TS 38.213 v16.5.0-----------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

If a UE is provided one PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH-r16 in the PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in the PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the PUCCH-Config

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH-r16 in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH-r16 in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH-r16 symbols in the second PUCCH-Config

----------------------------------------------------- End of text proposal ------------------------------------------------------

DOCOMO proposal:

Text proposal

---------------------------------Start of Text Proposal on TS 38.213 v16.5.0-----------------------

9	UE procedure for reporting control information

<Unchanged parts are omitted>

If a UE is provided one or two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

<Unchanged parts are omitted>

----------------------------------End of Text Proposal on TS 38.213 v16.5.0-------------------------

## 1st round proposal and discussion

Considering the TP from CATT, companies are encouraged to express your views:

- Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

<Unchanged text omitted>

If a UE is provided one PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in the PUCCH-Config is within the subslotLengthForPUCCH symbols in the PUCCH-Config

If a UE is provided two PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config

-	if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| CATT | We support the TP. |
| vivo | Fine in principle. |
| HW/HiSi | In principle fine with the TP. But we would slightly prefer the wording below, which we think is a little more compact description. 9 UE procedure for reporting control information <Unchanged text omitted> If a UE is provided one or two PUCCH-Config - if the UE is provided subslotLengthForPUCCH in the first PUCCH-Config, the PUCCH resource for any SR configuration with priority index 0 or any CSI report configuration in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the first PUCCH-Config If a UE is provided two PUCCH-Config - if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, the PUCCH resource for any SR configuration with priority index 1 in any PUCCH-Config is within the subslotLengthForPUCCH symbols in the second PUCCH-Config -------------------------------------------- End of text proposal ------------------------------------------- |
| Nokia, NSB | We support the TP (and prefer the separate handling compared to the HW/HiSi proposed version). |
| ZTE | No objection if most companies think this should be clarified. Actually, the original spec is clear and no misunderstanding here. |
| Samsung | Prefer Docomo’s proposal – it is simpler and there is no ambiguity (If a UE is provided one or two PUCCH-Config), can even be an editorial correction. |
| Apple | We are fine with the TP. The other versions of TPs are also fine with us, as long as the issue is clarified. |
| Intel | Fine with either the version. Slight preference for the more compact version from DCM. |

## Issue#6: Discussion on Msg3 or MsgA PUSCH overlapping with a high-priority PUCCH

## Inputs from Tdocs

Agreements: No PHY priority is defined for PRACH for intra-UE collision handling within a carrier. - It is per UE implementation for handling the collision.

The above agreement implies that it is up to UE implementation to handle the overlap of PRACH with PUCCH/PUSCH/SRS; this is regardless of whether the PUCCH/PUSCH is of high or low (PHY) priority.

Nokia proposal:

In TS 38.213 Sec. 9, the Rel-15/Rel-16 multiplexing and prioritization rules are defined. Our understanding is that, in these rules, a PUSCH may be a PUSCH corresponding to RAR UL grant or corresponding to MsgA. A PUSCH corresponding to RAR UL grant or corresponding to MsgA is treated as low priority (i.e. priority index 0). However, such PUSCH may overlap with a high-priority PUCCH, in which case the PUSCH is always dropped based on the current specifications of the PHY priority handling. Depending on the use case triggering the RACH procedure, a PUSCH corresponding to RAR UL grant or corresponding to MsgA would need to be prioritized over the overlapping channel (if any). In our view, a reasonable UE implementation should be able to select the suitable channel to prioritize for the case of high-priority PUCCH overlapping with PUSCH of Msg3 or Msg A, mainly depending on the urgency of the use case triggering the RACH procedure.

We thus believe that the agreement on PRACH (copied above) should be extended to also cover PUSCH corresponding to RAR UL grant or corresponding to MsgA. And the current specifications should be updated accordingly.

Proposal 2.1: When a PUSCH corresponding to RAR UL grant or corresponding to MsgA overlaps with a high-priority PUCCH, it’s up to UE implementation to handle the overlap.

Proposal 2.2: Adopt the following draft CR that handling the overlap between PUSCH corresponding to RAR UL grant or corresponding to MsgA and a high-priority PUCCH is left up to UE implementation (changes in green, to be shown as track changes in the final CR).

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.

If a UE receives a RAR UL grant or determines MsgA grant, and the corresponding PUSCH transmission overlaps with a transmission of a PUCCH of priority index 1, it’s up to the UE to decide which of the overlapping transmissions should be prioritized.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

## 1st round proposal and discussion

Considering the TP from Nokia, companies are encouraged to express your views:

- Adopt the following CR for TS 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9	UE procedure for reporting control information

*** Unchanged text is omitted ***

A UE does not expect to be scheduled to transmit a PUCCH or a PUSCH with smaller priority index that would overlap in time with a PUCCH of larger priority index with HARQ-ACK information only in response to a PDSCH reception without a corresponding PDCCH. A UE does not expect to be scheduled to transmit a PUCCH of smaller priority index that would overlap in time with a PUSCH of larger priority index with SP-CSI report(s) without a corresponding PDCCH.

If a UE receives a RAR UL grant or determines MsgA grant, and the corresponding PUSCH transmission overlaps with a transmission of a PUCCH of priority index 1, it’s up to the UE to decide which of the overlapping transmissions should be prioritized.

*** Unchanged text is omitted ***

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| CATT | We think it is not essential and current specification works. |
| vivo | The TP is not needed. In RAN1#101e-meeting, one conclusion was made about Msg A PUSCH in 2-step RACH session: Conclusion: · For single cell operation or for operation with carrier aggregation in a same frequency band, it is up to UE whether to transmit MsgA PUSCH and/or PUSCH/PUCCH/SRS within a same slot or when the gap is not satisfied. - Note: it is not intended to have any impact on UE capability signaling The conclusion can cover the case of overlapping and PUSCH/PUCCH can be with any priority. |
| HW/HiSi | We don’t think that a spec change is needed. It has been agreed to leave the handling of PRACH for intra-UE collision to the UE implementation. Therefore, for Msg3 and MsgA PUSCH, it shall also be left to the UE implementation. If companies think this needs to be clarified, we could make a RAN1 conclusion for that. |
| Nokia, NSB | Thanks to vivo for pointing out the related conclusion from the discussions on 2-step RACH (clarifying the handling for MsgA PUSCH). Having noted this, then at least we should make a similar conclusion or agreement on the case of ‘regular RACH’ procedure (i.e. 4 step) for the handling of RAR UL grant would be still needed. So we see 2 options here on how to proceed: - Alt. 1: We only clarify the same behavior (up to UE to handle the overlap of RAR UL grant with PUCCH of priority index 1) - Alt. 2: We basically capture both of these (for RAR UL grant and Msg A PUSCH) in the specifications by agreeing the TP. We have a slight preference on Alt. 2 from our side to capture this in the specifications by agreeing the TP, but clearly at least Alt. 1 would be needed to be clarified (aligned with the MsgA PUSCH conclusion for 2-step RACH). |
| ZTE | No need to change. As the proposal said, the behavior is up to UE implementation, it is not necessary to specify it if gNB already prepares to detect either PUSCH related to RACH procedure or PUCCH of priority index 1. On the other hand, we appreciate that UE prioritize the PUSCH transmission related to 4-step RACH procedure, as the UE behavior is deterministic for gNB. |
| Samsung | Agree with previous comments – the TP is unnecessary. |
| Apple | We also do not think spec change is needed. We are fine with making a conclusion if that addresses Nokia’s concern. |
| Qualcomm | Agree with previous comments – the TP is not essential, and current specification works. |
| Intel | Does not seem like an essential correction. |

## Issue#4: Clarification that Type 1 CB is not supported for sub-slot HARQ-ACK

## Inputs from Tdocs

The intention of the proposal is to clarify that Type-1 HARQ-ACK codebook is not supported for sub-slot based HARQ-ACK feedback.

CATT proposal:

Proposal 4: Adopt the following TP for Type- 1 HARQ-ACK codebook in TS38.213 section 9.1.2.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.1.2	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE does not expect to be configured with pdsch-HARQ-ACK-Codebook = semi-static for a codebook if a UE is provided subslotLength-ForPUCCH for the codebook.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Nokia proposal:

Proposal 4: Adopt the following draft TP from [CATT R1-2100337] that Type 1 CB is not supported with sub-slot HARQ-ACK (changes in green, to be shown as track changes in the final CR).

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.1.2	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE does not expect to be configured with pdsch-HARQ-ACK-Codebook = semi-static for a codebook if a UE is provided subslotLength-ForPUCCH for the codebook.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.

----------------------------------------------------- End of text proposal ------------------------------------------------------

## 1st round proposal and discussion

Considering the TP from CATT, companies are encouraged to express your views:

- Adopt the following CR for TS 38.213 section 9.1.2.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.1.2	Type-1 HARQ-ACK codebook determination

This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE does not expect to be configured with pdsch-HARQ-ACK-Codebook = semi-static for a codebook if a UE is provided subslotLength-ForPUCCH for the codebook.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format.

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| CATT | We support the TP. |
| vivo | fine with the TP. |
| HW/HiSi | Agree with the TP |
| Nokia, NSB | We support the TP. |
| ZTE | No objection if most companies support this. But the agreement for not supporting sub-slot based Type-1 codebook is enough, it slightly seems the CR is not needed. |
| Samsung | No need for the TP – not an essential correction. |
| Apple | Support |
| Qualcomm | We don’t think the TP is needed. But we will not object if most companies support it. |
| Intel | Not really necessary, but would not object. |

## Issue#5: Number of PUCCH resource sets per PUCCH-config

## Inputs from Tdocs

Nokia proposal:

Proposal 5: Adopt the following draft TP from [NTT Docomo R1-2101584] that there may be up to four sets of PUCCH resources per PUCCH-Config (changes in green, to be shown as track changes in the final CR).

---------------------------------Start of Text Proposal on TS 38.213 v16.5.0-----------------------

9.2.1	PUCCH Resource Sets

<Unchanged parts are omitted>

A UE can be configured up to four sets of PUCCH resources in a PUCCH-Config. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

<Unchanged parts are omitted>

DOCOMO proposal:

---------------------------------Start of Text Proposal on TS 38.213 v16.5.0-----------------------

9.2.1	PUCCH Resource Sets

<Unchanged parts are omitted>

A UE can be configured up to four sets of PUCCH resources in a PUCCH-Config. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

<Unchanged parts are omitted>

----------------------------------End of Text Proposal on TS 38.213 v16.5.0-------------------------

## 1st round proposal and discussion

Considering the TP from DOCOMO, companies are encouraged to express your views:

- Adopt the following CR for TS 38.213 section 9.2.1.

---------------------------------Start of Text Proposal on TS 38.213 v16.5.0-----------------------

9.2.1	PUCCH Resource Sets

<Unchanged parts are omitted>

A UE can be configured up to four sets of PUCCH resources in a PUCCH-Config. A PUCCH resource set is provided by PUCCH-ResourceSet and is associated with a PUCCH resource set index provided by pucch-ResourceSetId, with a set of PUCCH resource indexes provided by resourceList that provides a set of pucch-ResourceId used in the PUCCH resource set, and with a maximum number of UCI information bits the UE can transmit using a PUCCH resource in the PUCCH resource set provided by maxPayloadSize. For the first PUCCH resource set, the maximum number of UCI information bits is 2. A maximum number of PUCCH resource indexes for a set of PUCCH resources is provided by maxNrofPUCCH-ResourcesPerSet. The maximum number of PUCCH resources in the first PUCCH resource set is 32 and the maximum number of PUCCH resources in the other PUCCH resource sets is 8.

<Unchanged parts are omitted>

----------------------------------End of Text Proposal on TS 38.213 v16.5.0-------------------------

| Company | Comments |
| --- | --- |
| CATT | We support the TP. |
| vivo | Fine with the TP. |
| Hw/HiSi | We think that the spec is clear, the description for PUCCH resource sets is within one codebook. |
| Nokia, NSB | We support. Could be referred to the editor CR. |
| ZTE | No objection to this clarification. |
| Samsung | OK for editorial correction although not necessary as it is evident from 38.331. |
| Apple | Support |
| Qualcomm | Fine with the TP. |
| Intel | Fine with the TP. |

## (editorial/clarification): Editorial clarifications related to DCI format 1_2

## Background

In RAN1#102-e, the following TP for section 9.1.2.1 of 38.213 was approved Error: Reference source not found. However, the second modification was not reflected in the specification. In addition, a similar correction is needed for section 9.1.4 of 38.213.

## 1st round proposal and discussion

Companies are encouraged to check the CR proposed by CATT for the editorial clarifications:

#### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel - <Unchanged text is omitted> If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with one serving cell, and - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release. If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with more than one serving cells, or - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE repeats times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release. - <Unchanged text is omitted>

---------------------------------------------------- Start of text proposal ------------------------------------------------------

9.1.4	Type-3 HARQ-ACK codebook determination

<Unchanged text omitted>

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions for a serving cell  and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell , and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided, the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 5: Adopt the following TP for DCI format 1_2.

---------------------------------------------------- Start of text proposal ------------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged text omitted>

If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if

-	the UE is configured with more than one serving cells, or

-	, and

-	PDSCH-CodeBlockGroupTransmission is provided to the UE

the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release.

<Unchanged text omitted>

9.1.4	Type-3 HARQ-ACK codebook determination

<Unchanged text omitted>

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions for a serving cell  and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell , and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided, the UE repeats  times the HARQ-ACK information for the transport block in the PDSCH.

----------------------------------------------------- End of text proposal ------------------------------------------------------

| Company | Comments |
| --- | --- |
| CATT | We support the TP. |
| vivo | Fine in principle. |
| HW/HiSi | Support |
| Nokia, NSB | We support – could be referred to the editor CR. |
| ZTE | No objection to this proposal. |
| Samsung | Support – it is editorial. |
| Apple | Support |
| Qualcomm | Fine with the TP. |
| Intel | Fine with the TP. |

# References

- R1-2102372	Maintenance on UCI enhancements	OPPO

- R1-2102594	Remaining issues on UCI enhancements	CATT

- R1-2102822	Maintenance of Rel-16 URLLC UCI enhancements	Nokia, Nokia Shanghai Bell

- R1-2102824	PHY priority handling for Msg 3 / Msg A and PHR / power scaling for UL skipping & UL CI			Nokia, Nokia Shanghai Bell

- R1-2102943	Maintenance on UCI enhancements	vivo

- R1-2103270	Remaining issues on UCI enhancements for eURLLC	Apple Europe Limited

- R1-2103557	Corrections on UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC.