3GPP TSG-RAN WG1 Meeting #105	Tdoc R1-2104218

Electronic meeting, May 10th – May 27th 2021

Agenda Item:	8.3.1.2

Source:	Ericsson

Title:	CSI Feedback Enhancements for IIoT/URLLC

Document for:	Discussion

# Introduction

Rel-17 IIoT/URLLC working has been approved and scope was revised recently [1]. Among other objectives, the first objective says:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering:

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

At RAN1#104 it was concluded to continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B in R1-2102132 [3]:

Case 1-1: Statistical CSI/SINR

Case 1-2: CSI prediction

Case 1-3: Interference statistics

Case 1-4: Interference covariance matrix

Case 1-5: CSI based on worst IMR occasion

Case 1-6: Worst-M CQI

Case 1-7: Worst-best criteria for subband CQI report

Case 1-8: 3-bit differential subband CQI report

Case 1-9: Reference wideband CQI excludes worst subbands

Case 1-10: CSI expiration time

Case 1-11: Partial information update

Case 2-1: Decoding margin

Case 2-2: Block error probability

Case 2-3: (Delta) CQI/MCS/SINR

Case 2-4: HARQ redundancy version sequence

Case 2-5: Reason for NACK

Case 2-6: Number of NACK values

At RAN1#104e-bis the following was concluded and agreed with respect to Case 1 and Case 2 reporting [4]:

Conclusion:

For new reporting Case 1, do not consider further the following schemes:

- Case 1-2: CSI prediction

- Case 1-4: Interference covariance matrix

- Case 1-9: Reference wideband CQI excludes worst sub-bands

- Case 1-10: CSI expiration time

Agreements:

For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3):

- Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.)

  - Companies are encouraged to provide more details in their analysis

- FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits)

- FFS: Whether quantity reported is relative to the scheduled MCS

Agreement: Focus study on the following for new reporting Case 1:

- Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection.

  - Downselect by RAN1#105 to at most a single method from the following options:

    - Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

    - CSI based on worst IMR occasion (FFS details)

    - Interference standard deviation (FFS details)

    - Worst-M CQI (FFS details)

  - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

  - Applicable for same reporting quantity as R16 for CQI.

  - FFS: Whether network configured channel and interference measurement interval can also be applied

  - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated

FFS: whether the CQI processing time can be is reduced compared to Rel-16 CSI processing delay

In this paper we present our view on the CSI feedback enhancements listed for further study.

# Discussion

## New CSI reporting Case 1

As presented earlier, the CSI feedback enhancement objective for Rel-17 IIoT/URLLC work item was to allow for more accurate MCS selection [1]. In our opinion the MCS selection is inaccurate due to the fundamental limitation that CSI is measured at a time instance before a PDSCH is transmitted. Due to this fact it is impossible to perform link adaption for a PDSCH having perfect knowledge of the radio quality experienced by the PDSCH. Therefore, there will always be some degree of uncertainty that link adaption has to deal with. However, link adaptation is in a much better position to select a more accurate MCS if the uncertainty in reported CSI is known. For URLLC with high reliability requirements the link adaptation needs to account for the uncertainty CSI such that link adaption accounts for events that can happen occasionally, i.e. low-likelihood events.

### CSI based on worst IMR occasion

This is referenced as Case 1-5 in [3] and in [5] the scheme reached a UE satisfactory of ~58% with sub-band CSI and ~49% with wide-band CSI in R15 AR/VR scenario without eMBB traffic. In our contribution [6] for last meeting we also evaluated this scheme in R15 AR/VR scenario including also eMBB traffic. In our scenario we could not even reach 25% URLLC UE satisfaction without introducing additional backoff in link adaptation.

This scheme suffers from the inadequacy that “worst CSI observed” is not the same as the worst CSI that can happen. To illustrate the problem with Case 1-5 schemes we can model SINR at a particular time instance to be the outcome of a random variable. If X1, …, Xn-1 are the random variables at time instances t1<t2 …< tn-1, then if the Xi are highly correlated then CSI determined from the outcome will be less un-certain than if the correlation is low. Suppose now that the X1, …, Xn-1 are random variables for SINR (in dB) outcome at n CSI measurement instances. Further assume the Xi to be IID normal distributed with mean μ and std σ. Figure 2.1 shows the PDF (Probability Density Function) of Xi with μ=10 and σ=2 together with 30 outcomes.

Figure 2.1: PDF of Xi with μ=10 and σ=2, and outcome of X1, …, Xn-1 with n=30.

From example in Figure 2.1, it’s clear that 30 observations are far too few to capture samples that rarely happens. For URLLC with high reliability, a CSI report consisting of the lowest CQI observed during a period will be better than the latest observed CQI but there will still be a high degree of uncertainty remaining. To fulfill the requirements, the link adaptation needs to assume a very low percentile of the SINR, i.e. a SINR threshold such as that the probability that the SINR outcome is below the threshold is very low (e.g., 10-5). From this simple model we can conclude that without some additional functionality, i.e. additional backoff in link adaption which need to be guessed or estimated by other means, we cannot expect that Case 1-5 will solve the problem. Link adaption hence faces essentially the same problem as with legacy CSI reports, although in a less severe form.

CSI based on worst IMR occasion suffers from the fact that worst measured occasion is not the worst that can happen.

With CSI based on worst IMR occasion, link adaption still faces the same problem as with legacy CSI report since link adaption need to account for low CSI that happen occasionally.

Based on our observations we propose:

No further consideration of CSI based on worst IMR occasion for R17 CSI enhancements for IIoT/URLLC.

### Worst-M CQI

This scheme is referenced as Case 1-6 in [3]. Case 1-5 and Case 1-6 schemes belong to same family of “CSI for worst measurement resource” where Case 1-5 reports worst CSI among a set of time measurements while Case 1-6 reports worst CSI among a set of frequency measurements. Both schemes are limited by the basic property of a statistical event that to observe a rare event with high confidence the number of observations needs to be large. Clearly, when performing link adaption for URLLC with high reliability requirement, say 1-10-5, link adaption need to account for events that happen at 10-5 probability. Hence, it’s not enough to base link adaptation on “worst observed” unless there is strong confidence that the “worst observed” is also the worst that can happen at the likelihood given by the reliability requirement. Clearly, in some scenarios where, for example, interference is predictable in time but not in frequency, the Worst-M CQI may result in good performance. For example, if all traffic in the network is periodic and especially when there is a common periodicity, then all interfering gNB tend to transmit at periodic intervals and w.r.t to time the interference looks the same. The main uncertainty that remains is where in frequency interfering gNB allocate their transmissions. A report such as Worst-M, where CSI for the worst M resource blocks is reported, will hence capture the un-certainty where in frequency interference will appear. However, if interference is also un-predictable in time, as is the case in general, the Worst-CQI scheme will no longer be effective.

Worst-M CQI scheme is expected to perform well in scenarios where interference is predictable in time, but not in frequency. If interference is un-predictable both in time and frequency Worst-M CQI scheme is expected to only capture the “worst observed” and not the “worst that can happen”.

The scheme has been evaluated in [8] for a Factory automation (FA) scenario and a R15 enabled AR/VR scenario. For both scenarios latency and reliability requirements were set to 1 ms and 1-10-5, respectively.  The link adaption was configured to operate with a 10-5 target BLER. For the FA scenario we understand the simulation assumptions that all traffic in the network is periodic and is only associated with URLLC while in AR/VR scenario traffic in network consists of both URLLC and eMBB traffic where packet arrival is non-deterministic (FTP model 3) for both URLLC and eMBB UEs.

The results in [8] shows that BLER performance for a Wost-2 CQI scheme is almost spot-on the BLER targeted by link adaption for FA scenario, while for AR/VR the resulting BLER is ~9∙10-5. For AR/VR, legacy R16 CSI reporting with sub-band information reaches 10-4 BLER performance. Hence, for AR/VR Worst-M CQI give essentially same performance as with R16 sub-band CSI reporting. According to Observation 3 this is precisely what is expected, i.e. Worst-M CQI performs well when interference is predictable in time but not in frequency while performs worse when interference is un-predictable both in time and frequency.

Based on results in [8], for Factory Automation scenario with periodic traffic, i.e. predictable interference w.r.t time but not frequency, Worst-M CQI performs well.

Based on results in [8], for R15 enabled AR/VR scenario with un-deterministic traffic, i.e. un-predictable interference w.r.t both time and frequency, Worst-M CQI give essentially same performance as R16 sub-band CSI.

Based on our observations we propose:

No further consideration of CSI based on Worst-M CQI for R17 CSI enhancements for IIoT/URLLC.

### Interference standard deviation

This scheme is referenced as Case 1-3 in [3]. In reference [9], the scheme reached 91% satisfied UEs compared to 68% for their baseline scheme. Their interference standard deviation (interference-std) scheme was better than their Case 1-1 scheme with std-SINR which only reached 81%. In Section 2.1.4 analysis is presented that their results does not show the full potential of mean- and std-CQI/SINR schemes.

Our understanding of the interference-std scheme in [9] is that interference-std is standard deviation of the strength of interference. The interference-std and latest observed SINR (derived from latest CQI) are used to determine a “predict” SINR that is used to select an MCS. The “predict” SINR SINRpredict was determined as

SINRpredict=SINRobserved-∆,

where ∆ was the (1-10-5)-th percentile value of interference assuming normal distribution having zero mean and a std equal to the report std-SINR. We understand this as [9] following the same principle as MRC (Maximum Ratio Combining) where in logarithm domain SINR equals SINR=S-I, where S is signal strength and I is interference strength. If UE receiver is MRC, then SINRpredict=SINRobserved-∆ would be good only if CQI would be determined based on average interference; e.g., only then SINRpredict would correspond to 10-5-th percentile of SINR. However, receivers in UEs typically use MMSE (Minimum Mean-Square Error) receiver, or similar, where spatial properties for both channel and interference is taken into account. With such receivers the relation between channel and interference is far more complex which would make the above “MRC-like” adjustment in-accurate. In conclusion, this is not a sufficiently accurate method to motivate the effort to standardize.

Interference standard deviation scheme in [9] seem to rely on MRC-like operation in link adaption.

UE receivers are usually more advanced than MRC which makes MRC-like operation in link adaptation to be in-accurate.

We believe that reporting of interference-std has several drawbacks:

- gNB does not know how interference impact UE decoding performance. This is one of the main reasons why not even eMIMO Work Item has introduced interference-only report to support reciprocity-based beamforming. For TDD, it is plausible that gNB can estimate the channel based on SRS and together with noise-and-interference covariance matrix gNB could in theory make an “optimal” MCS selection. However, this can only be done if gNB has detailed knowledge how each individual UE receiver is implemented, which is an unrealistic assumption. To our understanding, this is the reason why even in eMIMO Work Item, UE is expected to measure and report RI and CQI (but not PMI) based on pre-coded/beamformed CSI-RS, where precoding/beamforming of CSI-RS is determined from SRS.

- Reporting of interference standard deviation does not capture spatial properties of interference

- Interference standard deviation report cannot be used by gNB by itself, but still it is sent in a separate report. This violates the self-contained principle that CSI information gNB needs to make link adaptation decisions should not be sent as partial information in different reports.

Based on our observations and analysis, the following is proposed:

No further consideration of “interference standard deviation” for R17 CSI enhancements for IIoT/URLLC.

### Mean-CQI/SINR and std-CQI/SINR

This scheme is referenced as Case 1-1 in [3]. This scheme has been evaluated by several companies with various success. From the appendix in [3] it can be seen that evaluation of this scheme shows mostly good but there are exceptions. In our opinion, performing evaluation of URLLC performance is difficult and the results are heavily dependent on how well link adaptation achieve its task.

Contribution [6] for RAN1#104b-e describes our variant where UE reports mean and standard-deviation (std) CQI. In R15 enabled AR/VR scenario including for eMBB and URLLC un-deterministic traffic (FTP model 3) more than 97% of the URLLC UE met the latency and reliability requirements (1 ms & 99.999%) where resource utilization (RU) was ~76% when only WB information was available to gNB while RU reduced to ~60% when SB information was reported. The rather high RU is explained by the dominating eMBB traffic.

Among evaluated schemes in [6], reporting CQI mean and variance shows superior performance and do not require LA backoff parameter optimization.

In reference [8] a similar Case 1-1 scheme was evaluated but here mean and std of SINR (instead of CQI) was reported by UE. The Case 1-1 scheme shows relatively good performance, but the potential of the scheme was not shown due to that the reported mean-SINR was reported 3 dB too high by the UE while such discrepancy in reported quality was not present for the other evaluated schemes.

Results in [8] have shown one of the best performance of SINR mean and STD even with artificial 3 dB offset applied.

In reference [9], in their Case 1-1 scheme where, to our understanding, only std-SINR was reported reached 81% satisfied UEs compared to 68% for their baseline scheme. Although this is a significant performance gain the scheme in [9], it is worse than the scheme described in [6]. To our understanding of [9], the link adaption used only std-SINR and latest observed SINR (derived from latest CQI) to determine a “predict” SINR that is used to select a MCS. The “predict” SINR SINRpredict was determined as

SINRpredict=SINRobserved-∆,

where SINRobserved is the observed SINR and ∆ was the (1-10-5)-th percentile value for normal distribution having zero mean and a std equal to the report std-SINR. In our opinion, this method will only produce good results if SINRobserved coincide with mean SINR, which is not a correct assumption in general. In our opinion the results [9] is heavily dependent how much interference was filtered in UE when reporting the R16 CQI.

Although results in [9] showed good performance of 81% satisfied UEs with std-SINR, the results does not show the full potential of mean- and std-SINR scheme. Reported CQI with scheme in [9] need to be used with filtered interference to show good results.

In reference [5], their Case 1-1 scheme where mean- and std-CQI was reported only reached around 30% satisfied users compared ~30% (WB report) and ~50% (SB report) for baseline. Hence, essentially no gain was seen compared to R16 CSI. However, this is likely due to poor utilization of mean- and std-CQI report in link adaptation. To our understand of [5] the link adaption determined a “predicted” CQI, CQIpredict, as “Predict CQI = mean(CQI)-std(CQI)”. Although not stated, we assume that “predict” CQI is used as a quality measure that link adaption assume when selecting the MCS. In our opinion, this isfar too aggressive considering the reliability of 99.999% considered in [5]. In our opinion, significant better results would be obtained if a SINR that would correspond to 10-5-th percentile of an estimated/assumed SINR distribution determined would be used in link adaptation.

Results in [5] showed poor performance results for mean- and std-SINR due to poor utilization of the statistical report, i.e. link adaptation use mean- and std-SINR in a non-optimal way.

Based on the observations and analysis above, it is proposed that:

NR R17 supports enhanced CSI reporting of mean-CQI/SINR and std-CQI/SINR. FFS reporting details, e.g. number of bits, CQI or SINR as metric.

We further discuss difference between this scheme and “only update CQI” scheme in Section 2.1.6. For example, for mean- and std-CQIs/SINR the determination of RI and PMI could be based on multiple interference measurements instead of a single interference measurement.

### Increasing granularity of sub-band CSI

This scheme is referenced as Case 1-6 in [3]. Reference [10] noted a resource utilization of 25.1% with the R16 2-bit sub-band differential CQI which was reduced to 21.1% by increasing the granularity of sub-band CQI to 3 bits. This corresponds to ~16% resource consumption reduction. However, reference [11]  observed that enhanced granularity of sub-band CQI did not provide noticeable performance gains in R15 enabled use case UMa scenario with un-deterministic packet arrival (FTP model 3). Our understanding of the results in reference [8] is that 4-bit sub-band differential CQI improved performance considerably as compared to R16’s 2-bit report in a Factory Automation scenario where all packet arrivals are periodic.

The results in [8] and [11] can be understood as follows:

- If interference is predictable w.r.t time but un-predictable w.r.t frequency, as is the case when all packet arrival is periodic, an increased knowledge of frequency-dependent CSI will improve gNB’s knowledge about the CSI un-certainty. By assuming that interference is dominating the CSI, gNB can use CSI for worst sub-bands in link adaption although PDSCH has a different allocation to ensure a suitable MCS selection for the possible event that interference coincides with the PDSCH allocation.

- If interference is un-predictable w.r.t both time and frequency, as in the case with non-deterministic packet arrival, it does not help to improve knowledge of frequency-dependent CSI. W.r.t to time there is still a high degree of un-certainty that link adaption somehow need to account for.

The results in [8] and [11] indicate that Case 1-6 scheme can improve performance when interference is predictable w.r.t time but not frequency, while little is gained when interference is un-predictable w.r.t both time and frequency.

In general interference is un-predictable w.r.t both time and frequency and in our view a feature that does not give significant performance gain in the general case cannot be justified considering the specification work and increased CSI reporting cost.

In NR, one out of two sub-band sizes can be configured. For a BWP with 100 PRBs the sizes possible sizes are 8 and 16 PRBs, i.e. the number of sub-bands are either 13 or 7. Hence, number of bits nCQI to carry CQI information equals

nCQI=4+ ndCQINSB,

where ndCQI is the number of bits for differential CQI and NSB is the number of sub-bands. Table 1 shows the CQI reporting overhead increase on a BWP with 100 PRBs by an increased granularity of sub-band CSI as compared to the existing 2-bit sub-band differential CQI in R16.

Table 1: CQI overhead increase compared to R16 by increased granularity of sub-band CSI on a BWP with 100 PRBs.

|  | CQI overhead increase compared to R16 |  |
| --- | --- | --- |
| #bits for sub-band differential | 8-PRB sub-band size | 16-PRB sub-band size |
| 3 | +43.3% | +38.9% |
| 4 | +86.7% | +77.8% |

On a BWP of 100 PRBs the CQI overhead is increased by at least ~39% if the granularity of sub-band CSI is increased.

Based on our observations, the following is proposed.

No further consideration of “increasing granularity of sub-band CSI” for R17 CSI enhancements for IIoT/URLLC

### Only update CQI in a report

This scheme is referenced as Case 1-11 in [3] and proposed in [12]. Our understanding of this scheme is that CSI is reported as two components at different time instances where one component is RI/PMI and the other component is CQI. This may appear suitable if channel characteristics, especially spatial characteristics, does not change frequently while interference is fast varying. Then RI/PMI could be determined and reported with a lower frequency than CQI conditioned the RI/PMI could have a higher reporting frequency.

This distribution of CSI reports across multiple instances has been discussed a lot in 3GPP, especially in LTE MIMO standardization. In LTE, the PUCCH format 2 payload is only 11 bits and when FD-MIMO was introduced, the CSI feedback payload increased and it was hard to fit within this constraint. However, RAN1 refrained as much as possible to split a report across multiple CSI reporting instances, due to the risk of mis-detection and error propagation. So the use of this in LTE MIMO was only accepted in very rare case.

In our opinion, Case 1-11  is similar to one of this rare report modes in LTE.  On the other hand, when the payload limitation is not present in LTE, i.e. when CSI is sent on PUSCH, RI/PMI and CQI is kept together. In contrast, in NR, the PUCCH information size limit is much larger: 1706 bits. Hence, there is no size limitation at all to motivate reporting CQI only in NR.

In NR, all components of CSI are kept together in one report, i.e. the CSI is self-contained.  The self-contained principle was one of the key NR design principles for several reasons also beyond error propagation and inter-dependence of reports. For us the self-contained principle is more than just keeping related information together in one report, but also that if some information is reported then the information shall be useful by its own. Hence, RAN1 should not adopt any CSI reporting scheme that does not obey the important self-contained principle in NR.

To summarize, there are two main reasons why we think this principle is important:

- Minimize error propagation: In RI/PMI is first reported and then several CQI values is reported, then it is crucial that the RI/PMI report was received correctly. If the CSI instance containing RI/PMI is not received correctly by gNB, then all the subsequent CQI reports conditioned on the RI/PMI are useless. For URLLC/IIoT on unlicensed band, there is even more uncertainty that two CSI reports may not be both received correctly.

- Minimize standard impact:

  - CSI fields and CSI report construction. Early in standardization of NR there was a strong desire to avoid the complexity of defining a long list of PUCCH reporting modes and types as in LTE. For NR there is no reason to split CSI due to PUCCH size limitation. If RAN1 moves back to LTE-like approach (e.g., one CSI type for RI/PMI only, another for CQI only), there will be significant specification impact, starting with formulating the CSI fields and CSI report.

  - CPU occupancy. Currently, CPU occupancy in NR is defined for all the reporting quantities that are associated with a CSI report of CSI report configuration. If NR accepts reporting RI/PMI separately from CQI, different CPU occupancies may need to be defined for RI/PMI and CQI.

  - CSI timeline. Similarly, CSI timelines need to be discussed since it is quite likely that RI/PMI evaluation takes considerably longer time than determining the CQI value. Furthermore, if we persist to split CSI into two parts then numerous new RRC parameters need to be introduced, for example, period, time offset, PUCCH resource, reference resource timeline.

The Case 1-11 scheme violates the self-contained principle in NR.

It is also observed that the proposed scheme in [12] could essentially be achieved already in R16 framework utilizing two CSI report configurations, one with report quantity ‘cri-RI-PMI-CQI’ and the other with report quantity ‘cri-RI-CQI’, where first configuration report CSI with a lower frequency than second configuration. The RI/PMI reported for first configuration could be used to select a pre-coder to be used for pre-coded CSI-RS associated with second configuration where the CSI reported for second configuration would essentially be a “only update CQI” report. Figure 2.2 illustrates how the two CSI report configurations could be used.

Figure 2.2: Using two R16 CSI report configurations to achieve “only update CQI”.

Let number of CSI-RS ports be υ for both configurations used as illustrated by Figure 2.2. Assume that the UE reports a pre-coder P of rank r w.r.t first configuration. Then if gNB  selects the pre-coder P'=(P, 0, …,0) of dimension υ×υ for the pre-coded CSI-RS used for second configuration the UE would  report rank r with high probability and the reported CQI would be conditioned the pre-coder P. The UE may report a lower rank than r, but we believe those occasions to be rare. If rank for first configuration is restricted to 1, as is often assumed for URLLC, then reported CSI for second configuration is precisely a “only update CQI” report.

Case 1-11 scheme could already be achieved using the R16 CSI  with rank restricted to 1, then current R16 CSI framework already supports precisely the Case 1-11 scheme.

It has been argued during discussions that “mean- and std-CQI/SINR” scheme works essentially the same way as “only update CQI”, i.e. that mean- and std-CQI/SINR are based on interference measurements that occurs after RI/PMI has deduced. However, this is not the way “mean- and std-CQI/SINR” scheme must work. In a more preferred operation the UE determines RI/PMI/mean- and std-CQI jointly w.r.t a CSI-RS occasion and a collection of CSI-IM time occasion, where the CSI-IM time occasions are occurring at time instances before the CSI-RS time instance. Considering that timelines for deducing channel estimate is likely larger than timeline for deducing interference estimate a few of the CSI-IM time instances could occur after the CSI-RS time instance. Such operation of mean- and std-CQI/SINR would significantly out-perform “only update CQI”.

“Mean- and std-CQI/SINR” scheme significantly out-perform “only update CQI” scheme.

Based on our observations, Case 1-11 scheme is not a good solution, and should not be adopted by itself. Thus, the following is proposed:

No further consideration of “updating only CQI in a report” for R17 CSI enhancement for URLLC.

## New CSI reporting Case 2

In the last RAN1#104b-e meeting it was agreed to focus the new reporting Case 2 on the Case 2-3, reporting of delta-CQI/MCS. By letting the UE report suggested change in MCS, an improved link adaptation could be obtained with the possibility for the outer loop to converge without letting the communication degrade so far that block errors occur.

### Metrics for delta-MCS/CQI reporting

Both delta-CQI and delta-MCS were considered as the basis for reporting. While they are closely related to each other, the notion of reporting of delta-MCS is preferable compared to delta-CQI, as the reporting is based on the decoding of an actual package with a specific MCS, which may be different from what the UE previously had reported as CQI. The MCS granularity is also finer than the CQI granularity.

Reporting of delta-MCS is preferable compared to delta-CQI.

For the UE to recommend whether the MCS is adequate or should be adjusted downwards or upwards, it needs to have knowledge about the target BLER, a value that can be semi-statically configured. Depending on the difference between the estimated BLEP and the target BLEP, different delta-MCS may can be transmitted. Only using the values 1e-1 or 1e-5 corresponding to configured CQI tables for CQI reporting is likely not enough, as the gNB may want to target BLER in between. Trying to map reported delta-MCS values based on one target BLER to another MCS associated with a different target BLER may be error prone and depends on the actual implementation in the UE.

The UE needs to know the target BLER in order to recommend delta-MCS. A target BLER can be semi-statically configured.

To be able to make the outer loop converge while having only successful transmissions, the delta-MCS generated with ACKs needs to be possible to send with opposite signs, i.e. the MCS must be possible to either increase or decrease after an ACK.

UE needs to be able to send both positive and negative delta-MCS upon a positive ACK, in order for the outer loop to converge without block error events.

How to generate the delta-MCS values will typically be according to UE implementation. This could e.g. be based on LDPC coder iterations, soft values, or comparing flipped bits before and after LDPC decoding, leading to estimated BLER for certain MCS choices. Since the actual implementations may vary, the behavior could be verified by RAN4 tests, e.g. such that reported delta-MCS varies properly with varying SINR at a fixed MCS.

How generate the estimated BLEP and delta-MCS values may be up to UE implementation, e.g. using LDPC coder iterations, or comparing flipped bits before and after LDPC decoding.

### Where to send delta-MCS bits

The amount of additional information transmitted from the UE to the will increase with the addition of delta-MCS/CQI. This information can either be transmitted bundled into an updated HARQ-ACK codebook, or stand-alone, outside the HARQ-ACK codebook as a form of A-CSI.

#### Transmission in HARQ-ACK codebook

Addition of the extra information to the type-1 codebook may result in putting too much information into the codebook, leading to doubling, or more of the codebook size. A more realistic choice may be to add the delta-MCS to the type 2 codebook.

To limit the number of transmitted bits, the delta-MCS can be specified to only be transmitted if certain conditions occur. Here, however, care must be taken to avoid potential error cases, should e.g. the UE miss the first scheduling DCI. A possible restriction of when to send the delta-MCS can be, report the multi-bit HARQ-ACK for PDSCHs which are provided with high priority indicator in its scheduling DCI.

It is also a question whether a delta-MCS should be transmitted only for first transmissions, or also for retransmissions. Here, it can be expected that only a minor portion of the transmissions generate a retransmission, so the added cost for including retransmissions will be minor. In addition, this makes the standardization simpler and resolves ambiguity in cases where the UE misses the first DCI. The transmitted information related to a retransmission may also help the gNB to schedule properly to fulfil a latency bound. A possible alternative is to only let delta-MCS be transmitted when scheduled with a SPS-PDSCH, since an SPS PDSCH is always the initial transmission, without any ambiguities.

If supporting delta-MCS for dynamically scheduled PDSCH, same format of delta-MCS should be applied both for initial transmissions and retransmissions.

To limit impact to codebook size if adding delta MCS to the HARQ-ACK codebook, the transmission can be restricted to only use delta MCS together with the type 2 HARQ-ACK codebook. Further restrictions can be included, e.g., to only allow it with high priority indication  in DCI; to limit it to SPS PDSCHs.

An example of a table added to HARQ-ACK codebook is given below in Table 2. Typical values could be d0= d1=—1, d2=0, d3=1, but also non-integer values could be used, letting the gNB accumulate or average over multiple reports.

Table 2. Mapping table for 2-bit HARQ-ACK feedback that provides CSI in addition to ACK/NACK

| 2-bit HARQ-ACK feedback value | Information provided |  | Explanation |
| --- | --- | --- | --- |
|  | Feedback on data packet reception | Feedback on CQI/MCS (delta_MCS) |  |
| 00 | NACK | d0 | NACK: The data packet(s) was not successfully received, lower MCS recommended; d0 < 0 |
| 01 | ACK | d1 | ACK: The data packet(s) was successfully received, but lower MCS recommended; d1 < 0 |
| 10 | ACK | d2 | ACK: The data packet(s) was successfully received, acceptable MCS; d2=0 typically |
| 11 | ACK | d3 | ACK: The data packet(s) was successfully received, a higher MCS can be tolerated; d3 > 0 |

If the HARQ-ACK report should include also the extra bit(s) related to the delta-MCS information, this means that there is somewhat more the UE should do before being able to send off the HARQ-ACK. However, we expect the added operations to be limited in comparison to what already is done in the decoding process, and the minimum HARQ-ACK timing N1 does not need to change.

#### Transmission outside HARQ-ACK codebook

An alternative solution is to transmit the delta-MCS bits outside the HARQ-ACK codebook. In that case, it should be transmitted on PUCCH rather than on PUSCH, due to the limited amount of added information. Thus, it will be transmitted as a form of A-CSI on PUCCH.

If delta-MCS should be transmitted outside the HARQ-ACK codebook, it should be transmitted on PUCCH rather than on PUSCH.

With the delta-MCS transmitted on PUCCH, the UE can be configured with parameters related to when and how often to report, which PUCCH resource, and what BLER target to aim at.

When transmitting separately from HARQ-ACK feedback, without the mapping between a PDSCH transmission and the resulting HARQ-ACK, a reference PDSCH needs to be defined for the delta-MCS report to be compared to.

Transmission of delta-MCS on as A-CSI on PUCCH may occupy less resources.

Table 3 below shows an example for a 2-bit delta-MCS table, transmitted separately from the HARQ-ACK feedback. Compared to Table 2 above, this table does not need to provide space for the ACK/NACK feedback, and uses two different levels of suggested MCS decrease, a level that is generated based on the estimated BLEP in relation to the configured target BLER.

Table 3. Mapping table for delta-MCS feedback separate from HARQ-ACK report

| 2-bit delta-MCS feedback value | Information provided | Explanation |
| --- | --- | --- |
|  | Feedback on CQI/MCS (delta_MCS) |  |
| 00 | d0 | Estimated BEP significantly too high. Lower MCS recommended; d0 < 0 |
| 01 | d1 | Estimated BEP slightly too high. Slightly lower MCS recommended; d1 < 0 |
| 10 | d2 | Acceptable MCS; d2=0 |
| 11 | d3 | A higher MCS can be tolerated; d3 > 0 |

## CSI feedback for PDCCH enhancement

There have been several variations of proposals to provide CSI feedback for PDCCH.

While the intention of improving PDCCH reliability is acknowledged, we do not see a strong need for CSI feedback for PDCCH specifically. In our view, current CSI reporting schemes are flexible enough to provide gNB with an acceptable CSI accuracy. For example, a CSI process could be allocated with an enforced rank 1 restriction. This could simplify PDCCH link adaption since otherwise a conversion from rank r CSI to rank 1 CSI is required. For eMBB where high-rank transmissions can be expected, a dedicated CSI process for PDCCH link adaptation could simplify but even in this case it is questionable if the potential performance gain justify the additional complexity. For URLLC the need is even more questionable since low rank PDSCH may often be preferred due to better interference suppression capability in the receiver. Therefore, configuring CSI reporting with rank restriction may often be desired for URLLC. Hence, in case of service mix of eMBB and URLLC within same UE it may anyway be desired to configure UE with two CSI processes, one without rank restriction and one with rank restricted to perhaps even 1.

We agree that accuracy of current CSI can be improved due to differences between PDCCH and PDSCH, e.g. coding scheme, and that CSI reflecting a preferred aggregation level could improve accuracy. However, not even for PDSCH current CSI reporting is not completely accurate since CSI can only be reported for a sub-set of allocation sizes. For example, if CSI is a wideband report then CSI is only accurate if PDSCH is wideband. Hence, if PDSCH has a smaller allocation size then link adaption must be performed based on assumptions on what quality can be expected for the PDSCH. Typically, this is done based on a coding model of the LDPC code which can, given a quality measure e.g. SINR, map a transport format to a BLEP estimate. For link adaption of PDCCH the only difference is that the coding model is a model of polar code instead of LDPC. The accuracy in link adaption hence depends on the accuracy in the coding model. We do not see a reason why the accuracy of a polar code coding model would be much worse than that of a LDCP code. Considering also that PDCCH code rate adaption is very coarse compared to PDSCH we believe there will be no or a very small gain by introducing PDCCH-specific CSI feedback. Instead we believe that one of the main challenges for link adaptation is the SINR un-certainty often due to un-predictable interference variations. This un-certainty cannot be mitigated by improving the accuracy in reported CSI that reflects a past interference situation. For PDSCH we proposed in Section 2.3 statistical CSI to mitigate SINR un-certainty and we strongly believe that such CSI reporting scheme is beneficial also for PDCCH.

Statistical CSI is beneficial also for PDCCH.

Un-certainty in SINR is often mitigated by applying a back-off to “reported” SINR, where back-off could be fixed or adjusted. For adjusted back-off the adjustment can be based on SINR statistics estimated from CSI reports (or statistical CSI reporting if introduced). Alternatively, or additionally, an outer-loop component triggered by DTX detection of HARQ-ACK and/or PUSCH transmissions could be used to determine the back-off. Clearly, DTX detection is associated with potential error cases that can occur and therefore it may be tempting to introduce a more explicit feedback for DCI detection or un-detection. But such feedback will also be associated with error cases that can occur. In some cases, a new feedback could improve gNB ambiguity whether PDCCH or PDSCH failed. In our view, the benefit of such new feedback is limited compared to a behaviour that gNB take actions that PDCCH failed as well as PDSCH failed, especially if occasions for ambiguity are relatively rare events.

Based on discussion above we propose:

No Rel-17 CSI enhancements for PDCCH.

## CSI feedback for SPS PDSCH

It is well known that IIoT/URLLC traffic are periodic and deterministic. DL SPS is very important feature to support such traffic, especially considering that DL SPS allows elimination of latency due to PDCCH scheduling. Based on our simulation study we strongly believe that the reliability of SPS PDSCH can be significantly affected by inaccurate CSI knowledge when the SPS configuration is activated.

The existing CSI feedback is largely designed for dynamic PDSCH transmission. There is no CSI feature that’s designed to specifically fit the need of SPS activation, even though incorrect MCS allocation at SPS allocation has long lasting effect as compared to dynamically scheduled PDSCH. Thus, there is a need to investigate and define CSI feedback features to fit the needs of SPS activation. For example, the CSI-RS/IM configuration should fit the anticipated SPS configuration(s) as much as possible, including periodicity and antenna ports. The CSI feedback should take into account the long-term, periodic nature of DL SPS, e.g., configure a statistical CSI report to track the channel experienced by DL SPS. When a DL SPS configuration is deactivated, then reactivated with new configuration parameters such as periodicity and slot offset, the CSI-RS/IM should adapt to support the new DL SPS configuration.

Rel-17 CSI enhancements are specified to better fit the needs of SPS PDSCH(s).

# Conclusion

Based on the discussions above, the following observations were made:

Observation 1	CSI based on worst IMR occasion suffers from the fact that worst measured occasion is not the worst that can happen.

Observation 2	With CSI based on worst IMR occasion, link adaption still faces the same problem as with legacy CSI report since link adaption need to account for low CSI that happen occasionally.

Observation 3	Worst-M CQI scheme is expected to perform well in scenarios where interference is predictable in time, but not in frequency. If interference is un-predictable both in time and frequency Worst-M CQI scheme is expected to only capture the “worst observed” and not the “worst that can happen”.

Observation 4	Based on results in [8], for Factory Automation scenario with periodic traffic, i.e. predictable interference w.r.t time but not frequency, Worst-M CQI performs well.

Observation 5	Based on results in [8], for R15 enabled AR/VR scenario with un-deterministic traffic, i.e. un-predictable interference w.r.t both time and frequency, Worst-M CQI give essentially same performance as R16 sub-band CSI.

Observation 6	Interference standard deviation scheme in [9] seem to rely on MRC-like operation in link adaption.

Observation 7	UE receivers are usually more advanced than MRC which makes MRC-like operation in link adaptation to be in-accurate.

Observation 8	Among evaluated schemes in [6], reporting CQI mean and variance shows superior performance and do not require LA backoff parameter optimization.

Observation 9	Results in [8] have shown one of the best performance of SINR mean and STD even with artificial 3 dB offset applied.

Observation 10	Although results in [9] showed good performance of 81% satisfied UEs with std-SINR, the results does not show the full potential of mean- and std-SINR scheme. Reported CQI with scheme in [9] need to be used with filtered interference to show good results.

Observation 11	Results in [5] showed poor performance results for mean- and std-SINR due to poor utilization of the statistical report, i.e. link adaptation use mean- and std-SINR in a non-optimal way.

Observation 12	The results in [8] and [11] indicate that Case 1-6 scheme can improve performance when interference is predictable w.r.t time but not frequency, while little is gained when interference is un-predictable w.r.t both time and frequency.

Observation 13	On a BWP of 100 PRBs the CQI overhead is increased by at least ~39% if the granularity of sub-band CSI is increased.

Observation 14	The Case 1-11 scheme violates the self-contained principle in NR.

Observation 15	Case 1-11 scheme could already be achieved using the R16 CSI  with rank restricted to 1, then current R16 CSI framework already supports precisely the Case 1-11 scheme.

Observation 16	“Mean- and std-CQI/SINR” scheme significantly out-perform “only update CQI” scheme.

Observation 17	Reporting of delta-MCS is preferable compared to delta-CQI.

Observation 18	UE needs to be able to send both positive and negative delta-MCS upon a positive ACK, in order for the outer loop to converge without block error events.

Observation 19	How generate the estimated BLEP and delta-MCS values may be up to UE implementation, e.g. using LDPC coder iterations, or comparing flipped bits before and after LDPC decoding.

Observation 20	To limit impact to codebook size if adding delta MCS to the HARQ-ACK codebook, the transmission can be restricted to only use delta MCS together with the type 2 HARQ-ACK codebook. Further restrictions can be included, e.g., to only allow it with high priority indication  in DCI; to limit it to SPS PDSCHs.

Observation 21	Transmission of delta-MCS on as A-CSI on PUCCH may occupy less resources.

Observation 22	Statistical CSI is beneficial also for PDCCH.

Furthermore, the following proposals were made:

Proposal 1	No further consideration of CSI based on worst IMR occasion for R17 CSI enhancements for IIoT/URLLC.

Proposal 2	No further consideration of CSI based on Worst-M CQI for R17 CSI enhancements for IIoT/URLLC.

Proposal 3	No further consideration of “interference standard deviation” for R17 CSI enhancements for IIoT/URLLC.

Proposal 4	NR R17 supports enhanced CSI reporting of mean-CQI/SINR and std-CQI/SINR. FFS reporting details, e.g. number of bits, CQI or SINR as metric.

Proposal 5	No further consideration of “increasing granularity of sub-band CSI” for R17 CSI enhancements for IIoT/URLLC

Proposal 6	No further consideration of “updating only CQI in a report” for R17 CSI enhancement for URLLC.

Proposal 7	The UE needs to know the target BLER in order to recommend delta-MCS. A target BLER can be semi-statically configured.

Proposal 8	If supporting delta-MCS for dynamically scheduled PDSCH, same format of delta-MCS should be applied both for initial transmissions and retransmissions.

Proposal 9	If delta-MCS should be transmitted outside the HARQ-ACK codebook, it should be transmitted on PUCCH rather than on PUSCH.

Proposal 10	No Rel-17 CSI enhancements for PDCCH.

Proposal 11	Rel-17 CSI enhancements are specified to better fit the needs of SPS PDSCH(s).

# References

RP-201310 Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, June 2020.

R1-2100269, CSI Feedback Enhancements for IIoT/URLLC, Ericsson, WG1#104e, January 25th – February 5th 2021.

R1-2102131, Feature lead summay #4 on CSI feedback enhancements for enhanced URLLC/IIoT, WG1#104e, January 25th – February 5th, 2021

R1-2103956, Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT, WG1#104b-e, April 12th – April 20th, 2021

R1-2100102, Discussion on CSI feedback enhancements for eURLLC, ZTE, WG1#104e, January 25th – February 5th, 2021

R1-2102745, CSI Feedback Enhancements for IIoT/URLLC, Ericsson, WG1#104b-e, April 12th – April 20th, 2021

R1-2008862, CSI feedback enhancements for URLLC/IIoT use cases, Nokia, WG1#103e, October 26th- November 13th, 2020

R1-2103434, CSI feedback enhancements for URLLC/IIoT use cases, Nokia, WG1#104b-e, April 12th- April 20th, 2021

R1-2102759, CSI feedback enhancements for URLLC, FUTUREWEI, WG1#104b-e, April 12th- April 20th, 2021

R1-2102695, CSI feedback enhancements for URLLC, MediaTek, WG1#104b-e, April 12th- April 20th, 2021

R1-2100650, CSI feedback enahncements for URLLC/IIoT, Intel, WG1#104e, January 25th – February 5th, 2021

R1-21004337, CSI feedback enhancements for Rel-17 URLLC, Vivo, WG1#104e, January 25th – February 5th, 2021