3GPP TSG RAN WG1 #105-e		   	R1-2104292

e-Meeting, May 10th – 27th, 2021

Agenda Item:		8.1.1

Source:				InterDigital, Inc.

Title:					Remaining Issues on Rel-17 Multi-beam Operation

Document for:	Discussion and Decision

# Introduction

In RAN1#104b-e [1], further details on L1-based beam indication without DL assignment, advanced beam refinement/, beam reporting for L1/L2-centric inter-cell mobility, UE-initiated panel selection/activation for MP-UE and UE reporting on MPE were agreed. In this contribution, we discuss remaining details on Rel-17 beam management.

# Unified TCI framework for DL and UL

In this section, we provide discussions on the issues related to the unified TCI framework.

Advanced beam refinement/tracking

In RAN1#104b-e [1], following candidate schemes of group 1 are agreed to facilitate advanced beam refinement/tracking:

Group 1

- Opt 1-1A: Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request)

  - By allowing beam measurement and reporting triggered by beam indication, latency to acquire beam information can be significantly reduced. In addition, transmitted RS for beam measurement can be used as a final gNB confirmation for beam change.

- Opt 1-1B: UE-initiated beam selection/activation based on beam measurement (without beam indication or activation from NW)

  - UE-initiated beam selection/activation may reduce beam application latency, however, it is preferred to include beam group indication or activation from NW (e.g., TCI state group). UE is able to determine its best beam among beams within the indicated beam group.

- Opt 1-2: Semi-static NW-configured beam selection (without beam indication and measurement/reporting)

  - As we assume that RRC based beam configuration should be available for unified TCI framework as well as Rel-15/16 beam management, no additional specification support is needed.

- Opt 1-3: SSB grouping to reduce beam training

  - Hierarchical beam grouping was discussed from Rel-15, but there’s no specification support on it as no clear benefits are observed.

- Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency

  - As there’s no change on maximum number of beams in Rel-17 unified TCI framework, we don’t see clear technical motivation to allow multiple resource sets for aperiodic beam measurement/reporting.

Proposal 1: For group 1, support beam measurement/reporting/refinement/selection triggered by beam indication (opt 1-1A) for advanced beam refinement/tracking.

Proposal 2: For group 1, UE-initiated beam selection/activation based on beam measurement can be considered if gNB beam group indication or activation is supported.

In addition to group 1, following candidate schemes were agreed for group 2.

Group 2

- Opt 2-1A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking

- Opt 2-1B: Latency reduction for MAC CE based PL-RS activation

- Opt 2-1C: Latency reduction for MAC CE based PUCCH resource/resource group activation

- Opt 2-2: Direct SCell TCI state activation

- Opt 2-3: Replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update

- Opt 2-4: One-shot timing update for TCI state update

For MAC CE based activation methods, the activation time is determined based on whether indicated RSs are known or unknown. For example, for MAC CE based TCI state activation, the delay is based on HARQ delay and additional 3ms, if the TCI state is known. However, if the TCI state is unknown, the delay is based on measurement type, QCL type, RS type and measurement time. As general activation details are specified in RAN4, it is preferred to discuss whether/how to reduce latency for MAC CE based activation methods in RAN4.

RRC based QCL information in periodic RSs is a well-known problem from Rel-15 maintenance. Periodic RSs are crucial features for NR operation. For example, all QCL assumptions are based on periodic TRSs and aperiodic TRSs can be utilized as supplementary RSs of the periodic TRSs. In addition, FR2 beam failure recovery procedures are based on only periodic CSI-RSs and semi-persistent/aperiodic CSI-RSs can’t be utilized. As optimized beams in FR2 dynamically change, beams for periodic RSs should be dynamically changed to efficiently utilize NR system in FR2. However, current specification only supports RRC reconfiguration based QCL update for periodic RSs. Thus, the frequent RRC reconfiguration significantly reduces efficiency of NR operation.

Proposal 3: For group 2, it is preferred to discuss latency reduction of MAC CE related activation in RAN4.

Proposal 4: For group 2, replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update (Opt 2-3) is supported.

DCI formats for separate UL beam indication

When gNB utilizes the existing DCI formats 1_1 and 1_2 for separate UL beam indication, the overall operation may not be efficient as the existing DCI formats 1_1 and 1_2 are DL DCI formats. Figures 1 – 2 show exemplary operations with the DCI formats 1_1/1_2 and the DCI formats 0_1/0_2, respectively, for separate UL beam indication.

Figure 1 Separate UL beam indication with DCI formats 1_1 and 1_2

Figure 2 Separate UL beam indication with DCI formats 0_1 and 0_2

As shown in Figure 1, when gNB utilizes the DCI formats 1_1 and 1_2, UE needs to decode both DL DCI for new beam indication and UL DCI for PUSCH scheduling. In addition, UL scheduling offset and processing time for new beam application should be independently applied as the new beam indication and the PUSCH utilize different DCI formats. On the other hand, when gNB utilizes the DCI formats 0_1 and 0_2 for both new beam indication and PUSCH scheduling as shown in Figure 2, UE needs to decode only UL DCI, so PDCCH decoding latency can be reduced. In addition, processing time for PUSCH transmission and new beam application can be combined if UE implementation allows simultaneous processing of both operations.

Observation 1: For separate UL beam indication, utilization of existing UL DCI formats 0_1 and 0_2 is more efficient as it doesn’t require multiple PDCCH decoding and combined UE processing time is possible if UE implementation allows simultaneous processing.

Proposal 5: For separate UL beam indication, utilization of the existing UL DCI formats 0_1 and 0_2 is preferred.

Beam application time for unified TCI framework

In RAN1#103-e, regarding application time of the beam indication, following alternatives are provided:

- Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication

- Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication

When UE receives L1-based beam indication, corresponding PUCCH transmission for ACK/NACK should be based on previously indicated beam as the UE didn’t send the confirmation to gNB yet. In that regard, it is obvious that UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication. Therefore, it is preferred to apply newly indicate beam X ms or Y symbols after the acknowledgement of the beam indication.

Observation 2: As UE transmits PUCCH for ACK/NACK of beam indication with previously indicated beam, UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication.

Proposal 6: For beam application time, the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt 2) should be supported.

Configuration of SRS for BM as a source RS to represent a DL RX spatial filter

For the configuration of SRS as a source RS to represent a DL RS spatial filter, RAN1 should consider following aspects:

- Beam Correspondence

  - Since Rel-15, beam correspondence has been a mandatory UE capability for FR2 where its type of support, i.e., with or without network assistance is indicated through UE capability signaling. Meaning that, for FR2, beam correspondence is an essential feature that all UEs need to support for beam management operation. Based on this understanding, there’s no reason not to utilize existing channel information based on beam correspondence.

- Signaling overhead

  - For Rel-15 DL TCI state indication, a gNB needs to allocate CSI-RS resources for TCI state indication even if the gNB is already aware of channel information based on SRS transmissions. Such CSI-RS transmission induces large signaling overhead considering maximum 64 DL beams and should be minimized by utilizing existing information

- More accurate beam information

  - A UE in FR2 may experience beam change due to UE mobility and dynamic blockage and latest information can help better utilization of transmission beam. For example, if SRS transmission is a more recent RS transmission than CSI-RS, it may provide better information and corresponding performance benefits.

Observation 3: Introduction of SRS in DL TCI state provides performance benefits based on beam correspondence, less signaling overhead and more accurate beam information.

Proposal 7: DL TCI state based on SRS for beam measurement should be supported.

Parameters to be included in the joint TCI state

Rel-15 UL beam indication mechanism allows beam specific UL transmit power control mechanism based on associations between an SRS resource and a set of UL power control parameters. The beam specific UL transmit power control allows beam-specific power allocation by measuring dedicated pathloss RS and applying dedicated parameters. The benefits of beam-specific power control should be maintained in joint TCI and at least following UL power control parameters should be considered:

- Pathloss RS

- P0 and alpha set

- Closed-loop index.

Observation 4: Rel-15 UL power control mechanism allows beam-specific power controls by having a set of power control parameters associated with an SRI.

Proposal 8: At least pathloss RS, P0/alpha set and closed-loop index should be supported as a component of joint TCI.

# L1/L2-centric inter-cell mobility

In RAN1#104b-e [1], support of neighbor cell SSBs associated with the TCI state and/or QCL-info was agreed. In RAN1#104-e [2], following options were provided for the further discussion:

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - Providing non-serving cell PCI is a straightforward and flexible option for indicating neighbor cell SSBs. By associating the configured non-serving cell PCI with the associated non-serving cell information.

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - Introducing a flag can be sufficient for inter-cell multi-TRP case with minimal specification impacts.

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - Introduction of explicit/implicit grouping requires more complexity and specification impacts than option 1 and 2 without providing additional benefits.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Re-indexing the non-serving cell RS requires unnecessary specification impact to support inter-cell mobility without providing additional benefits.

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - Introduction of a new indicator is a more generalized form of Option 2, however with more complexity and specification impacts than Option 2. Option 2 should be sufficient for the support of inter-cell mobility.

Observation 5: Introduction of non-serving cell PCI or a flag provides enough flexibility with minimum specification impacts.

Proposal 9: SSB based beam measurement for inter-cell operation by supporting a physical cell ID or a flag should be supported.

Beam failure recovery

Given that beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2, enhancement of beam failure recovery should be an important part of inter-cell mobility. For the enhancement, monitoring, candidate beam selection and reporting for other cells can be considered. For example, as number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase. In that regard, enhanced operation of beam failure recovery should consider efficient operation with reasonable power consumption. For example, prioritization of beams from serving cells than other cells can be considered.

Observation 6: Beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2.

Observation 7: As number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase.

Proposal 10: Efficient beam failure recovery operation for inter-cell operation should be supported.

# MP-UE assumption to facilitate fast UL panel selection

CSI reporting for MP-UE assumption

In RAN1#103-e [1], UE-initiated UL panel selection/activation was agreed to facilitate fast uplink panel selection. For UE initiated UL panel selection/activation, CSI reporting can be a good method to indicate UE’s selection on UL panel to gNB. One possible enhancement with simple specification impact would be introducing a panel ID in a CSI-RS resource. For example, based on the panel ID, UE can report its panel selection/activation to gNB by reporting CRI and gNB can identify UE’s decision by determining associated panel ID with reported CRI.

Observation 8: CSI reporting can be a good method to indicate UE’s panel selection/activation with simple specification impact.

Proposal 11: Introduce panel ID in a CSI-RS resource for CSI reporting based on UE-initiated UL panel selection/activation.

NW-initiated panel selection/activation

In addition to UE-initiated UL panel selection/activation, NW-initiated panel selection/activation can be considered. While UE is able to identify UE-side information e.g., remaining battery power, MPE and interference from other UEs and gNB, gNB is able to identify network-side information e.g., scheduling information and traffic. In that regard, NW-initiated panel selection/activation can be beneficial as well as UE determination. For example, a panel ID can be introduced in a TCI state for joint beam indication and separate UL beam indication.

Observation 9: While UE is able to identify UE-side information for panel determination, gNB is able to identify gNB-side information for panel determination.

Proposal 12: Introduce NW-initiated panel selection/activation in addition to UE-initiated panel selection/activation.

# MPE mitigation

Support of UE reporting for MPE mitigation

In RAN1#104-e [1], reporting on P-MPR based on Rel.16 framework and SSBRI(s)/CRI(s) was agreed. In addition, down selection between beam-level and panel-select reporting should be further discussed. Panel-select reporting only reports P-MPR associated with a panel. Therefore, if gNB requires additional beam measurement information associated with the panel, additional procedures and latency are required to trigger associated RS transmissions and reports. However, if beam-level reporting is supported with association between beam and panel, beam-level reporting enables to report both P-MPR associated with a panel as well as preferred beam information in the reported panel. The association can be achieved by supporting simple specification enhancements. For example, a TCI state group ID can be configured in a TCI state or a panel ID can be configured in a CSI-RS configuration.

Observation 10: Beam-level reporting provides both panel and beam related information while panel-select reporting only provides information on a specific panel.

Proposal 13: Introduce reporting of SSBRI(s)/CRI(s) to indicate gNB beam(s) with an association with a specific panel (e.g., TCI state group ID or panel ID).

Support of TCI state group

Based on the association, more efficient beam indication methods can be considered. As discussed in the above, support of TCI state group should be considered. For example, as shown in Figure 3, two TCI state groups can be supported and each of the TCI state groups are associated with each panel. Based on the configured TCI state groups, gNB can indicate a TCI state group for UE operation. The actual TCI state to be used for UE operation can be determined based on UE reporting for MPE mitigation. For example, a TCI state, associated with reported SSBRI(s)/CRI(s), in the indicated TCI state group can be used.

igure 3 Example of two TCI state group configuration

Proposal 14: Introduce a TCI state group based indication and UE-centric determination of a TCI state in the indicated TCI state group

NW confirmation in response to the reported MPE event

When UE-centric determination methods are considered, major concern on the support is a mismatch of beam information between UE and gNB. For example, when the gNB couldn’t decode UE’s reporting on P-MPR, a different spatial filter may be applied and severely reduce uplink performance due to a loss of antenna gain.  In that regard, a gNB confirmation can be considered to align the understanding. For example, the gNB may transmit a PDCCH in a dedicated CORESET as a confirmation of the UE reporting on P-MPR.

Observation 11: Possible mismatch of beam information between UE and gNB may severely degrade uplink performance.

Proposal 15: Consider gNB confirmation mechanism to align beam information between UE and gNB.

# Conclusions

This contribution discussed potential enhancement for Rel-17 beam management. Based on the presented discussion, we make the following observations and proposals,

Observation 1: For separate UL beam indication, utilization of existing UL DCI formats 0_1 and 0_2 is more efficient as it doesn’t require multiple PDCCH decoding and combined UE processing time is possible if UE implementation allows simultaneous processing.

Observation 2: As UE transmits PUCCH for ACK/NACK of beam indication with previously indicated beam, UE’s beam changing procedure initiates after sending the PUCCH not after decoding PDCCH which provides L1-based beam indication.

Observation 3: Introduction of SRS in DL TCI state provides performance benefits based on beam correspondence, less signaling overhead and more accurate beam information.

Observation 4: Rel-15 UL power control mechanism allows beam-specific power controls by having a set of power control parameters associated with an SRI.

Observation 5: Introduction of non-serving cell PCI or a flag provides enough flexibility with minimum specification impacts.

Observation 6: Beam failure recovery is an essential procedure which provides reliability and enables standalone operation of FR2.

Observation 7: As number of cells to be monitored, selected and reported increases, UE implementation complexity and power consumption also increase.

Observation 8: CSI reporting can be a good method to indicate UE’s panel selection/activation with simple specification impact.

Observation 9: While UE is able to identify UE-side information for panel determination, gNB is able to identify gNB-side information for panel determination.

Observation 10: Beam-level reporting provides both panel and beam related information while panel-select reporting only provides information on a specific panel.

Observation 11: Possible mismatch of beam information between UE and gNB may severely degrade uplink performance.

Proposal 1: For group 1, support beam measurement/reporting/refinement/selection triggered by beam indication (opt 1-1A) for advanced beam refinement/tracking.

Proposal 2: For group 1, UE-initiated beam selection/activation based on beam measurement can be considered if gNB beam group indication or activation is supported.

Proposal 3: For group 2, it is preferred to discuss latency reduction of MAC CE related activation in RAN4.

Proposal 4: For group 2, replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update (Opt 2-3) is supported.

Proposal 5: For separate UL beam indication, utilization of the existing UL DCI formats 0_1 and 0_2 is preferred.

Proposal 6: For beam application time, the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt 2) should be supported.

Proposal 7: DL TCI state based on SRS for beam measurement should be supported.

Proposal 8: At least pathloss RS, P0/alpha set and closed-loop index should be supported as a component of joint TCI.

Proposal 9: SSB based beam measurement for inter-cell operation by supporting a physical cell ID or a flag should be supported.

Proposal 10: Efficient beam failure recovery operation for inter-cell operation should be supported.

Proposal 11: Introduce panel ID in a CSI-RS resource for CSI reporting based on UE-initiated UL panel selection/activation.

Proposal 12: Introduce NW-initiated panel selection/activation in addition to UE-initiated panel selection/activation.

Proposal 13: Introduce reporting of SSBRI(s)/CRI(s) to indicate gNB beam(s) with an association with a specific panel (e.g., TCI state group ID or panel ID).

Proposal 14: Introduce a TCI state group based indication and UE-centric determination of a TCI state in the indicated TCI state group

Proposal 15: Consider gNB confirmation mechanism to align beam information between UE and gNB.

# References

- “Final Report of 3GPP TSG RAN WG1 #104b-e”, 3GPP RAN1 Meeting #104b-e, e-Meeting, April 12th – 20th, 2021

- “Final Report of 3GPP TSG RAN WG1 #104-e”, 3GPP RAN1 Meeting #103-e, e-Meeting, January 25th – February 5th, 2021.