3GPP TSG-RAN WG1 Meeting #105-e	R1-2104303

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.2

Source:	Moderator (Thales)

Title:	FL Summary on enhancements on UL time and frequency synchronization for NR NTN

Document for:	Discussion

# Introduction

This feature lead summary document captures the issues related to UL time and frequency synchronization in NR NTN. It contains a summary of the contributions under 8.4.2 at TSG-RAN WG1 #105-e. together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is also to provide recommendation on prioritization of discussion and whether any issues should be postponed.

Please note the following checkpoints for agreements: on May 25, May 27

# Content

Introduction	1

Content	1

1	Issue#1: Feeder link timing drift handling	3

1.1	Company views	5

1.2	Updated proposal based on company views (First round of email discussions)	6

1.3	Updated proposal based on company views (2nd round of email discussions)	6

2	Issue#2: Indication of Common TA drift parameters	6

2.1	Company views	9

2.2	Updated proposal based on company views (First round of email discussions)	10

2.3	Updated proposal based on company views (2nd round of email discussions)	10

3	Issue#3: Granularity and signalling of Common TA	10

3.1	Company views	12

3.2	Updated proposal based on company views (First round of email discussions)	13

3.3	Updated proposal based on company views (2nd round of email discussions)	13

4	Issue#4: The need and the indication of TA margin	13

4.1	Company views	14

4.2	Updated proposal based on company views (First round of email discussions)	14

4.3	Updated proposal based on company views (2nd round of email discussions)	14

5	Issue#5: TA command in RAR	14

5.1	Company views	15

5.2	Updated proposal based on company views (First round of email discussions)	15

5.3	Updated proposal based on company views (2nd round of email discussions)	15

6	Issue#6 : TA update in RRC_CONNECTED state	15

6.1	Company views	19

6.2	Updated proposal based on company views (First round of email discussions)	22

6.3	Updated proposal based on company views (2nd round of email discussions)	22

7	Issue#7 : NTN UE Time Alignment Timers	22

7.1	Company views	23

7.2	Updated proposal based on company views (First round of email discussions)	24

7.3	Updated proposal based on company views (2nd round of email discussions)	24

8	Issue#8: TA reporting	24

8.1	Company views	25

8.2	Updated proposal based on company views (First round of email discussions)	25

8.3	Updated proposal based on company views (2nd round of email discussions)	25

9	Issue#9: Broadcasting the position of a reference point	25

9.1	Company views	26

9.2	Updated proposal based on company views (First round of email discussions)	26

9.3	Updated proposal based on company views (2nd round of email discussions)	26

10	Issue#10 : Common Doppler shift pre/post compensation on the feeder link	27

10.1	Company views	28

10.2	Updated proposal based on company views (First round of email discussions)	28

10.3	Updated proposal based on company views (2nd round of email discussions)	28

11	Issue#11: Indication of common frequency pre-compensation offset on DL service link	28

[TR38.821] :	29

6.3.2	DL synchronization	29

11.1	Companies views	31

11.2	Updated proposal based on company views (First round of email discussions)	32

11.3	Updated proposal based on company views (2nd round of email discussions)	32

12	Issue#12: Indication of common compensation frequency offset for Uplink	32

12.1	Companies views	32

12.2	Updated proposal based on company views (First round of email discussions)	33

12.3	Updated proposal based on company views (2nd round of email discussions)	33

13	Issue#13: Close control loop for UL frequency alignment	33

[TR38.821] :	33

13.1	Companies views	34

13.2	Updated proposal based on company views (First round of email discussions)	35

13.3	Updated proposal based on company views (2nd round of email discussions)	35

14	Issue#14: Serving satellite ephemeris format	35

14.1	Company views	39

14.2	Updated proposal based on company views (First round of email discussions)	40

14.3	Updated proposal based on company views (2nd round of email discussions)	40

15	Issue#15: Time and Frequency synchronization requirements	40

15.1	Updated proposal based on company views (First round of email discussions)	42

15.2	Updated proposal based on company views (2nd round of email discussions)	42

16	Conclusion	42

17	References	42

18	Appendix I: RAN1 agreements on UL time and frequency synchronization for NR NTN	43

19	Appendix II: Summary of proposals	46

# Issue#1: Feeder link timing drift handling

The Timing Advance to be applied by an NR NTN UE in the different RRC sates was discussed in RAN1 Meeting #104-bis-e and the following formula was agreed:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. NTA,common with value of 0 is supported. details of signalling including granularity is still FFS.

The indication of common TA has been agreed in RAN1#104b-e, while the common TA drift rate is still under discussion. A major issue that is still open and needs to be addressed before to solve/remove this FFS is the handling of the feeder link timing drift.

w.r.t Feeder link time compensation at UE, there are two aimed objectives that could be considered:

| Objective (a) | - Feeder link timing compensation at UE to reduce the amount of buffering at gateway or gNB due to DL and UL misalignment - Coarse granularity of Common TA is sufficient |
| --- | --- |
| Objective (b) | - Feeder link “real-time” timing compensation at UE to ensure aligned DL and UL symbol level processing (FFT and IFFT window) at RU/gNB - Finer granularity of Common TA is required - But gNB can compensate in a semi-static way part of feeder link RTD to mitigate/reduce the large timing offset between the DL and UL frame timing at the UE side (as per objective (d)) |

w.r.t Feeder link time compensation at Network, there are also two aimed objectives that could considered:

| Objective (c) | - Feeder link timing drift is compensated by the Network - Network compensates time-varying RTD on feeder link - But UE can compensate in a semi-static way part of feeder link RTD to ease network implementation (as per objective (a)) |
| --- | --- |
| Objective (d) | - The network compensates part of feeder link propagation delay to mitigate/reduce the large timing offset between the DL and UL frame timing at the UE side when reference point (RP) for uplink time synchronization is at the gNB - Compensated timing drift at the gNB is constant within a period of time instead of time-variant feeder link RTD to resolve the implementation complexities at the gNB |

Therefore, there are two potential solutions for Issue#1:

Solution #1: Feeder link timing drift is compensated by UE using indicated/broadcast Common TA parameters.

With Solution#1 UE compensates the timing drift on the feeder link with GOOD accuracy to ensure aligned DL and UL symbol level processing (FFT and IFFT window) at Radio Unit(RU)/gNB → Objective (b).

The network may compensate part of feeder link propagation delay → Objective (d).

The handling of Feeder link TA drift at UE side is shown is Figure 1 below:

Figure 1 Solution#1: Feeder link TA drift handled by UE

Solution #2: Feeder link timing drift is compensated by the Network in a way transparent to UE.

With Solution#2 Network compensates the timing drift on the feeder link with GOOD accuracy to ensure aligned DL and UL symbol level processing (FFT and IFFT window) at Radio Unit(RU)/gNB → Objective (c).

The UE may compensate part of feeder link propagation delay → Objective (a).

The handling of Feeder link TA drift at Network side is shown is Figure 2 below:

Figure 2 Solution#2: Feeder link TA drift handled by Network

Proposals related to this issue are summarized in the following table:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: The delay compensated by the gNB should consider the gNB implementation complexity. Proposal 2: The common TA is determined as the RTD from the reference point to the satellite, i.e. by subtracting the delay compensated at the gNB from the feeder link RTD. Proposal 3: NTA,common can be a positive or negative value to support the corresponding reference point at both service link and feeder link. |
| PANASONIC | Proposal 2: The common TA value NTA,common is always indicated in SIB. - FFS: the value range of NTA,common Proposal 3: We propose to allow for a non-zero DL-UL timing difference managed by the network. - FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation. |
| Qualcomm Incorporated | Observation 1: Compensation of feeder link timing drift and Doppler by UE can impose significant implementation complexity and its feasibility is highly questionable. It is more efficient to have network compensate feeder link timing drift and Doppler in a way transparent to UE. |
| OPPO | Proposal 2: Further study the acceptable SIB reading frequency for a UE. Whether reading SIB information every 3 seconds is acceptable when only NTA,common,drift,rate is broadcasted. |
| Ericsson | Proposal 4 The reference point for time and frequency in an NTN should be under control of the network and should at least support the option of having gNB as the reference point. |
| ZTE | Observation 1: The time-variant behavior of TA is preferred to be handled by UE instead of gNB or gateway. |
| Xiaomi | Proposal 2: When the common TA is not broadcasted by network, the UE should assume that NTA,common=0 . |

## Company views

On Issue#1, it seems that the common view shared by the majority of companies is that the UE shall compensate feeder link timing drift using Common TA parameters indicated by the Network.

But, in its contribution to RAN1#104-bis-e and RAN1#105-e [Qualcomm] rightly observed that compensation of feeder link timing drift by UE can impose significant implementation complexity and its feasibility is highly questionable. It is more efficient to have network compensate feeder link timing drift in a way transparent to UE. The reasons given by [Qualcomm] are the followings:

- Having a UE to compensate feeder link timing drift doubles the maximal TA adjustment rate (both service link and the feeder link) at the UE and imposes additional complexity of UE implementation.

- Impacts of UE compensation of feeder link timing drift on receiver performance should also be examined:

[[8]R1-2104668]: Existing UL timing requirements depending on subcarrier spacing (SCS) are less than 20% of CP duration. To ensure satisfactory performance, timing error requirement needs to consider EVM window, typically 50% of CP duration, and channel delay spread. For NTN, some relaxation on UL timing error requirement can be considered thanks to the smaller channel delay spread in NTN. Several timing error sources in NTN and associated maximal error are listed below: - Feeder link RTD with 30s update periodicity and signaling of second-order derivative: about 1 s [R1-2102752- MTK][ R1-2102360- Thales] - Ephemeris with 60s update periodicity: about 0.12 s [R1-2100595- MTK] - UE GNSS error at 100m: 0. 3s (100m is the legacy requirement on GNSS accuracy) - Satellite GNSS error at x meter: about 4x/3e8 second - DL synchronization error and clock drift/jitter Note that all the above errors can be additive and the aggregated error can be the sum of the above. It can be seen that even with a 30s update periodicity, feeder link timing error dominates among the first three error terms and is alone far larger than existing total UL timing error requirement with 15 kHz SCS (0.39 s). It is also clear from [R1-2102752- MTK] and [R1-2102360- Thales] that an update periodicity of less than 5 seconds is needed even assuming feeder link timing error is the only UL timing error. From the above, compensation of feeder link timing drift by UE for objective B can have significant impact on receiver performance and its feasibility is highly questionable.

On the other hand, [Ericsson] observed that using satellite as reference for time and frequency requirements severely affects compatibility with existing Rel-16 gNB.

And according to [ZTE] in LEO cases, the variation of propagation timing for feeder link will lead to time-variant common TA and the common TA drift rate can be up to 45.4 us/s for GW at 10 degree elevation angle. In this case, if such kind of variation over time is handled by gNB or gateway, additional complexity and degradation on the performance is expected. [ZTE] observed that the time-variant behaviour of TA is preferred to be handled by UE instead of gNB or gateway.

[Huawei] proposed that the delay compensated by the gNB should consider the gNB implementation complexity.

From [Moderator’s perspective] in view of the current situation, we have to rethink Feeder link time compensation design. The group needs to discuss the feasibility of both solution#1 and #2 and make a reasonable technical trade-off between both solutions to move forward. We can continue to investigate the viability of solution#1 by considering the different drawbacks raised by [Qualcomm] but solution#2 should be also studied and challenged as a potential way forward. In moderator’s view whatever the adopted solution is, the existing Rel-16 gNB and Radio unit (RU) need to be enhanced to support NTN and both solution#1 and #2 will affect compatibility with existing Rel-16 gNB.

In view of  the above discussion, the following Initial Proposal is made:

Initial Proposal 1:

RAN1 to discuss the feasibility of each of the following two options:

- Option 1: Feeder link timing drift is compensated by UE using Common TA parameters

- Option 2: Feeder link timing drift is compensated by the Network in a way transparent to UE

- Companies are encouraged to provide inputs within the following table – please elaborate:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Option 1 applies to Reference point for DL-UL subframe timing alignment at the gNB. This requires at least 2nd order derivation for common TA for viability of solution, and preferable 3rd order derivatives for the common TA over the feeder link. Otherwise the device will need to read the common TA on SIB every time it is broadcast – e.g. every second. Option 2 applies to Reference point for DL-UL subframe timing alignment at the satellite. Impact on gNB implementation could be further discussed. |
| Intel | In our view both options are feasible to implement. The question is which one has smaller impact on the UE/gNB. In our understanding timing pre-compensation for UL was never considered at the gNB side, while it was always supported at the UE side. What UE needs to do is just receive the broadcast information with common TA configuration and continuously update the common TA value. So, in our view Option 1 has minor impact on the UE. Regarding the argument of the accuracy for TA at the UE, in our understanding UE should be able to set accurate TA and it does not matter if there is one contributor to the TA or it is a sum of several values. Since Option 2 has almost 0 impact on the spec, we support both option 1 and option 2. |
| OPPO | Option 2 does not have spec impact. Thus, if the spec needs to support option 1, we suggest that we focus on the concrete design of option 1 rather than spending more time on feasibility of option 2. |
| Samsung | With the agreement to introduce N_{TA, common}, Option 2 is now supported. For Option 1, there may be some error between the exact feeder link delay and the common TA value that a UE applies based on the gNB indication. That error can be managed by the gNB. |
| Xiaomi | We prefer option 2. The signaling overhead is large, if option 2 is used. Meanwhile tracking time drift of feeder link in the UE side is difficult, especially in the LEO-600 scenario. |
| LG | We slightly prefer to support option 2. Instead, it may be desirable for network to provide the common TA in series as time changes, so that the UE can know the accurate common TA value. Therefore, modified Option 2 as follows can be also considered: • Modified Option 2: Feeder link timing drift is compensated by the Network in a way transparent to UE, and network can provide series of Common TA to maintain its accuracy. |
| CMCC | Option 2 has almost 0 impact on the spec. If it is feasible, we can support it. Nevertheless, the major vendors (e.g., Huawei, ZTE, Ericsson) seems to have concern on the feasibility of Option 2, which may significantly increase the gNB implementation complexity. If only few vendors can support one solution, we need to seek for another reliable solution which can be implemented by most vendors. Thus, according to the current progress, at least Option 1 should be supported. As a compromise, Options 1 and Options 2 can be both supported, it would be at the discretion of the network operator. |
| Nokia, Nokia Shanghai Bell | We strongly support Option 1, where the UE is able to compensate the feeder link timing drift using ephemeris data and Common TA parameters. The Reference Point location can be easily indicated to the UE by the Common TA value. Option 2 would create significant complexity to the gNB or the satellite subsystem, as these would need to deal with a time-varying feeder link delay and compensate this to avoid a misalignment of DL and UL signals; this would require to significantly change legacy gNB implementations. |
| ZTE | We should stick to support on the Option-1 to enable the deployment that taking the RP at BS side. It was aligned with previous compromise and discussion. Taking the tracking of time drift by gNB will significant complicate the implementation of gNB since the variation is much larger and more server than legacy TN system. According to our contribution, it’s clear that with proper parameters design, the sufficient accuracy can be achieved with indication of parameters. And this mechanisms is also aligned with previous UE’s behavior for TA adjustment. |
| Panasonic | We prefer Option 2 with the understanding that also in this case a common TA is employed. However, misaligned UL/DL frame timing is permissible and managed by the network. On the other hand, Option 1 implies the need for tight and accurate synchronization in order to achieve rigid UL/DL frame timing at the reference point. To achieve sufficient accuracy, it requires at least a first order approximation of the feeder link delay, which increases the overhead and forces the UE to read the SIB frequently. Another aspect is feeder-link switching which becomes increasingly complex for the system the more parameters the UE needs to handle. |
| Apple | We tend to agree with Option 2. With common TA broadcast, the major gNB buffer issue is addressed. However, solving all timing errors for UL/DL synchronization at gNB seems not easily achievable. Hence, we think the feeder link timing drift can be compensated by network. This also has the less specification impact and less signaling overhead. |
| Fraunhofer IIS, Fraunhofer HHI | We are supportive of option 1. In our contribution, we showed that sufficient accuracy (less than 10% of CP duration) can be achieved by providing assistance information to the UE for calculation of the feeder link timing drift. |
| Ericsson | We support option 1. We disagree with the claim regarding Option 1 that “its feasibility is highly questionable”. No evidence for this has been presented. On the contrary, it has e.g. been shown by MediaTek in R1-2105952 that using a linear drift rate, 2nd order and possibly 3rd order terms, it is feasible to e.g. predict the feeder link delay with an error of 0.03µs more than 10 s (2nd order approximation) or 27 s (3rd order approximation) ahead. Therefore, the NTN network can ensure that the contribution of the feeder link RTT to the total Rx timing offset at the UL receiver is insignificant. It is unclear if option 2 corresponds to Objective A described above (slot-level time alignment at gNB receiver) or to a solution where the entire feeder link RTT would be compensated by the network (i.e., reference point at the satellite). But regardless of this, we strongly oppose option 2 since it would impose a large and time-varying time misalignment between UL and DL that will have significant impact to gNB implementation. A large time misalignment in the order of several slots implies that large buffering will be needed in the gNB and complicates scheduling. Even if this misalignment is roughly pre-compensated with a common TA of slot granularity (Objective A), a residual time misalignment that is time-varying and up to one slot length would be left. This will have negative impact on the complexity and performance of the gNB receiver. Further, a coarse granularity of Common TA (Objective A above) will create disruptive changes to the TA that will have significant negative impact to the system. Each time the common TA is increased by a slot length, the transmission time of two UL slots n and n+1 will coincide. To avoid this collision, it is necessary to create gaps in the UL scheduling for all UE each time the common TA is increased. When the common TA is decreased by one slot length, a gap in the physical UL transmission will occur that cannot be used for anything. So, the coarse common TA updates waste UL resources and complicates scheduling. This is illustrated in the figure below. Assuming a feeder link RTT drift rate of up to 50 µs/s, there will be a common TA update up to every 20 seconds for SCS=15 kHz and every 1.25 seconds for SCS=240 kHz. (On a side note, this also implies that signaling a drift rate is necessary even with Objective A, to avoid SIB reading up to every 1.25 seconds.) Critical UL resources such as PRACH slots may be lost (for all UE) due to the skipped UL slots. Also, the number of transmitted slots during slot aggregation can be less than intended. To avoid these scheduling gaps and wasted UL resources, the applied TA should always be incremented/decremented in small steps (in the order of a few Ts time units, as for legacy UE), regardless of the granularity of the common TA signaling. As a consequence, TA drift rate will be the same even if the granularity of the common TA signaling is coarse. Therefore, the claimed UE implementation complexity due to the large TA drift might be similar for both options. |
| Sony | We should focus on option 1 which reference point is not at satellite case. This option needs to consider how UE updates the common TA accurately and has spec impact. For option 2, we think there is almost no spec impact because of gNB implementation issue. |
| Lenovo/MM | We think option 1 should at least be supported considering the buffer and tracking complexity at gNB side. Regarding option 2, if gNB implementation support it, it can be realized by a specific common TA value indication and there is no further spec impact. |
| Huawei, HiSilicon | We support Option 1. We share the similar concerns with ZTE, Nokia and Ericsson that Option 2 would impose non-trivial complexities to both gNB receiver as well as scheduling. We are not sure why a UE cannot handle the timing drift given a timing drift rate or drift variation is provided to the UE. It should be noted that a UE will anyway need to handle the timing drift for service link part which is not known by the gNB. The underlying assumption for the current TA compensation equation is that both Option 1 and Option 2 are supported. This is already a comprise and there is no need to go back and have the discussion again. |
| NTT DOCOMO, INC. | We support option 1, whose feasibility has been proved by MediaTek and Ericsson above. Option 2 has no spec impacts but will cause great complexity to gNB. Because TA compensation is not available for TN gNB and unaligned UL and DL transmission will cause great impacts on the system. But in order to move forward, we are also fine that both options are supported with option 1 being mandatory and option 2 being optional. |
| Spreadtrum | Both Option 1 and Option 2 can be supported. |

## Updated proposal based on company views (First round of email discussions)

## Updated proposal based on company views (2nd round of email discussions)

# Issue#2: Indication of Common TA drift parameters

Issue#2 is highly dependent on Issue#1. Under Issue#2 we can continue investigating the viability of the compensation of feeder link timing drift by UE.

In case of LEO/MEO based non-terrestrial access network, if feeder link timing drift is to be compensated by the UE, such compensation shall be performed with a very good accuracy to ensure aligned DL and UL symbol level processing (FFT and IFFT window) at RU/gNB. Thus finer granularity of Common TA will be required and indicating parameters to assist the UE deriving the actual Common TA will be needed.

Having the feeder link timing drift compensated by the UE will introduce another source of timing errors that will impact the total tolerable UL timing error.

Thereby, to analyse the impact of feeder link time drift compensation at the UE. there are mainly two questions to be addressed carefully:

- What is the acceptable TA error due to UE handling of feeder link TA drift?

- If the feeder link timing drift is to be compensated by the UE. What are the Common TA parameters to be indicated by the network to have this solution workable?

The following table recaps the proposals submitted to RAN1#105-e regarding Issue#2:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: At least support indicating a common TA drift rate as part of the common TA applied by UE. Proposal 6: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission. |
| THALES | Proposal 2: The Network may periodically broadcast: - in case of GEO based non-terrestrial access network: - Common TA in a field of 23 bits - in case of LEO/MEO based non-terrestrial access network: - Common TA in a field of 21 bits - Common TA drift rate in a field of 13 bits - Common TA drift rate variation in a field of 6 bits |
| CATT | Proposal 3: In order to save signaling overhead, common TA and common timing drift will be equal to 0 if not indicated. |
| MediaTek Inc. | Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field - Common delay drift rate variation NTA,common,drift,thrird_order,derivative in a 6 bit field |
| CMCC | Proposal 1: The Network may periodically broadcast: - Common TA - Common TA drift rate - Common TA draft rate with value of 0 is supported Proposal 3: Update relative values of Common TA to reduce the signaling overhead. Proposal 4: The common TA is determined as NTA,commont=NTA,common, t0+TAcommondrift, t0∙(t-t0+δt0) where: t is the time for targeted UL transmission at UE’s clock. t0 is the time for receiving common TA signaling at UE’s clock. NTA,common, t0 is the common TA (in 16∙642μ∙Tc units) at t0. TAcommondrift, t0 is the common TA drift rate. δt0 is used to pre-compensate the ageing of NTA,common, t0values due to the long propagation delay in NTN. and, NTA,common, t0=A+X where: A represents the synthetical impact of time-invariant parameters, e.g., TA margin and the minimum RTT on the feeder link. X are time-variant parameters, and they need to be updated with time. Proposal 5: Indication of parameter A (a time-invariant offset of NTA,common, t0) should be supported. - For DL and UL aligned at satellite, A can be used to absorb TA margin. - For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link. Proposal 6: Indication of parameter X at least should be supported. - For DL and UL not aligned at satellite (including aligned at gNB), X should be indicated and updated to capture the rapidly changed RTT on the feeder link. |
| OPPO | Proposal 2: Further study the acceptable SIB reading frequency for a UE. Whether reading SIB information every 3 seconds is acceptable when only NTA,common,drift,rate is broadcasted. |
| Ericsson | Proposal 8 The network broadcasts parameters describing the (one-way) common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay that may be implicitly (derived from DL subframe) or explicitly signaled Dcommon, base is the common one-way delay at time Tcommon,base Dcommon, drift is the common one-way delay drift rate Proposal 10 The characterization of the common delay should include drift rate information. |
| Nokia | Proposal 6: The Common TA value is provided along with ephemeris information. Proposal 7: It should be left up to the gNB to decide whether and how often to broadcast the Common TA drift rate Proposal 8: It is not needed to provide the Common TA drift rate variation. |
| Apple | Proposal 1: The broadcast of common TA drift rate is based on network implementation. If common TA drift rate is not indicated, then its default value is 0. FFS common TA drift variation rate. Proposal 3: The network-controlled common TA NTA, common is obtained by NTA, common=NTA,common, RX +DTA, common,RX ⋅Δt, where NTA,common, RX is the latest received common TA, DTA,common, RX is the latest received common TA drift rate (if indicated) and Δt is the time gap between the latest common TA transmission and the corresponding uplink transmission. |
| Sony | Proposal 3: RAN1 should support the signalling of timing drift rate information to the UEs in a beam specific manner. Proposal 4: UE should update the common TA with common timing drift rate when UE transmits uplink data. |
| ZTE | Proposal 1: At least the first order common TA drift rates should be supported along with common TA. Proposal 2: Following solutions can be considered to further optimize the overhead of signaling: - Indication of second order common TA drift rate can also be considered to further optimize signaling overhead. - Indication of multiple set of {common TA, 1st order TA drift rate } with different applicable timing, - FFS: whether to include the 2nd order TA drift rate |
| Lenovo, Motorola Mobility | Proposal 4: Support indication of common timing offset drift rate and indication of drift rate variation. |
| Samsung | Proposal 10: The gNB signals common TA drift rate to enable autonomous TA update at UE. |
| Xiaomi | Proposal 3: The common timing drift rate indicated by network should be supported. |
| InterDigital, Inc. | Proposal-1: support to introduce an additional parameter to indicate feeder link RTT separately from the common TA. |
| NTT DOCOMO, INC. | Proposal 1: Support NW broadcast of common TA drift rate. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 7 Support common TA drift rate broadcasted in system information to reduce signaling overhead. |

## Company views

- On the parameters to be indicated to the UE for the derivation of Common TA, different views were expressed within the contributions submitted to RAN1#105-e: Companies are encouraged to read each other’s proposals captured in the table above.

- For better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be jointly broadcast by the Network.

- [MediaTek] observed that workable solution to pre-compensate the common TA drift over the feeder link is needed to support option of reference point for DL-UL subframe timing at gNB workable.

[MediaTek] proposed to broadcast of NTA,common,drift,rate , NTA,common,drift,rate,variation and  NTA,common,third,order,derivative. Indicating these parameters can reduce the maximum delay error over the feeder

According to [ZTE] when both first and second order drift rates are indicated, the performance evaluations of autonomous common TA estimation by UE for LEO-600 are improved. In this case, the required periodicity for indication will be 6 s and 4 s for the case with tolerance range as CP/2 and Te, respectively.

[OPPO] proposed to further study the acceptable SIB reading frequency for a UE. Whether reading SIB information every 3 seconds is acceptable when only  NTA,common,drift,rate is broadcasted.

According to [Thales] maximum time during which the UE can track the Common RTD without having acquired new Common TA parameters is equal to 9.8 seconds when second-order approximation of Common TA is used and the maximum error on the Common TA estimated by the gNB is equal to 0.2 µs (corresponding to a max error of 30m on satellite position).

Some inputs regarding the maximum feeder link delay/RTD error have been provided in some contributions:

Note within the following results neither the maximum error on the Common RTD estimated by the gNB (before being broadcast) nor the error of  quantization of assistance information were taken into account:

| MediaTek [R1-2105952] | The maximum feeder link delay error @ 5,5 seconds 0.009 us if 2nd order derivative 0.007 us if 3rd order derivative | The maximum feeder link delay error @ 27.3 seconds 0.49 us if 2nd order derivative 0.03 us if 3rd order derivative |
| --- | --- | --- |
| Thales [R1-2104301] | The maximum feeder link Round-Trip delay error @ 5 seconds 0.15 us if 2nd order derivative and no quantization error | The maximum feeder link Round-Trip delay error @ 28 seconds 24.2 us if 2nd order derivative |
| ZTE [R1-2105190] | The maximum feeder link Round-Trip delay error @ 6 seconds 0.23 us if 2nd order derivative | The maximum feeder link Round-Trip delay error @ 28 seconds ? us if 2nd order derivative |

Base on the above discussion the initial proposal 2 is made as follows:

Initial Proposal 2:

If the feeder link timing drift is to be compensated by the UE. What are the Common TA parameters to be indicated by the network to have this solution workable?

    - Common delay

    - Common delay drift rate

    - Common delay drift second order derivative

    - Common delay drift third order derivative

- Companies are encouraged to provide inputs within the following table – please elaborate:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support Common delay drift rate, second and third order derivatives Same comment as in Issue#1 Option 1. This requires at least 2nd order derivation for common TA for viability of solution, and preferable 3rd order derivatives for the common TA over the feeder link. Otherwise the device will need to read the common TA on SIB every time it is broadcast – e.g. every second. On difference of performance between MediaTek, Thales, and ZT, this depends on the algorithm / method to process the Common TA parameters and granularity for the number of bits for each common TA parameter field. |
| Intel | From our perspective third order derivative is redundant. Reasonable common TA update periodicity can be supported with second order derivative of the common TA. |
| OPPO | It seems that the simulation result gap is quite large from different companies. From our simulation results, it shows that when common TA drift variation is not provided, the UE needs to update the common TA drift every 3 seconds. While if the common delay drift second order derivative is provided, the UE can update the common TA drift and second order derivative every 30 seconds. RAN1 needs to conclude the periodicity of the common TA parameter updating corresponding to different options. The decision should be made based on the signaling overhead and minimum acceptable updating periodicity. |
| Samsung | From our tdoc, we see the common TA and its drift rate are enough. So, we support - Common delay - Common delay drift rate |
| Xiaomi | Support common delay and common delay drift rate. From our perspective second order and third derivatives are redundant. |
| LG | As mentioned in Issue #1, we prefer to support providing series of common TA as time changes. |
| CMCC | In our view, the assumption that UE frequently read the common TA parameters on broadcasted SIB in connected mode may be not valid. According to TS 38.214 Clause 5.1, the UE is not expected to read SIB during a process of autonomous SI acquisition when it is overlapped with PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI. On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.. Regarding common TA is timing varying parameters, the field may be excluded when determining changes in system information, i.e. changes of common TA parameters should neither result in system information change notifications nor in a modification of valueTag in SIB1, just like timeInfoUTC field in SIB9. Hence, common TA parameter update may be carried during a process of autonomous SI acquisition, and UE is not required to read it in time. Therefore, in our view, study on common TA parameters may be focused on UE in initial access. For UE in connected mode, update of common delay drift rate ([second order derivative, third order derivative], etc.) may be involved in enhanced MAC CE TA command. |
| Nokia, Nokia Shanghai Bell | We see that providing the Common delay drift rate as an additional parameter to the Common delay can be in many cases beneficial for the UE. However, it should be left up to the network to decide whether and how often to provide the Common delay drift rate. The Common delay cannot be provided too infrequent, as this would increase the access delay time for UEs, but the Common delay drift rate can be provided less frequent or even not provided, if the network decides so. Further, we are a bit doubtful as to the usefulness of the higher order derivatives of the delay drift. |
| ZTE | The common TA and first order of drift should be indicated at least. And the finer adjustment as second order is beneficial with good performance on prediction. The signaling overhead can also be reduced. For the 3rd , the corresponding functionality can be replaced by introduce multiple sets of combination of common TA, 1st and 2nd order drift rate. |
| Panasonic | Similar to our comment to Issue#1 favoring Option 2, we prefer the Common Delay only. |
| Apple | We do not seem the necessity of introducing third order derivative. We are open on the second order derivative, together with common TA drift rate. |
| Qualcomm | For the sake of discussion, we summarize the timing error source and numbers below. Companies are welcome to comment. - Satellite GNSS error up to 30 m for service link: 30x2/3e8=0.2 us, assuming 0 elevation angle. For 30 degree elevation angle, the number will be slightly smaller - Satellite GNSS error up to 30 m for feeder link: 0.2 us - UE GNSS error 100m (current requirement):0.6 us for service link - Satellite ephemeris prediction error: 0.1x2=0.2 us for service link (less than 60 s update interval) - Feeder link prediction error (network predicts future satellite location, based on which, calculate drift rate and high-order derivitives): ~0.1 us assuming more frequent update than ephemeris - Feederlink approximation error: ~0.1 us assuming frequently updated and higher order derivative - UE downlink synchronization error and clock jitter/drift: subcarrier spacing and implementation dependent. - UE movement: 0 assume we tighten the GNSS requirement so that UE movement is absorbed in GNSS accuracy requirement Items 2,5, and 6 are common to all UEs and don’t have to be considered if feeder link timing drift is handled by the network. The sum of the three items alone can be 0.4 us, comparable to the CP duration of 120 kHz SCS. The rest is already more than 1 us and very challenging even for 15 kHz SCS. For Ka band, it’s clear that network side has to perform UL timing adjustment relative to DL timing. For S band, the sum of items 1, 3, and 4 is already 1.0 us. Clearly, some tightenting on these numbers and/or item 7 are needed. If feeder link timing drift is to be handled by UE, to the least, numbers for other items must be further squeezed/tightened. Do we want to support two different implementations for the network side? |
| Fraunhofer IIS, Fraunhofer HHI | If the feeder link timing drift is compensated by UE, common delay and delay drift rate can be provided to UE. However, it is clear that gNB should calculate the drift rate based on actual RTT measurement of the feeder link. Having this in mind, instead of sending the drift rate explicitly to the UE, instead two instances of the feeder link delay can be provided to the UE and UE autonomously calculate the drift rate. We have shown in our contribution that the same performance as the one where drift rate provided to the UE can be achieved. |
| Ericsson | Common delay, common delay drift rate and 2nd order term are needed. The need for 3rd order derivative could be further studied. |
| Sony | We share Samsung’s view. We support - Common delay - Common delay drift rate |
| Lenovo/MM | We support common delay, common delay drift rate and Common delay drift second order derivative to be indicated. |
| Huawei, HiSilicon | At least common delay and common delay drift rate are required. In addition, more than one TA drift rate can be provided to extend the validity period of common TA. |
| NTT DOCOMO, INC. | We think that common delay drift rate can be broadcasted to avoid frequency indication of common TA. Regarding to the common delay drift second/third-order derivative, further study is necessary to get good tradeoff between the signaling overhead and periodicity. Besides, for UEs with different capabilities on TA accuracy, multiple common TAs can be broadcasted. |

## Updated proposal based on company views (First round of email discussions)

Updated Proposal 2:

The Network may periodically broadcast:

  - Common delay

  - Common delay drift rate

  - Common delay drift second order derivative

  - FFS: Common delay drift third order derivative

## Updated proposal based on company views (2nd round of email discussions)

# Issue#3: Granularity and signalling of Common TA

Issue#3 is also dependent on Issue#1. Having the timing drift on the feeder link compensated by the UE will require from the Network to indicate  Common TA parameters with finer granularity e.g same as the granularity of NTA. While in case of  timing drift on the feeder link compensated by the Network a coarse granularity of Common TA is sufficient e.g granularity of a slot duration or Common TA equal to zero when the reference point for time synchronization is satellite.

These are the proposals on Common TA granularity submitted to RAN1#105-e:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 1: The granularity of Common TA is equal to 4∙64/2^μ*Tc |
| PANASONIC R&D Center Germany | Proposal 1: We propose a granularity of the common TA value NTA,common in the order of slot or half slot duration. |
| Spreadtrum Communications | Proposal 1: NTA, common should be expressed in the legacy granularity of Tc units. |
| CATT | Proposal 4: The granularity of common TA can apply 1024 Tc or 2048Tc of step size. |
| MediaTek Inc. | Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field - Common delay drift rate variation NTA,common,drift,thrird_order,derivative in a 6 bit field |
| CMCC | Proposal 2: The granularity of Common TA is set to be the same as the granularity of NTA, i.e., 16∙642μ∙Tc. |
| Qualcomm | Proposal 1: Network may broadcast a common timing offset value, X, with granularity of one slot assuming the subcarrier spacing of the SSB of the cell. - NTA,common is derived from X. |
| Ericsson | Proposal 11 The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length. |
| Apple | Proposal 2: The broadcast common TA is in the unit of Tc⋅(16⋅64/2μ). Proposal 6: RAN1 to study whether or not introducing the TA drift rate in TA command MAC CE. |
| ZTE | Proposal 3: The signaling granularity of common TA can be chosen as |
| Lenovo, Motorola Mobility | Proposal 1: Support the granularity of NTA,common to be same as granularity of TA command. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal 1: RAN1 to strive for a unified design of common RTT/delay signaling in order to avoid duplicate signaling of common RTT/delay. Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding signaling. Proposal 3: RAN 1 should agree on studying common delay signaling design with reduced signaling overhead. Proposal 4: RAN1 to consider the signaling of the parameter set NTA,scale, drift scale rate, NTA,cons, a constant, and NTA,power, an exponent (can take integer values), for signaling of the common RTT/delay in NTN. Proposal 5: RAN1 to consider UE autonomous drift rate calculation for common RTT/delay signaling. |
| Samsung | Proposal 3: The following options are considered for N_(TA,common) and N_TA. - Option 1 - NTA,common: its update is supported by group common signalling. - NTA: Defined as the same as Rel-15/16 - Option 2 - NTA,common: broadcasted by SIB. - NTA: Defined as the same as Rel-15/16 - Option 3 - NTA,common: broadcasted by SIB with drift rate information. - NTA: Defined as the same as Rel-15/16. Proposal 8: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information. |
| LG Electronics | Proposal 1. Support additional signaling by the network in order to apply proper common TA according to time changes at UE side. Potential solutions can include - Alt 1) providing the reference time corresponding to common TA - Alt 2) providing series of common TA Proposal 2. Within pre-defined set of TA offsets, the TA offset can be provided by gNB via higher layer signing (e.g., SIB or dedicated RRC signaling). - The TA offset can be independently corresponding to different ROs (or RO groups) |
| Xiaomi | Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred. |
| NTT DOCOMO, INC. | Proposal 3: One or multiple common TAs are broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 3 The granularity of common TA is set to be the same as the granularity of NTA, i. e. 16∙64/2μ*Tc. Proposal 4 The common TA configuration shall be cell specific via system information and the configuration can be associated with a group of cells to save signaling overhead. Proposal 5 To minimize spec impacts and signaling overhead, association between the existing TAG and the group of cells for the common TA configuration shall be considered. |

## Company views

On Common TA granularity, different view were expressed as follows:

- The same as the granularity of N_TA, i.e., 16∙642μ∙Tc: [Huawei, Spreadtrum Communications, CATT, CMCC, Apple, ZTE, Lenovo, Motorola Mobility, Asia Pacific Telecom]

- The granularity of 4∙642μ∙Tc: [THALES]

- The granularity of  642μ∙Tc: [Ericsson]

- The granularity of  2∙4∙642μ∙Tc: [Xiaomi, CATT]

- One slot: [PANASONIC, Qualcomm]

In view of the on-going discussion on Issue#1, the granularity of Common TA  is dependent on the solution adopted for the handling of timing drift on feeder link. Therefore, the initial proposal is made as follows:

Initial Proposal 3:

In case of LEO/MEO based non-terrestrial access network:

- If feeder link timing drift is compensated by UE using Common TA parameters, the granularity of Common TA is set to be:

  - Option 1:  The same as the granularity of NTA, i.e., 16∙642μ∙Tc.

  - Option 2: 4∙642μ∙Tc

  - Option 3: 642μ∙Tc

- If feeder link timing drift is compensated by the Network in a way transparent to UE, the granularity of Common TA is set to be:

  - Option 4: one slot duration assuming the subcarrier spacing of the SSB of the cell.

- Companies are encouraged to provide their comments and views:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Option 3: 642μ∙Tc Based on fixed-point modelling and simulations in our contribution, we proposed 18 bits for common TA is sufficient for optimum performance. Assuming LEO – 600 km, the maximum common delay is in the order of 6.4 ms with 10 degree elevation. This allows a granularity of 24.41 ns which is reasonable good fit for 642μ∙Tc=32.55 ns |
| Intel | Our preference is Option 1. |
| OPPO | It is not clear to us how to determine the subcarrier spacing, note that the common TA is used for connected UE also. While for connected UE, they may have different subcarrier spacing, but the common TA is the same for them. In this case, how to determine the subcarrier spacing for calculating the common TA? |
| Samsung | We prefer to use the same granularity for LEO/MEO/GEO cases not to make UE implementation complicated. Then, we need to think the range of feeder link delay for all satellite cases. In this regards, we prefer to support Option 1. |
| Xiaomi | We prefer option 4. |
| LG | In order to reduce the UE implementation complexity, we prefer to Option 1, regardless of if feeder link timing drift is compensated by UE or network. Moreover, it can be clarified whether common TA value is based on accumulation or absolute value. |
| CMCC | For the first main-bullet, Option 1 is preferred. We are fine with the second main-bullet, if feeder link timing drift compensated by the Network in a way transparent to UE is feasible. |
| Nokia, Nokia Shanghai Bell | In our opinion it is a bit premature to discuss the granularity of the signaling when there is not yet agreement on which parameters are provided to the UE to be able to derive the common TA values and the common TA values into the future. |
| ZTE | Firstly, same granularity is preferred to simplify the design and similar as the MAC CE based signalling, 16∙642μ∙Tc can be considered as baseline unless new requirement on the accuracy from RAN4 is defined. W.r.t the case that time drift is handled by gNB, it’s too early to discuss it without any agreement on this part. Moreover, the slot-level granularity is also not preferred in this case. Same level adjustment as MAC CE based TA command is still needed. |
| Panasonic | We prefer option 4. |
| Apple | For first bullet, we prefer common TA has the same granularity level as TA command MAC CE (i.e., Option 1) to save signaling overhead. Also, we are fine with the second bullet. |
| Ericsson | Option 3 since the quantization error should not make a significant contribution to the total TA error. The cost of adding a few more bits is well spent. As OPPO points out, the UL SCS may differ between UEs. The parameter μ should be the highest allowed for the given FR. |
| Sony | Our preference is option 1. |
| Lenovo/MM | Support option 1. |
| Huawei, HiSilicon | We prefer Option 1. |
| Spreadtrum | For the first main-bullet, we prefer Option1. We are also fine with the second bullet. |

## Updated proposal based on company views (First round of email discussions)

## Updated proposal based on company views (2nd round of email discussions)

# Issue#4: The need and the indication of TA margin

The need of TA-margin was discussed since RAN1#103-e meetings and it seems needed depending on requirements for UE-autonomous TA error, PRACH preamble format, common TA error, and common timing drift rate error.

In Moderator ‘s view:

- TA-margin is needed to account for TA estimation uncertainty when applying the TA pre-compensation in initial access: During the first acquisition of its UE-specific TA, the UE can overestimate the TA. In case of overestimation, the PRACH preamble will be received at gNB side in advance w.r.t. the PRACH occasion leading to unwanted interference with previous slot or PRACH occasion.

- If initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR. It is preferable to avoid such specification impact

W.r.t TA margin indication, there mainly two different views:

- including TA-margin within the Common TA. i.e.; Common TA configuration absorbs the maximum TA uncertainty.

- TA-margin indication in the SIB; for at least two main reasons:  In case when the RP is located at satellite, the Common TA will be zero and it may not be necessary to provide the Common TA by gNB. Further, the need of TA_margin is only relevant in case of TA acquisition for PRACH msg1/mgsA transmission.  New value of Common TA acquired by the UE in connected state should not include TA_margin.

The companies proposals on the need and indication of TA margin are collected in the following table:

| Companies | Proposals |
| --- | --- |
| Spreadtrum Communications | Proposal 4: Negative TA Command in RAR is not needed when TA margin is considered. |
| CATT | Proposal 5: TA margin can be one fixed value or not needed at all if UL timing requirement of RRC-IDLE mode and RRC-connected mode for UE is same. |
| THALES | Proposal 3: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements |
| CMCC | Proposal 7: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE. |
| OPPO | Proposal 1:TA margin is no longer needed to be discussed. |
| Nokia | Proposal 5: There is no need to indicate a TA margin. Any uncertainty related to TA should be covered by the Common TA value and the CP of the random access preamble. |
| Apple | Proposal 4: RAN1 to determine the necessity of specifying TA margin, depending on RAN4’s conclusion. |
| Lenovo, Motorola Mobility | Proposal 3: TA margin indication is not supported. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 6 Support TA margin for a PRACH transmission in case of Contention-based Random Access (CBRA) to prevent the use of negative TAC in RAR. |

## Company views

- The discussion on TA margin did not change much from previous meeting. It seems reasonable to postpone this discussion until RAN4 determines time synchronization requirements.

The initial proposal is made as follows:

- Initial proposal 4:

RAN1 to discuss the need and the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

- Companies are encouraged to provide their comments and views:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| OPPO | OK |
| Samsung | OK |
| Xiaomi | Ok |
| LG | OK |
| CMCC | OK |
| Nokia, Nokia Shanghai Bell | OK to wait for response from RAN4. |
| ZTE | Ok |
| Panasonic | Agree |
| Apple | Support |
| Qualcomm | OK |
| Ericsson | Ok |
| Sony | OK |
| Lenovo/MM | Agree with moderator’s proposal. |
| InterDigital | Ok |
| Huawei, HiSilicon | Agree. |
| NTT DOCOMO, INC. | OK |
| Spreadtrum | Support proposal |

## Updated proposal based on company views (First round of email discussions)

Based on the expressed views during  the first round of email discussions, the following FL Recommendation is made:

- FL Recommendation 4:

RAN1 to discuss the need and the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

# Issue#5:	TA command in RAR

In RAN1#103-e meeting, the following working assumption was made on TA command in RAR

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

There are some proposals in the contributions submitted to RAN1#105-e to confirm this working assumption:

| Companies | Proposals |
| --- | --- |
| Spreadtrum Communications | Proposal 2: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. |
| CATT | Proposal 6: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. |
| Apple | Proposal 5: The requirement that the existing TAC 12-bit field in Msg2/MsgB is reused is that a UE pre-compensates an accurate UE specific TA and TA margin in its Msg1/MsgA transmission. |

## Company views

- We will come back on this topic when the requirement on UL time pre-compensation for Msg1/MsgA transmission are defined.

The following initial proposal is made:

- Initial proposal 5:

Postpone the confirmation of the working assumption on non-extension of  TAC 12-bit field in msg2 (or msgB) until the requirements on UL time pre-compensation for Msg1/MsgA transmission are defined

- Companies are encouraged to provide the comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| OPPO | OK |
| Samsung | OK |
| Xiaomi | Ok |
| LG | OK |
| CMCC | OK |
| Nokia, Nokia Shanghai Bell | Although we do not expect that the UL time pre-compensation requirements will have an impact, we are ok to postpone the confirmation of the working assumption until those are defined. |
| ZTE | Ok |
| Panasonic | Agree |
| Apple | Fine with the proposal |
| Qualcomm | We don’t expect UL time pre-compensation requirements will have impact. |
| Ericsson | Ok |
| Sony | OK |
| Lenovo/MM | Agree with moderator’s proposal. |
| InterDigital | Ok |
| Huawei, HiSilicon | Agree. |

## Updated proposal based on company views (First round of email discussions)

Based on the expressed views during  the first round of email discussions, the following FL Recommendation is made:

- FL Recommendation 5:

Postpone the confirmation of the working assumption on non-extension of  TAC 12-bit field in msg2 (or msgB) until the requirements on UL time pre-compensation for Msg1/MsgA transmission are defined

# Issue#6 : TA update in RRC_CONNECTED state

In RAN1 Meeting #104-bis-e the following agreement was made:

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

In RAN1#104-e, the following agreements were made:

Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control

Many issues related to TA maintenance/update in connected mode are still open:

- FFS: Operation of closed loop and open loop TA control:

  - FFS: details of NTA update/accumulation

  - FFS: details of NTA,UE-specificupdate

  - FFS: details of NTA,commonupdate

- FFS: Details of the combination of open and closed loop TA control

The discussions on these FFS are highly dependent on RAN4 discussions on timing requirements for NR NTN and the issues related to the handling of timing drift on feeder link and Common TA parameters.

The following proposals on TA update in RRC-Connected state were provided by the different companies:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 4: Existing N_TA update in RRC_CONNECTED based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change |
| PANASONIC | Proposal 4: In RRC_CONNCTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation. |
| Spreadtrum Communications | Proposal 3: Existing NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change. |
| CATT | Proposal 7: On the close-loop and open-TA combination, UE can stop autonomous TA compensation or subtract the accumulated TA compensated by autonomous TA compensation during the gap between two neighboring TAC commands. |
| MediaTek Inc. | Proposal 3: In RRC_CONNECTED state: - NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change. - NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris - NTA,common is updated autonomously by the UE based on common TA parameters broadcast on SIB in case reference point for DL-UL subframe timing alignment is at the gNB. |
| CMCC | Proposal 8: For an NR NTN UE in RRC_CONNECTED, regular update of NTA,common can be disabled. Proposal 9: For an NR NTN UE in RRC_CONNECTED, TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc where NTA,common0 is the common TA obtained in initial access procedure. Proposal 10: The legacy MAC CE TA command and the enhanced MAC CE TA command consists of TA,δ,TA,drift to be both supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Proposal 11: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as NTAti=NTAaccumm+TA,driftm∙ti-tm' where, - m is the last received enhanced TA command before ti. - NTAaccumm is the m-th accumulated TA command. - Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0 |
| CAICT | Proposal2: FFS the condition when to apply the TA through combined closed loop and open loop |
| OPPO | Proposal 3: TA updating by MAC-CE or RAR can have the following relationship NTA_new = NTA_old + adjustment, where for TA updated by MAC-CE, the adjustment is and TA is the TAC indication in the range of [0, 63]; for TA updated by RAR, the adjustment is and the TA is the RAR TAC indication in the range of [0, 3846]. Proposal 4: If the frequency of reading SIB information every 3 seconds is acceptable, the network indicated common TA can be updated by a drift rate, e.g. NTA,common_new= NTA,common_old + NTA,common,drift,rate * t. |
| Ericsson | Proposal 1 RAN1 to discuss how to take into account satellite movement during DL signal propagation from satellite to UE when defining the UE-specific TA. Proposal 2 RAN1 to discuss how to take into account satellite movement during UL signal propagation from UE to satellite when defining the UE-specific TA. Proposal 3 RAN1 to discuss how to take into account satellite movement during signal propagation between gateway and satellite when defining the common TA. Proposal 5 The closed-loop component of the TA, NTA, should be defined as in Rel-16. Proposal 7: The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA. Proposal 8 The network broadcasts parameters describing the (one-way) common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay that may be implicitly (derived from DL subframe) or explicitly signaled Dcommon, base is the common one-way delay at time Tcommon,base Dcommon, drift is the common one-way delay drift rate. Proposal 9 The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite. |
| Nokia | Proposal 9: Open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages. |
| Apple | Proposal 6: RAN1 to study whether or not introducing the TA drift rate in TA command MAC CE. |
| Sony | Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs. Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE. |
| ZTE | Proposal 4: In RRC_CONNECTED state, the applied TA can be determined as by assuming the support for the 2nd order of TA drift: - , - where - is the service link TA at activation time of ephemeris, and is the variation of service link TA between activation time of ephemeris and UL transmission time, which can be estimated by UE based on GNSS and indicated information. - is the broadcast common TA, and are the first and second order common TA drift rate indicated by BS, is the interval between activation time of broadcast common TA and DL transmission time of scheduling information, and is the interval between DL transmission time of scheduling information and UL transmission time. - is indicated in MAC CE TA command. Proposal 5: When common TA and ephemeris are broadcast, indication of a close loop TA command can be considered to handle possible overcompensation in RRC_CONNECTED state. |
| Lenovo, Motorola Mobility | Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE. |
| Samsung | Proposal 6: Each of the following options are supported based on the gNB configuration:  Closed-loop TA control  Open-loop TA control  Combination of open&closed-loop TA control |
| LG Electronics | Proposal 4. - At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling. - In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling. Proposal 7: Define the appropriate TA control method for each TA as follows: - Closed loop TA control is applied to common TA (i.e., N_(TA,common)) - Open loop TA control is applied to UE specific TA (i.e., N_(TA,UE-specific)) Proposal 8: Support independent TA update for each TA using independent TA control methods. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 1 Legacy timing advance command (TAC) for NTA and NTA,offset in the case of RAR is the baseline for NTN. No specification change is needed to minimize MAC impacts, e.g., TAC MAC CE format. Proposal 2 Legacy timing advance (TA) adjustment for NTA and NTA,offset in RRC_CONNECTED is the baseline for NTN. No specification change is needed to minimize MAC impacts, e.g., TA timers, and groups. |

## Company views

w.r.t NTA update/accumulation, it was proposed to reuse existing N_TA update in RRC_CONNECTED as in Rel-16 without specification change [Thales, Ericsson, Spreadtrum Communications, MediaTek, ZTE, Asia Pacific Telecom, FGI, ITRI, III].

Enhancements on NTA update proposed by [OPPO, CMCC]

w.r.t operation of  open loop TA control, there were some inputs provide by [MediaTek, Ericsson, ZTE, CMCC, OPPO]. Further [Ericsson] made some observations on UE-specific TA and Common TA calculation and proposed to take into account satellite movement which gives good accuracy.

There were also some inputs submitted to RAN1#105-e on combination of open and closed-loop TA:

[CATT] proposed that UE can stop autonomous TA compensation or subtract the accumulated TA compensated by autonomous TA compensation during the gap between two neighboring TAC commands. For [Nokia] open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages. According to [Sony] in setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs. [LG] Support independent TA update for each TA using independent TA control methods.For [CAICT, Sony , Lenovo, Motorola Mobility ] combination of open and closed-loop TA is FFS.

[Moderator’s view] : In RRC_Connected state the NTN UE shall update (NTA,UE-specific+NTA,common) so that the cyclic prefix could absorb the propagation delay variation due to satellite movement . With Open loop TA update, the UE compensates the delay so that the residual delay can be absorbed by the CP in use.

That is said, details of combination of open and closed-loop TA should be defined in coordination with  RAN4. Timing requirements for NR NTN are under discussion is current RAN4 meeting. The discussions were already started at RAN4 Meeting #98-bis-e. Remaining FFS and WF from RAN4 #98-bis-e meeting are recalled hereafter:

RAN4 Meeting #98-bis-e: WF on timing requirements for NR NTN WF on UE specific TA estimation error: - UE specific TA estimation accuracy is FFS - FFS whether the UE specific TA estimation accuracy shall be also defined as a separate accuracy requirement - Specify UE behavior related to UE specific TA estimation and the detailed behavior is FFS - FFS on the update periodicity of UE specific TA value WF on timing requirement: - UE initial transmit timing error (Te) - Te requirement in NTN is consist of: - Same types of errors as terrestrial UE e.g. DL timing estimation accuracy and UL timing setting accuracy. and; - fic estimation accuracy; - FFS on whether and how much different relaxations are required for different sets of SCS of SSB and SCS of uplink signals - It is the total NTN UE Te error that decides UL performance, no matter the source of inaccuracy. - N_TA_offset - The existing N_TA offset value defined in Table 7.1.2-2 in TS38.133 can be reused in NTN - Gradual timing adjustment - FFS whether to define new gradual timing adjustment requirements for NTN UE - FFS whether and how to count the maximum delay variation for the round trip delay; - FFS: whether define different requirements for different NTN topologies in terms of, e.g. GEO, MEO, LEO, HAPS, HIBS, altitude, elevation angles for feeder/service links, UE speed, etc; - FFS the reference timing for the Gradual timing adjustment in NTN - One shot timing adjustment - Not introduce one shot timing adjustment requirement for NTN UE WF on TA adjustment accuracy requirement: - In RRC_idle mode - FFS whether to define TA adjustment accuracy requirement; - In RRC_CONNECTED mode - Option 1: Reuse the existing TA adjustment accuracy requirement defined in TS 38.133 with considering of UL timing quantization accuracy. - Option 2: FFS on whether relax the TA adjustment accuracy requirement. - FFS on UE position and satellite position estimation error; - FFS on propagation delay change from a slot when UE received timing advance command to a slot when the indicated TA.

Based on companies’ proposals submitted to RAN1#105-e and in view of the RAN4’s WF on timing requirements for NR NTN , the initial proposal is made as follows:

Initial Proposal 6-1:

In RRC_CONNECTED state in NR NTN:

- NTA update based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as in NR Release 16.

    - FFS: Whether some enhancements are needed .e.g. new gradual timing adjustment in NTN

- NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris

    - The UE calculates the NTA,UE-specific (in Tc units) as follows:

    - NTA, UE-specific=ds,DL+ds,UL/Tc
where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

- NTA,common is updated autonomously by the UE based on common TA parameters broadcast on SIB

    - FFS: Common TA parameters

    - FFS: How the UE calculates/update the NTA,common

Companies are encouraged to provide their comments on initial Proposal 6-1:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| Intel | OK with the proposal |
| OPPO | For NTA , it should be updated considering the positive and negative value in mag2/mag B same as updated in MAC-CE. Because when the NTA is starts to update after the initial access, there may be overestimated or underestimated. For UE specific TA, it may not be necessary to discuss the formula for calculation. It can be left for UE implementation. |
| Samsung | We are basically okay for this proposal. - NTA: OK. FFS can be removed. - NTA,UE-specific: the definition of ds,DL+ds,UL are unclear. - NTA,common: can be determined after the decision of drift rate discussion. |
| LG | Regarding NTA, we are fine with the proposal, but we think the first FFS sub-bullet is not necessary. Regarding UE specific TA, we agree with OPPO. It is reasonable to leave it as an UE implementation. Regarding common TA, it can be discussed after the issue #1 and #2 is determined. By the way, we can discuss further which DL signal/channel can be used for signalling the common TA and/or the common TA parameters, e.g., SIB1 and/or SIB-x and/or dedicated RRC signalling, etc. |
| CMCC | Regarding NTA update, disagree. In our view, the legacy MAC CE TA command as in NR Release 16 and the enhanced MAC CE TA command consists of TA,δ,TA,drift to be both supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Regarding NTA,UE-specific, we are fine in principle. Regarding NTA,common, disagree. The assumption that UE frequently read the common TA parameters on broadcasted SIB in connected mode may be not valid. According to TS 38.214 Clause 5.1, the UE is not expected to read SIB during a process of autonomous SI acquisition when it is overlapped with PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI. On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.. Regarding common TA is timing varying parameters, the field may be excluded when determining changes in system information, i.e. changes of common TA parameters should neither result in system information change notifications nor in a modification of valueTag in SIB1, just like timeInfoUTC field in SIB9. Hence, common TA parameter update may be carried during a process of autonomous SI acquisition, and UE is not required to read it in time. Thus, NTA,common updated autonomously by the UE based on common TA parameters broadcast on SIB can not be assumed. |
| Nokia, Nokia Shanghai Bell | First component of the Initial proposal 6-1 is OK to us. The second component we would prefer is described in a more generic format that matches the intention. That is, is simply describes the delay that the UE need to compensate, and then we leave the conversion into N_TA,UE-specific to UE implementation. For the third component we would prefer to have the description open to the fact that the UE autonomously determines the N_TA,common from provided parameters (and thereby avoid coupling it directly to the TA parameters). |
| Panasonic | We support the proposal. |
| Apple | Generally fine with the direction of the proposal. We may need to clarify the difference between ds,DL and ds,UL. |
| Qualcomm | On the second bullet, it is preferred to describe what is considered as the service link RTD for a transmission in a more general way. We don’t see the need of the third bullet as it is already captured in previous agreement(s). |
| Ericsson | We are fine with the proposal except that “e.g. new gradual timing adjustment in NTN” in the FFS could be removed since the gradual timing adjustment is under RAN4 responsibility. (Also, it should be applied to the total TA, not the closed-loop TA specifically.) |
| Sony | Support proposal. |
| Huawei, HiSilicon | We share similar views with several other companies that it is up to UE implementation to calculate NTA,UE-specific. For new gradual timing adjustment in NTN, it is not clear what should be done in RAN. We expect this will be defined by RAN4. |

Details of combination of open and closed-loop TA should be defined in coordination with  RAN4. Therefore the initial proposal is to send an LS to RAN4:

Initial Proposal 6-2:

Send an LS to RAN4 to enquire details of combination of open and closed-loop TA.

Companies are encouraged to provide their comments on Initial Proposal 6-2 :

| Companies | Comments and Views |
| --- | --- |
| MediaTek | First conclude the FFS in RAN1 |
| LG | Agree with MediaTek. |
| Nokia, Nokia Shanghai Bell | According to our understanding, we do not need to ask RAN4 on this topic. We would not expect any timing related requirements to be changed here. RAN1 should simply apply the needed changes to ensure that the combination of open loop and closed loop TA adjustments does not violate existing timing requirements. |
| ZTE | Agree with MTK, and we need to deal with details on the solution design. Otherwise, all thing will be coupled without any progress. |
| Panasonic | Agree |
| Qualcomm | We can conclude on a baseline assumption (UE always add the accumulated TA command and UE autonomous TA) and ask RAN4 if there are any concerns |
| Ericsson | We do not think an LS to RAN4 will help at this point. |
| Lenovo/MM | We share similar view with MTK that we should first have a clear picture then we can ask RAN4 accuracy related issue. |
| Huawei, HiSilicon | Agree with others that there is no need to send the LS now. |

## Updated proposal based on company views (First round of email discussions)

Support proposal : [MediaTek, Intel, Panasonic, Apple, Thales, Ericsson, Sony]

Support proposal with some modification: [Samsung, LG]: 	NTA: OK. FFS can be removed.

[OPPO, LG]: For UE specific TA, it may not be necessary to discuss the formula for calculation. It can be left for UE implementation.

Based on the views expressed during the first round of email discussions, several companies  share the view that it is up to UE implementation to calculate NTA,UE-specific.

[Moderator] share the same view as highlighted by Ericsson in R1-2104811  and copied hereafter: to accurately calculate the service link delay, it is necessary to take into account that the UL and DL signals are offset in time and therefore subject to different service link delays

[Ericsson R1-2104811]: #### UE-specific TA, NTA,UE-specific The purpose of the UE-specific TA is to compensate for the RTT of the service link. The UE-specific TA is autonomously determined by the UE based on GNSS-acquired UE position and serving satellite position from broadcast ephemeris. As discussed in Section 2, to accurately calculate the service link delay, it is necessary to take into account that the UL and DL signals are offset in time and therefore subject to different service link delays. It is also necessary to take into account when calculating the service link delay that the satellite moves during the transmission across the service link. NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL and ds,UL are the DL and UL service link delays derived by the UE, respectively.

Also, it is necessary to agree a baseline assumption on the combination of open and closed loop TA update then possibly discuss with RAN4 if there are some concerns.

Based on the expressed view during the first round of email discussions, the updated proposal is given hereafter:

Updated Proposal 6:

In RRC_CONNECTED state in NR NTN:

- NTA update based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as in NR Release 16.

    - FFS: Whether some enhancements are needed

- NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris

    - The UE calculates the NTA,UE-specific (in Tc units) as follows:

      - NTA, UE-specific=ds,DL+ds,UL/Tc
where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

- NTA,common is updated autonomously by the UE based on the parameters acquired from SIB

    - FFS: Common TA parameters

    - FFS: How the UE calculates/update the NTA,common

- UE always add the accumulated TA command and UE autonomous TA

Companies are encouraged to provide their comments on initial Proposal 6:

| Companies | Comments and Views |
| --- | --- |

## Updated proposal based on company views (2nd round of email discussions)

# Issue#7 : NTN UE Time Alignment Timers

The legacy timeAlignmentTimer is used for closed loop TA update. Similarly specific timers may be needed for open loop TA update.

This issue was discussed in RAN1#104-bis-e. Ccompanies were open to discuss the need of validly timer after the signalling of ephemeris, common TA, and requirements are clearer. The following FL recommendation was made:

FL recommendation 7:

Companies are encouraged provide inputs to RAN1#105-e about the need of the validity timers for Common TA and UE specific TA

Regarding the Time Alignment Timers for NTN UE these proposals were submitted to RAN1#105-e:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 5: A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA calculation. - This timer is restarted each time the UE receives new assistance information. - The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available. |
| MediaTek Inc. | Proposal 4: NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network. Proposal 5: Before it can be discussed whether there is a need for a validity timer configured for common TA tracking tracking, it is needed to discuss the aspects that are relevant for common TA tracking. Proposal 6: The common TA parameters can be signalled on the same SIB with the PV/ephemeris information, in which case the validity timer if used can be common to both information. |
| OPPO | Proposal 8: Two timers are introduced to ensure the accuracy of NTA,common and NTA,UE-specific. |
| Nokia | Proposal 11: A validity timer configured for both UE-specific TA and Common TA defining the maximum time during which the UE can track the RTD on service link and Common RTD without having acquired new ephemeris or assistance information shall be used. - This validity timer is restarted each time the UE receives new ephemeris and assistance information to be used for Common TA estimation. - The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris and assistance information is not available. Proposal 12: RAN2 to study whether the TA timer can be also used as a validity timer for open-loop update of both UE-specific TA and Common TA. |
| ZTE | Proposal 6: Indication of on the valid time for satellite ephemeris data is preferred instead of UE-specific TA. Proposal 7: Configuration on the validity timer should be supported for common TA and common TA drift rate, and it will be started/restarted when a common TA or common TA drift rate is activated |
| Lenovo, Motorola Mobility | Proposal 5: Support indication of TA validity timer value in the TA indication message |
| LG Electronics | Proposal 5. Support separate validity timers for common TA and UE specific TA in Rel-17 NTN. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 8 For RACH, no validity timer for common TA is needed, if common TA is provided in SIB1, and if common TA update triggers SI change indication, then UE shall update common TA by re-acquiring SIB1 after receiving a short message in its paging occasion. Proposal 9 For RRC_CONNECTED, no validity timer for common TA is needed if common TA is provided in SIB1 and if common TA update triggers SI change indication. Proposal 10 No validity timer for UE-specific TA is needed if a GNSS measurement gap is supported for the L band. NW and UE shall share a common understanding of when to update GNSS and ephemeris. |

## Company views

[THALES] proposed to introduce a validity timer for Common TA

According to [MediaTek ] before it can be discussed whether there is a need for a validity timer configured for common TA tracking tracking, it is needed to discuss the aspects that are relevant for common TA tracking. [MediaTek ] proposed to introduce  NTN UE time alignment timer for re-acquisition of the satellite ephemeris.

[OPPO, Nokia, LG] proposed to introduce both timers

For [ZTE] Indication of on the valid time for satellite ephemeris data is preferred instead of UE-specific TA. And proposed to support the validity timer for common TA.

[Lenovo, Motorola Mobility] Support indication of TA validity timer value in the TA indication message.

[Asia Pacific Telecom, FGI, ITRI, III] do not support the introduction of validity timer for Common TA and for UE-specific TA.

From [Moderatos’s perspective]:

For NTA,common update, the Common TA parameters acquired in the SIB will be valid only during a validity time period which depends on the maximum tolerable error on common TA estimation which is depending on the method adopted for feeder link timing drift handling (issue#1). At the reception of new Common TA parameters, the UE shall start/restart related timer. At expiry of this timer a new acquisition of Common TA estimation assistance information is required

For UE specific TA update the UE needs to acquire satellite ephemeris and use its own propagator to predict its specific TA during a certain time period without the need of acquiring new ephemeris data. The newly acquired ephemeris data will be valid only  during a period depending on propagator model used by the UE and the maximum tolerable error on the estimation of N_(TA,UE-specific). But as the orbital propagator model to be used at UE side is left to implementation as per the conclusion made in RAN1#104-bis-e, it will be difficult to configure a validity Timer for UE specific TA if the orbital propagator model used at UE side is not known by the Network.

Based on the above, the following  Initial Proposal is made:

Initial proposal 7:

A validity timer configured for Common TA defines the maximum time during which the UE can apply the Common TA without having acquired new Common TA parameters to be used for Common TA calculation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

FFS: NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network.

Companies are encouraged to provide their comments and views on Initial Proposal 7:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal if FFS part can be included with dropping FFS. Based on Issue #2, the curve fitting algorithm may lead to different performance and is implementation dependent. The UE has no way to know how long the common TA parameters are valid. It may depend on the orbit, elevation angle, prediction method in the Gatway/gNB, … FFS: NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network. |
| Intel | In principle we support the proposal. However, timers are usually discussed in RAN2. In RAN1 we can make agreement on the need of such timer and RAN2 should do relevant work to specify it. So, we prefer the following proposal: Modified proposal 7: A validity timer for common TA is needed to avoid loss of uplink time synchronisation - A validity timer configured for Common TA defines the maximum time during which the UE can apply the Common TA without having acquired new Common TA parameters to be used for Common TA calculation. - This timer is restarted each time the UE receives new assistance information. - The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available. FFS: NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network. |
| OPPO | Support |
| Samsung | RAN2 can discuss the issues related to UE behaviour about timers. |
| Xiaomi | Support |
| LG | Support the proposal Also, we prefer to support another validity timer for UE specific TA. |
| CMCC | In our view, in the case gNB unknowns when UE starts/restarts the timer, the UE may miss to read SIB carrying common TA parameters/ satellite ephemeris when it is overlapped with PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, thus, timer based solution seems unreliable. |
| Nokia, Nokia Shanghai Bell | We could support the proposal where one validity timer is configured, and this timer should be used for both Common TA and satellite ephemeris information. Both information will be used together by the UE to adjust its UL transmission in time. If one of the two timers would expire first, the UE would anyway assume it has lost UL synchronization. Thus we do not see the reason for having separate timers for Common TA and ephemeris information. |
| ZTE | We are supportive to introduce the valid timer for common TA as mentioned in the main bullet. Other details can be up to RAN2’s decision. |
| Panasonic | Agree |
| Qualcomm | The common TA is totally controlled by the network. Network does not necessarily guarantee the continuity of its signaling. Whenever, it is updated, UE needs to read it. |
| Ericsson | We are fine with the proposal. Regarding the FFS, we think that a validity timer for satellite ephemeris is also needed. There might be a need for a reference model for orbit propagation defined in RAN4. The network will know how fast the satellite orbit deviates from the reference model and can set validity timer accordingly. The UE TA accuracy requirements would be tested using the reference model. |
| Sony | Support. |
| Lenovo/MM | From our perspective, there is already a timer for UE to determine whether the UE can maintain uplink synchronization. The existing timer in legacy release corresponding all the effects of UE-specific TA calculation, common TA indication, TAC in MAC CE, etc. We prefer a similar timer as in legacy release is used for NTN rather than a timer for common TA and a timer for the total. We also want to mention that the “total timer” changes over time as the satellite elevation angle changes. The configuration of the total timer can be together with the MAC CE command containing the TAC. |
| InterDigital | It should be discussed in RAN2 as timer will be specified in RAN2 specification if supported. We can send an LS to RAN2 to indicate that the necessity of supporting validity from RAN1 perspective if it is agreed. We tend to agree with Qualcomm that the common TA is fully managed by gNB as well as validity timer (if agreed). gNB should send common TA frequent enough before validity timer expires at the gNB. Given this observation, we don’t see a good justification to support validity timer. |
| Huawei, HiSilicon | A validity timer configured for Common TA may not be needed. Common TA will be updated by the network according to its validity. If the UE does not acquire the updated Common TA, the UL timing error will accumulate and when it exceeds the range of TAC, the gNB can stop sending TAC then RLF failure can be triggered by the current timer for TAC. A validity Timer for UE specific TA may be dependent on orbital propagator model used at UE, an indication from the UE for the validity according to its capability is more reasonable in order to ensure a common understanding between gNB and UE. |
| Spreadtrum | Support the proposal |

## Updated proposal based on company views (First round of email discussions)

Based on the above proposal 7 is updated as follows:

Updated proposal 7:

- A validity timer configured for Common TA defines the maximum time during which the UE can apply the Common TA without having acquired new Common TA parameters to be used for Common TA calculation.

- NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network.

FFS: Whether only one validity timer is configured for both Common TA and satellite ephemeris information

## Updated proposal based on company views (2nd round of email discussions)

# Issue#8: TA reporting

TA reporting was already discussed in during the last two RAN1’s meetings. In RAN1#104-bis-e the following FL recommendation was made:

Handle TA Reporting proposals under A.I. 8.4.1 or under A.I. 8.4.2 once sufficient progress has been made in A.I. 8.4.1.

Within the contributions to RAN1#105-e, proponents companies provided the following proposals:

| Companies | Proposals |
| --- | --- |
| MediaTek Inc. | Proposal 7: Handle TA Reporting proposals under A.I. 8.4.1 |
| CMCC | Proposal 12: RAN1 to further study the details of UE reporting TA related information to facilitate network updating K_offset after initial access. The following reporting information can be further studied. - Option 1: UE location. - Option 2: UE self-estimated TA (NTA,UE-specific). - Option 3: the difference value (in unit of slot) between K_offset signaled in system information and a UE specific K_offset as suggest by UE. |
| Samsung | Proposal 5: A mechanism to align T_TA between the gNB and the UE is adopted, e.g., a UE reports N_(TA,UE-specific) when it updates N_(TA,UE-specific) or the gNB triggers the UE to update N_(TA,UE-specific) with a granularity of 16∙64∙T_c/2^μ. Proposal 7: UE’s estimated TA value is reported to gNB by MAC CE. |
| LG Electronics | Proposal 3. Support implicit reporting of UE specific TA estimated by the UE. • The different TA (or the range of TA) can be mapped to different ROs (or RO groups). Proposal 6. RAN1 should discuss how to report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states. |
| InterDigital, Inc. | Proposal-2: support the reporting of the UE-specific TA after initial access. |
| Mitsubishi Electric RCE | Proposal 1: UEs using self-acquired timing advance for initial access signal the pre-compensation value NTA to the network during the initial access process. |

## Company views

From moderator’s view TA Reporting should be handled under A.I. 8.4.1.

The following FL recommendation is made:

FL recommendation 8:

Handle the issue on TA Reporting under A.I. 8.4.1.

- Companies are encouraged to provide their comments on Moderator’s recommendation 8:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| OPPO | OK |
| Samsung | OK |
| Xiaomi | Ok |
| LG | OK |
| CMCC | OK |
| Nokia, Nokia Shanghai Bell | If TA reporting is to be supported, it would probably fit better under AI 8.4.1. |
| ZTE | Fine, but this issue should be supported eventually. |
| Panasonic | Agree |
| Apple | Agree |
| Qualcomm | Agree |
| Ericsson | We support the proposal. |
| Sony | Support. |
| InterDigital | Ok |
| Huawei, HiSilicon | Agree |
| Spreadtrum | Support proposal |

# Issue#9: Broadcasting the position of a reference point

Broadcasting the position of a reference point is an alternative solution that would simplify the time and frequency compensation mechanisms was proposed by some companies [Huawei, Ericsson, CMCC].

The principles of this UE centric pre-compensation solution is captured in the following observation made by [Ericsson]: If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signalled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Support indication of the location of reference point as an alternative solution for common TA indication. |
| CMCC | Proposal 13: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. A fixed common TA can be additional broadcasted, if needed. |
| Ericsson | Proposal 15 Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. |
| NTT DOCOMO, INC. | Proposal 7: It is not necessary to broadcast the location of reference point and define two types of calculation methods for TA and doppler shift. |

## Company views

From Moderator’s perspective, the main issue with this simple solution is the security and  regulatory aspects. The NTN GW position may or may not be a critical information depending on the deployment scenario. Then, it is essential to support the cases where it is not acceptable to share this information with the users. Further this solution may not work when there are inter-satellite links.

That is said, to move forward and to get more inputs regarding the security / regulatory aspects,  a discussion involving other Working groups i.e SA3 and possibly SA3-LI might be necessary.

Therefore, the initial proposal is made as follows:

Initial proposal 9

RAN1should send an LS to SA3 , SA1 and  possibly SA3-LI  to get more inputs regarding the security /regulatory aspects if the NTN GW position is broadcast

Companies are encouraged to provide their comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support propopal |
| Samsung | RAN1 needs to discuss whether other additional mechanism is needed or not. |
| Xiaomi | Broadcasting NTN GW position is not necessary because common TA mechanism has been agreed. |
| LG | Support. RAN1 can discuss this issue further after receiving a response to this LS. |
| CMCC | Support the proposal. In our view, Observation 3: Broadcasting the position of a reference point of the feeder link is beneficial to simplify the time compensation procedures and reduce signaling overhead for frequent update of NTA,common. Observation 4: For TA pre-compensation, broadcasting the position of a reference point of the feeder link with certain accuracy is feasible. Observation 5: Security issue can be fixed by broadcasting the position of a reference point of the feeder link with certain artificial bias. Proposal 13: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. A fixed common TA can be additional broadcasted, if needed. |
| Nokia, Nokia Shanghai Bell | We support sending an LS as indicated in the initial proposal 9. One thing to note is that for the feeder link information that is being discussed under Issue#1 in this document, it is possible for any device to do reverse engineering to derive the GW location, as the FL common delay (including TA drift rate) will provide the propagation delay during a fly-over, and having multiple orbits will allow the UE (or any device) to derive the GW location. |
| ZTE | It’s not prefer to initialize the discussion on other additional mechanism since we have already worked on the one approaches. Otherwise, the efforts will be wasted. Moreover, from scheduling perspective, it’s not only the security issue for NTN gateway, it’s also related to the broadcast of location of BS. We need to have a clear understanding that there are at least two cases for deployment: - Co-located gateway and gNB - Separated gateway and gNB For these two case, to achieve the DL-UL aligned scheduling, the location of gNB should be broadcast to UE in such additional mechanism. |
| Panasonic | OK |
| Apple | RAN1 needs to first get consensus that an alternative mechanism (besides common TA broadcast) is needed before sending the LS. |
| Ericsson | Ok |
| Huawei, HiSilicon | Agree |
| NTT DOCOMO, INC. | As stated in our tdoc, it is not only the security issue but also the unclear motivation to define two types of calculation methods. RAN1 should first discuss whether this new type of calculation method is necessary before sending the LS. |

## Updated proposal based on company views (First round of email discussions)

Based on the above discussion the Proposal 9 is updated as follows:

Updated  proposal 9

RAN1should send an LS to SA3 , SA1 and  possibly SA3-LI  to get more inputs regarding the security /regulatory aspects if the NTN GW position is broadcast.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#10 : Common Doppler shift pre/post compensation on the feeder link

In RAN1#103e, it was agreed that an NR NTN UE shall be capable to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. But there is not agreement yet on Doppler shift pre/post compensation on the feeder link. It is not yet clear if it has to be performed by the UE or the Network (gNB and/or GW/Satellite).

This issue was heavily discussed during RAN1#104-bis-e and it was proposed  that the Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

The following is proposed by the companies w.r.t to Common Doppler shift pre/post compensation on the feeder link:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 6:The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| PANASONIC R&D Center Germany | Proposal 5: The Doppler compensation on the feeder link should be transparent to the UE. |
| Spreadtrum Communications | Proposal 5: The gateway and satellite-payload compensate common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. |
| CATT | Proposal 9: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a transparent way at least to the UE, whether compensated by the gNB is up to RAN4 decision. |
| MediaTek Inc. | Proposal 8: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| CMCC | Proposal 14: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| Qualcomm | Proposal 2: The common Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| Ericsson | Proposal 13 The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link. Proposal 14 RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE. |
| Nokia | Proposal 14: The Doppler shift over the feeder link and any transponder frequency error in both DL and UL are compensated by the satellite subsystem in a way transparent at least to the UE and gNB. |
| Apple | Proposal 7: Confirm the working assumption that the gateway and satellite-payload compensate common Doppler shift/Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to UE and gNB. |
| Samsung | Proposal 11: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa. |
| Xiaomi | Proposal 6: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| NTT DOCOMO, INC. | Proposal 4: The doppler shift over the feeder link and any transponder frequency error for both downlink and uplink are compensated in a way transparent to UE. |

## Company views

Many companies [Thales, PANASONIC R&D Center Germany, CATT, MediaTek, CMCC, Qualcomm, Xiaomi, NTT DOCOMO] share the view that that the Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. Whether it is done in a transparent way to gNB is not a question to be answered by RAN1.

Within its contribution to RAN1#105-e, [Thales] provided inputs on the ephemeris accuracy expected to perform Doppler pre/post-compensation on feeder link. Based on simulation results [Thales] observed that even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Based on the view expressed on this topic the following proposal is made. It can be taken as a working assumption is which RAN1 can revisit later if it requires update.

Initial proposal 10

Working assumption:

The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

- Companies are encouraged to provide their comments on this working assumption in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| Intel | Support the proposal |
| OPPO | support |
| Samsung | OK |
| Xiaomi | Support |
| LG | Support |
| CMCC | Support |
| Nokia, Nokia Shanghai Bell | We would prefer to stick to the working assumption that was discussed (but not formally agreed) at RAN1#104bis-e. Hence, have as working assumption: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent at least to the UE and NR gNB. |
| ZTE | This part should also be transparent to gNB. |
| Panasonic | Agree |
| Apple | Support |
| Qualcomm | Agree |
| Ericsson | We prefer the solution that the gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. |
| Sony | Support. |
| Lenovo/MM | Support the proposal. |
| Huawei, HiSilicon | Agee. |
| NTT DOCOMO, INC. | We are fine with the proposal. Besides, we think that the doppler shift over the feeder link can be compensated by gNB or Gateway or Satellite. |
| Spreadtrum | Support the proposal |

## Updated proposal based on company views (First round of email discussions)

Since Ka/Q/V band are widely used for feeder link operation, having a UE to compensate feeder link Doppler may not be feasible. For example, in case of feeder link with carrier frequency  20 GHz , the two-way Doppler in a feeder link will have a maximal Doppler of 25ppm x 2x 20 GHz= 1 MHz. To compensate such a large Doppler in S band can be problematic, because of reduced available spectrum.

Updated proposal 10:

The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated by the GW and Satellite in a way transparent to UE.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#11: Indication of common frequency pre-compensation offset on DL service link

In RAN1#104-e, the following conclusion was made:

Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. • FFS: support of DL frequency compensation for the service link Doppler.

Regarding the FFS in the conclusion above, as already discussed in [TR38.821] It was observed via simulations that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift :

### [TR38.821] : ### 6.3.2 DL synchronization According to the simulation assumptions in Table 6.1.2-1, the performance evaluation on the DL synchronization performance is conducted. The corresponding results from [43], [44], [45], [46], [47] are summarized in [48]. It is observed that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift, e.g., conducted with respect to the spot beam center at network side, respectively. However, for the LEO without pre-compensation of the frequency offset, additional complexity is needed at UE receiver to achieve robust DL initial synchronization performance based on Rel-15 SSB. No further enhancement on the SSB is needed. Additionally, w.r.t the performance on the DL timing/frequency tracking, no issues have been identified based on Rel-15/16 NR design. Potential optimization can be further considered in potential normative phase if necessary.

This issue was discussed in RAN1#104-bis-e and the detail on signalling is still remained to be discussed.

Several companies shared the view that a common signalling for both earth-fixed and earth-moving cells is preferred.

It means that for both earth-fixed and earth-moving cells the amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE.

The most challenging case is the Earth-fixed cell: In this case if DL frequency compensation for the service link Doppler is applied, gNB needs to periodically update and broadcast the DL pre-compensated frequency offset (FO). This pre-compensated  FO is time-varying in case of Earth fixed cell.

The pre-compensated  FO will be used by the UE to perform its own estimation of the residual Doppler experienced on the DL. The UE needs to know this residual Doppler to be able to generate  UL frequency carrier for its uplink transmission. Any residual error will lead to a UE transmit frequency error twice in the uplink.

Therefore, the UE shall acquire the broadcast pre-compensated  Frequency offset (FO) and performs .e.g a first order approximation to self-calculate the pre-compensated  FO. Hence, the network needs to also indicate the drift rate of common pre-compensated FO. The error on self-calculated pre-compensated FO should be within a specified threshold .e.g 10% 0,1 ppm, that is 0.01 ppm.

As a consequence, If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters. every 2.3 seconds Based on simulation [2].

Therefore, in case of Earth fixed cell two Way-forwards could be envisaged:

WF 1:  Indicate the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB. The UE will assume that the pre-compensation changes continuously with time. Thereby,  the gNB DL frequency needs to be periodically updated so that the deviation between the UE self-calculated DL pre-compensation (based on indicated beam-specific ECEF co-ordinates and Satellite ephemeris) and actual pre-compensation remains within a defined threshold e.g. 10% of 0.1PPM.

WF2: Similar to Earth moving cell, the amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE. But in this case as mentioned above, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters. every 2.3 seconds. Which make this is WF questionable.

In all case, RAN1 to consider also indication to UE regarding frequency pre-compensation performed at neighbouring cells/beams.

Bases on the above another possible WF:

WF 3:  based on the following observation made in [R1-1909479] and recalled hereafter:

Observation 1: For the DL synchronization in NTN:

      - SSB design in Rel-15 can provide the robust performance (i.e., synchronization) in following cases

        - GEO

        - With pre-compensation for LEO

          - Note: The above observation can be revised if proved by other results

        - FFS: Without pre-compensation for LEO

          - With consideration on, e.g., latency, complexity for SSB detection

A possible way forward is to deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

Companies proposals regarding Issue#11 are collected in the following table:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 7: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing. Proposal 9: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment. |
| THALES | Proposal 7: Deprioritize support of DL frequency compensation for the service link Doppler in Release 17. |
| CATT | Proposal 8: Common Doppler shift compensation value of DL and UL for service link can be indicated to UE, which can be defined with Khz granularity |
| MediaTek Inc. | Proposal 9: Deprioritize support of DL frequency compensation for the service link Doppler in Release 17. |
| CMCC | Proposal 15: Support indication of DL frequency compensation for the service link Doppler, i.e., - If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. |
| OPPO | Proposal 9: It is preferential to consider the granularity of frequency compensation in kHz. Proposal 10: Indication the amount of frequency compensation should base on the SCS. |
| Ericsson | Proposal 12 If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. |
| Nokia | Proposal 15: In the downlink a common frequency offset on service link is pre-compensated to limit the UE search space for the synchronization signals. Proposal 16: The amount of common frequency pre-compensation in DL in a cell may be indicated to the UE and thereby be used for determining the amount of UL frequency pre-compensation. Proposal 17: A common signaling should be used to indicate the amount of applied frequency pre-compensation in DL for both earth-moving and earth-fixed cells. |
| Apple | Proposal 8: In downlink transmissions, support gNB pre-compensates and indicates a frequency offset for the service link Doppler shift with respect to a reference point. |
| ZTE | Proposal 9: To enabling the UL synchronization for frequency, network should periodically broadcast the amount of frequency compensation in a 12 bit field when DL frequency pre-compensation is applied on service link |
| Xiaomi | Proposal 4: Pre-compensation value for DL frequency should be indicated by network. |
| Sharp | Proposal 1: Unless we have further evidence the UE PSS/SSS search on initial access takes an unacceptable amount of time, perhaps more than “a few” seconds, greatly increases UE complexity, or is otherwise shown to be seriously detrimental, we propose UE post-compensating for fD,NU, and avoid gNB/gateway or serving satellite pre-compensation for fD,NU. In this way, we avoid the signaling issues involved here as well as any implementation complexities for fD,NU pre-compensation . |
| NTT DOCOMO, INC. | Proposal 5: DL doppler shift partial pre-compensation on service link at NW side is supported at least for high frequency band. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 11 For earth-moving cells, proving a fixed value of DL frequency compensation on the service link at the NW side shall be supported to enhance UL frequency pre-compensation at the UE side. Proposal 12 For quasi-earth-fixed cells, leveraging a fixed reference location of a cell agreed in RAN2 for indicating DL frequency compensation on the service link at the NW side shall be supported. |

## Companies views

On issue 11, the detail on signalling is still remained to be discussed.

Only few companies provided some inputs on the signalling of the amount of frequency compensation in DL service link:

[ZTE] proposed that network should periodically broadcast the amount of frequency compensation in a 12 bit field when DL frequency pre-compensation is applied on service link.

[Thales] observed If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters and at least 19 bits are needed to indicate the amount of frequency compensation and associated drift rate.

According to [Oppo, CATT] it is preferential to consider the granularity of frequency compensation in kHz.

[Huawei] The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing.

[Thales and MediaTek ] proposed to deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

Based on the above the initial proposal is made as follows:

Initial Proposal:

Discuss whether to deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | DL frequency compensation for the service link Doppler in Release 17 can be de-prioritized. One benefit is that not applying DL frequency compensation for the service link Doppler removes the Doppler discontinuity. This simplifies the AFC tracking algorithm in LEO system with smaller beams. If DL frequency compensation for the service link Doppler is not applied, the synchronization algorithm convergence times are still reasonable. |
| Intel | In our view there is no need to make agreement on de-posterization at this stage. |
| Samsung | Share the same view with Intel. |
| Xiaomi | We think it is no need to deprioritize this. |
| LG | Agree with Intel and Samsung. |
| Nokia, Nokia Shanghai Bell | We believe DL frequency compensation for the service link Doppler should be supported in Rel-17 and the amount of pre-compensation should be indicated to the UE. This feature will prevent the UEs from having to deal with a very large frequency offset in DL, which would add complexity to the DL synchronization and potentially delay the initial access procedure. |
| ZTE | Fine to not indicate the value, if the intention is to align the nominal frequency and improve the uplink pre-compensation accuracy, this part can be discussed together with UL post-compensation together and the usage including value of the indicated parameters are needed to be discussed. |
| Panasonic | We do not think that there is a need to deprioritize this now. |
| Apple | We do not think de-prioritization is needed. |
| Ericsson | We are fine with deprioritizing. |
| Lockheed Martin | We should not de-prioritize this issue at this stage. |
| Lenovo/MM | We think DL frequency pre-compensation for service link shouldn’t be de-prioritized, otherwise, new SSB design may be necessary, which may lead to large spec effort. |
| Huawei, HiSilicon | We share similar view with ZTE. |

## Updated proposal based on company views (First round of email discussions)

## Updated proposal based on company views (2nd round of email discussions)

# Issue#12: Indication of common compensation frequency offset for Uplink

The necessity of network indication of common frequency offset on UL was not clear to many companies because: It should be further discussed whether the post compensation is for the access link or feeder link:

-If the post compensation is for the feeder link. And if the working assumption on Doppler shift over the feeder link is agreed there is no need to indicate the post compensation.

-If the compensation is for the service link: It was recommended to use UL frequency compensation indication during release-16 NR NTN study Item under the assumption that a UE without GNSS capability cannot estimate residual Doppler. With GNSS capability, there is no need for UL frequency compensation indication if the UE pre-compensation of Doppler is done with sufficient accuracy.

The companies proposals on this issue are collected in the following table:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 8: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment. Proposal 9: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment. |
| PANASONIC | Proposal 6: The post-compensation value can be included in common frequency shift relative to the UE frequency source and indicated in SIB to all UEs. |
| Spreadtrum Communications | Proposal 6: Indication of common compensation frequency offset for uplink is not supported. |
| Apple | Proposal 9: UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions, and additional network indication of frequency offset to be pre-compensated by UE is not supported. |
| Samsung | Proposal 12: The gNB indicates the additional UL frequency offset value. |
| Xiaomi | Proposal 5: The residual offset value of UL frequency at the reference point should be indicated by network. |

## Companies views

Few companies provided input on this topic.

[Huawei, Apple, Spreadtrum Communications] proposed  to not indicate post-compensation. Only DL pre-compensation indication is needed and sufficient for UL frequency alignment[Huawei].

Companies supportive of the post-compensation indication: [ PANASONIC, Samsung].

According to [Xiaomi] The residual offset value of UL frequency at the reference point should be indicated by network.

Based on the above the Initial proposal is made as follows:

Initial Proposal 12:

Indication of common post-compensation frequency offset for Uplink is not needed

Companies are invited to provide their comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| Intel | Support the proposal |
| OPPO | We think whether or not to indicate a common frequency offset is up to gNB decision. RAN1 should support the function that the gNB wants UE to compensate a common frequency offset. |
| Samsung | If “common” refers to the feeder link, then ok. |
| Xiaomi | We think indication of common post-compensation frequency offset for Uplink can assistant pre-compensation frequency offset for uplink in the UE side. |
| LG | Support |
| CMCC | In our view, if common frequency post-compensation for Uplink is not supported by the gNB, indication of such common post-compensation frequency offset for Uplink is not needed. |
| Nokia, Nokia Shanghai Bell | Indication of a common frequency offset for UL can be beneficial in case of GNSS loss. |
| ZTE | This part can be discussed together with DL pre-compensation together and the usage including value of the indicated parameters are needed to be discussed. |
| Panasonic | The post-compensation value can be included in the common frequency offset relative to the UE frequency source. We prefer to explicitly indicate the employed value to all UEs. |
| Apple | Support the proposal |
| Qualcomm | Support |
| Ericsson | We support the proposal. |
| Lenovo/MM | Support the proposal. |
| Huawei, HiSilicon | Our views is that is the DL pre-compensation and UL post-compensation can be done with the same value, there is no need to signal both values. But it is still needed if they are different. |
| Spreadtrum | Support the proposal |

## Updated proposal based on company views (First round of email discussions)

Updated Proposal 12:

Indication of common post-compensation frequency offset for Uplink is not needed

## Updated proposal based on company views (2nd round of email discussions)

# Issue#13: Close control loop for UL frequency alignment

It was recommended to use UL frequency compensation indication during release-16 NR NTN study Item under the assumption that a UE without GNSS capability cannot estimate residual Doppler. With GNSS capability, there is no need for UL frequency compensation indication if the UE pre-compensation of Doppler is done with sufficient accuracy.

### [TR38.821] : For the UL frequency compensation, at least for LEO system, the following solutions are identified with consideration on the beam specific post-compensation of common frequency offset at the network side: ● Option-1: Both the estimation and pre-compensation of UE-specific frequency offset are conducted at the UE side. The acquisition of this value can be done by utilizing DL reference signals, UE location and satellite ephemeris. ● Option-2: The required frequency offset for UL frequency compensation at least in LEO systems is indicated by the network to UE. The acquisition on this value can be done at the network side with detection of UL signals, e.g., preamble

In RAN1#103e, it was agreed to adopt Option-1 discussed in [TR38.821]:

Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

Since RAN1#104-e, this issue was discussed and some companies want to consider option-2 in [TR38.821] based on a closed loop uplink frequency control.

However, several companies do not see the need for UL frequency control loop. The Moderator recommended for companies willing to support UL frequency control loop should provide more details on the benefits of such mechanism. And the following recommendation was made after the first round of email discussions:

FL recommendation : RAN1 to further investigate the needs and benefits to support closed-loop UL frequency compensation for GNNS equipped NR NTN UE

Closed-loop frequency control was discussed within 4contributions submitted to RAN1#105-e:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 10: For GNSS UE, closed-loop UL frequency compensation is not needed. |
| Spreadtrum Communications | Proposal 7: Close control loop for UL frequency alignment is not needed. |
| Qualcomm | Proposal 3: Support closed-loop frequency control commands by MAC-CE. Proposal 4: Consider group-common DCI for UL time and frequency control. |
| Nokia | Proposal 19: Study whether closed-loop control for UEs in RRC connected mode should be enabled for UE frequency alignment in UL in case of GNSS loss. |

## Companies views

Few companies provided inputs to RAN1#105e on the support of Closed control loop.

Closed-loop UL frequency compensation is not needed according to [Huawei, Spreadtrum Communications]

According to [Qualcomm]  there will be scenarios where CL UL frequency control would be needed due to UE movement and infrequent GNSS reading. Therefore, [Qualcomm] prosed to support closed-loop frequency control commands by MAC-CE and to consider group-common DCI for UL time and frequency control.

[Nokia] proposed to further study whether closed-loop control for UEs in RRC connected mode should be enabled for UE frequency alignment in UL in case of GNSS loss.

In  Moderator’s view with GNSS capability, there is no need for closed control loop as long as the UE pre-compensation of Doppler shift is done with sufficient accuracy.

Based on the above, the following proposal is made as follows:

Initial Proposal 13

RAN1 to further investigate the need and benefit to support closed-loop UL frequency compensation

Companies are invited to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Fine to further investigate need and benefit. |
| OPPO | If the feeder link frequency offset is handled by the gateway, there is no need for close loop UL frequency compensation. |
| Samsung | We think the additional closed loop compensation is needed for fine tuning similar to TA. |
| Xiaomi | Ok |
| LG | Support. And it should be better the “FL recommendation” not “proposal” |
| Nokia, Nokia Shanghai Bell | We support investigating this further in RAN1. Closed-loop UL frequency synchronization can be beneficially for the case of GNSS loss at the UE. |
| ZTE | No need to introduce this mechanism. And such assumption is considered as basic benefits during the SI when we take the GNSS based solution as baseline |
| Panasonic | OK |
| Apple | We do not think closed-loop UL frequency compensation is needed, with the working assumption that the feeder link Doppler shift is transparent to UE. |
| Ericsson | Ok |
| Lockheed Martin | Agree with the proposal. |
| Lenovo/MM | We don’t think close-loop frequency compensation is necessary. |
| Huawei, HiSilicon | Closed-loop UL frequency compensation is not needed. UE will track the DL frequency frequently which will ensure the accuracy of UL frequency compensation. |
| Spreadtrum | We think closed-loop UL frequency compensation is not needed. |

## Updated proposal based on company views (First round of email discussions)

FL recommendation 13

RAN1 to further investigate the need and benefit to support closed-loop UL frequency compensation

## Updated proposal based on company views (2nd round of email discussions)

# Issue#14: Serving satellite ephemeris format

In RAN1#104-bis-e, the following agreement was made:

Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported

The issue on implicit or explicit epoch time for serving satellite ephemeris was discussed in RAN1#104bis-e but no agreement was made.

Based on discussion during RAN1#104bis-e meeting, it seems that the implicit epoch time is preferred by many companies.

The principle of the implicit epoch time for serving satellite ephemeris is shown in the Figure below:

- Step 1: On board GNSS-based measurements of the satellite position and velocity are made and dated.

- Step 2: The measurements are collected in the NTN Control Center (NCC) where the satellite Orbit Determination (OD) is performed.

- Step 3: The satellite OD is performed in the NCC. This operation can be more or less complex depending on the models considered, the quantity of measurements available and the algorithms used.

- Step 4: Satellite orbit prediction is performed and ephemeris data with explicit Epoch time is shared with GW and gNB.

- Step 5: gNB broadcast the ephemeris data periodically. Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast by considering the reference point for epoch time on the satellite. The UE, based on its GNSS can estimate the ageing of ephemeris data corresponding to one way satellite-UE propagation delay.

On Issue#14, the following proposals were submitted to RAN1#105-e:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 11: At least Satellite ephemeris format based on the following orbital elements is supported - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time t0 |
| THALES | Proposal 8: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast |
| PANASONIC | Proposal 7: We propose to adopt both satellite ephemeris formats, orbital elements and PVT format. |
| CATT | Proposal 1: Ephemeris format based on satellite position and velocity state vectors and Ephemeris format based on orbital elements can be supported both for different scenarios. Proposal 2: Support on-demand finer orbital parameters indication with UE specific way. |
| MediaTek Inc. | Proposal 10: Epoch time is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast. Proposal 11: Support satellite ephemeris format bit allocations - Position and velocity state vector ephemeris format (16 bytes payload). - The field size for position [m] is 78 bits - The field size for velocity [m/s] is 54 bits - Orbital parameter ephemeris format (18 byte payload) - Semi-major axis α [m] is 33 bits - Eccentricity e is 19 bits - Argument of periapsis ω [rad] is 24 bits - Longitude of ascending node Ω [rad] is 21 bits - Inclination i [rad] is 20 bits - Mean anomaly M [rad] at epoch time to is 24 bits Proposal 12: Satellite ephemeris orbital is used for long-term prediction of satellite position for UE wake up from idle DRX for next satellite fly-by. |
| CMCC | Proposal 16: Support both ephemeris formats based on satellite position and velocity state vectors (Set 1) and based on orbital elements (Set 2), if possible. Proposal 17: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Set 1) should be supported for implicit compatibility to support HAPS and ATG scenarios. Proposal 18: Support implicitly indication of epoch time for ephemeris information |
| OPPO | Proposal 5: There is no necessary to pre-configure ephemeris information in advance. Proposal 6: Delay derived from the ephemeris information of two notifications should be within half CP. Proposal 7: Both ephemeris formats should be supported. |
| Ericsson | Proposal 17 RAN1 to discuss how to define the reference point for epoch time if implicit epoch time for satellite ephemeris is used. Proposal 18 RAN1 to discuss the impact of implicit epoch time on gNB. Proposal 19 RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation with accuracy specified by RAN4. |
| Nokia | Proposal 20: Include at least the true anomaly at epoch t0 (or equivalent) parameter as part of the Set2 parameters and consider it as delta correction parameter which needs to be updated and signalled more frequently compared to the other Set2 parameters. Proposal 21: Keep both Set1 and Set2 as method to provide satellite ephemeris data to the UEs, in order to allow energy efficient UE IDLE and RRC CONNECTED mode procedures. Proposal 22: The Set1 and Set2 satellite ephemeris data may have different update rates. Proposal 23: The epoch time is explicitly indicated along with ephemeris data. |
| Apple | Proposal 10: RAN1 to discuss the field size for each ephemeris parameter after receiving RAN4 response on timing synchronization requirements. Proposal 11: Support both ephemeris formats, i.e., state vectors and orbital elements. |
| ZTE | Proposal 9: At least, the indication of PV information of serving node should be supported. Proposal 10: For the indication of set-1 parameters set, following methods can be considered to further optimize the signaling load:  Only indicating the limited location region for serving node  Indicating the relative location of serving node Proposal 11: For the indication of set-2 parameters set, following methods can be considered to further optimize the signaling load:  Use delta correction in the form of PVT.  The epoch time should be indicated in implicit way. Proposal 12: The upper bound of periodicity for satellite information updates should be less than 60s or 30s based on the defined TA error range from RAN4 (e.g., CP/2 and Te), respectively. |
| Samsung | Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings: • index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type • satellite altitude offset • satellite position • satellite velocity • reference time for satellite position and velocity. Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors, i.e., Set 1. |
| LG Electronics | Proposal 9. Regarding satellite ephemeris format, • Support one set to reduce both specification work and UE implementation complexity. • If two sets need to be supported, only one set should be mandatory feature. Proposal 10. Support validity timing window for satellite ephemeris information in Rel-17 NTN. |
| InterDigital, Inc. | Proposal-4: For ephemeris format, both formats are supported. Proposal-5: Option 1 is used for GEO deployment and Option 2 is used for LEO deployment. |
| NTT DOCOMO, INC. | Proposal 6: Support both of the following two sets - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris |

## Company views

- w.r.t FFS: whether down-selection is needed or both sets are supported:

Different views were expressed within the contributions submitted to RAN1#105-e:

[Huawei]: At least support set 2

[ZTE, Samsung]: At least support set 1

[PANASONIC, CATT, MediaTek, CMCC, OPPO, Nokia, Apple, InterDigital, Thales, NTT DOCOMO] : Both set 1 and set 2.

LG Electronics support one set to reduce both specification work and UE implementation complexity.

The initial proposal is made as follows:

Initial Proposal 14-1:

- Support both ephemeris formats, i.e., state vectors and orbital elements.

- Companies are encouraged to provide their comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Intel | Support the proposal |
| OPPO | OK |
| Samsung | OK |
| LG | As mentioned in our contribution, we prefer to support single set to reduce both specification work and UE implementation complexity. Nevertheless, if lots of companies in this group think that above sets must be supported, we prefer to support the one set should be the mandatory feature and the other set should be the optional feature. Otherwise, every NTN UE must implement at least two different orbital propagator models. |
| CMCC | OK. Nevertheless, if down-selection is finally needed, ephemeris format based on satellite position and velocity state vectors (Set 1) should at least be supported for implicit compatibility to support HAPS and ATG scenarios. |
| Nokia, Nokia Shanghai Bell | We see beneficial aspects of both sets. Set 1 would be good to have for the short term information, while Set 2 would be beneficial for the UE to extrapolate the satellite ephemeris information into the future position(s) of the satellite. |
| Panasonic | Agree |
| Apple | Support |
| Qualcomm | Support |
| Ericsson | We are fine with the proposal. |
| Lockheed Martin | Agree with the proposal. |
| Lenovo/MM | Support the proposal. |
| InterDigital | Support |
| Huawei, HiSilicon | Agree. |
| NTT DOCOMO, INC. | Support |

- w.r.t epoch time to be used for the ephemeris data :

There were few proposals on this topic in the Tdocs submitted to RAN1#105-e:

- [Nokia] proposed to adopt explicit Epoch time indication :

According to [Nokia]  the most natural choice is to use the time reference currently used in GNSS satellite systems (e.g. as epoch in seconds within GNSS week). This time reference can then be linked to 3GPP signalling time reference (SFN number) via dedicated broadcast information (SIB9).

- [THALES, MediaTek, CMCC, ZTE] are supportive of implicit Epoch time indication:

[MediaTek] observed that signaling of epoch time on NTN SIB requires a payload of approximately 5 bytes.

[Ericsson] proposed for RAN1 to discuss how to define the reference point for epoch time if implicit epoch time for satellite ephemeris is used. And observed: if implicit epoch time is used, the gNB may need to know the absolute transmission time of the subframe containing the ephemeris data with an accuracy of 0.6 ms, assuming that the impact of the epoch time inaccuracy on the UE transmission time error should not exceed 5% of the shortest CP length. [Ericsson] then proposed for RAN1 to discuss the impact of implicit epoch time on gNB.

Based on the views expressed in companies’ contributions:

Initial Proposal 14-2:

- Epoch time is implicitly known as a reference time linked to DL slot where the SIB carrying satellite ephemeris is broadcast.

- The reference point for epoch time is satellite.

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Intel | Support the proposal |
| Samsung | Support |
| LG | We prefer to support a simple method, i.e., explicit signalling. So, in this meeting, we prefer to list feasible options (i.e., explicit signalling and implicit signalling), and it can be down-selected the one of options in the next meeting. |
| CMCC | Support |
| Nokia, Nokia Shanghai Bell | We do not agree to this proposal. In our preference, the epoch time is explicitly indicated along with the ephemeris information. Allowing to have explicit indication of epoch time allows for simpler implementation at the gNB, while at the same time offering the possibility to provide information on satellite position into the future. |
| Panasonic | Agree |
| Apple | Support |
| Qualcomm | Support |
| Ericsson | RAN1 should first analyse the impact on gNB and potential ephemeris inaccuracy due to mismatch between explicit epoch time and implicit epoch time (i.e., transmission time of the SIB). May depend on outcome of NCC-gNB interface discussion in RAN3. |
| Lenovo/MM | Support the proposal. |
| Huawei, HiSilicon | Agree. |

- w.r.t Payload to be used for  ephemeris data indication:

The field size for each ephemeris parameter is till FFS.

[ZTE] observed  that the basic principle should be that the quantization error will not significantly affect the pre-compensation accuracy of UE specific TA and Doppler.

[MediaTek] proposed to broadcast ephemeris data in 16 bytes payload in case of Position and velocity state vector ephemeris format  and in 18 byte payload in case of orbital parameter ephemeris format.

[Apple] proposed for RAN1 to discuss the field size for each ephemeris parameter after receiving RAN4 response on timing synchronization requirements.

The initial proposal is made  follows:

Initial Proposal 14-3:

RAN1 to discuss Payload to be used for  ephemeris data indication after RAN4 response to the LS on timing synchronization requirements

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Support proposal |
| Xiaomi | Support the proposal |
| CMCC | Support |
| Nokia, Nokia Shanghai Bell | Ok to wait to discuss the ephemeris payload, but we do not see this as strongly coupled to the RAN4 response on the timing synchronization requirements. |
| ZTE | Support |
| Panasonic | Agree |
| Apple | Support |
| Qualcomm | Support |
| Ericsson | Ok |
| Huawei, HiSilicon | Agree. |

## Updated proposal based on company views (First round of email discussions)

Based on the first round email discussion the following updated proposal are made:

Updated Proposal 14-1:

- Support both ephemeris formats, i.e., state vectors and orbital elements.

Updated Proposal 14-2:

- Epoch time is implicitly known as a reference time linked to DL subframe where the SIB carrying satellite ephemeris is broadcast.

- The reference point for epoch time is satellite.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#15: Time and Frequency synchronization requirements

RAN4 answered partially to RAN1 questions on this issue. RAN4 response can be found in  R4-2106174 LS reply on NTN UL time and frequency synchronization requirements. And it is captured hereafter:

RAN4 would like to thank RAN1 for the LS on NTN UL time and frequency synchronization requirements. During RAN4#98bis-e meeting, RAN4 had extensive discussion on these 2 issues and would like to provide the following feedbacks to RAN1. Regarding question 2 on frequency synchronization, the following UE RF frequency error requirement is concluded in RAN4 for both initial access and RRC connected state: - The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ±0.1 ppm observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the gNB. - RAN4 is still investigating whether there are any intra-gNB related aspects to consider associated with the above UE requirement Note: The gNB refers to RAN3 NTN architecture. Regarding question 1 on time synchronization, RAN4 discussion is still on-going. Additional LS will be sent once there is an agreement in RAN4.

However, in the LS Reply it is not clearly defined the exact percentage of Doppler shift pre-compensation to be taken into account in the budget. This is useful to determine .

Therefore, RAN1 to further clarify the NTN UL frequency synchronization requirements related to NTN UE Doppler pre-compensation.

Initial Proposal 15-1:

RAN1 to discuss how to define the budget considered for the tolerated error on NTN UE Doppler pre-compensation:

WF 1:

The NTN UE Doppler pre-compensation shall be accurate to within x% value of ±0.1 ppm as observed over a period of 1 ms by the gNB. The value of x% is for FFS.

Note: The gNB refers to RAN3 NTN architecture.

WF 2:

Send LS to RAN4 to enquire the budget considered for the tolerated error on NTN UE Doppler pre-compensation.

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | RAN4 already sent an LS for frequency requirement. RAN4 is still discussing TA. We can wait for RAN4 decision. If needed, we can take WF 2. |
| Nokia, Nokia Shanghai Bell | Requirements would rather belong to RAN4. |
| ZTE | Up to RAN4 |
| Ericsson | It is not RAN1 responsibility to define this budget but we are open to discuss to help determine the design. An LS to RAN4 will likely not help. |
| Huawei, HiSilicon | Agree with others that this is a RNA4 issue. |

Moreover, w.r.t timing synchronization requirement : The issue is still under discussion in RAN4 and additional LS will be sent to RAN1 once there is an agreement in RAN4. But according to RAN4 WF on timing requirements for NR NTN captured in R4-2105796 and recalled in section 6.1

In addition during RAN4#98-bis-e, it was also agreed

RAN4#98-bis-e Agreements: - The UE specific TA estimation accuracy is counted into the UE transmit timing error requirement - UE specific TA estimation accuracy is FFS - FFS whether the UE specific TA estimation accuracy shall be also defined as a separate accuracy requirement - Specify UE behavior related to UE specific TA estimation and the detailed behavior is FFS

However, this agreement has to take into account NTA,common and not only UE specific TA estimation (i.e. NTA,UE-specific). And it is not crystal-clear that RAN4 will define UE specific TA estimation (NTA,UE-specific) and self-estimated TA common (NTA,common) accuracy as a separate accuracy requirement.

The initial proposal 5-2 is made as follows:

Initial Proposal 15-2:

RAN1 to discuss whether to define as a separate accuracy requirement

- UE specific TA estimation (NTA,UE-specific) accuracy

- Common TA (NTA,common) accuracy

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| MediaTek | Wait for RAN4 response to the LS on timing synchronization requirements |
| Intel | In our understanding requirements should be discussed by RAN4. |
| Samsung | Wait for RAN4 conclusion |
| Xiaomi | Wait for RAN4 response on timing synchronization requirements. |
| LG | Wait for RAN4 discussion. |
| Nokia, Nokia Shanghai Bell | Requirements would rather belong to RAN4. |
| ZTE | It’s up to the on-going RAN4’s decision |
| Panasonic | Wait for RAN4 |
| Apple | We may wait for RAN4 response on timing synchronization requirements. |
| Ericsson | This should be decided by RAN4. |
| Huawei, HiSilicon | Agree with others that this is a RNA4 issue. |

## Updated proposal based on company views (First round of email discussions)

## Updated proposal based on company views (2nd round of email discussions)

# Conclusion

# References

- R1-2104256	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

- R1-2104301	Considerations on UL timing and frequency synchronization in NTN	THALES

- R1-2104315	Enhancements on UL time and frequency synchronization	PANASONIC R&D Center Germany

- R1-2104425	Consideration on enhancements on UL time and frequency synchronization for NTN	Spreadtrum Communications

- R1-2104517	UL time and frequency synchronization for NTN	CATT

- R1-2105952	Enhancements on UL Time and Frequency Synchronisation for NR-NTN	MediaTek Inc.

- R1-2104608	Enhancements on UL time and frequency synchronization for NTN	CMCC

- R1-2104668	UL time and frequency synchronization for NTN	Qualcomm Incorporated

- R1-2104722	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

- R1-2104771	Discussion on UL time and frequency synchronization	OPPO

- R1-2104811	On UL time and frequency synchronization enhancements for NTN	Ericsson

- R1-2104828	Discussion on time and frequency synchronization for NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2105102	Uplink Time and Frequency Synchronization for NR NT	Apple

- R1-2105165	Considerations on UL time synchronisation	Sony

- R1-2105190	Discussion on UL synchronization for NR-NTN	ZTE

- R1-2105214	Discussion on NTN uplink time synchronization	Lenovo, Motorola Mobility

- R1-2105272	Discussion on UL Time Synchronization for NTN	Fraunhofer IIS, Fraunhofer HHI

- R1-2105307	Enhancements on UL time and frequency synchronization for NTN	Samsung

- R1-2105478	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

- R1-2105560	Discussion on UL time and frequency synchronization for NTN	Xiaomi

- R1-2105634	Enhancements on frequency synchronization for NTN	Sharp

- R1-2105668	UL time/frequency synchronization for NTN	InterDigital, Inc.

- R1-2105698	Discussion on UL time and frequency synchronization enhancements for NTN	NTT DOCOMO, INC.

- R1-2105812	On satellite pass predictions under NTN discontinuous coverage	Sateliot, Gatehouse, ESA, Kepler

- R1-2105821	UL time and frequency synchronization in NTN	Asia Pacific Telecom, FGI, ITRI, III

- R1-2105947	UL time synchronization acquisition for NTN	Mitsubishi Electric RCE

# Appendix I: RAN1 agreements on UL time and frequency synchronization for NR NTN

RAN1 agreements on UL time and frequency synchronization for NR NTN achieved in RAN1 Meeting #104-bis-e: Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e. Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported Conclusion: The orbital propagator model to be used at UE side can be left to implementation. RAN1 Meeting #104-e (e-Meeting, January 25th – February 5th, 2021): Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control Conclusion: It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation Agreement: RAN1 should send an LS to RAN4 with the following questions: Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. - FFS: support of DL frequency compensation for the service link Doppler. Agreement: - RAN1 to support satellite ephemeris broadcast based at least on one of the following format options: - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format - FFS: Whether down-selection is needed or both options are supported RAN1 Meeting #103-e (e-Meeting, October 26th – November 13th, 2020): Agreement: An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. Agreement: An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: - In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset[+X]×Tc[+X] Where: NTA is derived from the User specific TA self-estimation X is derived at least from the common timing offset value if broadcasted by the network. The granularity of X and whether X is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. - NTA, offset depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. - Tc is specified in TS 38.211 section 4.1. - Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Working assumption: It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. RAN1 Meeting #102-e (e-Meeting, August 17th – 28th, 2020): Agreement: • In Rel-17 NR NTN, at least support UE which can derive based on its GNSS implementation one or more of: o its position o a reference time and frequency • And, based on one or more of these elements together with additional information (e.g., serving satellite ephemeris or timestamp) signalled by the network, can compute timing and frequency, and apply timing advance and frequency adjustment at least for UE in RRC idle/inactive mode. • FFS: Details on additional information signalled from network Agreement: In case of GNSS-assisted TA acquisition in RRC idle/inactive mode, the UE calculates its TA based on the following potential contributions: • The User specific TA which is estimated by the UE: o Option 1: The User specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network:  FFS: Details on serving satellite ephemeris indication o Option 2: The User specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network • The Common TA if indicated by the network: o FFS: The need and details of Common TA indication • FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty)

# Appendix II: Summary of proposals

| TDoc | Source | Proposals |
| --- | --- | --- |
| R1-2104256 | Huawei, HiSilicon | Proposal 1: The delay compensated by the gNB should consider the gNB implementation complexity. Proposal 2: The common TA is determined as the RTD from the reference point to the satellite, i.e. by subtracting the delay compensated at the gNB from the feeder link RTD. Proposal 3: NTA,common can be a positive or negative value to support the corresponding reference point at both service link and feeder link. Proposal 4: Support indication of the location of reference point as an alternative solution for common TA indication. Proposal 5: At least support indicating a common TA drift rate as part of the common TA applied by UE. Proposal 6: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission. Proposal 7: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing. Proposal 8: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment. Proposal 9: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment. Proposal 10: For GNSS UE, closed-loop UL frequency compensation is not needed. Proposal 11: At least Satellite ephemeris format based on the following orbital elements is supported - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time t0 |
| R1-2104301 | THALES | Proposal 1: The granularity of Common TA is equal to 4∙64/2^μ*Tc Proposal 2: The Network may periodically broadcast: - in case of GEO based non-terrestrial access network: - Common TA in a field of 23 bits - in case of LEO/MEO based non-terrestrial access network: - Common TA in a field of 21 bits - Common TA drift rate in a field of 13 bits - Common TA drift rate variation in a field of 6 bits Proposal 3: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements Proposal 4: Existing N_TA update in RRC_CONNECTED based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change Proposal 5: A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA calculation. - This timer is restarted each time the UE receives new assistance information. - The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available. Proposal 6:The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. Proposal 7: Deprioritize support of DL frequency compensation for the service link Doppler in Release 17. Proposal 8: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast |
| R1-2104315 | PANASONIC R&D Center Germany | Proposal 1: We propose a granularity of the common TA value NTA,common in the order of slot or half slot duration. Proposal 2: The common TA value NTA,common is always indicated in SIB. - FFS: the value range of NTA,common Proposal 3: We propose to allow for a non-zero DL-UL timing difference managed by the network. - FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation. Proposal 4: In RRC_CONNCTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation. Proposal 5: The Doppler compensation on the feeder link should be transparent to the UE. Proposal 6: The post-compensation value can be included in common frequency shift relative to the UE frequency source and indicated in SIB to all UEs. Proposal 7: We propose to adopt both satellite ephemeris formats, orbital elements and PVT format. |
| R1-2104425 | Spreadtrum Communications | Proposal 1: NTA, common should be expressed in the legacy granularity of Tc units. Proposal 2: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Proposal 3: Existing NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change. Proposal 4: Negative TA Command in RAR is not needed when TA margin is considered. Proposal 5: The gateway and satellite-payload compensate common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. Proposal 6: Indication of common compensation frequency offset for uplink is not supported. Proposal 7: Close control loop for UL frequency alignment is not needed. |
| R1-2104517 | CATT | Proposal 1: Ephemeris format based on satellite position and velocity state vectors and Ephemeris format based on orbital elements can be supported both for different scenarios. Proposal 2: Support on-demand finer orbital parameters indication with UE specific way. Proposal 3: In order to save signaling overhead, common TA and common timing drift will be equal to 0 if not indicated. Proposal 4: The granularity of common TA can apply 1024 Tc or 2048Tc of step size. Proposal 5: TA margin can be one fixed value or not needed at all if UL timing requirement of RRC-IDLE mode and RRC-connected mode for UE is same. Proposal 6: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Proposal 7: On the close-loop and open-TA combination, UE can stop autonomous TA compensation or subtract the accumulated TA compensated by autonomous TA compensation during the gap between two neighboring TAC commands. Proposal 8: Common Doppler shift compensation value of DL and UL for service link can be indicated to UE, which can be defined with Khz granularity. Proposal 9: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a transparent way at least to the UE, whether compensated by the gNB is up to RAN4 decision. |
| R1-2105952 | MediaTek Inc. | Proposal 1: The UE acquires UE-gNB RTT as the sum of UE-satellite RTT and Satellite-gNB RTT, where - UE-satellite RTT is calculated by UE by using its position and the satellite ephemeris - Satellite-gNB RTT is acquired by the UE as broadcast by gNB. Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field - Common delay drift rate variation NTA,common,drift,thrird_order,derivative in a 6 bit field Proposal 3: In RRC_CONNECTED state: - NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change. - NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris - NTA,common is updated autonomously by the UE based on common TA parameters broadcast on SIB in case reference point for DL-UL subframe timing alignment is at the gNB. Proposal 4: NTN UE time alignment timer for re-acquisition of the satellite ephemeris on NTN SIB is configured by the network. Proposal 5: Before it can be discussed whether there is a need for a validity timer configured for common TA tracking tracking, it is needed to discuss the aspects that are relevant for common TA tracking. Proposal 6: The common TA parameters can be signalled on the same SIB with the PV/ephemeris information, in which case the validity timer if used can be common to both information. Proposal 7: Handle TA Reporting proposals under A.I. 8.4.1 Proposal 8: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. Proposal 9: Deprioritize support of DL frequency compensation for the service link Doppler in Release 17. Proposal 10: Epoch time is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast. Proposal 11: Support satellite ephemeris format bit allocations - Position and velocity state vector ephemeris format (16 bytes payload). - The field size for position [m] is 78 bits - The field size for velocity [m/s] is 54 bits - Orbital parameter ephemeris format (18 byte payload) - Semi-major axis α [m] is 33 bits - Eccentricity e is 19 bits - Argument of periapsis ω [rad] is 24 bits - Longitude of ascending node Ω [rad] is 21 bits - Inclination i [rad] is 20 bits - Mean anomaly M [rad] at epoch time to is 24 bits Proposal 12: Satellite ephemeris orbital is used for long-term prediction of satellite position for UE wake up from idle DRX for next satellite fly-by. Proposal 13: The lowest level of knowledge in network of when a UE will be in coverage of a satellite is the time when the UE last accessed the satellite cell. Proposal 14: The network should page the UE at the right time when - UE enters active period or idle DRX; - UE is within coverage. |
| R1-2104608 | CMCC | Proposal 1: The Network may periodically broadcast: - Common TA - Common TA drift rate - Common TA draft rate with value of 0 is supported Proposal 2: The granularity of Common TA is set to be the same as the granularity of NTA, i.e., 16∙642μ∙Tc. Proposal 3: Update relative values of Common TA to reduce the signaling overhead. Proposal 4: The common TA is determined as NTA,commont=NTA,common, t0+TAcommondrift, t0∙(t-t0+δt0) where: t is the time for targeted UL transmission at UE’s clock. t0 is the time for receiving common TA signaling at UE’s clock. NTA,common, t0 is the common TA (in 16∙642μ∙Tc units) at t0. TAcommondrift, t0 is the common TA drift rate. δt0 is used to pre-compensate the ageing of NTA,common, t0values due to the long propagation delay in NTN. and, NTA,common, t0=A+X where: A represents the synthetical impact of time-invariant parameters, e.g., TA margin and the minimum RTT on the feeder link. X are time-variant parameters, and they need to be updated with time. Proposal 5: Indication of parameter A (a time-invariant offset of NTA,common, t0) should be supported. - For DL and UL aligned at satellite, A can be used to absorb TA margin. - For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link. Proposal 6: Indication of parameter X at least should be supported. - For DL and UL not aligned at satellite (including aligned at gNB), X should be indicated and updated to capture the rapidly changed RTT on the feeder link. Proposal 7: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE. Proposal 8: For an NR NTN UE in RRC_CONNECTED, regular update of NTA,common can be disabled. Proposal 9: For an NR NTN UE in RRC_CONNECTED, TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc where NTA,common0 is the common TA obtained in initial access procedure. Proposal 10: The legacy MAC CE TA command and the enhanced MAC CE TA command consists of TA,δ,TA,drift to be both supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Proposal 11: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as NTAti=NTAaccumm+TA,driftm∙ti-tm' where, - m is the last received enhanced TA command before ti. - NTAaccumm is the m-th accumulated TA command. - Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0 Proposal 12: RAN1 to further study the details of UE reporting TA related information to facilitate network updating K_offset after initial access. The following reporting information can be further studied. - Option 1: UE location. - Option 2: UE self-estimated TA (NTA,UE-specific). - Option 3: the difference value (in unit of slot) between K_offset signaled in system information and a UE specific K_offset as suggest by UE. Proposal 13: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. A fixed common TA can be additional broadcasted, if needed. Proposal 14: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. Proposal 15: Support indication of DL frequency compensation for the service link Doppler, i.e., - If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. Proposal 16: Support both ephemeris formats based on satellite position and velocity state vectors (Set 1) and based on orbital elements (Set 2), if possible. Proposal 17: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Set 1) should be supported for implicit compatibility to support HAPS and ATG scenarios. Proposal 18: Support implicitly indication of epoch time for ephemeris information |
| R1-2104668 | Qualcomm Incorporated | Proposal 1: Network may broadcast a common timing offset value, X, with granularity of one slot assuming the subcarrier spacing of the SSB of the cell. - NTA,common is derived from X. Proposal 2: The common Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. Proposal 3: Support closed-loop frequency control commands by MAC-CE. Proposal 4: Consider group-common DCI for UL time and frequency control. Proposal 5: Support SRS with multiple coherent symbols with configurable gaps. |
| R1-2104722 | CAICT | Proposal1: Support to adopt cell-specific or beam-specific NTA,common for TA pre-compensation by the UEs without capability of location acquisition. Proposal2: FFS the condition when to apply the TA through combined closed loop and open loop |
| R1-2104771 | OPPO | Proposal 1:TA margin is no longer needed to be discussed. Proposal 2: Further study the acceptable SIB reading frequency for a UE. Whether reading SIB information every 3 seconds is acceptable when only NTA,common,drift,rate is broadcasted. Proposal 3: TA updating by MAC-CE or RAR can have the following relationship NTA_new = NTA_old + adjustment, where for TA updated by MAC-CE, the adjustment is and TA is the TAC indication in the range of [0, 63]; for TA updated by RAR, the adjustment is and the TA is the RAR TAC indication in the range of [0, 3846]. Proposal 4: If the frequency of reading SIB information every 3 seconds is acceptable, the network indicated common TA can be updated by a drift rate, e.g. NTA,common_new= NTA,common_old + NTA,common,drift,rate * t. Proposal 5: There is no necessary to pre-configure ephemeris information in advance. Proposal 6: Delay derived from the ephemeris information of two notifications should be within half CP. Proposal 7: Both ephemeris formats should be supported. Proposal 8: Two timers are introduced to ensure the accuracy of NTA,common and NTA,UE-specific. Proposal 9: It is preferential to consider the granularity of frequency compensation in kHz. Proposal 10: Indication the amount of frequency compensation should base on the SCS. |
| R1-2104811 | Ericsson | Proposal 1 RAN1 to discuss how to take into account satellite movement during DL signal propagation from satellite to UE when defining the UE-specific TA. Proposal 2 RAN1 to discuss how to take into account satellite movement during UL signal propagation from UE to satellite when defining the UE-specific TA. Proposal 3 RAN1 to discuss how to take into account satellite movement during signal propagation between gateway and satellite when defining the common TA. Proposal 4 The reference point for time and frequency in an NTN should be under control of the network and should at least support the option of having gNB as the reference point. Proposal 5 The closed-loop component of the TA, NTA, should be defined as in Rel-16. Proposal 6 The fixed TA offset, NTA,offset, should be defined as in Rel-16. Proposal 7 The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA. Proposal 8 The network broadcasts parameters describing the (one-way) common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay that may be implicitly (derived from DL subframe) or explicitly signaled Dcommon, base is the common one-way delay at time Tcommon,base Dcommon, drift is the common one-way delay drift rate Proposal 9 The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite. Proposal 10 The characterization of the common delay should include drift rate information. Proposal 11 The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length. Proposal 12 If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. Proposal 13 The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link. Proposal 14 RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE. Proposal 15 Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. Proposal 16 RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage. Proposal 17 RAN1 to discuss how to define the reference point for epoch time if implicit epoch time for satellite ephemeris is used. Proposal 18 RAN1 to discuss the impact of implicit epoch time on gNB. Proposal 19 RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation with accuracy specified by RAN4. |
| R1-2104828 | Nokia, Nokia Shanghai Bell | Proposal 1: Any UE should only attempt to access the 5G system over NTN for situations where it is absolutely sure that proper time and frequency compensation is applied. Proposal 2: The cyclic prefix of the random access preamble must be able to cover the aggregate contribution of all sources of time inaccuracy and multipath propagation delays. Proposal 3: The GNSS-assisted pre-compensation solution used by the UE shall meet the demands of the preamble format chosen by the operator. The UE shall ensure that requirements in TA adjustment and frequency pre-compensation for all preamble formats are met at any time. Proposal 4: Self-estimated UE-specific TA in RRC idle or inactive mode based on GNSS-provided time reference in conjunction with the time provided by referenceTimeInfo-R16 is independent on satellite ephemeris and should be standardized as well. Proposal 5: There is no need to indicate a TA margin. Any uncertainty related to TA should be covered by the Common TA value and the CP of the random access preamble. Proposal 6: The Common TA value is provided along with ephemeris information. Proposal 7: It should be left up to the gNB to decide whether and how often to broadcast the Common TA drift rate. Proposal 8: It is not needed to provide the Common TA drift rate variation. Proposal 9: Open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages. Proposal 10: For RRC connected mode, self-estimated UE-specific TA estimation based on GNSS-provided time reference and the time provided by referenceTimeInfo-R16 should be standardized as well. Proposal 11: A validity timer configured for both UE-specific TA and Common TA defining the maximum time during which the UE can track the RTD on service link and Common RTD without having acquired new ephemeris or assistance information shall be used. - This validity timer is restarted each time the UE receives new ephemeris and assistance information to be used for Common TA estimation. - The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris and assistance information is not available. Proposal 12: RAN2 to study whether the TA timer can be also used as a validity timer for open-loop update of both UE-specific TA and Common TA. Proposal 13: The location and determination of the frequency reference point must be agreed before developing further solutions for open technical aspects in NTN standardization. Proposal 14: The Doppler shift over the feeder link and any transponder frequency error in both DL and UL are compensated by the satellite subsystem in a way transparent at least to the UE and gNB. Proposal 15: In the downlink a common frequency offset on service link is pre-compensated to limit the UE search space for the synchronization signals. Proposal 16: The amount of common frequency pre-compensation in DL in a cell may be indicated to the UE and thereby be used for determining the amount of UL frequency pre-compensation. Proposal 17: A common signaling should be used to indicate the amount of applied frequency pre-compensation in DL for both earth-moving and earth-fixed cells. Proposal 18: UE frequency adjustment based on GNSS-acquired frequency reference, DL signals and the time provided by referenceTimeInfo-R16 has benefits and should be standardized as well. Proposal 19: Study whether closed-loop control for UEs in RRC connected mode should be enabled for UE frequency alignment in UL in case of GNSS loss. Proposal 20: Include at least the true anomaly at epoch t0 (or equivalent) parameter as part of the Set2 parameters and consider it as delta correction parameter which needs to be updated and signalled more frequently compared to the other Set2 parameters. Proposal 21: Keep both Set1 and Set2 as method to provide satellite ephemeris data to the UEs, in order to allow energy efficient UE IDLE and RRC CONNECTED mode procedures. Proposal 22: The Set1 and Set2 satellite ephemeris data may have different update rates. Proposal 23: The epoch time is explicitly indicated along with ephemeris data. |
| R1-2105102 | Apple | Proposal 1: The broadcast of common TA drift rate is based on network implementation. If common TA drift rate is not indicated, then its default value is 0. FFS common TA drift variation rate. Proposal 2: The broadcast common TA is in the unit of Tc⋅(16⋅64/2μ). Proposal 3: The network-controlled common TA NTA, common is obtained by NTA, common=NTA,common, RX +DTA, common,RX ⋅Δt, where NTA,common, RX is the latest received common TA, DTA,common, RX is the latest received common TA drift rate (if indicated) and Δt is the time gap between the latest common TA transmission and the corresponding uplink transmission. Proposal 4: RAN1 to determine the necessity of specifying TA margin, depending on RAN4’s conclusion. Proposal 5: The requirement that the existing TAC 12-bit field in Msg2/MsgB is reused is that a UE pre-compensates an accurate UE specific TA and TA margin in its Msg1/MsgA transmission. Proposal 6: RAN1 to study whether or not introducing the TA drift rate in TA command MAC CE. Proposal 7: Confirm the working assumption that the gateway and satellite-payload compensate common Doppler shift/Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to UE and gNB. Proposal 8: In downlink transmissions, support gNB pre-compensates and indicates a frequency offset for the service link Doppler shift with respect to a reference point. Proposal 9: UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions, and additional network indication of frequency offset to be pre-compensated by UE is not supported. Proposal 10: RAN1 to discuss the field size for each ephemeris parameter after receiving RAN4 response on timing synchronization requirements. Proposal 11: Support both ephemeris formats, i.e., state vectors and orbital elements. |
| R1-2105165 | Sony | Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs. Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE. Proposal 3: RAN1 should support the signalling of timing drift rate information to the UEs in a beam specific manner. Proposal 4: UE should update the common TA with common timing drift rate when UE transmits uplink data. |
| R1-2105190 | ZTE | Proposal 1: At least the first order common TA drift rates should be supported along with common TA. Proposal 2: Following solutions can be considered to further optimize the overhead of signaling: - Indication of second order common TA drift rate can also be considered to further optimize signaling overhead. - Indication of multiple set of {common TA, 1st order TA drift rate } with different applicable timing, - FFS: whether to include the 2nd order TA drift rate Proposal 3: The signaling granularity of common TA can be chosen as. Proposal 4: In RRC_CONNECTED state, the applied TA can be determined as by assuming the support for the 2nd order of TA drift: , where - is the service link TA at activation time of ephemeris, and is the variation of service link TA between activation time of ephemeris and UL transmission time, which can be estimated by UE based on GNSS and indicated information. - is the broadcast common TA, and are the first and second order common TA drift rate indicated by BS, is the interval between activation time of broadcast common TA and DL transmission time of scheduling information, and is the interval between DL transmission time of scheduling information and UL transmission time. - is indicated in MAC CE TA command. Proposal 5: When common TA and ephemeris are broadcast, indication of a close loop TA command can be considered to handle possible overcompensation in RRC_CONNECTED state. Proposal 6: Indication of on the valid time for satellite ephemeris data is preferred instead of UE-specific TA. Proposal 7: Configuration on the validity timer should be supported for common TA and common TA drift rate, and it will be started/restarted when a common TA or common TA drift rate is activated. Proposal 8: Explicitly defined GNSS measurement gap should be supported with the benefits to ensure the accuracy for UL pre-compensation and interference mitigation. Proposal 9: To enabling the UL synchronization for frequency, network should periodically broadcast the amount of frequency compensation in a 12 bit field when DL frequency pre-compensation is applied on service link. Proposal 10: At least, the indication of PV information of serving node should be supported. Proposal 11: For the indication of set-1 parameters set, following methods can be considered to further optimize the signaling load: - Only indicating the limited location region for serving node - Indicating the relative location of serving node Proposal 12: For the indication of set-2 parameters set, following methods can be considered to further optimize the signaling load: - Use delta correction in the form of PVT. - The epoch time should be indicated in implicit way. Proposal 12: The upper bound of periodicity for satellite information updates should be less than 60s or 30s based on the defined TA error range from RAN4 (e.g., CP/2 and Te), respectively. |
| R1-2105214 | Lenovo, Motorola Mobility | Proposal 1: Support the granularity of NTA,common to be same as granularity of TA command. Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE. Proposal 3: TA margin indication is not supported. Proposal 4: Support indication of common timing offset drift rate and indication of drift rate variation. Proposal 5: Support indication of TA validity timer value in the TA indication message |
| R1-2105272 | Fraunhofer IIS, Fraunhofer HHI | Proposal 1: RAN1 to strive for a unified design of common RTT/delay signaling in order to avoid duplicate signaling of common RTT/delay. Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding signaling. Proposal 3: RAN 1 should agree on studying common delay signaling design with reduced signaling overhead. Proposal 4: RAN1 to consider the signaling of the parameter set NTA,scale, drift scale rate, NTA,cons, a constant, and NTA,power, an exponent (can take integer values), for signaling of the common RTT/delay in NTN. Proposal 5: RAN1 to consider UE autonomous drift rate calculation for common RTT/delay signaling. |
| R1-2105307 | Samsung | Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings: index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type satellite altitude offset satellite position satellite velocity reference time for satellite position and velocity. Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors, i.e., Set 1. Proposal 3: The following options are considered for N_(TA,common) and N_TA. Option 1 N_(TA,common): its update is supported by group common signalling. N_TA: Defined as the same as Rel-15/16 Option 2 N_(TA,common): broadcasted by SIB. N_TA: Defined as the same as Rel-15/16 Option 3 N_(TA,common): broadcasted by SIB with drift rate information. N_TA: Defined as the same as Rel-15/16. Proposal 4: RAN1 discusses the granularity after concluding the signaling and update of N_(TA,common) and N_TA. Proposal 5: A mechanism to align T_TA between the gNB and the UE is adopted, e.g., a UE reports N_(TA,UE-specific) when it updates N_(TA,UE-specific) or the gNB triggers the UE to update N_(TA,UE-specific) with a granularity of 16∙64∙T_c/2^μ. Proposal 6: Each of the following options are supported based on the gNB configuration: Closed-loop TA control Open-loop TA control Combination of open&closed-loop TA control Proposal 7: UE’s estimated TA value is reported to gNB by MAC CE. Proposal 8: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information. Proposal 9: Multiple reference points and common TA values should be considered for extremely large cells. Proposal 10: The gNB signals common TA drift rate to enable autonomous TA update at UE. Proposal 11: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa. Proposal 12: The gNB indicates the additional UL frequency offset value. |
| R1-2105478 | LG Electronics | Proposal 1. Support additional signaling by the network in order to apply proper common TA according to time changes at UE side. Potential solutions can include Alt 1) providing the reference time corresponding to common TA Alt 2) providing series of common TA Proposal 2. Within pre-defined set of TA offsets, the TA offset can be provided by gNB via higher layer signing (e.g., SIB or dedicated RRC signaling). The TA offset can be independently corresponding to different ROs (or RO groups) Proposal 3. Support implicit reporting of UE specific TA estimated by the UE. The different TA (or the range of TA) can be mapped to different ROs (or RO groups). Proposal 4. At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling. In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling. Proposal 5. Support separate validity timers for common TA and UE specific TA in Rel-17 NTN. Proposal 6. RAN1 should discuss how to report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states. Proposal 7: Define the appropriate TA control method for each TA as follows: Closed loop TA control is applied to common TA (i.e., N_(TA,common)) Open loop TA control is applied to UE specific TA (i.e., N_(TA,UE-specific)) Proposal 8: Support independent TA update for each TA using independent TA control methods. Proposal 9. Regarding satellite ephemeris format, Support one set to reduce both specification work and UE implementation complexity. If two sets need to be supported, only one set should be mandatory feature. Proposal 10. Support validity timing window for satellite ephemeris information in Rel-17 NTN. |
| R1-2105560 | Xiaomi | Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred. Proposal 2: When the common TA is not broadcasted by network, the UE should assume that NTA,common=0 . Proposal 3: The common timing drift rate indicated by network should be supported. Proposal 4: Pre-compensation value for DL frequency should be indicated by network. Proposal 5: The residual offset value of UL frequency at the reference point should be indicated by network. Proposal 6: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE. |
| R1-2105634 | Sharp | Proposal 1: Unless we have further evidence the UE PSS/SSS search on initial access takes an unacceptable amount of time, perhaps more than “a few” seconds, greatly increases UE complexity, or is otherwise shown to be seriously detrimental, we propose UE post-compensating for fD,NU, and avoid gNB/gateway or serving satellite pre-compensation for fD,NU. In this way, we avoid the signaling issues involved here as well as any implementation complexities for fD,NU pre-compensation . |
| R1-2105668 | InterDigital, Inc. | Proposal-1: support to introduce an additional parameter to indicate feeder link RTT separately from the common TA. Proposal-2: support the reporting of the UE-specific TA after initial access. Proposal-3: a PRACH resource set is configured per distance group from the UE to satellite and a UE determine a PRACH resource set for Msg1 transmission based on the distance group the UE belongs to. Proposal-4: For ephemeris format, both formats are supported. Proposal-5: Option 1 is used for GEO deployment and Option 2 is used for LEO deployment. |
| R1-2105698 | NTT DOCOMO, INC. | Proposal 1: Support NW broadcast of common TA drift rate. Proposal 2: UE capability on UE-specific TA self-estimation/pre-compensation is defined to indicate the self-estimation accuracy. Proposal 3: One or multiple common TAs are broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy. Proposal 4: The doppler shift over the feeder link and any transponder frequency error for both downlink and uplink are compensated in a way transparent to UE. Proposal 5: DL doppler shift partial pre-compensation on service link at NW side is supported at least for high frequency band. Proposal 6: Support both of the following two sets - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris Proposal 7: It is not necessary to broadcast the location of reference point and define two types of calculation methods for TA and doppler shift. |
| R1-2105812 | Sateliot, Gatehouse, ESA, Kepler | Observation 1: When estimating at the IoT device the start of next satellite passes using simple orbit propagators and instantaneous orbital elements broadcast by the satellite, prediction errors could be in the order of ~20 seconds for prediction windows of ~12 hours, up to ~230 seconds for prediction windows of ~84 hours. Observation 2: Prediction errors have to be compared to almost two minutes of visibility time at 500 km altitude. At higher altitudes the prediction error would remain comparable but with longer visibility times. |
| R1-2105821 | Asia Pacific Telecom, FGI, ITRI, III | Proposal 1 Legacy timing advance command (TAC) for NTA and NTA,offset in the case of RAR is the baseline for NTN. No specification change is needed to minimize MAC impacts, e.g., TAC MAC CE format. Proposal 2 Legacy timing advance (TA) adjustment for NTA and NTA,offset in RRC_CONNECTED is the baseline for NTN. No specification change is needed to minimize MAC impacts, e.g., TA timers, and groups. Proposal 3 The granularity of common TA is set to be the same as the granularity of NTA, i. e. 16∙64/2μ*Tc. Proposal 4 The common TA configuration shall be cell specific via system information and the configuration can be associated with a group of cells to save signaling overhead. Proposal 5 To minimize spec impacts and signaling overhead, association between the existing TAG and the group of cells for the common TA configuration shall be considered. Proposal 6 Support TA margin for a PRACH transmission in case of Contention-based Random Access (CBRA) to prevent the use of negative TAC in RAR. Proposal 7 Support common TA drift rate broadcasted in system information to reduce signaling overhead. Proposal 8 For RACH, no validity timer for common TA is needed, if common TA is provided in SIB1, and if common TA update triggers SI change indication, then UE shall update common TA by re-acquiring SIB1 after receiving a short message in its paging occasion. Proposal 9 For RRC_CONNECTED, no validity timer for common TA is needed if common TA is provided in SIB1 and if common TA update triggers SI change indication. Proposal 10 No validity timer for UE-specific TA is needed if a GNSS measurement gap is supported for the L band. NW and UE shall share a common understanding of when to update GNSS and ephemeris. Proposal 11 For earth-moving cells, proving a fixed value of DL frequency compensation on the service link at the NW side shall be supported to enhance UL frequency pre-compensation at the UE side. Proposal 12 For quasi-earth-fixed cells, leveraging a fixed reference location of a cell agreed in RAN2 for indicating DL frequency compensation on the service link at the NW side shall be supported. |
| R1-2105947 | Mitsubishi Electric RCE | Proposal 1: UEs using self-acquired timing advance for initial access signal the pre-compensation value NTA to the network during the initial access process. Proposal 2: In NTN, the network broadcasts a beam-specific default value NTA_default of the user-specific NTA value for initial access purposes. Proposal 3: Support network assistance indicating to the UE whether to skip timing advance acquisition during handover. |