3GPP TSG RAN WG1 #105-e		R1-2104384

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Other enhancements to Rel-17 IAB nodes

Agenda Item:	8.10.3

Document for:	Discussion and Decision

- Introduction

New WID [1] on Enhancements to Integrated Access and Backhaul has been approved and there is one objective as following.

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]: - Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including: - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx). - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing. - Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we provide our view on some critical points which is relevant but not explicitly included in WID, i.e., PRACH resource selection issue.

- Discussion

  - PRACH occasion selection

In Rel-16, IAB specific PRACH resource can be configured via RACH-ConfigCommonIAB besides the normal PRACH resource configured via RACH-ConfigCommon. IAB specific PRACH resource is used only by IAB-node, while normal PRACH resource can be used by both IAB-node and UE. The motivations of IAB specific PRACH configuration include: a) addressing the issue that IAB node cannot transmit and receive simultaneously on a single PRACH resource, i.e., half-duplex constraint; b) longer PRACH format can be configured via RACH-ConfigCommonIAB, which is beneficial for child IAB-node to access parent IAB-node in case of long geo-distance between IAB-nodes.

Furthermore, if both rach-ConfigCommonIAB and rach-ConfigCommon are acquired by an IAB-node, the IAB-node will prioritize the PRACH resource derived from rach-ConfigCommonIAB catering to the aforementioned motivations. However, such behavior will incur large latency for IAB-node to access parent node, since period of IAB specific PRACH occasion is usually long for saving resource as the number of IAB nodes in the coverage of a macro cell is not large and an IAB node is assumed to be always connected when it is in operation. The large radio access latency is problematic, especially for the case of radio link re-establishment after radio link failure between child and parent IAB-nodes.

Observation 1: When PRACH resources configured by both rach-ConfigCommonIAB and rach-ConfigCommon are available, Rel-16 child IAB-node will prioritize the PRACH resource configured by rach-ConfigCommonIAB for radio access purpose.

Observation 2: Always using the PRACH resources configured by rach-ConfigCommonIAB once rach-ConfigCommonIAB is configured may incur large random access latency for an IAB-node.

Firstly, even with half constrain restriction, it does not mean that all IAB node cannot use the normal PRACH resources configured by rach-ConfigCommon. In practice, it should be the case that many IAB nodes can still have the opportunity to access normal PRACH resources when the resource allocation for these IAB nodes does not conflict with at least some of the PRACH occasions. Secondly, in Rel-17, the aforementioned half-duplex constraint will be alleviated somehow, since Rel-17 IAB node will support simultaneous TX and RX. Thirdly, an IAB node initiates from idle states does not have half-duplex constraint at all. Moreover, the distance between not all IAB-nodes have far distance to its candidate parent IAB-node such that the PRACH resource configured by rach-ConfigCommonIAB has to be used, which means that some IAB-node should be still able to access the candidate parent IAB-node via normal PRACH occasions. Hence, it is believed to be beneficial for an IAB-MT to have the flexibility to select either the IAB specific PRACH resources or the normal PRACH resources, thus radio link re-establishment/initial access latency will be reduced. Based on the above analysis, the following proposal is made

Proposal 1: Support a PRACH occasion selection mechanism, which gives the child IAB-node sufficient flexibility to use PRACH resources configured by both RACH-ConfigCommonIAB and rach-ConfigCommon.

Proposal 2: Specify a mechanism for an IAB node to select between the PRACH resources configured by rach-ConfigCommon and that configured by rach-ConfigCommonIAB.

- Conclusion

This contribution focus on IAB power sharing mechanism in dual-connectivity scenarios and PRACH resource selection issue, and the following are proposed,

Observation 1: When PRACH resources configured by both rach-ConfigCommonIAB and rach-ConfigCommon are available, Rel-16 child IAB-node will prioritize the PRACH resource configured by rach-ConfigCommonIAB for radio access purpose.

Observation 2: Always using the PRACH resources configured by rach-ConfigCommonIAB once rach-ConfigCommonIAB is configured may incur large random access latency for an IAB-node.

Proposal 1: Support a PRACH occasion selection mechanism, which gives the child IAB-node sufficient flexibility to use PRACH resources configured by both RACH-ConfigCommonIAB and rach-ConfigCommon.

Proposal 2: Specify a mechanism for an IAB node to select between the PRACH resources configured by rach-ConfigCommon and that configured by rach-ConfigCommonIAB.

- Reference

- RP-193251, “New WID on Enhancements to Integrated Access and Backhaul”, RAN#86, Spain, December 2019.