3GPP TSG RAN WG1 #105-e		R1-2104442

e-Meeting, May 10th – 27th, 2021

Agenda Item:     8.12.1

Source:	Spreadtrum Communications

Title:	             Discussion on MBS group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and decision

Introduction

In this paper, we will present our opinions on MBS group scheduling for RRC_CONNECTED UEs.

Discussion

Group scheduling mechanism

In RAN1#102e meeting [1], we have agreed to support PTM1. PTP has been agreed in RAN2. Compared with PTM1, obviously PTM2 will bring in more PDCCH overhead and transmission delay. The benefit and use case for PTM2 are not clear to us, especially on top of PTM1.

Proposal 1: For RRC_CONNECTED UEs for NR MBS, not support PTM2 transmission scheme.

For PTM scheme 1, CORESET/Search space set related agreements have been achieved in RAN1#104b-e meeting [2].

Agreement: If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options. - Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. - Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following alternatives (to be decided in RAN1#105): - Alt 1: support Type-3 CSS - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS - Alt 2: support a new Type-x CSS - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS. - Alt 3: support both Alt 1 and Alt 2 Agreement: For group-common PDCCH of Rel-17 MBS, support at least two DCI formats. - DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI. - DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline - FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields

For the retransmission scheme for PTM1, it has been agreed to support PTM1 and PTP. However, it is not preferable to support PTP and PTM1 simultaneously. We have not seen the necessity. Either PTM1 or PTP is enough.

Proposal 2: If initial transmission for multicast is based on PTM transmission scheme 1, not simultaneously support PTM1 and PTP together as the retransmission scheme.

In Rel-15/Rel-16, CSS is used for fallback DCI scheduling common PDSCH, unicast PDCCH scheduling unicast PDSCH, and group common PDCCH without PDSCH, and USS is only for unicast PDCCH. On the other hand, in RAN1#103e, we have agreed that the CCE indexes are common for different UEs in the same MBS group. In addition, in RAN1#104e meeting, CSS has been supported for idle state. Thus, in our opinion, it is reasonable to use CSS for Rel-17 PTM1 for connected state.

In current specification, CSS set includes five types: Type0-PDCCH CSS set, Type0A-PDCCH CSS set, Type1-PDCCH CSS set, Type2-PDCCH CSS set, and Type3-PDCCH CSS set, where Type0/Type0A/Type 2 are only applicable for primary cell of the MCG, and Type1/Type3 when applied for scheduling are only applicable for primary cell.  For some MBS, e.g., video streaming, for the sake of load balance, they could be carried on Scell. Thus, in our opinion, one new CSS type, e.g., Type4 could be defined for Rel-17 MBS, which could be used for both Pcell and Scell.

Proposal 3: For search space type for Rel-17 MBS, support to define a new search space type for multicast.

In Rel-15, for PDCCH overbooking case, network would ensure CSS related PDCCH not to be dropped, while USS related PDCCH could be dropped based on USS ID.  Given that the priority of MBS service could be lower, equal, or higher than unicast, it is reasonable that the PDCCH corresponding to CSS type for MBS should be in the same level with USS related PDCCH, i.e., monitoring priority should be depending on search space index.

Proposal 4: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS.

Last meeting, it has been agreed that DCI format 1_0, and DCI format 1_1 or 1_2 are as the baseline of the first DCI format and second DCI format with CRC scrambled with G-RNTI respectively. Generally, in our opinion, DCI format 1_0 could be directly reused as the first DCI format with some enhancements. For example, for the ‘Frequency domain resource assignment’ field, if the first DCI format is scheduled in CSS, the bitwidth and interpretation depends on the CORESET configuration and CFR configuration for MBS in idle state; if the first DCI format is scheduled in USS, the bitwidth and interpretation depends on the CFR configuration for MBS in RRC connected. For ‘Time domain resource assignment’ field, the bitwidth and interpretation depends on the specific TDRA table for multicast MBS. Other fields seem to be unnecessary to be enhanced. For DCI format 1_1 or 1_2, traditionally it is UE specific DCI, and the payload size, the bitwidth and interpretation for each field are depending on UE specific configuration. For the second format, we also think DCI format 1_1 or 1_2 could be directly reused, but should be based on PDSCH-config for MBS. In detail, the bitwidth for each field in the DCI is common to all member UEs in a group and related to PDSCH-Config for MBS, and for each member UE, and each field could be interpreted in light of its specific configuration.

Proposal 5: For connected UE, DCI 1_0 can be directly reused as group-common PDCCH for MBS, where

- If DCI 1_0 is scheduled in CSS, then the bitwidth and interpretation of  ‘FDRA’ field depends on the CORESET configuration and CFR configuration for MBS in idle state

- If DCI 1_0 is scheduled in USS, then  the bitwidth and interpretation of  ‘FDRA’ field  depends on the CFR configuration for MBS in RRC connected

Proposal 6: For connected UE, DCI 1_1 or 1_2 can be directly reused as group-common PDCCH for MBS, where

- The bitwidth for each field in the DCI is common to all member UEs in a group, and

- For each member UE, each field could be interpreted  in light of its specific configuration

In RAN1#104e meeting, some agreements about SPS group-common PDSCH has been achieved for MBS for RRC_CONNECTED UEs [3].

Agreement: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. - FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

Regarding the retransmission scheme for SPS PDSCH, in general, it is up to gNB’s implementation. It could be unicast scheduling or group scheduling for retransmission. For example, if only a few of member UEs are not successful, unicast dynamic scheduling for retransmission could be adopted; if the majority are not successful, group scheduling could be considered. However, similar to PTM1 retransmission scheme proposed in proposal 1 above, simultaneously scheduling unicast and group-common retransmission shall be avoided.

Proposal 7: Not support simultaneously scheduling unicast and group-common retransmission for SPS group-common PDSCH.

Simultaneous reception of unicast and multicast

In RAN1#104b-e meeting [2], great progress has been achieved about TDMed multiplexing among unicast PDSCH and/or group-common PDSCH in a slot as shown below.

Agreement: At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

In Rel-15, up to 2, 4, 7 unicast PDSCHs per slot per CC only in TDM for different TBs could be supported based on UE capability FG5-11/5-11a/5-11b. The MBS WID has clearly stated that in order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided). Thus, for the above case 1~case 3, the number of TDMed PDSCHs including group-common PDSCH and/or unicast PDSSCH should not excess R15 UE capability. For example, if UE supporting FG5-11a, then M could be 2 or 3.

Proposal 8: The number of TDMed multiplexing group-common PDSCHs and unicast PDSCHs should not exceed R15 UE capability.

Conclusion

In this contribution, we provide our opinions on MBS group scheduling for RRC_CONNECTED UEs:

Proposal 1: For RRC_CONNECTED UEs for NR MBS, not support PTM2 transmission scheme.

Proposal 2: If initial transmission for multicast is based on PTM transmission scheme 1, not simultaneously support PTM1 and PTP together as the retransmission scheme.

Proposal 3: For search space type for Rel-17 MBS, support to define a new search space type for multicast.

Proposal 4: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS.

Proposal 5: For connected UE, DCI 1_0 can be directly reused as group-common PDCCH for MBS, where

- If DCI 1_0 is scheduled in CSS, then the bitwidth and interpretation of  ‘FDRA’ field depends on the CORESET configuration and CFR configuration for MBS in idle state

- If DCI 1_0 is scheduled in USS, then  the bitwidth and interpretation of  ‘FDRA’ field  depends on the CFR configuration for MBS in RRC connected

Proposal 6: For connected UE, DCI 1_1 or 1_2 can be directly reused as group-common PDCCH for MBS, where

- The bitwidth for each field in the DCI is common to all member UEs in a group, and

- For each member UE, each field could be interpreted  in light of its specific configuration

Proposal 7: Not support simultaneously scheduling unicast and group-common retransmission for SPS group-common PDSCH.

Proposal 8: The number of TDMed multiplexing group-common PDSCHs and unicast PDSCHs should not exceed R15 UE capability.

Reference

- Draft Report of 3GPP TSG RAN WG1 #102e

- Draft Report of 3GPP TSG RAN WG1 #104b-e

- Draft Report of 3GPP TSG RAN WG1 #104e