3GPP TSG RAN WG1 #105-e	                                      R1-2104509

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	PDSCH/PUSCH enhancements for up to 71GHz operation

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

The work item on supporting NR from 52.6 GHz to 71 GHz was update and approved in RAN#90-e meeting [1]; objectives of WI are listed in the following.

- Physical layer aspects including [RAN1]: - In addition to 120 KHz SCS, specify new SCS, 480 KHz and 960 KHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480 KHz to 960 KHz - Time line related aspects adapted to 480 KHz and 960 KHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120 KHz SCS for SSB and 120 KHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240 KHz, 480 KHz, 960 KHz) for SSB, and additional SCS (480 KHz, 960 KHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480 KHz, 960 KHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480 KHz and/or 960 KHz), study, and specify if needed, potential enhancement for shared spectrum operation - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120 KHz SCS, 480 KHz SCS and/or 960 KHz SCS, as well as DMRS enhancement for 480 KHz SCS and/or 960 KHz SCS.

In this document, issues on PDSCH/PUSCH enhancements for up to 71GHz operation are discussed, including time line related aspects and multi-PDSCHs/PUSCHs scheduled by a single DCI.

- Potential impacts of physical signal/channel with higher SCS

3.1 Timing issues for scheduling

The UE processing capability would increase along with the increase of the maximum system bandwidth or/and the increase of SCS for control and data channels. The UE processing time, such as PDCCH decoding, PDSCH processing and associated HARQ-ACK generation and feedback, would require additional investigation when 480 KHz and 960 KHz SCS are supported for control and data channels. A new set of UE capability needs to be specified as well for the processing time associated with the 480 KHz and 960 KHz SCS control and data channels.

In last meeting, how to derive the processing timelines was discussed, the main idea is the absolute time duration should be computed based on 120 KHz SCS and the value can be optimized according to the detail scenarios. The agreements reached are listed as following:

Agreement: - RAN1 use the absolute time duration for 120 KHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 KHz and 960 KHz SCS for NR operation in 52.6 to 71 GHz - RAN1 strives to reduce the absolute time durations from the upper bound if feasible - FFS: How to derive timeline values - Case by case study - FFS: model-based approach for selected timelines, e.g. exponential models, projection based on log-linear regression, etc. Agreement: - The following UE processing timelines are prioritized for discussion - PDSCH processing time (N1), PUSCH preparation time (N2), HARQ-ACK multiplexing timeline (N3) - configuration(s)/default values of k0 (PDSCH), k1 (HARQ), k2 (PUSCH) - CSI processing time, Z1, Z2, and Z3, and CSI processing units - Note: the order of the above sub-bullets represents the priority for discussion in descending order - Companies are encouraged to provide preferred values/ranges of timelines for discussion

- PDSCH processing time (N1) and PUSCH preparation time (N2)

In Rel-15, PDSCH processing time (N1) and PUSCH preparation time (N2) were discussed carefully and two levels   of UE processing capability are defined. Capability 2 is for UE with high processing capability, which is defined to support service with tight processing timeline, such as URLLC service. Capability 1 is for UE with general processing capability and service with moderate processing timeline, such as eMBB. The N1 values of capability 1 in Rel-15 are shown as follows.

Table 1: PDSCH processing time for PDSCH processing capability 1   [4]

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB (CASE1 ) | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured (CASE 2) |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

The N1 value is specified based on the UE processing time required to complete the PDSCH decoding of the maximum system bandwidth. The vast majority of time consumption for UE PDSCH processing is the LDPC decoding of code blocks. The maximum LDPC code block size is 8192 bit. The LDPC would be in pipeline processing when there are more than one LDPC code blocks. The UE processing time would depend on the number of code blocks within the maximum system bandwidth for each SCS. When the symbol duration is reduced for larger SCS, the fraction of processing time for RF/Baseband processing other than LDPC decoding would increase in terms of number of symbols. When the SCS is doubled, the symbol and slot duration are reduced in half. The N1 processing time in terms of number of OFDM symbols would require double the original time intuitively. However, the number of PRBs would be also reduced in half within the same maximum system bandwidth when the SCS is doubled. The number of code blocks would be roughly reduced in half when the number of PRBs is reduced in half. We can see the N1 processing capability in the table above that the processing time in term of OFDM symbols are not in the same range when SCS is doubled. From the table, the fraction of RF/baseband processing other than LDPC decoding increases when SCS increases. Since the maximum system BW increases in proportion to the increase of the SCS (maximum 275 PRBs), the number of LDPC code blocks remains the same for all SCS. The absolute time of LDPC decoding should be the same for all SCS if the maximum number of LDPC code blocks is the same.

The system bandwidth is expected to increase from 400 MHz with the introduction of 480 KHz and 960 KHz SCS.  For 480 KHz SCS, the maximum system bandwidth of 1600MHz was agreed, and the number of PRBs is the same as that of 120 KHz SCS with 400 MHz bandwidth. Comparing with the slot duration of 120 KHz SCS, the slot duration of 480 KHz SCS is only ¼ the time duration of that for 120 KHz SCS.  UE needs to set the sync signal from local oscillator for signal processing of 480 KHz SCS at the receiver at ¼ the duration of that of 120 KHz, which is equivalent to tune up the clock speed 4 times comparing to that of 120 KHz SCS. The relative processing speed for 480 KHz SCS is also expected to increase in order to process the received signals in time. The N1 value of 480 KHz SCS should be less than 4 times of N1 of 120 KHz SCS since UE has similar time with faster processing speed in channel decoding based on maximum system BW and UE can complete the signal processing in 1/4 of the time for 480 KHz SCS comparing to that of 120 KHz SCS. Thus, we can see that the number of symbols for N1 for 480 KHz SCS would be roughly around 30 symbols, which is about 1.5 times that of 120 KHz SCS.

For SCS=960 KHz, the maximum system bandwidth of 2000 or 2160MHz was agreed. Comparing to 480 KHz SCS, the slot duration of 960 KHz SCS is half of that of 480 KHz. The number of PRBs for 960 KHz SCS also does not double with maximum system BW at 2000 or 2160 MHz and is about 62.5% increase from that of 480 KHz SCS. The N1 value of 960 KHz SCS can be 1.25 times that of 480 KHz SCS. Therefore, the N1 value for 960 KHz SCS is around 52 symbols.

Similar analysis can be used for PUSCH preparing time line (N2), the preferred values/ranges of timelines of N1/N2 is shown in the following tables.

Table 2: PDSCH processing time arrange for PDSCH processing capability 1 for SCS=480/960 KHz

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 3 | 20 | 24 |
| 5 | 30 | 36 |
| 6 | 42 | 50 |

Table 3: PUSCH preparation time for PUSCH timing capability 1 for SCS=480/960 KHz

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 3 | 36 |
| 5 | 60 |
| 6 | 72 |

Proposal 1: Compared with the processing time of 120 KHz SCS, considering the maximum system bandwidth and maximum PRB number can be scheduled, N1/N2 is proposed as follows

- For SCS=480 KHz, the N1/N2 can be 1.5 times as 120 KHz N1/N2 value.

- For SCS=960 KHz, the N1/N2 can be 1.25 times as 480 KHz N1/N2 value.

- On k0, k1, k2 configuration(s)

It was identified that appropriate configuration(s) of k0 (PDSCH), k1 (HARQ), K2 (PUSCH) can be studied for supporting the new subcarrier spacing SCS=480 KHz/960 KHz.

K0 is defined as the number of slots from the scheduling DCI to the scheduled PDSCH transmission. K0 is included in the TDRA table for dynamic indication of PDSCH transmission. As specified in section 5.1.2 of [4], the range of k0 would not be affected by UE PDCCH monitoring capability if both same slot and cross slot scheduling are supported. The range of k0 value should not be changed for 480 KHz and 960 KHz SCS.

K1 is defined as the slot number from the end of scheduled PDSCH to the first symbol of PUCCH carrying HARQ-ACK feedback. The default value k1= {1, 2, 3, 4, 5, 6, 7, 8} will be used if UE is in idle mode or if RRC does not configure the k1 value. The range of k1 value shall match PDSCH processing time (N1). Generally, the time duration between the last symbol of PDSCH and the first symbols of PUCCH shall be more than N1. Otherwise the UE can’t feedback the HARQ-ACK information on the scheduled slot.

For operation of PDCCH and PUSCH with 480 KHz/960 KHz SCS, considering N1 will be increased as discussed above, the default k1 value for R15/R16 will need to shifted to accommodate large value of N1 processing time, as shown in the table below.

Table 4: Expected k1 default arrange for SCS=480 KHz/960 KHz

|  | Current default K1 | N1(symbol number) | N1(slot number) | invalid default K1 |
| --- | --- | --- | --- | --- |
| 5 | {1,2,3,4,5,6,7,8} | 36 | {3,4,5,6,7,8,9,10,11,12 } | {1,2} |
| 6 | {1,2,3,4,5,6,7,8} | 50 | {4,5, 6, 7, 8, 9, 10, 11} | {1,2,3} |

As shown in the table above, we assume that N1 processing time for 960 KHz SCS is 50 symbols (4 slots). The default value of k1= {1, 2, 3} does not meet the requirement of the processing time N1.

To resolve this problem, for SCS=480 KHz/960 KHz operation, k1_offset can be introduced. The value of k1_offset can be predefined. Considering the margin of PUCCH first symbol position, and the margin of PDSCH last symbol position (not always the first symbol or the last symbol of the slot), k1_offset can be predefined toN1/14, as shown in the table below.

Table 5: Expected k1 default value for SCS=480 KHz/960 KHz

|  | Current default K1 | N1(symbol/slot) | Maximum margin of PUCCH | K1_offset |
| --- | --- | --- | --- | --- |
| 5 | {1,2,3,4,5,6,7,8} | 36/ (2.6 slots) | 10 | N1/14=2 |
| 6 | {1,2,3,4,5,6,7,8} | 50/(3.5 slots) | 11 | N1/14=3 |

The range of k2 values shall match PUSCH preparation time (N2). The value of k2_offset should be sufficient large in order to cover the minimum PUSCH processing time N2 for 480 KHz and 960 KHz SCS. Considering the margin of PUSCH first symbol position (e.g. it's not always on the first symbol of slot), and considering the margin of DCI last symbol position (e.g.  it’s not always on the last symbol of slot), k2_offset can be predefined  to  N2/14. This k2 offset for 480 KHz and 960 KHz SCS can be applied in the following table in section 6.1.2 of [4] for PUSCH scheduling.

Table 6: Definition of value j [4]

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 5 | N2/14 |
| 6 | N2/14 |

Proposal 2:  The range of k1 value specified for PDSCH HARQ process operation for 480 KHz/960 KHz SCS should take the N1 processing time into account with the starting slot from the valueN1/14.

Proposal 3:  The range of k2 value specified for PUSCH process for 480 KHz/960 KHz SCS should take the N2 processing time into account with the starting slot from the valueN2/14.

3.2 PDSCH/PUSCH scheduling design

- On non-continuous in time-domain

- In RAN 1 #104b-e meeting, it was agreed that Alt 2 is used for TDRA of multiple PUSCHs/PDSCHs. This means the scheduling can be non-continuous or continuous in time-domain. The agreement is as follows.

Agreement: - For a DCI that can schedule multiple PUSCHs, - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signaled in DCI.), as per agreement made in RAN1#104-e - FFS: signaling details - Note: Alt 2 does not preclude continuous resource allocation in time-domain. - For a DCI that can schedule multiple PDSCHs, - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signaled in DCI. - FFS: signaling details - Note: This does not preclude continuous resource allocation in time-domain. - Note: Multi-PDSCH scheduling for the case of 120 KHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- For 480 KHz/960 KHz SCS operation, the maximum number of PUSCHs/PDSCHs can be up to 8. Each PUSCH/PDSCH has separate SLIV for supporting flexible scheduling, and the multiple PUSCHs/PDSCHs can be non-continuous in time-domain. Generally, the concept of non-continuous in time-domain can be based on slot level or symbol level. Here it is noted that “non-continuous in time-domain” in the agreement is referred to slot based level.

- The first issue needs to be clarified is whether supporting multiple PUSCH/PDSCHs to be transmitted in one slot. Considering allowing multiple PUSCH/PDSCH transmitted within one slot will result in a complicated feedback design, only one PUSCH/PDSCH shall be transmitted within one slot.

- Proposal 4: For multiple PDSCH/PUSCH scheduling, no more than one PUSCH/PDSCH shall be transmitted in one slot.

- The second issue is how to specify “non-continuous”. There are two possible options as follows.

- Option 1：Non-continuous time-domain allocation is indicated by invalid SLIV

- For each row index of TDRA, there is a common k0 parameter. The k0 indicates a difference of slot number between the scheduling PDCCH and the first scheduled PDSCH. The k0 of subsequent scheduled PDSCH is increased by 1. When any SLIV is configured as invalid, the corresponding slots will be skipped, and no PDSCH will be scheduled. When all SLIVs are valid, multiple PDSCHs are consecutive at the slot level. An example is shown as following table,

Table 7: An example of TDRA configuration by RRC

| Row index | K0 | SLIV1 | SLIV2 | SLIV3 | SLIV4 | SLIV5 | SLIV6 | SLIV7 | SLIV8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 |
| 1 | 1 | Invalid | Invalid | S=2;L=12 | Invalid | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 |
| 2 | 1 | S=2;L=12 | Invalid | Invalid | Invalid | S=2;L=12 | S=2;L=12 | S=2;L=12 | S=2;L=12 |

- As shown in the above table, a maximum number of 8 PDSCHs are configured by RRC in row index 0. This enables gNB to schedule up to 8 PDSCHs by a DCI. The PDSCHs are continuous in time-domain at slot level. In other words, if gNB would transmit 8 PDSCHs by a single DCI scheduling, those PDSCHs are only continuously transmitted in time-domain. When row index 1 is indicated by DCI, for the five PDSCHs scheduled by one DCI, the third PDSCH and 5th PDSCH are not continuous because SLIV4 is invalid. For row index 2，there are three blank slots between the first PDSCH and the 4th PDSCH since SLIV2/SLIV3/SLIV4 are all invalid.

- Option 2：Non-continuous time-domain allocation is indicated by non-continuous k0 value

- In each row index of TDRA, there is a common k0 parameter. The k0 indicates a difference of slot number between the scheduling PDCCH and the first scheduled PDSCH. For all the subsequent PDSCH, each PDSCH will have one additional value (e.g. offset) for computing the k0. When the k0 of adjacent PDSCHs is not continuous, the two adjacent PDSCHs are non-continuous in time domain. When the k0 of adjacent PDSCHs is same, it means that adjacent PDSCHs are scheduled in a same slot. An example is shown as following table.

Table 8: An example TDRA configured by RRC

| Row index | K0 | SLIV1 | SLIV2 | SLIV3 | SLIV4 | SLIV5 | SLIV6 | SLIV7 | SLIV8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | S=2;L=12 Offset=0 | S=2;L=12 Offset=1 | S=2;L=12 Offset=2 | S=2;L=12 Offset=3 | S=2;L=12 Offset=4 | S=2;L=12 Offset=5 | S=2;L=12 Offset=6 | S=2;L=12 Offset=7 |
| 1 | 0 | S=2;L=12 Offset=0 | S=2;L=12 Offset=1 | S=2;L=12 Offset=3 | S=2;L=12 Offset=5 | S=2;L=12 Offset=6 | S=2;L=12 Offset=7 | S=2;L=12 Offset=8 | S=2;L=12 Offset=9 |
| 2 | 0 | S=2;L=6 Offset=0 | S=8;L=6 Offset=0 | S=2;L=12 Offset=2 | S=2;L=12 Offset=3 | S=2;L=12 Offset=4 | S=2;L=12 Offset=5 | S=2;L=12 Offset=6 | S=2;L=12 Offset=7 |

- As shown in the table above, there are 8 PDSCHs configured by RRC in row index 0, and they are continuous in time-domain. When row index 1 is indicated by DCI, the 8 PDSCHs are scheduled by one DCI, and the third PDSCH and the fourth PDSCH are not continuous because the k0 corresponding to the PDSCHs are not consecutive. Note in this option, two PDSCHs transmitted in one slot can be supported, although this is not preferred choice.  In row index 2，the first PDSCH and the second PDSCH are scheduled in a same slot because they have same k0 value. Although option 2 may be more flexible in term of scheduling as each PDSCH has a separate k0, it will cause HARQ-ACK procedure to become really complicated, especially during type-0 HARQ-ACK codebook generation.

- Proposal 5: Non-continuous time-domain allocation is indicated by invalid SLIV value in the configuration.

- Collision with unavailable slot/symbols

For a DCI scheduling multiple PUSCHs/PDSCHs, the PUSCHs/PDSCHs may be transmitted for up to 8 consecutive slots. For unpaired spectrum operation, it could happen that one or more uplink slots exist within these consecutive slots on which PDSCHs cannot be transmitted. In order to resolve this issue for multiple PUSCHs/PDSCHs scheduling, there are three possible options.

Option 1：The row index is not allowed to be used if valid SILV is mapped to unavailable symbols/slot

In the TDRA list table, up to 16 row index(s) can be configured by RRC, and each row index has to up to 8 SLIVs. When uplink slot/symbols are defined for unpaired spectrum operation, the UE does not expect to be scheduled with any PDSCH (having valid SLIV value) corresponding this row index. This option will restrict the scheduling flexibility.

Option 2:  When the PDSCH is mapped to an unavailable slot, the corresponding SLIV value can be regarded as invalid

The option allows PDSCH (having valid SLIV value) map to an uplink slot/symbol, but when this occurs, both gNB and UE will regard the scheduled PDSCH as invalid, i.e., labeling the SLIV as invalid due to the collision with uplink slot/symbols. This option can provide flexible scheduling. At the same time, it is easy for UE to determine which PDSCH scheduled is invalid according to TDD DL/UL configuration. Also, this option will not affect Type-1 HARQ-ACK codebook generation.

Option 3: The PDSCHs are postponed until downlink slots are available.

When the scheduled PDSCH is mapped to uplink slots/symbols according to DCI, the PDSCH transmission shall be postponed until available downlink slot/symbols are obtained. The subsequent PDSCHs can be transmitted according to the available time-domain resource. This option will make sure that each scheduled PDSCH will be transmitted, but it will affect Type-1 HARQ-ACK codebook generation since this will cause increased complexity, especially for candidate PDSCH occasion determination.

Figure 1: Three options for handling collision with unavailable symbols

As discussed above, when there are unavailable slot/symbols for scheduled multiple PUSCHs/PDSCHs, option 2 is preferred.

Proposal 6: When the scheduled PDSCH/PUSCH overlaps with unavailable slots/symbols, the corresponding SLIV value can be regarded as invalid.

For multiple PUSCHs/PDSCHs scheduled by a single DCI, the HARQ process IDs for different PUSCHs/PDSCHs are also consecutive. The HPN of first PUSCH is indicated by the DCI, and the HARQ process ID is then incremented by 1 for each subsequent PDSCH/PUSCHs. When the scheduled PDSCH/PUSCHs include unavailable slots/symbols, the corresponding SLIV can be seen as invalid. Whether the HARQ process ID is still consecutive when one or more SLIVs value is invalid can be further discussed.

Proposal 7: Whether the HARQ process ID is still consecutive when one or more SLIVs value is invalid can be further discussed.

- On restrictions of 4 maximum PDSCHs for 480KHz SCS

- Previously RAN1 agreed the maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 KHz. It was also discussed whether restrictions for 480 KHz (lower the maximum PDSCHs to 4) is needed but no conclusion was reached.

- The total duration of 8 PDSCHs will be 0.125ms for SCS of 960 KHz, and this is equal to the duration of one slot of 120 KHz SCS. Therefore, UE can keep the same PDCCH blind detection capability as for operation with 120 KHz SCS. When the maximum number of PDSCHs is 8 for 480 KHz SCS, the total duration will be 0.25ms, this does not require a new capability for PDCCH blind detection. Moreover, the maximum number of PDSCHs scheduled by a DCI can be configured by RRC, so the actual value can be 4 or even less if it is necessary.

- Proposal 8: For SCS of 480 KHz, it is not needed to restrict the maximum number of PDSCHs to 4.

In Rel-15/16, fallback DCI (e.g. format 1_0/0_0) is defined and is used for scheduling common messages and UE specific data. The common messages include system information and paging information. UE specific data includes MSG2/MSG3/MSG4, RRC/NAS and traffic data. Generally, the format of fallback DCI shall be compatible to Rel-15/Rel-16 UE. Also, one TB can carry overload of SIB, paging, Msg 2/3/4, or RRC/NAS messages. So for the fallback DCI, multi-PDSCH/PUSH in one DCI is not supported.

Proposal 9:  Fallback DCI (e.g. format 1_0/0_0) does not support multi-PDSCH/PUSH transmission.

3.3 HARQ-ACK codebook enhancements

In last meeting, the enhancement on type-1 HARQ-ACK codebook was discussed, and three possible options are listed as follows.

Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

For supporting one DCI scheduling multiple PDSCHs, the gNB will configure TDRA list with one or more SLIV rows. Each row could include up to 8 SLIV parameters. According to the time-domain parameter, UE can compute the maximum configured number of PDSCHs N_max. The k1 set will be extended according to N_max. One example is shown in the following figure with assumption that k1 set ={3,4,5], and N_max =4.

Figure 2: Option 1 for candidate PDSCH reception occasion determination

As shown in the above figure, when the serving cell is configured as type-1 HARQ-ACK codebook, the extension k1 set will be k1 set + N_max-1= {3,4,5,6,7,8}. The maximum number of candidate PDSCH reception occasions is six when there is only one PDSCH received in a slot.  The loop number of determination for the candidate PDSCH reception occasions is increased to. When there is TDD DL/UL configuration, R15 rule can be reused to discard unavailable PDSCHs reception occasions.

For option 1, the maximum of candidate PDSCH reception occasions is computed for the worst case. If the row index 0 is used and k1 is indicated as 3 in DCI, the scheduling signals indicating k1=4/5 will not occur. Enhancement on discarding some candidate PDSCH reception occasions should be further studied.

Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table

For this option, k1 set will not change, i.e. the loop number of ‘while ’ for candidate PDSCH reception occasion determination is not increased. For each k1 value loop, UE shall determine the corresponding candidate PDSCH reception occasions  . The total number of candidate PDSCH reception occasions for the cell is equal to . One example is shown in the figure below with the assumption that k1 set = {3, 4, 5] and N_max =4.

Figure 3：Option 1a for candidate PDSCH reception occasion determination

As shown in the figure above, when the serving cell is configured as type-1 HARQ-ACK codebook, the invalid candidate PDSCH reception occasions shall be computed according to each row and each SLIV of configured TDRA. The maximum candidate PDSCH reception occasions is equal to, which is 12 in the example. N_max is the maximum configured number of PDSCHs.

If the option 1a is used, the efficiency of HARQ-ACK feedback will be affected. Also, enhancement mechanism for discarding candidate PDSCH reception occasions need be studied, especially for the UE which does not receive more than one unicast PDSCH per slot.

Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

For this option, k1 set will not change. The determination procedure of candidate PDSCH reception occasions in R15/R16 can be reused. The procedure of HARQ-ACK information bit generation for each candidate PDSCH shall be enhanced, such as one occasion needs N_max HARQ-AC information. The total number of HARQ-ACK information will be.  For this example, the total number of HARQ-ACK information is 12.

If option 2 is used, the efficiency of HARQ-ACK feedback is also low and enhancement mechanism for discarding feedback of HARQ-ACK information is needed, especially for the UE which does not receive more than one unicast PDSCH per slot.

Proposal 10: The enhancement mechanism for discarding candidate PDSCH reception occasions and HARQ-ACK feedback information shall be studied before determination on the options for Type-1 HARQ-ACK can be made.

For Type-2 HARQ-ACK, in last meeting, the enhancement on type-2 HARQ-ACK codebook was discussed and following three possible options are listed.

Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs

Alt-1 can reuse the R15/R16 procedure to generate Type-2 HARQ-ACK codebook without DAI field size increasing. When multi-PDSCH is configured for a serving cell, similar method as how 2nd sub-codebook is defined to handle CBG-based scheduling can be used. To reduce overhead of HARQ-ACK feedback, simultaneous configuration for both CBG-based scheduling and multi-PDSCH scheduling shall be avoided. When there is a lot of invalid SLIV parameter in each row of TDRA table, the feedback efficiency will become worse since the size of HARQ ACK information depends on the maximum configured number of PDSCHs.

Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs

Alt-2a will increase DAI field size. Including counter DAI and total DAI, the size of DAI can be modified as and this is good for at most 3 consecutive DCI missing cases. In total    bits are needed for the maximum number of PDSCHs scheduling case across different cells. This alternative is suit for the case when there are lots of invalid SLIVs in each row.

Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs

Alt-3 is a configurable scheme with regard to alt 1 and alt 2 with the introduction of the parameter M. If M equals to N_max, it is same as Alt 1. If M equals to 1, it is the same as Alt 2.

It is noted that the main motivation of introducing multiple PDSCHs scheduling is that PDCCH monitoring period is based on multiple slots, so most likely when gNB schedules PDSCH, it is expected that the number of PDSCHs will be the maximum number of PDSCH configured by RRC.  For low bit rate service such as VOIP, the gNB can schedule using fall back DCI, or reconfigure the number of PDSCHs that can be scheduled by a DCI.

Proposal 11:  Considering the motivation of introducing multiple PDSCHs scheduling is that PDCCH monitoring period is based on multiple slots, Alt-1 for Type-2 HARQ-ACK codebook generation is preferred.

- Conclusion

This contribution discussed the new waveform for the support of NR operation up to 71 GHz. We have the following proposals,

Proposal 1: Compare to processing time of 120 KHz SCS, considering the maximum system bandwidth and maximum PRB number can be scheduled, N1/N2 is proposed as follows

- For SCS=480 KHz, the N1/N2 can be 1.5 times as 120 KHz N1/N2 value.

- For SCS=960 KHz, the N1/N2 can be 1.25 times as 480 KHz N1/N2 value.

Proposal 2: The range of k1 value specified for PDSCH HARQ process operation for 480 KHz/960 KHz SCS should take the N1 processing time into account with the starting slot from the valueN1/14.

Proposal 3:  The range of k2 value specified for PUSCH process for 480 KHz/960 KHz SCS should take the N2 processing time into account with the starting slot from the valueN2/14.

- Proposal 4: For multiple PDSCH/PUSCH scheduling, no more than one PUSCH/PDSCH shall be transmitted in one slot.

- Proposal 5: Non-continuous time-domain allocation is indicated by invalid SLIV value in the configuration.

Proposal 6: When the scheduled PDSCH/PUSCH overlaps with unavailable slots/symbols, the corresponding SLIV value can be regarded as invalid.

Proposal 7: Whether the HARQ process ID is still consecutive when one or more SLIVs value is invalid shall be further discussed.

Proposal 8: For SCS of 480 KHz, it is not needed to restrict the maximum number of PDSCHs to 4.

Proposal 9: Fall back DCI (e.g. format 1_0/0_0) does not support multi-PDSCH/PUSH transmission

Proposal 10: The enhancement mechanism for discarding candidate PDSCH reception occasions and HARQ-ACK feedback information shall be studied before determination on the options for Type-1 HARQ-ACK can be made.

Proposal 11:  Considering the motivation of introducing multiple PDSCHs scheduling is that PDCCH monitoring period is based on multiple slots, Alt-1 for Type-2 HARQ-ACK codebook generation is preferred.

- Reference

RP-202925, “Revised WID:  Extending current NR operation to 71 GH”, CMCC

RP-192263, “TR 38.807-100: Study on requirements for NR beyond 52.6 GHz”, Intel

TR 38.808 v1.0.0: “Study on supporting NR from 52.6 GHz to 71 GHz ".

TS38.214v16.4.0, “Physical layer procedures for data”.

TS38.133v16.6.0, “Requirements for support of radio resource management”.

- [6]       R1-2101310 PDSCH-PUSCH Enhancements	Ericsson

- [7]       R1-2102237 Summary-3-[104-e-NR-52-71GHz-05]-v111 Moderator (vivo)

- [8]      3GPP TSG RAN WG1 Meeting RAN1 #104b-e chairman notes, May 12th – May 20th, 2021.