3GPP TSG RAN WG1 #105-e            	   	                         R1-2104520

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Agenda item:	8.5.1

Document for:	Discussion and Decision

Introduction

At RAN1#104-e and RAN1#104b-e meetings, a lot of agreements and conclusions were achieved related to accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors [1][2].

This contribution further discusses mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors for DL, UL and DL+UL positioning methods with UE-based and UE-assisted positioning solutions, based on our previous contribution in RAN1#104b-e meeting [3]. Section 2 discusses the configuration of measurement time windows. Section 3 discusses the mitigation of UE/gNB Rx/Tx timing errors. Section 4 summarizes the proposals with conclusions.

Configuration of Measurement Time Windows

In this section, we discuss the configuration of measurement time windows. The following agreement was made in RAN1#104-e related to the measurement time windows [1]:

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

For DL, UL and DL+UL positioning methods, UE reports one or more measurement instances in a single measurement report to LMF for UE-assisted positioning, and/or TRP reports one or more measurement instances in a single measurement report to LMF. The measurement instances should be within a configured measurement time window, and each measurement instance is reported with its own timestamp.

The measurement time window can help LMF to eliminate the influence of timing errors of TRPs and UE. The purposes for introducing measurement time window are as follows:

- Limit the measurement behaviour of UE or TRP, and only DL-PRS/ SRS-Pos resources within the measurement time window will be measured.

- Limit the measurement time of each measurement instance, and support the measurement instance which only corresponds to one DL-PRS/SRS-Pos occasion for one-shot measurement.

- Facilitate the timestamps matching among various measurement instances, e.g., among UE Rx-Tx time difference measurement instances and gNB Rx-Tx time difference measurement instances for multi-RTT positioning method.

- Indicate whether the measurement instances are measured within the same measurement time window.

- Help LMF to track and mitigate the timing error drift over time.

As mentioned above, there are many purposes for introducing measurement time windows, and different purposes may need different configurations of measurement time windows. Therefore, the measurement time windows should be configurable to support various purposes, in which the UE or TRP measurement instances are measured.

Proposal 1: The measurement time windows should be configurable, in which the UE or TRP measurement instances are obtained.

For UE and TRP, the measurement time windows need to be configured independently in order to measure the DL-PRS/SRS-Pos resources in the windows. The configuration parameters of UE measurement time windows and TRP measurement time windows can be the same or different to support different purposes. As shown in Figure 1 and Figure 2 below, the UE measurement time windows and TRP measurement time windows are configured in the same manner.

UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources. TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources. And UE or TRP is not expected to report the measurement instances corresponding to a measurement time window from DL-PRS or SRS-Pos outside of the measurement time window.

Proposal 2: UE measurement time windows and TRP measurement time windows can be configured independently. They can be configured to be the same or different.

- UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources.

- TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources.

Proposal 3:  UE (or TRP) is not expected to measure DL-PRS (or SRS-Pos) outside of the measurement time window.

Two configuration methods will be discussed in the following: one indirectly configures the length of the measurement time window, and the other directly configures the length of the measurement time window.

Configuration method 1: Indirectly configure the length of the measurement time window

In this section, the configuration method 1 will be discussed, which indirectly configure the length of the measurement time window.

In order to configure the UE/TRP measurement time window, the following parameters for UE/TRP measurement time window should be configured and indicated via LPP and NRPPa signalling:

- The periodicity of UE/TRP measurement time window.

- The start time of UE/TRP measurement time window.

- The number of UE/TRP measurement instances included in the UE/TRP measurement time window.

- The number of instances of DL-PRS/SRS-Pos resource set or DL-PRS/SRS-Pos occasions contained by each UE/TRP measurement instance.

Figure 1: Configuration method 1 of UE measurement time windows (UE MTWs)

Figure 2: Configuration method 1 of TRP measurement time windows (TRP MTWs)

As shown in Figure 1, UE is configured with two measurement time windows, i.e., UE MTW#1 and MTW#2.

For UE MTW#1, its start time is T1, and only one measurement instance (MI#1) is configured and it contains the measurement results from one instance of DL-PRS resource set (R1). And the measurement report MR#1 contains MI#1, whose timestamp is t1.

For UE MTW#2, its start time is T2, and there are two measurement instances (MI#2 and MI#3) being configured. MI#2 contains the measurement results from two instances of DL-PRS resource sets (R3&R4). MI#3 contains the measurement results from one instance of DL-PRS resource sets (R5). And the measurement report MR#2 contains MI#2 and MI#3, whose timestamp is t2 and t3, respectively.

Based on the above discussion, we have the following proposal:

Proposal 4: (Configuration method 1): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- J: The number of UE measurement instances included in the UE measurement time window.

- Ni: The number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- K: The number of TRP measurement instances included in the TRP measurement time window.

- Mi: The number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance.

When UE or TRP reports a measurement instance, it also reports the time stamp of the measurement instance. If the measurement instance contains more than one DL-PRS or SRS-Pos resource set, the timestamp of the measurement instance should correspond to one time instance between the first and the last DL-PRS or SRS-Pos resource set contained by the measurement instance. For example, as shown in Figure 2, measurement instance MI#2 contains two SRS-Pos resource sets (R3 and R4); the timestamp of MI#2 should be one time instance t2 between R3 and R4.

About the start time of each UE/TRP measurement time window and timestamp of each measurement instance, it can be defined with an absolute time (UTC time) or a relative time (SFN + slot index).

Proposal 5: When UE reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to any of the time instances between the first and the last DL-PRS resource set contained by the measurement instance.

Proposal 6: When TRP reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one time instance between the first and the last SRS-Pos resource set contained by the measurement instance.

With the above parameters of UE/TRP measurement time window, the length of UE/TRP measurement time window can be defined as follows,

Proposal 7: For configuration method 1 and the periodic DL-PRS, the length of UE measurement time window MTWUE can be defined as:

MTWUE=PDDL-PRS×i=1JPNPi

        - PDDL-PRS is the periodicity of DL-PRS resource set;

        - JP is the number of UE measurement instances included in the UE measurement time window, JP≥1;

        - NPi is the number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance，NPi≥1.

Proposal 8: For configuration method 1 and the periodic/semi-persistent SRS-Pos, the length of TRP measurement time window MTWTRP can be defined as:

MTWTRP=PDSRS-Pos×i=1KPMPi

        - PDSRS-Pos is the periodicity of SRS-Pos resource set;

        - KP is the number of TRP measurement instances included in the TRP measurement time window, KP≥1;

        - MPi is the number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance，MPi≥1.

The number of instances of the DL-PRS resource set or SRS-Pos resource set contained by one UE/TRP measurement instance (i.e., Ni and Mi) should be configurable.

About whether Ni or Mi can be configured with 1, in order to support the measurement instance which only corresponds to one DL-PRS/SRS-Pos occasion for one-shot measurement, each UE or TRP measurement instance can be configured to only contain one instance of DL-PRS resource set or SRS-Pos resource set. Therefore, Ni or Mi can be configured with 1. As shown in Figure 2, measurement instance MI#1 only contains one instance of SRS-Pos resource set (R1) with the timestamp t1, and measurement report MR#1 only include the measurement results from one instance of SRS-Pos resource set (R1). The TRP measurement report MR#1 can be seen as one-shot measurement without the combination cross more than one instance of SRS-Pos resource sets.

Configuration method 2: Directly configure the length of the measurement time window

In this section, the configuration method 2 will be discussed, which directly configure the length of the measurement time window.

In order to configure the UE/TRP measurement time window, the following parameters for UE/TRP measurement time window should be configured and indicated via LPP and NRPPa signalling:

- The periodicity of UE/TRP measurement time window.

- The start time of UE/TRP measurement time window.

- The length of UE/TRP measurement time window.

Figure 3: Configuration method 2 of UE measurement time windows (UE MTWs)

Figure 4: Configuration method 2 of TRP measurement time windows (TRP MTWs)

As shown in Figure 3, UE is configured with two measurement time windows, i.e., UE MTW#1 and MTW#2. For UE MTW#1, its start time is T1, with the length L1. For UE MTW#2, its start time is T2, with the length L2.

Based on the above discussion, we have the following proposal:

Proposal 9: (Configuration method 2): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- L1: The length of UE measurement time window.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- L2: The length of TRP measurement time window.

Comparison between Configuration method 1 and Configuration method 2

In section 2.1 and section 2.2, we discussed Configuration method 1 and Configuration method 2 for the measurement time window. Compared with Configuration method 1, the merit of Configuration method 2 is reducing the complexity and overhead of configuration signalling for the measurement time window. And a lot of details of the configuration of the measurement time window will be up to UE/TRP implementation.

However, Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2. For example, Configuration method 1 can indicate or configure the number of UE/TRP measurement instances included in the UE/TRP measurement time window, as well as the number of instances of DL-PRS/SRS-Pos resource set contained by each UE/TRP measurement instance. It will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE. Therefore, Configuration method 1 is preferred to configure the measurement time window.

Observation 1: Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2.

Proposal 10: Configuration method 1 is preferred to configure the measurement time window. It will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE.

Mitigation of UE/gNB Rx/Tx timing errors

In this section, two candidate schemes for the mitigation of Rx/Tx timing errors are analysed. Scheme 1 is to measure and compensate Rx/Tx timing errors; scheme 2 is to introduce reference devices to eliminate Rx/Tx timing errors.

Scheme 1: Mitigation methods based on measurements and compensations of Rx/Tx timing errors

In this section, we analyse the mitigation methods based on measurements and compensations of Rx/Tx timing delays, for DL-TDOA, UL-TDOA as well as DL+UL positioning methods, with UE-based and UE-assisted positioning solutions.

- 3.1.1 DL positioning methods (DL-TDOA)

The following conclusion was made in RAN1#104-e related to the options for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA [1]:

Conclusion: Study the following options for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA: - Option 1: - Support a TRP to provide the association information of DL PRS resources with Tx TEGs to LMF - Option 2: - Support LMF to provide the association information of DL PRS resources with Tx TEGs to UE for UE-based positioning - Option 3: - Support a TRP to provide the Tx timing errors per Tx TEG to LMF - Option 4: - Support LMF to provide the Tx timing errors per Tx TEG of TRP to a UE for UE-based positioning - Option 5: - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to LMF when the UE reports the RSTD measurements to LMF - Option 6: - Support LMF to provide Rx timing errors per Rx TEG to a UE for UE-based positioning - Option7: - Support a UE to provide Rx timing errors per Rx TEG to LMF for UE-assisted positioning - Option 8: - Support a TRP to provide the Tx timing error differences between Tx TEGs of the TRP to LMF - Option 9: - Support LMF to provide the Tx timing error differences between Tx TEGs of a TRP to a UE for UE-based positioning - Option10: - Support a UE to provide Rx timing error differences between Rx TEGs to LMF for UE-assisted positioning - FFS: details of the signalling, procedures, and UE capability - FFS: How the TEGs are determined by the UE or TRP (could be by implementation, i.e., no specification impact) - Note: Other options are not precluded. - Note: Depending on the discussion results, none/one/multiple of the above options may be adopted in Rel-17.

The following agreement was achieved in RAN1#104b-e related to the options for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA [2]:

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

In addition to the options agreed in RAN1#104b-e, we believe some of the other options are reasonable and should also be supported for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA. Firstly, DL positioning methods includes UE- assisted and UE- based positioning solutions. Therefore, we will discuss these options for UE-assisted DL positioning methods and UE-based DL positioning methods, respectively.

For DL positioning methods with UE-assisted solutions, there are three methods to help LMF eliminate the influence of timing errors of TRPs and UE:

- Method 1: Provide LMF with the association information of DL PRS resources or RSTD measurements with Tx or Rx TEGs.

- Method 2: Provide LMF with the Tx timing errors per Tx TEG.

- Method 3: Provide LMF with the Tx timing error differences between Tx TEGs.

For method 1, its application scenario is that TRP/UE does not know the timing errors corresponding to each TEG or the timing error differences between TEGs, but only know the association information of DL PRS resources or RSTD measurements with Tx or Rx TEGs. According to the agreement in RAN1#104b-e, method 1 had been supported.

For method 2, TRP knows the Tx timing errors corresponding to each Tx TEG, but it will not adjust them when transmitting DL-PRS resources. Instead, TRP provides such information to LMF so that LMF will adjust or compensate the Tx timing errors for DL-PRS resources (i.e., Option 3 in the above conclusion). It should be noted that only Tx timing errors of TRP need to be provided to LMF, and UE Rx timing errors can be compensated directly by UE when UE reports RSTD measurements, so that UE Rx timing errors don’t need to be provided to LMF (i.e., Option 7 in the above conclusion is not needed).

For method 3, TRP knows the Tx timing error differences between Tx TEGs of the TRP, but it will not adjust them when transmitting DL-PRS resources. Instead, TRP provides such information to LMF so that LMF will adjust or compensate the Tx timing errors differences for DL-PRS resources (i.e., Option 8 in the above conclusion). It should be noted that only Tx timing error differences of TRP need to be provided to LMF, and UE Rx timing error differences can be compensated directly by UE when UE reports RSTD measurements, so that UE Rx timing error differences don’t need to be provided to LMF (i.e., Option 10 in the above conclusion is not needed).

Observation 2: For UE-assisted DL-TDOA positioning, only Tx timing errors or timing error differences of TRP need to be provided to LMF, and UE Rx timing errors or timing error differences can be compensated directly by UE when UE reports RSTD measurements.

Based on the above analysis and observation, we have the following proposal:

Proposal 11: For UE-assisted DL-TDOA positioning, the following two methods should also be supported to help LMF eliminate the influence of timing errors of TRPs and UE:

- Provide LMF with the Tx timing errors per Tx TEG.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 3 | TRP | LMF | Tx timing errors per Tx TEG of TRP |

- Provide LMFwith the Tx timing error differences between Tx TEGs.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 8 | TRP | LMF | Tx timing error differences between Tx TEGs of the TRP |

For DL positioning methods with UE-based solutions, there are three methods to help UE eliminate the influence of timing errors of TRPs:

- Method1: Provide UE with the association information of DL PRS resources with Tx TEGs.

- Method2: Provide UE with the Tx timing errors per Tx TEG.

- Method3: Provide UE with the Tx timing error differences between Tx TEGs.

For method1, its application scenario is that LMF does not know the timing errors corresponding to each TEG or the timing error differences between TEGs, but only know the association information of DL PRS resources with TEGs. According to the agreement in RAN1#104b-e, method 1 had been supported.

For method2, LMF knows the Tx timing errors corresponding to each Tx TEG, but TRP will not adjust them when transmitting DL-PRS resources. Instead, LMF provides such information to UE so that UE will adjust or compensate the Tx timing errors for DL-PRS resources (i.e., Option 4 in the above conclusion).

For method3, LMF knows the Tx timing error differences between Tx TEGs of the TRP, but TRP will not adjust them when transmitting DL-PRS resources. Instead, LMF provides such information to UE so that UE will adjust or compensate the Tx timing errors differences for DL-PRS resources (i.e., Option 9 in the above conclusion).

About Option 6 in the above conclusion, it proposes to support LMF to provide Rx timing errors per Rx TEG to a UE for UE-based positioning. The rationale from the discussion in the last meeting is that LMF may obtain the Rx timing errors per Rx TEG of UE from other positioning methods in advance (e.g., UL-AoA); however, we don’t think it is a general method to mitigate Rx timing errors of UE. Moreover, UE Rx timing errors can be obtained by UE itself, so that LMF don’t need to provide UE Rx timing errors to UE.

Based on the above analysis, we have the following proposal:

Proposal 12: For UE-based DL-TDOA positioning, the following two methods should also be supported to help UE eliminate the influence of timing errors of TRPs:

- Provide UE with the Tx timing errors per Tx TEG.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 4 | LMF | UE | Tx timing errors per Tx TEG of TRP |

- Provide UE with the Tx timing error differences between Tx TEGs.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 9 | LMF | UE | Tx timing error differences between Tx TEGs of a TRP |

- 3.1.2 UL positioning methods (UL-TDOA)

The following conclusion was made in RAN1#104-e related to the options for mitigating TRP Rx timing errors and/or UE Tx timing errors for UL-TDOA [1]:

Conclusion: Study the following option(s) for mitigating UE Tx and TRP Rx timing errors for UL TDOA: - Option 1: - Support a TRP to provide the association information of RTOA measurements with Rx TEGs to LMF when the TRP reports the RTOA measurements - Option 2: - Support a UE to provide the association information of SRS resources for positioning with UE Tx TEG(s) to LMF for UL TDOA positioning. - Option 3: - Support a UE to provide Tx timing errors per Tx TEG to LMF for UL TDOA positioning. - Option 4: - Support a UE to provide Tx timing error differences between Tx TEGs to LMF for UL TDOA positioning. - FFS: the details of the signalling, procedures, and UE capability - FFS: How the TEGs are determined by the UE or TRP (could be by implementation, i.e., no specification impact) - Note: Other options are not precluded. - Note: Depending on the discussion results, none/one/multiple of the above options may be adopted in Rel-17.

The following agreement was achieved in RAN1#104b-e related to the options for mitigating TRP Tx timing errors and/or UE Rx timing errors for UL TDOA [2]:

Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF; - FFS: the details of the Signaling, procedures, and UE capability

For UL positioning methods, there are three methods to help LMF eliminate the influence of timing errors of TRPs and UE:

- Method 1: Provide LMF with the association information of RTOA measurements or SRS resources for positioning with Rx or Tx TEGs.

- Method 2: Provide LMF with the Tx timing errors per Tx TEG.

- Method 3: Provide LMF with the Tx timing error differences between Tx TEGs.

For method 1, its application scenario is that TRP/UE does not know the timing errors corresponding to each TEG or the timing error differences between TEGs, but only know the association information of RTOA measurements or SRS resources for positioning with Rx or Tx TEGs. According to the agreement in RAN1#104b-e, method 1 had been supported. Regarding to the remain issues for method 1, firstly, we support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs, since RAN1 had agreed that UL SRS resources for MIMO can be reused for NR UL RTOA, AoA and gNB RSRP measurements for positioning in NR. Moreover, we prefer the association information is sent directly from UE to LMF via LPP signalling to reduce the latency and overhead, instead of  being first provided to gNB and then forwarded to LMF.

For method 2, UE knows the Tx timing errors corresponding to each Tx TEG, but it will not adjust them when transmitting SRS resources for positioning. Instead, UE provides such information to LMF so that LMF will adjust or compensate the Tx timing errors for SRS resources for positioning (i.e., Option 3 in the above conclusion). It should be noted that only Tx timing errors of UE need to be provided to LMF, and TRP Rx timing errors can be compensated directly by TRP when TRP reports RTOA measurements, so that TRP Rx timing errors don’t need to be provided to LMF.

For method 3, UE knows the Tx timing error differences between Tx TEGs of the UE, but it will not adjust them when transmitting SRS resources for positioning. Instead, UE provides such information to LMF so that LMF will adjust or compensate the Tx timing errors differences for SRS resources for positioning. It should be noted that only Tx timing error differences of UE need to be provided to LMF, and TRP Rx timing error differences can be compensated directly by TRP when TRP reports RTOA measurements, so that TRP Rx timing error differences don’t need be provided to LMF.

Observation 3: For UL-TDOA positioning, only Tx timing errors or timing error differences of UE need to be provided to LMF, and TRP Rx timing errors or timing error differences can be compensated directly by TRP when TRP reports RTOA measurements.

Based on the above analysis and observation, we have the following proposal:

Proposal 13: For UL-TDOA positioning, the following two methods should also be supported to help LMF eliminate the influence of timing error of TRPs and UE:

- Provide LMF with the Tx timing errors per Tx TEG.

| UL-TDOA | From | To | Signalling for mitigating UE Tx timing errors |
| --- | --- | --- | --- |
| Option 3 | UE | LMF | Tx timing errors per Tx TEG |

- Provide LMF with the Tx timing error differences between Tx TEGs.

| UL-TDOA | From | To | Signalling for mitigating UE Tx timing errors |
| --- | --- | --- | --- |
| Option 4 | UE | LMF | Tx timing error differences between Tx TEGs |

- 3.1.3 DL+UL positioning methods

The following agreement was achieved in RAN1#104b-e related to the options for mitigating UE Rx/Tx timing errors in DL+UL positioning [2]:

Agreement: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS; - Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to the one of the 2 following options: - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS. - For both alternatives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the signalling, procedures, and UE capability

For DL+UL positioning methods, there are three alternatives to help LMF eliminate the influence of timing errors of UE:

- Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS.

- Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF.

- Alt.3: Provide LMF with the timing errors per UE Rx/Tx TEG, or the timing error differences between the UE Rx/Tx TEGs

For Alt.1, its application scenario is that UE does not know the timing errors corresponding to each TEG or the timing error differences between TEGs, but only know the association information of UE Rx-Tx time difference measurements with TEGs. Therefore, UE can only provide the association information of UE Rx-Tx time difference measurements with Rx and Tx TEGs to LMF. With such information, LMF knows which UE Rx-Tx time difference measurements associated with the same UE Rx TEGs and with the same UE Tx TEGs and it will help LMF eliminate the influence of timing errors of UE.

For Alt.3, UE knows the timing errors per Rx/Tx TEG, or the timing error differences between the Tx/Rx TEGs, therefore such timing errors or timing error differences can be directly compensated by UE when UE reports UE Rx-Tx time deference measurements, so that such Tx and Rx timing errors or timing error differences don’t need to be provided to LMF.

Observation 4: For DL+UL positioning, UE Tx and Rx timing errors or timing error differences don’t need to be provided to LMF, and UE Tx and Rx timing errors or timing error differences can be compensated directly by UE when UE reports UE Rx-Tx time difference measurements.

About Alt.2, it is related to providing the association information or timing errors/time error differences per RxTx TEG. In our point of view, it will make the signalling for mitigating UE Rx/Tx timing errors more complicated and increase the overhead, if RxTx TEGs are introduced for DL+UL positioning methods. Taking Alt.1 and Alt.2 as an example, if a UE is configured with 4 Tx TEGs and 4 Rx TEGs, if the association information per UE Rx/Tx TEGs is needed to be provided to LMF (i.e., Alt.1), only 8 timing errors (corresponding to 4 Tx TEGs and 4 Rx TEGs, respectively) should be provided to LMF. However, if the timing errors per RxTx TEG are needed to be provided to LMF (i.e., Alt.2), UE has to provide 16 timing errors (corresponding to 4 Tx TEGs × 4 Rx TEGs = 16 RxTx TEGs, respectively) should be provided to LMF. Compared withAlt.1, the signalling overhead of Alt.2 will be doubled. In addition, LMF can obtain the association information or timing errors/time error differences per RxTx TEG by combining corresponding information per Rx TEG and per Tx TEG. Therefore, we failed to see the need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for UE.

Observation 5: For mitigating UE Rx/Tx timing errors in DL+UL positioning, considering the signalling overhead and the fact that LMF can obtain the association information or timing errors/time error differences per RxTx TEG of UE by combining corresponding information per Rx TEG and per Tx TEG of UE, there is no need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for UE.

Based on the above analysis and observation, we have the following proposal:

Proposal 14: For DL+UL positioning methods, the following Alt.1 should be supported to help LMF eliminate the influence of timing error of UE:

- Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS.

The following agreement was achieved in RAN1#104b-e related to the options for mitigating gNB Rx/Tx timing errors in DL+UL positioning [2]:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures

For DL+UL positioning methods, there are three alternatives to help LMF eliminate the influence of timing errors of TRPs:

- Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF.

- Alt.2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs.

- Alt.3: Provide LMF the timing errors per TRP Rx/Tx TEG, or the timing error differences between the TRP Rx/Tx TEGs.

For Alt.1, its application scenario is that TRP does not know the timing errors corresponding to each TEG or the timing error differences between TEGs, but only know the association information of gNB Rx-Tx time difference measurements with TEGs. Therefore, TRP can only provide the association information of gNB Rx-Tx time difference measurements with Rx and/or Tx TEGs to LMF. With such information, LMF knows which gNB Rx-Tx time difference measurements associated with the same TRP Rx TEGs and/or with the same TRP Tx TEGs and it will help LMF eliminate the influence of timing errors of TRP.

For Alt.3, TRP knows the timing errors per Rx/Tx TEG, or the timing error differences between the Tx/Rx TEGs, therefore such timing errors or timing error differences can be directly compensated by TRP when TRP reports gNB Rx-Tx time deference measurements, so that such Tx and Rx timing errors or timing error differences don’t need to be provided to LMF.

Observation 6: For DL+UL positioning, TRP Tx and Rx timing errors or timing error differences don’t need to be provided to LMF, and TRP Tx and Rx timing errors or timing error differences can be compensated directly by TRP when TRP reports gNB Rx-Tx time difference measurements.

About Alt.2, it is related to providing the association information or timing errors/time error differences per RxTx TEG. In our point of view, it will make the signalling for mitigating TRP Rx/Tx timing errors more complicated and increase the overhead, if RxTx TEGs are introduced for DL+UL positioning methods. Taking Alt.1 and Alt.2 as an example, if a TRP is configured with 4 Tx TEGs and 4 Rx TEGs, if the association information per TRP Rx/Tx TEGs is needed to be provided to LMF (i.e., Alt.1), only 8 timing errors (corresponding to 4 Tx TEGs and 4 Rx TEGs, respectively) should be provided to LMF. However, if the timing errors per RxTx TEG are needed to be provided to LMF (i.e., Alt.2), TRP has to provide 16 timing errors (corresponding to 4 Tx TEGs × 4 Rx TEGs = 16 RxTx TEGs, respectively) should be provided to LMF. Compared with Alt.1, the signalling overhead of Alt.2 will be doubled. In addition, LMF can obtain the association information or timing errors/time error differences per RxTx TEG by combining corresponding information per Rx TEG and per Tx TEG. Therefore, we failed to see the need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for TRP.

Observation 7: For mitigating TRP Rx/Tx timing errors in DL+UL positioning, considering the signalling overhead and the fact that LMF can obtain the association information or timing errors/time error differences per RxTx TEG of TRP by combining corresponding information per Rx TEG and per Tx TEG of TRP, there is no need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for TRP.

Proposal 15: For DL+UL positioning methods, the following Alt.1 should be supported to help LMF eliminate the influence of timing error of TRPs:

- Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF.

Scheme 2: Mitigation methods based on reference devices

The following agreement was made in RAN1#104-e related to the reference devices [1]:

Agreement: - Study specification impact for enabling a reference device with known location to support the following functionalities: - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF; - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF; - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations; - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information - FFS: The device with the known location being a UE and/or a gNB - FFS: Precision to which location of reference device is known - Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI

In this section, three candidate methods for mitigation of Rx/Tx timing errors with reference devices are discussed and pros and cons of these methods are analysed. Method 1 is real-time UE& gNB Rx/Tx timing error mitigation method by double differential scheme with reference UE. Method 2 is one-shot gNB Rx/Tx timing error estimation and compensation method with reference UE. Method 3 is one-shot gNB Rx/Tx timing error estimation and compensation method with reference TRP.

- 3.2.1 Method 1：Real-time UE& gNB Rx/Tx timing error mitigation method with reference UE

Method 1 can be used in DL, UL and DL+UL positioning. In the following, method 1 is discussed using DL positioning [3].

DL positioning:

The DL TOA with TRPi and TRPj measured by target UE and reference UE at UE’s BB is as follows:

ToATRPi→UE=TETRPi,Txi+PropTRPi→UE+TEUE,Rx                                                      (3-1)

ToATRPj→UE=TETRPj,Txi+PropTRPj→UE+TEUE,Rx                                                           (3-2)

ToATRPi→UE_ref=TETRPi,Txi+PropTRPi→UE_ref+TEUEref,Rx                                             (3-3)

ToATRPj→UE_ref=TETRPj,Txi+PropTRPj→UE_ref+TEUE_ref,Rx                               (3-4)

where PropTRPi/j→UE/UE_ref indicates propagation delay between TRPi/TRPj and UE/reference UE in the air, and TEUE/UE_ref,Rx/Tx indicates residual reception/transmission timing error from antenna/baseband to baseband/antenna for UE receiving or TRP transmitting DL-PRS. Note that clock error and measurement error are not considered from (3-1) to (3-4).

Subtracting (3-2) from (3-1) and (3-4) from (3-3), we can get the single differential (SD) TOA (TDOA / RSTD):

SD(UE)=TETRPi,Txi-TETRPj,Txi+PropTRPi→UE-PropTRPj→UE               (3-5)

SDUEref=TETRPi,Txi-TETRPj,Txi+PropTRPi→UE_ref-PropTRPj→UE_ref        (3-6)

Note that residual UE Rx timing error from antenna to baseband is eliminated from (3-5) to (3-6).

Subtracting (3-6) from (3-5), the real-time double differential (DD) TOA is as follows:

DDDL(UE, UE_ref)=PropTRPi→UE-PropTRPj→UE-(PropTRPi→UEref-PropTRPj→UEref)        (3-7)

From (3-7), we can know that the impact of Tx/ Rx timing error of target UE and TRPs are eliminated by the real-time double differential operation.

If the location coordinate information of reference UE is known by LMF or reported to LMF, then SD measurements of target UE from DD measurement can be constructed as in (3-8).

PropTRPi→UE-PropTRPj→UE=DDDLUE, UEref+(PropTRPi→UEref-PropTRPj→UEref)        (3-8)

When we get (3-8), the location of target UE can be further calculated by using Chan’s algorithm or other algorithm.

UL positioning:

The UL single differential TOA (UL TDOA) is as follows：

ToAUE→TRPi-ToAUE→TRPj=TETRPi,Rxi-TETRPj,Rxi+PropUE→TRPi-PropUE→TRPj            (3-9)

ToAUE_ref→TRPi-ToAUE_ref→TRPj=TETRPi,Rxi-TETRPj,Rxi+PropUE_ref→TRPi-PropUE_ref→TRPj(3-10)

Using the same analysis method as DL positioning, SD measurement for target UE from DD measurement DDULUE, UEref can be constructed as follows:

PropUE→TRPi-PropUE→TRPj=DDULUE, UEref+(PropUE_ref→TRPi-PropUE_ref→TRPj)          (3-11)

DL+UL (RTT) positioning:

The single differential RTT is as follows:

RTTUE_TRPi-RTTUE_TRPj=2PropTRPj→UE-2PropTRPi→UE+TETRPi,Rx+TETRPi,Tx-TETRPj,Rx-TETRPj,Tx(3-12)

RTTUE_ref_TRPi-RTTUE_ref_TRPi  =2PropTRPj→UE_ref-2PropTRPi→UE_ref+TETRPi,Rx+TETRPi,Tx-TETRPj,Rx-TETRPj,Tx(3-13)

Using the same method as in DL positioning, SD measurement for target UE from DD measurement DDRTTUE, UEref can be constructed as follows:

PropTRPi→UE-PropTRPj→UE=DDRTTUE, UEref2+(PropTRPi→UEref-PropTRPj→UEref)       (3-14)

Note that the impact of Tx/ Rx timing error of target UE and TRPs are eliminated by the real-time double differential operation from (3-7), (3-11) and (3-14).

The impact of method 1 on the protocol is as follows: Firstly, the location coordinate information of reference UE should be reported to LMF. Secondly, the same set of TRPs is required to be used for DL, UL and DL+UL method. For DL positioning, the target UE and the reference UE are required to measure DL-PRS from the same set of TRPs, and report the DL RSTD measurements in (3-5) and (3-6) to LMF. For UL positioning, the same set of TRPs are required to measure the UL SRS-Pos from the target UE and the reference UE, and report the UL RTOA measurements to LMF. For DL+UL positioning, the same set of TRPs are required to measure the UL SRS-Pos from the target UE and the reference UE, and report the gNB Rx-Tx time difference measurements to LMF; the target UE and the reference UE are required to measure DL-PRS from the same set of TRPs, and report the UE Rx-Tx time difference measurements to LMF.

Observation 8: The impact of Rx/Tx timing error of target UE and TRPs are eliminated using the real-time double differential operation with reference UE.

- 3.2.2 Method 2：One-shot gNB Rx/Tx timing error estimation and compensation method with reference UE

Method 2 can be used in DL, UL and DL+UL positioning. In the following, method 2 is discussed using DL positioning.

First step: estimation of TE difference value

From (3-6), if both SDUEref and (PropTRPi→UE_ref-PropTRPj→UE_ref) have been known by LMF, the difference of the Tx timing error between TRPi and TRPj can be calculated as follows:

TETRPi,Txi-TETRPj,Txi=SDUEref-(PropTRPi→UE_ref-PropTRPj→UE_ref)           (3-15)

where SDUEref=ToATRPi→UE_ref-ToATRPj→UEref denotes the single difference of TRPi and TRPj.

Second step: compensation of TE difference value

From (3-5) and (3-15), single difference (SD) measurements for target UE after compensating the impact of TRP Tx timing error can be calculated as follows:

PropTRPi→UE-PropTRPj→UE=SDUE-(TETRPi,Txi-TETRPj,Txi)              (3-16)

where SDUE is single differential TOA measurement from (3-5), and (TETRPi,Txi-TETRPj,Txi) is get from (3-15).

Note that time variation of TRP Tx timing error is not considered in (3-16).

There are two options for how to estimation of TE in the first step.

Option 1 (UE-assisted positioning): LMF estimates the difference of Tx TE between different TRPs

In Option 1, LMF estimates the difference of Tx timing errors between different TRPs according to the reported positioning measurement from the UE/TRPs, which is used to compensate the measurement in the second step.

- For DL positioning, the reference UE receives the DL PRS from different TRPs and reports DL positioning measurement using the same way of signalling as target UE and reports its location to the LMF.

- For UL positioning, the reference UE transmits the UL SRS reference signal to different TRPs using the same way of signalling as target UE, and reports its location to the LMF; gNB reports UL positioning measurements from target UE and reference UE to LMF. To achieve high accuracy, the time-frequency resource of SRS-Pos by target UE and reference UE should be orthogonal.

Option 2 (UE- assisted and UE-based positioning): Reference UE estimates the difference of Tx TE between different TRPs

In Option 2, the reference UE estimates the difference of Tx timing errors between different TRPs, and reports estimated TE value to LMF for UE-assisted positioning or to target UE via LMF for UE-based positioning.

Observation 9: For method 2, both Option 1 and Option 2 should be supported.

- 3.2.3 Method 3：One-shot gNB Rx/Tx timing error measurement and compensation method with reference TRP

The principle of method 3 can be referred to the reference [4]. There are two options on how to calculate the difference of timing error (TE).

- Option 1: TRP calculates the TE difference, and reports the Tx TE difference between TRPs to LMF.

- Option 2: LMF calculates the TE difference, and TRP only needs to report the measurement to LMF.

For Option 1, the reference signal and measurement between different TRPs are limited to TRP implementation and only a measurement report needs to be defined. For Option 2, only measurement report needs to be defined by the protocol. Option 1 is preferred as a result of little protocol impact.

Observation 10: For method 3, Option 1 has little protocol impact compared to Option 2.

- 3.2.4 Comparison of method 1/2/3

The pros and cons of method 1/2/3 are given in the following Table.

| Methods | Pros | Cons |
| --- | --- | --- |
| Method 1 | 1) Method 1 has little impact on protocol: The reference UE needs to report its location to LMF, and uses the same way to report the DL or DL+UL positioning measurement to LMF. 2) Method 1 will not be impacted by time variation of UE&TRP Rx/Tx timing error. | 1) In method 1, the target UE and the reference UE are required to measure the same set of TRPs. For example, to support 3-D positioning, the number of same TRPs should be no less than X, where X is greater than 4. - For UL positioning, UL RTOA measurements from all TRPs are based on a fixed value, which is the RTOA Reference Time is defined in TS 38.455 [5]. - For DL positioning, DL TDOA is a single differential value, and the reference UE and the target UE can choose a reference TRP by itself. For the real-time double differential method, the reference UE and the target UE may use different reference TRPs. In this case, the same reference TRP can be obtained through mathematical transformation if the target UE and the reference UE measure the same set of TRPs. 2) The requirement for the reference UE is high, as it needs to hold LOS channel type to all TRPs used for positioning at all measurement instance. |
| Method 2 | 1) Method 2 has little impact on protocol: The reference UE needs to report its location to LMF, and uses the same way to report the DL or DL+UL positioning measurement to LMF. | 1) In method 2, the target UE and the reference UE are required to measure the same set of TRPs. For example, to support 3-D positioning, the number of same TRPs should be no less than X, where X is greater than 4. 2) Method 2 will be impacted by time variation of UE&TRP Rx/Tx timing error. |
| Method 3 | 1) For option 1 of method 3, the reference signal and measurement between different TRPs is limited to TRP implementation and only measurement report needs to be defined; 2) Method 3 has lower cost compared to method 1 and method 2, as no reference UE is introduced. | 1) In order to ensure the accuracy of the measurements between TRPs, it is required that the channel types between different TRPs are LOS. However, the current typical TRP antennas all have down-tilt angle. When the distance between TRPs is large, it is difficult to guarantee LOS channel type between TRPs. 2) Method 3 will be impacted by time variation of TRP Rx/Tx timing error. |

The above three methods can be supported. Based on the above analysis, we can get the following proposal.

Proposal 16: NR Rel-17 should support reporting the location coordinate information of reference UE from UE to LMF for mitigating the Rx/Tx timing error of UE/TRPs, with the double differential positioning method.

Proposal 17: NR Rel-17 should support reference UE and target UE using the same way of signalling of DL/UL reference signal, and reporting the measurements for compensation the Rx/Tx timing error of target UE /TRPs, with Rel-16 DL/UL-TDOA / Multi-RTT positioning method.

Proposal 18: NR Rel-17 should support reference UE reporting the value of Rx/Tx timing error difference between different TRPs to LMF for UE-assisted positioning or to target UE via LMF for UE-based positioning.

Proposal 19: NR Rel-17 should support reporting the parameters related to gNB Rx/Tx timing error from gNB to LMF for UE-assisted positioning (or from gNB/LMF to UE for UE-based positioning).

- 3.2.5 Location accuracy of the reference device

In this section, we discuss the location accuracy of the reference device. First of all, the positioning error of the reference device (UE/TRP) should be less than the estimation error of Rx/Tx the timing error and the positioning error of the target UE. Specifically, the following four options can be considered.

- Option 1: The reference device is pre-placed in a known position (e.g., by laser mapping).

- Option 2: The location of reference UE is calculated by RAT-independent positioning scheme (such as GPS / BDS etc.).

- Option 3: The location of reference UE is calculated by DL-TDOA+DL-AoD or other solutions to obtain the preliminary position.

- Option4: The reference device is selected/placed at the location of TRP.

The pros and cons of the above four options are given in the following Table. As Option 3 has lower accuracy compared to other options, Option 1, Option 2, and Option 4 can be supported.

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1 | high accuracy | high cost |
| Option 2 | high accuracy and low cost | cannot work if the GPS / BDS signal is blocked in outdoor or in indoor |
| Option 3 | low cost | low accuracy |
| Option 4 | high accuracy and low cost | difficult to guarantee LOS channel type between different TRPs |

Proposal 20: The following approaches can be supported to obtain the location coordinates of a reference device.

- The reference device is placed in a known position.

- The location of reference UE is calculated by RAT-independent positioning scheme (such as GPS etc.).

- The reference device is selected/placed at the location of a TRP with a known position.

Conclusions

In this contribution, we discuss accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors, and give the following observations and proposals:

Proposal 1: The measurement time windows should be configurable, in which the UE or TRP measurement instances are obtained.

Proposal 2: UE measurement time windows and TRP measurement time windows can be configured independently. They can be configured to be the same or different.

- UE measurement time window refers to the time window in which UE measures DL-PRS resources. In this time window, UE obtains at least one UE measurement instance by measuring DL-PRS resources.

- TRP measurement time window refers to the time window in which TRP measures SRS-Pos resources. In this time window, TRP obtains at least one TPR measurement instance by measuring SRS-Pos resources.

Proposal 3: UE (or TRP) is not expected to measure DL-PRS (or SRS-Pos) outside of the measurement time window.

Proposal 4: (Configuration method 1): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- J: The number of UE measurement instances included in the UE measurement time window.

- Ni: The number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- K: The number of TRP measurement instances included in the TRP measurement time window.

- Mi: The number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance.

Proposal 5: When UE reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to any of the time instances between the first and the last DL-PRS resource set contained by the measurement instance.

Proposal 6: When TRP reports a measurement instance, it also reports the time stamp of the measurement instance, which corresponds to one time instance between the first and the last SRS-Pos resource set contained by the measurement instance.

Proposal 7: For configuration method 1 and the periodic DL-PRS, the length of UE measurement time window MTWUE can be defined as:

MTWUE=PDDL-PRS×i=1JPNPi

        - PDDL-PRS is the periodicity of DL-PRS resource set;

        - JP is the number of UE measurement instances included in the UE measurement time window, JP≥1;

        - NPi is the number of instances of DL-PRS resource set or DL-PRS occasions contained by the i-th UE measurement instance，NPi≥1.

Proposal 8: For configuration method 1 and the periodic/semi-persistent SRS-Pos, the length of TRP measurement time window MTWTRP can be defined as:

MTWTRP=PDSRS-Pos×i=1KPMPi

        - PDSRS-Pos is the periodicity of SRS-Pos resource set;

        - KP is the number of TRP measurement instances included in the TRP measurement time window, KP≥1;

        - MPi is the number of instances of SRS-Pos resource set or SRS-Pos occasions contained by the i-th TRP measurement instance，MPi≥1.

Proposal 9: (Configuration method 2): UE/TRP measurement time window should be configured with the following parameters by LMF:

- For UE measurement time window (via LPP signalling):

- P1: The periodicity of UE measurement time window (for periodic UE MTW).

- T1: The start time of UE measurement time window.

- L1: The length of UE measurement time window.

- For TRP measurement time window (via NRPPa signalling):

- P2: The periodicity of TRP measurement time window (for periodic TRP MTW).

- T2: The start time of TRP measurement time window.

- L2: The length of TRP measurement time window.

Observation 1: Configuration method 1 has more flexibility on indication or configuration of the concrete composition of measurement report than Configuration method 2.

Proposal 10: Configuration method 1 is preferred to configure the measurement time window. It will help LMF to more effectively eliminate the influence of timing errors of TRPs and UE.

Observation 2: For UE-assisted DL-TDOA positioning, only Tx timing errors or timing error differences of TRP need to be provided to LMF, and UE Rx timing errors or timing error differences can be compensated directly by UE when UE reports RSTD measurements.

Proposal 11: For UE-assisted DL-TDOA positioning, the following two methods should also be supported to help LMF eliminate the influence of timing errors of TRPs and UE:

- Provide LMF with the Tx timing errors per Tx TEG.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 3 | TRP | LMF | Tx timing errors per Tx TEG of TRP |

- Provide LMFwith the Tx timing error differences between Tx TEGs.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 8 | TRP | LMF | Tx timing error differences between Tx TEGs of the TRP |

Proposal 12: For UE-based DL-TDOA positioning, the following two methods should also be supported to help UE eliminate the influence of timing errors of TRPs:

- Provide UE with the Tx timing errors per Tx TEG.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 4 | LMF | UE | Tx timing errors per Tx TEG of TRP |

- Provide UE with the Tx timing error differences between Tx TEGs.

| DL-TDOA | From | To | Signalling for mitigating TRP Tx timing errors |
| --- | --- | --- | --- |
| Option 9 | LMF | UE | Tx timing error differences between Tx TEGs of a TRP |

Observation 3: For UL-TDOA positioning, only Tx timing errors or timing error differences of UE need to be provided to LMF, and TRP Rx timing errors or timing error differences can be compensated directly by TRP when TRP reports RTOA measurements.

Proposal 13: For UL-TDOA positioning, the following two methods should also be supported to help LMF eliminate the influence of timing error of TRPs and UE:

- Provide LMF with the Tx timing errors per Tx TEG.

| UL-TDOA | From | To | Signalling for mitigating UE Tx timing errors |
| --- | --- | --- | --- |
| Option 3 | UE | LMF | Tx timing errors per Tx TEG |

- Provide LMF with the Tx timing error differences between Tx TEGs.

| UL-TDOA | From | To | Signalling for mitigating UE Tx timing errors |
| --- | --- | --- | --- |
| Option 4 | UE | LMF | Tx timing error differences between Tx TEGs |

Observation 4: For DL+UL positioning, UE Tx and Rx timing errors or timing error differences don’t need to be provided to LMF, and UE Tx and Rx timing errors or timing error differences can be compensated directly by UE when UE reports UE Rx-Tx time difference measurements.

Observation 5: For mitigating UE Rx/Tx timing errors in DL+UL positioning, considering the signalling overhead and the fact that LMF can obtain the association information or timing errors/time error differences per RxTx TEG of UE by combining corresponding information per Rx TEG and per Tx TEG of UE, there is no need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for UE.

Proposal 14: For DL+UL positioning methods, the following Alt.1 should be supported to help LMF eliminate the influence of timing error of UE:

- Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS.

Observation 6: For DL+UL positioning, TRP Tx and Rx timing errors or timing error differences don’t need to be provided to LMF, and TRP Tx and Rx timing errors or timing error differences can be compensated directly by TRP when TRP reports gNB Rx-Tx time difference measurements.

Observation 7: For mitigating TRP Rx/Tx timing errors in DL+UL positioning, considering the signalling overhead and the fact that LMF can obtain the association information or timing errors/time error differences per RxTx TEG of TRP by combining corresponding information per Rx TEG and per Tx TEG of TRP, there is no need to introduce and provide the association information or timing errors/time error differences per RxTx TEG to LMF for TRP.

Proposal 15: For DL+UL positioning methods, the following Alt.1 should be supported to help LMF eliminate the influence of timing error of TRPs:

- Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF.

Observation 8: The impact of Rx/Tx timing error of target UE and TRPs are eliminated using the real-time double differential operation with reference UE.

Observation 9: For method 2, both Option 1 and Option 2 should be supported.

Observation 10: For method 3, Option 1 has little protocol impact compared to Option 2.

Proposal 16: NR Rel-17 should support reporting the location coordinate information of reference UE from UE to LMF for mitigating the Rx/Tx timing error of UE/TRPs, with the double differential positioning method.

Proposal 17: NR Rel-17 should support reference UE and target UE using the same way of signalling of DL/UL reference signal, and reporting the measurements for compensation the Rx/Tx timing error of target UE /TRPs, with Rel-16 DL/UL-TDOA / Multi-RTT positioning method.

Proposal 18: NR Rel-17 should support reference UE reporting the value of Rx/Tx timing error difference between different TRPs to LMF for UE-assisted positioning or to target UE via LMF for UE-based positioning.

Proposal 19: NR Rel-17 should support reporting the parameters related to gNB Rx/Tx timing error from gNB to LMF for UE-assisted positioning (or from gNB/LMF to UE for UE-based positioning).

Proposal 20: The following approaches can be supported to obtain the location coordinates of a reference device.

- The reference device is placed in a known position.

- The location of reference UE is calculated by RAT-independent positioning scheme (such as GPS etc.).

- The reference device is selected/placed at the location of a TRP with a known position.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104b-e.

- R1-2102635, Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, CATT, RAN1 #104b-e.

- R1-2100657, Mitigation of UE and gNB TX/RX Timing Errors, Intel Corporation, RAN1 #104-e.

- TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa) (Release 16).