3GPP TSG RAN WG1 #105-e                                            R1-2104529

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	Discussion on other aspects related to complexity reduction

Agenda Item:	8.6.1.4

Document for:	Discussion and Decision

Introduction

In RAN#90e, a new Rel-17 WI on support of reduced capability NR devices, i.e. RedCap, was approved [1]. The WID was further updated in RAN#91e [2]. It was agreed to support reduced maximum number of DL MIMO layers and relaxed maximum modulation order for RedCap UE:

… - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. - Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE.

The RAN1 leading features were discussed during RAN1#104e and RAN1#104bis-e, wherein the agreements on DL MIMO layers and MCS/CQI were reached [3] [4]. In this contribution, we provide our views on the DL MIMO layers and relaxed MCS and CQI for RedCap UE, based on the agreements so far.

Discussion

DL MIMO layers

In RAN1#104e, the following agreement was reached for DL MIMO layers [3]:

Agreements: - For relaxed maximum number of DL MIMO layers: - FFS: need for modification of DCI fields/formats - FFS: need for modification of CSI measurement/reporting

For the case when a RedCap UE is equipped with 2 Rx branches, it was agreed in the WID that 2 DL MIMO layers are supported. Thus, the maximum number of DL MIMO layers are clear, i.e., maximum 2 layers for Rx=2 UE, maximum 1 layer for Rx=1 UE, respectively.

Observation 1: The maximum number of DL MIMO layers for RedCap UE is already clear.

There are some open issues related to the reduced number of DL MIMO layers:

- Need for modification of DCI fields/formats

Reduced number of DL MIMO layers may have direct impact on the Antenna Port(s) field in DCI formats that indicating DMRS port(s) in PDSCH. Currently, for DCI format 1_1, the bitwidth of Antenna Port(s) field can be 4/5/6 bits. Similarly, for DCI format 1_2, the bitwidth of Antenna Ports can be 0 bit or 4/5/6 bits. For DCI format 1_0, there is no Antenna Port(s) field.

Ideally, for 1 Rx UE, the bitwidth to indicate DMRS may be reduced compared to the case of 4/5/6 bits. However, the bit reduction may not be huge. For example, for 2 Rx UE, 4 bits are required to indicate 10 different states of the ports, i.e. {0, 1, 01} for 1 CDM group without data and port {0, 1, 2, 3, 01, 23, 02} for 2 CDM groups without data. Hence there is no bit reduction compared to the current Antenna Port(s) field. For 1 Rx UE, similarly, it may still need 3 bits to indicate 6 different states of the ports, i.e. port {0, 1} for 1 CDM group without data and port {0, 1, 2, 3} for 2 CDM groups without data. There is only 1 bit reduction compared to the current Antenna Port(s) field.

So, for the case of 4/5/6 bits Antenna Port(s) field in DCI format 1_1/1_2, the coverage improvement is marginal with potential bit reduction for 1 Rx UE. For other cases, i.e. 2 Rx UE, or the case of 0 bit Antenna Port(s) field in DCI format 1_0/1_2, there is no coverage improvement. It is not worthy to bring additional complexity to the standard, at least for Rel-17.

Proposal 1: Modification of DCI fields/formats for reduced DL MIMO layers is not considered.

- Need for modification of CSI measurement/reporting

Current CSI measurement/reporting mechanism is flexible enough to accommodate the DL MIMO layer reduction of RedCap UE. For instance, Table 6.3.1.1.2-3~6 in TS 38.212 have specified the CSI reporting information with different ranks, layers and codebook types, etc., wherein the cases of 1 and 2 layers are already included. There is no need to modify the mechanism of CSI measurement or reporting.

Proposal 2: Modification of CSI measurement/reporting mechanism for reduced DL MIMO layers is not considered.

MCS and CQI

In RAN1#104e, it was agreed to reuse MCS and CQI tables for RedCap UE, and no new MCS or CQI tables to be introduced. But several open issues still need to be studied [3]:

Agreements: - The MCS tables currently defined are re-used for RedCap UEs - FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries) - FFS mandatory/optional of the MCS tables - Note: there is no new MCS table to be introduced for RedCap Ues Agreements: - The CQI tables currently defined are re-used for RedCap UEs. - FFS mandatory/optional of the CQI tables - There is no new CQI table to be introduced for RedCap UEs

There are 3 MCS tables specifies in the NR standard TS 38.214:

- Table 5.1.3.1-1: default MCS table for normal UE. The maximum modulation order is 64QAM.

- Table 5.1.3.1-2: a MCS table applied to UE when the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256'. The maximum modulation order is 256QAM. Since it has been agreed that 256QAM is optional for RedCap UE, Table 5.1.3.1-2 should be optional for RedCap UE.

- Table 5.1.3.1-3: the low SE MCE table, which may apply to a UE when configured with MCS-C-RNTI or the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE'. The maximum modulation order is also 64QAM.

Comparing Table 5.1.3.1-1(default one for normal UE) and Table 5.1.3.1-3 (low SE one), low SE table may have advantage on coverage which may compensate the coverage loss due to reduced Rx number and small form factor. However, we believe that the default MCS table for normal UE should also be the default and mandatory table for RedCap UE, for the following reasons:

- Early identification of RedCap UE is configurable by the operator. If the operators choose no early identification, the gNB does not know the UE type during initial access. In this case, the gNB cannot determine correct MCS indication for a UE if the default MCS tables for RedCap UE and non-RedCap UE are different.

- There is no serious coverage issue to apply default MCS table during the evaluation in SI phase.

- Low SE table improves the coverage at the cost of spectrum efficiency reduction, which is unnecessary for the RedCap UE in good or normal coverage, so should not be a mandatory table.

- Low SE table is applicable only after the dedicated RRC parameter configuration.

Based on the above analyses, we have the following proposals for MCS tables:

Proposal 3: The default MCS table for non-RedCap UE (i.e. Table 5.1.3.1-1 in TS 38.214) is also the default MCS table for RedCap UE.

Proposal 4: The default MCS table for non-RedCap UE is mandatory for RedCap UE, while the 256QAM MCS table and low SE MCS table are optional for RedCap UE.

Similarly, there are 3 CQI tables specifies in the NR standard TS 38.214:

- Table 5.2.2.1-2: the CQI table with maximum 64QAM and a TB error probability not exceeding 0.1.

- Table 5.2.2.1-3: the CQI table with maximum 256QAM and a TB error probability not exceeding 0.1.

- Table 5.2.2.1-4: the CQI table with maximum 64QAM and a TB error probability not exceeding 0.00001 (low SE one).

Note that CQI Table 5.2.2.1-2/3/4 are corresponding to MCS Table 5.1.3.1-1/2/3. Hence, with similar analyses and motivation, we have the following proposals for CQI tables:

Proposal 5: The default CQI table for non-RedCap UE (i.e. Table 5.2.2.1-2 in TS 38.214) is also the default CQI table for RedCap UE.

Proposal 6: The default CQI table for non-RedCap UE is mandatory for RedCap UE, while the 256QAM CQI table and low SE CQI table are optional for RedCap UE.

Conclusion

In this contribution, we provide our view on DL MIMO layers and MCS/CQI tables for RedCap UE. The observation and proposals are summarized as follows:

Observation 1: The maximum number of DL MIMO layers for RedCap UE is already clear.

Proposal 1: Modification of DCI fields/formats for reduced DL MIMO layers is not considered.

Proposal 2: Modification of CSI measurement/reporting mechanism for reduced DL MIMO layers is not considered.

Proposal 3: The default MCS table for non-RedCap UE (i.e. Table 5.1.3.1-1 in TS 38.214) is also the default MCS table for RedCap UE.

Proposal 4: The default MCS table for non-RedCap UE is mandatory for RedCap UE, while the 256QAM MCS table and low SE MCS table are optional for RedCap UE.

Proposal 5: The default CQI table for non-RedCap UE (i.e. Table 5.2.2.1-2 in TS 38.214) is also the default CQI table for RedCap UE.

Proposal 6: The default CQI table for non-RedCap UE is mandatory for RedCap UE, while the 256QAM CQI table and low SE CQI table are optional for RedCap UE.

Reference

- RP-202933, New WID on support of reduced capability NR devices, Ericsson, RAN#90e, December 7th – 11th, 2020.

- RP-210918, Revised WID on support of reduced capability NR devices, Ericsson, RAN#91e, March 6th – 16th, 2021.

- Chairman's Notes RAN1#104-e, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021.

- Chairman's Notes RAN1#104bis-e, 3GPP TSG RAN WG1 Meeting #104bis-e, e-Meeting, April 12th – April 20th, 2021.