3GPP TSG RAN WG1 Meeting #105-e	R1-2104585

e-Meeting, May 10th – 27th, 2021

Source:	ZTE

Title:	Enhancements on Multi-beam Operation

Agenda Item:	8.1.1

Document for:	Discussion and Decision

- Introduction

In RAN1#104b-e, enhancements on multi-beam operation were discussed and agreements [1] were reached on unified TCI framework, L1/L2-centric inter-cell mobility, UL fast panel selection and enhancement related to MP-UE. In this contribution, we analyze further details and provide our views on these topics.

- Enhancement for efficient DL/UL beam management

  - Unified TCI framework for both DL and UL

The objective of unified TCI framework is to achieve a flexible beam update scheme for DL/UL data and control transmission/reception with low signalling overhead and latency. The at least one source RS of a TCI state provide a reference for determining QCL assumption and/or spatial filter.

    - Configuration framework for unified TCI

In previous RAN1 meetings, it was agreed that both joint and separate TCI state indication schemes are supported in Rel-17. Meanwhile, the common RRC configured TCI state pool for joint TCI state indication has been agreed. And it is straightforward to support a common TCI state set activated in a MAC CE for joint TCI state indication scheme.

But, for separate DL and UL TCI state indication, whether a common or a separate RRC TCI state pool should be adopted is still FFS. Besides, whether a common or a separate TCI state sets activated in MAC CE is also FFS.

- Regarding RRC configured TCI state pool for separate DL and UL TCI state indication scheme, we suggest a common RRC configured TCI state pool. Because if there are two separate pools for DL and UL TCI state indication, a total independent configuration for UL TCI state (related to all candidate UL beams, e.g., up to 64 states) is needed, like Rel-15/16 spatialRelationInfo configuration. It may cause a huge RRC overhead and redundancy while considering DL TCI state configuration.

- As for MAC CE activated TCI state set for separate DL and UL TCI state indication scheme, it seems that a single common TCI state set in MAC-CE is also sufficient, like reusing the TCI state pool activated for PDSCH by MAC CE.

  - A common TCI state set in MAC-CE can comprise at least one TCI state without specifying for DL or UL. In this case, DCI which indicates the TCI state (codepoint) would further indicate the mode for DL or UL. It may be suitable for a DCI without DL assignment and can provide more flexibility. Meanwhile, for DCI with DL assignment, the highest bit of 3-bit TCI field can be used for indicating the mode for DL or UL, and the rest 2-bit is to indicate a TCI state (codepoint).

  - Alternatively, a common TCI state set can comprise at least one TCI state with specifying for DL or UL, which means DL TCI state and UL TCI state are in the same set. In such case, it may be suitable for a normal DCI with DL assignment and without DL assignment but some flexibility especially for DCI without DL assignment may be degraded.

Proposal 1: The common TCI state pool in RRC should be supported for both joint and separate DL and UL TCI state indication.

Proposal 2: The common TCI state/codepoint set in MAC CE should be supported for both joint and separate DL and UL TCI state indication.

- For separate indication, one of the following alternatives could be supported

  - Alt1: TCI state/codepoint in MAC-CE is NOT specified for DL or UL, and then the mode for DL or UL of the TCI state/codepoint is indicated by DCI.

  - Alt2: TCI state/codepoint in MAC-CE is specified for DL or UL, which means that the legacy TCI field in DCI can be reused but with some flexibility degradation.

    - Power control for UL TCI state

Regarding power control for unified TCI state, there were agreements as follows in RAN1#104b-e.

Agreement On Rel.17 unified TCI framework, in RAN1#105-e, further discuss to down select or combine from the following three alternatives for PL-RS (note: the text below is based on the agreed description in RAN1#104-e): - AltA. PL-RS can be included in UL TCI state (or, if applicable, joint TCI state). - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - AltB. PL-RS can be associated with (but not included in) UL TCI state (or, if applicable, joint TCI state) - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - AltC. UE calculates path-loss based on periodic DL RS configured as the source RS for determining spatial TX filter in UL or (if applicable) joint TCI state - FFS: If a PL RS is not included in or associated with the UL TCI state (or, if applicable, joint TCI state), whether the UE can estimate path-loss based on the PL-RS of an UL RS provided in an UL TCI state (or, if applicable, joint TCI state) as a source RS for determining the spatial TX filter. In addition: - FFS (to be decided in RAN1#105-e) whether a fallback scheme is needed and, if so, the details - FFS: Support additional UE capability to report whether above PLRS determination mechanism is supported - Note: As agreed in RAN1#104-e, the total number of maintained PL-RSs per CC is no more than 4 - FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates based on UE capability - FFS: UE capability for maximum number of active PL-RS across CCs per band Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, for each of PUSCH, PUCCH, and SRS, in RAN1#105-e, further discuss to down-select or combine from the following alternatives: - AltA. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - AltB. The setting of (P0, alpha, closed loop index) is also included with UL or (if applicable) joint TCI state - AltC. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state Note: It has been agreed that the setting of (P0, alpha, closed loop index) is associated with UL channel or UL RS (therefore the setting is channel- and signal-specific).

From above agreements, a TCI state which can be used for UL may be associated with power control (PC) parameters involving P0/alpha, PL RS, and closed loop index. For backward compatibility, the UL PC parameter can be independently configured and be mapped with a TCI state, if required, instead of adding the UL PC parameter directly into RRC parameter TCI-State. In such case, there may be no further change for the current RRC structure of TCI state.

The PL RS can be applied to all types of UL channel/RSs, but P0/alpha and closed loop index should be further channel/RS-specific as in Rel-15/16.

- Regarding PL RS, both the explicit and implicit methods of determining PL RS should be supported for different purposes.

  - Considering that DL RS corresponding to PL RS and QCL Type-D may be different due to the imbalance of number of activated PL RS (up to 4) and activated TCI state (up to 8) and different time-domain behavior requirement for PL RS (periodic only) and TCI state (all types), the explicit method through configuring PL RS as in Rel-15 is useful.

  - But, in the case that the number of activated TCI state is less than or equal to the maximum number of activated PL RS and a periodic RS is configured as in QCL-Type D RS, the implicit method seems to be more efficient with lower RRC signaling overhead and UE RS monitoring flexibility.

- Regarding other PC parameters (P0/alpha, and closed loop index):

  - If a TCI state is applied to a PUCCH/SRS, the other PC parameters for PUCCH/SRS associated with the TCI state should be applied to the PUCCH/SRS accordingly.

  - If a TCI state is applied to a PUSCH which may refer to one or more SRS resources for CB or non-CB, the other PC parameters associated with the TCI state can be directly applied to the subsequent PUSCH transmission rather than a legacy Rel-15/Rel-16 approach of deriving other PC parameters mapped to the indicated SRI, considering all SRS resources for CB or non-CB in the set correspond to the same unified TCI state (i.e., the same Tx beam) in the unified TCI framework.

Therefore, the above AltB+AltC for PL-RS and AltA for other PC parameters are preferred due to flexibility and simplicity.

Proposal 3: Beam-specific power control (involving P0/alpha, PL RS, and closed loop index) is supported (analogous to Rel.15/16)

- For PUSCH, PUCCH and SRS resource set (for CB, NCB and AS(antenna switching)), PL-RS is determined according to the indicated UL TCI state or (if applicable) joint TCI state (AltB+AltC is supported)

  - PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state

  - If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state

- For PUSCH, PUCCH and SRS resource set (for CB, NCB and AS), other PC parameters (involving P0/alpha and closed loop index) is determined according to the indicated UL TCI state or (if applicable) joint TCI state (AltA)

  - Other PC parameters (involving P0/alpha and closed loop index) can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state

    - Common beam update cross carriers

In RAN1#103-e meeting, a general framework for common DL/UL beam indication cross carriers (CCs) under the unified TCI framework was agreed as follows. In this subsection, we further discuss the details on common beam update cross CCs.

Agreement On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs: - The above applies to intra-band CA - The above applies to joint DL/UL and separate DL/UL beam indications - Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS - The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs - FFS: The above also applies to inter-band CA - FFS: TCI state pool for CA - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state. - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs - Opt-2: configuring RRC TCI state pool per individual CC - FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels

As a remaining issue, the TCI state pool for CA is still FFS, and the following two candidate solutions for TCI state pool for CA have been identified:

- Opt-1: sharing a single RRC TCI state pool for the set of configured CCs;

- Opt-2: configuring RRC TCI state pool per individual CC.

Specifically, we have the following analysis about the two candidate options above:

- Due to sharing a single RRC TCI state pool for a CC group rather than respective pools per CC, Opt-1 can significantly save RRC overhead over Opt-2, e.g., 1.12kB vs 143.36 kB [2].

- Configuration flexibility of Opt-1 is sufficient for unified TCI indication, considering the identical RS configuration (e.g., TRS) across multiple CCs under the requirement of updating a common beam for all RSs/channels, although losing some flexibility compared with Opt-2. Therefore, gNB only needs to guarantee that the RSs (e.g., TRS) with same ID share the same Tx beam across the CC group, and so the loss related to RRC configuration flexibility should be acceptable.

Observation 1:

- Due to sharing a single RRC TCI state pool for a CC group rather than respective pools per CC, Opt-1 can significantly save RRC overhead over Opt-2, e.g., 1.12kB vs 143.36 kB.

- Configuration flexibility of Opt-1 is sufficient for unified TCI indication, considering the identical RS configuration (e.g., TRS) across multiple CCs under the requirement of updating a common beam for all RSs/channels, although losing some flexibility compared with Opt-2.

Further, for cross-CC beam indication based on a single RRC TCI state pool, we have the following analysis on how to determine QCL-TypeA&D parameters (RSs) in a local (or target) CC/BWP according to the QCL-TypeA&D RS of TCI state in a reference CC/BWP.

- QCL-TypeA RS: Considering that QCL-TypeA parameter can NOT be applied for cross-CC indication due to demodulation requirement for accurate time/frequency tracking, the QCL-TypeA RS in a target CC/BWP needs to be located in the local CC/BWP. The QCL-TypeA RS in a target CC/BWP can be obtained according to the QCL-TypeA RS ID indicated by the TCI state in the reference CC/BWP. Specifically, a CC ID for QCL-TypeA RS is absent in a TCI state, and the CC ID for QCL-TypeA RS is determined according to a target CC of the TCI state, which is shown in Figure 1.

Figure 1 Examples for common TCI state pool, where PDSCH TCI state pool in a reference CC is reused.

- QCL-Type D RS: In Rel-15, when QCL-TypeD RS is a CSI-RS with repetition, QCL-TypeD RS can be applied for cross-CC beam indication. But, when TRS is used as a reference QCL-TypeD RS, there is a rule of same TRS to be applied for both QCL-TypeA and QCL-TypeD. Therefore, the following two candidates are identified based on email discussion:

  - Alt-1: Separate QCL-TypeD RSs can be determined from the common TCI state(s) for each of the CCs, as for above QCL-TypeA.

  - Alt-2: Single QCL-TypeD RS is determined from the common TCI state(s) for set of the CCs, and support enhanced QCL chain, e.g., the QCL-TypeA TRS and, if any, QCL-TypeD TRS, in the same/different CSI-RS resources.

Since the same QCL-TypeD RS are used for all CC(s) as agreed, Alt-2 is more aligned with the existed agreement and the unified TCI framework. In order to avoid the misalignment of Tx beam applied to QCL-TypeA RS in each of CC and the single QCL-TypeD RS for facilitating UE RS monitoring, e.g., in a band, we can have a rule that the QCL-TypeA RS in each of CC in a CC group corresponding to the indicated TCI state should be QCLed with the single QCL-TypeD RS.

Proposal 4: For intra/inter-band CA, support the mechanism of sharing a single RRC TCI state pool for the set of configured CCs.

- Regarding QCL-TypeA, a CC ID for QCL-TypeA RS is absent in a TCI state, and the CC ID for QCL-TypeA RS is determined according to a target CC of the TCI state.

- Regarding QCL-TypeD, a single QCL-TypeD RS is determined from the common TCI state(s) for set of the CCs, and support enhanced QCL chain, i.e., the QCL-TypeA TRS and, if any, QCL-TypeD TRS, in the same/different CSI-RS resources (Alt-2)

  - QCL-TypeA RS(s) in each of CCs corresponding to the indicated TCI state should be QCLed with the single QCL-TypeD RS.

    - Source RS and target RS/channel for DL TCI state

In RAN1#104-e, it was agreed to further study whether more RS types can be allowed as source RS for DL and UL TCI state in Rel-17. Then, in RAN1#104b-e, the following conclusion was reached that no consensus in supporting non-BM CSI-RS other than for tracking and non-BM SRS as source RS types for UL TX spatial filter reference. But source RS for DL TCI state is still open.

Agreement (RAN1#104-e) On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types Conclusion(RAN1#104b-e) On Rel.17 unified TCI framework, at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC, there is no consensus in supporting non-BM CSI-RS other than for tracking and non-BM SRS as source RS types for UL TX spatial filter reference No further discussion in Rel-17

In Rel-16, a DL RS (either CSI-RS or SSB) can be configured as a source RS to represent a DL RX filter,  and a DL/UL RS (CSI-RS, SSB or SRS) can be configured as a source RS to represent an UL TX filter. It is well-known that an UL TX spatial filter can be obtained from a DL RX spatial filter, and it is undisputed that the reversed direction is also viable. This may mean that SRS, at least for BM, can be configured as a source RS to represent a DL RX filter.

- Generally, beam management procedure is preformed from DL measurement firstly. Based on the DL measurement results (e.g., multiple candidate DL Tx beams), SRS is configured with spatialRelationInfo of a reference DL RS, and the SRS should be transmitted using the same spatial domain filter as receiving the reference DL RS by the UE.

- It may be argued that we can use the reference DL RS directly in the TCI state, and benefits of introducing SRS become weak. However, considering beam refinement of SRS around a reference DL RS, like RRC parameter Repetition for CSI-RS, it would benefit DL transmission to allow the refined UL beam sate which is reflected by SRS to be source RS for DL TCI state.

Besides, SSB and CSI-RS for CSI are both downlink signals and should be allowed to be source RS for DL TX filter, especially for CSI-RS for CSI that has already been supported in the Rel-15/16 QCL rule.

As for target RS of DL TCI state or joint DL and UL TCI state, we support aperiodic CSI-RS for CSI and for BM that can be applied based on the default beam mechanism in Rel-15/16. For instance, CSI-RS for CSI is mainly used for DL transmission and most probably share the same beam as DL transmission. However, it should be noticed that periodic and semi-persistent CSI-RS should be precluded for this enhancement.

- Firstly, the periodic CSI-RS is usually configured as a source RS for determining the QCL assumption, and if the periodic CSI-RS is indicated as both reference and target RS, the QCL chain becomes a weird closed-loop.

- Then, in order to save RS overhead, the periodic RS is cell-specific RS from gNB perspective, and we have to experience very huge RS overhead once we allow to configure UE-specific periodic RS for facilitating the dynamic/UE-specific TCI update, unless that the TCI state can further adjust a time-domain offset of CSI-RS for CSI in order to match cell-specific CSI-RS/TCI time pattern in gNB side.

Proposal 5: Regarding source RS and target RS/channels for DL TCI state,

- SSB, CSI-RS for CSI, and/or SRS for BM can be supported as source RS types for DL TCI state;

- Aperiodic CSI-RS for CSI can be supported as target RS types for DL TCI state;

  - Note that periodic and semi-persistent CSI-RS (e.g., periodic/semi-persistent CSI-RS for CSI, and periodic TRS) should NOT be supported as target RS types for DL TCI state

Besides, for RSs/channels (e.g., periodic and semi-persistent CSI-RS, and SRS for BM) to which Rel-17 unified TCI can NOT apply, Rel-15/16 signaling mechanisms are used. For DL, those RSs/channels also share the same/single RRC TCI state pool as for unified TCI framework, and for UL, those RSs/channels can reuse the RRC spatial relation pool as in Rel-15/16.

Proposal 6: For RSs/channels to which Rel-17 unified TCI can NOT apply, Rel-15/16 signaling mechanisms are reused.

- For DL RSs/channels, the same/single RRC TCI state pool as for unified TCI framework can be shared.

- For UL RSs/channels, the RRC spatial relation pool as in Rel-15/16 can be reused.

    - The number of TCI states to be indicated for transmission

Regarding the number of TCI states to provide QCL/spatial relation information in an indication command, i.e. M for DL TCI and N for UL TCI as agreed in RAN1#102e, whether the number should be larger than 1 still remains undecided. In our opinion, the number of TCI states to be indicated for transmission greater than 1 is used for mTRP scenario. For example, in mTRP with S-DCI, a TCI codepoint in a DCI can indicate two TCI states, each of which corresponds to one respective TRP.

Note that it has been discussed several rounds for sTRP (i.e., M=N=1) in recent RAN1 meetings. Remaining issues for sTRP are expected to be addressed in upcoming one or two RAN1 meetings. On the other hand, mTRP is an essential enhancement in Rel-16 and Rel-17 for 5G MIMO, and is reasonable to be supported by unified TCI framework. Regarding the design for DCI and/or MAC CE, even for RRC signaling, how to fulfil the requirements of both sTRP and mTRP is a key consideration when we compare different methods and make final decision for unified TCI framework.

- For example, a new type MAC CE may be introduced for unified TCI framework. If only considering sTRP, the MAC CE will be simple. But if mTRP is introduced for unified TCI framework later, the new MAC CE for sTRP may not be enhanced for supporting mTRP, then an additional new MAC CE may be needed which seems redundant. Therefore, a new unified MAC CE suitable for both sTRP and mTRP is preferred for facilitating forward compatibility.

Proposal 7: Regarding the number of TCI states to be indicated by a single command for DL and/or UL transmission,

- M=N=1 for sTRP should be discussed with high priority

- M>1, N>1 for mTRP should be postponed till August meeting (RAN1#106).

  - Signaling design for DCI based TCI state indication

In this subsection, we discuss signalling design for the DCI based TCI state indication, including application time of the TCI state indication and applicable scope of the indicated TCI state.

    - Application time of TCI state indication

In RAN1#104-e, the applicable timing of TCI state indication has been discussed, and the following agreements have been reached.

Agreement On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered: - Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication - Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication - Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16) - Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A - Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE - FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s

The further latency corresponding to decoding HARQ-ACK information and DCI retransmission is needed for the processing of gNB side. Consequently, for both of the DCI with or without DL assignment, we prefer “Alt2A: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication”.

For the sake of presentation, one example for Alt2A considering ACK mechanism directly in response to DCI is shown in Figure 2. Note that, for semi-static HARQ-ACK codebook, the virtual PDSCH should be assumed in the same slot as the DCI without DL assignment, and the time-domain allocation of the virtual PDSCH within the given slot is determined according to the SLIV of indicated TDRA field.

- Furthermore, considering the flexibility of beam/panel indication/activation, the beam application time can also be dynamically indicated by the DCI, like indication of the start of PDSCH transmission, especially for DCI without DL assignment.

- The consideration on the starting point for applying the unified TCI should be taken on for simplifying the beam indication procedure, and then Rel-15/16 default beam/power control mechanism for PDSCH/PUCCH/SRS should NOT be supported on Rel-17 unified TCI framework.

Proposal 8: Regarding application time of the TCI state indication for both of the DCI with or without DL assignment, the following alternative from RAN1#104-e, should be supported:

- The first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt2A)

  - The beam application time can be indicated by DCI, like Rel-15/16 PDSCH.

  - On Rel.17 unified TCI framework, Rel-15/16 default beam/power control mechanism for PDSCH/PUCCH/ SRS is NOT supported.

  - Note: it was agreed for configuration of beam application time by gNB based on UE capability.

Figure 2 Application time of TCI state indication (i.e., X ms or Y symbols after ACK) for semi-static HARQ-ACK codebook, where virtual PDSCH is assumed in the same slot of the DCI by UE

    - Applicable scope of the indicated TCI state

DCI format 1_1 and 1_2 can be used for both joint and separate DL and UL TCI state indication. An agreement was reached in RAN1#104-e as follows for application scope of the indicated TCI state.

Agreement On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives: - Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI - Details on dynamic indication are FFS - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI - Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling - Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling - Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling - Details on how this is signaled in relation to TCI activation are FFS

Among the above alternatives, Alt1+Alt2B is most flexible, which means that based on UE capability reporting, gNB can configure one of three modes of joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling, and then MAC-CE + DCI can be used for further indicating the applicable scope if the modes of separate DL/UL TCI or both of joint and separate DL/UL TCI is enabled by the RRC.

- Regarding how to dynamically indicate a TCI state as joint DL/UL or separate DL or UL, as analyzed in Section 2.1.1, the applicable scope such as DL only, UL only, joint DL and UL for a TCI state can be specified by a MAC CE, or by a DCI.

Proposal 9: Regarding joint DL/UL TCI v.s. separate DL/UL TCI, Alt1 and Alt2B should be supported.

- A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling

  - When the UE is configured with modes of separate DL/UL TCI or both joint and separate DL/UL TCI, the UE can be dynamically indicated the applicable scope of a TCI state by dynamic signaling.

  - L1/L2-centric inter-cell mobility

    - Dynamic activation/deactivation SSB of non-serving cell

In RAN1#104b-e, there is following agreement about L1/L2-centric inter-cell reporting.

Agreement #1 On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - On the value of K (defined in RAN1#104-e as the number of beam qualities associated at least with non-serving cell(s) can be reported in a single CSI reporting instance), - For the supported maximum value(s) of K, down-select at least one from the following candidates {4, 8, 16} - FFS: whether the maximum value of K is a UE capability - Periodic, semi-persistent, and aperiodic reporting (and the respective measurements) are supported. - Note: Semi-persistent and aperiodic reporting (and their respective measurements) are NW-Initiated Agreement #2 On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, for L1-RSRP measurement and at least aperiodic reporting, investigate and, if needed, specify MAC CE based dynamic activation/deactivation of a subset of higher-layer-configured measurement for non-serving cell SSBs

For semi-persistent and aperiodic reporting for L1/L2-centric inter-cell mobility, the beam measurement related to non-serving cell SSB(s) also can be dynamically activate/deactivate by a MAC-CE in order to save UE complexity. In such case, the UE only need to measure the non-serving cell SSB(s) once the corresponding measurement for semi-persistent or aperiodic reporting is activated by MAC-CE.

- Regarding semi-persistent reporting, the current MAC-CE for activating semi-persistent CSI-RS set can be reused to activate a non-serving cell SSB set. In the MAC-CE, cell information as a new non-serving related parameter set, can be indicated for each SSB resource in a SSB set, rather than indicating TCI state.

  - The non-serving cell information includes PCI, and SSB related information such as half frame index, periodicity, carrier information, SCS information and Case information, wherein the Case includes Case A~Case E which is to determine time pattern of SSB as described in 38.213.

  - Then, there are two candidate solutions for configuring the non-serving cell information for candidate SSB:

    - Alt-1: There are multiple SSB sets corresponding to different non-serving cell, and non-serving cell information can be configured per SSB resource in RRC level;

    - Alt-2: Multiple candidates of non-serving cell information is pre-configured in RRC, and then the MAC-CE further activate one of multiple candidates for an SSB resource in one activated SSB resource set for measurement.  It means that the association between SSB resource and non-serving cell information is performed by MAC-CE instead of RRC.

  - In our views, compared with Alt-1, Alt-2 can significantly save RRC signaling overhead for candidate non-serving cell SSB sets (e.g., up to 64 neighboring cells for UE mobility in Rel-15/16) with the same flexibility. Therefore, Alt-2 is preferred.

- Regarding aperiodic reporting, in RRC level, the TCI state of an aperiodic CSI-RS in current trigger states can be replaced with non-serving cell information accordingly. Specifically, the trigger state includes non-serving cell information for each SSB resource in an SSB resource set. Then, the UE only need to measure the non-serving cell SSB set associated with a trigger state activated by the legacy MAC-CE for activating AP-CSI triggering state(s).

  - In Rel-15/16, there is a rule that the legacy MAC-CE is required only when the number of RRC candidate triggering states is greater than the maximum number of candidate codepoints of CSI request field in DCI. But, for saving the UE complexity for non-serving cell measurement, this rule of disabling the legacy MAC-CE should be removed. To be more specific, even if the number of trigger states configured by RRC is smaller than or equal to , the legacy MAC-CE for activating AP-CSI triggering state(s) is also needed, where NTS is the number of bits of the CSI request field.

Proposal 10: For L1-RSRP measurement for semi-persistent and aperiodic reporting in L1/L2-centric inter-cell mobility and inter-cell mTRP, the measurement of non-serving cell SSB(s) can be activated/deactivated by MAC-CE

- Regarding semi-persistent reporting, non-serving cell information (rather than TCI state in the legacy MAC-CE for SP-CSI-RS activation) can be activated for each SSB resource in a RRC configured SSB set by MAC-CE.

- Regarding aperiodic reporting, aperiodic CSI-RS in a trigger state can be associated with non-serving cell information in RRC (rather than TCI state per aperiodic CSI-RS), and then the legacy MAC-CE for activating AP-CSI triggering sate is reused for dynamically enabling the measurement.

  - Notes that, compared with Rel-15/16, the legacy MAC-CE is always needed, regardless whether the number of RRC candidate triggering states is greater than the maximum number of candidate codepoints of CSI request field in DCI.

    - UE-Initiated L1/L2 reporting of neighboring cell

As shown in section 2.3.1, NW initiated L1/L2 reporting of neighboring cell has been agreed in RAN1#104b-e meeting. Besides NW initiated L1/L2 reporting of neighboring cell, the UE initiated L1/L2 reporting should also be supported. For instance, L1-RSRP reporting can be carried by a MAC-CE after initialized by an event (e.g., when a neighboring cell has a better channel quality than a serving cell). Instead of specifying UE measurement and this triggering event in MAC level, we can consider that the event can be directly initialized from an L3 mobility measurement.

Compared with NW-initiated L1/L2 reporting of neighboring cell, the latency of UE-initiated L1/L2 reporting should be much smaller because the UE can initiate the L1 reporting for a neighboring cell by trigger event without waiting for the signaling from gNB, and meanwhile the L1 reporting on demand can save UE power consumption significantly as BFR procedure. As the trigger event is based on L3 mobility, it can avoid too frequency reporting of neighboring cell. It can get good trade-off between power consumption and reporting in time.

Proposal 11: Besides NW-Initiated reporting, event-driven mechanism for L1/L2-centric inter-cell mobility should be supported

- L1-RSRP reporting involving new candidate beams and non-serving cell information (e.g., PCI) can be carried by a MAC-CE after initialized by an event triggered by L3 mobility measurement.

    - Differential L1-RSRP and transmitting power per non-serving cell

In RAN1#104b-e, some discussion on reporting format for L1/L2-centric inter-cell mobility was made, and then the following agreement was reached.

Agreement On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - In one reporting instance, depending on NW configuration, beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell - FFS: whether this applies to periodic, semi-persistent, and/or aperiodic - FFS: How to report the K beams and corresponding qualities if the Tx power among the non-serving cell and with serving-cell is not the same - Note: The supported numbers of non-serving cells (in terms of measurement/reporting) have not yet been decided. The above description doesn’t imply only one non-serving cell is allowed to be configured for measurement. Nor does this imply that only one non-serving cell is allowed in one reporting instance.

In order to report beams of serving cell and neighboring cell in one reporting and increase diversity and robustness of beams, the offset of L1-RSRP between beams of different cells can be larger than offset of L1-RSRP between beams of one cell. Then, in our views, differential RSRP should be performed between beams of each of non-serving/serving cell. Specifically, the UE reports the largest L1-RSRP per non-serving cell/serving cell using absolute value and then reports the differential L1-RSRP of remaining beams of the non-serving/serving cell using differential L1-RSRP.

Proposal 12: Differential L1-RSRP should be performed per non-serving cell/serving cell, when beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell in a reporting instance.

The transmit power of SSBs from different cells can be different. If the L1/L2 reporting of neighboring cell is for scheduling downlink transmission, the information of transmit power may NOT be needed, and herein the UE selects cell(s) and beam(s) to be reported only based on receive power. However, if the L1/L2 reporting of non-serving cell is used for scheduling uplink transmission, then the transmit power should be considered: the UE selects cell(s) and beam(s) based on difference between receive power and transmit power corresponding to SSB. So, it is recommended that, through configuring different reporting quantity parameters, the gNB can initialize an appropriate CSI-report with sufficient flexibility, e.g., based on L1-RSRP or difference between receiving L1-RSRP and transmit power, like coupling loss.

Proposal 13: For L1/L2 reporting for non-serving cell measurement, gNB can select for a CSI reporting configuration a report quantity from ‘L1-RSRP’ and ‘difference between receiving L1-RSRP and transmit power’ (e.g., like coupling loss).

    - Different TCI state groups associated with  different configuration

There is following agreement in RAN1#104-e about scheme of L1/L2 inter-cell mobility and the LS [3] has been sent to RAN2 after email discussion.

Agreement Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS: RRC configuration issues Serving cell issues C-RNTI issues Issues related to CU-DU split Inter-band CA issues Inter-frequency issues

In our opinion, the serving cell and non-serving cell should be in a same serving cell group instead of in different serving cell groups which is too complexity. There are following schemes to allow the UE communicate with serving cell and non-serving cell simultaneously in a same serving cell group.

- Scheme-1: serving cell and non-serving cell correspond to different serving cells which refer to separate configuration logic units in current specification, i.e a serving cell of a MCG/SCG. The former serving cell just represents a cell with a PCI configured in current ServingCellConfigCommon signaling and the later serving cell represents a serving cell of MCG/SCG. The non-serving cell represents a cell with a PCI which is not configured in any current ServingCellConfigCommon signaling

- Scheme-2: serving cell and non-serving cell corresponds to different BWPs;

- Scheme-3: serving cell and neighboring cell corresponds to the same BWP with different configurations of parameters such as: PCI, SSB information, RNTI, rate mating, power control information, wherein the RNTI comprises C-RNTI, SPS-RNTI, and so on. The power control information can include , close loop index, etc.

Along with the order from Scheme-1 to Scheme-3, the configuration flexibility between non-serving cell and serving cell decreases, the specification impact increases, and the UE complexity decreases. In addition, considering discussion of inter-cell multi-TRP, TCI state can be associated with cell information such as PCI and non-serving cell SSB information. Then different TCI state groups of a BWP can be associated with different cell configurations. So we slightly prefer Scheme-3, and thus the related RRC parameters are applied upon the corresponding TCI indication.

- For example, multiple candidates of non-serving information can be preconfigured by RRC, and then one of the multiple candidates can be associated with one TCI state group, or one SSB resource for measurement as described in section 2.3.1. The association between TCI state groups and non-serving cell information can also be done by MAC-CE. The non-serving cell information is configured in UE-Specific signalling instead of cell specific signalling.

- Note that, if there is no association with non-serving cell information, it implies that the TCI state group corresponds to configurations of the current serving cell, i.e some configurations in common signalling.

Proposal 14: Serving cell and non-serving cell can correspond to the same BWP but can be associated with different TCI state groups, each of which can be associated with a respective parameter configuration (e.g., non-serving cell information) including: PCI, SSB information, RNTI, rate matching, power control information .

- The parameter configuration is applied upon the corresponding TCI indication.

- The association between configuration of cell and TCI state group can be done by MAC-CE.

  - Note: The TCI state group corresponds to configuration of the current serving cell, if the association is absent

- UL beam selection for UE with multiple panels

In Rel-17, it should be enhanced to facilitate UL beam selection for UE(s) equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection.

  - Fast panel selection and antenna switching for UL transmission

In RAN1#104b-e, the following agreements including the support of UE-initiated UL panel selection/activation were reached for facilitating UL panel selection. In this subsection, we discuss the potential enhancement on UE fast panel switching.

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/reporting, down select and/or modify from the following candidates: - Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW - FFS: How to inform through CSI/beam reporting framework - FFS: Detailed design of the correspondence including the conveyed information - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16) - Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID - Note: The association between the new panel ID and the panel entity is determined by the UE - Opt1-3: No additional specification support - The duration in which the above panel entity reference is valid and the respective setting are FFS Note: “panel entity” is only used for discussion purpose Agreement On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following: - UE reporting of panel-specific information as a UE capability, for example: - Information related to the total number of DL/UL panel entities - Information related to the number of (max) antenna ports/layers per panel entity - Information related to the maximum number of resources per panel entity for SRS BM - Information related to panel selection delay - Information related to panel activation delay - UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling - UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling) - UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status - Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation - Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed Agreement On Rel.17 enhancements for MPUE, for codebook based UL transmission, decide by August RAN1 meeting whether to support CB-based SRS resources with different numbers of ports - FFS details (e.g. per resource or per resource set) - Note: the above is not for Rel-16 full power transmission but for Rel-17 panel-specific UL transmission - FFS: non-codebook based UL transmission for MPUE - FFS whether existing BWP switch based mechanism (discussed previously in Rel-16 power saving WI) can serve such purpose

    - UE-initialized UL panel selection/activation

Regarding UE-initiated UL panel selection/activation, UE can also control the panel activation and deactivation as a UE driven event, and, in order to guarantee panel indication for UL transmission, the state of panel, e.g., active or inactive, to be used for DL/UL transmission should be reported through panel-specific reporting to gNB side.

- As agreed above, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s). Straightforwardly, a list of activated DL+UL and DL-only UE panel ID(s) can be reported by UE via group based reporting (i.e., panel-specific reporting as discussed in item 2-c), and the corresponding state of UE panel(s) can be reported together (e.g., a 1-bit flag to represent ‘DL+UL’ or ‘DL only’). One example for state switching for UE panel operation can be found in Figure 3.

- Note that the maximum number of antenna groups and activated antenna groups are based on UE capability signaling, and the global ID for UE panel(s) is used for reporting and indication.

  - For instance, a UE can report that it supports up to 3 UE panels, but only up to 2 UE panels can be activated for DL and UL transmission. Then, a global ID for UE panels, i.e., from 0 to 2, to be reported is used rather than a relative ID within the report.

Figure 3 State switching for UE panel operation

Proposal 15: For UE-initialized UL panel selection/activation, information about state(s) of UE panel(s) can be reported along with panel-specific group based reporting.

- Candidate state(s) of UE panel(s) to be reported comprises active state for both DL and UL, active state for DL only;

- The maximum number of antenna groups and activated antenna groups are based on UE capability signaling, and the global ID for UE panel(s) is used for reporting and indication (Opt1-2)

    - Codebook and non-codebook based PUSCH transmission

Based on the agreed assumption for MPUE, the number of antenna ports and UL power control parameter corresponding to different UE panels may be different. In order to facilitate independent number of ports and independent UL power control for different UE panels, more than one SRS resource set can be introduced for both CB and NCB PUSCH transmission. For CB, the SRS resources from different sets can support different number of ports; for NCB, the SRS sets may have different number of SRS resources.

Besides, the consideration of dynamical TRP switching for different SRS resource sets has been taken on for Rel-17 mTRP-PUSCH transmission, and then the corresponding DCI signaling enhancement can be reused directly.

Proposal 16: For both CB and NCB PUSCH transmission, more than one SRS resource sets can be introduced for facilitating independent number of ports of different UE panel and UL power control

- For CB, the SRS resources from different sets may have different number of ports.

- For NCB, the SRS sets may have different number of SRS resources

- Reuse DCI signaling for Rel-17 mTRP-PUSCH (dynamical TRP switching) for SRS resource set indication besides SRI

    - Inter-panel antenna switching

Also, we need to consider the AP-SRS triggering with a large triggering offset for panel activation, which is similar to AP-CSI-RS beam switching in Rel-15, e.g., 224 or 336 OFDM symbols. In such case, the sounding procedure of antenna switching may be equivalent to that of fast panel switching. For instance, one example for inter-panel antenna switching is described in Figure 4. In such case, there may be different spatial relations applied to the respective UE panels, and the restriction about “same spatial relation for AP-SRS resources in a set for antenna switching” in the current spec may become invalid herein. It can be observed that the working assumption on the architecture of UE panels is very essential for the subsequent enhancement for fast UE panel switching.

Figure 4 Diagram for inter-panel antenna switching

Proposal 17: Sounding procedure for antenna switching can be further studied for supporting UE fast panel switching.

  - FR2 MPE mitigation

In RAN1#104b-e meeting, the following agreement was reached for FR2 MPE mitigation. In this subsection, we discuss the potential enhancement on FR2 MPE mitigation.

Agreement On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options: - Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported. - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - FFS: how to determine the virtual PHR or the modified version. - Opt 1D. {Rel.16 P-MPR based (beam/panel-level)} - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting FFS: If differential report is supported when multiple UL beams are reported in the same report

    - Beam/panel-specific P-MPR

In NR Rel-15, RAN4 introduced P-MPRf,c for reducing the maximum transmission power, i.e., Pcmax, with MPE impacts. In Rel-16, an event-driven P-MPR reporting through PHR MAC-CE was introduced for informing the real-time MPE impact.

- There is no doubt that the MPE impact for respective UL beams may be different, but, unfortunately, there is no beam/panel-specific P-MPR definition in either Rel-15 or Rel-16. Consequently, beam-specific P-MPR should be introduced firstly.

- For instance, “P-MPRf,c” is replaced by “P-MPRs,f,c” where s denotes the UL beam index, e.g., RS resource/spatial relation/panel index.

Proposal 18: Beam/panel-specific P-MPR is introduced in UL power control framework.

- For instance, “P-MPRf,c” is replaced by “P-MPRs,f,c” where s denotes the UL beam/panel index, e.g., TCI state/spatial relation/panel index.

    - Enhancement on UE reporting for FR2 MPE mitigation

On Rel.17 enhancements to facilitate MPE mitigation, UE-initiated (event-driven) reporting for MPE is prioritized over NW-initialized one in general. It is due to the fact that the MPE event is determined by UE side and gNB can NOT identify when MPE-related report is needed. Therefore, in our view, Opt 2A should be precluded considering that it may be more suitable for NW-initialized one, and meanwhile the motivation of introducing a modified L1-RSRP version that accounts for MPE effect, rather than reusing Rel-15 L1-RSRP definition + virtual PHR, is also unclear.

Proposal 19: On Rel.17 enhancements to facilitate MPE mitigation, UE-initiated (event-driven) reporting for MPE is prioritized over NW-initialized one

Regarding two candidate solution of Opt 1A and Opt 1D, we prefer Opt 1A due to the following reason:

- The real UL transmission power is determined by the min value between Pcmax and required power. For PUSCH transmission, we have the following formula for calculating transmission power.

- In general, whether the performance is degraded by MPE impact is up to the real transmission.

  - For instance, the required UL power is 15 dBm for PUSCH transmission i, and consequently there is still no impact even the Pcmax is reduced to 16dBm from 23dBm due to MPE.

- If beam reporting is only based on MPE rather than both MPE and the virtual/real transmission, the UL beam (with low MPE but large path loss) recommended by reporting may be useless, and more power is wasted for keeping the same performance. On the other hand, beam specific PHR reporting, i.e., difference between Pcmax and the required transmission power, can well present the MPE impact transparently, and a UL beam re-indication can be performed by gNB if a low PHR, e.g., 0-dB or negative, is received for the current beam.

For the candidate pool in Opt 1A, there are two alternatives: activated UL/joint TCI set and RRC configured SSBRI(s)/CRI(s) set. The former may have low measurement, but the reported beam domain is limited within an activated UL/joint TCI set which are activated for transmission, e.g., a set of TCI states with beams around a main beam direction. But RRC configured SSBRI(s)/CRI(s) set may have larger coverage for beam selection, and UE may recommend better TCI states considering both MPE and PHR. Therefore, we prefer RRC configured SSBRI(s)/CRI(s) set as candidate pool in Opt 1A.

In order to detect the candidate UL beams with low MPE impact, the virtual PHR should be further enhanced to report for the selected SSBRI(s)/CRI(s). And in such case, the PL value for the virtual PHR calculation is determined according to the SSBRI(s)/CRI(s) instead of using pusch-PathlossReferenceRS-Id = 0, as in Rel-15/16.

- For instance, once the beam/panel-specific P-MPR is beyond the threshold, the virtual PHR is triggered to be reported. The UE shall report SSBRI(s)/CRI(s) along with the virtual PHR with the objective of maximizing PHR value, i.e., minimizing the value of P-MPR and PL.

In order to minimizing spec impacts, its virtual PHR(s) can be reported together with its real PHR reusing current MAC-CE with enhancements of adding more specific fields related to virtual PHR(s).

Proposal 20: Reporting SSBRI(s)/CRI(s) along with a virtual PHR is supported for the purpose of indicating feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account (i.e. RRC configured SSBRI(s)/CRI(s) set as candidate pool in Opt 1A)

- The virtual PHR is determined according to the SSBRI(s)/CRI(s)

- MPE event detection as for R16 PHR/P-MPR MAC-CE can be reused as a baseline.

In addition, common beam updated cross multiple CCs is under discussion as in subsection 2.1.3. MPE which is tightly related to beam direction should also be considered to be same or similar among different CCs for a given beam direction. For the sake of overhead reduction, a set of beam specific MPE (P-MPR) values may be shared among a group of CCs.

- Advanced beam training/tracking with latency reduction

In this section, we elaborate the advanced beam training/tracking with latency reduction.

  - Beam management with reduced DL signaling to reduce latency (Group 1)

In RAN1#104b-e, the following agreement was reached for group 1 to reduce DL signaling to reduce latency.

Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 1: - Opt 1-1A: Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request) - Opt 1-1B: UE-initiated beam selection/activation based on beam measurement (without beam indication or activation from NW) - Opt 1-2: Semi-static NW-configured beam selection (without beam indication and measurement/reporting) - Opt 1-3: SSB grouping to reduce beam training - Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency - Note: Aim for at most one solution for Group 1 in Rel-17 to address issue 6

In the current spec, aperiodic beam measurement/reporting can only be based on one resource set which is limited to one slot only. This imposes the following limitations:

- P1 operation is not supported in aperiodic beam reporting

- Number of beams is limited by the number of downlink symbols in a slot for P2 and P3 operation

Only periodic or semi-persistent CSI-RS can achieve P1 and P2/P3 with large number of beams. Relying only on periodic/semi-persistent signals causes large latency and overhead and reduces the flexibility and efficiency of the system. Besides, the triggering request and the number of candidate RS(s) should be considered to be reported by UE for the assistance of P2/P3 related measurement/report configuration. Therefore, we have the following proposal:

Proposal 21: Regarding Group 1, ‘Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency’ should be supported

- Enabling joint DL TX and RX beam refinement/tracking (P2+P3).

- Additional UE report to aid P2/P3 related measurement/report configuration, such as triggering request, the number of candidate RS(s) or periodicity) should be considered.

  - Reducing activation delay of TCI states and PL-RS (Group 2)

In RAN1#104b-e, the following agreement was reached for group 2 to reduce activation delay of TCI states and PL-RS, and, in this sub-section, we will further elaborate our views on this group.

Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 2: - Opt 2-1A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking - Opt 2-1B: Latency reduction for MAC CE based PL-RS activation - Opt 2-1C: Latency reduction for MAC CE based PUCCH resource/resource group activation - Opt 2-2: Direct SCell TCI state activation - Opt 2-3: Replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update - Opt 2-4: One-shot timing update for TCI state update - Note: Aim for at most one solution for Group 2 in Rel-17 to address issue 6 - Note: At least for Opt 2-1A/B, 2-2, and 2-4, RAN2 and RAN4 will at least have to be involved (some may be exclusively RAN2 and/or RAN4 work)

In Rel-15, there is an additional latency for known beam switching in RAN4 over RAN1 solution. Note that the latency reduction of unknown beam switching is handled in L1/L2-centric inter-cell mobility in Section 2.3

Regarding the activation delay of TCI states,

- In NR the beam of PDSCH/PDCCH can be updated by a MAC-CE, the beam in the  MAC-CE can be applied for PDSCH/PDCCH starting from the first slot after slot n+3Nslotsubframe,µ according to TS 38.214, wherein the UE transmits HARQ-ACK for the PDSCH in slot n.

- But the delay of applying the new TCI state in the MAC-CE is much longer in 38.133 than in 38.214 as shown in Figure 5.

  - The known TCI state switch (i.e., TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement) occurs no later than in slot n+ THARQ +(3 ms +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length and the activation delay of the known TCI state doesn’t include a duration for TL1-RSRP as shown in Figure 5, where Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE, TSSB-proc = 2 ms and TOk = 1 if target TCI state is not in the active TCI state list for PDSCH. Technically, the UE needs to achieve time-frequency synchronization after beam switching.

    - That means, in such case, it additionally includes a latency for waiting for the first transmission of the SSB which is the QCL-TypeA or QCL-TypeC to the TCI state. The additional latency is determined according to the periodicity of SSB which belongs to {5,10, 20,40,80,160}ms.

Figure 5 Applicable timing for updating TCI state(s) in RAN1 and RAN4

Regarding the activation delay of PL RS,

- There is no explicit specification in RAN1 spec although we had RAN1 agreements in Rel-16, and the corresponding requirement is specified in RAN4.

  - The UE shall be able to apply the activated PL RS no later than slot i + THARQ+ 3ms + 5*Ttarget_PL-RS + 2msNR slot length , where THARQ and Ttarget_PL-RS denotes the latency of HARQ feedback and periodicity of PL-RS. That means that before applying the activated PL RS, 5 PL RS measurement should be considered additionally.

Observation 2: For the activation latency of TCI state and PL-RS, it can be observed that, besides for normal latency for MAC-CE activation latency,

- Waiting for the first transmission of the SSB and additional duration of 2 ms should be additionally considered for TCI state activation

- Five PL-RS periodicities and additional duration of 2 ms should be additionally considered for PL-RS switching.

For reducing the duration for the first SSB after beam switching command, e.g., MAC-CE. Some mechanisms of pre-frequency/time synchronization should be further studied. For example, the UE may be configured/activated with one or more SSB(s) in a pool and perform frequency/time synchronization of SSB(s) in the SSB pool. If a SSB corresponding to a new activated TCI state is from the SSB pool, the UE can apply the new TCI state directly without waiting for the first transmission of the SSB. For example the UE can track frequency and time synchronization of 4 SSB, but the activated TCI state only corresponds to 2 SSB with up to 8 TCI states. Then the UE can keep the SSB pool with 4 SSBs.

Proposal 22: Regarding Group 2, Opt-2A/B ‘Latency reduction for MAC CE based TCI state/PL-RS activation, or frequency/time/beam tracking’ should be supported

- UE can maintain a different/independent pool of QCL properties and PL-RSs for a time period from the pool of activated TCI state(s) by MAC-CE, and then the maintained QCL/PL-RS can be applied directly upon indication, e.g., preventing from ‘waiting for first SSB’ or ‘ first RS periodicities‘ after MAC-CE.

- Conclusion

In this contribution, we discuss the enhancements on multi-beam operation in Rel-17. We have the following observations and proposals.

Unified TCI framework for both DL and UL

Observation 1:

- Due to sharing a single RRC TCI state pool for a CC group rather than respective pools per CC, Opt-1 can significantly save RRC overhead over Opt-2, e.g., 1.12kB vs 143.36 kB.

- Configuration flexibility of Opt-1 is sufficient for unified TCI indication, considering the identical RS configuration (e.g., TRS) across multiple CCs under the requirement of updating a common beam for all RSs/channels, although losing some flexibility compared with Opt-2.

Proposal 1: The common TCI state pool in RRC should be supported for both joint and separate DL and UL TCI state indication.

Proposal 2: The common TCI state/codepoint set in MAC CE should be supported for both joint and separate DL and UL TCI state indication.

- For separate indication, one of the following alternatives could be supported

  - Alt1: TCI state/codepoint in MAC-CE is NOT specified for DL or UL, and then the mode for DL or UL of the TCI state/codepoint is indicated by DCI.

  - Alt2: TCI state/codepoint in MAC-CE is specified for DL or UL, which means that the legacy TCI field in DCI can be reused but with some flexibility degradation.

Proposal 3: Beam-specific power control (involving P0/alpha, PL RS, and closed loop index) is supported (analogous to Rel.15/16)

- For PUSCH, PUCCH and SRS resource set (for CB, NCB and AS(antenna switching)), PL-RS is determined according to the indicated UL TCI state or (if applicable) joint TCI state (AltB+AltC is supported)

  - PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state

  - If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state

- For PUSCH, PUCCH and SRS resource set (for CB, NCB and AS), other PC parameters (involving P0/alpha and closed loop index) is determined according to the indicated UL TCI state or (if applicable) joint TCI state (AltA)

  - Other PC parameters (involving P0/alpha and closed loop index) can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state

Proposal 4: For intra/inter-band CA, support the mechanism of sharing a single RRC TCI state pool for the set of configured CCs.

- Regarding QCL-TypeA, a CC ID for QCL-TypeA RS is absent in a TCI state, and the CC ID for QCL-TypeA RS is determined according to a target CC of the TCI state.

- Regarding QCL-TypeD, a single QCL-TypeD RS is determined from the common TCI state(s) for set of the CCs, and support enhanced QCL chain, i.e., the QCL-TypeA TRS and, if any, QCL-TypeD TRS, in the same/different CSI-RS resources (Alt-2)

  - QCL-TypeA RS(s) in each of CCs corresponding to the indicated TCI state should be QCLed with the single QCL-TypeD RS.

Proposal 5: Regarding source RS and target RS/channels for DL TCI state,

- SSB, CSI-RS for CSI, and/or SRS for BM can be supported as source RS types for DL TCI state;

- Aperiodic CSI-RS for CSI can be supported as target RS types for DL TCI state;

  - Note that periodic and semi-persistent CSI-RS (e.g., periodic/semi-persistent CSI-RS for CSI, and periodic TRS) should NOT be supported as target RS types for DL TCI state

Proposal 6: For RSs/channels to which Rel-17 unified TCI can NOT apply, Rel-15/16 signaling mechanisms are reused.

- For DL RSs/channels, the same/single RRC TCI state pool as for unified TCI framework can be shared.

- For UL RSs/channels, the RRC spatial relation pool as in Rel-15/16 can be reused.

Proposal 7: Regarding the number of TCI states to be indicated by a single command for DL and/or UL transmission,

- M=N=1 for sTRP should be discussed with high priority

- M>1, N>1 for mTRP should be postponed till August meeting (RAN1#106).

Signaling design for DCI based TCI state indication

Proposal 8: Regarding application time of the TCI state indication for both of the DCI with or without DL assignment, the following alternative from RAN1#104-e, should be supported:

- The first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication (Alt2A)

  - The beam application time can be indicated by DCI, like Rel-15/16 PDSCH.

  - On Rel.17 unified TCI framework, Rel-15/16 default beam/power control mechanism for PDSCH/PUCCH/ SRS is NOT supported.

  - Note: it was agreed for configuration of beam application time by gNB based on UE capability.

Proposal 9: Regarding joint DL/UL TCI v.s. separate DL/UL TCI, Alt1 and Alt2B should be supported.

- A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling

  - When the UE is configured with modes of separate DL/UL TCI or both joint and separate DL/UL TCI, the UE can be dynamically indicated the applicable scope of a TCI state by dynamic signaling.

L1/L2-centric inter-cell mobility

Proposal 10: For L1-RSRP measurement for semi-persistent and aperiodic reporting in L1/L2-centric inter-cell mobility and inter-cell mTRP, the measurement of non-serving cell SSB(s) can be activated/deactivated by MAC-CE

- Regarding semi-persistent reporting, non-serving cell information (rather than TCI state in the legacy MAC-CE for SP-CSI-RS activation) can be activated for each SSB resource in a RRC configured SSB set by MAC-CE.

- Regarding aperiodic reporting, aperiodic CSI-RS in a trigger state can be associated with non-serving cell information in RRC (rather than TCI state per aperiodic CSI-RS), and then the legacy MAC-CE for activating AP-CSI triggering sate is reused for dynamically enabling the measurement.

  - Notes that, compared with Rel-15/16, the legacy MAC-CE is always needed, regardless whether the number of RRC candidate triggering states is greater than the maximum number of candidate codepoints of CSI request field in DCI.

Proposal 11: Besides NW-Initiated reporting, event-driven mechanism for L1/L2-centric inter-cell mobility should be supported

- L1-RSRP reporting involving new candidate beams and non-serving cell information (e.g., PCI) can be carried by a MAC-CE after initialized by an event triggered by L3 mobility measurement.

Proposal 12: Differential L1-RSRP should be performed per non-serving cell/serving cell, when beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell in a reporting instance.

Proposal 13: For L1/L2 reporting for non-serving cell measurement, gNB can select for a CSI reporting configuration a report quantity from ‘L1-RSRP’ and ‘difference between receiving L1-RSRP and transmit power’ (e.g., like coupling loss).

Proposal 14: Serving cell and non-serving cell can correspond to the same BWP but can be associated with different TCI state groups, each of which can be associated with a respective parameter configuration (e.g., non-serving cell information) including: PCI, SSB information, RNTI, rate matching, power control information .

- The parameter configuration is applied upon the corresponding TCI indication.

- The association between configuration of cell and TCI state group can be done by MAC-CE.

  - Note: The TCI state group corresponds to the current serving cell, if the association is absent

UL beam selection for UE with multiple panels and MPE

Proposal 15: For UE-initialized UL panel selection/activation, information about state(s) of UE panel(s) can be reported along with panel-specific group based reporting.

- Candidate state(s) of UE panel(s) to be reported comprises active state for both DL and UL, active state for DL only;

- The maximum number of antenna groups and activated antenna groups are based on UE capability signaling, and the global ID for UE panel(s) is used for reporting and indication (Opt1-2)

Proposal 16: For both CB and NCB PUSCH transmission, more than one SRS resource sets can be introduced for facilitating independent number of ports of different UE panel and UL power control

- For CB, the SRS resources from different sets may have different number of ports.

- For NCB, the SRS sets may have different number of SRS resources

- Reuse DCI signaling for Rel-17 mTRP-PUSCH (dynamical TRP switching) for SRS resource set indication besides SRI

Proposal 17: Sounding procedure for antenna switching can be further studied for supporting UE fast panel switching.

Proposal 18: Beam/panel-specific P-MPR is introduced in UL power control framework.

- For instance, “P-MPRf,c” is replaced by “P-MPRs,f,c” where s denotes the UL beam/panel index, e.g., TCI state/spatial relation/panel index.

Proposal 19: On Rel.17 enhancements to facilitate MPE mitigation, UE-initiated (event-driven) reporting for MPE is prioritized over NW-initialized one

Proposal 20: Reporting SSBRI(s)/CRI(s) along with a virtual PHR is supported for the purpose of indicating feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account (i.e. RRC configured SSBRI(s)/CRI(s) set as candidate pool in Opt 1A)

- The virtual PHR is determined according to the SSBRI(s)/CRI(s)

- MPE event detection as for R16 PHR/P-MPR MAC-CE can be reused as a baseline.

Advanced beam training/tracking with latency reduction

Observation 2: For the activation latency of TCI state and PL-RS, it can be observed that, besides for normal latency for MAC-CE activation latency,

- Waiting for the first transmission of the SSB and additional duration of 2 ms should be additionally considered for TCI state activation

- Five PL-RS periodicities and additional duration of 2 ms should be additionally considered for PL-RS switching.

Proposal 21: Regarding Group 1, ‘Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency’ should be supported

- Enabling joint DL TX and RX beam refinement/tracking (P2+P3).

- Additional UE report to aid P2/P3 related measurement/report configuration, such as triggering request, the number of candidate RS(s) or periodicity) should be considered.

Proposal 22: Regarding Group 2, Opt-2A/B ‘Latency reduction for MAC CE based TCI state/PL-RS activation, or frequency/time/beam tracking’ should be supported

- UE can maintain a different/independent pool of QCL properties and PL-RSs for a time period from the pool of activated TCI state(s) by MAC-CE, and then the maintained QCL/PL-RS can be applied directly upon indication, e.g., preventing from ‘waiting for first SSB’ or ‘first RS periodicities‘ after MAC-CE.

- References

- Chairman's Notes RAN1#104b-e final

- R1-2102667, Further details on Multi-beam and Multi-TRP operation, ZTE

- R1-2102248, LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility, Samsung