3GPP TSG RAN WG1 Meeting #105-e	R1-2104649

May 10th – 27th, 2021

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues in Physical Layer Procedure

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining issues and corrections for NR V2X specifications regarding physical layer procedure.

# Power Control for Groupcast and Broadcast

The power control procedure defined in TS 38.213 has cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) when sl-P0-PSSCH-PSCCH is provided but the transmission is groupcast or broadcast as can be observed from the following definitions:

elseif sl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm]

- if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm]

Observation 1: The current power control procedure has a cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided.

Proposal 1: adopt the following text proposal in 38.213 to resolve the cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided.

| Reason for change: | Current specifications has a cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided. |
| --- | --- |
| Summary of change: | Only utilize PPSSCH,SL(i) in calculation of PPSSCH,D(i) for unicast transmissions and when sl-P0-PSSCH-PSCCH is provided. |
| Consequences if not approved: | Cyclic dependency in specifications leading to infeasible or undefined UE behavior. |
| Clauses affected: | 16.2.1 |

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.2.1 PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX;

-	if dl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	elseif sl-P0-PSSCH-PSCCH is provided and the PSSCH transmission includes a cast type indicator field indicating unicast

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

>>>> unchanged text omitted <<<<

----------------------------------------------------end text proposal for 38.213----------------------------------------------------

# Conclusion

Observation 1: The current power control procedure has a cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided.

Proposal 1: adopt the following text proposal in 38.213 to resolve the cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided.

| Reason for change: | Current specifications has a cyclic dependency between PPSSCH,D(i) and PPSSCH,SL(i) for groupcast and broadcast transmissions when sl-P0-PSSCH-PSCCH is provided. |
| --- | --- |
| Summary of change: | Only utilize PPSSCH,SL(i) in calculation of PPSSCH,D(i) for unicast transmissions and when sl-P0-PSSCH-PSCCH is provided. |
| Consequences if not approved: | Cyclic dependency in specifications leading to infeasible or undefined UE behavior. |
| Clauses affected: | 16.2.1 |

----------------------------------------------------begin text proposal for 38.213----------------------------------------------------

16.2.1 PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX;

-	if dl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	elseif sl-P0-PSSCH-PSCCH is provided and the PSSCH transmission includes a cast type indicator field indicating unicast

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

>>>> unchanged text omitted <<<<

----------------------------------------------------end text proposal for 38.213----------------------------------------------------

# References

- 3GPP TS 38.214 V16.3.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.3.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.3.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.3.0, NR, Physical Layer Procedures for Data (Release 16).