3GPP TSG RAN WG1 Meeting #105-e	R1- 2104656

May 10th – 27th, 2021

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	Enhancements on beam management for multi-TRP

Document for:	Discussion/Decision

- Introduction

The following agreements have been achieved for enhanced beam management for mTRP in RAN1 #104bis-e.

Agreement For beam reporting option 2 - On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e: - Alt1: Support maximum value N = {1, 2} - Alt2: Support maximum value N = {1, 2, 3, 4} - FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17 - On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e - Alt1: The value of N is fixed by RRC configuration - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE Agreement On CMR resource configuration for beam reporting option 2, adopt the following alternative: - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS: extension to aperiodic CMR resource setting - Each reported beam pair in a single CSI-report consists of M = 2 SSB-RI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset. - Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above. Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC. Agreement - Support S-DCI and M-DCI in TRP-specific BFR in Rel.17 - S-DCI is low priority, M-DCI is high priority - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement Adopt the following beam failure detection criteria for each BFD-RS set - The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold - X is max{minimal periodicity of BFD RS in the set, 2ms} Agreement A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1) Agreement On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

- Enhanced Group Based Beam Report

There are a few remaining topics from last meeting on the maximum number of beam groups (N) in a single report. To accommodate various implementation complexities, the maximum configured N can be up to 4, which can be further limited by corresponding UE capability. In addition, it is possible that the number of reported beam groups is less than N. In this case, it would be sensible for UE to determine the number of actual reported beam groups, which is further indicated in the report.

Proposal 1: On the maximum number of beam pairs (N) in a single CSI report, support RRC configured candidate value of N = {1, 2, 3, 4}.

- Introduce UE capability on the maximum value of N, which limits the configured N.

- Actual reported number of beam pairs in a single report is dynamically determined by UE and is upper bounded by the configured N.

In the context of inter-cell mTRP, group-based beam report should be extended such that the two reported beams per group can be from serving and non-serving cells, respectively. For example, one reported RS can have the root QCL source RS as SSB from a non-serving cell, while the other reported RS can have the root QCL source RS from the serving cell. Both reported beams from serving and non-serving cells in a beam group can be simultaneously received by the UE.

Proposal 2: For group-based beam report in inter-cell mTRP, support that the two reported beams per group can be from serving and non-serving cells, respectively.

Another remaining topic is whether to use different CMR resource sets or subsets to differentiate TRPs. It would be more flexible to use resource sets, since each set can accommodate more resources than a subset. Therefore, more candidate beam pairs can be measured for each reporting instance.

Proposal 3: For group-based beam report, the beam set (or TRP) index per candidate beam is implicitly determined by the corresponding CMR resource set.

- Bit width per reported SSBRI/CRI is determined based on the number of SSB/CSI-RS resources from the associated CMR resource set.

Another potential issue with the current group-based beam reporting framework arises when the report quantity is set to ‘cri-SINR’ or ‘ssb-Index-SINR’. In this scenario, the reported SINR metric of one CRI or SSBRI may not correctly reflect the cross-beam interference due to the other CRI or SSBRI, depending on the CMR and IMR configurations of the respective CRI or SSBRI. Consequently, if the gNB schedules a multi-TRP and/or multi-panel communication using the reported two different CRIs or SSBRIs, the actual L1-SINR seen at the UE would be different, potentially degrading the performance. For addressing this situation, we have the following proposal:

Proposal 4: For L1-SINR based group report, gNB configures multiple candidate beam groups, among which UE reports beam group(s) such that the two beams per group can be received simultaneously.

- The corresponding CMR/IMR per beam in each candidate group should be configured such that the reported L1-SINR per beam reflects cross-beam interference from the other beam in the group.

In R16, when the UE is configured with groupBasedBeamReporting set to ‘enabled’, the supported values for reportQuantity are ‘cri-RSRP’, ‘cri-SINR’, ‘ssb-Index-RSRP’ or ‘ssb-Index-SINR’ of the reported two different CRI or SSBRI. However, the throughput enhancements and/or the macro-diversity benefits using simultaneous multi-beam communication may not be accurately captured using L1-RSRP and/or L1-SINR of the individual resources. For this we propose to support additional report metric, such as sum of CQI that more closely represent the achievable data rate using the reported beam group, when the UE is configured with groupBasedBeamReporting set to ‘enabled’.

Proposal 5: For group-based beam report, it can be considered for UE to report throughput related metric per reported beam group, including sum of CQI, capacity, mutual info.

One remaining issue on the enhanced group based report is whether UE could report the two beams are received with different Rx beams. We believe this info is beneficial to indicate the potential usage for this reported beam group, e.g. for throughput enhancement or diversity. Therefore, gNB can apply the appropriate transmission scheme.

Proposal 6: For group-based beam report, UE can report the two beams are received by the same or different Rx beams.

For the group-based report, gNB may have various purposes for the two reported beams. They could be used to maximize throughput or improve reliability. In R15/16, the beam pair selection is up to UE, which may not be aware of gNB’s intention. If the intention is to maximize throughput, UE may report two gNB Tx beams, which can be simultaneously received by two UE Rx beams with good SINR per Rx beam. If the intention is to maximize reliability, UE may report two gNB Tx beams, which can be simultaneously received by a single UE Rx beam with best combined SINR. In addition, gNB can also instruct UE to order the beam pairs based on certain criterion, e.g. based on sum or minimum of the two SINRs, where the former may favour the total throughput, while the latter may provide better reliability against beam blocking by selecting two beams with similar strengths.

Proposal 7: For group report, gNB can signal the purpose (for throughput or reliability), operation mode (FDM/SDM/TDM), and corresponding beam pair selection criterion (based on sum or minimum of metrics of the two reported beams).

- TRP Specific BFR

As one remaining topic for TRP specific BFR, it would be beneficial to configure either cell-level or TRP-level TRP in a CC to limit complexity. If both are simultaneously configured, UE may need to perform separate BFD measurement and generate separate PHY indicator for both BFR types per CC. The MAC-CE may also need more bits to indicate which BFR type is triggered for a given CC. For TRP specific BFR, it can also indicate cell-level BFR by reporting the failure of both TRPs. Therefore, one type of BFR per CC should be sufficient.

Proposal 8: Either cell-specific or TRP specific BFR, but not both, can be configured in the same CC.

On the maximum RS number per BFD-RS set, it could be up to UE capability with a candidate value of 1 to accommodate various implementation choices.

Proposal 9: Support UE capability on the maximum number of RS per BFD-RS set, which includes a candidate value of 1.

Another remaining issue is when 2 PUCCH-SR resources are configured in a cell group, should they be mapped to different SR IDs or the same SR ID. We slightly prefer the same SR ID to reduce the SR ID number. Upon detection of TRP failure, the same SR ID will be triggered regardless which TRP fails. The 2 PUCCH-SR resources can be configured under the same schedulingRequestConfig, and the PUCCH-SR resource corresponding to the failed TRP is used to transmit the SR.

Proposal 10: When two PUCCH-SR resources are configured in a cell group, they can be mapped to the same schedulingRequestId to avoid two schedulingRequestIds.

- The PUCCH-SR resource associated with the failed BFD-RS set is used to transmit the SR.

In RAN1 #104e, 2 BFD-RS sets have been agreed per DL BWP. A remaining issue is whether the maximum number of resources per BFD-RS set and across all BFD-RS sets per BWP should be fixed or up to UE capability. For flexibility of UE implementation, we prefer UE capability on those parameters.

Proposal 11: Support UE capability on the total number of resources across all BFD-RS sets per DL BWP.

For TRP specific BFR, a remaining issue from #103-e is whether to support explicit and/or implicit BFD RS. Both options have their advantages. The explicit BFD RS can provide gNB more flexibility in the BFD RS determination, while the implicit BFD RS can save signalling overhead for the BFD RS update. Therefore, it would be beneficial to keep both options at least for mDCI mTRP.

Proposal 12: For mDCI mTRP, support both explicit and implicit per-TRP BFD.

For mDCI mTRP, each explicit BFD-RS set and the corresponding NBI-RS set can be configured with the same CORESETPoolIndex, which identifies the 1-to-1 association between them.

Proposal 13: For mDCI mTRP, the 1-to-1 association between each explicit BFD-RS set and the corresponding NBI-RS set can be indicated by configuring the same CORESETPoolIndex for them.

For mDCI mTRP, the set of implicit BFD RSs associated with a CORESETPoolIndex can be the QCL-TypeD RSs in up to X TCI states for CORESETs sharing the same CORESETPoolIndex. In addition, in absence of QCL-TypeD RS in the TCI state, the single QCL source RS in the TCI state can be the implicit BFD RS. The same rule has been applied to the legacy cell level BFD.

Proposal 14: For mDCI mTRP, the implicit BFD RSs associated with a CORESETPoolIndex can be the QCL-TypeD RSs in up to X TCI states for CORESETs sharing the same CORESETPoolIndex.

- X can be determined in spec or via UE capability.

- In absence of QCL-TypeD RS, the single QCL source RS in the TCI state can be the implicit BFD RS.

As agreed in RAN1 #104e, up to two dedicated PUCCH-SR resources can be configured in a cell group. On the number of spatial relations per PUCCH-SR, two candidate spatial relations can be configured for the single PUCCH-SR and can be associated with different CORESETPoolIndex in case of mDCI mTRP. If one spatial relation is associated with the CORESETPoolIndex of a failed TRP, UE will select the other for PUCCH-SR transmission. If none of the two spatial relations is associated with a failed TRP, UE can select any one spatial relation for transmission. This may require gNB to monitor both spatial relations simultaneously.

To further reduce PUCCH overhead, the same PUCCH-SR can be shared by both per-TRP BFR and SCell BFR, i.e. it can be triggered whenever per-TRP BFR or SCell BFR happens for any CC. This way does not need to define separate PUCCH-SR for per-TRP BFR and SCell BFR. For PCell BFR, it may have to rely on RACH procedure as in current spec.

Proposal 15: For each PUCCH-SR resource in a cell group:

- Two candidate spatial relations can be configured for the one PUCCH-SR resource.

- Among the two spatial relations, UE can select the one not associated with failed TRP for transmission.

- The same PUCCH-SR can be triggered by both per-TRP BFR and SCell BFR.

Similar to PUCCH-SR, the existing BFR MAC-CE can be modified to be shared by both per-TRP BFR and PCell/SCell BFR. Specifically, each CC can be configured for per-TRP BFR, SCell BFR, or PCell BFR. Whenever per-TRP BFR or SCell BFR happens to any CC but without cell level failure of PCell, the PUCCH-SR will be triggered for UL grant to transmit the modified BFR MAC-CE. However, in presence of PCell BFR, the RACH procedure will be triggered to transmit the modified BFR MAC-CE in Msg3/A.

Once transmitted, the modified BFR MAC-CE can indicate corresponding BFR info per CC based on the configured BFR type. For example, if a CC is configured with per-TRP BFR, the failed TRP ID(s) of the two TRPs can be indicated by a 2-bit field reserved for this CC. If a CC is configured with PCell or SCell BFR, the failed CC ID can be indicated by a 1-bit field reserved for this CC. The corresponding new beam ID per failed TRP/CC can be listed sequentially after the fixed fields indicating the failed TRP/CC IDs. The above indication of failed TRP/CC ID and corresponding new beam is similar to the structure of the existing BFR MAC-CE.

Proposal 16: A CC can be configured with either per-TRP BFR or PCell/SCell BFR. A modified BFR MAC-CE from R16 can indicate corresponding BFR info per CC based on the configured BFR type.

- If a CC is configured with per-TRP BFR, MAC-CE will indicate CC ID, failed TRP ID, new beam ID.

- If a CC is configured with SCell BFR, MAC-CE will indicate failed CC ID and new beam ID.

- If a CC is configured with PCell BFR, MAC-CE will indicate the failure event.

In addition to the cell level failure of PCell, UE may have to rely on the RACH procedure to transmit the modified BFR MAC-CE if no dedicated PUCCH-RS is configured.

Proposal 17: If no dedicated PUCCH-SR or if cell level failure of PCell happens, the modified BFR MAC-CE can be transmitted via RACH procedure.

After transmitting the modified BFR MAC-CE, UE is expected to receive the corresponding gNB response, which can be the next DCI scheduling new PUSCH transmission with same HARQ ID as the PUSCH carrying the modified BFR MAC-CE. Similar to SCell BFR, after 28 symbols form receiving the gNB response, both gNB and UE will reset the beam(s) and/or power control parameters at least for PDCCH/PUCCH associated with the failed TRP, if a new candidate beam is reported for the failed TRP in the modified BFR MAC-CE. In addition, in presence of PDCCH repetition, the 28 symbols start from the last repetition. If the CC for BFR response and the CC with failed TRP have different SCSs, the SCS of the 28 symbols can be determined as the largest SCS of them.

Proposal 18: After 28 symbols from receiving the gNB response, at least PDCCH/PUCCH beam will be reset for the failed TRP if a candidate beam is reported for it.

- In presence of PDCCH repetition, the 28 symbols start from the last repetition.

- The SCS of the 28 symbols is the largest SCS of the response receiving CC and the CC with failed TRP.

- Other Enhancements for mTRP BM

In R15, if a UE, configured for single cell operation or intra-band CA, monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETS that have QCL-TypeD properties on active DL BWPs of one or more cells, the prioritization rule is specified. That is, the UE monitors PDCCHs only in a CORESET (or multiple CORESETS having same QCL-TypeD as the CORESET) corresponding to the CSS set with the lowest index in the cell with the lowest index containing CSS. If no cell contains CSS, the UE monitors PDCCH in a CORESET corresponding to the USS set with the lowest index in the cell with lowest index.

In case of mDCI based mTRP, the above QCL prioritization rule for overlapped CORESETs can be extended such that one QCL is prioritized per TRP by apply the rule among the CORESETs associated with the same TRP. Therefore, PDCCHs from two TRPs can be received simultaneously at the UE.

Proposal 19: In multi-DCI based mTRP, the existing QCL prioritization rule for overlapped CORESETs should be applied within CORESETs with same CORESETPoolIndex.

In R16, simultaneous dual default PDSCH beams have been introduced in multi-TRP. The two features related to multi-DCI based and single-DCI based multi-TRP/panel transmission are agreed to be part of UE capability. While these features are indicated by separate UE capability, the UE may prefer to disable it for power saving purposes. For example, a capable UE with short remaining battery life may only activate a single Rx panel at one time and keep the other panels in sleep mode. In this scenario, it would be beneficial for UE to recommend disabling the dual default PDSCH beam operation.

Proposal 20: For power saving, UE can request to disable the dual default PDSCH beams in mTRP.

- Conclusion

In this contribution we discussed beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception. Below are the observations and proposals.

Proposal 1: On the maximum number of beam pairs (N) in a single CSI report, support RRC configured candidate value of N = {1, 2, 3, 4}.

- Introduce UE capability on the maximum value of N, which limits the configured N.

- Actual reported number of beam pairs in a single report is dynamically determined by UE and is upper bounded by the configured N.

Proposal 2: For group-based beam report in inter-cell mTRP, support that the two reported beams per group can be from serving and non-serving cells, respectively.

Proposal 3: For group-based beam report, the beam set (or TRP) index per candidate beam is implicitly determined by the corresponding CMR resource set.

- Bit width per reported SSBRI/CRI is determined based on the number of SSB/CSI-RS resources from the associated CMR resource set.

Proposal 4: For L1-SINR based group report, gNB configures multiple candidate beam groups, among which UE reports beam group(s) such that the two beams per group can be received simultaneously.

- The corresponding CMR/IMR per beam in each candidate group should be configured such that the reported L1-SINR per beam reflects cross-beam interference from the other beam in the group.

Proposal 5: For group-based beam report, it can be considered for UE to report throughput related metric per reported beam group, including sum of CQI, capacity, mutual info.

Proposal 6: For group-based beam report, UE can report the two beams are received by the same or different Rx beams.

Proposal 7: For group report, gNB can signal the purpose (for throughput or reliability), operation mode (FDM/SDM/TDM), and corresponding beam pair selection criterion (based on sum or minimum of metrics of the two reported beams).

Proposal 8: Either cell-specific or TRP specific BFR, but not both, can be configured in the same CC.

Proposal 9: Support UE capability on the maximum number of RS per BFD-RS set, which includes a candidate value of 1.

Proposal 10: When two PUCCH-SR resources are configured in a cell group, they can be mapped to the same schedulingRequestId to avoid two schedulingRequestIds.

- The PUCCH-SR resource associated with the failed BFD-RS set is used to transmit the SR.

Proposal 11: Support UE capability on the total number of resources across all BFD-RS sets per DL BWP.

Proposal 12: For mDCI mTRP, support both explicit and implicit per-TRP BFD.

Proposal 13: For mDCI mTRP, the 1-to-1 association between each explicit BFD-RS set and the corresponding NBI-RS set can be indicated by configuring the same CORESETPoolIndex for them.

Proposal 14: For mDCI mTRP, the implicit BFD RSs associated with a CORESETPoolIndex can be the QCL-TypeD RSs in up to X TCI states for CORESETs sharing the same CORESETPoolIndex.

- X can be determined in spec or via UE capability.

- In absence of QCL-TypeD RS, the single QCL source RS in the TCI state can be the implicit BFD RS.

Proposal 15: For each PUCCH-SR resource in a cell group:

- Two candidate spatial relations can be configured for the one PUCCH-SR resource.

- Among the two spatial relations, UE can select the one not associated with failed TRP for transmission.

- The same PUCCH-SR can be triggered by both per-TRP BFR and SCell BFR.

Proposal 16: A CC can be configured with either per-TRP BFR or PCell/SCell BFR. A modified BFR MAC-CE from R16 can indicate corresponding BFR info per CC based on the configured BFR type.

- If a CC is configured with per-TRP BFR, MAC-CE will indicate CC ID, failed TRP ID, new beam ID.

- If a CC is configured with SCell BFR, MAC-CE will indicate failed CC ID and new beam ID.

- If a CC is configured with PCell BFR, MAC-CE will indicate the failure event.

Proposal 17: If no dedicated PUCCH-SR or if cell level failure of PCell happens, the modified BFR MAC-CE can be transmitted via RACH procedure.

Proposal 18: After 28 symbols from receiving the gNB response, at least PDCCH/PUCCH beam will be reset for the failed TRP if a candidate beam is reported for it.

- In presence of PDCCH repetition, the 28 symbols start from the last repetition.

- The SCS of the 28 symbols is the largest SCS of the response receiving CC and the CC with failed TRP.

Proposal 19: In multi-DCI based mTRP, the existing QCL prioritization rule for overlapped CORESETs should be applied within CORESETs with same CORESETPoolIndex.

Proposal 20: For power saving, UE can request to disable the dual default PDSCH beams in mTRP.

- References

- RP-193133 Further enhancement on MIMO for NR