3GPP TSG RAN WG1 #105-e	R1-2104659

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.2.1

Source: 	Qualcomm Incorporated

Title: 	Initial access aspects for NR to support operation between 52.6 GHz and 71 GHz

Document for:	Discussion/Decision

# Introduction

In RAN #90e meeting, a Rel-17 work item for NR operation in a frequency regime between 52.6GHz and 71GHz was approved, and the WID was further revised in [1]. As part of the work item, the study on the physical layer aspects of initial access includes the following:

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

The Appendix lists the agreements made in RAN1#104e and RAN1#104b-e related to initial access.

In this contribution, based on the WID and the additional RAN1 agreements, we present our views on the initial access aspects for NR operation in the frequency regime between 52.6GHz and 71GHz, including design aspects for:

- SSB

- SSB and CORESET0 multiplexing patterns

- RACH

# Discussion

## SSB Design Aspects

According to the WID, the SCS for the SSB includes 120 kHz and, if needed, study additional SCS values:

- 240 kHz, 480 kHz, 960 kHz for initial access

- 480 kHz, 960 kHz for cases other than initial access

### SSB Numerology

In RAN1#104e, the following agreement was made:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

In RAN1#104b-e, the following agreement was made:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

To summarize, what has been agreed so far includes:

- SSB SCS of 120 kHz for initial access case

- SSB SCS of 480 kHz and 960 kHz for non-initial access case

  - SSB location and SCS are explicitly provided to the UE

  - SSB does not configure Type-0 PDCCH

RAN1 can continue to discuss other options for the SSB SCS support, but the additional discussion should not block the progress on the already agreed SSB SCS.

Proposal 1: RAN1 can continue to discuss other options for the SSB SCS support, but prioritize design on the already agreed choices (120 kHz SCS for initial access and 480 kHz and 960 kHz for non-initial access case where SSB location and SCS are explicitly provided to the UE and SSB does not configure Type-0 PDCCH)

In the remaining part of this section, we present our views related to SSB numerology for initial access and non-initial access with ANR including the following:

- UE complexity for initial search

- Timing resolution impact (channels (PDCCH/PDSCH) using timing estimation from SSB signals)

- Performance of the SSB signals/channels

- ANR case

#### UE Complexity for Initial Search

During the initial search (e.g., cold start search, gap search (inter-frequency), and idle mode), a UE searcher may use a sliding window and correlation technique to search for the PSS​ sequence. The PSS sequence is a length 127 frequency domain-based M-sequence (mapped to 127 sub-carriers) ​which can be initialized using 3 possible values. The PSS sequence determines part of the cell ID (NID(2)) which can take 3 possible values. Also, the UE during its PSS search would try multiple frequency hypotheses to account for Doppler, internal clock frequency shifts, and any other frequency errors​. Hence, the initial search for PSS involves a sliding window correlator where for each window time location, the UE tries 3 sequences and multiple frequency hypotheses.

​For higher SCSs, the symbols are shorter (e.g., 960 kHz symbol is 8x shorter compared to 120 kHz), hence, the number of UE correlations for the initial PSS search per unit time increases accordingly. In addition, the buffering requirements and correspondingly the memory size requirement also increases. On the other hand, increasing the SCS may require fewer frequency hypotheses. This depends on the UE implementation and several other factors including the nature of the channel/deployment and the maximum frequency and oscillator error assumptions.

For example, for 60 GHz carrier frequency, if the oscillator requirement is 10 ppm, the maximum frequency error is 600 kHz.

- For 120 kHz SCS, the bin could be [-540:60:540]. 19 bins

- For 960 kHz SCS, the bin could be [-480:480:480]. 3 bins (~ a factor of 8 reduction)

Observation 1: increasing the SSB SCS will have an effect on the UE initial search complexity and potentially increase the UE memory requirements depending on multiple factors including the number of frequency bins needed and the number of correlations in time

#### Timing Resolution Impact on Higher SCS Channels

The timing resolution of the initial cell search needs to be smaller than the CP of the other channels (e.g., PDCCH or PDSCH). The theoretical timing resolution may be approximated by:

| SSB SCS (kHz) | 120 | 480 | 960 |
| --- | --- | --- | --- |
| PSS Symbol bandwidth (MHz) | 17.3 | 69.1 | 138.2 |
| Timing resolution based on PSS (ns) | 57.9 | 14.5 | 7.2 |
| Symbol Containing SSS and DMRS bandwidth (MHz) | 28.8 | 115.2 | 230.4 |
| Timing resolution based on SSS and DMRS from PBCH (ns) | 34.7 | 8.7 | 4.3 |

The UE may use the SSS symbol (using SSS + DMRS) to further refine the timing.

The CP length for SCS 480 kHz and 960 kHz are approximately 146.5 ns and 73.2 ns, respectively. It can be observed that the timing resolution for SSB SCS of 120 kHz is less than the CP of the larger SCS symbols. However, this is based on theoretical calculations which may not consider any implementation aspects for time tracking loops.

Figure 1 and Figure 2 show the performance effect on PDSCH using SCS = 960 kHz that is using the timing estimation from a PSS using SCS = 120 kHz. As can be seen from the figures, the timing acquisition accuracy from a low SCS SSB may not always be sufficient for high SCS PDSCH and may cause some performance degradation.

In the same figures, additional performance curves are shown for using WB PDSCH DMRS for timing offset correction. It can be noted that the performance loss may be recovered. Similar observation may be extended if WB PDCCH DMRS based correction is used.

Another approach to recover the performance loss due to a smaller SCS SSB is to use a cell-specific TRS for additional timing offset correction as shown in Figure 3.

However, for certain conditions, a denser FD TRS design may be needed. This is shown in Figure 4, Figure 5, and Figure 6 where a larger FD TRS density (e.g., 12) may be needed for larger timing offsets.

Hence, we make the following observations:

Observation 2: the timing acquisition accuracy from low SCS SSB (e.g., 120 kHz) may not always be sufficient for high SCS PDSCH​ (e.g., 960 kHz) reception

Observation 3: using a WB DMRS or a cell-specific TRS for timing offset correction may help reduce the performance gap

Observation 4: the FD density for cell-specific TRS may need to be increased to get a good resolution and timing offset correction for high SCS PDSCH​

Proposal 2: for initial access, in cases where the SSB SCS is smaller than other channels’ SCS (e.g., PDCCH/PDSCH), consider WB DMRS or cell-specific TRS for further timing error corrections

- For cell-specific TRS, consider studying the FD density needed

| Figure 1: TTL effect on PDSCH (MCS 7) | Figure 2: TTL effect on PDSCH (MCS 16) |
| --- | --- |
| Figure 3: Timing Offset Correction (TOC) using TRS | Figure 4: TOC using TRS (FD Density = 3) |
| Figure 5: TOC using TRS (FD Density = 6) | Figure 6: TOC using TRS (FD Density = 12) |

#### Performance of the SSB

In [2], the performance of the SSB signals/channels was presented for different SCSs. The following observations were made:

Observation 5: for the PSS and SSS detection performance of different numerologies in the high frequency regime,

- The performance is degraded as the SCS increases due to the higher frequency selectivity.

  - The impact is more pronounced in NLOS channels (i.e., CDL-B and TDL-A) with larger delay spreads: ~2dB loss for 960kHz SCS compared to 120kHz SCS

Observation 6: for the PBCH performance of different numerologies in the high frequency regime,

- The performance is degraded as the SCS increases due to the higher frequency selectivity.

  - The impact is more pronounced in NLOS channels (i.e., CDL-B and TDL-A) with larger delay spreads: ~1.7dB loss for 960kHz SCS compared to 120kHz SCS

#### ANR Case

For the non-initial access ANR case (SSB does configure Type-0 PDCCH), given the arguments above about UE search complexity, the higher SCSs (480/960 kHz) may be supported in this case provided the timing of the SSB is known to the UE within a certain tolerance. This tolerance can be assumed to be the same as the “maximum receive timing difference requirement for inter-band NR carrier aggregation” as defined in Table 7.6.4-2 of TS 38.133.

Based on the above considerations/observations, we propose the following:

Proposal 3: for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE (within limits defined in Table 7.6.4-2 of TS 38.133): support SCS = 480/960 kHz

### SSB Pattern

According to the WID, up to 64 SSB beams is to be supported in this frequency range.

In RAN1#104b-e, the following agreement was made:

“For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB”

For SSBs using SCS = 240 kHz, there is no clear motivation to change the NR Rel-16 SSB pattern, hence, the same SSB pattern used for FR2 in NR Rel-16 specification may be reused.

For SSBs using SCS = 480 kHz or 960 kHz, a shorter symbol length means a shorter CP length. A gNB may need to have some time to switch from one beam to another which can be in the order of [100-200 ns] (this number is to be determined based on RAN4 response to LS [3]). In NR Rel 16, back-to-back SSBs from different beams was possible because the beam switching time can be absorbed in the CP and hence no explicit switching gap is needed (CP for SCS 120 and 240 kHz = 596 and 293 ns, respectively which is > [100 ns] needed). However, for higher SCS SSBs, the CP is smaller (CP for SCS 480 kHz and 960 kHz are 146.5 ns and 73.2 ns, respectively) and the beam switching time may be larger than the CP or takes a considerable portion of the CP. Hence, an explicit switching gap may be needed. Therefore, a simple extension of the existing patterns to the new numerologies may not work and a new pattern may be needed.

The beam switching delay may be in the order of [100-200 ns]. The CP length for SCS 480 kHz and 960 kHz are 146.5 ns and 73.2 ns, respectively, while the usable symbols lengths are 2083.3 ns and 1041.7 ns, respectively. Therefore, a beam switching gap may be in the order of 1 symbol.

Observation 7: explicit beam switching gap between SSBs with larger SCS (SCS = 480 kHz and 960 kHz) may be required and to accommodate these switching gaps, a new SSB pattern may be required

- A symbol-level (1 symbol) SSB beam switching gap may be required

In addition, due to the larger SCS (hence shorter symbols), the DL/UL switching delay is not trivial and may take more than few symbols (e.g., [7.015 µsec] depending on the RAN4 response to LS [3]). Therefore, to account for UL segments within an SSB burst, a considerable number of symbols need to be reserved for DL/UL switching delays.

Observation 8: for larger SSB SCS (480 kHz and 960 kHz), accommodating UL segments within the SSB burst may require accounting for DL/UL switching delays taking considerable number of symbols

To accommodate UL/DL switching and beam switching gaps, an “SSB slot” may be introduced where a repeated slot structure is defined and symbols for one or more SSB beams are contained in the SSB slot with beam switching gaps between them.

In addition, control symbols (e.g., PDCCH) may be also included in these SSB slots. These control symbols may be generic and may not need to be associated with the corresponding SSB beam as long as the design allows for a gap between the control symbols and an SSB of different beams.

Another option is to have a self-contained structure where the SSB and the corresponding CORESET0 may be confined in the same SSB slot.

Some examples for an “SSB slot” are shown in Figure 7.

Figure 7: “SSB Slot” Examples

If 2 SSBs (for example) are allocated in an SSB slot, 32 SSB slots are needed for all 64 beams to be swept. In case all SSB slots are back-to-back with no gap slots in between, the 32 slots span 0.5 ms for 960 kHz SCS. While it is much faster than the 5 ms for SCS 120 kHz, the 0.5 ms may be too long and may block URLLC traffic. Hence, the design may consider having gap slots within the SSB slots of the burst. An example is shown in Figure 8.

Figure 8: SSB Pattern with “Gap Slots” Example

The number of “SSB slots” and the “gap slots” in a burst/span may be such that:

- The burst/span is small enough so the DMTC window is short (not longer than Rel-16)

- The number of gap slots is reasonable for DL/UL transmission TDM multiplexing

- The pattern of SCS 480 and 960 kHz matches in time domain (i.e., number of SSB/gap slots for SCS 960 = 2 x number of SSB/gap slots for SCS 480)

Aligning Beams with Different SCSs

Potentially, a system operating between 52.6 GHz and 71 GHz may be designed to have SSB SCS values of 120, 240, 480, and/or 960 kHz. This system may be deployed with carrier aggregation (CA) where one component carrier (CC) may be using a low SCS (e.g., 120 kHz) and another CC may be using a higher SCS (e.g., 480 or 960 kHz). For e.g., PCell may be using 120 kHz to get a better coverage and can be aggregated with another secondary SCell using 960 kHz for higher data rate.

In such cases, it may be beneficial to align the beam directions of these component cells (PCell and SCell). If the same antenna is used for both cells, a single beam forming can be used at a time. If SSB beams are not aligned, this may lead to beam direction blockage overhead (beam direction needs to be reserved for PCell SSBs and SCell SSBs in different times).

For unlicensed band, it is possible that the EIRP will be limited, and FDM’ing SSB’s from different CCs may limit the power transmitted for each SSBs, hence, TDM’ing the SSBs while minimizing the beam switching needs may be another option to consider.

Figure 9 shows a design for the SSB for higher SCSs (960 kHz) to align with the lower SCS SSBs (120 kHz).

Figure 9: Examples for Nested SSB Structure for Different SCS

Proposal 4: for the SSB for NR operation in the frequency between 52.6GHz and 71GHz and SCS = 480 kHz and 960 kHz, consider defining an SSB pattern consisting of multiple “SSB slots” where SSB symbols for one or more beams are contained in the “SSB slot”

- A beam switching gap of 1 symbol is inserted between SSBs within the “SSB slot”

- Additional control symbols may be defined in the SSB slots with beam switching gaps between control and SSB symbols of different beams

- Additional “gap slots” may be inserted between “SSB slots” to account for URLLC and UL traffic

- Consider the option of aligning the higher SCS SSBs with the corresponding beams for the lower SCS SSB

### DRS Related Aspects

#### The Need for DBTW

A transmission may be classified as a short control signal transmission if its duration does not exceed 10 ms within an observation period of 100 ms. For SCS = 120 kHz, assuming 64 beams and SSB periodicity of 20 ms, the SSB transmission duration is ~ 11.4 ms within a 100 ms period, which exceeds the allowed limit to classify as short control signal transmission.

The maximum number of beams that can be configured without exceeding this limit can be calculated to be 56.

Observation 9: with 120 kHz SCS SSB, the number of transmitted SSB beams may need to be less than 64 in order to classify as short control signal transmission

In RAN1#104b-e, the following was agreed:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

An important part of the agreement is “Duration of DBTW is no greater than 5 ms”. Per the agreement, the DBTW window should be kept to within 5 ms​. With 120 kHz SCS, 64 SSB positions occupy the 5 ms DBTW window​. Maintaining the gaps between the SSBs may be important to facilitate URLLC and UL traffic​. Hence, additional candidate SSB positions may not be available​ within the 5 ms window. Therefore, if we need a design that has more than 1 candidate SSB per beam within this 5 ms DBTW, the number of SSB beams need to be reduced to less than 64.

Observation 10: with 120 kHz SCS SSB, to maintain the gaps between the SSBs to facilitate URLLC and UL traffic, additional candidate SSB positions may not be available​ within the 5 ms DBTW, and to support more than 1 candidate SSB per beam, the number of beams need to be less than 64

From the previous arguments and observations, it can be noted that reducing the number of beams may be needed in both cases:

- Case 1: having 1 candidate SSB per beam and satisfying the limit for short control signaling

- Case 2: having more than 1 candidate SSB per beam

Considering the high beam directivity for 60 GHz range compared to FR1, LBT failure rates may be low. Hence, case 2 may not add any benefits but may add to the specification and UE complexity, e.g.:

- Signaling the Q factor may be challenging (without changing the PBCH payload and DMRS sequence, per the agreement)

- Enabling/disabling DBTW depending on licensed/unlicensed bands

Hence, given the above argument we propose the following:

Proposal 5: for an unlicensed band that requires LBT, do not support discovery burst transmission window (DBTW) for SSB for all SCSs

#### If DBTW for SSB is Adopted for 120KHz SSB

Although there may not be a strong motivation to support SSB DBTW, in the following we present our views in the case it was concluded that it will be supported.

Based on the agreement above, the changes needed to support the DBTW should not include any additional PBCH payload or any additional DMRS sequences. This means that the bits needed to signal the Q factor and to enable/disable DBTW should come from either the existing fields or by reinterpreting some values in the existing fields. In both cases, some restrictions may be needed to the existing fields. Thus, to reduce this restriction impact, it may be desirable to reduce as much as possible the number of bits needed (e.g., to 1 or 2 bits) for Q and thus the allowed values (e.g., to 2 or 4 values).

In addition, there may not be a need to explicitly reserve bits for DBTW enabling/disabling as it can be implicitly assumed from the Q factor value.

Consider the 4-bits used to signal controlResourceSetZero. We can restrict it to be only 3-bits (8 possible values) to spare one bit for Q. In the current specs, only 3 bits are used for 120 kHz SCS SSB for the valid combinations for this WI. However, this may not become available if more combinations are supported (e.g., 96 RBs).

The second bit (if needed) may come from the 4-bits for searchSpaceZero field. For multiplexing patterns 2 and 3, only one value is used, hence, 1 bit is easily available. For multiplexing pattern 1, restrictions to the available configurations may be needed to get the 1 extra bit.

Another option to get the second bit (if needed) is from ssb-SubcarrierOffset by restricting the available values.

One more option to consider is the subCarrierSpacingCommon in case 120 kHz SSB and 480/960 kHz CORESET0 is not adopted. In this case, it frees up the subCarrierSpacingCommon bit.

As for the number of candidate SSB positions per DBTW, there may not be a strong motivation to increase it from the available 64 for FR2, i.e., there may not be a need to define new candidate SSB positions outside the FR2 Case D pattern. This is mainly to maintain the gaps needed for UL and/or URLCC traffic.

The following however may be considered: The gNB may do an LBT before an SSB burst:

- If the LBT passes, then the gNB sends all SSBs in the burst​

- If the LBT fails, the gNB may send the SSBs by dividing the SSB positions in an SSB burst into the following subsets:​

  - Subset 1: includes SSB beams that are transmitted under the short control signal assumption​

  - Subset 2: includes SSB beams that may be transmitted​

    - Option 1: one SSB candidate position can be assigned per SSB ​

      - If LBT fails on 1 opportunity, no transmission occurs for that beam in this burst​ (best effort)

    - Option 2: more than 1 SSB candidate positions can be assigned per SSB ​

      - If LBT fails on 1 opportunity, other opportunities are available in the burst​ (requires Q)

Proposal 6: for an unlicensed band that requires LBT, if DBTW for SSB is adopted for 120KHz SSB:

- Minimize the number of bits needed to signal Q (1 or 2 bits) and thus the values (2 or 4 values)

- Enabling/disabling DBTW can be implicit in the Q value

- Based on other agreements/designs, consider getting the bits needed from one or more of the following: controlResourceSetZero, searchSpaceZero, ssb-SubcarrierOffset, subCarrierSpacingCommon (in case 120 kHz SSB and 480/960 kHz CORESET0 is not adopted)

- Do not introduce new candidate SSB positions outside the FR2 Case D pattern, and the QCL relationship is introduced among the existing 64 candidate SSB positions

- Consider having a subset of the SSBs (< 64) transmitted under the short control signal assumption while another subset can be best effort or have multiple positions per beam (have a Q factor within the subset)

## SSB/CORESET0 Multiplexing Aspects

### Allowed SCS combinations

If a single numerology deployment is considered, the important part is to have the same numerology for the non-SSB channels/signals. For example, if SSB is 120 kHz while CORESET0 uses 480/960 kHz, then it may qualify as a same numerology deployment if other data/control use 480/960 kHz. Having 120 kHz SSB and 120 kHz CORESET0 with 480/960 kHz data/control may be the case for a different numerology deployment. Therefore, if 120 kHz SCS for SSB is used, it may be desirable to support 480/960 kHz SCS for CORESET0.

Observation 11: to support a single numerology deployment using 120 kHz SCS SSB (and 240 kHz SCS SSB if supported) and 480/960 kHz SCS data/control, supporting 480/960 kHz SCS for CORESET0 may be needed

For the other options:

- If 480/960 kHz is supported for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE, then it may be desirable to support the same SCS (480/960 kHz) for CORESET0.

- If 240 kHz SSB is supported, then support CORESET0 120 kHz (same as FR2) and CORESET0 480/960 kHz (to allow for single numerology deployment)

A proposal for the allowed combinations for the SSB and CORESET0 is presented in Table 1.

Proposal 7: consider the following SSB and CORESET0 SCS combinations:

- SSB SCS = 120 kHz, CORESET0 SCS = 120, 480, 960 kHz

- If SSB SCS = 240 kHz is supported, support CORESET0 SCS = 120, 480, 960 kHz

- If SSB SCS = 480/960 kHz is supported for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE, support CORESET0 SCS = SSB SCS

Table 1: Allowed SSB/CORESET0 SCS Combinations

| SSB SCS (kHz) ↓ | CORESET0 SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 120 | 480 | 960 |
| 120 | Yes | Yes | Yes |
| 240 [Note 1] | Yes | Yes | Yes |
| 480 [Note 2] | No | Yes | No |
| 960 [Note 2] | No | No | Yes |
| Note 1: if 240KHz SSB agreed to be supported Note 2: for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE |  |  |  |

### Indication of subCarrierSpacingCommon in MIB

The subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages is indicated in the MIB using 1-bit which can be scs15or60, scs30or120. In the case of SSB SCS = 120 kHz, and if 3 different values for the CORESET0 SCS are possible, then the 1-bit indication in the MIB is not enough. Several options may be considered, e.g.:

- Reduce the number of allowed combinations for 120 kHz SCS SSB

- Reuse 1-bit from the “pdcch-ConfigSIB1” field if all 8 bits are not needed

- Carry the extra-1 bit needed in PSS/SSS/DMRS, by some implicit MIB/PBCH signaling, or using different SSB design

Proposal 8: consider ways to have 2 bits (1 extra bit compared to FR2) to indicate the common SCS in the SSB structure or contents in case more than 2 values for the common SCS are allowed

### SSB/CORESET0 Multiplexing Patterns

In NR Rel-16, SSB and CORESET0 for FR2 may be multiplexed using one of 3 possible “multiplexing patterns”:

- Multiplexing pattern 1:

  - The SCS of the SSB and CORESET0 may or may not be the same

  - The SSB and CORESET0 symbols are TDM’ed

    - CORESET0 offset = 0, 2.5, 5, or 7.5 ms from the frame start

  - UE monitors PDCCH over 2 consecutive slots starting from slot n0

    - For SSB index i: n0=O∙2μ+i∙Mmod Nslotframe,μ

    - That is in a frame with SFN satisfying

      - SFNCmod 2=0 if O∙2μ+i∙M/Nslotframe,μ mod 2=0

    - Or in a frame with SFN satisfying

      - SFNCmod 2=1 if O∙2μ+i∙M/Nslotframe,μ mod 2=1

    - μ={0,1,2,3} based on the SCS for PDCCH receptions in the CORESET

  - O and M are defined in Table 13-12 of TS 38.213

- Multiplexing pattern 2:

  - The SCS of the SSB and CORESET0 are different

  - The SSB and CORESET0 symbols are FDM’ed/TDM’ed

    - They are TDM’ed to allow for the same numerology across the BWP

    - These are located in symbols between the SSBs

  - UE monitors PDCCH over 1 slot with CORESET0 periodicity equal to the periodicity of SSB

  - Defined in Tables 13-13 and 13-14 of TS 38.213

- Multiplexing pattern 3:

  - The SCS of the SSB and CORESET0 are the same

  - The SSB and CORESET0 symbols are FDM’ed

  - UE monitors PDCCH over 1 slot with CORESET0 periodicity equal to the periodicity of SSB

  - Defined in Table 13-15 of TS 38.213

The configuration is indicated in pdcch-ConfigSIB1 (in the MIB), which consists of:

- controlResourceSetZero (4 bits), determines:

  - Multiplexing pattern

  - CORESET0 frequency offset/#RBs/#symbols

- searchSpaceZero (4 bits), determines:

  - CORESET0 time location

  - For multiplexing patterns 2 and 3, only one value “0” is used

For the NR operation in the frequency between 52.6GHz and 71GHz, the following consideration may be taken for the SSB and CORESET0 multiplexing.

#### Multiplexing pattern 1

The existing NR Rel-16 design may be reused (equations and general table structure: 13-12 of TS 38.213). However, depending on the adopted SSB burst pattern, some numbers in the table may need modification. For example, the current offset of the 1st symbol of the CORESET0 from the frame start can be 0, 2.5, 5, or 7.5 ms. However, for higher SCS values, the SSB burst/span may be less than 2.5 ms. To have the option of back-to-back SSB and CORESET0 multiplexing, a value of < 2.5 ms may be considered.

Proposal 9: NR Rel-16 SSB/CORESET0 multiplexing pattern 1 design may be reused with possibly some changes to the table (e.g., the need for < 2.5 ms options for the start of the CORESET0 wrt frame boundary) which depends on the outcome of the SSB pattern design

#### Multiplexing pattern 2

This can be considered for the SSB + CORESET0 SCS combinations: 120 kHz + 480/960 kHz and 240 kHz + 120 kHz.

For the 240 kHz + 120 kHz combination, the same design for NR Rel-16 may be reused.

For the 120 kHz + 480/960 kHz combination, the CORESET0 symbols may be placed in the gap symbols between the SSBs. It can be a simple extension to the existing NR Rel-16 design where the CORESET0 symbols are placed in the symbol gaps before the corresponding SSBs (Figure 10).

Proposal 10: SSB/CORESET0 multiplexing pattern 2:

- For the 240 kHz + 120 kHz combination (if supported): reuse the same design as in NR Rel-16

- For the 120 kHz + 480/960 kHz combination (if supported): the CORESET0 symbols may be placed in the gap symbols between the SSBs (similar to the existing NR Rel-16 design)

Figure 10: SSB/CORESET0 Multiplexing Pattern 2 Example for 120 + 480/960 kHz combination

#### Multiplexing pattern 3

For this multiplexing pattern, the SCS of the SSB and CORESET0 need to be the same, hence the valid combinations are: 120 + 120 kHz, 480 + 480 kHz, and 960 + 960 kHz.

Generally, the same design concept and table used in NR Rel-16 may be used here.

Proposal 11: NR Rel-16 SSB/CORESET0 multiplexing pattern 3 design may be reused for the valid combinations of 120 + 120 kHz, 480 + 480 kHz, and 960 + 960 kHz

In addition to multiplexing patterns 1, 2, and 3 defined in NR Rel-16, additional designs may be considered.

#### Additional Design Option 1

A time-domain fixed location for CORESET0/SIB1:

- SSBs are located at {4,8,16,20} +28*n (n={0,1,2,3}, {5,6,7,8}, {10,11,12,13}, {15,16,17,18})

- First 16 beams of the CORESET0/SIB1 are placed at slots 8 and 9 of 120 kHz (slots 32 to slot 39 of 480 kHz or slots 64 to slot 79 of 960 kHz) such that

  - In case of 120 + 480: 2 CORESET0/SIB1 per 480 kHz slot (7 symbols for one CORESET0/SIB1)

    - Symbol 0-1 (CORESET0), symbol 2-5 (SIB1), symbol 6 (Gap)

    - Symbol 7-8 (CORESET0), symbol 9-12 (SIB1), symbol 13 (Gap)

  - In case of 120 + 960: 1 CORESET0/SIB1 per 960 kHz slot (14 symbols for one CORESET0/SIB1)

    - Symbol 0-1 (CORESET0), symbol 2-12 (SIB1), symbol 13 (Gap)

  - Similar offset for second/third/forth set of 16 CORESET0/SIB1 corresponding to SSBs

    - I.e., repeat 120 kHz slot {8,9} design for slots {18,19}, {28,29}, {38,39} to cover all 64 beams

- The advantages of this design may include:

  - After acquiring SSB i, if the RSSI of the SSB is good enough and the UE decides not to detect any other SSBs, the UE may sleep until the corresponding CORESET0/SIB1, thus achieve some power saving

  - Smaller delay between SSB and CORESET0/SIB1 (within the same frame)

- A possible disadvantage of this design may occur when the gNB does not transmit all 64 Beams (i.e., some SSB symbols are un-occupied), the gNB may not schedule CORESET0/SIBs in those un-occupied symbols.

Figure 11 and Figure 12 show the design for 120 + 480 kHz and 120 + 960 kHz combinations, respectively.

Proposal 12: consider introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where a time domain fixed location for the CORESET0 and SIB1 is considered

Figure 11: SSB/CORESET0 Multiplexing Pattern Example for 120 + 480 kHz combination (Additional Design 1)

Figure 12: SSB/CORESET0 Multiplexing Pattern Example for 120 + 960 kHz combination (Additional Design 1)

#### Additional Design Option 2

For higher SCS SSB, to minimize the beam switching gap overhead (which may be explicit), consider grouping (TDM’ing) the SSB and CORESET0 symbols. Grouping multiple channels/signals for 1 beam together may be particularly useful for higher SCS cases where symbol-level beam switching gaps may be required and the grouping may help reduce the gap overhead/duplication (e.g., between different beam SSBs and between different beam CORESET0’s/SIB1s). Figure 13 shows some examples for this proposed multiplexing pattern.

- Option A: in section 2.1.2, we proposed the “SSB slot” concept and the possibility that control symbols may be multiplexing with SSB in the same SSB slot. This design may have several advantages: e.g., beam switching gaps may be minimized, self-contained structure, and generic control symbols may be multiplexed with the SSB

- Option B: back-to-back SSB+CORESET0 of a beam followed by SIB1 and a gap (as in example 2 below)

Although the figure shows an SSB slot design, the SSB/COREST0/SIB1 may or may not be confined in an “SSB slot”.

The SCS of the SSB and CORESET0 used in this multiplexing pattern option may be different or the same.

Proposal 13: consider introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where TDM grouping of the SSB and the corresponding CORESET0/SIB1 is considered

Figure 13: SSB/CORESET0 Multiplexing Pattern Example (Additional Design 2)

## RACH Design Aspects

### PRACH Sequence Length

In [2], the performance of the PRACH was presented for different SCSs. The following observation was made:

Observation 12: for the PRACH performance of different numerologies in the high frequency regime,

- No noticeable difference in the misdetection performance is identified among SCSs.

- With the same CINR, the false alarm rate increases as the SCS or sequence length (i.e., bandwidth) increases

For NR operation in the range of 52.6 GHz to 71 GHz, unlicensed band has been identified within the frequency range. Similar to unlicensed band in FR1, there will be a PSD limit and an EIRP constraint in higher bands as well. For example, in use cases other than outdoor, there is PSD limit of 23 dBm/MHz with up to 40 dBm EIRP. While some mobile terminals may have TRP of 23 dBm, some other user terminals (e.g., CPE) may be able to operate at higher EIRP up to 40 dBm. For these high capability UEs, to fully take advantage of the 40 dBm EIRP limit, ~50 MHz transmission bandwidth is needed under the 23 dBm/MHz PSD limit.

To accommodate this, an increase of the BW of the PRACH preamble may be considered, and a longer sequence length (compared to FR2) was considered. In RAN1#104-e, it was agreed that “for initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.”

However, for higher SCS PRACH, the agreement was: “For non-initial access use cases, if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively. FFS: support of sequence length L = 571, 1151.”

Table 2 shows the required PRACH preamble BW for multiple SCS and sequence lengths (LRA) options.

Table 2: PRACH Preamble BW

| SCS (kHz) | LRA | BW (MHz) |
| --- | --- | --- |
| 120 | 139 | 16.7 |
|  | 571 | 68.5 |
|  | 1151 | 138.1 |
| 480 | 139 | 66.7 |
|  | 571 | 274.1 |
|  | 1151 | 552.5 |
| 960 | 139 | 133.4 |
|  | 571 | 548.2 |
|  | 1151 | 1105.0 |

Observation 13: SCS = 480/960 kHz with sequence length = 139 is enough to achieve the desired BW requirement for the maximum EIRP allowed

Proposal 14: consider only using PRACH sequence length = 139 for SCS = 480 kHz and 960 kHz

### RO Considerations

In RAN1#104b-e, the following agreement was made:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

In NR Rel-16, up to 8 RACH occasions (RO) can be FDM’ed. For higher bands, as agreed in RAN1#104-e, a longer preamble sequence length of 571 or 1151 may be used for SCS = 120 kHz and if all 8 FDM’ed ROs are configured, the overall BW would be 548 MHz and 1.105 GHz, respectively, which are higher than the 400 MHz maximum BW for SCS 120 kHz. Hence:

Proposal 15: a maximum of 4 and 2 FD multiplexed ROs for SCS = 120 kHz and sequence length = 571 and 1151, respectively

Due to the possible reduction in the maximum number of FDM’ed ROs, more time domain ROs may be needed to maintain the same RACH capacity​. We can have several options to do this:

- Option A: Re-use the existing design but use larger association period​

  - This may slow down initial access​ and increase UE power consumption

- Option B: Explicitly add more reference slots in a configuration period in Table 6.3.3.2-4 in TS 38.211​

  - Non-trivial spec work/time​

- Option C: Add more reference slots in a configuration period by:

  - Alt 1: adding N additional slots every M reference slot​

    - Reuse existing Table 6.3.3.2-4 in TS 38.211​ (minimal spec impact)

    - N and M can be specified or indicated​

    - Example: PRACH Config. Index 0:​

      - Current table: Slot number = 4,9,14,19,24,29,34,39​

      - New values (N = 1, M = 2): Slot number = 4,5, 9,14,15,19,24,25, 29,34,35,39​

  - Alt 2: adding one or more offseted version(s) (offset = L) of the slot number pattern to the existing one​

    - Reuse existing Table 6.3.3.2-4 in TS 38.211​ (minimal spec impact)

    - L can be specified or indicated and can be either added or subtracted to the existing slot number​

    - Example: PRACH Config. Index 0:​

      - Current table: Slot number = 4,9,14,19,24,29,34,39​

      - New values (L = 2): Slot number = 2,4,7,9,12,14,17,19,22,24,27,29,32,34,37,39​

Proposal 16: for SCS = 120 kHz, if the maximum number of FD ROs are reduced, consider ways to increase the TD ROs (to maintain the same capacity) with minimal specification impact

In the time domain, the same argument presented for the SSB beam switching delays may apply for PRACH. NR Rel-16 can multiplex RO’s back-to-back because the gNB beam switching delay can be absorbed by the CP (i.e., the CP is long enough to absorb the beam switching delays). However, in higher bands and SCS (e.g., SCS = 480 and 960 kHz), the CP length may not be long enough to absorb this beam switching delay requirement.

Observation 14: for higher RACH SCS (480 and 960 kHz), the gap and CP length may not be long enough to absorb the gNB beam switching delay requirement

To mitigate this issue, the following options may be considered:

- Option A: Do not include an explicit gap (reuse NR Rel16 design)

  - Some PRACH symbol repetitions may be lost

  - Leave it to gNB implementation on how to handle any “possible” repetition preamble losses

- Option B: Include a PRACH symbol level gap between ROs

  - This makes sure all repetitions are usable by the gNB and the coverage can be predicted

- Option C: Include a RO-level gap between ROs

  - This has larger gaps between ROs

  - May have an advantage of scheduling other UL traffic (e.g., for other users) in the gap

  - gNB may also chose to interlace the gaps and ROs in adjacent cells to reduce interference

To reduce the initial access delay and possible losses, and to reduce any possible gap overhead, option B is preferred.

Proposal 17: for higher RACH SCS (480 and 960 kHz), consider including a symbol-level gap between ROs to allow for gNB beam switching delay

Regarding the number of 480/960 kHz RACH slots within a RACH reference slot, consider the following situations:

- A longer PRACH format may be needed (e.g., for coverage). In this case, the same number of ROs defined for the 120 kHz SCS cannot be contained in the reference slot. Therefore, having the flexibility of going beyond 2 RACH slots per reference slot can be useful

- If beam switching and/or LBT gaps are introduced, the number of ROs (per RACH slot) that was defined for 120 kHz may not fit within the RACH slot, and the ROs may need to extend to a following slot​. Hence, the currently defined 1 or 2 RACH slots in a RACH reference slot may not be enough for the higher SCSs

The 2 situations above may not necessarily lead to an increase of RACH processing load.

From the argument above, we made the following observation and proposal:

Observation 15: for higher RACH SCS (480 and 960 kHz), due to gaps and/or coverage enhancement needs, more than 2 RACH slots per RACH reference slots may be needed. This may not necessarily lead to an increase of RACH processing load.

Proposal 18: for higher RACH SCS (480 and 960 kHz), consider ways to support more than 2 RACH slots per RACH reference slot

### RA-RNTI Considerations

The RA-RNTI associated with the PRACH occasion in which the preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

The calculation above uses t_id < 80 which assumes a 10 ms RAR window and a maximum SCS of 120 kHz (80 slots in a frame). For higher bands and higher SCSs (480 and 960 kHz), the number of slots in a frame is > 80 (320 slots for SCS = 480 kHz and 640 slots for SCS = 960 kHz). Hence, keeping the equation unchanged (with t_id < 80 slots) mean the maximum RAR window can be 2.5 ms and 1.25 ms for 480 and 960 kHz, respectively. This may be restrictive.

In addition, in NR Rel-16, MSGB RAR window and NR-U RAR window were extended to 40 ms by including the 2-bit LSB of the SFN corresponding to the PRACH occasion used to transmit the preamble of MSGA in RAR PDCCH.

However, the issue existence and the need to resolve it may be dependent on the actual RO design. If there is a need to address this, there may be several solutions to consider. In this paper, we present a 2 options.

Option A:

From the argument above, we may need to extend the range of the t_id to beyond 80 slots. For future compatibility (higher SCSs), we can make the maximum number of slots to be dependent on the maximum SCS supported in that frequency range.

RA-RNTI = (1 + s_id + 14 × t_id + 14 × max80,Nslotframe,μmax  × f_id + 14 × max80,Nslotframe,μmax  × 8 × ul_carrier_id) mod 216

where μmax is the maximum μ for the FR used.

With this proposal however, the RA-RNTI can be more than FFFF and modular operation needs to be applied. Due to the modular operation, some ROs:

- May have the same RA-RNTI

- May collide with FFF0–FFFD (reserved) or P-RNTI (FFFE) or SI-RNTI (FFFF)

In this case, the ROs with RA-RNTI conflicting with the pre-allocated RNTIs should not be used.

When multiple ROs have the same RA-RNTI but not conflicting with the pre-allocated RNTIs, only one of the ROs can be used (e.g., the first RO among those ROs with the same RA-RNTI) or rely on the existing contention resolution mechanisms.

Option B:

Reuse the NR RA-RNTI equation while conveying additional system timing information in the DL DCI that schedules the MSG2/MSGB (similar to the 40 ms RAR window extension concept in NR Rel-16).

As an example (960 kHz PRACH SCS), using the existing RA-RNTI equation and 10 ms RAR window, the maximum RAR window can be 1.25 ms. If we desire a 10 ms RAR window, there will be 8 segments if we use the RA-RNTI equation in the NR Rel-16. Therefore, we can signal a 3-bit indication in the DL DCI for MSG2/B to indicate which segment the PRACH was sent on. Similarly, for SCS 480 kHz, we need 2-bits (corresponding to 4 segments).

With this approach we can keep the same RA-RNTI as in NR Rel-16 while allowing higher SCS PRACH preamble without any RO restriction. In addition, the approach is future compatible when longer RAR window or even larger PRACH SCS is needed (where we can use more bits in the DCI).

Proposal 19: for higher RACH SCS (480 and 960 kHz), consider the following options for the RA-RNTI (if needed depending on the RO design):

- Option A: using the following equation for the RA-RNTI calculations (μmax is the maximum μ for the FR used) and defining rules in case RA-RNTI conflicts with pre-allocated RNTIs or in case multiple ROs have the same RA-RNTI

  - RA-RNTI = (1 + s_id + 14 × t_id + 14 × max80,Nslotframe,μmax  × f_id + 14 × max80,Nslotframe,μmax  × 8 × ul_carrier_id) mod 216

- Option B: reuse the same RA-RNTI equation in NR Rel-16, divide the RAR window into N segments (each segment is 80 slots using the used SCS), and signal the segment index in the DCI that schedules the MSG2/B

# Conclusions

In this contribution, based on the WID, we presented our views on the initial access aspects for NR operation in the frequency regime between 52.6GHz and 71GHz, including design aspects for:

- SSB

- SSB and CORESET0 multiplexing patterns

- RACH

The following observations were made:

Observation 1: increasing the SSB SCS will have an effect on the UE initial search complexity and potentially increase the UE memory requirements depending on multiple factors including the number of frequency bins needed and the number of correlations in time

Observation 2: the timing acquisition accuracy from low SCS SSB (e.g., 120 kHz) may not always be sufficient for high SCS PDSCH​ (e.g., 960 kHz) reception

Observation 3: using a WB DMRS or a cell-specific TRS for timing offset correction may help reduce the performance gap

Observation 4: the FD density for cell-specific TRS may need to be increased to get a good resolution and timing offset correction for high SCS PDSCH​

Observation 5: for the PSS and SSS detection performance of different numerologies in the high frequency regime,

- The performance is degraded as the SCS increases due to the higher frequency selectivity.

  - The impact is more pronounced in NLOS channels (i.e., CDL-B and TDL-A) with larger delay spreads: ~2dB loss for 960kHz SCS compared to 120kHz SCS

Observation 6: for the PBCH performance of different numerologies in the high frequency regime,

- The performance is degraded as the SCS increases due to the higher frequency selectivity.

  - The impact is more pronounced in NLOS channels (i.e., CDL-B and TDL-A) with larger delay spreads: ~1.7dB loss for 960kHz SCS compared to 120kHz SCS

Observation 7: explicit beam switching gap between SSBs with larger SCS (SCS = 480 kHz and 960 kHz) may be required and to accommodate these switching gaps, a new SSB pattern may be required

- A symbol-level (1 symbol) SSB beam switching gap may be required

Observation 8: for larger SSB SCS (480 kHz and 960 kHz), accommodating UL segments within the SSB burst may require accounting for DL/UL switching delays taking considerable number of symbols

Observation 9: with 120 kHz SCS SSB, the number of transmitted SSB beams may need to be less than 64 in order to classify as short control signal transmission

Observation 10: with 120 kHz SCS SSB, to maintain the gaps between the SSBs to facilitate URLLC and UL traffic, additional candidate SSB positions may not be available​ within the 5 ms DBTW, and to support more than 1 candidate SSB per beam, the number of beams need to be less than 64

Observation 11: to support a single numerology deployment using 120 kHz SCS SSB (and 240 kHz SCS SSB if supported) and 480/960 kHz SCS data/control, supporting 480/960 kHz SCS for CORESET0 may be needed

Observation 12: for the PRACH performance of different numerologies in the high frequency regime,

- No noticeable difference in the misdetection performance is identified among SCSs.

- With the same CINR, the false alarm rate increases as the SCS or sequence length (i.e., bandwidth) increases

Observation 13: SCS = 480/960 kHz with sequence length = 139 is enough to achieve the desired BW requirement for the maximum EIRP allowed

Observation 14: for higher RACH SCS (480 and 960 kHz), the gap and CP length may not be long enough to absorb the gNB beam switching delay requirement

Observation 15: for higher RACH SCS (480 and 960 kHz), due to gaps and/or coverage enhancement needs, more than 2 RACH slots per RACH reference slots may be needed. This may not necessarily lead to an increase of RACH processing load.

Based on these observations, we have the following proposals:

Proposal 1: RAN1 can continue to discuss other options for the SSB SCS support, but prioritize design on the already agreed choices (120 kHz SCS for initial access and 480 kHz and 960 kHz for non-initial access case where SSB location and SCS are explicitly provided to the UE and SSB does not configure Type-0 PDCCH)

Proposal 2: for initial access, in cases where the SSB SCS is smaller than other channels’ SCS (e.g., PDCCH/PDSCH), consider WB DMRS or cell-specific TRS for further timing error corrections

- For cell-specific TRS, consider studying the FD density needed

Proposal 3: for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE (within limits defined in Table 7.6.4-2 of TS 38.133): support SCS = 480/960 kHz

Proposal 4: for the SSB for NR operation in the frequency between 52.6GHz and 71GHz and SCS = 480 kHz and 960 kHz, consider defining an SSB pattern consisting of multiple “SSB slots” where SSB symbols for one or more beams are contained in the “SSB slot”

- A beam switching gap of 1 symbol is inserted between SSBs within the “SSB slot”

- Additional control symbols may be defined in the SSB slots with beam switching gaps between control and SSB symbols of different beams

- Additional “gap slots” may be inserted between “SSB slots” to account for URLLC and UL traffic

- Consider the option of aligning the higher SCS SSBs with the corresponding beams for the lower SCS SSB

Proposal 5: for an unlicensed band that requires LBT, do not support discovery burst transmission window (DBTW) for SSB for all SCSs

Proposal 6: for an unlicensed band that requires LBT, if DBTW for SSB is adopted for 120KHz SSB:

- Minimize the number of bits needed to signal Q (1 or 2 bits) and thus the values (2 or 4 values)

- Enabling/disabling DBTW can be implicit in the Q value

- Based on other agreements/designs, consider getting the bits needed from one or more of the following: controlResourceSetZero, searchSpaceZero, ssb-SubcarrierOffset, subCarrierSpacingCommon (in case 120 kHz SSB and 480/960 kHz CORESET0 is not adopted)

- Do not introduce new candidate SSB positions outside the FR2 Case D pattern, and the QCL relationship is introduced among the existing 64 candidate SSB positions

- Consider having a subset of the SSBs (< 64) transmitted under the short control signal assumption while another subset can be best effort or have multiple positions per beam (have a Q factor within the subset)

Proposal 7: consider the following SSB and CORESET0 SCS combinations:

- SSB SCS = 120 kHz, CORESET0 SCS = 120, 480, 960 kHz

- If SSB SCS = 240 kHz is supported, support CORESET0 SCS = 120, 480, 960 kHz

- If SSB SCS = 480/960 kHz is supported for non-initial access where SSB does configure Type-0 PDCCH and timing of the SSB is known to the UE, support CORESET0 SCS = SSB SCS

Proposal 8: consider ways to have 2 bits (1 extra bit compared to FR2) to indicate the common SCS in the SSB structure or contents in case more than 2 values for the common SCS are allowed

Proposal 9: NR Rel-16 SSB/CORESET0 multiplexing pattern 1 design may be reused with possibly some changes to the table (e.g., the need for < 2.5 ms options for the start of the CORESET0 wrt frame boundary) which depends on the outcome of the SSB pattern design

Proposal 10: SSB/CORESET0 multiplexing pattern 2:

- For the 240 kHz + 120 kHz combination (if supported): reuse the same design as in NR Rel-16

- For the 120 kHz + 480/960 kHz combination (if supported): the CORESET0 symbols may be placed in the gap symbols between the SSBs (similar to the existing NR Rel-16 design)

Proposal 11: NR Rel-16 SSB/CORESET0 multiplexing pattern 3 design may be reused for the valid combinations of 120 + 120 kHz, 480 + 480 kHz, and 960 + 960 kHz

Proposal 12: consider introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where a time domain fixed location for the CORESET0 and SIB1 is considered

Proposal 13: consider introducing an SSB/CORESET0 multiplexing pattern for higher SCS SSB (480 and 960 kHz), where TDM grouping of the SSB and the corresponding CORESET0/SIB1 is considered

Proposal 14: consider only using PRACH sequence length = 139 for SCS = 480 kHz and 960 kHz

Proposal 15: a maximum of 4 and 2 FD multiplexed ROs for SCS = 120 kHz and sequence length = 571 and 1151, respectively

Proposal 16: for SCS = 120 kHz, if the maximum number of FD ROs are reduced, consider ways to increase the TD ROs (to maintain the same capacity) with minimal specification impact

Proposal 17: for higher RACH SCS (480 and 960 kHz), consider including a symbol-level gap between ROs to allow for gNB beam switching delay

Proposal 18: for higher RACH SCS (480 and 960 kHz), consider ways to support more than 2 RACH slots per RACH reference slot

# References

[1] RP-210862, “Revised WID: Extending current NR operation to 71GHz”, RAN #91e, CMCC

[2] 	R1-2008615, “Required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz”, RAN1 #103-e, Qualcomm Incorporated

[3] R1-2102202, “LS on beam switching gap for 60 GHz band”, RAN1 #104-e

# Appendix

## RAN1#104-e Agreements

Agreement:

Send an LS to RAN4 to get input on gap required for gNBs and UEs for beam switching and for UL/DL and DL/UL switching.

Agreement:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Agreement:

For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS

- If DB supported

  - FFS: What signals/channels are included in DB other than SS/PBCH block

- If DBTW is supported

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

- The following points are additionally FFS:

  - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size

  - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

  - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

Agreement:

For CORESET#0 and Type0-PDCCH search space configured in MIB:

- Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz

  - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz.

    - FFS: Supporting additional values

  - FFS: Supported values for SSB to CORESET#0 offset RBs

- If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz

- If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz

- If 240 kHz SSB SCS is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz

- FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960)

  - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz)

Agreement:

For 480 kHz and 960 kHz SSB SCS (if agreed)

- Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels)

  - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz.

- Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern

- Study should account for inputs from RAN4

Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

- For non-initial access use cases,

  - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively.

    - FFS: support of sequence length L = 571, 1151

- FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access

Agreement:

If 480 and/or 960 kHz PRACH SCS is supported, RAN1 should study whether or not the current RA-RNTI calculation and PRACH identification in RAR correctly provides unique identification of PRACH.

## RAN1#104b-e Agreements

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation