3GPP TSG RAN WG1 Meeting #105e				R1-2104722

e-Meeting, May 10th – 27th, 2021

Source 	: CAICT

Title 	: Considerations on Enhancements on UL Time Synchronization in NTN

Agenda Item	: 8.4.2

Document for	: Discussion and decision

Introduction

In last meeting of RAN1#104-be, the following agreements have been reached for UL time synchronization in NTN.

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e. Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported

In this contribution, we will give our considerations on the FFS issues about UL time synchronization and put forward the observations and proposals accordingly.

Discussions

Full TA Pre-compensation

In last meeting, a new version of TA calculation formula for full TA pre-compensation has been agreed. In our observation, the cell-specific NTA,common can have multiple definitions and which definition to use is up to network implementation. As for the definition that NTA,common is corresponding to the TA needed to be pre-compensated at the reference point as shown in Fig.1., we think that a UE without capability of location acquisition can also make use of the NTA,common for RACH attempt.

FIG. 1 Illustration of the TA components in NTN (For simplicity, TA offset is not plotted)[1]

For instance, the role of the parameter of UE specific TA NTA,UE-specific and common offset NTA,common in the proposed TA calculation formula for UE with capability of location acquisition can be replaced by the common offset NTA,common for UE without capability of location acquisition. Then we can have the following TA formula:

TA =NTA+NTA,UE-specific+NTA,offset++NTA,common×Tc for UE with capability of location acquisition

=NTA+NTA,common+NTA,offset×Tc for UE without capability of location acquisition

Moreover, if the configured NTA,common is beam specific, the NTA,common associated to the selected SSB for RACH attempt shall be used in the TA calculation. When a gNB detects a RACH signal associated to one SSB, gNB can use the NTA,common associated to the SSB for future UL transmission scheduling, e.g. to determine the cell-specific or UE-specific K_offset.

Observation1: The configured cell-specific or beam-specific NTA,common can be used for TA pre-compensation by UEs without capability of location acquisition.

Proposal1: Support to adopt cell-specific or beam-specific NTA,common for TA pre-compensation by UEs without capability of location acquisition.

TA update

According to the discussion, how to update the TA of the following formula through combined open and closed loop control needs further discussions.

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Generally, the network controlled parameter NTA and the parameter NTA,UE-specific autonomously calculated by the UE based on UE-satellite distance are controlled in closed loop manner and open loop manner, respectively.

From our observation, in the scenario of fast change like LEO, the network controlled parameter NTA might be outdated if the duration from the time that UE receives the TAC to the time that UE applies the TA is quite long. In such case, it might be better to apply the TA for UE without considering the network controlled TA information from initial RACH attempt. Therefore, the condition when to apply the TA through combined closed loop and open loop needs FFS. It might need to introduce a timer here to deactivate the network controlled TA parameter.

Proposal2: FFS the condition when to apply the TA through combined closed loop and open loop.

Conclusions

In this contribution, we have present our considerations on the FFS issues about UL time synchronization and put forward the following observations and proposals:

Observation1: The configured cell-specific or beam-specific NTA,common can be used for TA pre-compensation by the UEs without capability of location acquisition.

Proposal1: Support to adopt cell-specific or beam-specific NTA,common for TA pre-compensation by the UEs without capability of location acquisition.

Proposal2: FFS the condition when to apply the TA through combined closed loop and open loop.

References

- 3GPP TR 38.821 v1.0.0, Solutions for NR to support non-terrestrial networks (NTN), Dec.2019

- 3GPP TSG RAN WG1 Meeting #103-e, RAN1 Chairman’s Notes, Nov. 2020.

- 3GPP TSG RAN WG1 Meeting #104-e, RAN1 Chairman’s Notes, Feb. 2021.

- 3GPP TSG RAN WG1 Meeting #104-e, FL Summary on enhancements on UL time and frequency synchronization for NR NTN (R1-2102215) , Feb. 2021.