3GPP TSG RAN WG1 #105-e	R1-2104799

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Maintenance on PDCCH for UE PDSCH processing time for DCI format 1_2

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In last RAN1 meeting, the issue of UE PDSCH processing time for DCI format 1_2 was raised but unfortunately, companies did not reach consensus. In this contribution, we continue to discuss how to include the new defined RRC parameters for DCI format 1_2 in section of UE PDSCH processing time.

Discussion

In Rel-15, UE PDSCH processing time depends on the UE reported capability and the configuration of additional DMRS and is defined as table 5.3-1 and table 5.3-2 in TS 38.214. To our understanding, the table 5.3-1 and table 5.3-2 are mainly used for guidance/restriction of gNB scheduling instead of UE implementation. In other words, UE will always maintain one processing chain in Rel-15 no matter the reported processing capability. For example, if UE reports processing capability 2 and UE is configured with dmrs-AdditionalPosition = 'pos0' for DCI format 1_1, UE will use the processing chain with PDSCH processing capability 2 for PDSCH processing. In such a case, if UE is scheduled by DCI format 1_0 with long PDSCH, UE would use longer processing time compared to processing time for PDSCH scheduled by DCI format 1_1 since there appears additional DMRS for the scheduled PDSCH. However, the processing chain for processing PDSCH(s) scheduled by DCI format 1_1 and DCI format 1_0 remains the same, that is, only the PDSCH processing time is different depending on if there is additional DMRS. In such a case, gNB is responsible for addressing the time gap between the right column of table 5.3-1 and table 5.3-2 to avoid out-of-order scheduling.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

In RAN1 #104b-e, the issue of absent of new DMRS parameters for DCI format 1_2， i.e. dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 in Rel-16 specification was raised and the most controversial point is whether the PDSCH processing time is dependent on DCI formats. With the understanding above, UE would maintain one processing chain for PDSCH processing scheduled by both DCI format 1_1 and 1_2, while the processing time for PDSCH scheduled by DCI format 1_1 and DCI format 1_2 can be different depending on the dmrs-AdditionalPosition configuration for DCI format 1_1 and DCI format 1_2. This would be beneficial for reducing the scheduling latency for DCI format 1_2. For example, UE reports processing capability 2 and UE is configured with dmrs-AdditionalPosition = 'pos0' for DCI format 1_2 but dmrs-AdditionalPosition = 'pos1' for DCI format 1_1. In such a case, gNB can schedule HARQ-ACK for PDSCH scheduled by DCI format 1_2 under the guidance of table 5.3-2 and no need to restrict the scheduling of HARQ-ACK to follow the right column of table. On the contrary, if the processing time is independent of DCI format, then UE uses shorter PDSCH processing time only if dmrs-AdditionalPosition = 'pos0' is configured for both DCI format 1_1 and DCI format 1_2. This would enlarge PDSCH processing time and further scheduling latency for DCI format 1_2. Based on the above discussion, it is preferred to define PDSCH processing time per DCI format on the premise of only one processing chain is maintained by UE.

Proposal 1: Define PDSCH processing time per DCI format on the premise of only one processing chain is maintained by UE. Adopt the following Text Proposal:

5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

Conclusion

In this contribution, we further discuss how to include the new defined RRC parameters for DCI format 1_2 in section of UE PDSCH processing time and the following proposal is made:

Proposal 1: Define PDSCH processing time per DCI format on the premise of only one processing chain is maintained by UE. Adopt the following Text Proposal:

5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

Reference

- R1-2104132	 Summary of email discussion [104b-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements	Moderator (Huawei)