3GPP TSG-RAN WG1 Meeting #105-e	R1-2104813

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.4

Source:	Ericsson

Title:	On other enhancements for NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description is updated in [1]. One objective is to specify the following topics if beneficial and needed:

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

In this contribution, we discuss these topics for NTN.

# 2	Beam management and BWP operation

In this section, we discuss the necessity of potential enhancement for beam management and BWP operation.

To this end, it is necessary to first identify and clarify the underlying problem if any for this discussion. An overview of NR BWP can be found in [2].

## 2.1	Background for BWP and BM discussion in NTN

During Rel-16 SI as well as Rel-17 WI for NR NTN, some enhancements for BWP and BM operation have been proposed. These enhancements are mainly motivated by the following considerations:

- Using L1 beam switching procedures for satellite service link switching, mainly for LEO satellite operation. The alternative here is to use handover procedures when UEs need to be switched to a new satellite.

- Providing frequency reuse scheme for NTN deployment using NR BWP concept for multiple beams per cell layout and using BM for handling UE mobility within the same cell. The alternative solution is to use one beam per cell or handover procedures.

The main motivation and foreseen benefit, in both cases, is to reduce signalling overhead by using L1/L2 BM instead of L3 handover signalling and procedures.

There are two options for mapping the satellite beams and PCI for NTN:

- Option a: Multiple beams per PCI

- Option b: One beam per PCI

Furthermore, according to the current specifications, either of these options can be realized by implementation choice and no further specification effort is required.

Both multiple-beam and one-beam per PCI mapping schemes can be realized for NTN with current NR specifications without any enhancement.

## 2.2	Service link switching using NR beam management

One of the two motivated use cases for BM/BWP related enhancements is service link switching. We discuss the necessity of utilizing NR BM framework for this issue in this subsection.

For LEO earth-fixed beams, NR needs to manage a frequent switching of the service link, from a first serving satellite to a second serving satellite. Since this switching of the service links associated with different satellites can occur as frequently as about once every 1-10 minutes depending on e.g. target operational elevation angle, a solution for managing this scenario for UEs in RRC connected is required. To support this scenario, handover-based mobility management can be used. There are also proposals that NR beam management framework can be used to reduce the overhead associated with handover.

RAN1 to discuss the scope of beam management, i.e., whether NR beam management framework (TCI state and spatial relations) should be restricted within the same satellite or support the switching of the service links associated with different satellites.

Satellite devices are in many cases equipped with highly directive beams with gains exceeding 40 dBi with 3-dB beam widths of around 1º. Due to this, it will be challenging for a satellite device to autonomously and rapidly detect reference signals, e.g. SSBs, from target satellite, point its receive and transmit beam towards the new satellite. To facilitate the switch and find the new satellite, we believe that the network, i.e. source satellite, should indicate the ephemeris of the target satellite. If NR beam management framework can be used to support the switching of the service links associated with different satellites, the TCI state and spatial relations can then be associated with the satellite ephemeris.

A first satellite providing coverage before a service link switch should assist UEs in RRC connected with signaling of the ephemeris of the second satellite providing coverage after the switch.

Finally, we believe that a UE should refrain from initiating an RRC connection towards a satellite that is just about to hand over its service link to a next satellite. Doing so may reduce unnecessary network signaling, reduce PRACH transmissions from UEs, and obviously eliminate the service link switch which may pose as a weak link in an RRC connection. To support this functionality, the network may indicate to the UEs in RRC idle that it will soon initiate a service link switch.

The NR network should be able to indicate the timing of the service link switch to UEs in RRC idle and RRC inactive modes.

## 2.3	Beam and BWP association for frequency reuse scenario

The other main BM/BWP related enhancement is providing Frequency Reuse Factor (FRF) larger than 1 within the same cell footprint for interference mitigation for NTN. In this case, each cell is divided into and covered with multiple beams and each beam is associated with a BWP. To realize the frequency reuse, neighbor beams are assigned with different BWP indices. Figure 1 illustrates an example deployment of this scenario where each cell is covered with 3 different beams.

Figure 1: Example of NTN frequency reuse deployment with multiple beams per PCI.

The detailed design of the BWP to beam association is being discussed. According to the RAN1#103-e FL summary [3], the proposals can be divided into four different options. In the following we discuss and provide our view on each option.

Option-1: Same beam layout for BWP#0 and BWP#x

In this case, gNB configures BWP#0 and one BWP#x on each beam. Simultaneous transmission and reception can be expected from gNB perspective on both BWPs for different UEs. Transmissions in BWP#0 across different beams of the cell can be time multiplexed, as is built in the fundamental design of SSBs that are time multiplexed. In other words, the beam for transmitting BWP#0 can sweep over the area defining the cell comprising multiple beams.

Option-2: Hierarchical beam layout for BWP#0 and BWP#x

In this case, BWP#0 acts as a cell-specific anchor beam covering the whole cell footprint, enabling initial access and broadcast transmission including SSB, SIB1, etc. However, for UEs in connected mode, the cell comprises multiple smaller spotbeams, each associated with a non-zero BWP#x for the transmission of dedicated signals and channels. This option is depicted in Figure 2.

Figure 2: An illustration of using BWP for frequency reuse with BWP#0 as anchor beam.

If the anchor beam covers the whole cell, this might limit the supported system capacity. Release 15 supports mobile terminated (MT) and mobile originated (MO) system access using the initial BWP#0. System access is already expected to pose a capacity bottleneck. Increasing the cell size will not improve the situation. It is of course possible to discuss an enabling of MT/MO access also for BWP#1-#3, but this will then raise new questions, e.g. on how idle mode mobility between these BWPs are to be supported.

It was mentioned that this scenario is one of the main deployment choices of existing satellite networks [3]. Similar to Option 1, this scenario can be realized using the current NR specification. It is up to network choice and implementation.

Using BWP-BM association to enable a frequency reuse can already be supported by existing NR specification. It is a choice of network configuration and implementation.

Option-3: Mapping between SSB index and BWP index

According to [3], the intention with this option is to have a predefined or configurable linking between SSB indices and BWP indices in the specification. Other than that, the operation appears not to be different from Options 1 and 2.

Based on the description, the main point here is to define a linking between BWP index and beam index. However, this is already supported in existing specification.

Mapping between BWP index and beam (SSB) index is already supported by current specification.

Option-4: SSB transmission in beam-specific initial BWP

In this case, the transmissions of CORESET#0, Type-0 CSS and SIB1 are carried out in each different BWP, potentially with or without frequency division multiplexed transmission of SSBs. This contrasts with current NR specification where they are transmitted within BWP#0.

This option is a significant deviation from the fundamental access design in Rel-15 NR. It requires considerable amount of specification effort and the actual effect of this option is almost equivalent to 1-beam per cell setup, that is, each beam become almost like a cell.

BWP specific transmission of CORESET#0, SIB1 and SSBs requires significant specification effort. The actual effect is equivalent to 1-beam per cell scenario.

## 2.4	On the issues identified in RAN1#104-e

In RAN1#104-e, the following conclusion was made:

RAN1#104-e:

Conclusion:

Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management:

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

In this section, we address each of the issues raised.

Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

When UE moves around and/or serving satellite/beam moves, it is not always necessary to switch the BWP upon beam switching or vice versa for mitigating interference and managing intra-cell UE mobility. First of all, UE can still communicate via the same BWP even when switching to new beam. One example of this scenario is that it is not necessary to switch the UE beam indicated by QCL type-D. This is especially relevant for satellite communication as the transmission point has a large distance from the UE and the direction of the UE beam can be the same for satellite neighbor beams. Similarly, even if the UE switches the beam, the BWP is not necessarily required to be changed. Therefore, the BWP/BM procedures including the BWP/beam switching should be performed dynamically with respect to UE position, interference level, channel condition, etc. And it is not desirable to associate the BWP switching with beam switching. The current design of NR BM/BWP procedures can fully support dynamic interference management and BWP/beam switching at the same time if necessary.

Issue #1 is not valid, because it is not necessary to directly associate a NR BWP with a beam and there is no need to necessarily trigger BWP switching upon beam switching.

Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

As pointed out in our response to Issue 1, the main motivation of having FRF>1 is to mitigate the interference between different beams, and the interference patterns are usually different for UL and DL. It would be more flexible that DL and UL are addressed separately for the UE. Therefore, the simultaneous DL and UL BWP switching is not desirable.

Issue #2 is not valid, because it is not necessary to trigger BWP switching upon beam switching, let alone simultaneous BWP switching in both DL and UL.

Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

This issue aims to motivate DCI based BWP switching without data scheduling. Such design was discussed in Rel-15 already, but it was decided that there was no need and thus it was not included. The proponent should justify why it is necessary in NTN. In our view, it is not justified to introduce DCI based BWP switching without data scheduling.

Issue #3 is not valid, because it is not justified to introduce DCI based BWP switching without data scheduling.

Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

It is not clear why resynchronization is required when BWP switching occurs. UE operates on the same carrier before and after the BWP switching. According to current specification, the UE can utilize the same set of SSBs for synchronization.

Issue #4 is not valid, because it is not necessary for the UE to perform re-synchronization upon BWP switching.

Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

The beam switching also depends on the UE mobility as well as interference level. Therefore, in general, it is not a predicable event, and depends on the measurements performed by the UE. So, even if BWP switching and beam switching were coupled, BWP switching would not be a predictable event. Further, as pointed out above, it is not necessary to couple BWP switching and beam switching, casting further doubt on the claim that BWP switching is predictable. In short, the configured BWP switching is against the dynamic and flexible scheduling principles.

Issue #5 is not valid, because BWP switching is not predictable and there is no need for configured BWP switching.

Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

How to deal with BWP switching triggered by bwpInactivityTimer and RA procedure are well defined in the specification. It is unclear what would need to be discussed here. The part on increasing throughput lacks motivation as well.

Issue #6 is not valid, because it is unclear what this issue is in the first place.

Issue 7: NR BWP switching/beam switching is done with UE specific signaling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signaling overhead.

The scenario mentioned in Issue 7 is not representative for satellite communication. For earth fixed beam scenario, the beam/BWP switching also depends on the UE mobility and level of interference and needs to be addressed for each UE separately and dynamically, based on the UE measurements and condition. For earth moving beam scenario, the satellite beam moves gradually on the surface of earth and the switching point in time is different for different UEs, unless the UEs are in a close distance in relation with the beam edge, which is not predictable and not valid all the time. Therefore, the need of group switching of UEs is not motivated.

Issue #7 is not valid, because the need of group switching is not motivated.

RAN1 to conclude that there is no need for additional enhancements for using BWPs to enable a frequency reuse.

# 3	Signaling of polarization

At the last RAN1 meetings, the following agreements were made to support polarization signaling in NR NTN.

RAN1#102-e:

Agreement:

Potential enhancements for support of polarisation signalling in NR NTN can consider at least the following:

- Configuration of DL and UL transmit polarization including Right hand and Left hand circular polarizations (RHCP, LHCP)

- Network broadcast DL and UL transmit polarization configuration

- UE polarization capability (RHCP, LHCP, Linear)

- Dependence of polarisation signaling on deployment scenarios. For example,

  - Resource reuse mode with/without polarization for the beam management enhancement

  - Fixed polarization per cell/beam for polarization reuse and circular polarisation with intra-UE and inter-UE multiplexing (intra-UE and inter-UE) signalling

RAN1#103-e:

Agreement:

Indication of polarization information for DL and UL by the network is supported.

- FFS: Signaling details

RAN1#104-e:

Agreement:

Support at least explicit indication of polarization information for DL by the network

- FFS: whether the indication is done by SIB, other RRC signaling, DCI.

- FFS: Whether separate signaling is needed for the UL and if so, whether or not a same polarization is indicated for DL and UL

Conclusion:

Discuss the necessity of reporting UE polarization capability considering at least following aspects,

- Deployment scenarios.

- UE implementation aspects with respect to polarization.

- Satellite implementation aspects for switching between polarization states.

- Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

Most satellite networks make use of polarization. Right hand and left-hand circular polarizations (RHCP, LHCP) are popular choices as they travel unaffected through the atmosphere. It was observed in TR 38.821 that polarization can be used in cell planning as a tool to reduce inter-cell interference. It is also useful for supporting rank-2 MIMO, and for supporting co-existence in shared frequency bands. The last item is a highly relevant aspect due to the many plans to deploy mega NGSO constellations in shared Ka-, Ku- and V-bands. High profile constellations such as the one provided by Starlink will e.g. make use of the same spectrum. Not only is the spectrum sharing important between these NGSO constellations, but it is also required between them and existing GEO deployments. ITU Recommendation ITU-R S.1431 provides methods to enhance sharing between NGSO FSS systems and e.g. points out that two systems can share the same spectrum over the same geographical region by means of using different polarizations. Supporting such cases is important for NR NTN to be an attractive alternative for NGSO operators.

In pure LOS conditions, the satellite transmit polarization should be detectable by a UE with high precision. In theory it could be required that a UE detects the DL polarization and makes use of the same polarization in the uplink. In practice, however, there are cases where reflections in buildings, landscape and water will impair the polarization before it reaches the UE. Assume e.g. that a satellite device receives a radio wave that was reflected in a window at the Brewster angle, at which the polarization component aligned with the direction of the wave cancels out. Even though the signal was circularly polarized at the transmitter, the receiving UE will only observe a linearly polarized wave, and it will be impossible for the UE to determine the polarization used by the satellite. To support a well-defined polarization both in the DL and the UL, we believe that the network should at least broadcast to the UEs the UL transmit polarization. With this signaling in place, we believe that it is reasonable to also support signaling of the transmit polarization used for the DL.

In some cases, a UE cannot be expected to reliably detect the used DL polarization.

Support broadcast signaling in SIB that allows a gNB to indicate the gNB’s DL transmit polarization mode and UL receive polarizations mode to UE.

Support signaling that allows the gNB to configure a UE’s polarization modes including the UE’s receive polarization mode in the DL and the UE’s transmit polarization mode in the UL.

It is further expected that a mega NGSO constellation will be utilized to serve multiple types of devices such as reduced capability devices, normal handhelds, vehicular devices, fixed devices, among others. Some of these devices will use linear polarization, some will use circular polarization, and others may support both. For optimal performance, it will be necessary for network to know the polarization capability of these devices.

NTN UE should report its polarization capability (RHCP, LHCP, Linear) to the network.

# Conclusion

In the previous sections, we discuss enhancements for NTN that fall in the category of “specify if beneficial and needed”. We made the following observations:

Observation 1	Both multiple-beam and one-beam per PCI mapping schemes can be realized for NTN with current NR specifications without any enhancement.

Observation 2	Using BWP-BM association to enable a frequency reuse can already be supported by existing NR specification. It is a choice of network configuration and implementation.

Observation 3	Mapping between BWP index and beam (SSB) index is already supported by current specification.

Observation 4	BWP specific transmission of CORESET#0, SIB1 and SSBs requires significant specification effort. The actual effect is equivalent to 1-beam per cell scenario.

Observation 5	Issue #1 is not valid, because it is not necessary to directly associate a NR BWP with a beam and there is no need to necessarily trigger BWP switching upon beam switching.

Observation 6	Issue #2 is not valid, because it is not necessary to trigger BWP switching upon beam switching, let alone simultaneous BWP switching in both DL and UL.

Observation 7	Issue #3 is not valid, because it is not justified to introduce DCI based BWP switching without data scheduling.

Observation 8	Issue #4 is not valid, because it is not necessary for the UE to perform re-synchronization upon BWP switching.

Observation 9	Issue #5 is not valid, because BWP switching is not predictable and there is no need for configured BWP switching.

Observation 10	Issue #6 is not valid, because it is unclear what this issue is in the first place.

Observation 11	Issue #7 is not valid, because the need of group switching is not motivated.

Observation 12	In some cases, a UE cannot be expected to reliably detect the used DL polarization.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to discuss the scope of beam management, i.e., whether NR beam management framework (TCI state and spatial relations) should be restricted within the same satellite or support the switching of the service links associated with different satellites.

Proposal 2	A first satellite providing coverage before a service link switch should assist UEs in RRC connected with signaling of the ephemeris of the second satellite providing coverage after the switch.

Proposal 3	The NR network should be able to indicate the timing of the service link switch to UEs in RRC idle and RRC inactive modes.

Proposal 4	RAN1 to conclude that there is no need for additional enhancements for using BWPs to enable a frequency reuse.

Proposal 5	Support broadcast signaling in SIB that allows a gNB to indicate the gNB’s DL transmit polarization mode and UL receive polarizations mode to UE.

Proposal 6	Support signaling that allows the gNB to configure a UE’s polarization modes including the UE’s receive polarization mode in the DL and the UE’s transmit polarization mode in the UL.

Proposal 7	NTN UE should report its polarization capability (RHCP, LHCP, Linear) to the network.

# References

RP-210908, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #91e, March 2021.

X. Lin, D. Yu, and H. Wiemann, “A primer on bandwidth parts in 5G new radio,” in 5G and Beyond: Fundamentals and Standards, X. Lin and N. Lee, Eds. Springer, 2021. Available at https://link.springer.com/chapter/10.1007/978-3-030-58197-8_12

R1-2009736, “Summary #4 of 8.4.4 Other Aspects of NR-NTN,”	MediaTek, November 2020.