3GPP TSG-RAN WG1 #105-e		R1-2105080

e-Meeting, May 10th – 27th, 2021

Source: 	Apple Inc.

Title:                     Clarification on NR-U RRM measurement operation

Agenda item:	7.2.2

Document for:	Discussion and Decision

# 1 Introduction

One LS [1] was received from RAN2 about the random subframe offset value generation for NR-U RRM measurement, if rmtc-SubframeOffset is not configured.

In this document, we discuss these two questions, i.e., how to generate the random value for subframe offset and whether the same approach can be extended to LAA case.

# 2. Discussions

The RSSI measurement defined for LAA was reused for NR-U operation. Referring to [2], a list of RRC parameters for RSSI measurement are given below:

RMTC-Config-r16 ::= SEQUENCE { rmtc-Periodicity-r16 ENUMERATED {ms40, ms80, ms160, ms320, ms640}, rmtc-SubframeOffset-r16 INTEGER(0..639) OPTIONAL, -- Need M measDurationSymbols-r16 ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60}, rmtc-Frequency-r16 ARFCN-ValueNR, ref-SCS-CP-r16. ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP}, ... }

Figure 1 provides one example of RSSI measurement based on the agreed RRC parameters.

Figure 1:  RSSI Measurement Configuration

According to [2], the UE shall setup the setup the RSSI measurement timing configuration (RMTC) in accordance with the received rmtc-Periodicity and, if configured, with rmtc-SubframeOffset i.e., the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10); subframe = rmtc-SubframeOffset mod 10; with T = rmtc-Periodicity/10;

The RSSI measurement configuration for LAA was extensively discussed in RAN1 83 meeting. There have been some discussions on the usefulness to introduce an OFDM symbol offset parameter and the necessity to make it configurable and optional:

- Compared to always performing RSSI measurement at the start of periodicity, the advantage of introducing this parameter and make it configurable is to provide desirable flexibility for gNB to control the timing where the UE performs RSSI measurement. It was considered to be useful and can be leveraged by network to detect the hidden nodes at the end of subframes where the eNB may also be performing measurements for LBT before transmission on the next subframe.

- During the email discussions, it was pointed out that RSSI measurement, if always periodic, could potentially overlap with DRS transmissions from other LAA cells or other periodic transmission, thus the measured RSSI or channel occupancy results can be biased. This is particularly concerned and problem for RSSI measurement, which is intendedly configured by eNB to assist it choosing other channels. Hence, it is beneficial to provide some flexibility in randomly moving the subframe offset in every period to help mitigating the problem of periodic pattern for this use case, as illustrated in FIG.1 above. This justifies the need to make this parameter optional. When this parameter is not configured, UE can randomly generate a subframe offset value in a period and therefore avoids that the RSSI measurement location is completely periodic.

Therefore, the following was agreed as the outcome of discussions in RAN1 83 meeting:

Agreements: - Parameter “Subframe offset for UE reported RSSI measurement duration” is made optionally configurable for inter-frequency measurement - When the parameter is configured - UE measures according to the configured offset - When the parameter is not configured - For inter-frequency measurements, UE [should be allowed to flexibly] chooses a random starting offset for “RSSI measurement duration”

Keeping in mind of the design spirit above, it is nature that UE generate the random value per rmtc-Periodicity if the rmtc-SubframeOffset is not configured. On the other hand, we are also flexible to leave how to generate random value to UE implementation to avoid specification changes for this case.

Proposal 1:

- If the rmtc-SubframeOffset is not configured, UE either generates a random value for subframe offset per rmtc-Periodicity to perform RSSI measurement (Alt.1, preferred) or is up to UE implementation (Alt.2).

- Use the same handling for LAA.

# 3. Conclusion

In this contribution, we have presented our views on relaxed maximum modulation order for Redcap. Based on the discussions, we proposed the following for RAN2 LS reply

Proposal 1:

- If the rmtc-SubframeOffset is not configured, UE either generates a random value for subframe offset per rmtc-Periodicity to perform RSSI measurement (Alt.1, preferred) or leave it for UE implementation (Alt.2).

- Use the same handling for LAA.

# References

- R1-2104163, LS to RAN1 on random value generation for RMTC-SubframeOffset, RAN2, Apple

- 3GPP TS 38.331, 5G NR RRC specification