3GPP TSG RAN WG1 #105-e			R1-2105103

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.3

Source:	Apple

Title:	On HARQ Enhancements for NR NTN

Document for:	Discussion/Decision

# Introduction

One of the RAN1-lead objectives in the NTN WID [1] is HARQ enhancements for NTN. A list of topics related to HARQ enhancements was provided [2]. The following agreements have been made in RAN1 meeting #104-e [3].

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

In this contribution, we provide our views on HARQ process number indication, mechanism for disabling HARQ feedback, HARQ-ACK codebook constructions and PDSCH scheduling restriction.

# Discussion

## HARQ process number indication

It was agreed [4] that 32 is taken as the maximal supported HARQ process number for both uplink and downlink. Since the maximum bit field size for HARQ process number in existing DCI formats is 4 bits, the HARQ process number indication needs to be enhanced.

In RAN1 #103-e meeting, four options to indicate up to 32 HARQ process numbers were identified for non-fallback DCI formats, i.e., 0_2/1_2 and 0_1/1_1. The first two options make use of the slot index of the DCI to indicate either MSB or LSB of HARQ process number. Although this approach does not increase the DCI payload size or restrict the usage of other DCI fields, it does pose the restrictions on scheduling. For example, half of the HARQ processes are restricted to be scheduled at a slot.

This scheduling restriction may not be aligned with SPS configuration. In existing NR SPS configuration, the HARQ process number is associated with slot index. The additional limitation on HARQ process number based on slot index may break the current association between HARQ process number and slot index. Furthermore, the impact of using this approach to the case of a single DCI scheduling multiple PDSCH/PUSCH is unclear.

Considering the fields and field sizes of DCI 0_2/1_2 are configurable, the option of extending the HARQ process number field up to 5 bits works well.

Proposal 1: Enhanced HARQ process number indication is supported for DCI 0-2/1-2 by extending the HARQ process number field up to 5 bits.

Unlike DCI 0_2/1_2, the other DCI formats, i.e., 0_0/1_0 or 0_1/1_1, have non-configurable field and field size. Hence, it is not desirable to extend the size of the HARQ process number field. In existing DCI formats 0_0/1_0/0_1/1_1, the HARQ process number field is fixed of 4 bits. The same field size should be maintained even if the number of HARQ processes is increased to 32. We prefer to re-interpret DCI fields to indicate up to 32 HARQ process numbers. Specifically, the existing 4-bit HARQ process number field, together another bit field in DCI, are jointly used to indicate HARQ process number.

One possible solution is that 1 redundancy version (RV) bit is used to indicate the MSB of HARQ process number, and only RV0 and RV3 are used when the supported HARQ process number is 32. One benefit of this scheme is that the soft buffer size per HARQ process is reduced by restricting to RV0 and RV3 at high code rates. Hence, the total soft buffer size over all HARQ processes may not increase even if the number of HARQ processes is increased to 32.

Note that the limitation of RV version is already used in NR release 16. For example, in DCI formats 0_2 and 1_2 for URLLC feature, one bit to indicate RV is supported, where only RV0 and RV3 are used (cf. Table 7.3.1.2.3-1 in [5]). In another example, in DCI format 0_1 for NR-U feature, one bit to indicate RV is supported, where only RV0 and RV2 are used (cf. Table 7.3.1.1.2-34 in [5]).

Proposal 2: Enhanced HARQ process number indication is supported for DCI 0_0/1_0 or 0_1/1_1 by reusing one bit from another DCI bit field (e.g., RV field).

## Mechanism for disabling HARQ feedback

It was discussed [5] that the parameters (e.g., for PDSCH or PUSCH) could be different for HARQ processes with enabled HARQ feedback and for HARQ processes with disabled HARQ feedback. For example, the configured aggregation factor for HARQ processes with disabled HARQ feedback can be larger than the configured aggregation factor for HARQ processes with enabled HARQ feedback. This is to enhance the data transmission reliability for HARQ processes with disabled HARQ feedback.

Proposal 3: Support to have different configurations for HARQ processes with or without HARQ feedback.

## Enhancement on the HARQ-ACK codebook

Type-1 HARQ-ACK codebook is semi-statically constructed. The size of type-1 HARQ-ACK codebook depends on the total number of candidate PDSCH reception occasions, rather than whether or not there is actually scheduled PDSCH. If no PDSCH is received in a candidate PDSCH reception occasion due to not detecting a corresponding DCI, then NACK is generated. Here, overhead reduction of type-1 HARQ-ACK codebook is not the main design consideration.

The reduction of type-1 HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback may lead to ambiguity on interpreting HARQ-ACK codebook, especially in DTX scenario. If a UE fails decoding a DCI, it does not know whether HARQ feedback is enabled or disabled for the corresponding HARQ process number. If this HARQ process number is configured with disabled feedback, then gNB expects ACK/NACK feedback for this transmission. If this HARQ process number is configured with enabled feedback, then gNB does not expect ACK/NACK feedback for this transmission. In either way, UE may generate a HARQ-ACK codebook with a different size as expected from gNB.

Hence, we should not reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback.

If one bit is to be generated for a HARQ process with disabled HARQ feedback, then one option is always setting that bit as NACK. However, this does not deliver useful information to gNB. Especially, it does not distinguish between DTX and the case of HARQ process with disabled HARQ feedback.

A better option is that UE generates ACK if a DCI is received which indicates a HARQ process with disabled feedback. On the other hand, UE generates NACK if no DCI is detected. This bit information facilitates gNB’s scheduling of higher layer retransmission.

Proposal 4: In type-1 HARQ-ACK codebook construction, UE does not reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback.

Type-2 HARQ-ACK codebook construction for NTN has been discussed in RAN1 # 104-e meeting. It was agreed [3] that the type-2 HARQ codebook size is reduced to only include HARQ-ACK of PDSCH with feedback-enabled HARQ processes. The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback enabled/disabled HARQ processes are still open.

In our view, the C-DAI and T-DAI counting for DCI of PDSCH with feedback enabled HARQ processes should follow the legacy way, i.e., C-DAI and T-DAI are given their true values.

There are several alternatives of C-DAI and T-DAI for the DCI of PDSCH with feedback disabled HARQ process. In the first alternative, the C-DAI and T-DAI are given their true values (i.e., count of feedback-enabled processes), despite they are not incremented. In the second alternative, the C-DAI and T-DAI are given a reserved value. In the third alternative, a separate set of C-DAI and T-DAI counts for HARQ process with disabled feedback is used. In the fourth alternative, the C-DAI is a reserved value and the T-DAI is given its true value (i.e., count of feedback-enabled processes).

These different alternatives are shown in the columns in Figure 1.

Figure 1: Different options/alternatives of C-DAI and T-DAI

With respect to the C-DAI and T-DAI count for PDSCH with feedback disabled HARQ processes, we do not see the benefit and necessity of keeping a separate set of C-DAI and T-DAI for feedback disabled HARQ processes (i.e., Alternative 3).

The second alternative has the benefit of saving the DCI bit fields of C-DAI and T-DAI. However, the first alternative has the advantage of indicating the total number of HARQ-ACK bits, which benefits in addressing the last DCI missing issue.

Consider an example of two carriers where carrier 1 delivers PDSCH with feedback disabled HARQ processes and carrier 2 delivers PDSCH with feedback enabled HARQ processes. Suppose a UE decodes the PDCCH on carrier 1 but does not decode the PDCCH on carrier 2. If the second alternative is used, then UE does not know how many HARQ-ACK bits to feedback. If the first alternative is used, then UE knows the exact number of HARQ-ACK bits to feedback based on the T-DAI detected on carrier 1.

Actually, we do not think the C-DAI for PDSCH with feedback disabled HARQ processes is needed. Hence, we propose that in the DCI of PDSCH with feedback disabled HARQ processes, the T-DAI is given its true value and the C-DAI is given a reserved value. This is shown as the fourth alternative in Figure 1.

Proposal 5: In type-2 HARQ-ACK codebook construction,

- In DCI of PDSCH with feedback enabled HARQ processes, C-DAI and T-DAI are given their true values.

- In DCI of PDSCH with feedback disabled HARQ processes, T-DAI is given its true value and C-DAI is given a reserved value.

The type-2 HARQ-ACK codebook construction for SPS PDSCH is open. The similar issue is also open for type-1 HARQ-ACK codebook only for SPS PDSCH reception.

In release 16 terrestrial network, a SPS configuration may include multiple HARQ processes. The ordering of HARQ-ACK bits for SPS PDSCH follows the “serving cell index first, SPS configuration index second, and slot index third” rule.

In NTN, enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process. It is unclear whether this principle is applicable to the HARQ processes in a SPS configuration. If it is applicable to SPS configuration, i.e., the HARQ processes in a SPS configuration can have different HARQ feedback settings, then the ordering rule of HARQ-ACK bits for SPS PDSCH needs to be further studied.

Considering a SPS configuration is aimed to serve a certain type of data, it may not be necessary to apply different feedback settings on the HARQ processes in a SPS configuration. In case all the HARQ processes in a SPS configuration have the same HARQ feedback setting (i.e., enabled or disabled HARQ feedback), then the existing ordering rule of HARQ-ACK bits for SPS PDSCH could be modified to “serving cell index first, SPS configuration index with HARQ feedback enabled second, and slot index third”.

Proposal 6: For type-1 HARQ-ACK codebook only for SPS PDSCH and for type-2 HARQ-ACK codebook for SPS PDSCH, consider whether to support the case where SPS configuration includes HARQ processes with different feedback settings.

## PDSCH scheduling restriction

The PDSCH scheduling restriction was discussed in RAN1 #104-e meeting. It was agreed [3] that for a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X

after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. The working assumption is X=Tproc,1. It is open whether this time gap can be further extended to max(Tproc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1=0.

The unit of K1 is in slots, while Tproc,1 is in absolute time. It is not directly comparable between these two values. Also, in fallback DCI, the time gap between PDSCH reception and PUCCH transmission does not rely on configurations and has range of 1 to 8 slots. If a UE has configured dl-DataToUL-ACK for non-fallback DCIs, it is still possible for this UE to report HARQ-ACK within a time gap smaller than the minimum configured k1 value, in case scheduled by fallback DCI. Hence, we think the minimum configured k1 value should not be used to determine the minimum time gap of two PDSCHs of the same HARQ process number with feedback disabled.

Proposal 7: Confirm the working assumption of X=Tproc,1, where X is the minimum time gap for a UE to receive two PDSCHs or set of slot-aggregated PDSCHs with the same HARQ process number with feedback disabled.

# Conclusion

In this contribution, we provided our views on HARQ enhancements for NTN. Our proposals are as follows:

Proposal 1: Enhanced HARQ process number indication is supported for DCI 0-2/1-2 by extending the HARQ process number field up to 5 bits.

Proposal 2: Enhanced HARQ process number indication is supported for DCI 0_0/1_0 or 0_1/1_1 by reusing one bit from another DCI bit field (e.g., RV field).

Proposal 3: Support to have different configurations for HARQ processes with or without HARQ feedback.

Proposal 4: In type-1 HARQ-ACK codebook construction, UE does not reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback.

Proposal 5: In type-2 HARQ-ACK codebook construction,

- In DCI of PDSCH with feedback enabled HARQ processes, C-DAI and T-DAI are given their true values.

- In DCI of PDSCH with feedback disabled HARQ processes, T-DAI is given its true value and C-DAI is given a reserved value.

Proposal 6: For type-1 HARQ-ACK codebook only for SPS PDSCH and for type-2 HARQ-ACK codebook for SPS PDSCH, consider whether to support the case where SPS configuration includes HARQ processes with different feedback settings.

Proposal 7: Confirm the working assumption of X=Tproc,1, where X is the minimum time gap for a UE to receive two PDSCHs or set of slot-aggregated PDSCHs with the same HARQ process number with feedback disabled.

# References

- RP-210908, “Solutions for NR to support non-terrestrial networks (NTN),” Mar. 2021.

- R1-2102143, “Summary#4 of AI 8.4.3 for HARQ in NTN,” Feb. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104-e Meeting, Feb. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- 3GPP TS 38.212, NR; Multiplexing and channel coding, v16.5.0, Mar. 2021.