3GPP TSG RAN WG1 #105-e	R1-2105109

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.5.5

Source:	Apple Inc.

Title:	Views on potential enhancements for NLOS mitigation in Rel-17 positioning

Document for:	Discussion, Decision

# Introduction

In 3GPP RAN#90-e, a work item for Rel-17 NR positioning enhancements was approved and was updated in RAN#91-e [1]. One of the objectives of Rel-17 positioning WI, is about possible enhancements to mitigate/detect NLOS, as it can be found below [1]:

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

In this contribution, we present our views on potential techniques and required signalling to enhance NR positioning accuracy by mitigating multi-path/NLOS.

# NLOS mitigation and specification impact

Although the multi-path/NLOS may impact the accuracy of any of positioning techniques, but it is understood that in general, multi-path/NLOS may have a bigger impact on the accuracy of angle-based positioning techniques, like DL-AoD/UL-AoA, The reason is that if RSRP measurements associated to positioning RS, is not tied with the first detected path in time, i.e. if RSRP measurement is not path-specific but for each beam it is measured on the received power across all the detected paths, in case there is a NLOS path that is stronger than the LOS, the estimated DL-AoD (UL-AoA) based on such RSRP calculations is wrongly biased toward the NLOS path. For the rest of discussion, we focus on DL-AoD but the proposals are dual for UL-AoA.

In order to mitigate NLOS path, RAN1 has already made couple of agreements. For example, it was agreed in RAN1# 104e that path specific PRS measurement and report is studied, as follows [2]:

Agreement: For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path

  - Option 1: Information corresponds to PRS-RSRP of the first arriving path

  - Option 2: Information corresponds to the angle of departure of the first arriving path

  - Option 3: Information corresponds to the arrival time of the first path

  - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path

  - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path

- FFS: Reporting of additional path to the first arriving path.

- FFS: Measurement definition details

- FFS: additional assistance data to support these enhancements

- FFS: how the “first path” is selected among PRS resources in a PRS resource set

- Note 1: Supporting multiple options as well as none of the options above is not precluded.

In our view, Option 1 in which PRS-RSRP is determined based on the first detected arrival path, can potentially mitigate NLOS/multi-path, but if the first detected path is not associated with LOS, e.g. for the case that LOS path is highly blocked, then option 1 fails to eliminate NLOS contribution to PRS-RSRP report. Similarly, option 2 may fail if the first detected arrival path is not associated with LoS. Option 3 is useful when multiple reports from transmissions of the same TRP is compared together, then LMF (or UE in case of UE-B) will know which PRS transmission(s) has gone through NLOS. However, in case all PRS transmissions from a specific TRP experience reflection and are received through NLOS, with highly blocked LoS, then option 3 fails as well.

As mentioned in our companion paper [3], we think the path-specific PRS-RSRP measurement should be performed within a configured time window. The concept of time window for positioning is already defined in Rel-16 specification. For example, in 38.214 Sec. 5.1.6.5, the UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS. Similarly, a “search window” is defined for UL-RTOA technique, which “indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Rreference Time” (38.455, Sec. 9.2.26). This concept can be extended for PRS-RSRP measurements at a UE, where UE is assisted by LMF about a timing window per TRP, wherein the LOS path for a PRS reception from that TRP is expected to arrive, as shown in Figure 1. The configuration of such window needs some priori knowledge of UE’s location, similar to the configuration of nr-DL-PRS-ExpectedRSTD. Once the window is indicated by LMF, UE may be further indicated to perform PRS-RSRP measurement only based on paths that lie within the window, i.e., all paths that arrive out of the window duration will not contribute into PRS-RSRP measurements.

Based on what we discussed above, we make the following proposal:

Proposal 1: For DL-AoD technique, support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

- Alternatively or additionally, for DL-AoD technique, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

Figure 1: Paths that arrive out of the indicated time window W don’t contribute in PRS-RSRP measurements.

Another procedure that may identify NLOS impact on PRS-RSRP measurements is obtained once the relative PRS-RSRP, which is the relative power of the first detected path to the measured RSRP, is also measured and reported. The larger the relative PRS-RSRP, the larger contribution of the first detected path to the PRS-RSRP. In the example shown in Fig. 2, while PRS-RSRP from Beam2 is larger than that of Beam1, but the ratio of the first detected path to the PRS-RSRP is less for Beam2 in comparison with Beam1. The drawback of this approach comes from the case that the first detected path itself is not associated with a LOS path, e.g., due to severely blocked LOS path.

Proposal 2: For both UE-based and UE-assisted methods of DL-AoD technique, the relative power of the first detected path to the measured RSRP is also measured and reported.

Figure 2: Relative PRS-RSRP measurements.

# Conclusions

In this contribution, we provided our thoughts on potential techniques to enhance the positioning accuracy for the NR by meeting multi-path/NLOS. Based on views shared in this paper, we make the following proposals:

Proposal 1: For DL-AoD technique, support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

- Alternatively, or additionally, for DL-AoD technique, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

Proposal 2: For both UE-based and UE-assisted methods of DL-AoD technique, the relative power of the first detected path to the measured RSRP is also measured and reported.

# References

- RP-210903, “New WID on NR Positioning Enhancements”, March 2021

- Chairman’s Notes RAN1#104-e final

- R1-2105107, Accuracy enhancements for DL-AoD positioning technique, Apple Inc.