3GPP TSG RAN WG1 #105-e			R1-2105138

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.15.1

Source:	Apple

Title:	On Link Budget Analysis of IoT NTN

Document for:	Discussion/Decision

# Introduction

The study item description of NB-IoT/eMTC support for NTN was provided in [1]. One objective of this study item is to identify scenarios applicable to NB-IoT/eMTC. The following agreements have been made in RAN1 #104b-e meeting [2], [3].

Agreement:

Capture in TR 36.763 the summary of link budget results from contributing companies in Appendix 1, Section 6.1.1

NOTE 1: The summary in Appendix 1, Section 6.1.1 can be further checked and revised during the drafting of Text Proposal as necessary.

NOTE 2: The summary of link budget results will be captured with alignment between contributing companies

Agreement:

Capture the detailed link budget results from contributing companies in a separate spreadsheet

Agreement:

Capture parameters associated with Set 4 for maximum beam diameter of 1700 km in a separate table in TR 36.763:

NOTE: There is no impact on Table 6.1-1 : IoT NTN reference scenario parameters in TR 36.763

| 3GPP TR 36.763 V0.1.0 Table 6.1-1 parameters that could be impacted by the beam size revision: | Current values in TR 36.763 V0.1.0 for LEO 600 km | Assumed and computed values under the consideration of a beam of 1700 km in diameter pointed at Nadir: | Comment |
| --- | --- | --- | --- |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 30° for service link and 10° for feeder link | Assumed value for service link is higher than value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max distance between satellite and C-IoT device at min elevation angle | 1,932 km | 1075.8 km (Computed for a terminal located at the beam edge, corresponding to an elevation angle of 30 degrees) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max Round Trip Delay (propagation delay only) | 25.77 ms (service and feeder links) | 20.05 ms (Computed for a terminal located at the beam edge, corresponding to an elevation angle of 30 degrees. Feeder link elevation angle kept at 10º) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max differential delay within a cell | 3.12 ms | 1.58 ms (Computed as the maximum differential delay between a device at beam edge and one at beam center) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6)” | 24 ppm | 19,95 ppm (Computed for a terminal at beam edge, corresponding to an elevation angle of 30 degrees) | Computed value is lower than current value in Table 6.1-1 in TR 36.763. No revision needed. |

Agreement:

Include the following in TR36.763

Add MEO scenario D in Table 4.2-1 in TR 36.763.

Table 4.2-1: IoT NTN reference scenarios (TR 36.763)

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |
| MEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km) | Scenario D |

Agreement:

Add MEO IoT NTN reference scenario parameters in Table 6.1-1 in TR 36.763:

Table 6.1-1: IoT NTN reference scenario parameters (TR 36.763)

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C | MEO based non-terrestrial access network -Scenario D |
| --- | --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth | circular orbiting at medium altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km | 10,000 km |
| Frequency Range | < 6 GHz (e.g. 2 GHz in S band) |  |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL) - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |  |
| Payload | Transparent type | Transparent Type | Transparent type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) | Scenario D: The beams move with the satellite |
| Max beam footprint size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) | 4018 km |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) | 14018 km |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) | 95.19 ms (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km | 13.4 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) | 7.5 ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) | 0.003 ppm/s |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |  |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam. NOTE 4: The use of a Circular polarized antenna is optional. NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888 for device can be assumed for link budget. The noise figure is device vendor implementation specific. NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 7: System bandwidth is FFS |  |  |  |

Agreement:

Include MEO Set-5 parameters for link budget analysis in a new Table 6.2-8 in TR 36.763, as a representative characterization of NTN-IoT scenarios with MEO altitude and characteristics:

Table 6.2-8: Sets of satellite parameters for link budget and system level evaluations

|  | Proposed MEO Scenarios (Set 5) |
| --- | --- |
| Satellite orbit | MEO |
| Satellite altitude | 10,000 km |
| Payload characteristics for DL transmission |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| Satellite EIRP density | 45.4 dBW/MHz |
| Satellite Tx max Gain | 28.1 dBi |
| 3dB beamwidth | 6.5 degrees |
| Satellite beam diameter (at nadir pointing) | 1140 km |
| Payload characteristics for UL reception |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| G/T | 3.8 dB/K |
| Satellite Rx max Gain | 28.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter for the parabolic reflector modelled in Sec. 6.4.1 of TR 38.811. Other antenna models can be considered. |  |

Agreement:

Add MEO Set-5 satellite parameters for system level simulator calibration in a new Table 6.2-9 in TR 36.763:

Table 6.2-9: Set-5 parameters for link budget analysis

| Set 5 | MEO |
| --- | --- |
| 3 dB Beam width (HPBW) | 6.5 degrees |
| Central beam center elevation | 90 degrees |
| Central beam edge elevation | 81.6 degrees |
| Central beam edge satellite-UE distance | 10042 km |

Agreement:

Add observation in TR 36.763:

The doppler shift/variation and the delay variation for MEO are smaller than for LEO. The maximum delay for MEO is smaller than for GEO. The IoT-NTN enhancements for LEO and GEO should be sufficient to support MEO.

NOTE: The parameter set for MEO is only for information/reference and evaluation/enhancements are mainly considered for GEO and LEO. These enhancements can be applicable for MEO.

In this contribution, we provide link budget analysis based on the newly agreed IoT NTN reference scenario parameters.

# Discussion

Besides three different IoT NTN scenarios and their respective reference parameters [4], a new IoT NTN scenario and its respective reference parameters have been agreed [2].

A common set of link budget parameters (e.g., UE power class, UE noise figure, scintillation loss, atmospheric loss, polarization loss, shadow margin, etc) have been agreed [5]. Furthermore, besides four existing satellite parameter sets (i.e., Set 1 and Set 2 in [6], Set 3 and Set 4 in [5]), one additional satellite parameter set (i.e., Set 5) has been agreed to be used for IoT NTN link budget calculations.

These parameters are used in our link budget analysis.

For set 5 satellite parameters in [2], the link budget evaluations for downlink are provided in Table 1 and the link budget evaluations for uplink is provided in Table 2. In summary, we have the following observations.

Observation 1: For set 5 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.41 dB.

Observation 2: For set 5 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 3.75/15 30/45/90/180/360/1080 kHz is -0.12/-6.14/-9.15/-10.92/-13.93/-16.94/-19.95/-24.72 dB, respectively.

Table 1: DL NB-IoT/eMTC link budget based on set 5 satellite parameters in [2]

| Satellite orbit | MEO-10000 (DL) |  |
| --- | --- | --- |
|  | NB-IoT | eMTC |
| Satellite EIRP density (dBW/MHz) | 45.4 |  |
| Channel bandwidth (MHz) | 0.18 | 1.08 |
| Satellite EIRP (dBm) | 67.95 | 75.73 |
| Central beam edge elevation (degree) | 81.6 |  |
| Max. distance between satellite and IoT device (km) | 10042 |  |
| Carrier frequency (GHz) | 2 |  |
| Free space path loss (dB) | 178.51 |  |
| Shadowing (dB) | 3 |  |
| Atmospheric path loss (dB) | 0.1 |  |
| Scintillation loss (dB) | 2.2 |  |
| Polarization loss (dB) | 3 |  |
| IoT antenna temperature (K) | 290 |  |
| Thermal noise (dBW/Hz) | -174 |  |
| Noise floor (dBm) | -121.45 | -113.67 |
| IoT noise figure (dB) | 9 |  |
| IoT device antenna gain (dBi) | 0 |  |
| CNR (dB) | -4.41 | -4.41 |

Table 2: UL NB-IoT/eMTC link budget based on set 5 satellite parameters in [2]

| Satellite orbit | MEO-10000 (UL) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT device max Tx power (dBm) | 20 |  |  |  |  |  |  |  |
| IoT device antenna gain (dBi) | 0 |  |  |  |  |  |  |  |
| IoT device EIRP (dBm) | 20 |  |  |  |  |  |  |  |
| Central beam edge elevation (degree) | 81.6 |  |  |  |  |  |  |  |
| Max. distance between satellite and IoT device (km) | 10042 |  |  |  |  |  |  |  |
| Carrier frequency (GHz) | 2 |  |  |  |  |  |  |  |
| Free space path loss (dB) | 178.51 |  |  |  |  |  |  |  |
| Shadowing (dB) | 3 |  |  |  |  |  |  |  |
| Atmospheric path loss (dB) | 0.1 |  |  |  |  |  |  |  |
| Scintillation loss (dB) | 2.2 |  |  |  |  |  |  |  |
| Polarization loss (dB) | 3 |  |  |  |  |  |  |  |
| Antenna temperature (K) | 290 |  |  |  |  |  |  |  |
| G/T (dB/K) | 3.8 |  |  |  |  |  |  |  |
| Satellite Rx gain (dBi) | 28.42 |  |  |  |  |  |  |  |
| Channel bandwidth (kHz) | 3.75 | 15 | 30 | 45 | 90 | 180 | 360 | 1080 |
| CNR (dB) | -0.12 | -6.14 | -9.15 | -10.92 | -13.93 | -16.94 | -19.95 | -24.72 |

# Conclusion

In this contribution, we provided our link budget analysis on IoT over NTN. Our observations are as follows:

Observation 1: For set 5 satellite parameters, the CNR for DL NB-IoT/eMTC is -4.41 dB.

Observation 2: For set 5 satellite parameters, the CNR for UL NB-IoT/eMTC with bandwidth 3.75/15 30/45/90/180/360/1080 kHz is -0.12/-6.14/-9.15/-10.92/-13.93/-16.94/-19.95/-24.72 dB, respectively.

# References

- RP-202689, “New study WID on NB-IoT/eMTC support for NTN,” Dec. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104b-e Meeting, Apr. 2021.

- R1-2103962, “Summary #3 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC,” Apr. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104-e Meeting, Feb. 2021.

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.0.0, Dec. 2019.