3GPP TSG RAN WG1 #105-e		R1-2105158

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.2.5

Source:	Sony

Title:	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz

Document for:	Discussion / Decision

Introduction

In the WID [1], the following parts are included in the objective.

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

In this contribution, PDSCH/PUSCH enhancements for above 52.6 GHz operation are discussed.

Discussion

Multi-PUSCH scheduling

In RAN1#104-e meeting [2], the following agreements were made.

Agreement: The multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17. - FFS: Applicability to multi-PDSCH scheduling. Agreement: - For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling. - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled). - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline). - TDRA: Down-select among - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI. - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs - Applicability to multi-PDSCH scheduling in Rel-17. - Note: Other enhancements are not precluded.

CBGTI

CBG-based transmission is beneficial when time domain channel characteristic is drastically different between symbols where PUSCH is scheduled. However, for 480 kHz and 960 kHz, since slot length is very short, time domain channel characteristic on duration of a slot would be quite similar. Therefore, there is no motivation to support CBG-based transmission for multi-PUSCH scheduling.

Proposal 1: CBG-based transmission should not be supported for multi-PUSCH scheduling.

CSI-request

In Rel-16 NR-U, AP-CSI is carried by the last scheduled PUSCH to reduce dropping probability due to LBT failure. We don’t see strong motivation to apply different rule to Rel-17 multi-PUSCH scheduling compared to Rel-16.

Proposal 2: For AP-CSI feedback, the same rule as Rel-16 multi-PUSCH should be applied to Rel-17 multi-PUSCH scheduling.

URLLC related fields

According to TR 38.807 [3], URLLC use case was considered in NR above 52.6 GHz, e.g. factory automation and industrial IoT. Therefore, it is straightforward to support URLLC related fields if NR above 52.6 GHz supports URLLC use case.

Remaining issue is whether multiple PUSCHs scheduled by one DCI indicate the same priority or different priority. If all PUSCH have the same priority, the same DCI overhead as single scheduling is considered. However, scheduling flexibility would be decreased. On the other hand, if it is allowed to carry multiple PUSCHs that have different priority by multi-PUSCH scheduling, scheduling flexibility would be increased. On the other hands, more indication bits will be necessary in this case, which has impact on DCI size.

Proposal 3: URLLC related fields should be supported for multi-PUSCH scheduling

- Further study whether single or multiple fields related to URLLC are applied to multiple PUSCH scheduled by single DCI.

Multi-PDSCH scheduling

In this section, we discuss about multi-PDSCH scheduling

MCS/NDI/RV

In RAN1#104-e meeting [2], the following agreement related to MCS/NDI/RV/HPN was made.

Agreement: For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

In current NR spec, 2nd TB is used when the number of transmission layer is greater than 4. It is unlikely to use more than 4 transmission layer on frequencies above 52.6 GHz considering multipath property on mmWave. Therefore, scheduling of 2nd TB for PDSCH is not beneficial for NR above 52.6 GHz.

Observation 1: Scheduling of 2nd TB for each PDSCH is not beneficial for NR above 52.6 GHz.

Even in current NR spec, felids of MCS/NDI/RV for the 2nd TB could be disable when maxNrofCodeWordsScheduledByDCI equals 1. No optimization would be necessary.

CBG-based transmission

As we discussed on section 2.1.1, CBG-based transmission for PDSCH would not be beneficial when symbol length is shorter compared to time varying of fading channel. If CBG-based transmission is supported, DCI overhead would be increased. therefore, CBG-based transmission should not be supported for multi-PDSCH scheduling.

Proposal 4: CBG-based transmission should not be supported for multi-PDSCH scheduling.

Priority indicator

As we discussed on section 2.1.3, it is straightforward to support URLLC related fields if NR above 52.6 GHz supports URLLC use case. Therefore, priority indicator should be supported for multi-PDSCH scheduling.

If only single PUCCH is supported for multi-PDSCH scheduling, there is only one option that multiple PDSCH scheduled by single DCI is the same priority, because low priority HARQ-ACKs are always drooped due to single PUCCH. On the other hand, if multiple PUCCH is supported for multi-PDSCH scheduling, it could be further discussed whether single or multiple fields to indicate priority are applied to multiple PDSCH scheduled by the single DCI. For example, it could be considered that 1st PUCCH is associated with high priority HARQ-ACKs and 2nd PUCCH is associated with low priority HARQ-ACKs, respectively.

Proposal 5: Priority indicator should be supported for multi-PDSCH scheduling

- Further study whether single or multiple fields related to URLLC are applied to multiple PDSCH scheduled by single DCI if multiple PUCCH scheduled by the single DCI is supported.

HARQ feedback/HARQ process

In RAN1#104-e meeting [2], the following agreement related to PUCCH resource indication was made.

Agreement: - For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1, - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. - It is noted that granularity of K1 can be separately discussed. - FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

Table 1 and 2 in appendix show PDSCH decoding time N1 for PDSCH processing capability 1 and 2, which is specified in TS 38.214 [2]. For PDSCH processing capability 1, PDSCH decoding time N1 is more than 20 symbols if SCS is 120 kHz. Although PDSCH decoding time N1 for 480 kHz and 960 kHz SCS has not been decided, based on processing time N1 in the case of 120 kHz SCS, we can predict that processing time N1 for 480 kHz and 960 kHz SCS would be about 80 symbols (5.7 slots) and 160 symbols (11.4 slots), respectively. This implies that HARQ feedback latency would be higher for 480 kHz and 960 kHz SCS.

Large HARQ feedback delay causes shortage of HARQ process. Figure 1 shows issue about HARQ process shortage. In the case of 960 kHz SCS, 8 slots span of PDCCH monitoring occasion would be reasonable, considering the same PDCCH monitoring frequency as 120 kHz SCS. However, as mentioned above, PUCCH cannot be scheduled in a slot where K1 is less than 12 slots. In this case, the situation that PDSCH cannot be scheduled in several slots may happen since all HARQ processes are occupied. Since DL resource will not be efficiently utilized, it causes degradation of DL throughput.

Figure 1. Shortage of HARQ process caused by higher HARQ feedback timing

To solve such issue, the following three options could be considered.

- Multiple PUCCH resource indication by one DCI

- Multiple DCI in a slot

- Increasing the number of HARQ process

For option 1 and option 2, since multiple HARQ feedback timing could be indicated, HARQ feedback delay could be decreased and HARQ process could be released earlier. However, for option 1, since multiple PRI and PDSCH-to-HARQ_feedback timing indication needs to be contained in one DCI, DCI size would be increased. For option 2, although DCI size is not changed, PDCCH blocking would be increased since multiple DCIs are scheduled in the same CORESET. Moreover, it has impact on UE implementation complexity because UE has to decode more than one DCI in a slot. Option 3 can solve the issue on shortage of HARQ process and would be less specification impact since the support of increasing the number of HARQ processes has already agreed and been specifying in Rel-17 NTN. However, HARQ feedback delay is not improved by option 3. Moreover, it has impact on UE implementation since more HARQ buffer needs to be implemented.

Proposal 6: If PDSCH processing time is long, at least one of the following solutions should be considered

- Multiple HARQ feedback timing indication by one DCI

- Multiple DCI in a slot

- Increasing the number of HARQ process

Type-1 HARQ codebook

In RAN1#104-e meeting, the following agreement related to Tyep-1 HARQ codebook was made.

Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered, - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set - Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table - Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table - FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

In option 2, there may be an issue when some SLIV overlap with invalid symbols (e.g. UL symbol). Especially, if last SLIV overlaps with invalid symbols, UE would consider the all SLIVs are invalid then not reserve HARQ-ACK bits corresponding to PDSCHs possibly scheduled on the invalid SLIVs.

On the other hands, in option 1, each SLIV has virtual K1. For example, when 8 convective PDSCHs are scheduled and it is indicated as K1 = 1, other PDSCHs than the last PDSCH have virtual K1 of which value = 8, 7, 6, 5, 4, 3, and 2, respectively. Even when some SLIV overlaps with invalid symbols, other SLIVs could be considered as valid using virtual K1, then UE would reserve HARQ-ACK bits associated with PDSCHs possibly scheduled on the other valid SLIVs. Therefore, option 1 could solve the issue when SLIV overlaps with invalid symbols.

Proposal 7: Support Option 1 (The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set)

DAI

In RAN1#104-e meeting, the following agreement related to DAI was made.

Agreement: For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Alt 1 is similar approach for sub-codebook of CBG-based and TB-based transmission. In Alt 1, single PDSCH scheduling and multi-PDSCH scheduled are associated with different HARQ-ACK sub-codebook. If missed PDCCH is detected, UE would reserve N HARQ-ACK bits where N is the maximum configured number of PDSCHs scheduled by one DCI, regardless of the number of actual scheduled PDSCHs. Therefore, Alt 1 would lead increasing HARQ-ACK payload size.

Compared to Alt 1, Alt 2 could keep small HARQ-ACK payload size because HARQ-ACK codebook size depends on the number of actual scheduled PDSCHs, while C-DAI/T-DAI size needs to be increased. Considering the benefit of Type-2 HARQ codebook is smaller HARQ-ACK payload size compared to Type-1 HARQ codebook, Alt 2 would be more preferable design for Type-2 HARQ codebook.

Proposal 8:C-DAI/T-DAI for multi-PDSCH scheduling should be counted per PDSCH.

NR-U related fields

LBT is required for 60 GHz unlicensed operation, which means the case that HARQ-ACKs cannot be transmitted due to LBT failure sometime occurs. In Rel-16 NR-U, to recover missed HARQ-ACK transmission due to LBT failure, enhanced Type-2 HARQ CB and Type-3 HARQ CB has been introduced. it is straightforward to support NR-U HARQ enhancement features (non-numerical K1, enhanced Type-2 HARQ CB, and Type-3 HARQ CB) also for multi-PDSCH scheduling on 60 GHz unlicensed operation.

If single PUCCH is only supported for multi-PDSCH scheduling, no enhancement from Rel-16 NR-U is necessary. On the other hands, if multiple PUCCH is supported for multi-PDSCH scheduling, NR-U related fields may need to be enhanced e.g. multiple fields (one-shot HARQ-ACK request, PDSCH group index, NFI, and number of requested PDSCH groups) needs to be contained in one DCI and each fields corresponds to each PUCCH.

Proposal 9: NR-U HARQ enhancement features (Non-numerical K1, enhanced Type-2 HARQ CB, and Type-3 HARQ CB) for multi-PDSCH scheduling should be supported.

- Further study how to indicate/determine PDSCH group if multiple PUCCH for multi-PDSCH scheduling is supported.

DCI for multi-PDSCH/PUSCH scheduling

As agreed in RAN1#104-e meeting, the multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17. Since the multi-PUSCH scheduling defined in Rel-16 NR-U is supported by DCI format 0_1, it is natural to support multi-PUSCH scheduling in Rel-17 by using DCI format 0_1. Moreover, on Rel-16 multi-PUSCH scheduling, single DCI can be indicated whether single PUSCH scheduling or multi-PUSCH scheduling. Since the same mechanism could be reused to Rel-17 multi-PUSCH scheduling, no new DCI format is needed for multi-PUSCH scheduling.

For DCI format used for multi-PDSCH scheduling, the same principle as multi-PUSCH scheduling is applicable to multi-PDSCH scheduling. Therefore, no new DCI format is needed and DCI format 1_1 can be used for multi-PDSCH scheduling.

Proposal 10: No new DCI format is needed for multi-PUSCH/PDSCH scheduling.

- The same DCI format is used for both single PUSCH/PDSCH scheduling and multi-PUSCH/PDSCH scheduling.

Proposal 11: At least DCI format 0_1 should be supported for multi-PUSCH scheduling.

Proposal 12: At least DCI format 1_1 should be supported for multi-PDSCH scheduling.

Multi-PDSCH/PUSCH may cause increasing DCI overhead compared to current DCI format 1_1. Large DCI size would cause increasing DCI decoding error and large delay due to retransmissions. In the case that a lot of DCI overhead for multi-scheduling is required, DCI enhancement may need to be additionally considered, e.g. 2-stage DCI.

Observation 2: DCI enhancement may need to be additionally considered in the case that a lot of DCI overhead for multi-PDSCH/PUSCH scheduling is required.

Conclusions

In this contribution, based on the above discussion, we have the following observations and proposals:

Proposal 1: CBG-based transmission should not be supported for multi-PUSCH scheduling.

Proposal 2: For AP-CSI feedback, the same rule as Rel-16 multi-PUSCH should be applied to Rel-17 multi-PUSCH scheduling.

Proposal 3: URLLC related fields should be supported for multi-PUSCH scheduling

- Further study whether single or multiple fields related to URLLC are applied to multiple PUSCH scheduled by single DCI.

Observation 1: Scheduling of 2nd TB for each PDSCH is not beneficial for NR above 52.6 GHz.

Proposal 4: CBG-based transmission should not be supported for multi-PDSCH scheduling.

Proposal 5: Priority indicator should be supported for multi-PDSCH scheduling

- Further study whether single or multiple fields related to URLLC are applied to multiple PDSCH scheduled by single DCI if multiple PUCCH scheduled by the single DCI is supported.

Proposal 6: If PDSCH processing time is long, at least one of the following solutions should be considered

- Multiple HARQ feedback timing indication by one DCI

- Multiple DCI in a slot

- Increasing the number of HARQ process

Proposal 7: Support Option 1 (The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set)

Proposal 8:C-DAI/T-DAI for multi-PDSCH scheduling should be counted per PDSCH.

Proposal 9: NR-U HARQ enhancement features (Non-numerical K1, enhanced Type-2 HARQ CB, and Type-3 HARQ CB) for multi-PDSCH scheduling should be supported.

- Further study how to indicate/determine PDSCH group if multiple PUCCH for multi-PDSCH scheduling is supported.

Proposal 10: No new DCI format is needed for multi-PUSCH/PDSCH scheduling.

- The same DCI format is used for both single PUSCH/PDSCH scheduling and multi-PUSCH/PDSCH scheduling.

Proposal 11: At least DCI format 0_1 should be supported for multi-PUSCH scheduling.

Proposal 12: At least DCI format 1_1 should be supported for multi-PDSCH scheduling.

Observation 2: DCI enhancement may need to be additionally considered in the case that a lot of DCI overhead for multi-PDSCH/PUSCH scheduling is required.

References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz,” TSG RAN Meeting #90-e, December 2020.

- RAN1#104-e chairman’s note

- TR 38.807

- TS 38.214

Appendix

The following tables are PDSCH processing time specified in TS 38.214.

Table 1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |