3GPP TSG RAN WG1 #105-e					    	                     R1-2105223

e-Meeting, May 10th – 27th, 2021

# Source:		     ETRI

# Title:		 Discussion on beam management for NTN

# Agenda item:    8.4.4

# Document for:  Discussion

Introduction

In RAN1#104-e, the following has been concluded on the NTN beam management [1].

Conclusion: Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management: - Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching. - Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL. - Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling. - Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP. - Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it. - Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility. - Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

With these elements, we discuss NTN beam management in more detail.

Discussion

Issue 1:

Figure 1 shows the case of applying BWP per beam within one cell. When an existing beam fails due to movement of the UE, the UE selects a new beam and reports it to the gNB. Thereafter, the UE monitors the response of the gNB to the new beam.

For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index   provided by higher layers, the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format

Fig 1. BWP per beam within one cell

with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4 within a window configured by BeamFailureRecoveryConfig [2].

RecoverySearchSpaceId of BeamFailureRecoveryConfig field is search space to use for BFR RAR. The network configures this search space to be within the linked DL BWP (i.e., within the DL BWP with the same BWP id) of the UL BWP in which the BeamFailureRecoveryConfig is provided. The CORESET associated with the recovery search space cannot be associated with another search space. Network always configures the UE with a value for this field when contention free random access resources for BFR are configured [3].

Accordingly, when the BWP id for measurement and the BWP id of the new beam are the same, the NR process can be applied. When the BWP id for measurement and the BWP id of the new beam are different, the UE may perform BWP switching by the gNB or the UE may implicitly perform BWP switching. When the UE implicitly performs BWP switching, overhead or processing time for BWP switching can be reduced. In this case, it is necessary to consider when the UE switches BWP and how the gNB can know the BWP switching of the UE.

Proposal 1:	When NTN FRF>1, implicit BWP switching of the UE may be considered according to the configuration of the BWP for beam measurement.

Issue 4:

This issue is related to the TA update procedure. In [1], an NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. The operation of closed loop and open loop TA control is FFS. In addition, it is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation.

Therefore, when FRF>1, re-synchronization in the switched BWP may be revisited after discussion of the TA update procedure and GNSS measurements in RAN4.

Proposal 2:	When NTN FRF>1, re-synchronization in the switched BWP may be revisited after discussion of the TA update procedure.

Issue 7:

Figure 2 shows the case where the serving satellite for the UE is changed from sat#0 to sat#1 for the fixed beam scenario.

Fig 2. Beam tracking and switching according to the movement of the satellite in the fixed beam scenario

In this case, the direction of the serving beam may be rapidly changed at a specific moment for all UEs within the coverage. This change may be distinguished from the beam tracking procedure for sat#0. Switching between beams having a large characteristic difference may affect TA update. In addition, a long processing time for beam measurement or RRC reconfiguration may be required. Accordingly, fast beam switching may be possible by distinguishing the TCI according to the time, angle, or information provided for the CHO, rather than tracking the beam for the same TCI.

Proposal 3:	In a fixed beam scenario, TCI may be configured according to beam characteristics (time, angle, etc.) to reduce overhead due to beam switching.

Conclusions

In this contribution, we present our considerations for beam management in NTN scenarios. Based on the discussion, we make the following proposals.

Proposal 1:	When FRF>1, implicit BWP switching of the UE may be considered according to the configuration of the BWP for beam measurement.

Proposal 2:	When FRF>1, re-synchronization in the switched BWP may be revisited after discussion of the TA update procedure.

Proposal 3:	In a fixed beam scenario, TCI may be configured according to beam characteristics (time, angle, etc.) to reduce overhead due to beam switching.

References

- R1-2102281, “Report of RAN1#104-e meeting”

- 3GPP TS 38.213 V16.5.0, “NR; Physical layer procedures for control (Release 16)”

- 3GPP TS 38.331 V16.4.1, “NR; Radio Resource Control (RRC) protocol specification (Release 16)”