3GPP TSG RAN WG1 Meeting #105-e	R1-2105267

e-Meeting, May 10– May 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev | - | Current version: | V16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE procedure for uplink MTB scheduling in TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When ce-PUSCH-MultiTB-Config is configured, if the PDSCH repetition number N=1 and UL index in DCI format 6-0A is set as ‘11’, the HARQ process number for corresponding TB needs clarification for the case of multiple TBs scheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following UE behavior: The UE is not expected to receive DCI format 6-0A with both the MSB and LSB of the UL index set to 1 for the case that multiple TB are scheduled when ce-PUSCH-MultiTB-Config is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The HARQ process number derived from ‘UL index’ and ‘Scheduling TBs for Unicast’ may be conflicted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

  - 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

-	for TDD UL/DL configuration 0 and a BL/CE UE in CEModeA, if the MSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, x = 7. The UE is not expected to receive DCI format 6-0A with both the MSB and LSB of the UL index set to 1 when N>1 or multiple TBs are scheduled when ce-PUSCH-MultiTB-Config is configured. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 6-0A

<Unchanged parts are omitted>