3GPP TSG RAN WG1 #105-e	R1-2105297

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.2.1

Source:	Samsung

Title:	Initial access aspects for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

In RAN1#104b-e [1], the following agreements have been made regarding the initial access aspects for 52.6 to 71 GHz [1]:

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

This contribution discusses the further design details for the initial access aspects for NR 52.6 to 71 GHz.

# Numerology of SS/PBCH block

## Numerology of SS/PBCH block for non-initial access case

In the last meeting [1], it has been agreed to support 480 kHz and 960 kHz SCS for SS/PBCH blocks in non-initial access wherein the SS/PBCH block doesn’t provide the CORESET#0/Type0-PDCCH configuration, and the sub-case of SS/PBCH block providing the CORESET#0/Type0-PDCCH configuration has been discussed without consensus.

The case of SS/PBCH block location and SCS are explicitly provided to the UE (non-initial access) and the SS/PBCH block provides the CORESET#0/Type0-PDCCH configuration includes at least the following two scenarios:

- Scenario 1: ANR purpose, wherein the SS/PBCH block location and SCS are provided by measurement object, and the UE needs to read the MIB of the corresponding SS/PBCH block to acquire the CORESET#0/Type0-PDCCH configuration, when the SS/PBCH block is from another operator.

- Scenario 2: cell reselection, wherein the SS/PBCH block location and SCS are provided by the SIBx, and the UE needs to read the MIB of the corresponding SS/PBCH block to acquire the CORESET#0/Type0-PDCCH configuration, when the SS/PBCH block is from a neighboring cell.

The first scenario has been discussed in the last meeting, while the technical aspects for the two scenarios are the same, hence, we focus on the first scenario in this contribution, with the understanding that both scenarios will be included.

A baseline procedure for ANR in Rel-15 and Rel-16 is shown in Figure 1, wherein the pre-step for ANR purpose is a regular RRM to report the physical cell ID and cell quality. In Rel-15 and Rel-16, the acquisition of the CGI is based on the acquisition of MIB and SIB1 from the neighboring cell of another operator.

Figure 1 Illustration of ANR procedure.

For 52.6 to 71 GHz, it is clear that the necessity of ANR, especially for inter-operator PCI confusion resolution, is still valid, particularly considering most the frequency band will operate on an unlicensed spectrum. Supporting the ANR purpose only with 120 kHz SCS is obviously not sufficient, since the network then has to implement two different types of SS/PBCH blocks, if 480 or 960 kHz control and data channels are used, which is against the motivation of implementing the system in a single numerology.

Regarding the method of providing the CORESET#0/Type0-PDCCH configuration for ANR purpose, two approaches were discussed in the last meeting:

- Approach 1: based on the MIB of the SS/PBCH block.

- Approach 2: based on dedicated signalling.

In our view, Approach 2 cannot be used for resolving the inter-operator PCI confusion. As described in Figure 1, the UE needs to read SIB1 of Cell B from another operator to acquire the CGI, but the CORESET#0/Type0-PDCCH configuration of Cell B may not be known by the serving operator (e.g. Cell A). In this sense, the serving operator cannot use dedicated signalling to let its UE know the CORESET#0/Type0-PDCCH configuration from the neighboring cell, and the UE has to read the MIB of the neighboring cell to acquire the configuration.

Moreover, Approach 2 will also lead to different UE behavior on determining the CORESET#0/Type0-PDCCH configuration for 120 kHz and 480/960 kHz SCS, since for 120 kHz, Approach 1 should be used by default, which is the same as Rel-15/Rel-16.

Based on above discussion, we have the following observation and proposal.

Observation 1: Using dedicated signalling to provide the CORESET#0/Type0-PDCCH configuration cannot resolve the inter-operator PCI confusion.

Proposal 1: Support ANR and inter-operator PCI confusion resolution for all supported SS/PBCH block subcarrier spacings, and the CORESET#0/Type0-PDCCH configuration is provided by the MIB of the SS/PBCH block.

## Numerology of SS/PBCH block for initial access case

For SS/PBCH block in initial access case, in addition to the aspects considered for SS/PBCH block in non-initial access case, the UE complexity for initial cell search should be the most important aspect to consider. More precisely, the UE complexity can be reflected by the number of extra searching synchronization raster entries for the new SCSs, in addition to the ones the UE needs to search for 120 kHz SCS.

There have been two types of synchronization and channel raster design in Rel-15 and Rel-16, for licensed spectrum and unlicensed spectrum, respectively.

Figure 2 Illustration of synchronization and channel raster in Rel-15 and Rel-16.

For Type 1 synchronization and channel raster design, channel raster interval is as small as subcarrier spacing level to provide full flexibility for operators to allocate the channels, and the synchronization raster is designed to guarantee at least one SS/PBCH block being implemented within the minimum channel bandwidth (MCBW). To achieve this purpose, the synchronization raster interval is chosen as Δsync=BWCH-BWSSB, wherein BWCH is the minimum channel bandwidth to include the corresponding SS/PBCH block with respect to the targeted SCS (not including the guard band), and BWSSB is the SS/PBCH block bandwidth with respect to the targeted SCS. For example, Type 1 synchronization and channel raster design is applicable to Rel-15 FR2. When the default SCS of SS/PBCH block is 120 kHz, BWCH=46.08 MHz, BWSSB=28.80 MHz, such that Δsync=17.28 MHz; when the default SCS of SS/PBCH block is 240 kHz, BWCH=92.16 MHz, BWSSB=57.60 MHz, such that Δsync=34.56 MHz.

For Type 2 synchronization and channel raster design, channel raster and synchronization raster are both fixed as single location for a given channel. There is one and only one synchronization raster entry within a channel with minimum channel bandwidth, and the synchronization raster interval is approximately same as minimum carrier bandwidth, subject to some rounding to integer number of RBs to be compatible with GSCN values. For example, Type 2 synchronization and channel raster design is applicable to Rel-16 NR-U, wherein Δsync=18.72 or 20.16 MHz.

For NR operated from 52.6 GHz to 71 GHz, both licensed and unlicensed bands could be included in this carrier frequency range, hence, both types of synchronization and channel raster design are being considered in RAN4. This section analyzes the complexity of each type of synchronization and channel raster design, and especially its impact to the selection of default SCS for SS/PBCH block for initial cell search.

Table 1 summarizes the synchronization raster interval for different SCS of SS/PBCH block, for both Type 1 and Type 2 synchronization and channel raster design, wherein the values of the minimum channel bandwidth is based on the RAN4 agreement (i.e., 100 MHz for 120 kHz SCS of data, 400 MHz for 480 kHz and 960 kHz SCS of data). Note that the value of synchronization raster interval is an approximation, and it can be further subject to changes due to the consideration of different rounding to integers and nesting of different rasters, but the trend across different SCSs should maintain the same. The following observation can be made from Table 1.

Observation 2: Supporting 240 kHz SCS SS/PBCH block for initial access has the largest complexity.

Table 1 Synchronization raster interval for NR 52.6 to 71 GHz.

| Sync Raster Interval (GSCN) | SCS of SSB (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 120 | 240 | 480 | 960 |
| Type 1 Sync and Channel Raster Design | 3 | 2 | 15 | 8 |
| Type 2 Sync and Channel Raster Design | 6 | 6 | 24 | 24 |

Table 2 summarizes the estimated number of synchronization raster entries for bands from 57 to 66 GHz, wherein a 10% guard band for each band is assumed.

Table 2 Number of synchronization raster entries for NR 57 to 66 GHz.

| Number of Sync Raster Entries |  | SCS of SSB (kHz) for initial access |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120 | 240 | 480 | 960 | 120+240 | 120+480+960 | 120+240+480+960 |
| Type 1 Sync and Channel Raster Design | 66-71 GHz band | 86 | 130 | 17 | 32 | 216 | 135 | 265 |
|  | 57-71 GHz band | 243 | 364 | 48 | 91 | 607 | 382 | 746 |
| Type 2 Sync and Channel Raster Design | 66-71 GHz band | 43 | 43 | 10 | 10 | 86 | 63 | 106 |
|  | 57-71 GHz band | 121 | 121 | 30 | 30 | 242 | 181 | 302 |

We also note that in NR FR2, at most 344 synchronization raster entries were supported for a given band, then in order to keep the same or similar UE complexity in initial searching a band between 52.6 GHz to 71 GHz, there is no issue with the initial access complexity for supporting both 480 kHz and 960 kHz SCS for SS/PBCH blocks.

Proposal 2: Support 480 kHz and 960 kHz SCS for SS/PBCH block in initial access case.

## Compromised proposal

In the last meeting, there was an intensive discussion on the SCS of SS/PBCH block to be supported for 52.6 to 71 GHz, and different components expressed their own preference and justifications. To facilitate the process, there can be a compromised proposal to try to maximize the use cases to be supported, and satisfies the interest of the most components. In order to control the initial access complexity and specification impact, some further restriction can be added. More precisely, we are supportive of a compromised proposal as follow.

Proposal 3:

- Support SSB with 240/480/960 kHz for initial and non-initial access with support of CORESET0/Type0-PDCCH configuration in the MIB.

  - SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

  - only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS

- It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries in the 57 – 71 GHz band no larger than [400] (Note: the total number of synchronization raster entries in FR2 for band n259 is 344). If the assumption cannot be satisfied, it’s up to RAN4 to decide which of 240/480/960 kHz SCS are supported for initial access of such band.

- RAN1 prioritizes time-domain multiplex of SSB and CORESET0 to minimize the number of needed synchronization raster entries.

- Supporting 480 kHz SCS and 960 kHz SCS for SSB are UE capabilities:

  - UE is not expected to support 480 kHz SCS for SSB if it doesn’t support 480 kHz SCS for data/control channels.

  - UE is not expected to support 960 kHz SCS for SSB if it doesn’t support 960 kHz SCS for data/control channels.

- Send an LS to RAN2 and RAN4.

# SS/PBCH block pattern for new SCSs

In Rel-15 and Rel-16, SS/PBCH block is only supported for 15 kHz and 30 kHz for FR1, and 120 kHz and 240 kHz for FR2, as shown in Figure 3. For 52.6 to 71 GHz, the SS/PBCH block pattern needs to be defined for the new SCSs. Scaling from existing SS/PBCH block pattern can be considered as the baseline, but design aspects including the switching time between neighboring SS/PBCH block beams shall be taken into account. For example, in Rel-15, the beam sweeping time of neighboring SS/PBCH blocks can be accommodated within the CP length of 240 kHz SCS. However, if a larger SCS is used for the new carrier frequency range 52.6 to 71 GHz (e.g. 480 or 960 kHz SCS), the beam switching duration of the SS/PBCH blocks cannot be restricted within the CP length. This restriction may not be achievable in implementation, so at least one symbol should be guaranteed in time domain between the neighboring SS/PBCH blocks in a slot.

Figure 3 Illustration of supported SS/PBCH block pattern in Rel-15 and Rel-16.

Meanwhile, as described in the TR, multiplexing SS/PBCH block with CORESET and HARQ transmission should be taken into account when designing the SS/PBCH block pattern for the new SCSs, and one design aspect to consider is, which SCS should be used as the reference SCS for CORESET and HARQ to determine the SS/PBCH block multiplexing pattern. As analyzed in the previous sections, same numerology implementation is the baseline to support, hence, using same SCS for CORESET and HARQ to determine the SS/PBCH block multiplexing pattern should be considered as baseline as well.

Based on the above requirement, existing SS/PBCH block pattern (e.g. Case A or Case C) can already support at least one symbol gap between neighboring SS/PBCH blocks, and also enables multiplexing of CORESET and HARQ with same SCS. Hence, we propose to reuse such a pattern for SS/PBCH block with SCS of 480 kHz and 960 kHz.

Proposal 4: Support the same SS/PBCH block pattern for 480 kHz and 960 kHz SCSs.

- At least one symbol should be reserved between neighboring SS/PBCH block for beam sweeping delay.

- Symbols should be reserved for CORESET and HARQ with SCS same as the SS/PBCH block.

- For SS/PBCH block candidate locations in a slot, Case A or Case C can be reused.

# CORESET#0 configuration

## CORESET#0 configuration enhancement for 120 kHz SCS

The design of CORESET#0 configuration highly relies on the type of channelization and synchronization raster interval, and in NR Rel-15 FR2, CORESET#0 configuration was designed based on the Type 1 channelization with a synchronization raster interval of 12 RB for 120 kHz SCS of SS/PBCH block (i.e., 17.28 MHz) and 12 RB for 240 kHz SCS of SS/PBCH block (i.e., 34.56 MHz).

If Type 1 channelization is reused for NR 52.6 GHz to 71 GHz, and the minimum channel bandwidth is enlarged from FR2 (e.g. 50 MHz), the synchronization raster interval can be larger than Rel-15 FR2, even using 120 kHz SCS of SS/PBCH block. Hence, the CORESET#0 configuration, especially the frequency offset between SS/PBCH block and CORESET#0 in multiplexing Pattern 1, should be enhanced accordingly. For example, the minimum number of RB offsets for CORESET#0 bandwidth as 48 RB is increased when the minimum channel bandwidth increased to be 100 MHz, which means for some channelization, using Rel-15 FR2 CORESET#0 configuration table may not be able allocate a CORESET#0 within the minimum channel bandwidth. Hence, enhancement to the RB offset is needed.

Also, NR Rel-15 FR2 only supports at most 100 MHz CORESET#0 bandwidth (i.e., 48 RBs for 120 kHz SCS), and if this bandwidth can be enlarged for NR 52.6 to 71 GHz, e.g. to support 480 kHz or 960 kHz SCS for SS/PBCH block in initial access, 96 RB CORESET#0 can be supported to increase the coverage of PDCCH in CORESET#0.

Lastly, the need for supporting {SSB SCS, CORESET#0 SCS} other than {120 kHz, 120 kHz} is not strong. Note that the SCS of initial BWP can be reconfigured in RMSI, so the benefit of supporting CORESET#0 SCS other than 120 kHz and indicated in MIB is only beneficial for implementing PDCCH and PDSCH of RMSI, which may not be significantly needed.

Based on above discussion, we have the following proposal for CORESET#0 configuration for 120 kHz SCS:

Proposal 5: For SS/PBCH block with 120 kHz SCS,

- only support CORESET#0 SCS as 120 kHz;

- additional CORESET#0 RB offsets are needed;

- support 96 RB as the number of RBs for CORESET#0.

## CORESET#0 configuration for 480 kHz and 960 kHz SCS

For 480 kHz and 960 kHz SCS, the intention is to facilitate same numerology implementation of SS/PBCH block and other signals and channels, hence, at least same numerology between SS/PBCH block and CORESET#0 should be supported, and other combination may not be essentially needed. The table for 120 kHz SCS can be used as a baseline to design the configuration tables for 480 kHz and 960 kHz, with the understanding to support at least the same SS/PBCH block and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, and number of symbols as in 120 kHz SCS. The design of RB offset may need further progress from RAN4 on the channelization and synchronization raster.

Proposal 6: For SS/PBCH block with 480 kHz SCS and 960 kHz,

- only support CORESET#0 SCS same as SS/PBCH block SCS;

- support at least the same SS/PBCH block and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, and number of symbols as in 120 kHz SCS;

- support 96 RB as the number of RBs for CORESET#0;

- Further study the RB offset based on RAN4 design of channel and synchronization rasters.

# Discovery burst transmission window

In the study item, “short control signal” has been supported without applying channel access procedure to initialize a channel occupancy, wherein the content and condition of using “short control signal” is to be further investigated in the work item. The intention of supporting “short control signal” is analog to “discovery burst” in Rel-16 NR-U, and at least all the components supported in Rel-16 NR-U, e.g. SS/PBCH block, PDCCH/PDSCH of RMSI, and CSI-RS, should be included as part of “short control signal”. Moreover, we also note that there are cases “short control signal” cannot be applicable for SS/PBCH block transmission. For one example, for region where the regulation does not support “short control signal”. For another example, the condition for using “short control signal” cannot be met (e.g. duty cycle or duration limitation). For these cases, LBT is still needed to initialize the SS/PBCH block transmission, and there is still chance for channel access failure due to congestion. Hence, discovery burst transmission window (DBTW) is still needed to mitigate the impact from LBT, and since it is already a feature supported in Rel-16 NR-U, there is no need to remove it for 60 GHz unlicensed band.

In the last RAN1 meetings, the feasibility of supporting DBTW and the indication method were discussed, and the following aspects need to be further clarified and studied for supporting DBTW in 60 GHz unlicensed band.

- The indication of QCL parameter for SS/PBCH blocks (i.e., Q) can still follow the same principle as Rel-16 NR-U: best effort to indicate Q by MIB, and if not possible due to the limitation of payload size, to indicate Q in SIB1.

- The indication of enabling and disabling of the DBTW (or LBT mode) can be joint coded with the indication of Q. For example, one codepoint (e.g. a non-numerical value) in the indication of Q can be utilized for disabling the DBTW (or non-LBT mode), and the other codepoints (e.g. numerical values) in the indication of Q are used for enabling the DBTW (or LBT mode) and further deriving the QCL parameter value for SS/PBCH blocks.

- Based on current structure of PBCH payload, there are 8 physical layer bits not contained in MIB. By noting the TTI of PBCH is 80 ms, the indication of timing information for a particular half frame will cost at least log⁡(80/5)=4 bits, and there can be at most 4 physical layer bits available for indicating timing within a half frame. Combining the at most 3 bits timing information by DM-RS, there can be up to 128 SS/PBCH block candidate locations within a half frame for indication, regardless of the SCS of SS/PBCH blocks.

- For the carrier frequency of 60 GHz unlicensed band, part of the band bandwidth could be potentially overlap with a licensed band in a particular region, and mechanism to distinguish the licensed and unlicensed bands needs to be considered. For initial access case, this distinguish can be achieved by using different synchronization raster entries (i.e., different GSCN values), and for non-initial access case, there can be an explicit indication of licensed or unlicensed operation of a cell, when the cell is configured.

Proposal 7: Support discovery burst transmission window for 60 GHz unlicensed band.

- The indication of Q can be in MIB for a best effort, and if not possible, in SIB1;

- The indication of DBTW disabling can be joint coded with the indication of Q;

- Support more than 64 candidate SS/PBCH block locations within a half frame;

  - Current PBCH payload can support timing indication of up to 128 candidate SS/PBCH block candidate locations;

  - For example, for 120 kHz SCS, support 80 candidate SS/PBCH block locations within a half frame;

- For initial access, different synchronization raster entries are applied for licensed and unlicensed operations; for non-initial access, support an explicit indication of licensed or licensed operation when configuring a cell.

# RACH related issues

## PRACH format for new SCSs

In Rel-16 NR-U, the longer sequence lengths for PRACH format (e.g. 571 and 1151) have been introduced, mainly for the purpose of OCB requirement. For the 60 GHz unlicensed band, since the OCB requirement may not be an issue for PRACH transmission, there is no other strong needs to introduce extra new sequence length.

Then, for the time-domain formats, since the coverage of the use cases for 52.6 to 17 GHz is not extensive, e.g., which considers the indoor hotspot cases, even with higher SCS, the CP length might still be enough for the delay budget. Meanwhile, supporting same numerology for all UL signals and channels is beneficial for implementation, so the new SCSs (480 kHz and 960 kHz) should be applicable to PRACH format as well. Besides, there is no point to differentiate non-initial access and initial access for RACH SCS.

- From design history point of view, the random access procedure is always same for the so called initial access (assuming for connection purpose) and non-initial access;

- From signalling point of view, the RACH SCS is separately configured by gNB, it is not tied to the SSB or other signals;

- From implementation point of view, the random access channel is specially designed and filtered. Intentionally try to allow different SCS for RACH by separating the so called initial access and non-initial access will increase the implementation burden, e.g., gNB needs to consider two different SCS reception for one operation.

- The effort of including the trigger events for RACH seems no benefits, they will be many and they will be more to come, e.g., SDT transmission, consider positioning purpose, etc.

Thus, we support the larger SCS for PRACH in a general way.

Proposal 8: Support short PRACH format for all PRACH sequence lengths LRA∈139, 571, 1151 and all SCSs μ∈3, 5, 6, and don’t support long PRACH format.

## RO configuration for new SCSs

For RO configuration in case of new (larger) SCSs are adopted, the following several aspects need to be considered:

- RACH capacity provided by the network. With larger SCS, the conventional RACH frame (10ms) can have much more slots compare to before, e.g., if SCS = 960 kHz, then in one RACH frame, there will be 8 times more slots than those for SCS = 120 kHz, i.e., 640 potential RACH slots.

- The RO pattern designed for NR Rel-15/16 are listed in the configuration table and each row is being carefully checked and tons of efforts have been spent on it. So it’s not easy or not necessary to redesign the table for larger SCS cases, we can make use of the current table for 120 kHz SCS for the extension.

Proposal 9: Using the RO pattern for SCS = 120 kHz derived from the PRACH configuration table as the reference for larger SCS cases.

When we consider using the RO pattern for SCS = 120 kHz derived from the PRACH configuration table as the reference, there could be two directions:

Direction 1: keep the RACH frame = 10 ms (i.e., the RACH configuration period is at least 10 ms), then design the RO patterns for the 8*80 slots, with 80 slots RO pattern derived by a PRACH configuration index based on 120 kHz, the additional indication on which one(s) of the 8 eighty-slots needs to be configured with RACH resource, by bitmap indication or by some table-searching based methods. One alternative is that one 120 kHz RO could be counted as eight 960 kHz ROs, so the providing the indication which ones of the eight 960 kHz ROs within the 120 kHz RO is also a possible way to determine the RO pattern.

Direction 2: if we consider the RO patterns for eighty-slot are large enough for the RACH capacity, then another direction is to redesign the RACH duration, where the RACH duration is the time duration where the RACH resource occupied in one RACH period, e.g., duration containing 480/960 kHz PRACH slot pattern within 10 ms. For example, when SCS = 960 kHz, the 80slots now occupies 1.25 ms, while RAN1 agrees the RACH periodicity still needs to at least 10 ms. Then the RACH duration location within the 10 ms needs to be determined. Then UE could finally determine the RO pattern.

Proposal 10: For RO configuration, both direction 1 (indication on which one(s) of the 8 eighty-slots or which one(s) of the eight 960 kHz ROs within a 120 kHz RO) and direction 2 (keep 80 slots in total but redesign the RACH period and RACH duration location) can be considered.

## Non-consecutive RO for unlicensed operation

With this larger SCS, the symbol duration is much shorter than NR Rel-15/16. Given that if LBT is still required before each PRACH transmission, the consecutive allocation of ROs within a PRACH slot could lead to the scenario where LBT for a RO is failed due to a PRACH transmission utilizing the previous RO. In Rel-15/16 case, the RO length is relative long compared to the one with larger SCS, as shown in following figure, the length of RO with 120 kHz is 8 times of that for RO with 960 kHz. In 120 kHz case, if a UE selects the RO 1, it needs to do LBT whose period will overlap with RO 0, so whether RO 0 has been selected by others will be important to the LBT results. The same happened to the case if UE selects RO 2, the LBT period will overlap with RO 1 and LBT results will be impacted by the usage of RO 1 as well.

This issue will be even worse for 60 GHz unlicensed band wherein higher SCS is used. As shown in Figure 4, the length of RO for 960 kHz SCS will be much shorter than 120 kHz SCS, and the LBT period could overlap with multiple ROs. Thus, the LBT results of the RO 2 will depend on both RO 0 and RO 1 as shown in the figure. Any of these ROs being selected by other UEs will make the RO 2 unavailable for the UE to use.

Figure 4 Illustration of impact of consecutive ROs for LBT.

Thus, a LBT gap is needed before a RO. The gap duration between neighboring ROs can be achieved by configuring UEs to only utilize the RACH occasions with odd indexes 2n+1 (e.g., the 1-st, 3-rd, 5-th) within the RACH slot for PRACH transmissions, while the RACH occasions with even indexes 2n (e.g., the 0-th, 2-nd, 4-th) can be used for the LBT operation of UEs that will utilize RO with index 2n+1. Moreover, this method could be extended to using every nth RO for RACH, and the not-used RO could be leave to LBT gap. This approach can directly use the existing PRACH configuration table from Rel-15/16 and thus requires less modification from current spec.

Figure 5 An example of creating LBT gap through even/odd RO indexes.

Proposal 11: Support non-consecutive RO configuration to alleviate the RACH LBT failure.

# Conclusion

The observations and proposals made in this contribution are summarized below:

Observation 1: Using dedicated signalling to provide the CORESET#0/Type0-PDCCH configuration cannot resolve the inter-operator PCI confusion.

Proposal 1: Support ANR and inter-operator PCI confusion resolution for all supported SS/PBCH block subcarrier spacings, and the CORESET#0/Type0-PDCCH configuration is provided by the MIB of the SS/PBCH block.

Observation 2: Supporting 240 kHz SCS SS/PBCH block for initial access has the largest complexity.

Proposal 2: Support 480 kHz and 960 kHz SCS for SS/PBCH block in initial access case.

Proposal 3:

- Support SSB with 240/480/960 kHz for initial and non-initial access with support of CORESET0/Type0-PDCCH configuration in the MIB.

  - SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

  - only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS

- It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries in the 57 – 71 GHz band no larger than [400] (Note: the total number of synchronization raster entries in FR2 for band n259 is 344). If the assumption cannot be satisfied, it’s up to RAN4 to decide which of 240/480/960 kHz SCS are supported for initial access of such band.

- RAN1 prioritizes time-domain multiplex of SSB and CORESET0 to minimize the number of needed synchronization raster entries.

- Supporting 480 kHz SCS and 960 kHz SCS for SSB are UE capabilities:

  - UE is not expected to support 480 kHz SCS for SSB if it doesn’t support 480 kHz SCS for data/control channels.

  - UE is not expected to support 960 kHz SCS for SSB if it doesn’t support 960 kHz SCS for data/control channels.

- Send an LS to RAN2 and RAN4.

Proposal 4: Support the same SS/PBCH block pattern for 480 kHz and 960 kHz SCSs.

- At least one symbol should be reserved between neighboring SS/PBCH block for beam sweeping delay.

- Symbols should be reserved for CORESET and HARQ with SCS same as the SS/PBCH block.

- For SS/PBCH block candidate locations in a slot, Case A or Case C can be reused.

Proposal 5: For SS/PBCH block with 120 kHz SCS,

- only support CORESET#0 SCS as 120 kHz;

- additional CORESET#0 RB offsets are needed;

- support 96 RB as the number of RBs for CORESET#0.

Proposal 6: For SS/PBCH block with 480 kHz SCS and 960 kHz,

- only support CORESET#0 SCS same as SS/PBCH block SCS;

- support at least the same SS/PBCH block and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, and number of symbols as in 120 kHz SCS;

- support 96 RB as the number of RBs for CORESET#0;

- Further study the RB offset based on RAN4 design of channel and synchronization rasters.

Proposal 7: Support discovery burst transmission window for 60 GHz unlicensed band.

- The indication of Q can be in MIB for a best effort, and if not possible, in SIB1;

- The indication of DBTW disabling can be joint coded with the indication of Q;

- Support more than 64 candidate SS/PBCH block locations within a half frame;

  - Current PBCH payload can support timing indication of up to 128 candidate SS/PBCH block candidate locations;

  - For example, for 120 kHz SCS, support 80 candidate SS/PBCH block locations within a half frame;

- For initial access, different synchronization raster entries are applied for licensed and unlicensed operations; for non-initial access, support an explicit indication of licensed or licensed operation when configuring a cell.

Proposal 8: Support short PRACH format for all PRACH sequence lengths LRA∈139, 571, 1151 and all SCSs μ∈3, 5, 6, and don’t support long PRACH format.

Proposal 9: Using the RO pattern for SCS = 120 kHz derived from the PRACH configuration table as the reference for larger SCS cases.

Proposal 10: For RO configuration, both direction 1 (indication on which one(s) of the 8 eighty-slots or which one(s) of the eight 960 kHz ROs within a 120 kHz RO) and direction 2 (keep 80 slots in total but redesign the RACH period and RACH duration location) can be considered.

Proposal 11: Support non-consecutive RO configuration to alleviate the RACH LBT failure.

# Reference

[1] Chairman notes, 3GPP RAN1#104b-e, April, 2021.