3GPP TSG RAN WG1 #105	                                                                 R1-2105305

e-Meeting, May 10th – 27th, 2021

Agenda Item:		8.3.3

Source:		Samsung

Title:		Uplink intra-UE multiplexing and prioritization

Document for:		Discussion and decision

# Introduction

In RAN1#104e-bis, the following were agreed regarding intra-UE multiplexing and prioritization.

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

This contribution further discusses issues related to intra-UE multiplexing and prioritization of traffic with different priorities.

# Discussion

## Uplink intra-UE multiplexing

Multiplexing eMBB HARQ-ACK with URLLC UCI and/or data should ensure reliability and latency targets of URLLC traffic. Even if a UE indicates support of multiplexing of UCI/data with different priority values, whether the UE actually does so or prioritizes transmission of PUCCH/PUSCH with larger priority value should be under the control of the network. This can also simplify specification/implementation support for multiplexing as it can leave the decision to the gNB for each scenario and avoid considerations of various conditions, such as whether or not HARQ-ACK with priority 1 is multiplexed in a PUSCH or PUCCH that is within a sub-slot of a PUCCH that the UE would otherwise transmit to provide the HARQ-ACK. The Rel-16 procedures for multiplexing a UCI type can also remain applicable.

If multiplexing of PUCCHs with different priorities is enabled by the gNB, the overlapping PUCCHs are treated as overlapping PUCCHs with same priority in Rel-16 (i.e. priority does not matter) and multiplexing can follow Rel-16 timeline conditions assuming same priority among overlapping PUCCHs. If there is any concern for latency regarding the UCI in the HP PUCCH, the gNB may not enable such multiplexing. The gNB can separately configure whether or not multiplexing is enabled for UCI from LP PUCCHs to an HP PUCCH or for UCI from HP PUCCHs to an LP PUCCH (subject to timeline conditions).

Proposal 1: Support multiplexing UCI of different priorities subject to timeline conditions and RRC configuration and/or dynamic indication from gNB.

Supporting UE multiplexing of data/UCI with different priority values may require a UE to multiplex up to 5 UCI types on a same PUCCH and up to 3 UCI types on a PUSCH (“type” is identified by information content and priority) and, due to increased payload, may always require use of a large number of RBs for PUCCH transmission or of a large number of PUSCH REs. A large UCI payload can compromise overall reliability due to a requirement of larger transmission power and a larger number of RBs to achieve a target code rate. A large number of UCI REs can be controlled by a small value of the scaling factor α but that can also lead to limitations when the payload is only due to, e.g., eMBB HARQ-ACK for multiple cells and multiple PDSCH receptions with or without CBG-based HARQ-ACK feedback. Then, in some cases, the RE limitation may result to not multiplexing (or multiplexing) CSI while in other cases it may result to not multiplexing (or multiplexing) HARQ-ACK and corresponding consequences are different. Therefore, in addition to enabling/disabling intra-UE multiplexing, a network should be able to control the UCI types of a first priority that can be multiplexed on a PUCCH/PUSCH of a second priority.

Proposal 2: The UCI types with first priority that can be multiplexed on a PUCCH/PUSCH of a second priority are configurable by the network.

When UCIs with different priorities are multiplexed on a same PUCCH/PUSCH, whether to use separate coding or joint coding was discussed in RAN1#104-e-bis. It has been agreed to use separate coding for multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUCCH in Rel-17 when the total number of LP and HP HARQ-ACK bits is more than 2 with an FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

Multiplexing of 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK in a PUCCH has been discussed for several meetings. The probability of this case to happen is very small and it can be even smaller than the BER of LP HARQ-ACK, simply dropping the LP HARQ-ACK is preferred and gNB can schedule a retransmission for the corresponding PDSCH. There is no motivation to further optimize this case. For the other cases in the FFS, the motivation of using a different solution other than separate coding is not clear.

Proposal 3: Reuse Rel-16 for the case where a PUCCH with 1 bit LP HARQ-ACK overlaps with a PUCCH with 1 bit HP HARQ-ACK in the time domain.

In Rel-15, separate coding is used for multiplexing for Part 2 CSI in the PUCCH. When the payload of Part 2 CSI is less than 3 bits, zeros are appended to the information bits until the payload equals 3. Similar rules should apply if the payload of HP HARQ-ACK or LP HARQ-ACK is less than 3 bits when multiplexing HP HARQ-ACK and LP HARQ-ACK in a PUCCH.

Proposal 4: Zeros should be appended to the LP/HP HARQ-ACK information bits if the payload of LP/HP HARQ-ACK is less than 3 bits when multiplexing HP HARQ-ACK and LP HARQ-ACK in a PUCCH.

PRB determination and RE mapping should be addressed for multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH. One solution is to reuse Rel-15 rules for Part 2 CSI. However, the reliability of HP HARQ-ACK can be affected by the LP HARQ-ACK for Type-2 HARQ-ACK codebook the size of which is determined by the DAI values and a miss detection of a ‘last’ DCI format can lead to UE and gNB have different understanding of the size of HARQ-ACK codebook (e.g. in case of single-cell operation). Another solution is to perform PRB determination and RE mapping separately for HP HARQ-ACK and LP HARQ-ACK. The PRBs for HP HARQ-ACK and LP HARQ-ACK are concatenated in a PUCCH. The decoding of HP HARQ-ACK will not be affected by LP HARQ-ACK with this solution and thus to ensure the reliability of HP HARQ-ACK. As shown in Figure 1, an example is given to illustrate these two options.

Figure 1

Proposal 5: For PRB determination and RE mapping when multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH, consider following two options.

Option 1) Reuse Rel-15 rules of separate coding in a PUCCH.

Option 2) PRB determination and RE mapping are performed separately for HP HARQ-ACK and LP HARQ-ACK for PF2/3.

FFS: How to ensure the reliability of HP HARQ-ACK.

An important parameter for determining the PRB and RE is the maxCodeRate when multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH format. The maxCodeRate of HP HARQ-ACK can reuse the maxCodeRate configured in the second PUCCH-Config while the maxCodeRate of LP HARQ-ACK can reuse the maxCodeRate configured in the first PUCCH-Config of the same PUCCH format. Alternatively, RRC can configure a separate maxCodeRate for LP HARQ-ACK per PUCCH format in the second PUCCH-Config.

Proposal 6: When multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH format,

- Reuse the maxCodeRate configured in the second PUCCH-Config for HP HARQ-ACK.

- Consider following two options to determine the maxCodeRate for LP HARQ-ACK.

  - Option 1) Reuse the maxCodeRate configured in the first PUCCH-Config of the same PUCCH format.

  - Option 2) RRC configures a separate maxCodeRate for LP HARQ-ACK per PUCCH format in the second PUCCH-Config.

Another issue for multiplexing LP HARQ-ACK and HP HARQ-ACK on a same PUCCH is the supported scenario. There are four cases listed below.

Case a) Multiplexing of LP Type-1 HARQ-ACK codebook and HP Type-1 HARQ-ACK codebook

Case b) Multiplexing of LP Type-2 HARQ-ACK codebook and HP Type-2 HARQ-ACK codebook

Case c) Multiplexing of HP Type-1 HARQ-ACK codebook and LP Type-2 HARQ-ACK codebook

Case d) Multiplexing of HP Type-2 HARQ-ACK codebook and LP Type-1 HARQ-ACK codebook

At least multiplexing of HARQ-ACK codebooks with the same type (Case a) and Case b)) should be supported. As mentioned before, for Type-2 HARQ-ACK codebook, a DCI format miss detection can lead to UE and gNB having different understanding of the size of the HARQ-ACK codebook. How to ensure the reliability of HP UCI and/or data when multiplexed with LP HARQ-ACK needs further discussion at least for Cases b), c) and d). Whether the above cases are enabled can be configured by RRC. If it is not enabled, Rel-16 behavior can apply.

Observation 1: Multiplexing of LP HARQ-ACK codebook and HP HARQ-ACK codebook with same and/or different HARQ-ACK codebook types can be enabled via the configuration for HP/LP multiplexing.

Proposal 7: Consider solutions to ensure the reliability of multiplexing of LP Type-2 HARQ-ACK codebook and HP HARQ-ACK codebook and/or HP data.

In RAN1#104e, multiplexing a low-priority HARQ-ACK PF0/1 and a high-priority SR into a PUCCH for some HARQ-ACK/SR PUCCH format combinations were discussed and quite a few options are given.

Further, more than two SR PUCCHs overlapping with a LP HARQ-ACK were also discussed in RAN1#104e. That situation also existed in Rel-15 where the UE selects one positive SR to indicate among the overlapping SR configurations. If multiplexing of more than one positive SRs in a same PUCCH is supported in Rel-17, possible solutions should consider how to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK.

Another case is a LP HARQ-ACK PUCCH PF0/1 overlapping with both a HP SR PF0 and a HP SR PF1 and the two SR PUCCHs are not overlapping with each other. As shown in Figure 2, in such case dropping HR SR should be avoided.

Figure 2

Optimization for these cases may require a lot of standard efforts with marginal gain. For example, if LP HARQ-ACK (with PF1) is to be multiplexed in the HP SR resource with PF0, there is anyway no reason for the gNB to configure PF1 for the LP HARQ-ACK as such overlapping would always occur and PF0 will end up being used (assuming that the reason for using PF0 for SR is the short periodicity). If the HP SR (with PF0) is to be multiplexed with the LP HARQ-ACK using PF1, there is no reason to have PF0 for the SR. Therefore, it is preferable to maintain Rel-16 operation and drop LP HARQ-ACK PF 1. Droping LP HARQ-ACK only results in up to 2 PDSCH retransmissions in the worst case (and can be avoided by the gNB by configuring PF0 for 1-2 bits of LP HARQ-ACK if the SR needs to use PF0 – coverage is anyway same). Further, if LP HARQ-ACK PUCCH overlaps with both HP SR PUCCH and HP HARQ-ACK PUCCH, all UCI can be multiplexed on a HP HARQ-ACK PUCCH resource.

Proposal 8: Drop LP HARQ-ACK PUCCH when a LP HARQ-ACK PF0/1 overlaps with a HP SR PUCCH.

For more than 2 bits LP HARQ-ACK, multiplexing of LP HARQ-ACK and HP SR can be supported if the reliability and latency of HP SR can be satisfied. The baseline mechanism is reusing Rel-15 assuming same priority of HARQ-ACK and SR. For the cases where the reliability and/or latency of SR cannot be satisfied, dropping LP HARQ-ACK or simple solutions for multiplexing can be considered.

In Rel-15, when HARQ-ACK is multiplexed on PUCCH format 2/3/4, overlapped SRs are multiplexed with HARQ-ACK on a HARQ-ACK PUCCH resource. In Rel-17, reusing the Rel-15 mechanism can have reliability and latency issues. For PUCCH format 2, the latency might not be an issue. How to ensure the reliability should be further studied if joint coding is used, e.g., using a lower maxCodeRate. For PUCCH format 3/4, whether the latency is satisfied can depend on the number of symbols of the PUCCH or whether the LP HARQ-ACK is within the HP sub-slot of the overlapping HP SR PUCCH and the reliability issue can be solved similar as PUCCH format 2.

Proposal 9: Support multiplexing of LP HARQ-ACK and HP SR when HARQ-ACK is transmitted on PUCCH format 2/3/4

- Use Rel-15 mechanism as a baseline assuming HARQ-ACK and SR have the same priority.

- FFS: how to ensure the latency and reliability of HP SR.

It has been agreed to support multiplexing of LP HARQ-ACK, HP HARQ-ACK and HP SR on a PUCCH. In RAN1#104e, multiplexing of more than two overlapping PUCCHs with different priorities was further discussed. The time unit should be first clarified when solving the collision of more than two overlapping PUCCHs with different priorities.

In Rel-15, the time unit for solving the collision of multiple PUCCHs is a slot. All the PUCCHs within a single slot is put into a set Q and then PUCCH multiplexing is performed according to the pseudo and order function.

TS 38.213 9.2.5 UE procedure for reporting multiple UCI types … Set to the set of resources for transmission of corresponding PUCCHs in a single slot without repetitions where - a resource with earlier first symbol is placed before a resource with later first symbol - for two resources with same first symbol, the resource with longer duration is placed before the resource with shorter duration - for two resources with same first symbol and same duration, the placement is arbitrary - the above three steps for the set are according to a subsequent pseudo-code for a function - a resource for negative SR transmission that does not overlap with a resource for HARQ-ACK or CSI transmission is excluded from set

In Rel-16, sub-slot was introduced to reduce the latency of URLLC HARQ-ACK feedback and sub-slot length is configured per PUCCH-Config. If UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH as described in TS 38.213.  In Rel-16, PUCCH multiplexing is supported with the same L1 priority and Rel-15 rules are reused with the PUCCH multiplexing time unit replaced by subslotLengthForPUCCH, if provided.

TS 38.213 9 UE procedure for reporting control information … In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH.

In Rel-17, it has been agreed to support multiplexing of HP HARQ-ACK/SR and LP HARQ-ACK on a PUCCH, the time unit for solving the collision of UCI with different L1 priorities needs to be clarified first. The candidates can be a slot (14 symbols for NCP and 12 symbols for ECP), a HP PUCCH time unit, or a LP PUCCH time unit.

In RAN1#103e, it has been agreed to support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met. A typical configuration is that the HP PUCCH time unit is shorter than the LP PUCCH time unit. There can be three cases need to be considered.

Case 1) Each HP sub-slot contains zero or more HP SR PUCCH resource.

Case 2) Each HP sub-slot contains zero or one HP HARQ-ACK PUCCH resource.

Case 3) Each HP sub-slot contains zero or more HP SR PUCCH resource and zero or one HP HARQ-ACK PUCCH resource.

Observation 2: For determination of the PUCCH time unit for handling PUCCH collisions with different PUCCH time units (i.e. slot and sub-slot PUCCH configuration) of different priorities, when A LP HARQ-ACK PUCCH overlaps with more than one HP sub-slot, at least following cases should be considered.

- Case 1) Each HP sub-slot contains zero or more HP SR PUCCH resource.

- Case 2) Each HP sub-slot contains zero or one HP HARQ-ACK PUCCH resource.

- Case 3) Each HP sub-slot contains zero or more HP SR PUCCH resource and zero or one HP HARQ-ACK PUCCH resource.

- Note: Multiplexing of PUCCH with different priorities is performed within a PUCCH time unit (i.e. either within a slot or a sub-slot). E.g. for each PUCCH time unit, put all the associated PUCCHs (e.g. for a same priority, associated PUCCHs are the PUCCHs within the PUCCH time unit) in the corresponding set Q and then reuse Rel-15/Rel-16 rules.

- Aiming to reuse Rel-15/16 pseudo code for PUCCH multiplexing with limited optimization.

It has been agreed to use HP PUCCH resource for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH. The same rule can apply to the case of multiplexing a high-priority (HP) HARQ-ACK, a low-priority (LP) HARQ-ACK and HP SR(s) into a PUCCH. For case 2) and 3) a LP HARQ-ACK PUCCH can overlap with multiple HP HARQ-ACK PUCCHs, to ensure the latency of HP HARQ-ACK, a preferred solution is multiplexing the LP HARQ-ACK PUCCH and the first overlapped HP HARQ-ACK PUCCH and the result PUCCH should be a HP HARQ-ACK PUCCH. The other overlapped HP HARQ-ACK PUCCHs can be transmitted as well. To adopt this solution, the time unit for PUCCH multiplexing should be the HP PUCCH time unit. If a LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units, the LP HARQ-ACK PUCCH should be put into one of Q sets of the overlapped multiple HP PUCCH time units. Then, Rel-15 rules can be applied.

Proposal 10: The time unit for solving the collision of PUCCHs with different L1 priority indexes should be the HP PUCCH time unit.

- If a LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units, determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH.

  - FFS details.

Another issue for solving the collision of LP HARQ-ACK PUCCH, HP HARQ-ACK PUCCH and HP SR PUCCH is the multiplexing order. At least there can be the following options.

Option 1) All PUCCHs are viewed with same priority – Rel-15 multiplexing applies.

Option 2) First, multiplex overlapping HP HARQ-ACK PUCCH and LP HARQ-ACK, then multiplex resulting PUCCH and SR PUCCH (if there is overlapping)

For the above options, Rel-15 UCI multiplexing rules can be reused and potential enhancements can be considered. For example, considering the case in Figure 3.

Figure 3

Assume the timeline for multiplexing is satisfied. If Rel-15 UCI multiplexing rules are reused, the orders of the PUCCHs in a set Q are shown in the left figure. HP SR and LP HARQ-ACK will be multiplexed/prioritized first, however, LP HARQ-ACK will be dropped if the latency and/or reliability of HP SR cannot be satisfied. For example, in this case, the latency of HP SR cannot be satisfied if HP SR is multiplexed on the LP HARQ-ACK PUCCH and the LP HARQ-ACK PUCCH should be dropped. An enhanced order function can be considered to increase the transmission opportunity of LP HARQ-ACK as shown in the right figure. For example, HP/LP HARQ-ACK can be placed before HP SR in the set Q. HP HARQ-ACK can be placed before LP HARQ-ACK. With this order, LP HARQ-ACK and HP HARQ-ACK will be multiplexed first on a HP HARQ-ACK PUCCH resource and then HP SR will be multiplexed if it overlaps with the result PUCCH.

Proposal 11: Down select from the following options for multiplexing/prioritizing LP HARQ-ACK PUCCH, HP HARQ-ACK PUCCH and HP SR PUCCH on a same PUCCH. FFS potential enhancements.

- Option 1) All PUCCHs are viewed with same priority – Rel-15 multiplexing applies.

- Option 2) First, multiplex overlapping HP HARQ-ACK PUCCH and LP HARQ-ACK, then multiplex resulting PUCCH and SR PUCCH (if there is overlapping)

Next we discuss intra-UE multiplexing when there is PUSCH. In RAN1#104e-bis, it was agreed to support separate coding for the two HARQ-ACKs for multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUSCH in Rel-17. Separate coding can help protect HP HARQ-ACK from impacted by size mismatch of LP HARQ-ACK codebook. However, the impact on the reliability of HP data still exists. This issue needs to be addressed if Type-2 HARQ-ACK codebook is configured for LP, due to different reliability levels for the LP and HP HARQ-ACK payload size which may make the LP HARQ-ACK payload size mismatch as the bottleneck for the reliability of HP PUSCH. Solutions such as LP UL DAI, reservation of a fixed number of REs for LP HARQ-ACK can be considered.

Proposal 12: Consider solutions to ensure the reliability of data in a HP PUSCH if a LP Type-2 HARQ-ACK codebook is multiplexed in the HP PUSCH.

In Rel-16, multiplexing of overlapping PUCCH(s) and/or PUSCH(s) is performed with the same priority index. This can be used as a starting point for Rel-17 intra-UE multiplexing. After resolving overlapping PUCCH(s) and/or PUSCH(s) with the same priority index, overlapping PUCCH(s) and/or PUSCH(s) with different priority indexes can be resolved.

Proposal 13: Determine order for resolving overlapping among HP/LP PUCCHs and HP/LP PUSCHs.

When a PUCCH overlaps with multiple PUSCHs, how to select one from the multiple PUSCHs to multiplex UCI from an overlapped PUCCH was specified in Rel-15. In Rel-17 the overlapping can be among channels with different priorities and the PUSCH selection can be reconsidered as it can have different outcomes.

In RAN1#102e, it was agreed to support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. From a RAN1 perspective, there is no distinction between inter-band and intra-band CA and there is no apparent reason for a UE capable to transmit PUSCH on the PCell and an SCell to not be capable to transmit PUCCH on the PCell and PUSCH on the SCell. If any restriction is needed, that can be addressed by RAN4. To maximize the benefits of simultaneous PUCCH and PUSCH transmissions, that feature should be supported regardless of CA, i.e. it should be supported on the same (primary) cell. It is noted that such support has been specified in LTE. The conditions to actually utilize that feature in NR are substantially improved compared to LTE because (a) the waveform is OFDM, not DFT-S-OFDM, (b) the PUCCH transmission need not be at the edge of the cell bandwidth (even the active UL BWP need not be at the edge of the cell bandwidth), and (c) the PUCCH need not be over one RB (resulting to maximum PSD). An LS to RAN4 would be beneficial to determine MPR requirements for a few cases.

Observation 3: In RAN1 specifications, there needs to be no differentiation between intra-band CA and inter-band CA for simultaneous PUSCH and PUCCH transmissions from a UE.

Proposal 14: Send an LS to RAN4 to inquire about the feasibility/MPR for simultaneous PUCCH and PUSCH transmissions on a same cell.

Further, it should be noted that support of simultaneous PUSCH and PUCCH transmissions has nothing to do with whether they are LP or HP. There is no need to change Rel-16 multiplexing procedures by considering only LP vs. HP overlapping. Basically, when a UE supports and is configured for simultaneous PUSCH and PUCCH transmissions, the UE skips the part of the overlapping procedure for multiplexing UCI of an overlapping PUCCH in the PUSCH.

Observation 4: Support of simultaneous PUCCH and PUSCH transmissions need not have any RAN1 specification impact other than that a UE does multiplex UCI of an overlapping PUCCH in a PUSCH on an SCell (and instead transmits the PUCCH on the primary cell and the PUSCH on the SCell).

When UCI from a PUCCH is multiplexed on a PUSCH, the reliability requirements of the UCI on the PUSCH should be satisfied while maintaining the Rel-16 multiplexing procedure for UCI in a PUSCH. PUSCHs that do not satisfy the reliability requirements should be ineligible for (HP) UCI multiplexing.

When a LP PUSCH overlaps with a HP PUSCH on a same serving cell, the UE drops the LP PUSCH. If the UE would multiplex UCI in the LP PUSCH, the UCI will also be dropped. To increase the transmission opportunity of UCI, the UE should first determine whether the LP PUSCH is overlapping with another HP PUSCH on a same cell before multiplexing the UCI in the LP PUSCH, subject to timelines).

For example, as shown in Figure 4, a LP PUCCH overlaps with LP PUSCH #1 and LP PUSCH #2. In Rel-16, the UCI from the LP PUCCH should be multiplexed in the PUSCH with the smaller cell index, in this case LP PUSCH #1. However, as there is a HP PUSCH that overlaps with LP PUSCH #1, the UE will drop the LP PUSCH #1 with the LP UCI. Further optimizations can be considered to ensure the reliability of UCI on a LP PUSCH.

Figure 4

Proposal 15: For UCI to be multiplexed on a PUSCH, the following conditions should be satisfied.

- Simultaneous PUSCH and PUCCH transmission does not apply.

- The PUSCH satisfies the reliability requirements of the UCI.

- FFS potential solutions to ensure the reliability of UCI on a LP PUSCH.

## Uplink intra-UE prioritization

For intra-UE prioritization of traffic with different priorities, there are two remaining cases from Rel-16.

Case a) 1st DG PUSCH vs. 2nd CG PUSCH: the 2nd CG PUSCH with priority 1 cancels the 1st DG PUSCH with priority 0.

Case b) 1st CG PUSCH vs. 2nd DG PUSCH: the 2nd DG PUSCH with priority 1 cancels the 1st CG PUSCH with priority 0.

In RAN1#102e, it has been agreed to support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH. For this case, the main issue is to ensure that MAC provides MAC PDU for the prioritized grant before the required timeline for cancelation of the deprioritized grant. If the timeline is not satisfied, MAC should not provide MAC PDU to PHY. For handling prioritization, the MAC entity can use time domain resource allocation information for each grant to check whether or not it is overlapped with other grants. Since the MAC knows the time domain resource allocation for each grant, it is possible for the MAC to ensure that the PHY is able to cancel the 1st LP DG and transmit the 2nd HP CG.

Proposal 16: If transmission of a CG-PUSCH with priority 1 starts after a transmission of a DG-PUSCH with priority 0 from a UE on a same serving cell and the two PUSCHs overlap, the UE is expected to cancel the DG-PUSCH before the first overlapping symbol.

Regarding case b), it was agreed to support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17 in RAN1#103e. A serving gNB can ensure that the cancelation timeline is satisfied considering all the configured grant resources with lower priority; otherwise, case b) can be considered as an error case.

Proposal 17: If transmission of a DG-PUSCH with priority 1 starts after a transmission of a CG-PUSCH with priority 0 from a UE on a same serving cell and the two PUSCHs overlap, a UE is expected to cancel the CG-PUSCH before the first overlapping symbol.

# Conclusions

This contribution discusses the issues related to intra-UE multiplexing and prioritization of traffic with different priorities. The proposals are summarized below.

Proposal 1: Support multiplexing UCI of different priorities subject to timeline conditions and RRC configuration and/or dynamic indication from gNB.

Proposal 2: The UCI types with first priority that can be multiplexed on a PUCCH/PUSCH of a second priority are configurable by the network.

Proposal 3: Reuse Rel-16 for the case where a PUCCH with 1 bit LP HARQ-ACK overlaps with a PUCCH with 1 bit HP HARQ-ACK in the time domain.

Proposal 4: Zeros should be appended to the LP/HP HARQ-ACK information bits if the payload of LP/HP HARQ-ACK is less than 3 bits when multiplexing HP HARQ-ACK and LP HARQ-ACK in a PUCCH.

Proposal 5: For PRB determination and RE mapping when multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH, consider following two options.

Option 1) Reuse Rel-15 rules of separate coding in a PUCCH.

Option 2) PRB determination and RE mapping are performed separately for HP HARQ-ACK and LP HARQ-ACK for PF2/3.

FFS: How to ensure the reliability of HP HARQ-ACK.

Proposal 6: When multiplexing a HP HARQ-ACK and a LP HARQ-ACK in a PUCCH format,

- Reuse the maxCodeRate configured in the second PUCCH-Config for HP HARQ-ACK.

- Consider following two options to determine the maxCodeRate for LP HARQ-ACK.

  - Option 1) Reuse the maxCodeRate configured in the first PUCCH-Config of the same PUCCH format.

  - Option 2) RRC configures a separate maxCodeRate for LP HARQ-ACK per PUCCH format in the second PUCCH-Config.

Observation 1: Multiplexing of LP HARQ-ACK codebook and HP HARQ-ACK codebook with same and/or different HARQ-ACK codebook types can be enabled via the configuration for HP/LP multiplexing.

Proposal 7: Consider solutions to ensure the reliability of multiplexing of LP Type-2 HARQ-ACK codebook and HP HARQ-ACK codebook and/or HP data.

Proposal 8: Drop LP HARQ-ACK PUCCH when a LP HARQ-ACK PF0/1 overlaps with a HP SR PUCCH.

Proposal 9: Support multiplexing of LP HARQ-ACK and HP SR when HARQ-ACK is transmitted on PUCCH format 2/3/4

- Use Rel-15 mechanism as a baseline assuming HARQ-ACK and SR have the same priority.

- FFS: how to ensure the latency and reliability of HP SR.

Observation 2: For determination of the PUCCH time unit for handling PUCCH collisions with different PUCCH time units (i.e. slot and sub-slot PUCCH configuration) of different priorities, when A LP HARQ-ACK PUCCH overlaps with more than one HP sub-slot, at least following cases should be considered.

- Case 1) Each HP sub-slot contains zero or more HP SR PUCCH resource.

- Case 2) Each HP sub-slot contains zero or one HP HARQ-ACK PUCCH resource.

- Case 3) Each HP sub-slot contains zero or more HP SR PUCCH resource and zero or one HP HARQ-ACK PUCCH resource.

- Note: Multiplexing of PUCCH with different priorities is performed within a PUCCH time unit (i.e. either within a slot or a sub-slot). E.g. for each PUCCH time unit, put all the associated PUCCHs (e.g. for a same priority, associated PUCCHs are the PUCCHs within the PUCCH time unit) in the corresponding set Q and then reuse Rel-15/Rel-16 rules.

- Aiming to reuse Rel-15/16 pseudo code for PUCCH multiplexing with limited optimization.

Proposal 10: The time unit for solving the collision of PUCCHs with different L1 priority indexes should be the HP PUCCH time unit.

- If a LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units, determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH.

  - FFS details.

Proposal 11: Down select from the following options for multiplexing/prioritizing LP HARQ-ACK PUCCH, HP HARQ-ACK PUCCH and HP SR PUCCH on a same PUCCH. FFS potential enhancements.

- Option 1) All PUCCHs are viewed with same priority – Rel-15 multiplexing applies.

- Option 2) First, multiplex overlapping HP HARQ-ACK PUCCH and LP HARQ-ACK, then multiplex resulting PUCCH and SR PUCCH (if there is overlapping)

Proposal 12: Consider solutions to ensure the reliability of data in a HP PUSCH if a LP Type-2 HARQ-ACK codebook is multiplexed in the HP PUSCH.

Proposal 13: Determine order for resolving overlapping among HP/LP PUCCHs and HP/LP PUSCHs.

Observation 3: In RAN1 specifications, there needs to be no differentiation between intra-band CA and inter-band CA for simultaneous PUSCH and PUCCH transmissions from a UE.

Proposal 14: Send an LS to RAN4 to inquire about the feasibility/MPR for simultaneous PUCCH and PUSCH transmissions on a same cell.

Observation 4: Support of simultaneous PUCCH and PUSCH transmissions need not have any RAN1 specification impact other than that a UE does multiplex UCI of an overlapping PUCCH in a PUSCH on an SCell (and instead transmits the PUCCH on the primary cell and the PUSCH on the SCell).

Proposal 15: For UCI to be multiplexed on a PUSCH, the following conditions should be satisfied.

- Simultaneous PUSCH and PUCCH transmission does not apply.

- The PUSCH satisfies the reliability requirements of the UCI.

- FFS potential solutions to ensure the reliability of UCI on a LP PUSCH.

Proposal 16: If transmission of a CG-PUSCH with priority 1 starts after a transmission of a DG-PUSCH with priority 0 from a UE on a same serving cell and the two PUSCHs overlap, the UE is expected to cancel the DG-PUSCH before the first overlapping symbol.

Proposal 17: If transmission of a DG-PUSCH with priority 1 starts after a transmission of a CG-PUSCH with priority 0 from a UE on a same serving cell and the two PUSCHs overlap, a UE is expected to cancel the CG-PUSCH before the first overlapping symbol.