3GPP TSG RAN WG1 #105	        		             R1-2105336

e-Meeting, May 19th – 27th, 2021

Agenda item:	8.12.1

Source: 	Samsung

Title: 	Support of group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This contribution considers mechanisms to support group-scheduling for RRC_CONNECTED UEs with MBS.

# Group Scheduling

## CFR for MBS

A recurring issue is whether to define the common frequency resource (CFR) for MBS as an MBS-specific BWP (“Option 2A”) or as an MBS frequency region (“Option 2B”). Both options can work and a decision should be based on minimum specification impact.

Regardless of whether or not there is BWP-switching involved for a UE to receive in the unicast BWP and in the MBS BWP (there is no reason for BWP-switching delay assuming same numerology and the MBS BWP fully contained within the unicast BWP), using an MBS frequency region (“Option 2B”) has a smaller specification impact. If an MBS-specific BWP is introduced, the specifications would have to address at least the following aspects which do not exist for an MBS frequency region – it does not matter how those aspects will be specified, it only matters that they need to be specified

- Does the MBS-specific BWP increase the maximum number of 4 DL BWPs?

- Does the MBS-specific BWP require support of 2 active DL BWPs?

- Does the MBS-specific BWP have its own BWP ID?

- Does the MBS-specific BWP have a linked UL BWP or is it a new DL BWP type without a linked UL BWP?

- How is the bwp-InactivityTimer defined and do the conditions for decrementing or restarting the bwp-InactivityTimer need to be determined over both unicast active DL BWP and the MBS BWP?

- Do the measurements/procedures for RLM, LRR/BFR, etc. need to be defined for both the unicast active DL BWP and the MBS BWP?

The specification impact from introducing an MBS-specific BWP for RRC_CONNECTED UEs is clearly larger than the specification impact from introducing an MBS frequency region and therefore the latter option is preferred.

Proposal 1: A UE is configured an MBS frequency region within the active DL BWP (Option 2B).

Regarding the CFR, the following was agreed in RAN1#104bis-e.

Agreement:

One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: Whether more than one CFR is supported per dedicated unicast BWP

- FFS: Whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP

For the first FFS, the motivation for having multiple CFRs is for a UE to receive multiple corresponding MBS. Although scenarios can be envisioned where a first UE and a second UE have a same first CFR and the second UE and a third UE have a same second CFR (and therefore the second UE has two CFRs), such scenarios cannot arbitrarily exist for all UEs and the network needs to anyway limit them. For example, there is no reason for the network to not have a nested structure for the active DL BWPs of different UEs receiving MBS. Also, FDM receptions of multicast PDSCHs are not in scope [1].

For the second FFS, there is no need to configure a CFR to a UE when the CFR is same as the active DL BWP of the UE. However, other MBS-specific configurations need to be independently provided (e.g. PDCCH-Config, PDSCH-Config, etc.). The issue can be left to RAN2 based on information from RAN1 that, from a RAN1 perspective, the CFR does not need to be configured when it is same as the active DL BWP but that does not relate to whether or not other MBS-specific configurations are provided.

Observation 1: There is no need to support more than one CFR per active DL BWP for a UE.

Observation 2: RAN2 can determine whether or not configuration for a CFR is provided to a UE when the CFR is same as the active DL BWP for the UE.

In RAN1#104bis-e, it was agreed to further study whether or not a CORESET configured in PDCCH-config for unicast can be used for multicast (the CORESET needs to be contained in the CFR), and whether or not the CORESET configured in PDCCH-config for multicast can be used for unicast. Overall, the above is entirely a gNB implementation issue. Further, there is no reason for a CORESET associated with search space sets for unicast DCIs to not be possible to be associated with search space sets for multicast DCIs (and the reverse). It is assumed that, similar to Rel-16, the configuration of search space sets for detection of multicast DCIs is separately provided than the configurations of search space sets for detections of unicast DCIs (this is also necessary due to CSS and due to UE-common requirements of multicast DCIs).

Observation 3: Whether or not a UE monitors PDCCH for detection of unicast DCIs and multicast DCIs in a same CORESET is a gNB implementation issue.

Another discussion point was whether to increase the maximum number of CORESETs per BWP for a UE supporting multicast. Such an increase would mirror (and would be additional to) the one for a UE supporting M-TRP operation. An increase in the number of CORESETs is meaningful if there is an increase in the number of active TCI states that a UE needs to support. Increasing the number of active TCI states means that a UE needs to increase a number of CSI-RS that the UE maintains for QCL-D. That represents a substantial increase in UE hardware complexity and is one of the main reasons that M-TRP operation with increased number of CORESETs is a UE capability. Also, such an increase would be for FR2 operation where, in addition to increasing UE hardware complexity, new designs are not in scope [1].

Observation 4: Increasing the number of CORESETs to support multicast operation increases UE hardware complexity, requires additional specifications that are applicable only for FR2, and is not in scope.

## PDCCH Configuration for MBS

It has been agreed to support PDCCH monitoring for multicast scheduling according to CSS (i.e. Yp,-1=0) while support according to USS (i.e. Yp,-1=nG-RNTI) is FFS. For CSS, the search space set prioritization in case of overbooking is not concluded. Another issue with CSS is that all PDCCH candidates will always collide either with PDCCH candidates for Rel-16 CSS types or with candidates for other multicast services. Therefore, the number of PDCCH candidates of search space sets associated with multicast PDCCH will need to be over-dimensioned and, in return, that will always lead to overbooking considering that the corresponding CCE aggregation levels would need to be the larger ones. USS allows the Rel-16 rules to apply for the search space set prioritization, avoids the previous constant overlap of PDCCH candidates and an associated over-dimensioning for the number of PDCCH candidates, and does not require any specification impact or any new UE implementation.

Proposal 2: A UE can be configured to monitor PDCCH for multicast PDSCH scheduling according to USS.

For PDCCH monitoring according to CSS for multicast PDSCH scheduling and for handling overbooking events, a first option is to assign a monitoring priority to each CSS set. That would provide the Rel-16 functionality and enable a network to prioritize certain unicast services (e.g. URLLC or XR) over multicast. A second option is for the monitoring priority of search space sets for multicast to be hard-coded in the specifications to be larger than the one of search space sets for unicast. That is obviously a faulty design. Also, it is noted that an RRC_CONNECTED UE does not monitor Rel-16 CSS sets in every slot but rather with a periodicity that is at least 5 msec (and typically much larger than 5 msec) – therefore, it cannot be assumed that a UE monitors in a same slot PDCCH for search space sets of Rel-16 TypeX-PDCCH CSS and PDCCH for search space sets of multicast. It was also suggested to support both options but, in addition to increasing the specification impact for no reason (e.g. the first option can practically realize the second option by gNB implementation), that would be the worst alternative for a UE.

Proposal 3: The monitoring priority of CSS sets for multicast PDSCH is determined based on the CSS set index.

Another proposal that was discussed in RAN1#104bis-e is to increase the maximum numbers of PDCCH candidates and non-overlapping CCEs that a UE can monitor per slot. That proposal was associated with UEs supporting CA but has nothing to do with CA as MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are the maximum numbers of PDCCH candidates and non-overlapping CCEs that a UE can monitor per slot per scheduled cell and are independent of any CA capability. That proposal probably relates to introducing a new UE capability, similar to the Rel-16 M-TRP capability that was targeted for FR2. However, the objective of this WI is to enable multicast to be deployed quickly to address market needs while re-using as much as possible existing UE implementations and focusing on FR1. The WID is even explicit about avoiding additional UE hardware complexity (over Rel-15, not even Rel-16).

Observation 5: Increasing MPDCCHmax,slot,μ and CPDCCHmax,slot,μ does not relate to CA capability of a UE - it is a core aspect of a UE implementation that is hard-coded in the specifications - any increase would have substantial impact on modem design and is precluded by the WID.

In RAN1#104bis-e, it was agreed to support at least two DCI formats for scheduling multicast PDSCH where the first DCI format is based on DCI format 1_0 and the second DCI format is based on DCI format 1_1 or 1_2. Whether the at least two DCI formats for multicast PDSCH would have names that are same as or different than the unicast ones can be left to the editor of TS 38.212 – either alternative is possible. RAN1 should focus on the contents/fields and their sizes in the DCI formats for scheduling multicast PDSCH.

Multicast DCI formats should be considered together with unicast DCI formats in determining the DCI format size budget. Otherwise, a network will probably need to resize some DCI formats 2_x, increase them in size, and that is not realistic. Considering the “3+1” DCI size budget, the number of DCI formats can be as many as allowed by that budget. For example, when the UE is configured to monitor PDCCH for DCI formats 0_0/1_0, for DCI formats 0_1/1_1 (size matched) and for DCI formats 0_2/1_2 (same size or size matched), there is no reason to preclude a network from using 3 DCI formats with corresponding sizes to schedule multicast PDSCH (subject to no specification or UE implementation impact). For example, when a UE does not have unicast traffic, there is no reason for the DCI formats scheduling multicast PDSCH to be size matched to any unicast DCI format that a UE is not configured to monitor (and there is no reason to introduce virtual DCI formats). The fields in the DCI format can be based on the unicast ones but some fields may be removed (e.g. TPC command field), some new fields may be added (e.g. indication for enabling/disabling HARQ-ACK, if agreed) and, more importantly, the fields can have configurable sizes to enable the network to select a level of flexibility for the functionality of a given field – as is the case for DCI formats 0_2/1_2.

DCI formats 0_2/1_2 can have a smaller size than DCI format 0_0/1_0 and were introduced in Rel-16 URLLC to enhance PDCCH coverage/reliability – that objective also holds for multicast considering that cell-edge UEs will need to be addressed by practically every PDCCH and it is important to minimize consumption of CCEs. DCI formats 0_2/1_2 can also have same size as DCI formats 0_1/1_1 or as DCI formats 0_0/1_0. Basically, the DCI formats for scheduling multicast PDSCH can have sizes determined by the constraint to not exceed the Rel-16 unicast DCI size budget (3) but the sizes of their fields can be configurable following the same principles as for unicast DCI formats 0_2/1_2.

Observation 6: DCI formats scheduling multicast PDSCH can have same size with any unicast DCI format (i.e. any unicast DCI format can be used as ‘baseline’).

Observation 7: DCI formats scheduling multicast PDSCH have configurable fields and can follow DCI formats 0_2/1_2. This maximizes functionality and minimizes specification impact as size matching procedures do not need to be defined.

Proposal 4: The fields of DCI formats are based on the fields of DCI formats 0_2/1_2.

Proposal 5: The sizes of the DCI formats scheduling multicast PDSCH are counted together with the unicast ones and are such that the budget of 3 sizes is not exceeded.

## Transmission scheme and HARQ process management

In RAN1#104bis-e it was proposed to support a UE to receive a later/second TB for a HARQ process before acknowledging correct reception of a previous/first TB for the HARQ process. The proposal was conditioned on the initial TB reception to be in a multicast PDSCH but that is not relevant given that HARQ processes are shared between multicast and unicast and a UE capability to support such feature does not depend on whether the RNTI used to scramble the CRC of a DCI format scheduling the PDSCH is a first RNTI or a second RNTI. It is noted that the above scenario can already happen due to NACK-to-ACK errors. Overall, the motivation of the proposal is unclear as, regardless of retransmissions, the maximum number of HARQ processes is adequate for any UL-DL configuration (including the typically used DDDSU). A problem may occur in case of PUCCH repetitions for coverage enhancements, but that does not relate to multicast and coverage enhancements are addressed by another Rel-17 WI.

Observation 8: There is no need to specify a UE behavior for a case where the UE receives a new TB for a HARQ process before the UE provides a positive acknowledgement for a previous TB for the HARQ process.

Another proposal from RAN1#104bis-e was to allow a network to simultaneously retransmit a TB by multicast PDSCH and by unicast PDSCH. That is possible from a network perspective for example by configuring to a first UE a search space set for monitoring PDCCH for multicast PDSCH and not configuring to a second UE the search space set (and, in general, the network scheduling behavior is not specified). Then, retransmission of the TB can use a multicast PDSCH for the first UE and use a unicast PDSCH for the second UE. The first and second UEs do not know what the network is doing and there is no specification impact for the UE behavior.

The situation is different if a same UE is scheduled by a first DCI format and by a second DCI format to simultaneously receive a same TB, at least when the UE does not have such capability (although the FFS from RAN1#104bis-e was not for the same UE). It is noted that it doesn’t matter if both DCI formats are unicast, or multicast, or if one is unicast and the other is multicast (the UE does not care what RNTI is associated with a DCI format) or if the TB retransmission is for an initial transmission provided by a DCI-based PDSCH or a SPS PDSCH. As there is no need for a network to use two different DCI formats (not repetitions of a same DCI for PDCCH coverage enhancements as in Rel-17 M-TRP) to schedule simultaneous reception by a UE of a TB, the UE behavior can remain undefined for such event as in Rel-16.

Observation 9: There is no need to define a UE behavior for simultaneous receptions of PDSCHs that are scheduled by different DCI formats and provide a same TB.

In Rel-16, a UE can be scheduled to simultaneously receive two PDSCHs providing different TBs only when the corresponding priorities are different (and the UE drops reception of the low priority PDSCH based on a timeline). That principle should remain regardless of whether the two PDSCHs are unicast, multicast, or unicast and multicast (that does not matter to a UE). The case that a UE is scheduled to simultaneously receive two PDSCHs providing different TBs and the corresponding DCIs (or RRC configurations) indicate a same priority can be considered an error case and the UE behavior remains undefined as in Rel-16.

Observation 10: In Rel-16, when a UE would simultaneously receive two TBs, it is an error case when the TBs are associated with same priority; otherwise, the UE drops reception of the TB associated with low priority. The Rel-16 UE behavior can remain applicable regardless of whether the TBs are provided by multicast PDSCH or by unicast PDSCH.

# Conclusions

This contribution considered reliability improvements for MBS and proposes the following.

Proposal 1: A UE is configured an MBS frequency region within the active DL BWP (Option 2B).

Proposal 2: A UE can be configured to monitor PDCCH for multicast PDSCH scheduling according to USS.

Proposal 3: The monitoring priority of CSS sets for multicast PDSCH is determined based on the CSS set index.

Proposal 4: The fields of DCI formats are based on the fields of DCI formats 0_2/1_2.

Proposal 5: The sizes of the DCI formats scheduling multicast PDSCH are counted together with the unicast ones and are such that the budget of 3 sizes is not exceeded.

In addition, the following observations are made.

Observation 1: There is no need to support more than one CFR per active DL BWP for a UE.

Observation 2: RAN2 can determine whether or not configuration for a CFR is provided to a UE when the CFR is same as the active DL BWP for the UE.

Observation 3: Whether or not a UE monitors PDCCH for detection of unicast DCIs and multicast DCIs in a same CORESET is a gNB implementation issue.

Observation 4: Increasing the number of CORESETs to support multicast operation increases UE hardware complexity, requires additional specifications that are applicable only for FR2, and is not in scope.

Observation 5: Increasing MPDCCHmax,slot,μ and CPDCCHmax,slot,μ does not relate to CA capability of a UE - it is a core aspect of a UE implementation that is hard-coded in the specifications - any increase would have substantial impact on modem design and is precluded by the WID.

Observation 6: DCI formats scheduling multicast PDSCH can have same size with any unicast DCI format (i.e. any unicast DCI format can be used as ‘baseline’).

Observation 7: DCI formats scheduling multicast PDSCH have configurable fields and can follow DCI formats 0_2/1_2. This maximizes functionality and minimizes specification impact as size matching procedures do not need to be defined.

Observation 8: There is no need to specify a UE behavior for a case where the UE receives a new TB for a HARQ process before the UE provides a positive acknowledgement for a previous TB for the HARQ process.

Observation 9: There is no need to define a UE behavior for simultaneous receptions of PDSCHs that are scheduled by different DCI formats and provide a same TB.

Observation 10: In Rel-16, when a UE would simultaneously receive two TBs, it is an error case when the TBs are associated with same priority; otherwise, the UE drops reception of the TB associated with low priority. The Rel-16 UE behavior can remain applicable regardless of whether the TBs are provided by multicast PDSCH or by unicast PDSCH.

# References:

[1] RP-201038, “WID revision: NR Multicast and Broadcast Services”