3GPP TSG RAN WG1 #105	        		             R1-2105340

e-Meeting, May 19th – 27th, 2021

Agenda item:	8.13.2

Source: 	Samsung

Title: 				    On a single DCI format scheduling on multiple cells

Document for:	Discussion and Decision

# Introduction

This contribution overviews outcomes from the study on a single DCI format scheduling PDSCH (or PUSCH) on multiple cells [1] and considers possible next steps.

# Metrics for multi-cell scheduling with single DCI format

Using a single DCI format to schedule PDSCH (or PUSCH, although UL CA is much less frequent than DL CA) on two cells allows, without penalizing throughput, to avoid duplication of the following fields: (a) CRC, (b) fields related to PUCCH transmission (TPC command, slot indicator, PUCCH resource indicator), and (c) DAI field (at least in principle, for the case of Type-2 HARQ-ACK codebook). Reduction in size for other fields will have a negative impact on throughput and reverse any control overhead reduction, at least for the inter-band scenario of the study.

There was generally a good alignment in the link-level simulation results from all sources - on average, across SINRs, use of a DCI format that schedules PDSCH on 2 cells requires 1-2 fewer CCEs than use of 2, single-cell, scheduling DCI formats (DCI format 1_1). Translating those savings into system level gains led to divergent conclusions.

The basic principle for translating BW savings into capacity/throughput savings is Shannon’s capacity formula. That is a transparent approach that removes any ambiguity for calibration issues or system simulation errors. Then, the throughput gain is proportional to the BW/CCE savings and, depending on the BW of the scheduling cell, the maximum throughput gains are in the range of 0%-1%. To achieve the maximum throughput gains (i.e. CCE savings from using a multi-cell scheduling DCI format directly scale overall control overhead), several optimistic (and unrealistic) assumptions were made, including:

- All scheduling, all the time, is for PDSCHs on two cells. That means no common control, no UL scheduling, no UEs without CA configuration, no scheduling on an odd number of cells - ever.

- The CCE savings can always be translated into throughput savings. That means that either (a) no other PDCCH exists in the CORESET, not even PDCCH scheduling the same UE in different cells, or (b) the CORESET is dimensioned assuming that the only use will be for single-DCI scheduling on two cells – always.

- Some additional second-level, “ideal-case”, assumptions such as

  - no impact on PDCCH overhead from padding DCI formats scheduling PUSCH, or from using the 2-cell scheduling DCI to scheduled on one cell, in order to maintain the “3+1” DCI format size budget,

  - no coverage loss from using a larger DCI format size, due to UEs with 2 Rx antennas in lower frequency bands or due to larger path-loss in larger frequency bands (in FR1).

Observation 1: No throughput gains would exist in practice from using one DCI format to schedule on two cells.

Another metric that was considered for potential benefits from using one DCI format to schedule on two cells was the PDCCH blocking probability. The expectation is that due to the average CCE savings of 1-2 CCEs, a single DCI format can result to a reduced PDCCH blocking probability. However, some of that reduction would be offset by the use of larger CCE aggregation levels for a PDCCH transmission with the larger 2-cell scheduling DCI format size and, at the (theoretical, not actual) limit where the 2-cell scheduling DCI format would require ~2x the CCEs of a single cell scheduling DCI format, the blocking probability would be larger for the 2-cell scheduling DCI format. Nevertheless, as the actual increase in the number of required CCEs is somewhat less than 2x, a reduction in PDCCH blocking probability is possible and then the question is for the level of such reduction and whether it has any operational benefit. The evaluations for the blocking probability had similar optimistic (and unrealistic) assumptions as some of the evaluations for the throughput gains, namely:

- All scheduling, all the time, in any CORESET, is for PDSCHs on two cells - no common control, no UL scheduling, no UEs without CA configuration, no scheduling on an odd number of cells.

- A large number of UEs (at least 5, typically more than 10) are scheduled in every slot with DL CA on carriers of relatively small BWs – that is never the case in practice

- Assumption of only one CORESET – use of more CORESETs is mathematically guaranteed to reduce PDCCH blocking probability

- Blocking probabilities that are already too large for both 1 PDCCH and 2 PDCCHs for scheduling 2 PDSCHs

- No consideration for increase in PDCCH blocking probability due to requirements for size matching DCI formats (thereby using larger CCE aggregation levels) in order to maintain the DCI format size budget

Observation 2: Although some reduction in PDCCH blocking probability may exist from using a single DCI format to schedule PDSCHs on two cells, it is not expected to have any practical impact on scheduling.

Although using a single DCI format to schedule on two cells is not meaningful to further consider, large PDCCH overhead reductions could result to non-negligible throughput gains in heavy DL CA deployments (heavy UL CA deployments do not exist and are expected to remain unlikely at least in the near future). To achieve large PDCCH overhead reductions, focus can be on intra-band CA and on a number of cells that is larger than two as such scenarios offer a possibility for material PDCCH overhead reductions due to non-duplicated fields or due to a reduction in sizes of fields relative to corresponding combined sizes across scheduled cells in case of a DCI format scheduling on a single cell. Further, existing limitations in translating CCE savings into throughput gains (i.e. being able to use varying available RBs within a CORESET for PDSCH receptions) or offsetting effects from padding DCI formats due to the “3+1” requirement for the number of DCI format sizes need to be addressed in order to utilize potential PDCCH overhead reductions. Coverage may also need to be considered in case the size of the DCI format is relatively large (e.g. >130 bits).

Further discussions on a single DCI format scheduling on multiple cells may depend on available time in Rel-17 and may also be subject to RANP guidance. If such discussions occur, focus should be on identifying meaningful deployment scenarios where scheduling by a single DCI format on multiple cells may provide throughput gains and potentially on mechanisms to realize such gains. The study should also include multi-cell PUSCH scheduling, for overall design flexibility/efficiency, although UL CA is not a likely scenario for throughput gains in practice.

Proposal 1: If there is further study on a single DCI format scheduling on multiple cells, focus should be on scenarios and mechanisms that may offer throughput gains under realistic/typical operating conditions.

# Conclusions

This contribution considered outcomes from the study on a single DCI format scheduling PDSCH (or PUSCH) on multiple cells and possible next steps, and proposes the following.

Proposal 1: If there is further study on a single DCI format scheduling on multiple cells, focus should be on scenarios and mechanisms that may offer throughput gains under realistic/typical operating conditions.

In addition, the following are observed.

Observation 1: No throughput gains would exist in practice from using one DCI format to schedule on two cells.

Observation 2: Although some reduction in PDCCH blocking probability may exist from using a single DCI format to schedule PDSCHs on two cells, it is not expected to have any practical impact on scheduling.

# References:

- R1-2102138, “Feature lead summary#4 on multi-cell scheduling via a single DCI”