3GPP TSG RAN WG1 #105-e                                 R1-2105423

e-Meeting, May 10th –27th, 2021

Agenda Item:	8.2.6

Source: 	LG Electronics

Title: 	Considerations on channel access mechanism to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

The following WID objectives related to the channel access mechanism were made in RAN#90-e meeting [1]:

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access Study, and if needed specify, energy detection threshold enhancement

In this contribution, we discuss and provide views on the channel access mechanism to support NR above 52.6 GHz, in terms of the channel access mode switching, the introduction of CAPC, CWS adjustment mechanism and Type 2 (2A/2B2C) channel access procedure to the beam-based LBT/transmission, and the ED threshold enhancement considering the relationship between the LBT beam and transmission beam(s).

- The channel access mode

■ Mechanism and condition(s) to switch between LBT and No-LBT

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between - Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication - Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication - FFS: Whether the indication of the decision on applying LBT mode or no-LBT mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation) - FFS: Whether a gNB and its UE(s) can have different mode - FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

In the RAN1#102-e meeting, it was agreed that both channel access with LBT mechanism(s) and a channel access mechanism without LBT are supported to initiate a channel occupancy for gNB and UE [2]. Basically, LBT mechanisms such as omni-directional LBT and directional LBT can be used to initiate the channel occupancy. However, if the local regulation allows initiating channel occupancy without LBT and at least one of the specific conditions are met, the channel access with LBT can be switched to the channel access mechanism without LBT.

For example, gNB/UE can switch to the channel occupancy initiated by channel access mechanism without LBT (during limited time) only when it can assume low interference environment e.g., by estimating interference level or by using HARQ information of (consecutive) successful transmissions. Once the channel access mechanism is switched to one without LBT, the timer can be started and the channel access mode can be fallback to the channel access mode with LBT mechanism when the timer is expired or at least one of the specific conditions (e.g., when NACKs were received consecutively for the PDSCH/PUSCH transmitted without LBT) for switching the channel access mechanism is met. This switching mechanism between the LBT and No-LBT mode can be beneficial for coexistence with different RAT/operator or for high traffic load scenario.

Proposal #1: The channel access mechanism can be switched from LBT mode to no-LBT mode based on timer operation when receiving the information of the local regulation from the gNB (by cell specific or UE specific signaling) and satisfying certain conditions such as a low interference environment.

■ Contention exempt short control signalling

Agreement: - Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) - FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc Agreement: For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission. - RMSI PDCCH and RMSI PDSCH - Other broadcast PDSCH - PDSCH without user-plane data - PDCCH - CSI-RS - PRS - Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst) Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval. FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

In RAN1#104b-e meeting [5], it was agreed that the contention exempt short control signalling rules can be applicable to the transmission of SS/PBCH and the candidate signals/channels that can be multiplexed with the SS/PBCH block transmission are listed.

Based on the latest ETSI EN 302 567, the contention exempt short control signalling can be supported in NR above 52.6 GHz for the transmission(s) initiated by gNB with SS/PBCH multiplexing with non-unicast information (e.g., SIB1, CSI-RS), where the transmission(s) duration is not exceed 10ms within an observation period of 100ms. However, for the signals and channels other than SSB (e.g., PRACH), the clarifications on whether the constraints/conditions such as duty cycle are per cell or per UE (for UL only) should be preceded.

Proposal #2: The contention exempt short control signalling can be supported for SS/PBCH multiplexed with non-unicast information (e.g., SIB1, CSI-RS), where the transmission(s) duration is not exceed 10ms within an observation period of 100ms.

■ LBT bandwidth

Agreement: For LBT for single carrier transmission, continue down selection between - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, continue down selection between - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

In RAN1#104b-e meeting [5], the narrowed alternatives for the LBT bandwidth on which channel access mechanism is performed by gNB/UE were captured for single-carrier and multi-carrier transmission. In NR-U, considering the regulatory requirement such as the nominal channel bandwidth defined in ETSI EN 302 893 and coexistence with the incumbent system (e.g., Wi-Fi), 20 MHz is always considered as the unit LBT bandwidth.

Meanwhile, in 60 GHz band which is considered in Rel-17, the regulatory requirement defined in EN 302 567 does not mandate a specific LBT bandwidth or a relation between the LBT BW and the multiple declared channel bandwidths. Although the definition of LBT bandwidth is ambiguous in the regulation, it may be necessary to define a unit LBT bandwidth that enables gNB and UE to efficiently perform LBT and transmit/receive transmission before transmission in the 60GHz band.

Even though the main purpose of LBT is to mitigate the potential interference, the channel access probability may be reduced if the gNB/UE should always perform channel access procedure on large bandwidth more than necessary. On the contrary, if the gNB/UE adjusts the LBT bandwidth per transmission (i.e., according to the transmission bandwidth), it may cause coexistence issues with other incumbent system (e.g., WiGig) and also create high overhead for regulatory testing.

Therefore, it is necessary to define a unit of LBT bandwidth and for gNB/UE to perform LBT in all the LBT units (to be transmitted in) in the channel bandwidth. The unit of LBT bandwidth for a UE can be configured by the gNB and the size of LBT bandwidth can be adjusted to manage the channel access probability and the spectral efficiency considering the bandwidth capability of UE and the interference level of the cell. Unlike NR-U, the LBT bandwidth of 60GHz may have a structure in which multiple BWPs/CCs are confined within only a single LBT bandwidth. In other words, one BWP/CC smaller than a LBT bandwidth does not overlap with multiple LBT bandwidths. When the unit LBT bandwidth is defined, it is no need to distinguish between single-carrier LBT and multi-carrier transmission LBT. Moreover, the operating channel bandwidth of the baseline ED threshold formula can also be calculated by replacing with the unit LBT bandwidth.

Proposal #3: Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth.

■ Introduction of CAPC and CWS adjustment mechanism

Agreement: Use the CCA check procedure in EN 302 567 (per RAN1 understanding as from RAN1 #102-e) as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work. - Whether CAPC and contention window adjustment mechanisms are introduced - Whether ED threshold change is needed, e.g., due to changes in bandwidth, beamforming gain etc. - Whether contention window range needs to be adjusted

In RAN1#103-e meeting [3], the agreement was made that the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied and whether to introduce CAPC and CWS adjustment mechanism will be further discussed during WI phase.

Because the purpose of CAPC and CWS adjustment are to prioritize high priority traffic and resolve the collision between the transmissions, the introduction of CAPC and CWS adjustment mechanism can be beneficial in highly congested scenario. Moreover, considering the fair coexistence with the incumbent system (e.g., WiGig) operating in the above 52.6GHz, it is necessary to consider the introduction of CAPC and CWS adjustment procedure. The procedures specified for the CAPC and CWS adjustment mechanism in Rel-16 NR-U can be reused as baseline for operation in the 60 GHz band.

Proposal #4: Introduce channel access priority class and the contention window adjustment mechanisms when LBT is used in NR above 52.6 GHz, similar to Rel-16 NR-U.

- Considerations for LBT schemes and LBT types

Since the cell coverage can be reduced in the high frequency band, the beam-based transmission using a multi-antenna beamforming technique may be useful. Therefore, the directional CCA can be utilized for channel access mechanism to enhance the performance of beam-based transmission in above 52.6GHz band and the following issues related to directional CCA operation and COT sharing can be discussed.

■ Directional LBT and receiver assisted LBT

  - CCA threshold

    - CCA threshold for directional transmission may need to be different from the case of omni-directional transmission since the coverage and interference from a directional transmission will be different from an omni-directional transmission

  - Relationship between transmission direction and CCA direction

    - In the actual implementation of the UE or gNB, the receiving direction and the transmitting direction may not match well. Moreover, applying same spatial direction as transmission direction for CCA may not be effective since interference to the receiver will be estimated in the opposite direction to the transmission direction as depicted in figure 1.

Figure 1. Illustration of interference caused by directional transmission

  - Receiver assisted LBT

    - Considering that the difference between the interference measured by the transmitter and the receiver can be significant, it can be helpful to utilize the assistance information sent by the receiver. However, since the contents of the assistant information or feedback mechanisms can be fully supported by the current Rel-15/16 specification, there is no need to specify additional mechanisms other than the indication of which LBT to be used at receiver side when the receiver assisted LBT is used.

Proposal #5: The directional CCA and the receiver assisted LBT can be beneficial to increase cell coverage and spatial reuse, and whether or not the receiver assisted LBT can have an impact on specification except for indicating LBT type to responder should be first investigated.

Agreement: For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion - Alt 1. Legacy RSSI measurement and reporting with possible enhancements - Alt 2. AP-CSI report with possible enhancements - Alt 3. LBT at receiver - Alt 3.1 eCCA - Alt 3.2 Cat2 LBT

In RAN1#104-e meeting [4], the three alternatives were captured as set of tools for the receiver assisted LBT. As described before, the additional or new mechanism for the receiver assisted LBT is not necessary because the assistance information or feedback mechanism is already supported by the current specification. In this regard, the legacy RSSI measurement and reporting with possible enhancements are enough for the receiver to provide assistance.

Regarding the potential enhancements of Alt 1, the directional RSSI and L1 RSSI reporting might need to be introduced considering that the beam-based aspects are not captured in the legacy mechanism and L1 RSSI can decrease the RSSI reporting delay.

Proposal #6: For the receiver to provide assistance, support Alt 1 (i.e., legacy RSSI measurement and reporting with possible enhancements), and the introduction of directional RSSI and L1 RSSI reporting can be considered as the potential enhancements.

■ Transmission with multiple candidate directions

For some broadcast signals/channels (e.g. SSB) in NR, multi-beam sweeping transmission is necessary. Depending on the further discussion on the LBT operation for directional transmission, directional CCA at transmitter side may be necessary. In that case, it should be further studied how to perform omni-directional or directional CCA for multiple-beam sweeping transmission.

In addition, if either transmitter or receiver side channel sensing is introduced per transmission direction or reception direction, it should be also discussed how to define CWS management, e.g., per-direction management or across-direction management. It can be also considered how to adjust CWS if omni-directional LBT as well as directional LBT are operated at the same time. To be specific, CWS increment/reset event in a particular direction may affect the CWS managed for other directions. For example, if the CWS for directional LBT increase occurs in a particular direction due to collision, the CWS managed for omni-directional LBT may also be affected and may be increased. Conversely, if the CWS reset event is occurred in omni-directional LBT, the CWS managed in each direction may also be affected and may be reset. Also, if the back-off counter value is also defined per-direction, the same principle can be applied to back-off counter value decrease in Type1 LBT procedure.

Proposal #7: If the directional CCA procedure is introduced the followings points can be considered:

- How to perform the CCA procedure for multiple-beam sweeping transmission

- How to define CWS management (e.g., per-direction or across-direction management)

- How to manage the back-off counter value

■ Type 2 channel access procedures and the indication of LBT types

Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select from - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission - FFS: Value for X - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT - FFS: Value for Y - FFS: How to define the one-shot LBT

In RAN1#104-e meeting [4], there were discussions related to the Cat-2 LBT and the maximum gap for a responding device in the shared COT. Even the EN 302 567 does not explicitly define the gap allowed for COT sharing, it is beneficial to introduce the maximum gap and the Cat-2 LBT for efficient COT sharing to support NR above 52.6GHz. In addition, the several use cases of Cat-2 LBT other than COT sharing is identified to be studied during the RAN1#104-e meeting as follows: Cat-2 LBT may be used before switching to a new transmission beam in a COT with TDM beams, and Cat-2 LBT may be used for sensing at the receiver as a responding device for RX-assistance measurement and associated signalling.

In NR-U, the initiating device can transmit the transmission(s) in its own COT by supporting multiple switching points when the conditions such as the gap are met and the responding device may transmit a transmission immediately after sensing the channel to be idle for at least a specific sensing interval. If the gap between the transmissions is larger than the maximum gap, the COT acquired by the initiating device cannot be maintained and the collisions may occur by the transmissions from other devices that sensed the channel to be idle.

The same principle can be applied to the COT sharing between the gNB and UE in the COT acquired by the directional LBT. Moreover, a gNB can transmit the multiple beams in a TDM manner, resulting in transmissions gaps on a beam, within a COT after sensing the channel in the corresponding directions at the beginning of the COT. Therefore, the maximum gap Y between the transmissions in the COT should be defined and the Type 2 (e.g., Type 2A/2B/2C) channel access procedure can be performed before starting the transmission to avoid the collisions. The definition of Type 2 (e.g., 2A/2B/2C) channel access procedure in Rel-16 NR-U can be reused with possible modifications to the parameters such as the gap duration for each type of LBT.

Proposal #8: Type 2 (e.g., 2A/2B/2C) channel access procedure can be introduced for the use cases such as COT sharing, multi-beam LBT, and Rx-Assistance and the maximum gap Y between the transmissions within the COT can be defined for above 52.6 GHz.

If the directional LBT can be used to initiate channel occupancy together with omni-directional LBT for initiating a channel occupancy, it is necessary to indicate both the direction of LBT (e.g., omni-directional LBT or directional LBT) and the type of LBT (e.g., Type 1 or Type 2A/2B/2C channel access procedure in NR-U) when scheduling a UL transmission to a UE. Moreover, it is necessary to determine which type and direction of LBT should be indicated to a UE inside of gNB’s channel occupancy acquired by omni-directional LBT or directional LBT. For example, the omni-directional Type 2A LBT can be indicated to UE inside of the channel occupancy acquired by omni-directional Type 1 LBT by gNB.

Proposal #9: It should be discussed how to indicate the direction of LBT (e.g., omni-directional LBT or directional LBT) and the type of LBT (e.g., Type 1 or Type 2A/2B/2C channel access procedure in NR-U) when scheduling a UL transmission inside or outside of a channel occupancy.

■ Tx Burst construction for COT sharing

If the directional LBT is performed with a specific LBT beam using a specific directional antenna, the CCA range should cover the interference range of the transmission beam(s). Within the COT acquired by the LBT beams with specific direction and ED threshold, the transmission of beams that have directions different from the CCA range (due to large transmit power than ED threshold) shall not be allowed to be multiplexed (SDM/TDM) in the COT. For example, if the COT is acquired by the directional LBT with Beam A, the transmission of beam(s) is only allowed to the beam having the same direction as Beam A in the COT. When LBT is performed with wide beam A, it may be necessary to discuss whether to allow transmission of the multiple narrow beams included in the beam A (i.e., multiple beam sweeping transmission) and whether to allow the transmission of beam other than the LBT beam in the COT.

Proposal #10: The relationship between the LBT beam with a specific direction to acquire the COT and the transmission beam(s) allowed to transmit in that COT should be defined considering the relationship between the CCA range of the LBT beam and the interference range of the transmission beam(s).

If the directional LBT is performed to transmit a beamformed transmission, it may be desirable that all DL signals/channels (or UL signals/channels) belonging to the same TX burst have QCL relationship. For example, the gNB performs directional LBT to Beam A to acquire COT and transmits DL signals across four slots. If gNB’s transmission is beamformed to Beam A for the first three slots but the last slot is redirected to Beam B, it may collide with other nodes (e.g., WiGig AP) that has transmitted in the direction of beam B after the success of directional LBT to Beam B. The same principle can be applied to the COT sharing between the gNB and UE in the COT acquired by the directional LBT. When the spatial relationship between the DL signals/channels and the UL signals/channels are associated, it is desirable that the COT acquired by directional LBT is shared between the DL signals/channels and UL signals/channels having spatial QCL relationship.

Proposal #11: It would be beneficial for coexistence that channel occupancy acquired by directional LBT is shared only for DL and UL signals/channels having spatial QCL relationship.

##### Proposal 2.9.3-1: - 3GPP specification defines the relative relationship between all applicable sensing beams and the transmission beam(s), at least sensing beam “covers” the transmission beam(s) - Alt 1. To define “cover”, the angle included in the [3]dB beamwidth of the transmission beam(s) is included in the [3]dB beamwidth of the sensing beam - Alt 2. Extending the beam correspondence and/or QCL/TCI framework to define “cover” - Alt 3. Leave RAN4 to define “cover”

As cited from [6] above, whether/how to specify the relationship between sensing beam and transmission beam was discussed in previous meetings. Regarding this issue, we think that the relative relationship between all applicable sensing beams and the transmission beam should be defined in 3GPP specification. For the sensing beam “covers” the transmission beam(s), Alt 2 (Extending the beam correspondence and/or QCL/TCI framework) in the Proposal 2.9.3-1 [6] can be used to define “cover”.

Proposal #12: To define the relative relationship between all applicable sensing beams and the transmission beam in 3GPP specification, adopt Alt-2 (Extending the beam correspondence and/or QCL/TCI framework to define “cover”).

■ Multiplexing of multi-beams within a COT

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In RAN1#104b-e meeting [5], the clarification on independent per-beam LBT sensing was discussed and the multiple alternatives based on each proposing company were captured. For Alt B (for both SDM and TDM case), we think that when and what conditions need to support Alt B should be clarified first. For example, only the transmitter equipped with multi-TRP may perform the per-beam LBT for different beams simultaneously.

For Alt A-2, it is equivalent to the independent single beam transmission in an each separate COT. Hence, it is not relevant to the multiplexing of multi-beam transmission. For Alt A-3, it seems that it is not aligned with the LBT procedures described in ETSI EN 302 567 regulation. For a given back-off counter, the CCA check cannot be skipped or assumed to be idle based on measurement of a different spatial channel.

For a COT with MU-MIMO (SDM) and TDM of beams transmission, the per-beam LBT sensing should be performed at the beginning of COT for all beam directions to be transmitted within a COT. In other words, the node should complete the eCCA on all of the beam(s) transmitted in COT and then can start transmission. Therefore, Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) should be supported. For a concern on the large LBT latency, the additional single wide beam (or omnidirectional LBT) of Cat-2 LBT can be used after back-to-back eCCA is finished.

Proposal #13: For a COT with MU-MIMO (SDM) and TDM of beams transmission, adopt Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) when independent per-beam LBT sensing at the start of COT.

- ED threshold enhancement

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

In RAN1#104-e meeting [4], the baseline ED threshold formula was agreed as a function of Pout where Pout is the RF output power (EIRP) and the operating channel BW, the formula should be further modified to account for aspects such as the relationship between the sensing beam and the transmission beam etc.

To explain a method of the ED threshold setting according to the channel bandwidth of NR, the output power and the PSD, the reference LBT parameter set can be defined as {reference (LBT) BW, reference (max output) power, reference ED (threshold)}. The value of each parameter in the reference LBT parameter set can be determined based on the regulatory requirements and/or the (nominal) system bandwidth, the maximum output power (allowed for that BW size), and the ED threshold defined by other RATs. For example, when considering the WiGig system as a reference, the reference LBT parameter set can be expressed as {BWiGig = 2.16GHz, PWiGig = 40dBm, TWiGig = -48dBm}.

If the unit LBT BW of NR is considered to be the same as its CC BW (=BNR) and if BNR = BWiGig, the maximum output power of NR (=PNR) and ED threshold of NR (=TNR) can be set to the same values as in the reference LBT parameter sets (used for WiGig). In other words, the LBT parameter set of NR can be expressed as {BNR = 2.16GHz, PNR = 40dBm, TNR = -48dBm}, and in this case, it would be obvious that NR can fairly coexist with WiGig.

Even in case when BNR <BWiGig, the maximum output power and ED threshold of NR could be set to the same values as in the reference LBT parameter sets used for WiGig as long as it does not violate the regulatory requirement. However, since NR performs LBT with a smaller bandwidth than WiGig, PNR and TNR might need to be adjusted with consideration of coexistence. For example, in case when BNR < BWiGig, PNR can be set to be lower than PWiGig, and TNR can be adapted to be higher than TWiGig. As a result, the NR can perform the channel access procedure using the LBT parameter set by {BNR (<BWiGig), PNR (<PWiGig), TNR (> TWiGig)}.

For the relationship between the LBT beam and the transmission beam(s), if the directional LBT is performed with a specific LBT beam (i.e., Rx beam pattern) using a specific directional antenna, the CCA range should cover the interference range of the transmission beam(s). In other words, the value of ED threshold and the CCA range should be determined considering the interference range of the transmission beam(s) in the COT. Within the COT acquired by the LBT beams with specific direction and ED threshold, the transmission of beams that have directions different from the CCA range (due to large transmit power than ED threshold) shall not be allowed to be multiplexed (SDM/TDM) in the COT. For example, if the COT is acquired by the directional LBT with relatively narrow beam A, the transmission of beam(s) is only allowed to the beam having the same direction as Beam A in the COT. When LBT is performed with wide beam A, it can be allow transmission of the multiple narrow beams included in the beam A in TDM/SDM manner.

In this regard, the ED threshold can be further adjusted by reflecting the relationship between the sensing beam and transmission beam and it may be closely related to the beam correspondence between Tx/Rx beams. The relationship can be determined based on the beam correspondence capability/requirement of UE. For example, the lower (i.e., more sensitive) ED threshold can be applied to the UE satisfying the relaxed requirement for the beam correspondence capability/requirement

Proposal #14: The ED threshold provided by the ETSI 302 567 can be enhanced considering the following points:

- The size of LBT bandwidth

- Transmit power of beam(s) in the COT

- The beam correspondence capability/requirement of UE.

- Conclusions

In this contribution, the channel access mechanism to support NR in high frequency range from 52.6 GHz to 71 GHz was discussed, and the followings are proposed.

Proposal #1: The channel access mechanism can be switched from LBT mode to no-LBT mode based on timer operation when receiving the information of the local regulation from the gNB (by cell specific or UE specific signaling) and satisfying certain conditions such as a low interference environment.

Proposal #2: The contention exempt short control signalling can be supported for SS/PBCH multiplexed with non-unicast information (e.g., SIB1, CSI-RS), where the transmission(s) duration is not exceed 10ms within an observation period of 100ms.

Proposal #3: Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth.

Proposal #4: Introduce channel access priority class and the contention window adjustment mechanisms when LBT is used in NR above 52.6 GHz, similar to Rel-16 NR-U.

Proposal #5: The directional CCA and the receiver assisted LBT can be beneficial to increase cell coverage and spatial reuse, and whether or not the receiver assisted LBT can have an impact on specification except for indicating LBT type to responder should be first investigated.

Proposal #6: For the receiver to provide assistance, support Alt 1 (i.e., legacy RSSI measurement and reporting with possible enhancements), and the introduction of directional RSSI and L1 RSSI reporting can be considered as the potential enhancements.

Proposal #7: If the directional CCA procedure is introduced the followings points can be considered:

- How to perform the CCA procedure for multiple-beam sweeping transmission

- How to define CWS management (e.g., per-direction or across-direction management)

- How to manage the back-off counter value

Proposal #8: Type 2 (e.g., 2A/2B/2C) channel access procedure can be introduced for the use cases such as COT sharing, multi-beam LBT, and Rx-Assistance and the maximum gap Y between the transmissions within the COT can be defined for above 52.6 GHz.

Proposal #9: It should be discussed how to indicate the direction of LBT (e.g., omni-directional LBT or directional LBT) and the type of LBT (e.g., Type 1 or Type 2A/2B/2C channel access procedure in NR-U) when scheduling a UL transmission inside or outside of a channel occupancy.

Proposal #10: The relationship between the LBT beam with a specific direction to acquire the COT and the transmission beam(s) allowed to transmit in that COT should be defined considering the relationship between the CCA range of the LBT beam and the interference range of the transmission beam(s).

Proposal #11: It would be beneficial for coexistence that channel occupancy acquired by directional LBT is shared only for DL and UL signals/channels having spatial QCL relationship.

Proposal #12: To define the relative relationship between all applicable sensing beams and the transmission beam in 3GPP specification, adopt Alt-2 (Extending the beam correspondence and/or QCL/TCI framework to define “cover”).

Proposal #13: For a COT with MU-MIMO (SDM) and TDM of beams transmission, adopt Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) when independent per-beam LBT sensing at the start of COT.

Proposal #14: The ED threshold provided by the ETSI 302 567 can be enhanced considering the following points:

- The size of LBT bandwidth

- Transmit power of beam(s) in the COT

- The beam correspondence capability/requirement of UE.

- References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz”, RAN#90-e, CMCC.

- RAN1#102-e Chairman’s note

- RAN1#103-e Chairman’s note

- RAN1#104-e Chairman’s note

- RAN1#104b-e Chairman’s note

- R1-2104040	Email discussion summary for channel access mechanism for 52.6GHz-71GHz band, ver03 Moderator (Qualcomm Incorporated)