3GPP TSG RAN WG1 #105-e     	                                       R1-2105462

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Maintenance on NR sidelink mode-1 resource allocation mechanism

Agenda Item:	7.2.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues of mode-1 of following aspects:

  - PDCCH candidate for SL DCI

  - DCI size alignment

  - HARQ reporting for multiple PSFCH pools

  - HARQ reporting on PUCCH for an ‘incomplete’ PSFCH period

  - Power control of PUCCH for SL HARQ reporting

  - PUCCH resource for sidelink configured grant type 2

- SL DCI

  - PDCCH candidate location for DCI format 3_0/3_1

When performing blind decoding of a PDCCH candidate of a Uu DCI format, the UE determines the CCE index of the PDCCH candidate according to the following formula, where nCI in the formula refers to the value in CIF field of the cell for which the DCI format is monitored, if the cross-carrier indication is configured, otherwise, nCI is zero.

TS 38.213 For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCIare given by L⋅Yp,ns,fμ+ms,nCI⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i for any CSS, Yp,ns,fμ=0; for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537; i=0,⋯,L-1; NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set; nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0; ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI; for any CSS, Ms,maxL=Ms,0L; for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ; the RNTI value used for nRNTI is the C-RNTI.

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

}

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13].

The IE SearchSpace defines how/where to search for PDCCH candidates. For a scheduled cell in the case of cross-carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpace information element for a scheduled cell

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

}

Figure 1. Example of cross-carrier scheduling Uu

For example, as shown in Figure 1, if UL on cell#1 is cross-carrier scheduled by Uu DCI#2 on cell#0, then the nCI and nrofCandidates used to identify the CCE for the Uu DCI#2 detection are determined based on CIF=1 and nrofCandidates=8 respectively.

Observation 1. In R15, nCI of a PDCCH candidate for self-scheduling is set to 0, while for cross-carrier scheduling, nCI is set to the value of the corresponding CIF field.

Observation 2. In R15, Ms,nCIL corresponding to a Uu DCI format in the search space scheduling a serving cell is determined based on the nrofCandidates of the serving cell corresponding to nCI.

Nevertheless, for SL scheduling, the location (i.e., CCE index) of the PDCCH candidate corresponding to DCI format 3_0/3_1 is not clear. More specifically, if the SL is cross-carrier scheduled as shown in Figure 2, it is unclear which cell’s nCI and nrofCandidates are used for SL DCI detection. Therefore, the value of nCI of PDCCH candidates of a SL DCI format should be specified. Moreover, how many blind detections of DCI 3_0/3_1 the UE should perform is also not clear. As shown in Figure 2, for the same search space (e.g., SS ID = 5), there are 4 BDs configured for cell#0 and 8 BDs configured for cell#1, then how many BDs, e.g., 4, 8, or even 12, are for SL DCI?

Figure 2. Example of cross-carrier scheduling SL

There are two options for defining a SL DCI candidate. For example, the PDCCH candidate for self-scheduling can be used for blind decoding the SL DCI, which means nCI for SL DCI is always set to zero. Another possible way to determine the CCE index is to use the ID of the scheduled cell (i.e., the CIF value in a Uu DCI format scheduling that cell).

- Option1. nCI of PDCCH candidates of a SL DCI format = 0

- Option2. nCI of PDCCH candidates of a SL DCI format = CIF value in a Uu DCI format scheduling the SL cell

With option2, it is unclear how to handle the case of SL deployed on ITS band, since there is no CIF value for a SL cell on ITS band. Thus, option1 is preferred.

Proposal 1. The value of nCI of PDCCH candidates for a SL DCI format is set to zero, and the number of PDCCH candidates associated with a SL DCI format blind decoding is set to the value of nrofCandidates corresponding to the scheduling cell.

- TP#1 for 38.213: PDCCH candidate

------------------------------------------- Start of Draft TP of 213------------------------------------- 10.1 UE procedure for determining physical downlink control channel assignment For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCIare given by L⋅Yp,ns,fμ+ms,nCI⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i where for any CSS, Yp,ns,fμ=0; for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537; i=0,⋯,L-1; NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set; nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS or any USS configured with DCI format 3_0 or DCI format 3_1, nCI=0; --------------------------------------------- End of Draft TP --------------------------------------------

  - DCI size alignment

In the previous meeting, it was agreed that the Uu DCI size budget of the scheduling cell on which SL DCI is received is used for SL DCI size alignment. In TS 38.212, ‘DCI formats configured to monitor for a cell’ in the specification text is used to represent the Uu DCI format corresponding to the concerned Uu DCI size budget. However, it does not include the group common DCI format configured on the scheduling cell.

Noted that in 7.3.1.0.1, group common DCI format 2-x are summarized as ‘DCI formats for other purposes.’

TS 38.212 7.3.1.3 DCI formats for other purposes 7.3.1.3.1 Format 2_0 DCI format 2_0 is used for notifying the slot format, COT duration, available RB set, and search space set group switching. ==================Omitted================== 7.3.1.3.2 Format 2_1 ==================Omitted================== ……

Thus, we suggest using ‘DCI formats for other purposes as described in 7.3.1.3’ to represent the group common DCI format and propose the following TP:

Proposal 2. Adopt the TP#2 regarding DCI size budget for SL DCI alignment in TS 38.212.

- TP#2 for 38.212: size alignment

------------------------------------------- Start of Draft TP of 212------------------------------------- 7.3.1.0.1 DCI size alignment for DCI formats for scheduling of sidelink If DCI format 3_0 or DCI format 3_1 is monitored on a cell, DCI size alignment for DCI format 3_0 and DCI format 3_1 is performed as described in this clause after performing the DCI size alignment described in Clause 7.3.1.0. The size(s) of the DCI formats configured to monitor for a cell and DCI formats for other purposes as described in 7.3.1.3 on the same cell if configured in this clause refers to that after performing the DCI size alignment described in Clause 7.3.1.0. If DCI format 3_0 or DCI format 3_1 is monitored on a cell and the total number of DCI sizes of the DCI formats configured to monitor for the cell, DCI formats for other purposes as described in 7.3.1.3 on the same cell if configured and DCI format 3_0 or DCI format 3_1 is more than 4, zeros shall be appended to DCI format 3_0 if configured and DCI format 3_1 if configured, until the payload size of DCI format 3_0 or DCI format 3_1 equals that of the smallest DCI format among the DCI formats configured to monitor for the cell and DCI formats for other purposes as described in 7.3.1.3 on the same cell if configured that is larger than DCI format 3_0 or DCI format 3_1. The UE is not expected to handle a configuration that results in: - the total number of different DCI sizes configured to monitor for the cell, DCI formats for other purposes as described in 7.3.1.3 on the same cell if configured and DCI format 3_0 or DCI format 3_1 is more than 4; and - the payload size of DCI format 3_0 or DCI format 3_1 is larger than the payload size of all other DCI formats configured to monitor for the cell and DCI formats for other purposes as described in 7.3.1.3 on the same cell if configured. ---------------------------------------------- End of Draft TP ---------------------------------------------

- HARQ

  - Multiple PSFCH pools

According to the previous discussion, gNB can configure multiple resource pools containing PSFCH, where PSFCH occasions of different resource pools can overlap on the same occasion. However, the current specification only specifies how the UE generates HARQ-ACK codebook for one pool, so it is unclear how to construct HARQ-ACK codebook when there are multiple resource pools containing PSFCHs in this case. A straightforward solution is to iterate through the resource pools containing PSFCHs in the order of their pool IDs, determining the HARQ-ACK bits corresponding to each pool separately based on the pseudo-code, and then concatenating these HARQ-ACK bits.

Proposal 3. Agree the TP#3 for HARQ reporting for multiple pools with PSFCH.

- TP#3 for 38.213: HARQ reporting for multiple pools with PSFCH

16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel For the set of slot timing values K1, the UE determines a set of MA occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions according to the following pseudo-code. Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions Set MA=∅ Set CK1 to the cardinality of set K1 Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1 Set NPSFCH_ResourcePool to the number of resource pool containing PSFCH in the set of resource pool provided by sl-TxPoolScheduling Set l=0– index of resource pool, in ascending order of the pool id provided by sl-ResourcePoolID of resource pool, in the set of resource pool containing PSFCH while l< NPSFCH_ResourcePool Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool l while k< CK1 if mod nU-K1,k+1,max2μUL-μSL,1=0 Set nS=0 – index of a SL slot within an UL slot while nS<max2μSL-μUL,1 if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell nS=nS+1; else if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and sl-PSFCH-Period, where K1,k is the k-th slot timing value in set K1 Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while end if nS=nS+1; end if end while end if k=k+1; end while l=l+1; end while

  - HARQ reporting on PUCCH for an ‘incomplete’ PSFCH period

Agreements: - Option B is agreed for the number of logical slots in a resource pool. - Option A: UE is not expected to be configured with a resource pool where the number of logical slots of the resource pool, counted within 10240 ms period, is not an integer multiple of the PSFCH resource period - Option B: It is clarified in the specification that a PSFCH slot is associated with up to N PSSCH slots where N is the PSFCH resource period. - Option 1 is agreed for the location of PSFCH slots where the PSFCH resource period is N. - Option 1: Logical slot index #0, #N, #2N, …. within 10240 ms period - Option 2: Logical slot index #N-1, #2N-1, #3N-1, … within 10240 ms period - Option 3: Logical slot index …, #M-2N, #M-N, #M within 10240 ms period, where logical slot #M is the last slot of a resource pool

It is was discussed in the previous meetings whether the number of SL slots of a resource pool in a DFN/SFN cycle must be multiple of PSFCH period NPSSCHPSFCH. The outcome of the discussion is that the number of SL slots of a resource pool in a DFN/SFN cycle may not always be multiple of NPSSCHPSFCH, which means there may be an ‘incomplete’ PSFCH period at the DFN/SFN cycle boundary.

An example of an ‘incomplete’ PSFCH period is shown below. With PSFCH period NPSSCHPSFCH and minimum PSSCH-PSFCH gap K, it can be derived that the time gap between a PSFCH and its corresponding candidate PSSCH occasions should be within the distance range [K, NPSSCHPSFCH+K-1]. However, when the number of SL slots in a 10240ms period is not a multiple of NPSSCHPSFCH, the interval between the last PSFCH in the previous cycle and the first PSFCH in the next cycle is less than NPSSCHPSFCH. Moreover, in this case, the number of PSSCH associated with the latter PSFCH at a distance of [K, NPSSCHPSFCH+K-1] is less than NPSSCHPSFCH.

Figure 3. Example of an incomplete PSFCH period

This ‘special’ PSFCH occasion that has fewer than NPSSCHPSFCH associated PSSCH occasions is usually the first PSFCH occasion in a 10240ms period. Donating the number of candidate PSSCH occasions associated with a PSFCH occasion by NPSSCH_actualPSFCH, as illustrated in Figure 3, the PSFCH occasion in red has only NPSSCH_actualPSFCH=1 associated PSSCH occasion while NPSSCHPSFCH=4.

According to the previous agreement, NPSSCHPSFCH HARQ-ACK bits should be reported for each PSFCH occasion involved in a type1 SL HARQ codebook based on the PSFCH detection result corresponding to each candidate PSSCH occasion. If a candidate PSSCH occasion that is not scheduled for SL, NACK will be reported.

TS 38.213 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel ==================Omitted================== Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while ==================Omitted================== If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions.

However, it is not clear what should be reported in response to the ‘special’ PSFCH occasion with fewer than NPSSCHPSFCH associated PSSCH occasion.

There are two options:

Option1: TX UE reports NPSSCHPSFCH bit HARQ-ACK for the ‘special’ PSFCH occasion (i.e., the same number of bits as for a normal PSFCH occasion), including NPSSCHPSFCH-NPSSCH_actualPSFCH bit(s) NACK and NPSSCHPSFCH HARQ-ACK bit(s) corresponding to the actual associated PSSCH occasions.

Option2: TX UE reports NPSSCH_actualPSFCH bit HARQ-ACK for the ‘special’ PSFCH occasion, which means UE only generates HARQ-ACK for the actual associated PSSCH occasions.

Figure 4. Example of option1 and option2

Option2 has the merit of reducing the codebook size but has the disadvantage that the type1 codebook size would dynamically change. To be specific, if the gNB allocates two K1 values, and one of them corresponds to a normal PSFCH occasion and the other one corresponds to a special PSFCH occasion, the TX UE will report NPSSCHPSFCH+NPSSCH_actualPSFCH HARQ-ACK bits on the corresponding PUCCH, while if both K1 values correspond to normal PSFCHs for another scheduled PUCCH, the TX UE shall feedback 2NPSSCHPSFCH HARQ-ACK bits. This dynamically changing codebook size will not only increase the complexity of gNB detection but also deviates from the original intent of introducing type1 codebook. In this late stage, further optimization is not preferred. Therefore, option1 is suggested.

Proposal 4. For a PSFCH occasion associated with NPSSCH_actualPSFCH PSSCH occasion, TX UE reports NPSSCHPSFCH bit HARQ-ACK for the special PSFCH occasion, including NPSSCHPSFCH-NPSSCH_actualPSFCH bit NACK and NPSSCH_actualPSFCH HARQ-ACK corresponding to the associated PSSCH occasion, where NPSSCHPSFCH is the periodicity of PSFCH and NPSSCH_actualPSFCH<NPSSCHPSFCH.

- TP#4 for 38.213: HARQ reporting option1

------------------------------------------- Start of Draft TP of 213 ------------------------------------- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for 0 ≤j<M, as described in Clause 16.5. If an occasion for candidate PSSCH transmission with index j with corresponding PSFCH reception occasions with index k is not associated with the PSFCH reception occasions with index k, the HARQ-ACK information bit for the occasion for candidate PSSCH transmission with index j with corresponding PSFCH reception occasions with index k is set to NACK. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions. ---------------------------------------------- End of Draft TP ---------------------------------------------

  - Power control of PUCCH for SL HARQ reporting

The transmission power of PUCCH on active UL BWP  of carrier  of primary cell is determined based on the following formula, where  is a PUCCH transmission power adjustment component which is depended on the PUCCH format as well as the number of UCI bits to be transmitted:

[dBm]

- For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where - - - is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided pdsch-HARQ-ACK-Codebook, if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, - is a number of SR information bits that the UE determines as described in Clause 9.2.5.1 - is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

If a PUCCH format2/3/4 is used for SL HARQ-ACK reporting and the number of HARQ-ACK bits is larger than 11, the current spec is not clear on how to determine the  for PUCCH power control. It seems to imply that other UL UCI (e.g., SR, CSI, etc.) are multiplexed with SL HARQ-ACK reporting (as highlight above), which is not aligned with the previous agreement. Some clarification is needed, i.e.,  and  should be set to 0 since multiplexing between SL HARQ-ACK and CSI/SR is not allowed, and  should be set to the number of the SL HARQ-ACK bits OACK determined in Clause 16.5.1 for type1 codebook or Clause 16.5.2 for type2 codebook.

Proposal 5. Adopt the TP#5 on PUCCH power control in 38.213.

- TP#5 for 38.213: TX power for PUCCH for SL HARQ-ACK reporting

------------------------------------------- Start of Draft TP of 213------------------------------------- 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as nHARQ-ACK=m=0M-1Nmreceived where Nmreceived is a number of HARQ-ACK information bits determined for corresponding PSSCH transmissions with corresponding PSFCH reception occasions in PSFCH reception occasion m. If OACK>11, and if the PUCCH transmission uses PUCCH format 2 or PUCCH format 3 or PUCCH format 4, the UE determines a transmission power for the PUCCH, as described in Clause 7.2.1, except that - =OACK. - =0 - =0 16.5.2.1 Type-2 HARQ-ACK codebook in physical uplink control channel If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as nHARQ-ACK=VSAI,mlastSL-USAImod4+m=0M-1Nmreceived+NCG where - VSAI,mlastSL is a value of a counter SAI field in a last DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions that the UE detects within the M PDCCH monitoring occasions - VSAI,mlastSL=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions in any of the M PDCCH monitoring occasions - USAI is a total number of DCI format 3_0, scheduling PSSCH transmissions associated with PSFCH reception occasions, that the UE detects within the M PDCCH monitoring occasions. USAI=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions in any of the M PDCCH monitoring occasions - Nmreceived is a number of DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions that the UE detects in PDCCH monitoring occasion m - NCG is a number of SL configured grants for which the UE transmits corresponding HARQ-ACK information in a same PUCCH as for HARQ-ACK information corresponding to PSFCH reception occasions within the M PDCCH monitoring occasions If OACK>11, and if the PUCCH transmission uses PUCCH format 2 or PUCCH format 3 or PUCCH format 4, the UE determines a transmission power for the PUCCH, as described in Clause 7.2.1, except that - =OACK. - =0 - =0 ------------------------------ -------------- End of Draft TP ---------------------------------------------

  - PUCCH resource for sidelink configured grant type 2

At RAN1#104-e meeting, RAN1 reached the following agreement and sent an LS to RAN2. RAN2 agreed to introduce a separate IE sl-N1PUCCH-AN-Type2 for PUCCH determination for sidelink configured grant type2 according to the LS reply[1].

Agreements:  Send an LS to RAN2 describing that:  Per RAN1 agreements, the parameter sl-N1PUCCH-AN-r16 should be used for SL CG Type 2 (only for PSCCH/PSSCH transmissions without a corresponding PDCCH).  In RAN1’s understanding, the parameter cannot be configured for SL CG type-2 and asking RAN2 whether they can provide a solution that would be compatible with the existing RAN1 agreements.

SL-ConfiguredGrantConfig field descriptions

sl-N1PUCCH-AN-Type2 This field indicates the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

In this case, for a sidelink configured grant type2, the corresponding PUCCH is determined based on sl-N1PUCCH-AN-Type2 if provided and k value for the DCI activating the configured grant. Thus, we propose the following TP according to RAN2’s updates:

Proposal 6. Agree the TP#6 for determining PUCCH for sidelink configured grant type2 without PDCCH.

- TP#6 for 38.213: PUCCH for sidelink configured grant type2

16.5 UE procedure for reporting HARQ-ACK on uplink With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Clause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-PSFCH-ToPUCCH-r16 for a transmission scheduled by a DCI format or for a SL configured grant type 2, or by sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant type 1. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the start of the sidelink frame is same as the start of the downlink frame [4, TS 38.211]. For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. For a SL configured grant Type 2 PSSCH transmission without PDCCH, if sl-PSFCH-ToPUCCH-CG-Type2 is provided by high layer, a PUCCH resource can be provided by sl-PSFCH-ToPUCCH-CG-Type2, otherwise, no PUCCH resource is provided. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1. In R15, nCI of a PDCCH candidate for self-scheduling is set to 0, while for cross-carrier scheduling, nCI is set to the value of the corresponding CIF field.

Observation 2. In R15, Ms,nCIL corresponding to a Uu DCI format in the search space scheduling a serving cell is determined based on the nrofCandidates of the serving cell corresponding to nCI.

Proposal 1. The value of nCI of PDCCH candidates for a SL DCI format is set to zero, and the number of PDCCH candidates associated with a SL DCI format blind decoding is set to the value of nrofCandidates corresponding to the scheduling cell.

Proposal 2. Adopt the TP#2 regarding DCI size budget for SL DCI alignment in TS 38.212.

Proposal 3. Agree the TP#3 for HARQ reporting for multiple pools with PSFCH.

Proposal 4. For a PSFCH occasion associated with NPSSCH_actualPSFCH PSSCH occasion, TX UE reports NPSSCHPSFCH bit HARQ-ACK for the special PSFCH occasion, including NPSSCHPSFCH-NPSSCH_actualPSFCH bit NACK and NPSSCH_actualPSFCH HARQ-ACK corresponding to the associated PSSCH occasion, where NPSSCHPSFCH is the periodicity of PSFCH and NPSSCH_actualPSFCH<NPSSCHPSFCH.

Proposal 5. Adopt the TP#5 on PUCCH power control in 38.213.

Proposal 6. Agree the TP#6 for determining PUCCH for sidelink configured grant type2 without PDCCH.

- Reference

- R2-2104463, Reply LS to RAN1 on SL HARQ-ACK reporting to the gNB.