3GPP TSG RAN WG1 #105-e		R1-2105465

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Maintenance on PDCCH enhancement

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we share our view on maintenance on PDCCH enhancement.

- Correction on UE PDSCH processing time for DCI format 1_2

In RAN1#104b, correction on UE PDSCH processing time for DCI format 1_2 are discussed. But no agreement is achieved.

In Rel-15, the different PDSCH processing time between DCI format 1_0 and 1_1 scheduling exists since UE always assumes additional DMRS with ‘pos2’ for a PDSCH with mapping type A scheduled by DCI format 1_0 while PDSCH with additional DMRS with ‘pos0’ can be scheduled by DCI format 1_1.

For Rel-15, for a single DCI scheduling different PDSCH length, the different processing time also exists due to the different d1,1 value. According to 38.214, the difference of d1,1 for PDSCH mapping type A scheduled by a DCI format could be up to 5 OS e.g., the last symbol of a 2-symbol PDSCH on the second symbol of a slot and a PDSCH scheduling with d1,1 =0, as shown in the following box,

5.3 UE PDSCH processing procedure time ……. - For the PDSCH mapping type A as given in clause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,1 = 7 - i, otherwise d1,1 = 0 …….

Secondly, for a UE supporting Capability 1, the N1 difference could be up to 6 OS for PDSCH scheduled by DCI formats 1_0 and 1_1, e.g. for PDSCH with l1=12 scheduled by DCI formats 1_0, N1,0=14 while for PDSCH configured with ‘dmrs-AdditionalPosition = pos0’ scheduled by DCI formats 1_1, N1=8.

5.3 UE PDSCH processing procedure time …… - If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1, otherwise N1,0=13.

Therefore, the maximum difference could be 5+6 =11 OS for PDSCH scheduling for Rel-15. Thus, the same mechanism should be maintained as in Rel-15, i.e., the different PDSCH processing time can be defined for DCI format 1_0 and 1_x.

On the other hand, we think the same PDSCH processing time should be maintained for DCI format 1_1 and 1_2 considering implementation complexity perspective. Therefore, the following TP are suggested.

---------------------------------Start of Text Proposal on TS 38.214 ----------------------- 5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. “dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured”; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if none of the higher layer parameters is configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingType, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. 0; 3. 1; 4.5. 2; 9 for frequency range 1. ------------------------------------------End of Text Proposal -----------------------------------

Proposal 1: To correct the UE PDSCH processing time with the text proposal.

- Correction/clarification on new SLIV reference for Type 1 HARQ codebook

In RAN1#104b, new SLIV reference for Type 1 HARQ codebook is discussed and no agreement is achieved. Two interpretations are raised as following.

- Interpretation 1: The extended SLIV applies in every slot (i.e. even for the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0). Interpretation 1 ensures the same number of the set of row indexes of the TDRA tables for type-1 codebook construction for every slot. - Interpretation 2: The extended SLIV is only applied to the slot(s) with PDCCH monitoring occasion with starting symbol S0>0. The candidate PDSCHs in a slot can be impacted by both PDCCH Mos and the number of PDSCH repetitions as discussed in Samsung paper.

As Rel-15 specification described, Type 1 codebook includes HARQ-ACK bits for each PDSCH repetition unless all repetitions collide with UL symbols. From this point, HARQ-ACK corresponding to PDSCH repetitions across the slots with the SLIV reference should also be included in Type 1 codebook.

For Interpretation 1, Only some restrictions can be provided for clarification. On the contrary, For Interpretation 2, further solutions are needed. From simplicity perspective, we suggest to add a conclusion to fix this issue, i.e., if a UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.

Proposal 2: If a UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.

- Conclusion

In the contribution, we discuss remaining issues on PDCCH and propose that,

Proposal 1: To correct the UE PDSCH processing time with the text proposal.

---------------------------------Start of Text Proposal on TS 38.214 ----------------------- 5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. “dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured”; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if none of the higher layer parameters is configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingType, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. 0; 3. 1; 4.5. 2; 9 for frequency range 1. ------------------------------------------End of Text Proposal -----------------------------------

Proposal 2: If a UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.

References

- Chairman notes RAN1#104b e-meeting.