3GPP TSG RAN WG1 #105-e		                   R1-2105480

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.4.4

Source:	LG Electronics

Title:	Discussions on other aspects of NTN

Document for:	   Discussion and Decision

- Introduction

In RAN1#104-e meeting, following agreement and conclusions were made as captured below:

Agreement:

Support at least explicit indication of polarization information for DL by the network

- FFS: whether the indication is done by SIB, other RRC signaling, DCI.

- FFS: Whether separate signaling is needed for the UL and if so, whether or not a same polarization is indicated for DL and UL

Conclusion:

Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management:

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

Conclusion:

Discuss the necessity of reporting UE polarization capability considering at least following aspects,

- Deployment scenarios.

- UE implementation aspects with respect to polarization.

- Satellite implementation aspects for switching between polarization states.

- Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

In this contribution, we discuss on other aspects including PRACH, polarization signaling and Bm/BWP operation in NTN.

- Discussion

Polarization mode signaling

Inter-cell interference mitigation is important issue not only for NTN but also for TN. This interference mitigation can be achieved by configuring different polarization from neighboring cells. By applying polarization domain in addition to frequency domain, network can be more flexible on configuring resources, and therefore it is expected to achieve higher spectral efficiency or higher network throughput. In this regard, in RAN1#104-e meeting, polarization information is agreed to be supported in an explicit manner. There are two remaining issues: 1) which signaling is used for polarization information indication, and 2) whether separate signaling is needed for the UL. Regarding issue 1), in case of dynamical polarization configuration, the huge cell signaling overhead is expected which is not desirable for NTN. Also, use case for dynamic polarization configuration is not clear. Therefore, it seems sufficient that polarization mode (RHCP, LHCP) is signaled in semi-static way, and SIB can be a good candidate for polarization signaling. Regarding issue 2), it is depends on the UE polarization capability. If the UE is equipped with dual circular polarization, UL polarization signaling can be beneficial in terms of network scheduling perspective. Alternatively, UE can assume the same polarization for DL and UL, and therefore UL polarization is not needed. In case of a single circular polarization or a linear polarization, UL polarization signaling does not provide benefit. Thus, we think DL polarization indication seems enough unless significant benefit for separate polarization signaling for DL and UL is proved.

Observation 1. DL polarization indication seems sufficient.

Proposal 1. Indication of polarization mode (RHCP, LHCP) can be broadcasted via SIB.

According to the UE type (e.g., Handheld, VSAT, ESIM, etc.), the polarization capability (RHCP, LHCP, linear) can be varied. As mentioned above, at least for UEs with single circularly polarized antenna or linear antenna, the reporting of its capability does not provide benefits. It is expected some network scheduling flexibility in case of UE with dual circularly polarization, it polarization capability is reported by UE. However, without polarization capability reporting, UE can also properly work according to polarization indication from network. Thus, before agree on the UE polarization capability reporting, clear benefit should be firstly shown.

Observation 2. Without UE polarization capability reporting, UE can properly work.

BM/BWP operation in NTN

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

In the RAN1#104-e meeting, conclusion that includes several potential issues on BM and BWP operation in NTN was made. In this subsection, we discuss on these issues captured below.

In issue 1, some new feature such as beam-specific BWP is proposed, and it may lead large specification impact. The motivation of fixed linkage between beam and BWP is not strong. If the motivation of beam specific BWP is fast beam switching, the same functionality can be done by currently specified beam management. Issue 2 considers joint triggering of UL and DL BWP switching. It seems the motivation of issue 2 is some latency optimization. However, interference situation and traffic load for DL and UL are quite different, so that the joint triggering of UL and DL BWP switching is not necessary. Regarding issue 3, it is questionable that why do we trigger the BWP switching if there is no plan for data scheduling. Issue 4 considers re-synchronization when BWP is switched. In current NR specification, there is no re-synchronization procedure when BWP is switched. Thus, it should be firstly investigate the need for re-synchronization especially for NTN. In issue 5, prediction based BWP switching is considered. Since UE can be provided ephemeris information by gNB, it could be possible prediction based BWP switching. However, current DCI based BWP switching seems flexible enough to do the same functionality. In addition, in order to ensure prediction based BWP switching, gNB and UE should have the same understanding of switching timing. In issue 6, current statement seems not clear on what is the issue and it could be up to network configuration. Lastly, issue 7 deals with group based BWP/BM in order to reduce the signaling overhead in NTN. Since the # of UEs in NTN cell is relatively larger than TN due to its larger cell size, introducing group-based BM and/or BWP switching seems beneficial. In order to properly perform group-based BM, group-wise beam measurement and reporting can also be considered.

Observation 3. Most of listed issues can be handled by current BM and/or BWP operation. Group-based BM and BWP operation can be further studied for signaling overhead saving.

In NR, BWP switching is based on the DCI, MAC-CE or inactivity timer (bwp-InactivityTimer). For example of timer based operation as shown in Figure 1, when UE receives PDCCH scheduling DL or UL, the inactivity timer (re)starts. If there is no additional scheduling command before the expiration of timer, UE will perform BWP switching to initial BWP or default BWP. This is the way to resolve ambiguity when gNB and UE have different understanding on BWP switching. If a UE misses PDCCH carrying BWP switching indication, gNB can notice this situation at least after round-trip time + T_timer from the time of a PDCCH transmission. Thus, as RTT increase, this issue becomes non-negligible.

Figure 1. Illustration of timer based BWP switching

In NTN, the maximum RTT can be up to 21ms, 42ms and 542ms for LEO-600km, LEO-1200km and GEO, respectively. Hence, the careful BWP switching needs to be considered in order not to waste scheduling resources. To this end, potential enhancements on BWP switching can consider at least the following aspects:

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing.

- PDSCH and PUSCH transmission after ACK transmission of BWP switching command.

Proposal 2. For NTN, potential enhancement on BWP switching can consider at least following aspects:

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing,

- PDSCH transmission after transmission of ACK for BWP switching command.

Enhancement on PRACH

In general, GNSS-equipped UE can obtain its location and reference time through GNSS. In this case, existing Rel-15 PRACH formats and preamble sequences can be reused. However, if GNSS-equipped UE is in a particular situations (e.g., indoor, underground, etc.), it may be hard to obtain the accurate location and the reference time because the accuracy of the GNSS may be reduced. In this situation, existing Rel-15 PRACH formats and preamble sequences cannot be reused. Therefore, following four options can be considered for the enhanced PRACH formats and/or preamble sequences in TR38.821 [1].

- Option 1: A single Zadoff-Chu sequence based on larger SCS, repetition number.

- Option 2: A solution based on multiple Zadoff-Chu sequences with different roots.

- Option 3: Gold/m-sequence as preamble sequence with additional process, e.g., modulation and transform precoding.

- Option 4: A single Zadoff-Chu sequence with combination of scrambling sequence.

Regarding this issue, it is necessary to discuss further in Rel-17 NTN WI phase. If it is agreed that the enhanced PRACH formats and/or preamble sequences are necessary, and if one of the above options should be supported, the Option 1 is preferred over other options due to the minimum impact on specification considering the limited time budget in Rel-17 NTN.

Proposal 3. If enhanced PRACH formats and/or preamble sequences are necessary and supported in Rel-17 NTN, the option with simple modification, such as a single Zadoff-Chu sequence based on larger SCS and repetition number, is preferred.

- Conclusion

In this contribution, we discussed on the enhancement on PRACH, polarization signaling, and BWP operation in NTN. Based on the above discussion, we have following observations and proposals:

Observation 1. DL polarization indication seems sufficient.

Observation 2. Without UE polarization capability reporting, UE can properly work.

Observation 3. Most of listed issues can be handled by current BM and/or BWP operation. Group-based BM and BWP operation can be further studied for signaling overhead saving.

Proposal 1. Indication of polarization mode (RHCP, LHCP) can be broadcasted via SIB.

Proposal 2. For NTN, potential enhancement on BWP switching can consider at least following aspects:

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing,

- PDSCH transmission after transmission of ACK for BWP switching command.

Proposal 3. If enhanced PRACH formats and/or preamble sequences are necessary and supported in Rel-17 NTN, the option with simple modification, such as a single Zadoff-Chu sequence based on larger SCS and repetition number, is preferred.

Reference

[1] 3GPP TR 38.821 V1.0.0 (Release 16)