3GPP TSG RAN WG1 #105-e	R1-2105488

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.8.1.1

Source: 	LG Electronics

Title: 	Discussions on PUSCH repetition type A enhancements

Document for:	Discussion and decision

Introduction

In RAN1#104-e meeting, enhancement of PUSCH repetition type A were discussed including determination rule for the available slots and maximum number of repetitions. Also, several agreements and conclusion were made [1]. From the previous agreement, we continue to discuss on enhancement of PUSCH repetition type A.

Discussion

Determination rule for the available slots

In RAN1#104-e meeting, determination rule for the available slots was discussed. It was concluded to discuss further to select one of the alternatives.

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. - Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

The discussion point is whether to the determination of all available slots has to be done prior to the first actual transmission of repetitions. In case of Alt-a, if slot(s) which is/are determined as available slot prior first transmission is not allowable for UL transmission due to dynamic signaling, the UL transmission could be dropped. In this case, it is not hard to achieve all of expected number of repetition. Nevertheless, a benefit of ‘Alt-a’ is that the overall time duration for PUSCH repetition is kept. On the other hand, in case of ‘Alt-b’, if slot(s) which is/are expected as available slot prior first transmission is not allowable due to dynamic signaling, UL transmission could be postponed. In this case, it seems that overall time duration for single TB transmission is increased. It could be a tradeoff between spectral efficiency and coverage enhancement. If the design principle for PUCCH repetitions in Rel-16 is followed for PUSCH repetition in Rel-17 CE WI, ‘Alt-a’ which has a benefit to keep the overall time duration for PUSCH repetition can be selected.

Proposal 1: Select Alt-a (The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.).

Also, it was agreed to select one of alternatives for determination prior to the first actual transmission of the repetition.

Agreements: Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded) - Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC). - Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Main difference between Alt1 and Alt2 is whether to consider dynamic signaling for determination of the available slots. This discussion is related with above conclusion. Hence, as the same reason to select ‘Alt-a’ in the conclusion, we propose to select Alt1.

Proposal 2: Select Alt1 (‘Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC)’) for the determination of all the available slots prior to the first actual transmission of the repetitions

Since RRC configuration (at least tdd_ul_dl configuration) is listed in both Alt1 and Alt2, it is agreeable that both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are used for determining available slot. In addition, it is allowable that ssb-PositionInButst for SS/PBCH block is used for determination, which follows the determination rule of the available slots for PUCCH repetitions.

Proposal 3: For determining available slot, support to use both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. In addition, support to use ssb-PositionInButst for SS/PBCH block.

The maximum number of repetitions

In RAN1#104-e meeting, the maximum number of repetitions was discussion. And following agreements were made [1].

Agreements: The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH. Agreements: Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI. - FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

According to the previous agreement for the increase of maximum number of repetitions with repetition factors configured in a TDRA, entries in following parameter can be increased.

- numberofrepetitions-r16 = {1, 2, 3, 4, 7, 12, 16}

- The number of repetitions for DCI format 0_1/0_2 in PUSCH-TimeDomainResourceAllocationNew information element

Even if the maximum number of repetition is increased, it is unnecessary to increase the range of repetition for dynamic indication. In this aspect, it is proposed that the number of rows of the TDRA table should remain unchanged from Rel-16.

Proposal 4: The number of rows of the TDRA table should remain unchanged from Rel-16.

Furthermore, it needs to discuss increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig. Since it was agreed that the maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH, following two parameters can be possibly extended to support the increase of maximum number of repetitions with repetition factors.

- pusch-AggregationFactor = {2, 4, 8}

  - The number of repetitions for data in PUSCH-Config information element

- repK = {1, 2, 4, 8}

  - The number of repetitions of K in ConfiguredGrantConfig information element

Proposal 5: Increase the maximum number of repetitions for pusch-AggregationFactor and repK.

Conclusion

In this contribution, we discussed on enhancement of PUSCH repetition type A. From the discussion, we provide following proposals.

Determination rule for the available slots

Proposal 1: Select Alt-a (The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.).

Proposal 2: Select Alt1 (‘Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC)’) for the determination of all the available slots prior to the first actual transmission of the repetitions

Proposal 3: For determining available slot, support to use both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. In addition, support to use ssb-PositionInButst for SS/PBCH block.

The maximum number of repetitions

Proposal 4: The number of rows of the TDRA table should remain unchanged from Rel-16.

Proposal 5: Increase the maximum number of repetitions for pusch-AggregationFactor and repK.

Reference

- RAN1 chairman’s notes, RAN1 #104-e, e-Meeting, January 25th – February 5th, 2021