3GPP TSG RAN WG1 #105-e		R1-2105510

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.8.1.2 TB Processing over Multi-Slot PUSCH

Source: 		Sierra Wireless

Title: 		Design Considerations for TB Processing over Multi-Slot PUSCH

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for the following coverage enhancement objectives in the WID [2]:

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots

Agreements from previous meetings:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

- The same number of PRBs per symbol is allocated across slots for TBoMS transmission.

- Consecutive physical slots for UL transmission can be used for TBoMS for paired spectrum and the SUL band

  - FFS if non-consecutive physical slots for UL transmission are also supported for paired spectrum and the SUL band

For TBoMS, the maximum supported TBS should not exceed legacy maximum supported TBS in Rel-15/16, for the same number of layers.

- FFS: Details and further constraints on the applicability of TBoMS.

One or two of the following approaches will be considered as a starting point to decide howNinfo NInfo for TBoMS is calculated (aiming for down selection in RAN1 #104-bis-e):

- Approach 1: Based on all REs determined across the symbols or slots (FFS whether symbols or slots are used) over which the TBoMS transmission is allocated

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K

Note: L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

One or two of the following options will be considered (aiming for down-selection in RAN1#104b-e) to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols or slots (FFS whether symbol or slot are used) over which the TBoMS transmission is allocated.

  - FFS: if either the number of symbols or the number of slots is used.

  - FFS: if xOverhead is separately configured from the one in Rel-15/16.

Non-consecutive physical slots for UL transmission can be used to transmit TBoMS at least for unpaired spectrum.

- How TBoMS is transmitted over non-consecutive physical slots for UL transmission for unpaired spectrum is to be discussed further.

- Whether and how non-consecutive physical slots for UL transmission can be used to transmit TBoMS for paired spectrum and SUL band as well, is to be discussed further.

Working Assumption

The concept of transmission occasion for TBoMS (TOT) is utilized for the purpose of discussion, where a TOT is constituted of time domain resources which may or may not span multiple slots

- FFS: details, whether multiple slots which constitute a TOT are consecutive or non-consecutive physical slots for UL transmissions

- FFS: other details.

- FFS: whether such concept will be specified or not.

For the definition of a single TBoMS, down select among the following options:

- Option 1: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using a single RV.

  - FFS: whether and how the single RV is rate matched across the TOT, e.g., continuous rate-matching across the TOT, rate matched for each slot and so on.

- Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

  - FFS: how RV index is refreshed within the TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

- Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

  - FFS: whether and how RV index is refreshed within one TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- FFS: the exact TBS determination procedure.

- FFS: whether a single TBoMS can be repeated or not.

- FFS: other implications, e.g., power control, collision handling and so on.

# Multi-Slot Encoding

A common solution to achieve a lower code rate for a multi-slot transmission is to encode transport blocks over multiple slots. This is especially useful for a larger TBS into smaller number of PRBs. For example, in [1] the increased code rate going from TBS=888 with 8 repeats to TBS=1800 with 16 repeats led to a decline in performance. To determine the performance of multi-slot encoding, LLS simulations were conducted with different TBS, number of repetitions, and multi-slot encoding options. For a fair comparison, all results have a data rate of 100kbps. Full simulation assumptions this paper can be found in Appendix A.

Figure 1: LLS Results of Multi-Slot Encoding vs. no Multi-Slot Encoding at Fdop = 2Hz

As the graph clearly shows, there is no coverage gain between TBS=888 encoded over 1 slot and TBS=1780 encoded over >=2 slots at low mobility. The gain between the black line (no multi-slot encoding, TBS 1780) and Green and Red lines (with multi-slot encoding) only occurs because the code rate improved but improving code rate can also be done by increasing the number of PRBs used for encoding (e.g. from 4 to 8). Although there is no coverage improvement using TBoMS, the amount of spectral resources needed using TBoMS was 1/2 vs normal repetition.

TBoMS provides no coverage gain over repeats but provides increased spectral efficiency.

# Relationship between TBoMS and RV Repetitions Cycling

This relationship between TBoMS and PUSCH repetitions including RV cycling is a fundamental question. The FL has outlined two basic paths:

ALT 1: TBoMS is an enhancement to repetition where RV cycling of repeats is re-used.

ALT 2: TBoMS is a new feature where RV cycling of repeats is not used.

This led to the following agreement being made:

For the definition of a single TBoMS, down select among the following options:

- Option 1: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using a single RV.

  - FFS: whether and how the single RV is rate matched across the TOT, e.g., continuous rate-matching across the TOT, rate matched for each slot and so on.

- Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

  - FFS: how RV index is refreshed within the TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

- Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

  - FFS: whether and how RV index is refreshed within one TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- FFS: the exact TBS determination procedure.

- FFS: whether a single TBoMS can be repeated or not.

- FFS: other implications, e.g., power control, collision handling and so on.

Discussion of options:

Spectral Efficiency:

As specified earlier, the main objective of TBoMS is to improve the code rate so that large TB can be sent in narrow bandwidths which improves spectral efficiency in coverage-limited cases. All options can equally fulfill this requirement.

Coverage Enhancement:

To support coverage enhancement, UL transmission times need to be around 8 slots to support eMBB coverage and 24 slots for TDD VoIP. Since options 2 & 4 support repeats, option 2 or 4 would only need to support TBoMS over ~2 slots to provide adequate coverage. Since options 1 & 3 do not support repeats, option 1 & 3 would need to support TBoMS over a very large number of slots (e.g. 24 slots).

Specification and commercialization impact:

Options 1 & 3 have more complexity WRT to UCI multiplexing, special slot handling, and higher priority transmission interruption. If option 2 or 4 is chosen, the TBS determination procedure can follow legacy repetition procedures.

Given the above discussion and that commercialization impact is lowest with option 2 or 4, the following proposal is made:

TBoMS is an enhancement to repetition where RV cycling of repeats is re-used (i.e. Option 2 or 4 is chosen) where:

Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

- WRT to this:

- FFS: the exact TBS determination procedure.

If option 2 or 4 is chosen, the TBS determination procedure can follow legacy repetition procedures.

TBS determination procedure can follow legacy repetition procedures when option 2 or 4 is chosen.

# TDRA Considerations

The following agreement was made last meeting WRT TDRA design:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

PUSCH repetition type B like TDRA is mainly for the URLLC use case, not the eMBB and VoIP use cases, and PUSCH repetition type A like TDRA is simpler to commercialize. Thus, type A is preferred.

The starting point to design the time domain resource determination for TBoMS is:

PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

# Conclusions

- TBoMS provides no coverage gain over repeats but provides increased spectral efficiency.

- TBoMS is an enhancement to repetition where RV cycling of repeats is re-used (i.e. Option 2 or 4 is chosen) where:

Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

TBS determination procedure can follow legacy repetition procedures when option 2 or 4 is chosen.

The starting point to design the time domain resource determination for TBoMS is:

PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

# References

R1-2007930 Potential techniques for NR coverage enhancement, Sierra Wireless

RP-202928 China Telecom, “New WID on NR coverage enhancements”

# Appendix A: LLS Simulation Assumptions:

| Parameter | Value |
| --- | --- |
| Bandwidth Part | 20 MHz |
| Configuration | FDD |
| Carrier frequency | 700 MHz |
| RX Antenna ports | 2 |
| Channel model | TDL-C 300 ns |
| Number of PRBs | 4 |
| UE Speed | 3, 30, 100 km/hr As specified |
| Carrier Frequency Offset | None |
| Channel Estimation | Perfect |
| Frequency Hopping | Disabled |
| Frequency Averaging Window | 3 PRBs |
| DMRS Configuration | 2 DMRS per slot |
| Transform Pre-coding | Yes |
| SCS | 15 kHz |
| Modulation | QPSK |
| TBS | As specified |
| Repeats | As specified |
| HARQ | Disabled |
| iBLER Target | 10% |
| Gaps | 0, 9, 15, 31 ms As specified |
| Multi- Slots encoding | 1,2,4 As specified |