3GPP TSG RAN WG1 #105-e	R1- 2105512

e-Meeting, May 10th – 27th, 2021

Agenda item:		8.5.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Views on mitigating UE and gNB Rx/Tx timing errors

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#90 a new WI was approved and at RAN#91 that WID was updated on enhancements in Rel-17 to positioning [1]. This contribution discusses our views on the enhancements related to UE and gNB Rx/Tx timing errors. Our companion contributions discuss our others views [2-6]. The objective from the WID is to

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

During RAN1#104-e the following agreement was reached:

Agreement:

The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications):

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

- Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

- UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

- TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

# Discussion

## UE Antenna Array Phase Center Offset Errors

With reference to WID [1] one of the main objectives for the Rel-17 work on positioning enhancements is to improve the accuracy of the positioning methods. Especially for IIoT use cases the positioning accuracy target is challenging at < 20 cm and UE implementation impairments which may so far have been considered neglectable (i.e., 3 or 10 m accuracy target of Rel-16), and therefore left uncompensated, now become significant contributors to the overall positioning error.

Figure 1 depicts a simplified UE formfactor design with a 1:8 mmWave antenna array mounted at the top receiving or transmitting a positioning reference signal at a given angle. For time based positioning accuracy, e.g. UL/DL-TDOA or multi-RTT, at the UE, it is key to have a precise measure of the effective phase reference position for the signal received or transmitted via the UE mmWave antenna array. We name this position the antenna array phase center and ideally this antenna array phase center is fixed and aligned with e.g. the physical Antenna Reference Point (ARP).

However, the UE antenna array phase center may not always align with the physical Antenna Reference Point (ARP) and its position may in fact be highly sensitive to multiple parameters of which some will vary during live operation in the field. The UE antenna array phase center location is for example sensitive to UE formfactor, cover, coating, beam steering angle, polarization and angle of arrival/departure both for narrow and broad beam antenna array configurations.

As depicted in Figure 1 if, for a given beam steering angle and angle of arrival/departure, the antenna phase center is located at an offset compared to the assumed location (e.g. the ARP), the impact is an error on the estimated TOA/TOD. This TOA/TOD error can be several centimeters and therefore a significant contributor in the error budget for IIoT use cases if left uncompensated.

Figure 1: Dynamic antenna array phase center scenario for example 1:8 UE form-factor design

To show the effect CST EM simulations have been carried out on the UE formfactor design depicted in Figure 1 for the top mounted 1:8 antenna array with front and rear glas covers. The simulations of the antenna array phase center position with reference to the ARP have been conducted for different beam steering angles, for different signal angle of arrival (AOA) for each beam steering angle and for both co- and cross polarization. The phase center position has been evaluated in the direction of the incoming signal over an angular area of 12°x 12°.

The radiation phase of the UE formfactor with the 1:8 antenna array mounted at the top in broad beam configuration (single activated element) is depicted in Figure 2 for both polarizations (co polarization and cross polarization). This significant phase variation behavior manifest itself in sizable phase center offset variation over AOA and polarization as shown in Figure 3. The phase center offset variation is several centimeters and is highly dependent on both AOA and polarization.

Figure 2: Antenna array broad beam (single patch) configuration phase plots over AOA and polarization

Figure 3: Antenna array broad beam configuration phase center offset in [mm] ref. ARP over AOA and polarization. Each colour represent offset in the x-y-z direction as indicated in Figure 1.

Observation 1: For both narrow and broad beam configurations dynamic UE antenna array phase center offset may be a sizable contributor to TOA/TOD errors and thereby to the inaccuracy of the UE positioning estimation especially important for high accuracy IIoT use cases.

Observation 2: These offsets can be seen also as timing delays that are dependent on the formfactor of the device, antenna panel used as well as the beam configuration and the particular AoA/AoD.

Assuming that the UE is aware of the phase center offset (PCO) characteristics of all its antennas/antenna arrays over configurations and conditions for both DL receptions and UL transmissions and that the UE has obtained an estimate of the DL PRS AOA for a given gNB1 (AOA_gNB1).

In this scenario the UE may locally compensate the DL PRS TOA offset introduced by the receiving antenna PCO in the same way as compensating for other UE local RX timing errors.

Observation 3: The UE may know PRS AOA and the antenna phase center offset in that direction and may thus locally compensate for the associated PRS TOA offset.

Note that the PRS AOA can be a relative AOA value, e.g., relative to the receiving antenna, as it is only used locally at the UE side. Here PRS is an example but other reference signals could also be used by the UE (e.g., SSB).

By channel reciprocity the UE may assume that any subsequent UL SRS-Pos towards gNB1 is transmitted in the same direction and the UE may therefore compensate the UL SRS-Pos Time Of Departure (TOD) by the transmitting antenna PCO in the direction dictated by AOA_gNB1.

Since the PCO is transmission direction dependent this UE UL local compensation of the SRS-Pos TOD is feasible for scenarios where only the target gNB can hear the transmission or for gNB specific UL SRS-Pos transmissions conformed by decoding.

This is visualized in Figure 4 where the UE is transmitting SRS-Pos towards gNB1, 2 and 3. For narrow beam configuration (green beams) each SRS-Pos transmission towards gNB1/2/3 may be TOD offset compensated at the UE ref. the PCO in that direction (PCO1/2/3).

Observation 4: For the case of UE SRS-Pos transmission towards gNBs located in one direction the UE may know the SRS-Pos AOD and the antenna phase center offset in that direction and may thus locally compensate for the associated SRS-Pos TOD offset.

Figure 4: Example of UE with direction dependent PCO transmitting UL SRS-Pos towards gNBs with broad beams (orange) or narrow beams (green).

For for broad beam configuration (orange beams) two gNBs (gNB2 & 3) are receiving the same SRS-Pos transmission and since UE PCO2 and PCO3 are different it is not possible to apply a single accurate TOD offset at the UE transmitting side.

For such cases with more gNBs receiving a UE SRS-Pos transmission the PCO compensation will need to happen at the receiving side by compensating the UL SRS-Pos TOA for at each receiving gNB. To enable this the UE may report the TOD offset for each gNB to LMF which will then apply these offsets when calculating the UE positioning.

Observation 5: For the case of UE SRS-Pos transmission towards multiple gNBs in different directions the UE may know the SRS-Pos AODs and the antenna phase center offsets in those directions and may report the associated SRS-Pos TOD offsets to the LMF for compensation.

Proposal 1: UE to include reporting of gNB specific SRS-Pos TOD offsets to gNB/LMF for post-compensation of direction specific UE antenna phase center offsets thereby enhancing the positioning accuracy.

Proposal 2: UE to signal to gNB/LMF its capabiltiy to compensate for antenna phase center offsets for time based positioning. Note this could apply to both broad beam and narrow beam SRS-Pos transmissions.

In context of the UE TEG definition it is worth noting that the UE antenna PCO is adding a direction of transmission/reception aspect to the RX/TX timing error. Consequently, if the PCO impact on the timing error is not included then that may invalidate small “certain margin” TEG’s, at least in some directions.

Proposal 3: Include the impact of antenna PCO in the definition of RX/TX timing errors and associated TEGs .

## Reference Device

One component of the Rx/Tx timing delays is the group delay which may be partially calibrated in an offline manner but also contains a time varying component (e.g., due to temparture). One potential solution to mitigate the time varying part of the gNB Rx/Tx timing delays is to rely on reference devices, with known location, to calibrate for the variations in group delay on the transmitter side.

We discuss the addition of reference devices in detail in our Tdoc on DL-AoD [3]. The main message is that the current specification are not sufficient for enabling a reference device.

Proposal 4: RAN1 to specific support for enabling a selected device with known location to support configuration by the network for at least some positioning calibration measurements.

There is not sufficient specification support for such capabilities as highlighted above and in addition the current specifications would lead to high overhead and complex signaling that would not be optimal. For example, if a UE using GNSS is configured to report the location it estimates via GNSS, the LMF has little ability to control the timing of the report and the relation to other measurement reports (e.g., PRS-RSRP measurements). The current protocols would require multiple LPP procedures to run concurrently in order to facilitate this type of reporting. Which is highly inefficient and the signaling was not designed for such reference UEs. UEs which have fixed locations are also not currently supported and would need a method to report their locaitons. By not introducing direct specification support for reference devices we also limit the forward looking potential of such devices. To meet the very challenging accuracy requirements in Rel-17 it is needed to introduce reference device support.

Observation 6: The current specifications are not sufficient for enabling a reference device.

Observation 7: UEs with fixed location currently have no ability to report their location to the LMF.

Observation 8: Reference devices could have the ability for additional positioning measurement reporting capabilities.

## TEG Handling

At RAN1#104-bis the following agreements were reached on how to handle TEGs.

Agreement:

- Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA

  - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs

  - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs

  - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs

  - FFS: the details of the signalling, procedures, and UE capability

- Send an LS to RAN4 to check if there is any issue to support the above enhancements

Agreement:

Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA

- Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs

- Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF;

- FFS: the details of the Signaling, procedures, and UE capability

Agreement:

For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives:

- Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS;

- Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to the one of the 2 following options:

  - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs

    - FFS:  whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically

  - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS.

- For both alterntives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF

  - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF

- FFS: the details of the signalling, procedures, and UE capability

Agreement:

For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives:

- Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF

- Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options:

  - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs

    - FFS:  whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically

  - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS.

- For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs.

- FFS: the details of the signalling, procedures

The major open point in the above agreement is the decision and if the introduction of RxTxTEG is needed or not. In our understanding it may be useful for a UE/TRP to inform the network that the combined Tx and Rx errors of multiple Rx-Tx mesaurements are within a certain margin.

Observation 9: Reporting of RxTx TEG is useful for positioning.

The key point of reporting TEG association information is for the LMF to make smart decision when processing positioning measurements. Knowing the resources used for those measurements is clearly beneficial for the LMF as it does the processing. However we fail to see the benefit of having PRS association with Rx TEG for multi-RTT.

Proposal 5: Support Alt. 2, Option 1 in the prior agreement on UE Rx-Tx time difference measurements.

Propsoal 6: Don’t support UE providing association of PRS resources and Rx TEG to LMF for UE Rx-Tx measurements.

Similarly for gNB Rx-Tx time difference measurements we support the following:

Propsoal 7: Support Alt. 2, Option 1 in the prior agreement on gNB Rx-Tx time difference measurements.

Propsoal 8: Don’t support TRP reporting the association information of SRS resource to TRP Rx TEG for gNB Rx-Tx measurements.

## Measurement Timing Window

The below agreement was reached at RAN1#104.

Agreement:

Support enabling

- A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and

- A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and

- Each measurement instance is reported with its own timestamp

  - FFS: The measurement instances are within a [configured] measurement time window

- FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set

  - FFS: N (including N=1)

- FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions

  - FFS: M (including M=1)

- FFS: details of signalling, procedures, and UE capability if any

- FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric

- Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

- Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

Configuration measurement window to align measurement time between the UE and the TRP/gNB could be helpful to attain round trip time more accurately. However, it is not clear how much accuracy can be improved, and furthermore it is highly likely to restrict the gNB implementation. As an indirect way, we suggest the UE provides the gNB with its measurement window so that the gNB can consider it. In consideration of the UE mobility, UE’s suggestion on the measurement time window could be better than the suggestion by network. This way could be simplified solution without any impact on the current LPP and NRPPa.

Proposal 9: UE to provide gNB its measurement time window for UE Rx-Tx time difference measurement.

# Conclusion

We made the following observations and proposals in this paper:

Observation 1: For both narrow and broad beam configurations dynamic UE antenna array phase center offset may be a sizable contributor to TOA/TOD errors and thereby to the inaccuracy of the UE positioning estimation especially important for high accuracy IIoT use cases.

Observation 2: These offsets can be seen also as timing delays that are dependent on the formfactor of the device, antenna panel used as well as the beam configuration and the particular AoA/AoD.

Observation 3: The UE may know PRS AOA and the antenna phase center offset in that direction and may thus locally compensate for the associated PRS TOA offset.

Observation 4: For the case of UE SRS-Pos transmission towards gNBs located in one direction the UE may know the SRS-Pos AOD and the antenna phase center offset in that direction and may thus locally compensate for the associated SRS-Pos TOD offset.

Observation 5: For the case of UE SRS-Pos transmission towards multiple gNBs in different directions the UE may know the SRS-Pos AODs and the antenna phase center offsets in those directions and may report the associated SRS-Pos TOD offsets to the LMF for compensation.

Proposal 1: UE to include reporting of gNB specific SRS-Pos TOD offsets to gNB/LMF for post-compensation of direction specific UE antenna phase center offsets thereby enhancing the positioning accuracy.

Proposal 2: UE to signal to gNB/LMF its capabiltiy to compensate for antenna phase center offsets for time based positioning. Note this could apply to both broad beam and narrow beam SRS-Pos transmissions.

Proposal 3: Include the impact of antenna PCO in the definition of RX/TX timing errors and associated TEGs .

Proposal 4: RAN1 to specific support for enabling a selected device with known location to support configuration by the network for at least some positioning calibration measurements.

Observation 6: The current specifications are not sufficient for enabling a reference device.

Observation 7: UEs with fixed location currently have no ability to report their location to the LMF.

Observation 8: Reference devices could have the ability for additional positioning measurement reporting capabilities.

Observation 9: Reporting of RxTx TEG is useful for positioning.

Proposal 5: Support Alt. 2, Option 1 in the prior agreement on UE Rx-Tx time difference measurements.

Propsoal 6: Don’t support UE providing association of PRS resources and Rx TEG to LMF for UE Rx-Tx measurements.

Propsoal 7: Support Alt. 2, Option 1 in the prior agreement on gNB Rx-Tx time difference measurements.

Propsoal 8: Don’t support TRP reporting the association information of SRS resource to TRP Rx TEG for gNB Rx-Tx measurements.

Proposal 9: UE to provide gNB its measurement time window for UE Rx-Tx time difference measurement.

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

[2] R1-2105513, Views on enhancing UL AoA, Nokia, Nokia Shanghai Bell.

[3] R1-2105514, Views on enhancing DL AoD, Nokia, Nokia Shanghai Bell.

[4] R1-2105515, Views on PHY Latency Reductions, Nokia, Nokia Shanghai Bell.

[5] R1-2105516, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

[6] R1-2105517, Additional views on Inactive Mode Positioning and on-demand PRS, Nokia, Nokia Shanghai Bell.