3GPP TSG RAN WG1 #105-e	R1-21005595

e-Meeting, May 19 – 27, 2021

Agenda item:	8.6.1.4

Source:	NEC

Title: 	MIMO aspects for RedCap

Document for:	Discussion and Decision

Introduction

A revised WID on support of reduced capability device type (RedCap) was approved in RAN#91E [1]. In this contribution, we discuss about some MIMO aspects.

Discussion

Capability of maximum number of DL MIMO layers

UE radio access capability parameter for maximum number of DL MIMO layers, which is signaled to the network, is specified in 4.2.7.6 of TS 38.306 [2] as follows.

4.2.7.6	FeatureSetDownlinkPerCC parameters

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE does not support MIMO on this carrier. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

To accommodate support of RedCap UE, updates are needed to the description.

Proposal:

- DL MIMO capability of TS 38.306 needs update

- TP is provided as follows:

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. Except for RedCap type, Ffor single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. For RedCap type with 2Rx, it is mandatory with capability signaling to support 2 MIMO layers. For RedCap type with 1Rx, only single layer is supported. If absent, except for RedCap type the UE does not support MIMO on this carrier, or for RedCap type the UE only supports single layer. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Existing parameter does not support explicit indication of support of single layer in DL. On the other hand, as proposed in the TP, single layer can be indicated by absence of the parameter implicitly.

Proposal:

- Support of single layer only can be indicated by absence of the parameter maxNumberMIMO-LayersPDSCH

For RedCap type, maximum number of MIMO layers is equal to number of Rx antennas [1]. A RedCap UE can indicate its number of Rx antennas with maximum number of DL MIMO layers implicitly.

Proposal:

- Number of Rx branches of RedCap UE can be indicated implicitly by the parameter maxNumberMIMO-LayersPDSCH

DMRS configuration

For better co-existence of non-RedCap and RedCap UE and flexibility of gNB scheduling, it would be beneficial that RedCap UE supports any DMRS configuration and any single antenna port for 1Rx RedCap and any antenna ports up to two antenna ports for 2Rx RedCap.

Proposal:

- For better co-existence between RedCap and non-RedCap UE and scheduling flexibility of gNB,

  - RedCap UE supports any DMRS configuration

  - RedCap UE with 1Rx supports any single antenna port

  - RedCap UE with 2Rx supports any antenna port(s) up to two layers

- Existing tables to indicate antenna port(s) of DCI formats are reused

Summary

Proposal:

- DL MIMO capability of TS 38.306 needs update

- TP is provided as follows:

| maxNumberMIMO-LayersPDSCH Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. Except for RedCap type, Ffor single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. For RedCap type with 2Rx, it is mandatory with capability signaling to support 2 MIMO layers. For RedCap type with 1Rx, only single layer is supported. If absent, the UE does not support MIMO on this carrier except for RedCap type, or the UE only supports single layer for RedCap type. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal:

- Support of only single layer can be indicated by absence of the parameter maxNumberMIMO-LayersPDSCH

Proposal:

- Number of Rx branches of RedCap UE can be indicated implicitly by the parameter maxNumberMIMO-LayersPDSCH

Proposal:

- For better co-existence between RedCap and non-RedCap UE and scheduling flexibility of gNB,

  - RedCap UE supports any DMRS configuration

  - RedCap UE with 1Rx supports any single antenna port

  - RedCap UE with 2Rx supports any antenna port(s) up to two layers

- Existing tables to indicate antenna port(s) of DCI formats are reused

Reference

[1] RP-210918	Revised WID on Support of reduced capability NR devices

[2] TS 38.306