3GPP TSG RAN WG1 #105-e			R1-2105662

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.10.1

Source:	AT&T

Title:	Enhancements for IAB resource multiplexing

Document for:	Discussion/Approval

# Introduction

As part of the Rel-17 Work Item on Enhancements for Integrated Access and Backhaul for NR [1], 3GPP has agreed to the following RAN1 led objectives:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

This contribution discusses different physical layer enhancements to support simultaneous operation of child and parent links including aspects impacting resource allocation and RRM for IAB nodes.

# Resource Multiplexing Enhancements for IAB

During Rel-16, for in-band operation IAB nodes are assumed to operate with a half-duplex constraint as shown in Figure 1, which means they can only do the following at any given time:

- Receive on the access link (UE to IAB node) and/or backhaul link (IAB node to IAB node)

- Transmit on the access link and/or backhaul link

Figure 1: Half-duplex constraint for IAB in Rel-16

As a result, the frame structure, timing alignment, and initial access/RRM procedures for IAB nodes in Rel-16 focused on so-called TDM operation where either the access link or backhaul link was active in a given time/frequency resources. However in Rel-17 enhancements are considered which extend the resource multiplexing to additional scenarios such as spatial division multiplexing (SDM) and multi-panel Tx/Rx (MPTR) as shown in Figure 2. MPTR is a subset of potential “full duplex IAB” operation where the different panels are used for access and backhaul links in a given slot. Depending on the scenario, MPTR could be extended to support simultaneous operation within a panel, however potentially with stricter interference requirements.

SDM

MPTR

SDM

MPTR

Figure 2: SDM and MPTR for IAB in Rel-17

## Multiplexing enhancements for cell-specific channels/signals

In RAN1#104-e the following agreement was reached:

Agreement

Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17.

- FFS: IAB-MT behavior in case of conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT (e.g., dedicated slot configurations)

This enables coexistence with existing Rel-16 resource multiplexing by extending the hard resource definition to support simultaneous (half and full duplex) operation.

However, as noted in the agreement, since the IAB nodes have both DU functionality as well as MT functionality, the IAB node DU function must transmit signals and channels used for initial access/RRM as well as receive reports from connected devices which may be both access UEs and higher order IAB nodes. As a result, while the same physical signals may be used for both UEs and IAB nodes, different configuration of the resources and/or transmission period(s) of the signals used for RRM for access UEs and IAB nodes may be required and the parameters configuring RRM operation at the IAB node DU function need to consider the half-duplex constraint imposed by the MT function and can also take into account hop order and other topology/route management functionalities as shown in Figure 3 where three orthogonal patterns are configured to allow IAB nodes to transmit and measure SSBs in different 5 ms windows (i.e. STC/SMTC configurations).

Figure 3. Half-Duplex SSB-based STC/SMTC Configurations

If the IAB nodes instead support SDM or MPTR operation, the SSB patterns indicated by the STC/SMTC configurations can be fully overlapped, since in the same time slot an IAB-DU is transmitting a SSB, the co-located IAB node MT can be receiving SSBs transmitted by other IAB nodes. However, in order to avoid impact on access UEs and ensure backwards compatibility with TDM-only IAB nodes, it may be beneficial that alignment of SSB transmissions/receptions only occur in resources which are orthogonal to the resources used for access UEs and TDM-only IAB nodes.

In addition, when operating under SDM or MPTR, the network may configure resources for data transmission/reception for IAB nodes which are overlapping to resources for RRM. Two types of resources “hard” and “soft” resources may be configured for the data resources, where hard resources can only be used by the DU, while soft resources may be flexibly shared between the DU and MT based on traffic or interference considerations. In 3GPP Rel-16, resources for RRM are fixed to only be considered as hard resources when overlapping with the data resources due to the half-duplex constraint. Additionally configuring soft resources which overlap with RRM resources of the IAB-MT can be considered in Rel-17, however again it should be ensured that this does not impact performance, especially for access UEs and TDM-only IAB nodes.

Proposal 1: Specify support for mechanisms to enable non-TDM (half-duplex and full-duplex) cell-specific configurations (e.g. STC/SMTC, RACH, system information, periodic CSI-RS etc.) of the IAB-MT in resources which are not used for access UE transmissions, including overlapping hard and soft configured IAB-DU resources.

## Support for Dynamic multiplexing capability indications

The following agreements were reached in RAN1#104bis-e:

Agreement

Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

Rel-16 allows for the parent and child to exchange information about the supported multiplexing capabilities and IAB-specific guard symbols which can be used to enable more efficient utilization of resources, including guard symbols and especially soft resources at the child IAB-DU. However, multiple factors may influence whether a given multiplexing case is more or less appropriate for a given time/frequency resource for example a desire to avoid certain DL or UL slots used by access UEs or TDM-only IAB nodes, and also other factors such as self or cross-link interference, single or multi-parent operation, transmit or receive panel selection, and device capabilities. As a result, in order for the dynamic indication of availability of soft resources to be utilized effectively the parent and node should be able to take this potential variability of the multiplexing operation and guard symbols into account. Enhancements to DCI Format 2_5 and the MAC CE for indicating the number of desired/available guard symbols should be considered to support this dynamic indication.

Proposal 2: Support dynamic indication of multiplexing capability and guard symbols for a subset of time/frequency resources of given backhaul link via enhancements to DCI Format 2_5 and MAC-based signaling of guard symbols between an IAB node and its parent node.

## Extension of spatial resources to H/S/NA resource attributes

In RAN1#104-e the following agreements was reached:

Agreement

Further study whether/how to manage resources in the spatial domain. Candidate solutions are:

- Dynamic signaling between parent and child nodes for using/restricting/sharing antenna panels/beams

- Beam management / multi-panel enhancements for simultaneous operations

- Extension of H/S/NA resource indication to the spatial domain

Other solutions are not precluded.

During RAN1#104bis-e the following agreements were reached:

Agreement

To facilitate simultaneous operations and interference management, dynamic indication for restriction/usage/availability of beams (in upstream and/or downstream directions) is supported

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Whether IAB-specific enhancements beyond the existing beam management framework are needed to the support the functionality

- FFS: Impact on the semi-static resource configurations (e.g., extending the H/S/NA resource attributes to the spatial domain)

- FFS: Whether panel-based granularity is additionally supported

In addition, using the beam-management framework defined for UEs, IAB nodes which support multiple panels or even multiple beams within a same panel with sufficient isolation could conceivably be simultaneously utilized on different access and backhaul links. Similar to time/frequency subsets, the existing multiplexing capability signaling framework does not have sufficient granularity to allow differentiation based on spatial information. As a result, there may be a benefit to extend the H/S/NA resource indication to the spatial domain at the panel/beam level. RAN1 should further investigate mechanisms to associate a given H/S/NA resource at the IAB-DU with different QCL assumptions of a co-located IAB-MT using the existing beam-management framework (e.g. on the basis of different TCI states configured at the IAB-MT).

Proposal 3: Consider mechanisms to associate a given H/S/NA resource at the IAB-DU with different QCL assumptions of a co-located IAB-MT using the existing beam-management framework and potential extensions to support panel-level granularity.

## Extension of frequency resources to H/S/NA resource attributes

In order to extend the H/S/NA framework in case of FDM operation beyond inter-carrier granularity, the following agreements were made in RAN1#104bis-e:

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Agreement

For the semi-static DU resource configuration in the frequency domain within a carrier, the frequency-domain granularity is configurable

- FFS:  minimum resource size e.g. N PRBs/N RBGs

- FFS: Separate or joint TDM and FDM semi-static DU resource configurations

Agreement

Soft resource availability indications for frequency-domain resources are supported

- FFS enhancements to DCI Format 2_5

- FFS: Separate or joint TDM and FDM indications

In order to make an efficient tradeoff between overhead of the signaling and the flexibility to support FDM within a carrier between access and backhaul links, RBG-level frequency granularity and symbol-level granularity of the indication can be taken as the starting point for the resource configuration. This enables the network to avoid TDM-only H/S/NA configurations and RBG-level granularity for the frequency domain can coexist well with access UEs and IAB-MTs. To further maximize flexibility a separate indication for both the semi-static and dynamic (i.e. DCI Format 2_5) signaling should be introduced.

Proposal 4: Support the extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types with the following:

- Symbol-level time-domain granularity

- RBG-level frequency-domain granularity

- Separate indications for time/frequency resources in case of semi-static DU resource type indications and DCI_Format 2_5 soft resource availability indications

## Multi-parent scenarios

In addition, in Rel-16, the resource allocation and multiplexing framework focused on single-parent scenarios for a given IAB-node. However, to support redundancy and load-balancing, multi-parent scenarios are also possible.

Furthermore, during RAN1#104bis-e the following agreement was reached for multi-parent scenarios:

Agreement

The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity) to support simultaneous Tx and/or Rx at the child IAB-MT to/from both parent links:

- Extending the Rel-16 CA TDD conflict resolution framework for synchronous intra-band NR-DC operation

- Coordinating TDD configurations for the parent nodes (for both intra-donor and inter-donor operation) and coordinating H/S/NA configurations for the child node between donors (at least for inter-donor operation)

In order to address multi-parent operation, signaling for the IAB-DU resource configuration may need enhancements in Rel-17. For example, while per-DU resource configurations were considered sufficient for Rel-16, per-link resource configurations may be needed to coexist or replace the per-DU configurations to avoid severe resource restrictions and overhead. In addition, these per-link configurations may need to be exchanged across multiple IAB-DUs in the case of multi-parent operation, since to allow multiplexing which is more efficient than strict TDM requires awareness of the configured resources at the multiple parents to avoid scheduling conflicts.

Proposal 5: Per-backhaul link resource configurations and multiplexing capability indications independent of the per-IAB-DU configuration and multiplexing capability indication are supported in Rel-17. Coordination signaling to exchange the per-link resource configurations and multiplexing capability indications between up to two parent IAB-nodes/donors should be additionally supported in Rel-17.

# Conclusion

This contribution analyzed potential physical layer enhancements for IAB resource allocation and multiplexing in Rel-17. The following proposals were made:

Proposal 1: Specify support for mechanisms to enable non-TDM (half-duplex and full-duplex) cell-specific configurations (e.g. STC/SMTC, RACH, system information, periodic CSI-RS etc.) of the IAB-MT in resources which are not used for access UE transmissions, including overlapping hard and soft configured IAB-DU resources.

Proposal 2: Support dynamic indication of multiplexing capability and guard symbols for a subset of time/frequency resources of given backhaul link via enhancements to DCI Format 2_5 and MAC-based signaling of guard symbols between an IAB node and its parent node.

Proposal 3: Consider mechanisms to associate a given H/S/NA resource at the IAB-DU with different QCL assumptions of a co-located IAB-MT using the existing beam-management framework and potential extensions to support panel-level granularity.

Proposal 4: Support the extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types with the following:

- Symbol-level time-domain granularity

- RBG-level frequency-domain granularity

- Separate indications for time/frequency resources in case of semi-static DU resource type indications and DCI_Format 2_5 soft resource availability indications

Proposal 5: Per-backhaul link resource configurations and multiplexing capability indications independent of the per-IAB-DU configuration and multiplexing capability indication are supported in Rel-17. Coordination signaling to exchange the per-link resource configurations and multiplexing capability indications between up to two parent IAB-nodes/donors should be additionally supported in Rel-17.

# Reference

[1] RP-201293 New WID on Enhancements to Integrated Access and Backhaul, Qualcomm

[2] R1-2100777 Enhancements for IAB interference management, AT&T