3GPP TSG RAN WG1 #105-e			R1-2105695

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhancements for unlicensed band URLLC

Agenda Item:	8.3.2

Document for: 	Discussion and Decision

- Introduction

At RAN1#104bis-e meeting, following agreements and conclusions related to enhancements for unlicensed band URLLC were made [1]:

Agreements: - Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode. Agreements: - For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. Agreement: - For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell: - Do not support any additional period value Agreement: - For semi-static channel access mode, the starting point of first UE FFP for a serving cell - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement). Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation. FFS whether/how to enhance Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. Agreement: - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

As per the guidance from RAN1 chair as follows, in the following section, COT initiator determination when UE can operate as UE-initiated COT for FBE and harmonizing UL CG enhancements are discussed.

Only to handle topics of CG harmonization and decision between Alt-a/Alt-b for semi-static channel access mode when a UE can operate as UE-initiated COT for both CG and DG (see sections 2.3/2.4/2.5 of R1-2103960) - No contributions to other topics please

- COT initiator determination when UE can operate as UE-initiated COT for FBE

Regarding COT-initiator determination for configured/scheduled UL, the determination rule was agreed for a configured UL transmission which starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE as follows, while it was not agreed for the following cases:

- A configured UL transmission which is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE

- A scheduled UL transmission

Configured UL Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Scheduled UL Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreement: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission

Basically UE can determine whether the configured/scheduled UL is associated to gNB-initiated COT or UE-initiated COT according to the alignment between the configured/scheduled resources and the UE FFP boundary if UE has not initiated COT as shown in Cases 1 and 2 in Fig. 1.

Figure 1.  COT association for scheduled/configured UL (from R1-2009781)

However, as shown in Cases 3a and 3b in Fig. 4, it was discussed whether UE can initiate COT when gNB has already initiated COT. Based on the discussion in the last RAN1 meeting [2], companies may have different understanding on this point and we think it’s better to have common understanding to promote the discussion on the COT initiator determination as suggested by moderator as follows:

Alternative Proposed Conclusion 3-1: - When a first DL transmission at a gNB FFP boundary is assumed to initiate a gNB-COT for that gNB FFP, if a UL transmission at a UE FFP boundary is assumed to initiate a UE COT for that UE FFP and the UL transmission is confined within that gNB FFP, a second DL transmission in that gNB FFP after the UL transmission may be associated to gNB initiated COT. - When a first UL transmission at a UE FFP boundary is assumed to initiate a UE-COT for that UE FFP, if a DL transmission at a gNB FFP boundary is assumed to initiate a gNB COT for that gNB FFP and the DL transmission is confined within that UE FFP, a second UL transmission in that UE FFP after the DL transmission may be associated to UE initiated COT.

It is our understanding that the above proposed Conclusion 3-1 does not try to down-select the rule for COT initiator determination, but rather tries to have common understanding among companies whether gNB and UE can initiate COT when another device has already initiated COT or not as long as the regulation is fulfilled. We have the same understanding with the moderator that the above proposed Conclusion 3-1 means that when a device initiates a COT, it has the right to access the channel until the idle period of the FFP. In this FFP, it can also share its right to access the channel with other devices. It does not mean whether another device to use it or not, or the initiating device would lose its right to access the channel and loses its right to use the COT until the idle period of the FFP.

Proposal 1:

- When a first DL transmission at a gNB FFP boundary is assumed to initiate a gNB-COT for that gNB FFP, if a UL transmission at a UE FFP boundary is assumed to initiate a UE COT for that UE FFP and the UL transmission is confined within that gNB FFP, a second DL transmission in that gNB FFP after the UL transmission may be associated to gNB initiated COT.

- When a first UL transmission at a UE FFP boundary is assumed to initiate a UE-COT for that UE FFP, if a DL transmission at a gNB FFP boundary is assumed to initiate a gNB COT for that gNB FFP and the DL transmission is confined within that UE FFP, a second UL transmission in that UE FFP after the DL transmission may be associated to UE initiated COT.

Under the above assumption and agreements, for the case when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, the COT initiator for Alt-a and Alt-b can be shown as Fig. 2 as an example. For both Alt-a and Alt-b, gNB initiates COT by DL1 and UE can use shared gNB COT for configured UL1 transmission and hence, no LBT is necessary if a proper gap less than 16us is set. Then, gNB can transmit DL2 using gNB COT without LBT for a proper gap. For Alt-a, UE can use shared gNB COT for configured UL2 transmission again without LBT, while for Alt-b, UE has to initiate COT with LBT even if the gap is less than 16us. Considering the successful COT continuation until the idle period for low UL latency, we slightly prefer Alt-a. Though Alt-b allows UL transmission in the idle period for gNB FFP, it is not the main motivation to introduce the UE-initiated COT for FBE to reduce the UL latency.

Figure 2.  COT initiator for configured UL

Proposal 2:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP,

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

  - Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

For scheduled UL, following two alternatives were agreed to be down-selected, where Alt-b is preferred because of the same reason as configured UL. We don’t see the motivation to adopt dynamic indication by DCI as the rules applied for a configured UL transmission is enough for the COT-initiator determination. However, if the first FFS point for Alt-a is resolved, i.e., determination based on the rules applied for configured UL transmissions is applied if the corresponding field(s) is absent, we can live with Alt-a as a compromised solution.

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

- Alt-b: Determination based on the rules applied for a configured UL transmission

Proposal 3:

- For a scheduled UL transmission, support one of the following alternatives for the COT-initiator determination

  - Alt-a: Determination based on the content in the scheduling DCI

    - The corresponding field(s) can be absent in DCI. If absent, determination based on the rules applied for configured UL transmissions is applied

  - Alt-b: Determination based on the rules applied for a configured UL transmission

It was also discussed in the last RAN1 meeting whether to support channel access field in DCI x_2 as follows. From URLLC perspective, compact DCI is beneficial and hence, channel access field should be supported in DCI x_2 as well, while the field can be absent as the same reason as above.

Proposal 4-2: Select one of the following: - Option 1: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively. - Option 2: Study whether to support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

Proposal 4:

- Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively

  - The field can be absent in the DCI

- Harmonizing UL configured-grant enhancements in NR-U and URLLC

In Rel.16, CG-PUSCH has been enhanced both in URLLC/IIoT and NR-U and the enhanced features are summarized in Table 1. CG-DFI and CG-UCI has to be indicated in unlicensed band for CG-PUSCH in current spec. CG-DFI and CG-UCI are useful to improve UL HARQ operation when there is frequent LBT failure. CG-UCI is also useful to indicate COT sharing information to gNB. While for URLLC use case and in controlled environment, Rel.16 URLLC CG features would be enough. Therefore, it should be supported that both CG-PUSCH with/without CG-DFI or CG-UCI can be configured by gNB for flexible CG-PUSCH operation. At RAN1#103e, following options were discussed but no consensus was achieved at RAN1#104e, and down-selected to Options 1 or 2-a at RAN1#104bis-e:

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Following the RAN2 agreement as below, it is straightforward to select either Rel.16 NR-U or URLLC HPN/RV determination by the configuration of cg-RetransmissionTimer-r16. When is configured, “CG-DFI based procedures” should be enabled as they are highly related for the operation of automatic CG retransmission. Regarding “CG-UCI based procedure”, at least HPN/RV indication should be enabled together with “CG-DFI based procedures” as Rel.16 NR-U HPN/RV determination relies on their indication in CG-UCI. However, CG-UCI also includes COT sharing information. We think this should be separately enabled/disabled for the flexibility.

Agreements (RAN2#112-e):

From RAN2 perspective

1 	It is assumed that LBT failures only happen infrequently in UCE (unlicensed controlled environment).  A formal definition of UCE and its relationship to semi-static or dynamic access mode is not necessary in RAN2 specifications.

2	cg-RetransmissionTimer can be configured optionally for shared spectrum

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

5	As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U.

6	HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured.

7	FFS if LCH based prioritization can be configured with cg-RetransmissionTimer

8	The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell.  No optimizations will be pursued to allow the two features be configured together in Rel-16.  No CR is needed for this for now.

9	If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it.  FFS how to achieve it (using existing mechanisms should be considered as baseline)

Proposal 5:

- Support Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

  - Indication of COT sharing information included in CG-UCI is separately enabled/disabled from the configuration of cg-RetransmissionTimer-r16

Table 1.  Configured grant (CG) features supported in Rel.16 NR-U and Rel.16 URLLC (from R1-2005376)

| CG features | Rel.16 URLLC | Rel.16 NR-U |
| --- | --- | --- |
| Multiple CG configurations | Supported | Supported |
| HARQ process number/ ID determination | Associated with the configured/indicated first TO, calculated based on the equation defined in TS 38.321 | Decide and reported by the UE in CG-UCI |
| Management of HARQ process number/ ID among multiple CG configurations | Not shared between different CG configurations in the same BWP | Can be shared between different CG configurations in the same BWP |
| RV determination | One of the three RV sequence can be configured and associated with TO {0,0,0,0}; {0,3,0,3}; {0,2,3,1} | Decided and reported by the UE in CG-UCI |
| Flexible initial transmission occasion (TO) | If the CG is configured with Configuredgrantconfig-StartingfromRV0 set to ‘off’, the initial transmission only starts at the first TO of the K repetitions; otherwise, the initial transmission TO depends on the configured RV sequence and K repetitions. | Multiple consecutive potential TOs are configured by cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16, can start initial transmission at any TOs depending on the LBT results. |
| Repetition scheme(s) | PUSCH repetition Type A and PUSCH repetition Type B | Similar as PUSCH repetition Type B without supporting segmentation. (no support of cross-slot resource allocation, and if collide with invalid symbol(s), drop the repetition) |
| CG-Downlink feedback information (DFI) | No support. If Re-scheduling UL grant is not received, UE assumes ACK. | Support, If CG-DFI is not received, UE assumes NACK. |
| CG Re-transmission timer | No support | Support and always configured |
| CG Re-transmission scheme | Only support Re-transmission scheduled by UL grant | Support automatic re-Transmission on the same or different CG configuration decided by UE, and support re-Transmission scheduled by UL grant |

- Conclusion

In this contribution, we discussed the uplink enhancements for URLLC in unlicensed controlled environments. Based on the discussion, we made following proposals.

Proposal 1:

- When a first DL transmission at a gNB FFP boundary is assumed to initiate a gNB-COT for that gNB FFP, if a UL transmission at a UE FFP boundary is assumed to initiate a UE COT for that UE FFP and the UL transmission is confined within that gNB FFP, a second DL transmission in that gNB FFP after the UL transmission may be associated to gNB initiated COT.

- When a first UL transmission at a UE FFP boundary is assumed to initiate a UE-COT for that UE FFP, if a DL transmission at a gNB FFP boundary is assumed to initiate a gNB COT for that gNB FFP and the DL transmission is confined within that UE FFP, a second UL transmission in that UE FFP after the DL transmission may be associated to UE initiated COT.

Proposal 2:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP,

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

  - Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Proposal 3:

- For a scheduled UL transmission, support one of the following alternatives for the COT-initiator determination

  - Alt-a: Determination based on the content in the scheduling DCI

    - The corresponding field(s) can be absent in DCI. If absent, determination based on the rules applied for configured UL transmissions is applied

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Proposal 4:

- Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively

  - The field can be absent in DCI

Proposal 5:

- Support Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

  - Indication of COT sharing information included in CG-UCI is separately enabled/disabled from the configuration of cg-RetransmissionTimer-r16

References

- 3GPP, RAN1 #104bis-e meeting, chairman’s notes.

- Moderator (Ericsson), R1-2103960, Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band, April 2021.