3GPP TSG RAN WG1 #105-e		R1-2105776

e-meeting, May 10th – 27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements for PUCCH repetition

Agenda Item:   8.8.2

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on enhancements for PUCCH repetition enhancements including:

- dynamic indication for number of PUCCH repetitions

- DM-RS bundling (joint channel estimation) across multiple PUCCH repetitions.

# 2	PUCCH enhancements for coverage

## 2.1 Dynamic indication for PUCCH repetition factor

In RAN1#104-e, following agreement related to dynamic indication for PUCCH repetition factor has been made in [2]:

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication.

- Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

  - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc.

  - FFS: RRC signaling enhancement details

- Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI

  - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication

  - FFS whether there is a need for RRC update

Currently PUCCH format 1, format 3 and format 4 can each be configured by RRC with individual repetition factor. For dynamic indication of the repetition factor, two options have been considered in RAN1#104-e. One option is to enhance RRC signalling to allow configuration of PUCCH repetition factor per PUCCH resource, and the second option is to enhance DCI fields to explicitly indicate PUCCH repetition factor.

Regarding option 2 in the agreement, the overhead could be up to 3 bits to signal up to 8 repetitions, which is not desirable. Similar enhancements were considered for PUSCH repetition factor as well in Rel-16, but it was not considered due to similar overhead reasons. Therefore, option 2 should not be considered for dynamic indication of PUCCH repetition factor.

Regarding option 1 in the agreement, PUCCH resource indicator field can be considered for also indicating the repetition factor for PUCCH. Currently the PRI field can be configured with 0-3 bits, depending upon the number of PUCCH resources within a PUCCH resource set. The bitmap values map to one of the PUCCH resource ID within a PUCCH resource set, with the PUCCH resource set selected depending upon the payload to be transmitted in PUCCH. The current PRI mapping could be extended to also indicate the repetition factor, as illustrated in Table 1. A PRI to repetition factor mapping list can be RRC configured for a PUCCH resource set.

Table 1: Dynamic repetition indication with PRI table

| PUCCH resource indicator |  |  | PUCCH resource | Repetition Factor |
| --- | --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits |  |  |
| '0' | '00' | '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList | 2 |
| '1' | '01' | '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList | 2 |
|  | '10' | '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList | 2 |
|  | '11' | '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList | 4 |
|  |  | '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList | 8 |
|  |  | '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList | 4 |
|  |  | '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList | 16 |
|  |  | '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList | 2 |

Proposal 1: For supporting dynamic indication of the repetition factor for PUCCH repetitions in NR Rel-17 coverage enhancements, reuse PUCCH resource indicator (PRI) field to indicate the number of repetition corresponding each of the PUCCH resource by extending the PRI table.

## 2.2 DM-RS bundling for PUCCH repetitions

In RAN1#104-e, following agreements/conclusions related to DM-RS bundling for PUCCH repetitions have been made in [2]:

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

- FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.

- Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

Agreements:

Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

- FFS: the configuration is per UE or per PUCCH resource.

- FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

- FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

Conclusion: In Rel-17, deprioritize the study of DMRS pattern/location/granularity optimization for PUCCH coverage enhancement in AI 8.8.2. This conclusion could be revisited after the progress on the study of  DMRS pattern/location/granularity optimization for PUSCH coverage enhancement in AI 8.8.1.3.

Conclusion: For the study of enhancing inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, at least the following aspects can be considered:

- Performance tradeoff between maximizing # consecutive UL slots in one frequency hop (to achieve more DMRS bundling gain) and maximizing # hops (to achieve more diversity gain)

  - Note: the maximum # frequency hopping positions is still 2 as in Rel-15/16., which is signaled by startingPRB and secondHopPRB

- Interaction between hopping boundary determination and TDD configuration

Conclusion: For the simulations to study the enhancement of inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling, simulation assumptions in 38.830 are reused as a starting point.

Note: Additional simulation scenarios/assumptions are not precluded.

Based on the agreements, the intention for joint channel estimation for PUCCH should be to strive for common design with joint channel estimation for PUSCH. In RAN1#104b-e, following agreement related to time domain window for joint channel estimation for PUSCH has been made [3].

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain windown (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS whether or not to further consider impacting of timing advance

In our view, similar agreement can be directly applied to PUCCH as well and should be quite straightforward to agree.

Proposal 2: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

Another aspect related to joint channel estimation using DM-RS bundling is for multiple PUCCH with frequency hopping. If frequency hopping is enabled for multiple PUCCH transmissions, then the time domain window (for DM-RS bundling) should be contained within a single hop as it has been agreed to restrict the same frequency resource allocation across PUCCH repetitions. Alternatively, the time domain window could be same as frequency hop duration. Otherwise, phase continuity might be an issue for joint channel estimation across multiple frequency hops. However, if such limitation is applied, then the DM-RS bundling is not really beneficial as currently only intra-slot or inter-slot frequency hopping is supported. Therefore, multi-slot frequency hopping should be supported. This would provide better opportunity for joint channel estimation with DM-RS bundling. As illustrated in Figure 1, frequency hop duration is across 4 slots for PUCCH format 1 and therefore, the DM-RS bundling is also applied in multiples of 2 slots for each of the frequency hop.

Figure 1: Example multi-slot frequency hopping with DM-RS bundling for PUCCH format 1 repetition

Proposal 3: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, support multi-slot frequency hopping and multi-slot DM-RS bundling for joint channel estimation for entire hop:

- Association between frequency hop duration and time-domain window should be supported such that explicit indication of both the frequency hop duration and time-domain window is not needed

  - Time-domain window size can be equal to the frequency hop duration

- At least hop duration of 2 slots should be supported with DM-RS bundling

In addition, following details related to time-domain window should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUCCH transmissions (for maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be additionally configured/indicated (duration value could be smaller than the maximum duration) and the UE is expected to maintain phase continuity over at least each successive non-overlapping time-domain window duration starting from the first PUCCH transmission

- For a burst of PUCCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

Proposal 4: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, following details related to time-domain window should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUCCH transmissions (for maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be additionally configured/indicated (duration value could be smaller than the maximum duration) and the UE is expected to maintain phase continuity over at least each successive non-overlapping time-domain window duration starting from the first PUCCH transmission

- For a burst of PUCCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

# 3	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For supporting dynamic indication of the repetition factor for PUCCH repetitions in NR Rel-17 coverage enhancements, reuse PUCCH resource indicator (PRI) field to indicate the number of repetition corresponding each of the PUCCH resource by extending the PRI table.

Proposal 2: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

Proposal 3: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, support multi-slot frequency hopping and multi-slot DM-RS bundling for joint channel estimation for entire hop:

- Association between frequency hop duration and time-domain window should be supported such that explicit indication of both the frequency hop duration and time-domain window is not needed

  - Time-domain window size can be equal to the frequency hop duration

- At least hop duration of 2 slots should be supported with DM-RS bundling

Proposal 4: For supporting joint channel estimation with DM-RS bundling across multiple PUCCHs for coverage enhancements in NR Rel-17, following details related to time-domain window should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUCCH transmissions (for maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be additionally configured/indicated (duration value could be smaller than the maximum duration) and the UE is expected to maintain phase continuity over at least each successive non-overlapping time-domain window duration starting from the first PUCCH transmission

- For a burst of PUCCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

# 4	References

[1]	3GPP RP-210855, “Revised WID on NR coverage enhancements”, China Telecom

[2] 	3GPP RAN1#104-e. “Chairman Notes”

[3] 	3GPP RAN1#104b-e. “Chairman Notes”