3GPP TSG-RAN WG1 Meeting #105-e	Tdoc R1-2105831

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.14.3

Source:	Ericsson

Title:	Initial XR performance evaluation results

Document for:	Discussion

# 1	Introduction

In [1], a new SI on evaluations for XR was agreed. In this contribution, we provide some initial system simulations for one particular scenario and highlight the impact of some factors that affect the capacity.

During RAN1#104bis-e, there was significant progress in defining the traffic models and evaluation methodology for XR evaluations. In this contribution, we will present initial simulation results using the agreed assumptions for the following cases:

- DL capacity in Dense Urban Macro for cloud gaming

- DL capacity in Urban Macro for cloud gaming

- DL capacity in Dense Urban Macro for VR

- DL capacity in Urban Macro for VR

- UL capacity in Dense Urban for the pose traffic model

- UL capacity in Urban Macro for the pose traffic model

- Power consumption in Dense Urban Macro

Note that for the traffic model parameters, we rely on the common baseline proposed in [2].

# 2	Discussion

## 2.1	DL capacity for cloud gaming

For cloud gaming, RAN1 agreed to use a frame-based model, where the frame sizes followed a truncated gaussian distribution. Also, there is frame level jitter, where the jitter follows a truncated gaussian distribution.

For the evaluations in shown in this document, following parameters for frame size and jitter are used

- Bit rate: 8Mbps

- Frame rate: 60fps

- Frame size distribution:

  - Mean: 16.7kB

  - Standard deviation: 2.5kB

  - Max frame size: 25kB

  - Min frame size: 8.33kB

- Jitter distribution:

  - Mean: 0ms

  - Standard deviation: 2ms

  - Max: 4ms

  - Min: -4ms

### 2.1.1	Dense urban

We have performed system simulations for the Dense Urban scenario for the CG traffic model above. The results are shown in Figure 1, for PDB=10ms and 15ms, and for 99% of video frames successfully delivered within the PDB. Detailed simulation assumption can be found in the appendix.

We have simulated this for 32 and 128 gNB antennas.

Figure 1: Performance for CG in Dense Urban. 32 and 128 gNB antennas

We see that the performance clearly depends on the chosen PDB: the smaller the PDB, the smaller the capacity. If we define capacity as the maximum load where 90% of the users are satisfied, the capacities are around 11.8 and 15.8 users for 32 and 128 gNB antennas respectively.

### 2.1.2	Urban macro

We have performed system simulations for the Urban macro scenario for the CG traffic model above. The results are shown in Figure 1, for PDB=10ms and 15ms, and for 99% of video frames successfully delivered within the PDB. Detailed simulation assumption can be found in the appendix.

We have simulated this for 32 and 128 gNB antennas.

Figure 2: Performance for CG in Urban Macro. 32 and 128 gNB antennas

We see that the performance clearly depends on the chosen PDB: the smaller the PDB, the smaller the capacity. If we define capacity as the maximum load where 90% of the users are satisfied, the capacities are around 8.6 and 13.1 users for 32 and 128 gNB antennas respectively.

## 2.2	DL capacity for VR

For VR, RAN1 agreed to use a frame-based model, where the frame sizes followed a truncated gaussian distribution. Also, there is frame level jitter, where the jitter follows a truncated gaussian distribution.

For the evaluations in shown in this document, following parameters for frame size and jitter are used

- Bit rate: 30Mbps

- Frame rate: 60fps

- Frame size distribution:

  - Mean: 62.5kB

  - Standard deviation: 6.56kB

  - Max frame size: 93.8kB

  - Min frame size: 31.3kB

- Jitter distribution:

  - Mean: 0ms

  - Standard deviation: 2ms

  - Max: 4ms

  - Min: -4ms

### 2.1.1	Dense urban

We have performed system simulations for the Dense Urban scenario for the VR traffic model above. The results are shown in Figure 1, for PDB=10ms and 15ms, and for 99% of video frames successfully delivered within the PDB. Detailed simulation assumption can be found in the appendix.

We have simulated this for 32 and 128 gNB antennas.

Figure 3: Performance for VR in Dense Urban. 32 and 128 gNB antennas

We see that the performance clearly depends on the chosen PDB: the smaller the PDB, the smaller the capacity. If we define capacity as the maximum load where 90% of the users are satisfied, the capacities are around 0.8 and 1.0 users/cell for 32 and 128 gNB antennas respectively.

## 2.3	Performance for pose traffic model

During RAN1#104-e, the pose traffic model was also discussed. This traffic model is meant to represent the traffic that is sent from an XR device to describe the movement of, e.g., an HMD, so that a game server can change the view displayed on the device. The traffic characterized by small but frequent packets transmitted in UL, each with a fixed size. The parameters of the used traffic model are

- Packet interval: 4ms

- Packets size: 100 bytes

In Figure 4, we illustrate the performance for the Dense Urban scenario.

Figure 4: Performance for the pose traffic model.

From Figure 4, it is clear that the load has only a small impact on the performance on the quality-of-experience: the number of satisfied users is more or less independent of the load. For Dense Urban, more than 90% of the users are always satisfied.

However, for Urban Macro, less than 60% of the users are satisfied. To investigate this in more detail, we have studied the fraction of satisfied users for a fixed traffic load, for different PDBs. The result is illustrated in Figure 5.

Figure 5: Fraction of satisfied users for different PDBs. The load is fixed to 5 users per cell.

The results in Figure 5 clearly illustrates the sensitivity of the capacity to the chosen PDB. With a PDB of 15ms, almost all UEs are satisfied also for Urban Macro. However, occasional retransmissions lead to that users become unsatisfied.

Note that we have run the UL pose traffic with dynamic scheduling: there is thus an inherent delay delivery of the packets: SR->BSR->UL data. Although configured grant may sound like a perfect use case for the pose traffic, dynamic scheduling may be the only realistic option for applications that are deployed at a wide scale.

## 2.3	Power consumption

In this section we present initial results evaluating impact of UE power consumption reduction techniques on XR capacity. Results are shown for Dense Urban Macro deployment. To be able to align data and DRX period start for eCDRX parameters, the start of data period start for each users is selected based on the UE id and evenly distributed over the data period.

The following scenarios are evaluated:

- Traffic models

  - DL only -- XR CG traffic with 8/30Mbps average data rate,10/15ms PDB

  - DL+UL – XR CG traffic with 8/30Mbps average data rate + UL pose information (4ms periodicity and 100bytes payload)

- DRX configured with below settings

  - Baseline case with no DRX is configured

    - labelled as ‘data-8/30-drx-0’ in the figures

  - DRX with different settings labelled as ‘data-8/30-drx-p-On-q-iat-r’ in the figures with below settings for p,q,r

    - DRX Cycle p = 10ms; On duration timer q = 5ms; IAT r = 5ms

    - Enhanced CDRX, DRX period adjusted with data period DRX Cycle p = 16.66ms (16.5,16.5,17, …); On duration timer q = 8ms; IAT r = 3ms

- Power consumption is also shown for ‘genie’ case

  - Labelled as ‘genie’ in the figures.

  - For this case it is assumed that PDCCH monitoring is turned off for the UE using a ‘genie assisted’ mechanism whenever there is no data reception (sleep if no UL) and the UE is sent to sleep state. Power consumption for the sleep state is based on available sleep duration.

### 2.3.1	DL CG model

Figure 2.3.1-1 shows XR performance for baseline (no DRX) case vs. different DRX settings

Figure 2.3.1-1: XR performance for baseline (no DRX) case vs. different DRX settings

Figure 2.3.1-2a and 2.3.1-2b show CDFs of UE power consumption for medium/high loads (i.e., 4,10 UEs/cell respectively) for different DRX settings.

Figures 2.3.1-2a (left), 2.3.1-2b (right) : XR Power consumption for different cell loading (DL CG)

Figure 2.3.1-3a and 2.3.1-3b show the relative fraction of time consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.1-3a shows results for medium load (4UEs/cell) and 2.3.1-3b for high load (10UEs/cell)

Figures 2.3.1-3a (left), 2.3.1-3b (right) : Relative time fraction of different UE power consumption states (DL CG)

Figure 2.3.1-4a, 2.3.1-4b, show the relative fraction of energy consumed by the UE in different power consumption states. Figure 2.3.1-4a shows results for medium load (4UEs/cell) and 2.3.1-4b for high load (10UEs/cell)

Figures 2.3.1-4a (left), 2.3.1-4b (right) : Relative energy fraction of different UE power consumption states (DL CG)

Table 2.3-1 below shows the XR capacity vs. UE power savings tradeoff for the different DRX settings that are considered in the evaluations

Table 2.3.1-1: XR capacity vs. UE power savings tradeoff (DL CG – 8Mbps)

| DRX configuration (DRX cycle, On duration, IAT) | Mean PS gain compared to baseline (%) | 95%-tile PS gain (highest Energy) | 5%-tile PS gain (lowest Energy) | Capacity Cell load when 90% of the users are satisfied |
| --- | --- | --- | --- | --- |
| No DRX (baseline) | N/A | N/A | N/A | 16.1 |
| (10ms,5ms,5ms) | 12% | 8% | 15% | 14.5 |
| (16.66ms,8ms,3ms) | 27% | 22% | 30% | 14.6 |
| genie | 51% | 42% | 58% | N/A |

Table 2.3.1-2: XR capacity vs. UE power savings tradeoff (DL CG – 30Mbps)

| DRX configuration (DRX cycle, On duration, IAT) | Mean PS gain compared to baseline (%) | 95%-tile PS gain (highest Energy) | 5%-tile PS gain (lowest Energy) | Capacity 15ms | Capacity 10ms |
| --- | --- | --- | --- | --- | --- |
| No DRX (baseline) | N/A | N/A | N/A | 3.0 | 1.7 |
| (10ms,5ms,5ms) | 10% | 6% | 14% | 2.2 | 0.1 |
| (16.66ms,8ms,3ms) | 23% | 17% | 29% | 2.5 | 1.4 |
| genie | 47% | 36% | 56% | N/A | N/A |

### 2.3.2	DL CG + UL Pose model

Here we show UE power consumption also considering UL transmissions for pose updates.

The following scenarios are evaluated:

- DRX configured with below settings

  - Baseline case with no DRX is configured: labelled as ‘data-8-drx-0’ in the figures

  - DRX with different settings labelled as ‘data-8-drx-p-On-q-iat-r’ in the figures with below settings for p,q,r

    - DRX Cycle p = 10ms; On duration timer q = 5ms; IAT r = 5ms

  - genie case, labelled as ‘genie’ in the figures

Figure 2.3.1-1: XR performance for baseline (no DRX) case vs. different DRX settings

Figure 2.3.2-1a and 2.3.2-1b show CDFs of UE power consumption for medium/high loads (i.e., 5,10 UEs/cell respectively)

Figures 2.3.2-1a (left), 2.3.2-1b (right) : XR Power consumption for different cell loading (DL CG + UL pose)

Figure 2.3.2-2a, 2.3.2-2b, show the relative fraction of time consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.2-2a shows results for medium load (5UEs/cell) and 2.3.2-2b for high load (10UEs/cell)

Figures 2.3.2-2a (left), 2.3.2-2b (right) : Relative time fraction of different UE power consumption states (DL CG + UL pose)

Figure 2.3.2-3a, 2.3.2-3b, show the relative energy fraction of energy consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.2-3a shows results for medium load (8UEs/cell) and 2.3.2-3b for high load (15UEs/cell)

Figures 2.3.2-3a (left), 2.3.2-3b (right) : Relative energy fraction of different UE power consumption states (DL CG + UL pose)

Table 2.3-2-1 below shows XR UE power savings for genie case compared to baseline no DRX

Table 2.3.2-1: XR UE power savings (DL CG 8Mbps + UL pose)

| DRX configuration | Mean PS gain compared to baseline (%) | 95%-tile PS gain (highest Energy) | 5%-tile PS gain (lowest Energy) | Capacity |
| --- | --- | --- | --- | --- |
| No DRX baseline | N/A | N/A | N/A | 13.2 |
| genie | 20% | 14% | 25% | N/A |

Table 2.3.2-2: XR UE power savings (DL AR/VR, CG 30Mbps+ UL pose)

| DRX configuration | Mean PS gain compared to baseline (%) | 95%-tile PS gain (highest Energy) | 5%-tile PS gain (lowest Energy) | Capacity 15ms | Capacity 10ms |
| --- | --- | --- | --- | --- | --- |
| No DRX baseline | N/A | N/A | N/A | 1.9 | 1.1 |
| genie | 19% | 13% | 24% | N/A | N/A |

We make the following preliminary observations from the initial evaluations.

- When DL XR traffic is considered without taking into account the PUSCH impact

  - contribution of “PDCCH monitoring” (i.e., PDCCH monitoring without scheduled data) to overall UE power consumption is larger (e.g. as shown in Figures 2.3.1-4a/4b)

  - Results show that at least for the evaluated cases, appropriate DRX settings can achieve some UE power savings gain with reduction in capacity

    - For CG 8Mbps

      - 12% UEPS gain with 10% capacity loss with Rel15 settings (10ms,5ms,5ms)

      - 27% UEPS gain with 9% capacity loss with enhanced settings (16.66ms,8ms,3ms)

    - For CG 30Mbps

      - 10% UEPS gain with 27% capacity loss with Rel15 settings (10ms,5ms,5ms)

      - 23% UEPS gain with 17% capacity loss with enhanced settings (16.66ms,8ms,3ms)

- When both DL and UL PUSCH XR traffic is considered

  - PUSCH transmissions contribute a significant fraction to overall UE power consumption (e.g. as shown in Figures 2.3.2-3a/3b)

  - The achievable power savings is much reduced i.e., idealistic genie assumption shows only

    - 20% avg power savings potential compared to baseline for CG 8Mbps+UL pose (as shown in Table 2.3.2-1)

    - 19% avg power savings potential compared to baseline for CG 30Mbps+UL pose (as shown in Table 2.3.2-2)

# 3	References

RP-193241, Study on XR Evaluations for NR, Qualcomm, RAN#86, Sitges, December 2019.

R1-2105829, Traffic model for XR, Ericsson, RAN1#105-e, May 2021

# 4	Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz, SCS: 30 kHz, BW: 100 MHz |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (8, 2, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. gNB Antenna has mechanical down-tilt 12 degrees (DU) or 6 degrees (UMa) |
| Duplex method | TDD DDDSU |
| UE Antenna Configuration | (M, N, P) = (1, 2, 2), |
| Link adaptation | Outer loop adaptation + 32-port CSI-RS Type I codebook |
| Scheduling | PF, SU-MIMO |
| Control and RS overhead | Explicit modelling of control and RS overhead |
| Transmission scheme | Up to rank 4 transmission |
| Other potential impairments | Not modelled (assumed ideal) |
| Target BLER | 10% |
| Scenario | Dense Urban (macro-layer only, TR 38.913), 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 20% outdoor Urban macro, 500m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 20% outdoor |
| Channel model | UMa 5G (TR 38.901) |
| BS Tx power | 51dBm (DMa) 56dBm (UMa) |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | Randomly over the system |
| UE receiver | MMSE-IRC |
| CSI feedback | Realistic, including CSI quantization |