3GPP TSG-RAN WG1 Meeting #105e	Tdoc R1-2105832

Online, May 10th – 27th, 2021

Agenda Item:	8.14.4

Source:	Ericsson

Title:	Discussion on enhancements for XR

Document for:	Discussion

# 1	Introduction

In [1], a new SI on evaluations for XR was agreed. In this contribution, we discuss the potential study topics for NR enhancement in capacity, UE power saving and mobility.

# 2	Discussion on Potential Study Areas for XR

## 2.1 Study area: capacity

### 2.1.1 Application and traffic awareness in RAN

RAN has been typically designed to be service agnostic, so the functions and enhancements are usually not linked to a specific service, or application. Having a service-agnostic design offers many benefits and has proven to be a very successful approach. On the other hand, when new services and applications whose traffic characteristics and requirements are different from those of the traditional services and applications need to be supported in RAN, a service-agnostic design sets limitations to what the RAN can do to perform better.

XR services require high bit rate with bounded latency. The high bit rates lead to that a large application data unit (ADU) will be transmitted in several IP packets. When these IP packets arrive to RAN, RAN will treat all the packets as if they are uncorrelated from each other. However, the end-user performance depends on if all these IP packets belonging to a single ADU are successfully delivered. Application information in the RAN can assist the network to plan its resources to deliver the data according to the committed QoS. Thus, radio resource management may benefit of having information about the traffic characteristic of the application. Knowing which packets belong to the same application PDU, or the periodicity of the application PDUs are examples of application information which could be useful for RAN. The lack of application information may lead to sub-optimal radio resource allocation and utilization which will affect the network capacity.

Studying the feasibility and benefits of making available XR application traffic information at RAN/UE could be useful for enabling a more intelligent radio resource management.

Note that RAN1 can study the performance gains of this type of application awareness. However, the actual mechanisms to convey such information would have to be specified by other groups:

This type of study would need to be coordinated with SA2 and SA4 as well as other RAN WGs.

While cloud gaming and VR create a lot of traffic in the DL, AR traffic is dominantly generated by the UE and it creates a considerable amount of UL data with more than one traffic flow. As indicated above, application and traffic information can be beneficial for the network. Current protocols already support some mechanisms to assist the network to obtain information such as the buffer status, for instance. The BSR reports the amount of data buffered at the UE per logical channel group. However, existing BSR formats may not be sufficient for gaining multiple low-latency flow AR traffic information. Logical channel groups aggregate the information of all the logical channel identities mapped into the group. This results in that the network cannot distinguish which concrete logical channel identities have data and how much. In addition, video traffic generates large packet sizes and this will result in that the BSR will indicate high indexes from the existing BSR tables. High indexes result in a larger inaccuracy meaning that the UE buffer is within a value X and value Y, and the difference between Y and X is large. This may lead to that too large grants are provided to the UE, resulting in that the UE must transmit a lot of padding data with a waste of radio resources and UE power.

Knowing detailed and accurate information of AR traffic is key to minimizing the waste of radio resource and to deliver the packets on time. For example, as outlined above, knowing how many packets and which packets belong to a single application PDU can improve the RAN resource allocation.

Timely and accurate application traffic information report by the UE in the UL should be studied to improve capacity by efficiently allocating resource within a bounded latency and reducing waste of radio resources.

### 2.1.2 Support of efficient XR traffic dropping

Efficient XR traffic dropping is one example of how application information can be useful for radio resource management.

The traffic volume of XR application is order of magnitude larger than typical web-browsing and voice communications. This will require substantial amount of radio resources, often leading to high network utilization and long queuing delay. If there is not sufficient resource for multiple users with a large amount of video traffic, there will be growing buffers which result in increasing packet delays which potentially exceed the latency requirement for video rendering. From a video application perspective, it is common that future video frames are encoded based on previous frames to compress the amount of required data. Therefore, if one IP packet belonging to one particular ADU is too late, dropping all relevant IP packets which already arrived to RAN may be beneficial. This can avoid redundant transmissions of IP packets belonging to an ADU which may not be used anyway for rendering. Dropping of too late packets can save expensive radio resource so that the resource can be used for other users which potentially increase the overall system capacity. Therefore, it will be worthwhile studying dropping mechanisms to work efficiently with XR video traffic.

Studying efficient mechanisms of XR video traffic dropping is beneficial for capacity enhancement.

### 2.1.3 Support of efficient scheduling for XR services

Radio resource allocation is one of important areas for studying based on acquired XR traffic information in downlink and uplink. 5G NR already provides a number of tools to support efficient resource allocation via both dynamic and fixed – configured – grants.

XR video traffic is similar to eMBB services in the sense that its application PDU size is varying just as FTP or web browsing. On the other hand, the arrival time or periodicity of the traffic generation is more predictable than the eMBB services. This is due to the fact that video has a fixed frame refresh rate. From this angle, the traffic characteristic is more similar to periodic traffic such as voice and motion control in industries. Just as industrial control applications, XR requires bounded latency, and reasonably high reliability. The traditional mechanisms and strategies to perform resource allocation used for eMBB or for the voice/motion control may then be suboptimal for XR.

To deal with a varying frame size, for example, the dynamic scheduling may be the most flexible and best solution. However, dynamic scheduling comes at the costs of overhead control signalling (PDCCH, SR), and potentially increased latency. Considering the large application PDU size, the network will usually need to allocate several slots to deliver all the packets associated with one application PDU. Investigating how to avoid redundant control signalling overhead for multiple resource allocation would seem relevant:

Multiple resource allocation to handle one large video frame is expected and it would be beneficial to study potential of control signaling reduction for downlink and uplink video traffic.

For predictable arrival times, DL and UL configured grants (SPS and CG) have been specified. However, they may not be suitable to handle large and varying video frame sizes, due to their fixed resource allocation. Instead, these solutions are more suitable for fixed packet sizes. For example, CG may be relevant for pose or for the BSR transmissions resulting from AR video traffic in uplink. Hence, we observe

Dynamic scheduling is more appropriate mechanism to be used for XR video traffic.

SPS/CG alone are not suitable mechanisms to be used for XR video traffic with large and varying packet size, and/or jitter.

CG is relevant to more predictable UL traffic, e.g., pose or BSR triggered by UL video, but arrival pattern matching with existing CG configuration needs to be studied.

### 2.1.4 Link adaptation

Link adaptation has been part of the 3GPP standards for many releases. With link adaptation, the receiver estimates the quality of the channel. Based on this quality estimate, the NW decides which coding and modulation scheme (MCS) to use: if the channel quality is bad, a more robust MCS is chosen, and when the channel quality is good, a less robust coding scheme is used. For the DL, the UE feeds back the channel quality estimates to the NW, and for the UL, the NW directly uses the estimate in the scheduling grant to the UE.

3GPP systems also use HARQ retransmissions. When a TB is retransmitted, the resulting effective coding rate is decreased, and sometimes, HARQ is considered to be implicit link adaptation.

As already mentioned, the NW selects the MCS based on the estimate of the channel quality. If the receiver overestimates the channel quality, the resulting BLER will be high, resulting in HARQ retransmissions. For eMBB services, the additional delay caused by a few HARQ retransmissions is generally not problematic, since eMBB services are not sensitive to the delay. The performance impact of channel quality overestimation is in most cases not significant. However, for services that require bounded latency such as XR, this additional delay could be quite harmful.

One reason why channel quality estimates are inaccurate are interference variations. If the UE estimates the channel quality when the interference is low and performs the corresponding transmissions when the interference is high, there is a large risk that the MCS is not robust enough, causing failed receptions and subsequent HARQ retransmission. The opposite effect could also happen, which leads to that a too robust MCS is chosen, leading to an inefficient use of the channel. This phenomenon is known as the flashlight effect since the UE captures the channel conditions only during a short period of time. Again, for eMBB, HARQ can be used to overcome the detrimental impact of the flashlight effect, at the expense of an increased delay, but such an increased delay may be unacceptable for XR. Hence, we observe

Inaccurate channel quality estimation, caused by, e.g., interference variations may lead to larger delay, and/or inefficient use of the channel.

Hence, improved link adaptation may be beneficial for increased capacity for XR:

Improved link adaptation may be beneficial for increased XR capacity.

We note that improved link adaptation is studied in the Rel-17 URLLC WI. The outcome from this work will serve as a baseline for additional enhancements in Rel-18.

## 2.2 Study area: UE power saving

Low power consumption is essential for some XR devices, e.g., smart glasses. DRX is one of important features which already have been extensively studied and well expected to be used in reality. However, the existing DRX features is well optimized for uncertain traffic arrival of MBB services, e.g. ON duration with inactivity timer. In addition, it is already shown that having DRX will likely increase latency. One potential issue in DRX for XR is that XR traffic pattern is not well aligned with the existing DRX configuration parameters. For example, the XR video arrival interval may not be divisible by the time duration of radio resource allocation unit, leading to the potential mismatch between traffic arrival and the start time of ON duration in DRX. This would result in extra latency. Therefore, studying DRX in terms of compatibility with XR traffic pattern is needed to further decide potential enhancement.

Typical periodicity of burst video frame arrival will lead to mismatch with DRX pattern which result in unnecessary extra latency so that studying DRX pattern matching with common XR video traffic arrival time is useful.

## 2.3 Study area: Mobility

In general, users running VR applications are considered to be relatively static. On the other hand, cloud gaming and advanced AR, however, may be considered as services that can be used on the move. When looking at mobility, the traffic characteristics of the application need to be considered, as well as the service requirements. Legacy handover procedures will lead to an interrupt of some 40ms, which may impact the end user perception of the XR service quality. Multi-connectivity could handle situations with very low or limited mobility, transmitting data in the cell which offers the best radio. Conditional handover (CHO) can be considered in situations in which more robust mobility is desired. More advanced solutions e.g. DAPS can also be considered in cases where there is a need for a very short interruption time. DAPS may then need to be enhanced to operate with DC, CA or in FR2. In general, DAPS would need to be enhanced so that subsequent reconfigurations to e.g. configure features for high bit rate and low latency, are avoided. This issue – avoiding reconfigurations – is actually more generic. RRC reconfigurations take time to be executed. Part of this time is taken by the UE to apply the configuration i.e. RRC processing delay, and another part of the time is required to access the new cell. Note that UE complexity for DAPS is in general high since multiple radio chains are required in the UE.

In Rel-17, improved mobility supported as part of the MIMO WI [2]. Here the idea is to perform parts of the mobility actions on L1/L2, by relying on extensions of the multi-TRP/beam management functionality. An ongoing connection can then be transferred from one TRP to another TRP, even if these TRPs broadcast different PCIs, and thus constitute different cells. It is however foreseen that the Rel-17 functionality will be somewhat limited: only intra-DU operation will be supported, and there will restrictions in the configuration and deployment of the involved cells. On the other hand, the interrupt will be very short, and CA/DC will probably be supported at a reasonable UE complexity.

Introducing new features and capabilities are not the only way to improve the mobility procedures. This is due to the fact that mobility procedures are based on RRC reconfigurations and RRC reconfigurations add a considerable delay due to the RRC processing in the UE. Therefore, the current RRC processing delay requirements in the UE should also be revisited with the aim to enhance the UE mobility.

Mobility procedures and enhancements could take advantage of the traffic characteristics of the application when possible e.g. the periodicity of the traffic.

Methods to have fewer and faster reconfigurations should be studied including tightening the RRC processing delay requirements.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Studying the feasibility and benefits of making available XR application traffic information at RAN/UE could be useful for enabling a more intelligent radio resource management.

Observation 2	This type of study would need to be coordinated with SA2 and SA4 as well as other RAN WGs.

Observation 3	Timely and accurate application traffic information report by the UE in the UL should be studied to improve capacity by efficiently allocating resource within a bounded latency and reducing waste of radio resources.

Observation 4	Studying efficient mechanisms of XR video traffic dropping is beneficial for capacity enhancement.

Observation 5	Multiple resource allocation to handle one large video frame is expected and it would be beneficial to study potential of control signaling reduction for downlink and uplink video traffic.

Observation 6	Dynamic scheduling is more appropriate mechanism to be used for XR video traffic.

Observation 7	SPS/CG alone are not suitable mechanisms to be used for XR video traffic with large and varying packet size, and/or jitter.

Observation 8	CG is relevant to more predictable UL traffic, e.g., pose or BSR triggered by UL video, but arrival pattern matching with existing CG configuration needs to be studied.

Observation 9	Inaccurate channel quality estimation, caused by, e.g., interference variations may lead to larger delay, and/or inefficient use of the channel.

Observation 10	Improved link adaptation may be beneficial for increased XR capacity.

Observation 11	Typical periodicity of burst video frame arrival will lead to mismatch with DRX pattern which result in unnecessary extra latency so that studying DRX pattern matching with common XR video traffic arrival time is useful.

Observation 12	Mobility procedures and enhancements could take advantage of the traffic characteristics of the application when possible e.g. the periodicity of the traffic.

Observation 13	Methods to have fewer and faster reconfigurations should be studied including tightening the RRC processing delay requirements.

# References

RP-193241, Study on XR Evaluations for NR, Qualcomm, RAN#86, Sitges, December 2019.

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019