3GPP TSG RAN WG1 #105	R1-2105887

e-Meeting, May 10th – 27th, 2021

Agenda item:		8.7.1.2

Source:	Nordic Semiconductor ASA

Title:					On TRS design for idle/inactive UEs

Document for:		Discussion and Decision

# Introduction

RAN1#104b-e there has been good progress with Idle TRS design, agreements are summarized below

Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded

In this contribution, we focus on three aspects of design

- How to validate Idle TRS

- Which parameters are configurable and which are fixed

# Validation of a periodic iTRS occasions

Our assumption is that Idle TRS should be facilitating synchronization of a UE waking up to receive Paging DCI + PDSCH. If TRS would be 40ms ahead of paging occasion, they are useless. Therefore, we propose that idle TRS are configured such that all the resources of a resources set are within frame or half-frame prior to PO frame or half-frame.

Proposal-1: TRS are configured 5ms or 10ms before paging frame/half-frame.

- supported periodicities are thus 5ms (half-frame) and 10ms (frame)

Validation of a particular TRS is performed in the PEI. Indication indicates availability/unavailability of TRS within frame or half-frame before paging frame of half-frame, and it indicates an availability/unavailability for all the TRS resources associated with the SSBs indicated in ssb-PositionsInBurst.

Proposal-2: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support

  - Alt1: Availability/unavailability information for a set of configured TRS resources is indicated in the PEI by a single bit.

    - The set of TRS resources is given by TRS resources associated to SSBs indicated in ssb-PositionsInBurst, unless configured by gNB otherwise.

# Configuration of iTRS

## On set of configurable parameters

Feature lead summarized all the parameters for connected CSI-RS configuration. Some parameters have been already agreed as configurable in RAN1#104 for Idle TRS. Some parameters are not applicable to TRS, in general, and for the remaining fields, our opinion is summarized in the below table.

| # | Parameters | Need and details | Overhead |
| --- | --- | --- | --- |
| 1 | bwp-Id | iTRS were agreed not to be restricted within BWP. On the other hand, UE shall be guaranteed with minimum number of RBs for design to be functional. For CORESET#0 of 24 and 48RB, iTRS shall overlap with all PRBs of CORESET#0, for CORESET#0 of size 96 shall span 48RB of CORESET#0 Proposal-3: iTRS shall overlap with - All RBs of 24- and 48-RB CORESET#0 - 48 contiguous RBs of 96-RB CORESET#0 | 0 |
| 2 | resourceType {aperiodic, semiPersistant, periodic} | Field is not needed, as only periodic TRS are supported based on previous agreements. | 0 |
| 3 | repetition {on, off} | Not applicable to TRS | 0 |
| 4 | aperiodicTriggeringOffset | Field not needed, as only periodic TRS are supported | 0 |
| 5 | trs-Info {true} | Not applicable to TRS | 0 |
| 7 | powerControlOffset | Not applicable to TRS | 0 |
| 8 | powerControlOffsetSS | Configurability agreed already | 2 |
| 9 | scramblingID | Configurability agreed already | 10 |
| 10 | periodicityAndOffset | Reduce periodicity to 5ms,10ms only as proposed in P1 | 5bits (FR1) |
| 11 | qcl-InfoPeriodicCSI-RS | Simplify QCL framework to reduce configuration overhead Proposal-4: TCI-state ID points to a SS/PBCH block index only [0-63]. - UE assumes QCL-TypeC' with the SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block | 6 |
| 12 | frequencyDomainAllocation {row1, row2, row4, others} | Proposal-5: For iTRS, support configuration of row1 of frequencyDomainAllocation | 4 |
| 13 | nrofPorts | Not applicable to TRS | 0 |
| 14 | firstOFDMSymbolInTimeDomain | Configurability agreed already | 4 |
| 15 | firstOFDMSymbolInTimeDomain2 | Not applicable to TRS | 0 |
| 16 | cdm-Type | Not applicable to TRS | 0 |
| 17 | density | Not applicable to TRS | 0 |
| 18 | startingRB | Configurability agreed already | 9 |
| 19 | nrofRBs | Configurability agreed already | 8 |
| 20 | subcarrierSpacing (this is not part of CSI-RS resource configuration) | RAN1 agreed that SCS is the one of CORESET#0 | 0 |
| 21 | Others. (please provide any missing/additional parameters) |  | 0 |
|  |  | Total | Total 48bits |

Roughly 50bits are required for single iTRS resource set, and there would have to be one iTRS NZP-CSI-RS resource per transmitted SSB which is above the maximum size a SIB can carry (assuming 64beams). After all necessary parameters are identified, study further how to reduce configuration overhead.

Proposal-6: After all parameters necessary for iTRS are identified, study further how to reduce configuration overhead for iTRS resources.

# Conclusions

In this contribution we discussed iTRS design and we had the following observations and proposals:

Proposal-1: TRS are configured 5ms or 10ms before paging frame/half-frame.

- supported periodicities are thus 5ms (half-frame) and 10ms (frame)

Proposal-2: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support

  - Alt1: Availability/unavailability information for a set of configured TRS resources is indicated in the PEI by a single bit.

    - The set of TRS resources is given by TRS resources associated to SSBs indicated in ssb-PositionsInBurst, unless configured by gNB otherwise.

Proposal-3: iTRS shall overlap with

- All RBs of 24- and 48-RB CORESET#0

- 48 contiguous RBs of 96-RB CORESET#0

Proposal-4: TCI-state ID points to a SS/PBCH block index only [0-63].

- UE assumes QCL-TypeC' with the SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block

Proposal-5: For iTRS, support configuration of row1 of frequencyDomainAllocation

Proposal-6: After all parameters necessary for iTRS are identified, study further how to reduce configuration overhead for iTRS resources.