3GPP TSG RAN WG1 #105-e	R1-2105943

e-Meeting, May 10th - 27th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance for Resource allocation for sidelink - Mode 1

Agenda item:	7.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining issues relevant for mode 1.

# Discussion

## PUCCH resource for sidelink configured grant type 2

At RAN1#104-e we discussed PUCCH slot/resource determination and reached the following agreement:

Agreements:  Send an LS to RAN2 describing that:  Per RAN1 agreements, the parameter sl-N1PUCCH-AN-r16 should be used for SL CG Type 2 (only for PSCCH/PSSCH transmissions without a corresponding PDCCH).  In RAN1’s understanding, the parameter cannot be configured for SL CG type-2 and asking RAN2 whether they can provide a solution that would be compatible with the existing RAN1 agreements.

The LS was sent in R1-2102176. RAN2 replied in R1-2104160 that they defined a new parameter sl-N1PUCCH-AN-Type2-r16 with the following description:

SL-ConfiguredGrantConfig field descriptions

sl-N1PUCCH-AN-Type2 This field indicates the HARQ resource for PUCCH for PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

With the introduction of this new parameter the earlier RAN1 agreement can now be implemented such that the PUCCH resource for HARQ-ACK reporting for subsequent periods of configured grant type 2 can be provided by this new parameter.

Proposal 1: Agree the following text proposal regarding PUCCH resource for sidelink configured grant type 2:

============================= Start of text proposal for 38.213 ==========================

16.5	UE procedure for reporting HARQ-ACK on uplink

<Unchanged parts omitted>

For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. For a SL configured grant Type 2 PSSCH transmission without a corresponding PDCCH a PUCCH resource can be provided by sl-N1PUCCH-AN-Type2. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

============================= End of text proposal for 38.213 ==========================

# Conclusions

In this contribution we discussed remaining editorial issues in sidelink resource allocation mode 1 and made the following proposals:

Proposal 1: Agree the following text proposal regarding PUCCH resource for sidelink configured grant type 2:

============================= Start of text proposal for 38.213 ==========================

16.5	UE procedure for reporting HARQ-ACK on uplink

<Unchanged parts omitted>

For a PSSCH transmission by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSSCH transmission activated by a DCI format, the DCI format indicates to the UE that a PUCCH resource is not provided when a value of the PUCCH resource indicator field is zero and a value of PSFCH-to-HARQ feedback timing indicator field, if present, is zero. For a SL configured grant Type 1 PSSCH transmission, a PUCCH resource can be provided by sl-N1PUCCH-AN and sl-PSFCH-ToPUCCH-CG-Type1. For a SL configured grant Type 2 PSSCH transmission without a corresponding PDCCH a PUCCH resource can be provided by sl-N1PUCCH-AN-Type2. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.

============================= End of text proposal for 38.213 ==========================

# References

- RP-200129, “Revised WID on 5G V2X with NR sidelink”, 3GPP RAN #87e